3GPP TSG-RAN WG3 #117-e							R3-225063

15 - 24 Aug 2022

Online

Agenda Item:	9.2.8

Source:	ZTE - Moderator

Title:	Summary of Offline Discussion on CB: # 54_LRC

Document for:	Approval

- Introduction

CB: # 54_LRC

- Whether such case will happen, and whether there is any IoT issue caused by unexpected propagation of the Area Of Interest List IE between NG-RAN nodes?

(ZTE - moderator)

Summary of offline disc R3-225063

Please Note:

There would be two rounds of email discussion.

The 1st round is to be ended by Friday of first week (23:59 UTC, 19 Aug).

The 2nd round is to be ended 3 hours before the email deadline at second week (9:00 UTC, 23 Aug).

2 For the Chairman’s Notes

Propose to capture the following:

The common understanding is NG-RAN node performs location reporting of AOI when Event type is set as ‘UE presence in the area of interest’ and Area of Interest information is provided.

There is no consensus at this meeting whether it is necessary to specify NG-RAN’s behavior after receiving the area of interest information in NGAP.

Further discussion is contribution driven.

(NGAP CR R3-224884 rev in R3-225134 is noted )

4  Discussion (2nd round)

According to the first round discussion, most companies have acknowledged the possibility of the issue discussed and think it should be specified clearly about the behavior of NG-RAN node after receives the area of interest in NGAP to avoid problems. After all, It is beneficial to have the correction on NGAP to prevent unnecessary behaviour like storing the area of interest information when it is not useful and prevent unexpected propagation of the useless information.

Based on companies’ selection in the first round, Moderator propose the following:

Proposal 1: It is possible that AMF might send the area of interest with a reporting type other than ‘UE presence in the area of interest’.

Proposal 2: It should be specified in NGAP on the behavior of NG-RAN node after receiving the area of interest, to avoid any potential problem or confusion, for example, caused by bad implementation.

Proposal 3: Revise R3-224884 into only one single correction (procedure text) and agree on the revised CR (rev in R3-225134)

The draft revision is in the 2nd round folder, companies are welcome to provide comments.

Pls provide your comments if any.

| Company | Comments |
| --- | --- |
| Ericsson | We should focus on to “if the location reporting of AOI” is clearly specified. In our view it is already very clear as per TS 23.502, and also TS 23.501. After the first round of discussion, it is still unclear what the problem/confusion could be caused. The proposed CR is NBC in the specification. If the companies have concern, we could capture in Chainman notes a common understanding: It is a common understanding that NG-RAN node performs location reporting of AOI when Event type is set as “UE presence in the area of interest” and “Area Of Interest information” is provided. |
| Nokia | No. We are not convinced about the issue and the need for CR. Please check 23.501/2, it clearly defines the behavior when the Event type is set as “UE presence in the area of interest” and “Area Of Interest information” is provided. So it is clear when gNB can start to track the UE. if a gNB is implemented to track the UE only based one of the event type and AOI, it is not standard complaint. RAN3 should not consider such type of gNB. |

Summary:

Still two companies are against the correction propose, with the concern that the correction might result in NBC issue for some NG-RAN nodes with different implementation.

Moderator would like to accept the proposal in the comment of Ericsson as a way forward at this meeting. But since 5 companies are in favour of the NGAP correction, it is proposed to be captured as an issue to be continued at next meeting.

The proposal 2 and proposal 3 are revised into:

Proposal 2: It is a common understanding that NG-RAN node performs location reporting of AOI when Event type is set as ‘UE presence in the area of interest’ and Area of Interest information is provided.

Proposal 3:

Whether it is necessary to specify NG-RAN’s behavior after receiving the area of interest information in NGAP.

To be continued...

3 Discussion (1st round)

It is proposed in [1] that some procedure text or IE description should be added for the handling of Area of Interest List IE when RAN receives the Location Report Control message from AMF. The intention is to clarify in RAN3 spec that the NG-RAN shall store the received Area of Interest List IE when the event type IE is set to ‘UE presence in the area of interest’, to make the RAN3 spec aligns with SA2 spec (23.502).

During the online discussion, there was no consensus on whether the concerned case proposed in the CR would happen. According to the specification of SA2, it only says that ‘If the Reporting Type indicates to start the NG-RAN to report when UE moves out of or into the Area Of Interest, the AMF also provides the requested Area Of Interest information in the Location Reporting Control message’, but it does not preclude any other cases, for example, AMF may set the Request type to other values and provide the area of interest as well. The text in 23.502 is pasted below:

1. AMF to NG-RAN: Location Reporting Control (Reporting Type, Location Reporting Level, (Area Of Interest, Request Reference ID)). The AMF sends a Location Reporting Control message to the NG-RAN. The Location Reporting Control message shall identify the UE for which reports are requested and shall include Reporting Type and Location Reporting Level. The Location Reporting Control message may also include Area Of Interest and Request Reference ID. Location Reporting Level could be TAI+ Cell Identity. Reporting Type indicates whether the message is intended to trigger a single standalone report about the current Cell Identity serving the UE or start the NG-RAN to report whenever the UE changes cell, or ask the NG-RAN to report whenever the UE moves out or into the Area Of Interest. If the Reporting Type indicates to report whenever the UE changes cell and if PScell reporting is requested and Dual Connectivity is in use, the Master RAN node shall also report to the AMF whenever the PSCell changes. If the Reporting Type indicates to start the NG-RAN to report when UE moves out of or into the Area Of Interest, the AMF also provides the requested Area Of Interest information in the Location Reporting Control message. The AMF may include a Request Reference ID in the Location Report Control message to identify the request of reporting for an Area Of Interest. If multiple Areas Of Interest are included in the message, the Request Reference ID identifies each Area of Interest.

According to 23.502, it can only be assumed that ‘UE presence in area of interest’ + ‘Area of interest’ is a right pair to the RAN node, which would usually be the case, if it is correctly settled by AMF. But there is not a clue in SA2 specification that AMF would never send the area of interest with another Request type; hence RAN3 cannot make such an assumption that AMF would always send the right request type for area of interest.

Please companies provide views on this question below.

Q1: Will it happen that the Area of Interest List IE is present, with a reporting type other than ‘UE presence in the area of interest’?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | According to SA2 spec, the case that area of interest is sent with another request type is not precluded. There is not a clue in SA2 specification that AMF would never send the area of interest with another Request type. RAN3 cannot make such an assumption that AMF would always send the right request type for area of interest. |
| Huawei | Possibly, but should be avoided by “good” AMF implementation. | With the above procedure texts referred to the TS 23.502, a “good” AMF implementation should avoid its happening. So we understand the above may possibly happen, but should be avoided. |
| Nokia | No | This does not happen for an AMF which is implemented according to 23.502. |
| Qualcomm | Possible with a faulty node | Any Faulty Node can do that. |
| Ericsson | Only with a faulty implementation. | A faulty implementation which does not follow the specification could make many other mistakes … |
| Samsung | Possibly, but should be avoided by “good” AMF implementation. | We usually specify the receiving node behavior to ensure the good implementation. |
| CATT | Possible |  |

Summary:

7 companies commented.

1 company think it is impossible according to 23.502.

6 companies think it possible, e.g., by faulty implementation, in which one company pointed out that we should specify the receiving node behaviour to ensure good implementation.

There was not a proof that AMF would always send the area of interest with the right reporting type. So Moderator think it can be assumed that the case discussed in this question is possible.

Another argument is about whether there would be any IoT issue caused by the unexpected propagation of the useless information. Let’s consider the following case to further evaluate this issue.

If there are two NG-RAN nodes with different kinds of implementation based on their own understanding of the specifications:

- Implementation of NG-RAN node 1: no matter what the reporting type is, NG-RAN shall store the area of interest.

- Implementation of NG-RAN node 2: when the reporting type is ‘UE presence in the area of interest’, NG-RAN stores the area of interest and use it to track UE’s location.

And if for some reason, AMF sends the area of interest to NG-RAN with another reporting type to NG-RAN, the NG-RAN node 1 would store the area of interest and it is possible that this information might be propagated to NG-RAN node 2.

In this case, once the area of interest if propagated from NG-RAN node 1 to NG-RAN node 2, while the reporting type is not about UE presence in the area of interest, the NG-RAN node 2 would be confused and deem that the NG-RAN node 1 is problematic. This is a severe IoT issue that should definitely be avoided.

Companies are welcome to provide more opinions on any other IoT issue it may cause.

Q2: Whether there is any IoT issue caused by unexpected propagation of the Area Of Interest List IE between NG-RAN nodes?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | There would be IoT issue if the implementation of NG-RAN nodes is different, based on the designers’ different understanding on the specifications, as discussed above. So it should be clarified in RAN3 spec that when the reporting type is ‘UE presence in the area of interest’, NG-RAN node shall store the information and use it for UE location tracking. This problem due to different implementation should definitely be solved in RAN3. Maybe except from the current correction in the CR[1], a cause value is also needed in the LOCATION REPORTING FAILURE INDICATION, to notify that about the unmatched reporting type and area of interest. |
| Huawei | Not an IoT issue, but an implementation issue | We understand if this happens, the NG-RAN node should “ONLY when the reporting type is ‘UE presence in the area of interest’, NG-RAN stores the area of interest and use it to track UE’s location”. If the reporting type “UE presence in the area of interest” is not indicated, the NG-RAN should discard the area of interest. |
| Nokia | No | The NG-RAN node behavior shall follow 23.502. The above “Implementation of NG-RAN node 2” is not standard complaint. Moderator’s reply: the implementation of NG-RAN node 1 and NG-RAN node 2 are both possible if we do not specify clearly in NGAP about when the NG-RAN node shall store the area of interest. |
| Qualcomm | See comments | If a faulty node sends Areas of Interest List IE, then a good node should ignore it. If the receiver is again a faulty node, then the expected results might differ. |
| Ericsson |  | We are not convinced that there would be any issue at all. If the Event Type is not “UE presence in the area of interest’, nothing would happen related to area of interest reporting. Moderator’s reply: but the behavior of the receiver node differs, i.e., to store the information, or to discard it. This should be specified in NGAP to avoid bad implementation or confusion. |
| Samsung | See comments | We think storing useless information in NG-RAN should be avoided. If we specify the receiving node more accurate, the NG-RAN will store the information only when both “UE presence in the area of interest” and “Area of interest List” are included. In other cases, the NG-RAN doesn’t need to store and propagation to the target. |
| CATT |  | If we limit in the specification the condition that this information applied, there would be no problem. |

Summary:

7 companies commented.

1 company said yes, and 1 company said no.

1 company think there is not any issue.

4 companies showed concerns on the potential issue, among which 2 companies mentioned bad implementation issue, and 2 companies hold the view that NG-RAN behavior after receiving the area of interest information should be specified, to avoid any potential problem.

Most of companies are aware of the issue caused by the unclear behavior of NG-RAN node after receives the area of interest and think it should be specified in NGAP that NG-RAN shall store the area of interest information only when both “UE presence in the area of interest” and “Area of interest List” are included.

If RAN3 is to clarify the handling of area of interest in the RAN side, there are two kinds of corrections on 38.413 proposed in [1]:

- Option 1: modify the procedure text in section 8.12.1.2

If the Area Of Interest List IE is included in the Location Reporting Request Type IE in the LOCATION REPORTING CONTROL message, and if the Event Type IE in the Location Reporting Request Type IE is set to ‘UE presence in the area of interest’, the NG-RAN node shall store this information and use it to track the UE's presence in the area of interest as defined in TS 23.502 [10].

- Option 2: add a semantic description for area of interest List IE in section 9.3.1.65

| Area of Interest List |  | 0..1 |  | This IE only applies when the Event Type IE is set to ‘UE presence in the area of interest’. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

Q3: Which option above do you prefer?

| Company | Opt | Comments |
| --- | --- | --- |
| Huawei | Option 1 |  |
| Nokia | None | As commented above, no change is needed, since no IOT issue. |
| Qualcomm | Option 1 |  |
| Ericsson | None | Since it is clear in the specification and there is no issue, we should not change the specification. Please note that “the receiver side” behavior for this Optional IE is already correctly specified. |
| Samsung | Option 1 | Actually no strong view on option 1 or option 2. |
| ZTE | Option 1or 2 | Slightly prefer option 1 |
| CATT | Option 2 is preferred. However, option 1 is also acceptable to us. |  |

Summary:

7 companies commented.

2 companies selected option 1.

2 companies slightly prefer option 1 but no strong view.

1 companies preferred option 2 but can also accept option 1.

2 companies said none.

According to the discussion above, it is clear that most of the companies think it would cause problems if the NG-RAN node behavior after receiving area of interest. Based on the selections from companies, option 1 is the most acceptable one. So moderator would propose to revise [1] into a CR with the correction in option 1 and to agree on the revised CR.

Q4: Please provide here if you have any other suggestions on the corrections for NGAP.

| Company | Comments |
| --- | --- |

4 Conclusion, Recommendations

See section 2.

5 References

- R3-224884, Correction on Location reporting control (NGAP) (ZTE, Huawei, CATT, Samsung)