3GPP TSG-RAN WG3 Meeting #117-bis-e	R3-225509

E-meeting, 10 October – 18 October 2022

Title: 	(TP for AI/ML BLCR to TS38.423) Procedures for exchanging AI/ML-related information

Source: 	Ericsson, Inter Digital

Agenda item:	12.2.2.1

Document Type:	Discussion and Decision

# Introduction

The general procedure for exchanging AI/ML-related information in the RAN was discussed during the RAN3#117-e meeting, and the following agreements and open issues were captured:

Define a new procedure over Xn which can be used for AI/ML related information, e.g., predicted information

FFS whether to use the existing procedure or dedicated new procedure for other input, output and feedback information.

The new procedure for reporting of AI/ML related information, e.g., predicted information, should be based in a requested way, like resource status report procedure.

The new procedure over Xn used for AI/ML related information should be non-UE associated as a start point.

FFS on whether UE associated procedure is needed.

In this paper, we elaborate on the open issues related to information exchange for AI/ML in the RAN, regarding the standard impacts arising thereof.

# New XnAP procedures for exchanging AI/ML information

## Previous agreements and open issues

According to previous meetings progress, RAN3 agreed on defining a new procedure over Xn to exchange AI/ML-related information:

Define a new procedure over Xn which can be used for AI/ML related information, e.g., predicted information

Furthermore, it was also agreed that the new procedure for signalling the said AI/ML-related information should follow the general paradigm of a subscription-reporting mechanism:

The new procedure for reporting of AI/ML related information, e.g., predicted information, should be based in a requested way, like resource status report procedure.

Observation 1:  RAN3 has agreed on defining a new XnAP procedure for reporting AI/ML-related information. The new procedure should be based on a subscription-reporting mechanism, like the Xn: Resource Status Reporting procedure.

To provide the flexibility required by AI/ML algorithms and to guarantee future-proofness, we propose to introduce new procedures to assist AI/ML in RAN, defined in a use-case and information agnostic manner. The node which receives the request, i.e., the reporting node, does not need to know for which use case the data is needed or if it will be used as input or feedback for instance. The only thing it needs to know is what data should be reported. The latter is in line with the agreements taken during the study phase, where it was specified that processes like data preparation and validation are done at the Model Training/Inference function.

For the reasons above, the procedures should follow the general paradigm of a subscription-reporting mechanism, regardless of whether the type of information being transferred is an input(for the requesting node), an output (of the AI/ML model at the sending node), or feedback (for the requesting node). This can be an advantage if we consider that sometimes the very same information can be used for different purposes. Additionally, this would provide an NG-RAN node the necessary flexibility to subscribe for and receive/report, with the same procedure, different types of information needed by different AI/ML use cases.

Proposal 1:  The new procedure should be use-case agnostic. In this way, if the same information is needed for several use cases, it only needs to be signalled once.

The first FFS related to the new procedure is the following:

FFS whether to use the existing procedure or dedicated new procedure for other input, output and feedback information.

Similar to the above explanation, we propose that the new procedure be the same regardless of whether the type of information being transferred is an input, an output, or feedback. This avoids the duplication of signalling of information that is used for different purposes, e.g., as input for one AI/ML process and as feedback for another AI/ML process.

Proposal 2:  The new procedure should be information agnostic, i.e., it should be the same regardless of whether the type of information being transferred is an input, an output, or feedback. For example, if the same information is used as input and as feedback, it only needs to be signalled once.

The final agreement and open issue related to the new procedure are the following:

The new procedure over Xn used for AI/ML related information should be non-UE associated as a start point.

FFS on whether UE associated procedure is needed.

In this matter, we argue that a different UE-associated procedure is not needed as UE-specific information can be carried in the agreed non-UE-associated procedure. Note that adding UE-specific information in a non-UE associated procedure is a solution already adopted in NG-RAN specifications, see the ACCESS AND MOBILITY INDICATION procedure over Xn, where UE specific RACH reports are signalled, together with the UE XnAP ID of the UE. Furthermore, if RAN3 would agree to defining a new UE-associated procedure, this would result in a more complex system with multiple procedures for subscribing and reporting AI/ML-related information. A more complex system is more prone to errors.

Proposal 3:  UE-associated information can be carried in the new non-UE-associated procedure agreed by RAN3. This results in a simpler and more robust system.

We identified, however, one case where AI/ML related information may be signalled within an existing UE associated procedure and we do not exclude that more cases may be identified in the future. This is the case of the UE Trajectory Prediction information. In this case, the information is needed at the time of UE handover preparation, hence it seems plausible that the information is signalled as part of the existing HO signalling messages.

Proposal 4:  UE Trajectory predictions are needed at the time of HO preparation, and for this, they can be included in the legacy HO Signalling messages.

Following these agreements, we propose to introduce two new procedures to assist AI/ML in the RAN, defined in a use-case and information agnostic manner. The new procedures resemble the existing Resource Status Reporting Initiation and Resource Status Reporting procedures, and at the same time they can be designed to be more flexible, to satisfy the needs of future AI/ML processes. These procedures are:

- a new non-UE associated procedure, for handling the subscription mechanism. A tentative name for this procedure can be AIML Assistance Data Reporting Initiation.

- a new non-UE associated procedure, for handling the collection of subscribed information. A tentative name for this procedure can be AIML Assistance Data Reporting.

Proposal 5:  Introduce two dedicated non-UE-associated procedures in XnAP for collecting AI/ML assistance data, based on Resource Status Reporting Initiation and Resource Status Reporting:

- one procedure to handle the subscription for requesting data (“AIML Assistance Data Reporting Initiation”), i.e. where an NG-RAN node requests a neighbour node for reporting of AI/ML information

- one procedure to enable the reporting node to send the requested data (“AIML Assistance Data Reporting”)

## Possible implementation

As an example, the AI-ML ASSISTANCE DATA REQUEST message, which is part of the AIML Assistance Data Reporting Initiation procedure, is shown here:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the assistance data is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement object the NG-RAN node 2 is requested to report. First Bit = Predicted Radio Resources Periodic, Second Bit = Current Energy Efficiency Periodic, Third Bit = Predicted Energy Efficiency Periodic (FFS), Fourth Bit = UE Performance Indicator Periodic. Other bits shall be ignored by the NG-RAN node 2. | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting. | YES | ignore |

As a further example, the AI-ML ASSISTANCE DATA UPDATE message, which is part of the AIML Assistance Data Reporting procedure, is shown below. We note the proposed UE Assistant Identifier IE that can be used to signal UE-associated information in this non-UE-associated procedure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Energy Efficiency | O |  | INTEGER (0..100) | Value 1 indicates the minimum measured Energy Efficiency and 100 indicates the maximum measured Energy Efficiency. Energy Efficiency should be measured on a linear scale |  |  |
| Predicted Energy Efficiency (presence of this IE is FFS) | O |  | INTEGER (0..100) | Value 1 indicates the minimum Predicted Energy Efficiency and 100 indicates the maximum Predicted Energy Efficiency. Predicted Energy Efficiency should be measured on a linear scale |  |  |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.3.xx | Indicates the predicted values of reported resource measurements |  |  |
| >>UE Performance Indicator List | O | 1 .. < maxnoofFeedbackUEs > |  |  |  |  |
| >>>UE Performance Measurements |  |  | 9.2.3.yy |  |  |  |
| >>>UE Assistant Identifier | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | ignore |

One important detail is how the Predicted Radio Resources IE should be represented. The simplest way would be to define the Predicted Radio Resources IE as a sequence of predicted values for the IEs reported in the Xn: Resource Status Update message. One example of how the Predicted Radio Resources IE may look like is shown below:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | IE/Group Name | Presence |
| --- | --- | --- | --- | --- | --- | --- |
| Predicted Radio Resource Status | O |  | 9.2.2.50 | Predicted value of the Radio Resource Status IE | – |  |
| Predicted TNL Capacity Indicator | O |  | 9.2.2.49 | Predicted value of the TNL Capacity Indicator IE | – |  |
| Predicted Composite Available Capacity Group | O |  | 9.2.2.51 | Predicted value of the Composite Available Capacity Group IE | – |  |
| Predicted Slice Available Capacity | O |  | 9.2.2.55 | Predicted value of the Slice Available Capacity IE | – |  |
| Predicted Number of Active UEs | O |  | 9.2.2.62 | Predicted value of the Number of Active UEs IE | –- |  |
| Predicted RRC Connections | O |  | 9.2.2.56 | Predicted value of the RRC Connections IE | – |  |

While the above tabulars are not definitive and need to be discussed in RAN3 and refined, they can be taken as a baseline for further work.

Proposal 6:  Take as baseline the above tabulars for the definition of the AIML Assistance Data Reporting Initiation and AIML Assistance Data Reporting, resembling the XnAP Resource Status Reporting Initiation and Resource Status Reporting procedures.

# Conclusion

In this contribution, we have discussed the agreements and the open issues related to information exchange for AI/ML in the RAN. Corresponding observations and proposals are reported below.

Observation 1:  RAN3 has agreed on defining a new XnAP procedure for reporting AI/ML-related information. The new procedure should be based on a subscription-reporting mechanism, like the Resource Status Reporting procedure.

Proposal 1:  The new procedure should be use-case agnostic. In this way, if the same information is needed for several use cases, it only needs to be signalled once.

Proposal 2:  The new procedure should be information agnostic, i.e., it should be the same regardless of whether the type of information being transferred is an input, an output, or feedback. For example, if the same information is used as input and as feedback, it only needs to be signalled once.

Proposal 3:  UE-associated information can be carried in the new non-UE-associated procedure agreed by RAN3. This results in a simpler and more robust system.

Proposal 4:  UE Trajectory predictions are needed at the time of HO preparation, and for this, they can be included in the legacy HO Signalling messages.

Proposal 5:  Introduce two dedicated non-UE-associated procedures in XnAP for collecting AI/ML assistance data, based on Resource Status Reporting Initiation and Resource Status Reporting:

- one procedure to handle the subscription for requesting data (“AIML Assistance Data Reporting Initiation”), i.e. where an NG-RAN node requests a neighbour node for reporting of AI/ML information

- one procedure to enable the reporting node to send the requested data (“AIML Assistance Data Reporting”)

Proposal 6:  Take as baseline the above tabulars for the definition of the AIML Assistance Data Reporting Initiation and AIML Assistance Data Reporting, resembling the XnAP Resource Status Reporting Initiation and Resource Status Reporting procedures.

A CR to TS 38.423, mirroring the proposals above, is available in the appendix.

# Appendix - CR for TS 38.423

3GPP TSG-RAN3 #117-bis-e	R3-22xxxx

10th October – 18th October 2022

Online

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR |  | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BLCR to TS38.423: Support for AI/ML in NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN-Core |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to fulfil the objectives of the WID in RP-220635 (NR_AIML_NGRAN-Core) a number of changes are required. This CR implements such changes |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support for AI/ML based mechanisms in support for Network Energy Saving, Load Balancing and Mobility Optimisation use cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AI/ML based procedures and functions cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<< Start of CHANGEs >>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5QI	5G QoS Identifier

AI	Artificial Intelligence

AMF	Access and Mobility Management Function

BH	Backhaul

CAG	Closed Access Group

CGI	Cell Global Identifier

CHO	Conditional Handover

CP	Control Plane

CPA	Conditional PSCell Addition

CPAC	Conditional PSCell Addition or Change

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DL	Downlink

EN-DC	E-UTRA-NR Dual Connectivity

E-RAB	E-UTRAN Radio Access Bearer

GUAMI	Globally Unique AMF Identifier

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

MBS	Multicast/Broadcast Service

MCG	Master Cell Group

ML	Machine Learning

M-NG-RAN node	Master NG-RAN node

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NSSAI	Network Slice Selection Assistance Information

PEIPS	Paging Early Indication with Paging Subgrouping

PNI-NPN	Public Network Integrated Non-Public Network

ProSe 	Proximity Services

RANAC	RAN Area Code

RedCap	Reduced Capability

RSN	Redundancy Sequence Number

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SNPN	Stand-alone Non-Public Network

S-NG-RAN node	Secondary NG-RAN node

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

SDT	Small Data Transmission

TAC	Tracking Area Code

TAI	Tracking Area Identity

UL	Uplink

UPF	User Plane Function

V2X	Vehicle-to-Everything

<<<<<< NEXT CHANGE >>>>>>

# 8	XnAP procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| IAB Transport Migration Management | IAB TRANSPORT MIGRATION MANAGEMENT REQUEST | IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE | IAB TRANSPORT MIGRATION MANAGEMENT REJECT |
| IAB Transport Migration Modification | IAB TRANSPORT MIGRATION MODIFICATION REQUEST | IAB TRANSPORT MIGRATION MODIFICATION RESPONSE |  |
| IAB Resource Coordination | IAB RESOURCE COORDINATION REQUEST | IAB RESOURCE COORDINATION RESPONSE |  |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE | PARTIAL UE CONTEXT TRANSFER FAILURE |
| AI-ML Assistance Data Reporting Initiation | AI-ML ASSISTANCE DATA REQUEST | AI-ML ASSISTANCE DATA RESPONSE | AI-ML ASSISTANCE DATA FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| RAN Multicast Group Paging | RAN MULTICAST GROUP PAGING |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Retrieve UE Context Confirm | RETRIEVE UE CONTEXT CONFIRM |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| AI-ML Assistance Data Reporting | AI-ML ASSISTANCE DATA UPDATE |

<<<<<< NEXT CHANGE >>>>>>

### 8.4.xx	AI-ML Assistance Data Reporting Initiation

#### 8.4.xx.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML assistance data to another NG-RAN node.

The procedure uses non UE-associated signalling.

#### 8.4.xx.2	Successful Operation

Figure 8.4.xx.2-1: AI-ML Assistance Data Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the AI-ML ASSISTANCE DATA REQUEST message to NG-RAN node2 to start AI/ML assistance data reporting, stop assistance data reporting, or add cells to report for assistance data. Upon receipt, NG-RAN node2:

-	shall initiate the requested assistance data reporting according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells assistance data reporting and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report List IE to the assistance data reporting initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If assistance data reporting is already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored.

If the Registration Request IE is set to "start" in the AI-ML ASSISTANCE DATA REQUEST message and the Report Characteristics IE indicates cell specific assistance data, the Cell To Report List IE shall be included.

If the Registration Request IE is set to "add" in the AI-ML ASSISTANCE DATA REQUEST message, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all requested AI/ML assistance data, it shall initiate the assistance data reporting as requested by NG-RAN node1 and respond with the AI-ML ASSISTANCE DATA RESPONSE message.

Interaction with other procedures

When starting AI/ML assistance data reporting, the Report Characteristics IE in the AI-ML ASSISTANCE DATA REQUEST indicates the type of objects NG-RAN node2 shall perform assistance data reporting on. For each cell, NG-RAN node2 shall include in the AI-ML ASSISTANCE DATA UPDATE message:

-	the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resources Periodic" of the Report Characteristics IE included in the AI-ML ASSISTANCE DATA REQUEST message is set to "1";

- 	the UE Performance Indicator List IE, if the fourth bit, "UE Performance Indicator Periodic" of the Report Characteristics IE included in the AI-ML ASSISTANCE DATA REQUEST message is set to "1".

For the NG-RAN node2, NG-RAN node2 shall include in the AI-ML ASSISTANCE DATA UPDATE message:

-	the Energy Efficiency IE, if the second bit, "Current Energy Efficiency Periodic" of the Report Characteristics IE included in the AI-ML ASSISTANCE DATA REQUEST message is set to "1";

-	the Predicted Energy Efficiency IE (presence of this IE is FFS), if the third bit, "Predicted Energy Efficiency Periodic" of the Report Characteristics IE included in the AI-ML ASSISTANCE DATA REQUEST message is set to "1".

If the Reporting Periodicity IE in the AI-ML ASSISTANCE DATA REQUEST is present, this indicates the periodicity for the reporting of AI/ML assistance data. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

#### 8.4.xx.3	Unsuccessful Operation

Figure 8.4.xx.3-1: AI-ML Assistance Data Reporting Initiation, unsuccessful operation

If any of the requested assistance data cannot be initiated, NG-RAN node2 shall send the AI-ML ASSISTANCE DATA FAILURE message with an appropriate cause value.

#### 8.4.xx.4	Abnormal Conditions

For the same Measurement ID, if the initiating NG-RAN node1 does not receive either the AI-ML ASSISTANCE DATA RESPONSE message or the AI-ML ASSISTANCE DATA FAILURE message, the NG-RAN node1 may reinitiate the AI-ML Assistance Data Reporting Initiation procedure towards the same NG-RAN node, provided that the content of the new AI-ML ASSISTANCE DATA REQUEST message is identical to the content of the previously unacknowledged AI-ML ASSISTANCE DATA REQUEST message.

If the NG-RAN node2 receives an AI-ML ASSISTANCE DATA REQUEST message which includes the Registration Request IE set to "add" or "stop" and if the NG-RAN node2 Measurement ID value received in the AI-ML ASSISTANCE DATA REQUEST message is not used, the NG-RAN node2 shall initiate AI-ML ASSISTANCE DATA FAILURE message with an appropriate cause value.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the AI-ML ASSISTANCE DATA REQUEST message, then NG-RAN node2 shall initiate an AI-ML ASSISTANCE DATA FAILURE message with an appropriate cause value.

If the NG-RAN node2 receives an AI-ML ASSISTANCE DATA REQUEST message which includes the Registration Request IE set to "start" and the NG-RAN node1 Measurement ID IE corresponding to an existing on-going assistance data reporting, then NG-RAN node2 shall initiate an AI-ML ASSISTANCE DATA FAILURE message with an appropriate cause value.

### 8.4.yy	AI-ML Assistance Data Reporting

#### 8.4.yy.1	General

This procedure is initiated by an NG-RAN node to report the results of assistance data admitted by the NG-RAN node following a successful AI-ML Assistance Data Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.4.yy.2	Successful Operation

Figure 8.4.yy.2-1: AI-ML Assistance Data Reporting, successful operation

NG-RAN node2 shall report the results of the admitted assistance data in AI-ML ASSISTANCE DATA UPDATE message. The admitted assistance data were successfully initiated during the preceding AI-ML Assistance Data Reporting Initiation procedure.

#### 8.4.yy.3	Unsuccessful Operation

Not applicable.

#### 8.4.yy.4	Abnormal Conditions

Void

<<<<<< NEXT CHANGE >>>>>>

#### 9.1.3.xx	AI-ML ASSISTANCE DATA REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested assistance data reporting in support to AI/ML functions, according to the parameters given in the message.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the assistance data is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement object the NG-RAN node 2 is requested to report. First Bit = Predicted Radio Resources Periodic, Second Bit = Current Energy Efficiency Periodic, Third Bit = Predicted Energy Efficiency Periodic (FFS), Fourth Bit = UE Performance Indicator Periodic. Other bits shall be ignored by the NG-RAN node 2. | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.1.3.ww	AI-ML ASSISTANCE DATA RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested assistance data, for all of the objects included in the assistance data request, is successfully initiated.

Direction: NG-RAN node2 ® NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.yy	AI-ML ASSISTANCE DATA FAILURE

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for any of the requested assistance data the measurement cannot be initiated.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.zz	AI-ML ASSISTANCE DATA UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested assistance data.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Energy Efficiency | O |  | INTEGER (0..100) | Value 1 indicates the minimum measured Energy Efficiency and 100 indicates the maximum measured Energy Efficiency. Energy Efficiency should be measured on a linear scale |  |  |
| Predicted Energy Efficiency (presence of this IE is FFS) | O |  | INTEGER (0..100) | Value 1 indicates the minimum Predicted Energy Efficiency and 100 indicates the maximum Predicted Energy Efficiency. Predicted Energy Efficiency should be measured on a linear scale |  |  |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.3.xx | Indicates the predicted values of reported resource measurements |  |  |
| >>UE Performance Indicator List | O | 1 .. < maxnoofFeedbackUEs > |  |  |  |  |
| >>>UE Performance Measurements | M |  | 9.2.3.yy |  |  |  |
| >>>UE Assistant Identifier | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofFeedbackUEs | Maximum no. of UEs reporting feedback information. Value is FFS |

<<<<<< NEXT CHANGE >>>>>>

#### 9.2.3.xx	Predicted Radio Resources

This IE provides prediction values for Radio Resource parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | IE/Group Name | Presence |
| --- | --- | --- | --- | --- | --- | --- |
| Predicted Radio Resource Status | O |  | 9.2.2.50 | Predicted value of the Radio Resource Status IE | – |  |
| Predicted TNL Capacity Indicator | O |  | 9.2.2.49 | Predicted value of the TNL Capacity Indicator IE | – |  |
| Predicted Composite Available Capacity Group | O |  | 9.2.2.51 | Predicted value of the Composite Available Capacity Group IE | – |  |
| Predicted Slice Available Capacity | O |  | 9.2.2.55 | Predicted value of the Slice Available Capacity IE | – |  |
| Predicted Number of Active UEs | O |  | 9.2.2.62 | Predicted value of the Number of Active UEs IE | – |  |
| Predicted RRC Connections | O |  | 9.2.2.56 | Predicted value of the RRC Connections IE | – |  |

#### 9.2.3.yy	UE Performance Measurements

The UE Performance Measurements IE indicates performance measurements for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | 9.2.3.4 | Average overall user plane UE throughput in DL |
| Average UE Throughput UL | O |  | 9.2.3.4 | Average overall user plane UE throughput in UL |
| Average Packet Delay | O |  | FFS | Average value for the delay that a packet may experience. |
| Average Packet Error Rate | O |  | 9.2.3.13 | Average Packet Error Rate |
| Average RAN Visible QoE | O |  | FFS | Average RAN Visible QoE experienced by the UE on all the active services for which QoE is collected at the time of reporting |
| UE Energy Consumption | O |  | FFS | The energy consumption experienced by the UE at the time of reporting |