3GPP TSG-RAN WG3 Meeting #117bis-e	R3-225596

E-meeting, 10 – 18 October, 2022

Agenda item:	9.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Correction of 'set of NR cells' in UHI

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#116-e, an issue related to unclear definition of Last Visited NG-RAN Cell Information entry in UE History Information (UHI) was raised in [1]. We share the view with the authors of [1] that the current definition is unclear with risk of misoperation, and we therefore in this paper provide some background information from Rel-15 discussions and propose a corresponding correction.

# 2	Discussion

The issue relates to the following description of TS 38.413 clause 9.3.1.97:

#### 9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

The wording was introduced in RAN3 July adhoc meeting in 2018 and published in NGAP v15.1.0. In the SoD (R3-184235) is written: “The rationale of this clarification is related to the change introduced in the cell definition last meeting thanks to RAN2. A UE can be reconfigured to change the SSB, i.e cell in same gNB without any move… That’s why we characterized here the change of “physical” cell related to mobility and HO that the UE History is subject to solve.”

The intention behind the wording “set of NR cells” was therefore to describe the set of logical cells (CGIs) sharing the same physical cell in order to avoid that we don’t store changes that represent inter-PLMN HO within the same physical cell. This to avoid creating an entry in the UE history that could look like physical UE mobility, while actually only the PLMN was changed (it is still a question whether this is a very frequent scenario…). This means that the term "set of cells" was intended to correspond to the cells included in the CellAccessRelatedInfo IE in SIB1 (see TS 38.331).

Observation: The term "set of cells" was intended to correspond to the cells included in the CellAccessRelatedInfo IE in SIB1.

Additionally, probably for the purpose of keeping similar functionality as E-UTRAN, the wording “with the same NR ARFCN for reference point A” was to make clear that the new entries resulting from inter-frequency mobility (which may occur without physical UE mobility) are stored in the UHI.

However, due to absence of further definition of the term “set of NR cells” we believe that the current specification could be misunderstood so that implementations might avoid storing entries resulting from e.g. intra-gNB intra-frequency mobility. This would greatly reduce the benefit of the UHI functionality, and we therefore propose a correction as follows:

#### 9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells included in the CellAccessRelatedInfo IE in SIB1 (see TS 38.331 [18]) with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

Proposal: Correct description of Last Visited NG-RAN Cell Information IE as per TP above.

# 3	Conclusion

We have provided the following observation and conclusion:

Observation: The term "set of cells" was intended to correspond to the cells included in the CellAccessRelatedInfo IE in SIB1.

Proposal: Correct description of Last Visited NG-RAN Cell Information IE as per TP above.

# References

[1]		R3-223488, Correction of Last Visited NG-RAN Cell Information, CR to TS 38.413 (Rel-15), Huawei, Qualcomm, China Telecom, China Unicom

# Annex - CR to TS 38.413

3GPP TSG-RAN WG3 Meeting #117bis-e	R3-22xxxx

# E-meeting, 10 – 18 October, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | x | rev | - | Current version: | 15.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Last Visited NG-RAN Cell Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Risk for misoperation of functionality using UE History Information due to different possible interpretations of the Last Visited NG-RAN Cell Information IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification of the term 'set of cells' in IE definition. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated functional impact with the previous version of the specification (same release). The impact can be considered isolated because the change only affects the UE history information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification with risk for misoperation of functionality using UE History Information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.97 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells included in the CellAccessRelatedInfo IE in SIB1 (see TS 38.331 [18]) with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  |
| Cell Type | M |  | 9.3.1.98 |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells included in the CellAccessRelatedInfo IE in SIB1 (see TS 38.331 [18]) with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells included in the CellAccessRelatedInfo IE in SIB1 (see TS 38.331 [18]) with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. |