3GPP TSG-RAN WG3 Meeting #117bis-e	R3-225666

E-meeting, 10 – 18 October 2022

Title: 	(TPs to TS 38.300, TS 38.413 BL CRs) Support of Network-Controlled Repeater

Source: 	Huawei

Agenda item:	22.2

Document Type:	Other

# 1. Introduction

In RAN#97-e meeting, new WID for network-controlled repeater is approved [1]. The solution of NCR management (i.e., the identification and authorization/validation of NCR) is to be specified in WI phase, and the solution is to be down-selected from the following 4 solutions in section 8 of TR 38.867 [2]:

- In solution 1, the NCR firstly accesses to RAN and CN as a normal UE, NCR dedicated slice is deployed to enable NCR authorization, and afterwards the NCR validation is used to check the validity of NCR which is done at RAN side.

- In solution 2, the NCR is identified at RAN side and the authorization/validation are performed by local RAN OAM. CN is absent in this solution.

- In solution 3, NCR identification is done at RAN side, and NCR authorization is done at CN side, similar to the handling of IAB-MT.

- In solution 4, NCR authorization is performed at CN side, and the NCR is identified in the authorization process, e.g., based on the UE subscription. The NCR authorization information is sent from the AMF to the gNB.

In the paper of NCR WID, it is noted that the feasibility of NCR validation procedure in solution 1 and the feasibility of solution 2 are pending to the feedback from SA3 and SA5. When waiting for the LS back, we can first analyse the details of solution 3 and solution 4.

# 2. Discussion

Solution 1 and solution 2 respectively render RAN and OAM the function of legality validation that has not been considered as the function of RAN and OAM before. Thus SA3 is to check the security issues in solutions 1 and 2, and SA5 is to check the OAM related issues. How to support inter-vendor interoperability also needs to be clarified by the supporters of solution 1 (RAN validation) and solution 2 (OAM authorization/validation). Before receiving the feedback from SA3 and SA5 on solution 1/2’s feasibility, we should only discuss the details of solution 3 and solution 4. And if feasibilities of solution 1 and solution 2 are approved in the future, down-selection of these 4 solutions can be considered.

Proposal 1: Before receiving the feedback from SA3 and SA5 on solution 1/2’s feasibility, we should only discuss the details of solution 3 and solution 4 in RAN3.

In TR 38.867, the call flows for solution 3 and solution 4 are given as follows:

Figure 1. NCR management solution 3 call flow          Figure 2. NCR management solution 4 call flow

In Solution 3 and solution 4, the NCR authorization is performed by the CN, and CN provide NCR authorized indication to RAN once it is authorized.

To support NNSF, in solution 3, the NCR supporting capability of the AMF is transmitted to the gNB, and the gNB selects an NCR capable AMF for serving the NCR. In solution 4, it is assumed that all the connected AMFs of the gNB support NCR.

To support NCR identification, in solution 3, the NCR indication info is first transmitted from the NCR to the gNB, and gNB further forward this indication to the CN. In solution 4, AMF identifies and authorized the NCR based on the subscription data, and gNB identifies the NCR based on the NCR authorization indication from the CN.

From our view, both solution 3 and solution 4 are feasible and solution 3 is more flexible than solution4, considering that the NGAP change for solution 4 is a subset of solution3, we provide TPs related to solution3 and 4 in the section 5 and 6.

Proposal 2: Capture the TPs provided in section 5 and 6 about solution 3 and 4 into the BL CRs for TS 38.300 and 38.413.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1: Before receiving the feedback from SA3 and SA5 on solution 1/2’s feasibility, we should only discuss the details of solution 3 and solution 4 in RAN3.

Proposal 2: Capture the TPs provided in section 5 and 6 about solution 3 and 4 into the BL CRs for TS 38.300 and 38.413.

# 4. Reference

- RP-222673 New WID on NR network-controlled repeaters, RAN#97-e

- 3GPP TR 38.867, Study on NR network-controlled repeaters.

# 5 TP for TS 38.300 BL CR

-------------------------------------------------------------Start of change-------------------------------------------------------------

## 4.x	Network Controlled Repeater

### 4.x.x	NCR Identification and Authorization

NCR identification is done at RAN side, and NCR authorization is done at CN side. The general procedure for NCG identification and authorization is illustrated in Figure 4.x.x-1:

Figure 4.x.x-1. NCR Identification and Authorization

1/2. During NG-C setup procedure, the AMF should inform the gNB whether it supports NCR, by including “NCR-supported” indicator in NG SETUP RESPONSE message.

3.The gNB broadcasts the support of NCR access in the cell. (pending to RAN2 discussion)

4~8. NCR access to the cell and establishes RRC connection and provide the NCR indication in Msg5.

9. The gNB selects an AMF which supports NCR, and forwards the NCR indicator to the AMF.

10. the AMF performs NCR authorization based on the subscription information.

11. in case the NCR is authorized, the AMF provides NCR Authorization indication to the gNB in the Initial Context Setup Request procedure.

-------------------------------------------------------------End of change-------------------------------------------------------------

# 6 TP for TS 38.413 BL CR

-------------------------------------------------------------Start of change-------------------------------------------------------------

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

ACL	Access Control List

AMF	Access and Mobility Management Function

CAG	Closed Access Group

CGI	Cell Global Identifier

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DC	Dual Connectivity

DL	Downlink

EPC	Evolved Packet Core

FN-RG	Fixed Network Residential Gateway

GUAMI	Globally Unique AMF Identifier

HFC	Hybrid Fiber-Coax

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

MBS	Multicast/Broadcast Service

N3IWF	Non 3GPP InterWorking Function

NB-IoT	Narrow Band Internet of Things

NCR	Network Controlled Repeater

NID	Network Identifier

NGAP	NG Application Protocol

NPN	Non-Public Network

NRPPa	NR Positioning Protocol Annex

NSAG	Network Slice AS Group

NSCI	New Security Context Indicator

NSSAI	Network Slice Selection Assistance Information

OTDOA	Observed Time Difference of Arrival

PEIPS	Paging Early Indication with Paging Subgrouping

PNI-NPN	Public Network Integrated Non-Public Network

ProSe	Proximity Services

PSCell	Primary SCG Cell

PTP	Point to Point

PTM	Point to Multipoint

QMC	QoE Measurement Collection

QoE	Quality of Experience

RedCap	Reduced Capability

RIM	Remote Interference Management

RIM-RS	RIM Reference Signal

RSN	Redundancy Sequence Number

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SgNB	Secondary gNB

SMF	Session Management Function

S-NG-RAN node	Secondary NG-RAN node

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

TAC	Tracking Area Code

TAI	Tracking Area Identity

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNLA	Transport Network Layer Association

TWAP	Trusted WLAN Access Point

TWIF	Trusted WLAN Interworking Function

UL	Uplink

UP	User Plane

UPF	User Plane Function

V2X	Vehicle-to-Everything

W-AGF	Wireline Access Gateway Function

WUS	Wake Up Signal

-------------------------------------------------------------Start of change-------------------------------------------------------------

### 8.3.1	Initial Context Setup

#### 8.3.1.1	General

The purpose of the Initial Context Setup procedure is to establish the necessary overall initial UE context at the NG-RAN node, when required, including PDU session context, the Security Key, Mobility Restriction List, UE Radio Capability and UE Security Capabilities, etc. The AMF may initiate the Initial Context Setup procedure if a UE-associated logical NG-connection exists for the UE or if the AMF has received the RAN UE NGAP ID IE in an INITIAL UE MESSAGE message or if the NG-RAN node has already initiated a UE-associated logical NG-connection by sending an INITIAL UE MESSAGE message via another NG interface instance. The procedure uses UE-associated signalling.

For signalling only connections and if the UE Context Request IE is not received in the Initial UE Message, the AMF may be configured to trigger the procedure for all NAS procedures or on a per NAS procedure basis depending on operator’s configuration.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial context setup: successful operation

In case of the establishment of a PDU session the 5GC shall be prepared to receive user data before the INITIAL CONTEXT SETUP RESPONSE message has been received by the AMF. If no UE-associated logical NG-connection exists, the UE-associated logical NG-connection shall be established at reception of the INITIAL CONTEXT SETUP REQUEST message.

The INITIAL CONTEXT SETUP REQUEST message shall contain the Index to RAT/Frequency Selection Priority IE, if available in the AMF.

If the NAS-PDU IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall pass it transparently towards the UE.

If the Masked IMEISV IE is contained in the INITIAL CONTEXT SETUP REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

Upon receipt of the INITIAL CONTEXT SETUP REQUEST message the NG-RAN node shall

-	attempt to execute the requested PDU session configuration;

-	store the received UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9];

-	store the received Mobility Restriction List in the UE context;

-	store the received UE Radio Capability in the UE context;

-	store the received Index to RAT/Frequency Selection Priority in the UE context and use it as defined in TS 23.501 [9];

-	store the received UE Security Capabilities in the UE context;

-	store the received Security Key in the UE context and, if the NG-RAN node is required to activate security for the UE, take this security key into use;

-	if supported, store the received SRVCC Operation Possible in the UE context and use it as defined in TS 23.216 [31];

-	store the received NR V2X Services Authorization information, if supported, in the UE context;

-	store the received LTE V2X Services Authorization information, if supported, in the UE context;

-	store the received NR UE Sidelink Aggregate Maximum Bit Rate, if supported, in the UE context, and use it for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services;

-	store the received LTE UE Sidelink Aggregate Maximum Bit Rate, if supported, in the UE context, and use it for the concerned UE’s sidelink communication in network scheduled mode for LTE V2X services;

-	store the received PC5 QoS Parameters, if supported, in the UE context and use it as defined in TS 23.287 [33];

-	store the received Management Based MDT PLMN List information, if supported, in the UE context;

-	if supported, store the received IAB Authorization information in the UE context;

-	store the received 5G ProSe Authorization information in the UE context, if supported, and use it for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services;

-	store the 5G ProSe UE PC5 Aggregate Maximum Bit Rate in the UE context, if supported, and use it for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services;

-	store the 5G ProSe PC5 QoS Parameters, if supported, in the UE context and use it as defined in TS 23.304 [47];

-	store the received NCR Authorization information, if supported, in the UE context.

For the Initial Context Setup an initial value for the Next Hop Chaining Count is stored in the UE context.

If the PDU Session Resource Setup Request List IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall behave the same as defined in the PDU Session Resource Setup procedure. The NG-RAN node shall report to the AMF in the INITIAL CONTEXT SETUP RESPONSE message the result for each PDU session resource requested to be setup as defined in the PDU Session Resource Setup procedure.

-------------------------------------------------------------Start of change-------------------------------------------------------------

### 8.7.1	NG Setup

#### 8.7.1.1	General

The purpose of the NG Setup procedure is to exchange application level data needed for the NG-RAN node and the AMF to correctly interoperate on the NG-C interface. This procedure shall be the first NGAP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes, replaces it by the one received and clears AMF overload state information at the NG-RAN node. If the NG-RAN node and AMF do not agree on retaining the UE contexts this procedure also re-initialises the NGAP UE-related contexts (if any) and erases all related signalling connections in the two nodes like an NG Reset procedure would do.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation

The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF. The AMF responds with an NG SETUP RESPONSE message including the appropriate data.

If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the NG SETUP REQUEST message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

If the UE Retention Information IE set to “ues-retained“ is included in the NG SETUP REQUEST message, the AMF may accept the proposal to retain the existing UE related contexts and signalling connections by including the UE Retention Information IE set to “ues-retained“ in the NG SETUP RESPONSE message.

If the AMF supports IAB, the AMF shall include the IAB Supported IE in the NG SETUP RESPONSE message.

The AMF shall include the Backup AMF Name IE, if available, in the Served GUAMI List IE in the NG SETUP RESPONSE message. The NG-RAN node shall, if supported, consider the AMF as indicated by the Backup AMF Name IE when performing AMF reselection, as specified in TS 23.501 [9].

If the GUAMI Type IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall store the received value and use it for further AMF selection as defined in TS 23.501 [9].

//skip unchanged part

If the Onboarding Support IE is also included within the same PLMN Support Item IE, the NG-RAN node shall, if supported, consider that the AMF supports UE onboarding for the identified SNPN, as specified in TS 23.501 [9].

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the AMF supports NCR, the AMF shall include the NCR Supported IE in the NG SETUP RESPONSE message.

-------------------------------------------------------------Start of change-------------------------------------------------------------

### 8.3.4	UE Context Modification

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to partly modify the established UE context. The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE context modification: successful operation

Upon receipt of the UE CONTEXT MODIFICATION REQUEST message the NG-RAN node shall

-	if supported, store the received IAB Authorization information in the UE context;

-	if supported, store the received NCR Authorization information in the UE context.

If the Security Key IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall store it and perform AS key re-keying according to TS 33.501 [13].

-------------------------------------------------------------Start of change-------------------------------------------------------------

#### 9.2.2.1	INITIAL CONTEXT SETUP REQUEST

This message is sent by the AMF to request the setup of a UE context.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| …… | …… | …… | …… | …… | …… | …… |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.3.1.234 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| NCR Authorized | O |  | 9.3.1.xxx |  | YES | ignore |

-------------------------------------------------------------Next change-------------------------------------------------------------

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the AMF to provide UE Context information changes to the NG-RAN node.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| …… | …… | …… | …… | …… | …… | …… |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.3.1.234 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| NCR Authorized | O |  | 9.3.1.xxx |  | YES | ignore |

-------------------------------------------------------------Next change-------------------------------------------------------------

#### 9.2.5.1	INITIAL UE MESSAGE

This message is sent by the NG-RAN node to transfer the initial layer 3 message to the AMF over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| …… | …… | …… | …… | …… | …… | …… |
| NPN Access Information | O |  | 9.3.3.46 |  | YES | reject |
| RedCap Indication | O |  | 9.3.1.228 |  | YES | ignore |
| NCR Indication | O |  | ENUMERATED (true, ...) | Indication of an NCR. | YES | reject |

-------------------------------------------------------------Next change-------------------------------------------------------------

#### 9.2.6.2	NG SETUP RESPONSE

This message is sent by the AMF to transfer application layer information for an NG-C interface instance.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| …… | …… | …… | …… | …… | …… | …… |
| IAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for IAB. | YES | ignore |
| Extended AMF Name | O |  | 9.3.3.51 |  | YES | ignore |
| NCR Supported | O |  | ENUMERATED (true, ...) | Indication of support for NCR. | YES | ignore |

-------------------------------------------------------------Next change-------------------------------------------------------------

#### 9.3.1.xxx	NCR Authorized

This IE provides information about the authorization status of the NCR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NCR Authorized | M |  | ENUMERATED (authorized, not authorized, ...) | Indicates the NCR authorization status. |

-------------------------------------------------------------End of change-------------------------------------------------------------