3GPP TSG-RAN WG3 Meeting #117e-bis	R3-225750

# Electronic, 10 – 18 Oct 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on coarse UE location reporting for TS 38.300 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, Apple, CAICT, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-09-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA3’s LS R3-224220 NTN specific User Consent and UE location in connected mode in NTN, SA3 have the following suggestions for NTN specific user consent for Rel-17. - “For Rel-17, in regions where user consent is required for NTN, SA3 recommends that the user consent requirement be met via provisional means, e.g. per gNB/NTN-GW configuration (consent granted for all UEs subscribing for NTN) based on the service-level agreement between the operator and its NTN subscribers” - “it shall be the network that decides whether there is a user consent for the aforementioned location request (based on subscription-based means or proprietary mechanisms) and not the UE. In other words, the network should request for the location if there is user consent (based on subscription-based or proprietary mechanisms) and the network should not request for the location if there is no user consent. The UE should provide the location information (if available) if the network requests for it.” Based on the above recommandation from SA3, to protect the user location privacy in NTN, the network should be responsible for determining whether to send the request based on the user consent configured by OAM or RAN implementation. RAN2 had related discussion about the NTN specific user consent based on the SA3’s LS in RAN2 119e meeting, however, there’s no consensus in RAN2 since the determination of the above request is within the scope of network side, which should be discussed and specified by RAN3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a text about user consent for netwrok request in the charpter of coarse UE location reporting. Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated because the change only clarifies the netwrok request is based on the user consent. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network can reqeust for coarse location without user consent, which may violate user privacy |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.14.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

16.14.8	Coarse UE location reporting

Upon network request, after AS security is established in connected mode, a UE should report its coarse UE location information (most significant bits of the GNSS coordinates, ensuring an accuracy in the order of 2 km) to the NG-RAN if available. The network request is sent only if there is user consent (i.e. based on subscription-based or proprietary mechanism).

End of the change