3GPP TSG-RAN WG3 Meeting #117bis-e	 R3-225807

Online, 10th - 18th October 2022

Agenda Item:	10.2.3

Source:	CMCC

Title:	SON/MDT enhancement for RACH report

Document for:	Discussion and Decision

1	Introduction

A Rel-18 WI “Further Enhancement of Data Collection for SON_MDT in NR standalone and MR-DC WI” was first approved at RAN #94 meeting and updated at RAN #96 meeting in [1].

One of the objectives is related to RACH report enhancement. During the last meeting, at least the following scenarios for RACH optimisation has been identified for further discussion,

  - RACH optimization for feature or feature combinations involving RACH partitioning (SDT, RedCap, Coverage Enhancement, network slicing, …)

  - RACH report retrieval

  - SN RACH report in MR-DC

In this contribution, we further discuss the scenarios and potential specification impacts of SON/MDT optimisation for RACH report.

2	Discussion

2.1 	SN RACH report for MR-DC

This is a leftover issue in Rel-17, the use cases of RACH report for SgNB were first discussed in RAN3 and a LS was sent to RAN2 to request for support of RACH report for SgNB in MR-DC scenarios in R3-205647.

RAN2 confirmed the requirement via a reply LS at RAN3#114 meeting in R3-205662 and agreed that UE would report SN RACH report to MN for EN-DC, NGEN-DC and NR DC, and then MN could send the SN RACH report to SN.

RAN2 has the following agreement:

- UE reports the SN RACH report to the MN, and then MN sends the SN RACH report to SN.

Based on this agreement, UE could report the recorded SgNB related RACH information to the ‎current MN node, for (NG)EN-DC case and for NR-DC case, respectively. And the node ‎which received the information could send the SN related RACH report information to the ‎SgNB(s).‎ RAN2 is working on detailed signalling to support such a report.

With the feedback from RAN2, RAN3 worked on network interface signaling to support the RACH report for SN for EN-DC, NGEN-DC and NR-DC cases, e.g., introducing a new X2 AP message i.e. ACCESS AND MOBILITY INDICATION to transfer the RACH Report for SgNB from eNB to en-gNB. But for NE-DC, it is not supported in Rel-17, since the RACH Report container over Xn contains only the RACH report in NR format.

Observation 1: RAN3 supports SN RACH report for (NG)EN-DC, and NR-DC scenarios in Rel-17. NE-DC scenario is not supported.

Due to time limitation, RAN2 made further agreement that SN RACH report could not be supported in (NG)EN-DC and NE-DC cases in Rel-17.

- TS 36.331 modifications are not introduced to handle the scenario of LTE MN fetching the list of NR RA reports in Rel-17.

- TS 38.331 modifications are not introduced to handle the scenario of NR MN fetching the LTE RA report in Rel-17.

Observation 2: RAN2 does not support SN RACH report for (NG)EN-DC, NR-DC and NE-DC scenarios in Rel-17

So, there is misalignment between RAN2 and RAN3 specs on this aspect. The misalignment should be resolved in Rel-18.

Proposal 1: The misalignment between RAN3 and RAN2 about SN RACH report in MR-DC should be resolved.

In RAN2#119-e, RAN2 discussed Rel-18 scope for SN RACH report enhancements and agreed to focus only on leftover issues for completing the work done in RAN3 in Rel-17 for the SN RACH reporting.

Further questions are asked by RAN2 in R3-225312,

- Any leftover issues of the SN RA report that is done in Rel-17, and may impact RAN2 specification, including (if applicable):

- Clarification of the required SN RACH report for (NG)EN-DC and NE-DC scenarios.

For the scenario of SN RACH report for EN-DC or (NG) EN DC, SN RACH report will be encoded in NR format, and report via the LTE to the network, so the TS 36.331 needs to be modified. Then the MN (eNB or ng-eNB) needs to forward the SgNB RACH report to SN. In case of SN RACH report was not reported immediately while PScell change or inter-MN handover happens, the MN needs to forward the SN RACH report to the correct SN. Since the eNB or ng-eNB cannot decode the SN RACH report, the PScell ID may be included outside the SN RACH report container to help the network identify the proper SN without decoding the container.

For the scenario of SN RACH report for NE-DC, similar procedures can be applied with the difference of modification at TS 38.331. Considering RAN3 still not supports SN RACH report for NE-DC scenario over Xn interface, and the main benefit of SN report in NE-DC is to identify and optimize the parameters in LTE side which is not the focus of the WI. So it is proposed not to support SN RACH report for NE-DC in Rel-18 to save time.

Proposal 2: SN RACH report for NE-DC is not supported in Rel-18

Proposal 3: Send an reply LS to RAN2 to indicate RAN3 has supported SN RACH report for EN-DC, NGEN-DC and NR-DC scenario over network interfaces, which may impact RAN2 specification and request RAN2 to include PScell identity information outside the RACH report.

2.2	RACH report retrieval for CU-DU architecture

This is also a leftover issue in Rel-17 and a long discussion has been made. In Rel-16, it has been agreed to collect the RACH report for every RACH procedure and up to 8 reports can be logged in a list of RACH reports. RACH report can be triggered and logged for various purposes, shown below:

Among all different purposes there are some cases where the gNB-CU is aware of the performed RACH and logged RACH report while there are some other cases in which only the gNB-DU is aware of the performed RACH procedure. A list of RACH purposes for which RACH is triggered in a way not visible to the CU is given in the following:

- beamFailureRecovery

- ulUnSynchronized

- noPUCCHResourceAvailable

- requestForOtherSI

- sCellAdditionTAAdjestment

There are two ways to handle the issue,

- DU indicates to CU the availability of RACH reports when the occurrence of the RACH procedure is not known to the gNB-CU.

- UE indicates a RACH report availability indication to the network.

Both solutions work, a lot of debates on this issue happen in Rel-17, some companies think a trigger from DU to CU could notify CU when to retrieve a RACH Report which would be beneficial for the RACH optimization function. Other companies think that a trigger from DU to CU every time there is a RACH event is heavy on signaling and should be avoided.

Proposal 4: To make a decision between solution 1 and solution 2, and if solution 2 is not agreed, ask RAN2 views on the support of UE based solution.

  - RACH partitioning

During Rel-17 discussion, at least 4 features including RAN slicing, coverage enhancement, small data transmission and Redcap, have requirements to perform RACH partitioning, to satisfy the requirement of early identification and resource isolation. At last meeting, it was agreed to support RACH optimization for feature or feature combinations involving RACH partitioning (SDT, RedCap, Coverage Enhancement, network slicing, …).

To avoid too fragmented RACH resources due to partitioning, Rel-17 designed an unified solution. The network can associate a set of RACH resources with feature(s) applicable to a Random Access procedure. A set of RACH resources associated with a feature is only valid for random access procedures applicable to at least that feature; and a set of RACH resources associated with several features is only valid for random access procedures having at least all of these features. The UE selects the set(s) of applicable RACH resources, after uplink carrier (i.e. NUL or SUL) and BWP selection and before selecting the RA type.

So in order to help network optimize RACH partition configuration, some UE assistance information are needed.

Proposal 5: Send LS to RAN2 to work on the UE assistance information.

2.4	RACH optimization for SDT

Rel-17 has specified SDT mechanism to allow UE to transmit small data during 2-step/4-step RACH or configuration grant procedure. But whether and how the UE could send small data through RACH procedure follows the configuration from the gNB, e.g., RSRP threshold, data volume threshold to select SDT or non-SDT and time/frequency resources. All these configurations are different from normal RACH procedure. Improper configuration may deteriorate the performance of SDT. So SON optimization could be utilized to improve the performance of SDT.

Proposal 6: SDT are considered as one of the cases for Rel-18 RACH optimisation.

3	Conclusion

In this contribution, we discuss the scenarios and potential specification impacts of SON/MDT optimisation for RACH report. The following observations and proposals are made,

Observation 1: RAN3 supports SN RACH report for (NG)EN-DC, and NR-DC scenarios in Rel-17. NE-DC scenario is not supported.

Observation 2: RAN2 does not support SN RACH report for (NG)EN-DC, NR-DC and NE-DC scenarios in Rel-17

Proposal 1: The misalignment between RAN3 and RAN2 about SN RACH report in MR-DC should be resolved.

Proposal 2: SN RACH report for NE-DC is not supported in Rel-18

Proposal 3: Send an reply LS to RAN2 to indicate RAN3 has supported SN RACH report for EN-DC, NGEN-DC and NR-DC scenario over network interfaces, which may impact RAN2 specification and request RAN2 to include PScell identity information outside the RACH report.

Proposal 4: To make a decision between solution 1 and solution 2, and if solution 2 is not agreed, ask RAN2 views on the support of UE based solution.

Proposal 5: Send LS to RAN2 to work on the UE assistance information.

Proposal 6: SDT are considered as one of the cases for Rel-18 RACH optimisation.

4	Reference

[1] 	RP-221825, “New WID on Further Enhancement of Data Collection for SON_MDT in NR standalone and MR-DC”, CMCC