3GPP TSG-RAN WG3 Meeting #117bis-e	R3-225827

E-meeting, 10 – 18 October 2022

Title:	[DRAFT] LS on delay issue on configuring MRB

Response to:

Release:	Release 17

Work Item:	NR_MBS-Core

Source:	Huawei [will be RAN3]

To:	SA2, CT4

Cc:	RAN2

Contact person:

#### Name:	Yan Wang

E-mail Address:	wangyan7@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN3 considers that there will be extra latency for configuring multicast MRB due to generate initial PDCP COUNT value based on the receiving packet over NG-U tunnel. The NG-RAN node cannot configure multicast MRB for involved UEs as excepted during multicast session establishment procedure.

RAN3 proposed two solutions to solve this problem:

- Control plane solution: the MB-SMF gets the MBS QFI SN from MB-UPF and provides the MBS QFI SN to the gNB

  - In case of unicast transport of shared NG-U case, when a gNB triggers the shared NG-U establishment for an activated multicast session, the MB-SMF gets the MBS QFI SN information from the MB-UPF during the shared NG-U establishment procedure.

  - In case of IP multicast transport of shared NG-U case, when a gNB triggers the shared NG-U establishment for an activated multicast session, the MB-SMF retrieves the MBS QFI SN from the MB-UPF.

  - During the session deactivation procedure, the MB-UPF informs the MBS QFI SN information to the MB-SMF, then after that in case a gNB triggers the shared NG-U establishment for that deactivated multicast session, the MB-SMF informs the stored MBS QFI SN information to the gNB directly.

- User plane solution: the MB-UPF sends an N3mb packet containing the MBS QFI SN without user data payload to gNB-CU-UP

  - In case of unicast transport of shared NG-U case, when the gNB triggers the shared NG-U establishment for an activated multicast session, the MB-SMF invokes the MB-UPF to transmit empty packets to carry MBS QFI SN.

  - In case of IP multicast transport of shared NG-U case, when a gNB triggers the shared NG-U establishment for an activated multicast session, the MB-SMF invokes the MB-UPF to transmit empty packets to carry MBS QFI SN, and those empty packets will be received by all the gNBs joined the IP multicast group.

  - When the session is deactivated, when a gNB triggers the shared NG-U establishment, the MB-SMF still needs to invokes the MB-UPF to transmit empty packets to carry MBS QFI SN.

Then, RAN3 ask SA2 and CT4 to evaluate both solutions and give the preference if any.

# 2	Actions

To SA WG2, CT WG4 groups.

ACTION: RAN3 respectfully requests SA2 and CT4 to evaluate both solutions and give the preference if any.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#118			2022-11-14 - 2022-11-18		Toulouse, France

RAN3#119			2023-02-27 - 2023-03-03		Athens, GR