3GPP TSG-RAN WG3 Meeting #117bis-e	R3-225877

# E-meeting, 10 – 18 October 2022

Agenda item:		12.2.1

Source:	ZTE

Title:	Discussion on stage2 impacts for AIRAN feature

Document for:		Discussion and Decision

Introduction

The following are the agreements for stage2 impacts from RAN3#117e meeting:

Not to capture the flow charts right now, can be considered after the standard impacts are identified.

Both non-split architecture and split architecture are in scope. Focus on the non-split architecture first. Split architecture should be specified after the work of non-split architecture. The training/inference function location is referred to TR37.817.

Capture the Abbreviations of AI/ML in TS38.300. Capture the general introduction of AI/ML in TS38.300.

Focus on Xn interface first.

Start from SA and then consider DC.

According to the agreed CR to TS38.300, which involves the addition of abbreviation, general introduction of AIRAN and the deployment scenario. The description in the case of split-architecture should also be introduced, and the AI/ML related common procedures should be introduced.

Discussion

In the RAN3#117e meeting, following is the agreement to the TS38.300

Capture the Abbreviations of AI/ML in TS38.300. Capture the general introduction of AI/ML in TS38.300.

And it can be seen that the BLCR to TS38.300 involves the addition of abbreviation, general introduction of AIRAN and the deployment scenario in the case of non-split architecture. Since in the TR37.817 [1], the deployment across three use cases includes the split architecture, as shown below:

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

Therefore, since AIRAN may have impact over F1, the general introduction and deployment of AI/ML RAN should be introduced in the TS38.401. And the abbreviation should also be introduced.

Proposal 1: Capture the Abbreviations of AI/ML in TS38.401.

Proposal 2: Capture the general introduction of AI/ML in TS38.401.

As can be clearly seen in the TR37.817, there are two kinds of deployments across three use cases. One is that AI/ML Model Training at OAM and AI/ML Model Inference at NG-RAN, and the other is that AI/ML Model Training and AI/ML Model Inference at NG-RAN. From our perspective, the common procedures can be squeezed from these two flow charts. And for the input data collection, output collection and feedback information, the data collection procedure, predicted procedure and feedback procedure could be introduced. Following are the flowcharts for AI/ML RAN, which can be captured into the TS38.300.

AI/ML DATA COLLECTION PROCEDURE:

NG-RAN node initiates the AI/ML DATA COLLECTION procedure to request the input information from other NG-RAN nodes to perform AI/ML function (e.g., AI/ML model training/inference).

Figure 1. Flowcharts of AI/ML input information exchange between NG-RAN nodes.

AI/ML PREDICTED INFORMATION PROCEDURE:

NG-RAN node initiates the AI/ML PREDICTED INFORMATION procedure to request the predicted information from other NG-RAN nodes to perform AI/ML function (e.g., AI/ML model training/inference).

Figure 2. Flowcharts of AI/ML predicted information exchange between NG-RAN nodes.

AI/ML FEEDBACK INFORMATION PROCEDURE:

NG-RAN node initiates the AI/ML FEEDBACK INFORMATION procedure to request the feedback information from other NG-RAN nodes to evaluate or monitor the AI/ML model performance.

Figure 3. Flowcharts of AI/ML feedback information exchange between NG-RAN nodes.

Proposal 3: Capture the description of procedures above for AI/ML RAN which includes data collection procedure, predicted information procedure, and feedback procedure.

In the RAN3 meeting, it was acknowledged that a new procedure can be defined for AI/ML related information (e.g., predicted information), and the new procedure for reporting AI/ML related information should be based in a requested way, like resource report procedure. Therefore, with respect to the procedures for AI/ML function, e.g., data collection procedure, predicted information procedure and feedback information, should follow the reporting mechanisms like resource status initiation procedure and resource status update procedure.

Proposal 4: The procedures for AI/RAN can be defined as similar as resource status reporting initiation procedure and resource status reporting procedure.

Based the TR37.817, CU is also able to perform the model training/inference, and the optimization decision should be generated and executed by CU. Therefore, the data collection procedure and feedback procedure between F1 should also be captured into the TS38.401. And since CU generates the predicted information, there is no need to transfer the predicted information from CU to DU for now. Following are the procedures should be captured into the TS38.401.

AI/ML DATA COLLECTION PROCEDURE:

The gNB-CU initiates the AI/ML DATA COLLECTION procedure to request the input information from the gNB-DU(s) to perform AI/ML function (e.g., AI/ML model training/inference).

Figure 4. Flowcharts of AI/ML feedback information exchange in the case of split architecture

AI/ML FEEDBACK INFORMATION PROCEDURE:

The gNB-CU initiates the AI/ML FEEDBACK INFORMATION procedure to request the feedback information from the gNB-DU(s) to evaluate or monitor the AI/ML model performance..

Figure 5. Flowcharts of AI/ML feedback information exchange in the case of split architecture

Proposal 5: In the case of CU/DU split architecture, it is proposed to capture procedures description above for AI/ML RAN which includes data collection procedure and feedback procedure.

Above all, data collection procedures for AI/ML input information should involve:

-	Data collection reporting initiation: Data collection request and Data collection response

-  Data collection reporting: Data collection update

Predicted information procedures for AI/ML output information should involve:

-	Predicted information reporting initiation: Predicted information request and Predicted information response

-  Predicted information reporting: Predicted information update

Feedback information procedures for AI/ML feedback information should involve:

-	Feedback information reporting initiation: Feedback information request and Feedback information response

-  Feedback information reporting: Feedback information update

Proposal 6: The data collection procedures, predicted information procedures and feedback information procedures should involve the reporting initiation procedure (class1) and reporting procedure (class2).

Since it is proposed to capture the flowcharts for the AI/ML function, it is beneficial to capture the terminology of AI/ML function, and the terminology of AI/ML function listed below are acknowledged in TR37.817:

-	Data collection: Data collected from the network nodes, management entity or UE, as a basis for AI/ML model training, data analytics and inference.

-	AI/ML Model: A data driven algorithm by applying machine learning techniques that generates a set of outputs consisting of predicted information and/or decision parameters, based on a set of inputs

-	AI/ML Training: An online or offline process to train an AI/ML model by learning features and patterns that best present data and get the trained AI/ML model for inference.

-	AI/ML Inference: A process of using a trained AI/ML model to make a prediction or guide the decision based on collected data and AI/ML model.

Proposal 7: Capture the terminology of AI/ML function in the stage2 specification.

The corresponding CR[2] to TS38.401 and TP to TS38.300 below are provided.

Conclusion

We propose the following proposals to the stage2 specification:

Proposal 1: Capture the Abbreviations of AI/ML in TS38.401.

Proposal 2: Capture the general introduction of AI/ML in TS38.401.

Proposal 3: Capture the description of procedures above for AI/ML RAN which includes data collection procedure, predicted information procedure, and feedback procedure

Proposal 4: The procedures for AI/RAN can be defined as similar as resource status reporting initiation procedure and resource status reporting procedure.

Proposal 5: In the case of CU/DU split architecture, it is proposed to capture procedures description above for AI/ML RAN which includes data collection procedure and feedback procedure.

Proposal 6: The data collection procedures, predicted information procedures and feedback information procedures should involve the reporting initiation procedure (class1) and reporting procedure (class2).

Proposal 7: Capture the terminology of AI/ML function in the stage2 specification.

- References

- TR37.817

- R3-225878, CR to TS38.401 for addition of AI/ML RAN feature, ZTE

- TP to 38.300

<Unchanged Text Omitted>

- 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

Data collection: Data collected from the network nodes, management entity or UE, as a basis for AI/ML model training, data analytics and inference.

AI/ML Model: A data driven algorithm by applying machine learning techniques that generates a set of outputs consisting of predicted information and/or decision parameters, based on a set of inputs

AI/ML Training: An online or offline process to train an AI/ML model by learning features and patterns that best present data and get the trained AI/ML model for inference.

AI/ML Inference: A process of using a trained AI/ML model to make a prediction or guide the decision based on collected data and AI/ML model.

<Unchanged Text Omitted>

- X.X AI/ML for NG-RAN

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

For the deployments of RAN intelligence, following solutions may be supported:

•	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

•	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

- X.X.1 Procedures for AI/ML NG-RAN

Following are the procedures for AI/ML NG-RAN, including AI/ML DATA COLLECITON procdure used for AI/ML input information, AI/ML PREDICTED INFORMATION procedure used for AI/ML output information, and AI/ML FEEDBACK INFORMATION procedure used for AI/ML feedback information.

AI/ML DATA COLLECTION PROCEDURE:

NG-RAN node initiates the AI/ML DATA COLLECTION procedure to request the input information from other NG-RAN nodes to perform AI/ML function (e.g., AI/ML model training/inference).

AI/ML PREDICTED INFORMATION PROCEDURE:

NG-RAN node initiates the AI/ML PREDICTED INFORMATION procedure to request the predicted information from other NG-RAN nodes to perform AI/ML function (e.g., AI/ML model training/inference).

AI/ML FEEDBACK INFORMATION PROCEDURE:

NG-RAN node initiates the AI/ML FEEDBACK INFORMATION procedure to request the feedback information from other NG-RAN nodes to evaluate or monitor the AI/ML model performance.