3GPP TSG-RAN WG3 Meeting #117bis-e	R3-225973

# E-meeting, 10 – 18 October 2022	was R3-225656

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0042 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on initialRX-DELIV over E1AP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CBN, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Ericsson,CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN2#119e meeting, there are agreement about PDCP configuration: multicastHFN-AndRefSN is renamed to initialRX-DELIV and update the corresponding description to ‘Indicates an initial value of RX_DELIV for multicast MRB PDCP window initialization as specified in TS 38.323 [5]. In the current E1AP specification, the CU-UP may indicate the Initial HFN and Reference PDCP SN IE to CU-CP during MC Bearer Context Setup, MC Bearer Context Modification, and MC Bearer Context Modification Required procedures. In which, the Initial HFN and Reference PDCP SN IE is descripted as “Refer to the multicastHFN-AndRefSN IE as specified in the TS 38.331 [10].” E1AP specification should be updated to align with latest RAN2 agreement. In addition, the format of the IE in the E1AP is currently not clarified, e.g., which bits are used to identify the PDCP SN. Therefore, the format needs to be clarified to avoid misunderstanding. Thus, RAN3 shall rename the “MBS Initial HFN and Reference PDCP SN” IE in E1AP into “MBS PDCP COUNT”, and adjust its description into “Encoded in the same format as the initialRX-DELIV IE and to be taken into account to configure the UE, as specified in the TS 38.331 [10]” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rename the “MBS Initial HFN and Reference PDCP SN” IE in E1AP into “MBS PDCP COUNT”, and adjust its description into “Encoded in the same format as the initialRX-DELIV IE and to be taken into account to configure the UE, as specified in the TS 38.331 [10]”. Impact Analysis: This CR has an isolated impact towards the previous version of the specification, and will not cause non-backward compatible issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The E1AP spec is misaligned with RRC spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.35a, 9.3.3.33, 9.3.3.34, 9.3.35, 9.3.36, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of the first change--------------------

#### 9.3.1.35a	MBS PDCP COUNT

This IE includes the MBS PDCP Count information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS PDCP COUNT | M |  | BIT STRING (32) | Encoded in the same format as the initialRX-DELIV IE and to be taken into account to configure the UE, as specified in TS 38.331 [10]. |

--------------Start of the next change--------------------

#### 9.3.3.33	MC Bearer Context To Setup Response

This IE contains MBS session resource related information used to confirm MC Bearer Context Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MC Bearer Context NG-U TNL Info at NG-RAN | O |  | 9.3.1.121 |  |
| MC MRB Setup Response List |  | 1..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >MBS QoS Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  |
| >MBS QoS Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |
| >MBS PDCP COUNT | O |  | 9.3.1.35a |  |
| MC MRB Failed List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >Cause | M |  | 9.3.1.2 |  |
| Available MC MRB Configuration | O |  | MC MRB Setup Configuration 9.3.1.120 |  |

--------------Start of the next change--------------------

#### 9.3.3.34	MC Bearer Context To Modify

This IE contains MBS session resource related information used to request a modification of a multicast MC Bearer Context.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MC Bearer Context NG-U TNL Info at 5GC | O |  | 9.3.1.122 |  | - |  |
| MC Bearer Context NG-U TNL Info at NG-RAN Request | O |  | 9.3.1.123 | To request NG-U TNL information from the gNB-CU-UP, if not yet available at gNB-CU-CP | - |  |
| MBS Multicast F1-U Context Descriptor | C-ifSetupOrRemove |  | 9.3.1.125 |  | - |  |
| MC MRB To Setup or Modify List |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| >MC Bearer Context F1-U TNL Info at DU | O |  | 9.3.1.124 |  | - |  |
| >SDAP Configuration | O |  | 9.3.1.39 |  | - |  |
| >MBS PDCP Configuration | O |  | PDCP Configuration 9.3.1.38 |  | - |  |
| >MBS QoS Flows Information To Be Setup | O |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - |  |
| >MRB QoS | O |  | QoS Flow Level QoS Parameters 9.3.1.26 | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB. | - |  |
| >MBS PDCP COUNT Request | O |  | ENUMERATED (true, ...) | Indicates that the MBS PDCP COUNT is requested. | - |  |
| MC MRB To Remove List |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| MC Forwarding Resource Request | O |  | 9.3.1.134 | Requests MC Forwarding Resource related information for the peer node | YES | ignore |
| MC Forwarding Resource Indication | O |  | 9.3.1.135 | Provides MC Forwarding Resource related information from the peer node | YES | ignore |
| MC Forwarding Resouce Release | O |  | 9.3.1.137 | Requests the release of the MC Forwarding Resource | YES | ignore |

--------------Start of the next change--------------------

#### 9.3.3.35	MC Bearer Context To Modify Response

This IE contains MBS session resource related information used to confirm a MC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MC Bearer Context NG-U TNL Info at NG-RAN Modify Response | O |  | 9.3.1.127 |  | - |  |
| MBS Multicast F1-U Context Descriptor | C-ifSetupOrFailed |  | 9.3.1.125 |  | - |  |
| MC MRB Setup or Modify Response List |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| >MBS QoS Flow Setup List | O |  | QoS Flow List 9.3.1.12 |  | - |  |
| >MBS QoS Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - |  |
| >MC Bearer Context F1-U TNL Info at CU | O |  | UP Transport Layer Information 9.3.2.1 |  | - |  |
| >MBS PDCP COUNT | O |  | 9.3.1.35a |  | - |  |
| MC MRB Failed List |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| >Cause | M |  | 9.3.1.2 |  | - |  |
| Available MC MRB Configuration | O |  | MC MRB Setup Configuration 9.3.1.120 | In case the shared MBS NG-U termination had a different MRB Configuration applied. | - |  |
| MC Forwarding Resource Response | O |  | 9.3.1.136 | Provides MC Forwarding Resource related information destined to the peer node | YES | ignore |

--------------Start of the next change--------------------

#### 9.3.3.36	MC Bearer Context To Modify Required

This IE contains MBS session resource related information used to request MC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Multicast F1-U Context Descriptor | C-ifRemoved |  | 9.3.1.125 |  | - |  |
| MC MRB To Remove List Required |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| MC MRB To Modify List Required |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| > MBS PDCP COUNT | O |  | 9.3.1.35a |  | - |  |
| MC Forwarding Resource Release Indication | O |  | 9.3.1.138 | Indicates the release of an MC Forwarding Resource | YES | ignore |

--------------Start of the next change--------------------

### 9.4.5	Information Element Definitions

// Skip unchanged part

#### -- M

MaxDataBurstVolume  ::= INTEGER (0..4095, ..., 4096.. 2000000)

MaximumIPdatarate ::= SEQUENCE {

maxIPrate			MaxIPrate,

iE-Extensions		ProtocolExtensionContainer { {MaximumIPdatarate-ExtIEs} }	OPTIONAL,

...

}

MaximumIPdatarate-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MaxIPrate ::= ENUMERATED {

bitrate64kbs,

max-UErate,

...

}

MaxPacketLossRate ::= INTEGER (0..1000, ...)

MaxCIDEHCDL ::= INTEGER (1..32767, ...)

MBSAreaSessionID ::= INTEGER (0..65535, ...)

MBSF1UInformationAtCU ::= SEQUENCE {

mbs-f1u-info-at-CU				UP-TNL-Information,

iE-Extensions					ProtocolExtensionContainer	{ { MBSF1UInformationAtCU-ExtIEs } }	OPTIONAL,

...

}

MBSF1UInformationAtCU-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSF1UInformationAtDU ::= SEQUENCE {

mbs-f1u-info-at-DU				UP-TNL-Information,

iE-Extensions					ProtocolExtensionContainer	{ { MBSF1UInformationAtDU-ExtIEs } }	OPTIONAL,

...

}

MBSF1UInformationAtDU-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSNGUInformationAt5GC ::= CHOICE {

multicast 				MBSNGUInformationAt5GC-Multicast,

unicast					UP-TNL-Information,

choice-extension	ProtocolIE-SingleContainer	{{MBSNGUInformationAt5GC-ExtIEs}}

}

MBSNGUInformationAt5GC-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

MBSNGUInformationAt5GC-Multicast ::= SEQUENCE {

ipmcAddress		TransportLayerAddress,

ipsourceAddress	TransportLayerAddress,

gtpDLTEID		GTP-TEID,

iE-Extensions		ProtocolExtensionContainer { {MBSNGUInformationAt5GC-Multicast-ExtIEs} }	OPTIONAL,

...

}

MBSNGUInformationAt5GC-Multicast-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSNGUInformationAtNGRAN ::= CHOICE {

unicast					UP-TNL-Information,

choice-extension	ProtocolIE-SingleContainer	{{MBSNGUInformationAtNGRAN-ExtIEs}}

}

MBSNGUInformationAtNGRAN-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

MBSSessionAssociatedInformation ::= SEQUENCE {

mbsSessionAssociatedInformationList		MBSSessionAssociatedInformationList,

mbsSessionForwardingAddress				TransportLayerAddress,

iE-Extensions		ProtocolExtensionContainer { {MBSSessionAssociatedInformation-ExtIEs} }	OPTIONAL,

...

}

MBSSessionAssociatedInformation-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionAssociatedInformationList	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF MBSSessionAssociatedInformation-Item

MBSSessionAssociatedInformation-Item	::=	SEQUENCE {

mbs-QoS-Flow-Identifier					QoS-Flow-Identifier,

associated-unicast-QoS-Flow-Identifier	QoS-Flow-Identifier,

iE-Extensions							ProtocolExtensionContainer	{ { MBSSessionAssociatedInformation-Item-ExtIEs } }	OPTIONAL,

...

}

MBSSessionAssociatedInformation-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Support-Info-ToAdd-List ::= SEQUENCE (SIZE(1..maxnoofMBSSessionIDs)) OF MBS-Support-Info-ToAdd-Item

MBS-Support-Info-ToAdd-Item ::= SEQUENCE {

globalMBSSessionID					GlobalMBSSessionID,

iE-Extensions		ProtocolExtensionContainer { { MBS-Support-Info-ToAdd-Item-ExtIEs} }	OPTIONAL,

...

}

MBS-Support-Info-ToAdd-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Support-Info-ToRemove-List ::= SEQUENCE (SIZE(1..maxnoofMBSSessionIDs)) OF MBS-Support-Info-ToRemove-Item

MBS-Support-Info-ToRemove-Item ::= SEQUENCE {

globalMBSSessionID					GlobalMBSSessionID,

iE-Extensions		ProtocolExtensionContainer { { MBS-Support-Info-ToRemove-Item-ExtIEs} }	OPTIONAL,

...

}

MBS-Support-Info-ToRemove-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- MCBearerContextToSetup

MCBearerContextToSetup ::= SEQUENCE {

snssai								SNSSAI,

mcMRBToSetupList						MCMRBSetupConfiguration,

requestedAction									RequestedAction4AvailNGUTermination		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToSetup-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToSetup-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCMRBSetupConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBSetupConfiguration-Item

MCMRBSetupConfiguration-Item ::= SEQUENCE {

mrb-ID							MRB-ID,

sdap-config						SDAP-Configuration,

mbs-pdcp-config					PDCP-Configuration,

qoS-Flow-QoS-Parameter-List		QoS-Flow-QoS-Parameter-List,

qoSFlowLevelQoSParameters		QoSFlowLevelQoSParameters		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCMRBSetupConfiguration-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBSetupConfiguration-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- MCBearerContextToSetupResponse

MCBearerContextToSetupResponse ::= SEQUENCE {

mcBearerContextNGU-TNLInfoatNGRAN			MCBearerContextNGU-TNLInfoatNGRAN			OPTIONAL,

mcMRBSetupResponseList					MCMRBSetupResponseList,

mcMRBFailedList							MCMRBFailedList							OPTIONAL,

availableMCMRBConfig						MCMRBSetupConfiguration					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToSetupResponse-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToSetupResponse-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCBearerContextNGU-TNLInfoatNGRAN::= CHOICE {

locationindependent				MBSNGUInformationAtNGRAN,

locationdependent				LocationDependentMBSNGUInformationAtNGRAN,

choice-extension	ProtocolIE-SingleContainer	{{MCBearerContextNGU-TNLInfoatNGRAN-ExtIEs}}

}

MCBearerContextNGU-TNLInfoatNGRAN-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

MCMRBSetupResponseList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBSetupResponseList-Item

MCMRBSetupResponseList-Item ::= SEQUENCE {

mrb-ID								MRB-ID,

qosflow-setup						QoS-Flow-List,

qosflow-failed						QoS-Flow-Failed-List		OPTIONAL,

mBS-PDCP-COUNT						MBS-PDCP-COUNT				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCMRBSetupResponseList-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBSetupResponseList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-PDCP-COUNT	::= BIT STRING (SIZE (32))

MCMRBFailedList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBFailedList-Item

MCMRBFailedList-Item ::= SEQUENCE {

mrb-ID								MRB-ID,

cause								Cause,

iE-Extensions		ProtocolExtensionContainer { {MCMRBFailedList-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBFailedList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- MCBearerContextToModify

MCBearerContextToModify ::= SEQUENCE {

mcBearerContextNGUTNLInfoat5GC					MCBearerContextNGUTNLInfoat5GC				OPTIONAL,

mcBearerContextNGUTnlInfoatNGRANRequest			MCBearerContextNGUTnlInfoatNGRANRequest		OPTIONAL,

mbsMulticastF1UContextDescriptor					MBSMulticastF1UContextDescriptor				OPTIONAL,

-- This IE shall be present if either the MC MRB To Setup or Modify List IE or the MC MRB To Remove List IE or both IEs are included.

mcMRBToSetupModifyList							MCMRBSetupModifyConfiguration				OPTIONAL,

mcMRBToRemoveList								MCMRBRemoveConfiguration						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToModify-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToModify-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-MCForwardingResourceRequest		CRITICALITY ignore	EXTENSION MCForwardingResourceRequest		PRESENCE optional}|

{ID id-MCForwardingResourceIndication	CRITICALITY ignore	EXTENSION MCForwardingResourceIndication	PRESENCE optional}|

{ID id-MCForwardingResourceRelease		CRITICALITY ignore	EXTENSION MCForwardingResourceRelease		PRESENCE optional},

...

}

MCBearerContextNGUTNLInfoat5GC ::= SEQUENCE {

mbsNGUInformationAt5GC			MBSNGUInformationAt5GC,

mbsAreaSession-ID				MBSAreaSessionID		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextNGUTNLInfoat5GC-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextNGUTNLInfoat5GC-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCBearerContextNGUTnlInfoatNGRANRequest ::= SEQUENCE {

ngRANNGUTNLRequested				ENUMERATED {requested, ...},

mbsAreaSession-ID				MBSAreaSessionID		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextNGUTnlInfoatNGRANRequest-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextNGUTnlInfoatNGRANRequest-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCMRBSetupModifyConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBSetupModifyConfiguration-Item

MCMRBSetupModifyConfiguration-Item ::= SEQUENCE {

mrb-ID							MRB-ID,

f1uTNLatDU						MCBearerContextF1UTNLInfoatDU		OPTIONAL,

sdap-config						SDAP-Configuration				OPTIONAL,

mbs-pdcp-config					PDCP-Configuration				OPTIONAL,

qoS-Flow-QoS-Parameter-List		QoS-Flow-QoS-Parameter-List		OPTIONAL,

mrbQoS							QoSFlowLevelQoSParameters		OPTIONAL,

mbs-PDCP-COUNT-Req				MBS-PDCP-COUNT-Req				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCMRBSetupModifyConfiguration-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBSetupModifyConfiguration-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCBearerContextF1UTNLInfoatDU ::= SEQUENCE {

mbsF1UInfoatDU						UP-TNL-Information,

mbsMulticastF1UContextDescriptor	MBSMulticastF1UContextDescriptor,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextF1UTNLInfoatDU-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextF1UTNLInfoatDU-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastF1UContextReferenceE1 ::= OCTET STRING (SIZE(4))

MBSMulticastF1UContextDescriptor ::= SEQUENCE {

multicastF1UContextReferenceE1	MulticastF1UContextReferenceE1,

mc-F1UCtxtusage 	ENUMERATED {ptm, ptp, ptp-retransmission, ptp-forwarding, ...},

mbsAreaSession					MBSAreaSessionID					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MBSMulticastF1UContextDescriptor-ExtIEs } }	OPTIONAL,

...

}

MBSMulticastF1UContextDescriptor-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCMRBRemoveConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MRB-ID

MBS-PDCP-COUNT-Req ::=	ENUMERATED {true, ... }

##### -- MCBearerContextToModifyResponse

MCBearerContextToModifyResponse ::= SEQUENCE {

mcBearerContextNGU-TNLInfoatNGRANModifyResponse	MCBearerContextNGU-TNLInfoatNGRANModifyResponse		OPTIONAL,

mbsMulticastF1UContextDescriptor					MBSMulticastF1UContextDescriptor				OPTIONAL,

-- This IE shall be present if either the MC MRB Setup or Modify Response List IE or the MC MRB Failed List IE or both IEs are included.

mcMRBModifySetupResponseList						MCMRBSetupModifyResponseList							OPTIONAL,

mcMRBFailedList									MCMRBFailedList										OPTIONAL,

availableMCMRBConfig								MCMRBSetupConfiguration								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToModifyResponse-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToModifyResponse-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-MCForwardingResourceResponse	CRITICALITY ignore	EXTENSION MCForwardingResourceResponse		PRESENCE optional},

...

}

MCBearerContextNGU-TNLInfoatNGRANModifyResponse ::= SEQUENCE {

mbs-NGU-InfoatNGRAN	MBSNGUInformationAtNGRAN,

mbsAreaSession		MBSAreaSessionID					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextNGU-TNLInfoatNGRANModifyResponse-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextNGU-TNLInfoatNGRANModifyResponse-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCMRBSetupModifyResponseList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBSetupModifyResponseList-Item

MCMRBSetupModifyResponseList-Item ::= SEQUENCE {

mrb-ID								MRB-ID,

qosflow-setup						QoS-Flow-List			OPTIONAL,

qosflow-failed						QoS-Flow-Failed-List		OPTIONAL,

mcBearerContextF1UTNLInfoatCU		UP-TNL-Information		OPTIONAL,

mBS-PDCP-COUNT						MBS-PDCP-COUNT			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCMRBSetupModifyResponseList-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBSetupModifyResponseList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- MCBearerContextToModifyRequired

MCBearerContextToModifyRequired ::= SEQUENCE {

mbsMulticastF1UContextDescriptor					MBSMulticastF1UContextDescriptor				OPTIONAL,

-- This IE shall be present if either the MC MRB To Remove List Required IE is included.

mcMRBToRemoveRequiredList						MCMRBRemoveConfiguration						OPTIONAL,

mcMRBToModifyRequiredList						MCMRBModifyRequiredConfiguration				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToModifyRequired-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToModifyRequired-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-MCForwardingResourceReleaseIndication	CRITICALITY ignore	EXTENSION MCForwardingResourceReleaseIndication		PRESENCE optional},

...

}

MCMRBModifyRequiredConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBModifyRequiredConfiguration-Item

MCMRBModifyRequiredConfiguration-Item ::= SEQUENCE {

mrb-ID							MRB-ID,

mBS-PDCP-COUNT					MBS-PDCP-COUNT															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MCMRBModifyRequiredConfiguration-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBModifyRequiredConfiguration-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- MCBearerContextToModifyConfirm

MCBearerContextToModifyConfirm ::= SEQUENCE {

mbsMulticastF1UContextDescriptor					MBSMulticastF1UContextDescriptor				OPTIONAL,

mcMRBModifyConfirmList								MCMRBModifyConfirmList					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToModifyConfirm-ExtIEs} }	OPTIONAL,

...

}

MCMRBModifyConfirmList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBModifyConfirmList-Item

MCMRBModifyConfirmList-Item ::= SEQUENCE {

mrb-ID								MRB-ID,

iE-Extensions		ProtocolExtensionContainer { { MCMRBModifyConfirmList-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBModifyConfirmList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCBearerContextToModifyConfirm-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

-- MCForwardingResourceRequest

MCForwardingResourceRequest ::= SEQUENCE {

mcForwardingResourceID				MCForwardingResourceID,

mbsAreaSession-ID					MBSAreaSessionID					OPTIONAL,

mrbForwardingResourceRequestList	MRBForwardingResourceRequestList	OPTIONAL,

mbsSessionAssociatedInformation		MBSSessionAssociatedInformation		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCForwardingResourceRequest-ExtIEs} }	OPTIONAL,

...

}

MCForwardingResourceRequest-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MRBForwardingResourceRequestList ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF MRBForwardingResourceRequest-Item

MRBForwardingResourceRequest-Item	::=	SEQUENCE {

mrb-ID								MRB-ID,

mrbProgressRequestType				MRB-ProgressInformationType	OPTIONAL,

mrbForwardingAddressRequest			ENUMERATED {request, ...}	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ {MRBForwardingResourceRequest-Item-ExtIEs} }	OPTIONAL,

...

}

MRBForwardingResourceRequest-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

-- MCForwardingResourceIndication

MCForwardingResourceIndication ::= SEQUENCE {

mcForwardingResourceID				MCForwardingResourceID,

mrbForwardingResourceIndicationList	MRBForwardingResourceIndicationList		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCForwardingResourceIndication-ExtIEs} }	OPTIONAL,

...

}

MCForwardingResourceIndication-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MRBForwardingResourceIndicationList ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF MRBForwardingResourceIndication-Item

MRBForwardingResourceIndication-Item	::=	SEQUENCE {

mrb-ID								MRB-ID,

mrb-ProgressInformation				MRB-ProgressInformation		OPTIONAL,

mrbForwardingAddress				UP-TNL-Information			OPTIONAL,

iE-Extensions						ProtocolExtensionContainer	{ {MRBForwardingResourceIndication-Item-ExtIEs} }	OPTIONAL,

...

}

MRBForwardingResourceIndication-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

-- MCForwardingResourceResponse

MCForwardingResourceResponse ::= SEQUENCE {

mcForwardingResourceID			MCForwardingResourceID,

mrbForwardingResourceResponseList	MRBForwardingResourceResponseList		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCForwardingResourceResponse-ExtIEs} }	OPTIONAL,

...

}

MCForwardingResourceResponse-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MRBForwardingResourceResponseList ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF MRBForwardingResourceResponse-Item

MRBForwardingResourceResponse-Item	::=	SEQUENCE {

mrb-ID								MRB-ID,

mrb-ProgressInformation				MRB-ProgressInformation		OPTIONAL,

mrbForwardingAddress				UP-TNL-Information			OPTIONAL,

iE-Extensions						ProtocolExtensionContainer	{ {MRBForwardingResourceResponse-Item-ExtIEs} }	OPTIONAL,

...

}

MRBForwardingResourceResponse-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

-- MCForwardingResourceRelease

MCForwardingResourceRelease ::= SEQUENCE {

mcForwardingResourceID			MCForwardingResourceID,

iE-Extensions		ProtocolExtensionContainer { {MCForwardingResourceRelease-ExtIEs} }	OPTIONAL,

...

}

MCForwardingResourceRelease-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

-- MCForwardingResourceReleaseIndication

MCForwardingResourceReleaseIndication ::= SEQUENCE {

mcForwardingResourceID			MCForwardingResourceID,

iE-Extensions		ProtocolExtensionContainer { {MCForwardingResourceReleaseIndication-ExtIEs} }	OPTIONAL,

...

}

MCForwardingResourceReleaseIndication-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCForwardingResourceID ::= OCTET STRING (SIZE(2))

MDTPollutedMeasurementIndicator ::= ENUMERATED {

iDC,

no-IDC,

...

}

MRB-ID	::=	INTEGER (1..512, ...)

MRB-ProgressInformation ::= SEQUENCE {

mrb-ProgressInformationSNs	MRB-ProgressInformationSNs,

mrb-ProgressInformationType	MRB-ProgressInformationType,

iE-Extensions				ProtocolExtensionContainer { {MRB-ProgressInformation-ExtIEs} } OPTIONAL,

...

}

MRB-ProgressInformation-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MRB-ProgressInformationSNs ::= CHOICE {

pdcp-SN12			INTEGER (0..4095),

pdcp-SN18			INTEGER (0..262143),

choice-extension		ProtocolIE-SingleContainer { { MRB-ProgressInformationSNs-ExtIEs} }

}

MRB-ProgressInformationSNs-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

MRB-ProgressInformationType ::= ENUMERATED {oldest-available, last-delivered, ...}

--------------End of the changes--------------------