3GPP TSG-RAN WG3 #117bis-e                                                             R3-226051

10 Oct - 18 Oct 2022

Online

Agenda item:	11.4

Source: 	ZTE, China Telecom, China Unicom, Samsung, Ericsson

Title: 	TP to BL CR of 38.473 on RAN visible QoE

Document for:     Discussion and Approval

1 Introduction

This paper provides a TP to BL CR of 38.473 on RAN visible QoE, based on the discussion on the discussion in [1].

2 Reference

[1] R3-225822 Discussion on QoE R17 left-over issues

3 TP to BL CR of 38.473

Changes Start

### 9.2.16	QMC messages

#### 9.2.16.1	QOE INFORMATION TRANSFER

This message is sent by a gNB-CU to a gNB-DU, to indicate information related to RAN visible QoE.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| QoE Information List |  |  | 0..1 |  | YES | ignore |
| >QoE Information Item |  |  | 1..<maxnoofQoEInformation> |  | Each | ignore |
| >>QoE Metrics | O |  | 9.3.1.260 |  | - | - |
| >> DRB List |  |  | 0..1 |  |  |  |
| >>> DRB List Item |  |  | 1 .. <maxnoofDRBs> | The List of DRBs corresponding to the QoE Information. |  |  |
| >>>>DRB ID |  |  | 9.3.1.8 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoEInformation | Maximum no. of QoE information for one UE, the maximum value is 16. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |

End of Change