3GPP TSG-RAN WG3 Meeting #117bis-e	R3-226021

# E-meeting, 10 – 20 October, 2022

Title: 	(TP for AI&ML BLCR for TS38.300) Further stage 2 updates on the introduction of RAN AI&ML

Source: 	Huawei, Ericsson, Nokia, Nokia Shanghai Bell, ZTE, CATT, Lenovo

Agenda item:	12.2.2

Document Type:	Discussion and decision

1. Introduction

In last RAN3#117e meeting, BL CR to 38.300 was agreed in [1], this paper tries to make further text proposals.

2. Reference

- R3-225211, Draft CR to 38.300 on AI/ML for NG-RAN, CMCC, ZTE, Ericsson, Nokia, Nokia Shanghai Bell, Huawei, CATT, Samsung, Lenovo, Intel

Annex: TP to 38.300

******************************Start of  Change*****************************************

X.X AI/ML for NG-RAN

X.X.1 General

AI/ML for NG-RAN, as a RAN internal function, is achieved by using Artificial Intelligence (AI) and Machine Learning (ML) techniques.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

X.X.2 Mechanisms and Principles

The AI/ML function requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results), in support to AI/ML processes such as AI/ML Model Inference and AI/ML Model Training.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

For the deployments of RAN intelligence, following scenarios may be supported:

•	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

•	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

******************************End of  Change*****************************************