3GPP TSGRANWG3#118																				R3-226248

14th – 18th Nov 2022

Toulouse, France

Agenda Item:	14.3

Source:	ZTE

Title:	(TP for CHO with NR-DC to TS 37.340): Early data forwarding optimization for CHO with SCG procedure

Document for:	Approval

- Introduction

In the last meeting, we achieved the following progress on early data forwarding optimization.

There is a need to discuss the avoidance of unnecessary signaling between MN and target SN for CHO + MR-DC. Early Data Forwarding optimizations with involvement of the target SCG(s) in Rel-18 will be supported. Focus on optimizing duplicated data forwarding scenario. There is no issue to identify the same target candidate SN by the source in case direct data forwarding is used on all the forwarding paths/target MNs. WA: both direct and indirect data forwarding will be supported. RAN3 discusses whether to avoid unnecessary signaling between MN and target SN for CHO + MR-DC in Rel-18 WI. Reuse the same design principle for CHO with one target SCG and with multiple target SCGs.

This paper addresses the following issue for CHO with one target SCG.

- Direct data forwarding

- Indirect data forwarding

- Discussion

  - Direct data forwarding

In [R3-225366], it mentioned that the prerequisite of problem as follows.

- The target SN from different candidate target MN shall be the same.

- The data forwarding DRB from Source DRB to QoS Flow Mapping List shall be the same.

In [R3-225391], it provides the following view.

- When multiple target MNs prepare the same SN it is likely the requested bearer setup in all the Addition Preparations will be the same and the target SN will be able to identify matching bearers. The target SN may then allocate the same TEIDs for the same bearers for all target MNs.

- If the source MN/SN receives the same destination TEIDs from multiple target MNs, it can deduce that there is a common target SN and forward the data only once.

- The target SN shall be able to inform the target MNs that TEIDs allocated for data forwarding are shared with other target MNs. Also, these target MNs shall inform the source MN about it, so that in case all nodes involved in CHO preparation may enable direct data forwarding.

In SoD [R3-225958], after discussing in the last meeting, we achieved the following agreement.

There is no issue to identify the same target candidate SN by the source in case direct data forwarding is used on all the forwarding paths/target MNs.

Observation 1: In case of CHO with SCG, if all of the involved nodes are in the union IP address room, the direct data forwarding can be used.

S-NODE ADDITION REQUEST message

| CHO Information SN Addition | O |  |  |  |
| --- | --- | --- | --- | --- |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  |
| >Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source M-NG-RAN node |

Observation 2: In case of CHO with SCG, the target SN can know it is the same target SN from different target MNs via the CHO Information SN Addition IE included in the SN addition request messages.

Proposal 1: In case of CHO with SCG, the same target SN assigns the same data forwarding address via SN addition request ACK message to the different target MN.

According to the TS38.423, the target SN can know if it has direct data path availability with the indicated source NG-RAN node.

### 8.3.1 S-NG-RAN node Addition Preparation If the Source NG-RAN Node ID IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, use it to decide the direct data path availability with the indicated source NG-RAN node, and if the direct data forwarding path is available, include the Direct Forwarding Path Availability IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message. #### 9.1.2.1 S-NODE ADDITION REQUEST Source NG-RAN Node ID; O; Global NG-RAN Node ID 9.2.2.3; The NG-RAN Node ID of the source NG-RAN node or the source SN.. 9.1.2.2 S-NODE ADDITION REQUEST ACKNOWLEDGE Direct Forwarding Path Availability; O; ENUMERATED (direct path available, …); Indicates direct forwarding path is available between the target S-NG-RAN node and source NG-RAN node for intra-system handover or between the target S-NG-RAN node and the source SN..

Observation 3: In case of CHO with SCG, based on the existing of the Direct Forwarding Path Availability IE, the target MN can judge whether direct data forwarding between source and target SN is supported.

  - Indirect data forwarding

In case of CHO with SCG, if the Direct Forwarding Path Availability IE is absent within the SN addition request ACK message, the target MN has to use indirect data forwarding.

For indirect forwarding optimization, [R3-225366] suggests that the same target SN assigns a single data forwarding address to one target MN from multiple target MNs. Then only the target MN which receives data forwarding address, transmit the data forwarding address to the source, other target MNs will not join the data forwarding handling.

Proposal 2: In case of CHO with SCG, for indirect data forwarding optimization, the target SN assigns a single data forwarding address included in SN addition request ACK message to one of the target MNs.

- Conclusion

Based on the above analysis, we provide the following observations and proposals.

Observation 1: In case of CHO with SCG, if all of the involved nodes are in the union IP address room, the direct data forwarding can be used.

Observation 2: In case of CHO with SCG, the target SN can know it is the same target SN from different target MNs via the CHO Information SN Addition IE included in the SN addition request messages.

Observation 3: In case of CHO with SCG, based on the existing of the Direct Forwarding Path Availability IE, the target MN can judge whether direct data forwarding between source and target SN is supported.

Proposal 1: In case of CHO with SCG, the same target SN assigns the same data forwarding address via SN addition request ACK message to the different target MN.

Proposal 2: In case of CHO with SCG, for indirect data forwarding optimization, the target SN assigns a single data forwarding address included in SN addition request ACK message to one of the target MNs.

If the above proposals are agreed, the related TP shall be agreed.

- Reference

- R3-225366 (TP for CHO with NR-DC BLCR to TS 37.340/38.423/37.483) Data forwarding optimization

- R3-225391[TP to BL CR to TS 38.423, CHO w/ NR-DC] Data forwarding in case of CHO with DC

- R3-225958 Summary of Offline Discussion on CHO with NR-DC

- Text Proposal

  - TP to 37.340

Start CHANGE

## 10.x	Conditional Handover with SCG

### 10.x.1	EN-DC

<Skip unchanged part>

### 10.x.2	MR-DC with 5GC

The Conditional Handover with SCG procedure is used to transfer a UE context from a source MN to a target MN while CHO is configured with SCG (including CHO with SCG addition and CHO with/without SCG change). In case of the CHO with/without SCG change, the UE context at the SN is kept or moved to another SN.

Figure 10.x.2-1: Conditional Handover with SCG procedure

Figure 10.x.2-1 shows an example signaling flow for Conditional Handover with SCG.

NOTE 1:	For a CHO without SN change, the source SN and the target SN shown in Figure 10.x.2-1 are the same node.

NOTE 1a:	For a CHO with SCG addition, the source SN and steps involved with the source SN in Figure 10.x.2-1 are ignored.

1.	The source MN starts the conditional handover procedure by initiating the Xn Handover Preparation procedure including MCG configuration and, if the UE is configured with an SCG, SCG configuration. The source MN includes the (source) SN UE XnAP ID, SN ID, the UE context in the (source) SN and the Conditional Handover Information Request IE in the Handover Request message.

NOTE 2:	In case of the CHO with/without SCG change, the source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration before step 1.

2.	If the candidate MN decides to keep the UE context in the SN, the candidate MN sends the SN Addition Request message to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the candidate MN decides to change the SN allowing delta configuration, the candidate MN sends the SN Addition Request message to the candidate SN including the UE context in the source SN that was established by the source MN. Otherwise, the candidate MN may send the SN Addition Request message to the candidate SN including neither the SN UE XnAP ID nor the UE context in the source SN that was established by the source MN. Within the SN Addition Request message, the candidate MN also includes the CHO related information, i.e., CHO Information SN Addition IE.

3.	The (candidate) SN replies with the SN Addition Request Acknowledge message. The (candidate) SN may include the indication of the full or delta RRC configuration.

NOTE 2a: In CHO with SCG configuration, it is up to the candidate MN implementation to make sure that the CG-Config provided from the (candidate) SN can be used in all CHO preparations, it is up to the (target) SN to decide whether to assign the same data forwarding address for multiple data forwarding pathes, or to assign a single data forwarding address for one of data forwarding pathes.

3a.	For the SN terminated bearers using MCG resources, the candidate MN provides Xn-U DL TNL address information in the Xn-U Address Indication message.

4.	The candidate MN includes within the Handover Request Acknowledge message the MN RRC reconfiguration message to be sent to the UE in order to perform the conditional handover, and may also provide forwarding addresses to the source MN. If PDU session split is performed in the target side during handover procedure, more than one data forwarding addresses corresponding to each node are included in the Handover Request Acknowledge message. The candidate MN indicates to the source MN that the UE context in the SN is kept if the candidate MN and the SN decided to keep the UE context in the SN in step 2 and step 3.

5.	The source MN sends an RRC reconfiguration message to the UE, including the CHO configuration, i.e. a list of RRC reconfiguration* messages and associated execution conditions, in which each RRC reconfiguration* message contains the SCG configuration in the RRC reconfiguration** message received from the candidate SN in step 3 and an MCG configuration.

6.	The UE applies the RRC reconfiguration message received in step 5, stores the CHO configuration and replies to the MN with an RRC reconfiguration complete message.

Editor’s Note:	FFS. It’s up to RAN3 decision when to perform early data forwarding for SN-terminated bearers.

7/8. The UE maintains connection with the source MN and, if the UE is configured with an SN, the source SN, after receiving CHO configuration, and starts evaluating the CHO execution conditions for the candidate cell(s). If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source MN, applies the stored corresponding configuration for that selected candidate cell, synchronises to that candidate cell and completes the RRC handover procedure by sending RRC reconfiguration complete* message to the target MN. The UE releases stored CHO configurations after successful completion of RRC handover procedure.

NOTE 3:	In case the target SN includes the indication of the full RRC configuration, the MN performs release of the SN terminated radio bearer configuration and release and add of the NR SCG configuration part towards the UE.

9.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the (target) SN.

Editor’s Note:	FFS whether the Random Access procedure towards the (target) SN is mandatory or optional, depending on whether there must be an SCG in CHO with SN procedure.

NOTE 4:	The order the UE performs Random Access towards the MN (step 7) and performs the Random Access procedure towards the (target) SN (step 9) is not defined.

10.	If the RRC connection reconfiguration procedure was successful, the target MN informs the (target) SN via SN Reconfiguration Complete message.

11.	The target MN sends the Handover Success message to the source MN to inform that the UE has successfully accessed the target cell.

12a/b.	The source MN sends SN Release Request message to the (source) SN including a Cause indicating MCG mobility. The (source) SN acknowledges the release request. The source MN indicates to the (source) SN that the UE context in SN is kept, if it receives the indication from the target MN.

12c.	The source MN sends XN-U Address Indication message to the (source) SN to transfer data forwarding information. More than one data forwarding addresses may be provided if the PDU session is split in the target side.

12d. The source MN sends the Handover Cancel message toward the other signalling connections or other candidate MNs, if any, to cancel CHO for the UE.

12e/f. The target MN or/and other candidate MN(s) sends the SN Release Request message(s) to other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

13a. The source SN sends the Secondary RAT Data Usage Report message to the source MN and includes the data volumes delivered to and received from the UE over the NR/E-UTRA radio as described in clause 10.11.2.

NOTE 5:	The order the source SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SN may send the report when the transmission of the related QoS is stopped.

13b. The source MN sends the Secondary RAT Data Usage Report message to AMF to provide information on the used NR/E-UTRA resource.

14.	For bearers using RLC AM, the source MN sends the SN Status Transfer message to the target MN, including, if needed, SN Status received from the source SN. The target forwards the SN Status to the target SN, if needed.

15.	If applicable, data forwarding takes place from the source side. If the SN is kept, data forwarding may be omitted for the SN terminated bearers or QoS flows kept in the SN.

16-19.	The target MN initiates the Path Switch procedure. If the target MN includes multiple DL TEIDs for one PDU session in the Path Switch Request message, multiple UL TEID of the UPF for the PDU session should be included in the Path Switch Ack message in case there is TEID update in UPF.

NOTE 6:	If new UL TEIDs of the UPF for SN are included, the target MN performs MN initiated SN Modification procedure to provide them to the SN.

20.	The target MN initiates the UE Context Release procedure towards the source MN.

21.	Upon reception of the UE Context Release message from source MN, the (source) SN releases C-plane related resources associated to the UE context towards the source MN. Any ongoing data forwarding may continue. The SN shall not release the UE context associated with the target MN if the UE contest kept indication was included in the SN Release Request message in step 12a.

END OF CHANGE