3GPP TSG-RAN WG3 Meeting #118	R3-226293

Toulouse, FR, 14-18 Nov, 2022

Title: 	Discussion on the NG-RAN Access Point Position

Source: 	Huawei, CMCC, China Unicom

Agenda item:	9.2.4

Document Type:	Discussion

1. Introduction

This paper discusses a correction to the NG-RAN Access Point Position IE in NRPPa and F1AP.

2. Discussion

In NRPPa and F1AP specifications [1-2] , the NG-RAN Access Point Positioning IE is used to identify the geographical position of an NG-RAN Access Point. An issue is observed in the degree information of latitude and longitude: the current specification cannot represent the latitude of 90° and the longitude of +180° correctly.

The NG-RAN Access Point Position IE in NRPPa is cited below for reference.

### 9.2.10 NG-RAN Access Point Position The NG-RAN Access Point Position IE is used to identify the geographical position of an NG-RAN Access Point. It is expressed as ellipsoid point with altitude and uncertainty ellipsoid according to TS 23.032 [8]. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Latitude Sign; M; ENUMERATED (North, South). Degrees Of Latitude; M; INTEGER (0..223-1); The IE value (N) is derived by this formula: N223 X /90  N+1 X being the latitude in degrees (0°.. 90°).. Degrees Of Longitude; M; INTEGER (-223..223-1); The IE value (N) is derived by this formula: N224 X /360  N+1 X being the longitude in degrees (-180°..+180°).. Direction of Altitude; M; ENUMERATED (Height, Depth). Irrelevant part omitted.

By taking the degrees of latitude as an example, the range for altitude X is (0°.. 90°)  and the value of X satisfies the following inequality: N<=223 X /90 < N+1. However, when X takes the value of 90 degree, according to the inequality, N will equal to 223 which is beyond the maximum value of N, i.e., 223-1. This shows that the value of 90° cannot be represented correctly by current specification.

A solution is to extend the range of N to (0..223), however, this means to add an additional bit which is a non-backwards compatible change or to add a new IE to extend the range of  NG-RAN Access Point Position IE to make the change backwards compatible. This may also impact on other specifications, like in section 6.1 Point in TS 23.032, the definition of the ellipsoid point coordinates.

Therefore, it is proposed to fix this issue in RAN3 internally which is to represent the latitude of 90° by N= 223-1. Similarly, a longitude of +180° can be represented by N=223-1.

Proposal 1: For NG-RAN Access Point Positioning IE, the latitude of 90° is represent by N= 223-1, the longitude of +180° is represented by N=223-1.

3. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1: For NG-RAN Access Point Positioning IE, the latitude of 90° is represent by N= 223-1, the longitude of +180° is represented by N=223-1.

Proposal 1 affects the NRPPa specification from Rel-15 to Rel-17, and the F1AP specification from Rel-16 to Rel-17.

The corresponding CRs are provided in [3] ~ [7].

4. Reference

- 38.455 V17.2.0

- 38.473 v17.2.0

- R3-226294	Correction on the NG-RAN Access Point Position (NRPPa, Rel-15)

- R3-226295	Correction on the NG-RAN Access Point Position (NRPPa, Rel-16)

- R3-226296	Correction on the NG-RAN Access Point Position (NRPPa, Rel-17)

- R3-226297	Correction on the NG-RAN Access Point Position (F1AP, Rel-16)

- R3-226298	Correction on the NG-RAN Access Point Position (F1AP, Rel-17)