3GPP TSG-RAN WG3 #118 					                                                      R3-226320

14th – 18th Nov 2022

Toulouse, France

Source:	Nokia, Nokia Shanghai Bell

Title:	Stage-2 TP for NCR

Agenda item:	22.2

Document for:	Approval

Per the discussion in ([1]), this TP proposes Stage-2 changes to support NCR.

# References

- R3-226319, Discussion on Identification and authorization of Network Controlled Repeater

Proposed Stage-2 (TS38.401) TP:

## 8.x	NCR-node Integration Procedure

A high-level flow chart for NCR integration is shown below:

Figure 8.x.y-1: The integration procedure for NCR-node

Phase 1: NCR-MT setup. In this phase, the NCR-MT of the new NCR-node connects to the network in the same way as a UE, by performing RRC connection setup procedure with gNB-CU, authentication with the core network, and, OAM connectivity establishment by using the NCR-MT’s PDU session. The OAM server may configure the NCR-node about the list of NR cells that the NCR-MT can connect with.

Phase 2: The gNB-CU may configure the NCR node via the RRC.

Editor’s NOTE: The information to be configured via RRC pends on RAN2.

Phase 3: After the NCR is configured, the NCR-node can start serving the UEs.