3GPP TSG-RAN WG3 Meeting #118	R3-226333

Toulouse, France, 14-18 November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1083 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NR MBS over F1AP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CBN, Qualcomm Incorporated, Lenovo, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In previous meetings, the Source MRB ID IE was added in the UE Multicast MRB to Be Setup Item IEs IE in the UE CONTEXT SETUP REQUEST message, which is intended to be used in case of moblity. It is noticed that the upper layer IE UE Multicast MRB to Be Setup Item IEs IE is also included in the UE CONTEXT MODIFICATION REQUEST message. Therefore, the tabular of the UE CONTEXT MODIFICATION REQUEST message is inconsistent with the asn.1, i.e. Source MRB ID IE missing in the tabular. But the Source MRB ID IE should not be used in the UE CONTEXT MODIFICATION REQUEST message, therefore, to avoid the inconsistency between tabular and asn.1, it is needed to add the Source MRB ID IE in the message and add clarification in the Semantics description that the IE is not used. - In 9.2.14.1 MULTCAST GROUP PAGING message, the Paging Cell Item IEs IE includes the NR CGI IE only, but in asn.1 part, the PagingCell-Item was extended to include LastUsedCellIndication and PEISubgroupingSupportIndication, therefore the tabluar and asn.1 are also not aligned. Similar to (1), it is also needed to update the tabluar adding the two extra IEs, and mark in Semantics desctiption that these IEs are not used. - Wrong IE name and missing IE Reference in section 9.3.2.8 for the Multicast F1-U Context Reference F1 IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the Source MRB ID IE in the tabular of UE CONTEXT MODIFICATION REQUEST message and add clarification in the Semantics description that the IE is not used. Add Last Used Cell Indication and PEI Subgrouping Support Indication IEs in the Paging Cell Item IEs and add clarification in the Semantics description that the IE is not used. Update the IE name and add IE Reference in section 9.3.2.8. Impact Analysis: This CR has an isolated impact towards the previous version of the specification, and will not cause non-backward compatible issue. This CR only has an impact on the consistence between tabular and asn.1, and fix wrong IE name and missing IE reference in the tabular. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent tabular and asn.1 and errors exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.14.1, 9.2.2.7, 9.3.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------Start of the First Change---------------

#### 9.2.14.1	MULTICAST GROUP PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to multicast group page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| UE Identity List for Paging |  | 0..1 |  |  | YES | ignore |
| >UE Identity for Paging Item |  | 1..<maxnoofUEIDforPaging> |  |  | - |  |
| >>UE Identity Index value | M |  | 9.3.1.39 |  | - |  |
| >>Paging DRX | O |  | 9.3.1.40 |  | - |  |
| Paging Cell List |  | 0..1 |  |  | YES | ignore |
| >Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Last Used Cell Indication | O |  | ENUMERATED (true, …) | If received, This IE shall be ignored in this version of the specification. | YES | ignore |
| >>PEI Subgrouping Support Indication | O |  | ENUMERATED (true, …) | If received, This IE shall be ignored in this version of the specification. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEIDforPaging | Maximum no. of UE ID for multicast group paging. Value is 4096. |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |

----------------Start of the Next Change---------------

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| //skip unchanged part |  |  |  |  |  |  |
| UE Multicast MRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>MBS PTP Retransmission Tunnel Required | O |  | 9.3.2.10 |  | - |  |
| >>MBS PTP Forwarding Tunnel Required Information | O |  | MRB Progress Information 9.3.2.12 |  | - |  |
| >>Source MRB ID | O |  | 9.3.1.224 MRB ID | If received, This IE hall be ignored in this version of the specification. | - | ignore |
| UE Multicast MRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Released Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| SL DRX Cycle List |  | 0..1 |  |  | YES | ignore |
| //skip unchanged part |  |  |  |  |  |  |

----------------Start of the Next Change---------------

#### 9.3.2.8	MBS Multicast F1-U Context Descriptor

This IE contains a reference to a Multicast F1-U Context, information about the usage of the MBS Multicast F1-U Context and may contain an MBS Area Session ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Multicast F1-U Context Reference F1 | M |  | 9.3.2.11 |  |
| MC F1-U Context usage | M |  | ENUMERATED (ptm, ptp, ptp retransmission, ptp forwarding, ...) | "ptm" indicates that the Multicast F1-U Context is setup for ptm transmissions; decided by the DU. "ptp" indicates that the Multicast F1-U Context is setup for ptp transmissions; decided by the DU. "ptp retransmission" indicates that the Multicast F1-U Context is setup for ptp retransmissions (based on PDCP Status Report); requested by the CU "ptp forwarding" indicates that the Multicast F1-U Context is setup for transmitting from a defined MBS Progress Information status onwards; requested by the CU. |
| MBS Area Session ID | O |  | 9.3.1.221 |  |

----------------End of the Changes---------------