3GPP TSG-RAN WG3 #118-e	R3-226369

E-meeting, 14th – 18th Nov 2022

Source:	CATT

Title:	Discussion on E1AP and F1AP impacts of UE-associated metrics

Agenda Item:	12.2.2.2

Document for:	Discussion and decision

- Introduction

This discussion paper focuses on Stage-3 E1/F1 interface impact of AI/ML for UE-associated metrics.

- Discussion

  - Current/predicted UE traffic

The current UE traffic is almost the same as the M4 MDT measurement. Although categorised as information “from the local node”, it surely has impact on E1AP in split gNB scenario.

For the purpose of predicting traffic the granularity should be as stable as possible. Therefore the ideal granularity should be per-UE, per-session, or per-QoS-flow. Per-DRB traffic is not that ideal as the flow-to-RB mapping may change over time. However the M4 measurement is defined as a per-DRB one in TS 37.320, even though in TS 28.552 the only granularity is per-QoS-flow.

We don’t know whether we should follow TS 37.320 or TS 28.552. Our preference is to make the granularity configurable.

Proposal 1: The granularity of UE traffic (i.e. per-UE, per-session, per-DRB or per-flow) is FFS. Our preference is to make it configurable.

As proposed in R3-225730 [1], the baseline in split architecture is that the predicting AI/ML model is deployed in the gNB-CU-CP. Therefore it should be possible for gNB-CU-UPs to provide the current UE traffic toward the gNB-CU-CP.

Proposal 2: It should be possible for the current UE traffic to be provided from the gNB-CU-UP toward the gNB-CU-CP.

For traffic prediction located in gNB-CU-CP and metrics collected by the gNB-DU (e.g. PRB usage), gNB-DU shall provide the current UE traffic toward the gNB-CU-CP.

Proposal 3: It should be possible for the current UE traffic to be provided from the gNB-DU toward the gNB-CU-CP.

  - UE location information

In the TR this bullet is categorised as information from the UE. As of current RRC spec the UE may provide these information if some condition is met.

In addition, gNB-DUs may also provide some information about UE location, e.g. the current beam index. It may even coarsely position the UE by means such as by AoA+TA (Timing Advance). It should be FFS as well.

Proposal 4: It is FFS whether F1AP should be enhanced to facilitate collecting UE location information.

  - UE trajectory prediction, predicted handover target, etc

The predicted UE location, estimated arrival probability of CHO, etc., should be generated in the gNB/gNB-CU/gNB-CU-CP. It may be needed to provide this information to gNB-DU for radio resource optimization.

Proposal 5: It may be needed to provide UE trajectory prediction information from gNB-CU to gNB-DU.

  - UE location information of HO-ed UE as feedback

This information helps the gNB to monitor the accuracy of prediction beyond its coverage, which can be very important as the main use of location prediction is to determine the handover target, which is ordinarily beyond the predicting node’s coverage. As discuss above, it is FFS whether gNB-DU shall provide some information about UE location to gNB-CU.

Proposal 6: It is FFS whether F1AP should be enhanced to facilitate collecting UE location information. Details are FFS.

  - UE performance information of HO-ed UE as feedback

Likewise, this information can be used as the “reward” in reinforcement learning. In last RAN3 meeting, the content of UE performance information is agreed as below:

Support the following UE performance information to be sent for feedback purposes: Average Packet Delay, Average UE Throughput DL, Average UE Throughput UL, Average Packet Error Rate.

gNB-CU-UPs shall provide above UE performance information to gNB-CU-CP.

On how to define these metrics, we discuss it in [2].

Proposal 7: It should be possible for gNB-CU-UPs to collect UE performance of reconfigured UE and provide it to gNB-CU-CP for the purpose of feedback. UE performance information shall include: Average Packet Delay, Average UE Throughput DL, Average UE Throughput UL, Average Packet Error Rate.

- Conclusion

Proposal 1: The granularity of UE traffic (i.e. per-UE, per-session, per-DRB or per-flow) is FFS. Our preference is to make it configurable.

Proposal 2: It should be possible for the current UE traffic to be provided from the gNB-CU-UP toward the gNB-CU-CP.

Proposal 3: It should be possible for the current UE traffic to be provided from the gNB-DU toward the gNB-

Proposal 4: It is FFS whether F1AP should be enhanced to facilitate collecting UE location information.

Proposal 5: It may be needed to provide UE trajectory prediction information from gNB-CU to gNB-DU.

Proposal 6: It is FFS whether F1AP should be enhanced to facilitate collecting UE location information. Details are FFS.

Proposal 7: It should be possible for gNB-CU-UPs to collect UE performance of reconfigured UE and provide it to gNB-CU-CP for the purpose of feedback. UE performance information shall include: Average Packet Delay, Average UE Throughput DL, Average UE Throughput UL, Average Packet Error Rate.

The corresponding TPs on F1AP and E1AP are merged into [1].

- Reference

[1] R3-226368; Discussion on E1AP and F1AP impacts of non-UE-associated metrics; CATT.

[2] R3-226365; Discussion on XnAP impacts of AI/ML UE-associated metrics; CATT.