3GPP TSG RAN WG3 Meeting#118		R3-226407

Toulouse, France, November 14th – 18th, 2022

Agenda item:	13.3

Source:	KT Corp.

Title:	Discussion on mobility enhancements for mobile-IAB

Document for:	Discussion

# Introduction

In the last RAN3 #117bis e-meeting, following could not reach to agreement and further discussion will be made in this meeting [1]:

The donor CU should know that the IAB node is “mobile”.

RAN3 to discuss whether the target IAB-donor should know the migrating IAB-node is “mobile IAB-node” from the source IAB-donor.

Source donor CU of mobile IAB-MT informs the target donor CU of mobile IAB-MT that the migrating node is a mobile IAB-node, via explicit indication in XnAP HO Request message.

RAN3 to discuss whether source donor should know whether the target cell belongs to a mIAB-Node

RAN3 to further discuss the following options for TAC/RANAC issue:

-	Option 1: The TAC/RANAC for the mobile IAB cell can be changed in order to reflect the physical location when the mobile IAB-node moves.

-	Option 2: Using static TAC/RANAC for mobile IAB when it moves. Involvement of SA2 may be needed

In this contribution, above open issues from the last RAN3 meeting are discussed.

# Discussion

From operator perspective, one of the expected deployment case for m-IAB shall be on the city bus, trams, shuttle bus, etc. where dedicated route is used. For this case, operators see clear benefits such as reducing handover signalling overhead caused from frequent handover. In order to reduce signalling overhead, we need to make sure that on-board UEs are always connected with m-IAB and for this reason it is beneficial to introduce a flag indicating “m-IAB node” so that unnecessary handover attempts from on-board UEs are excluded and only m-IAB node are considered for handover.

Observation 1: It is beneficial to introduce a flag indicating “m-IAB node”

Proposal 1: The source donor informs the target donor via XnAP that the migrating node is a m-IAB node

While m-IAB is on the move, currently there are two options whether TAC/RANAC for the m-IAB should reflect the physical location or static TAC/RANAC should be used. As indicated above, for the use cases such as city bus, trams for shuttle bus with dedicated route it should be at least clear that the m-IAB is located within the dedicated route. Also, one of the clear benefits of introducing m-IAB to commercial network is that control signalling from many on-board UEs can be significantly decreased. To enjoy this benefit, KT suggests continuing discussion for TAC/RANAC issue based on Option 2. Also, as the main use case we are considering for the Option 2 is on the dedicated route, extra performance gain can be achieved when F1- and BAP- related configurations are pre-stored. KT would like to have this supported in the m-IAB node so that handover performance can significantly improved when m-IAB is travelling on the dedicated (i.e. known) routes.

Observation 2: Using Static TAC/RANAC for mobile IAB can reduce HO signalling

Proposal 2: Continue discussion on TAC/RANAC issue with priority on Option 2 (Using Static TAC/RANAC for mobile-IAB when it moves. Involvement of SA2 may be needed)

Note: Please note that we should not preclude Option 1 as this could be helpful on use cases where vehicle with m-IAB is not travelling on the dedicated routes.

Observation 3: Option 2 can further enjoy the benefit if F1- and BAP- related configurations are pre-stored

Proposal 3: A m-IAB node can pre-store F1- and BAP- related configurations when travelling on the dedicated routes

# Conclusion

Based on the discussion in the above section, the following proposal is suggested.

Proposal 1: The source donor informs the target donor via XnAP that the migrating node is a m-IAB node

Proposal 2: Continue discussion on TAC/RANAC issue with priority on Option 2 (Using Static TAC/RANAC for mobile-IAB when it moves. Involvement of SA2 may be needed)

Proposal 3: A m-IAB node can pre-store F1- and BAP- related configurations when travelling on the dedicated routes

# Reference

- R2-226100, 3GPP TSG RAN WG3 #118 Meeting Agenda