3GPP TSG-RAN WG3 Meeting #118	R3-226609

Toulouse, France, 14th - 18th November 2022

Title: 	(TP for NR_QoE BLCR for 37.340) on QoE measurement in NR-DC

Source: 	Huawei

Agenda item:	11.3

Document Type:	discussion and decision

1. Introduction

In this paper, we will provide the TP for 37.340 on QoE measurement in NR-DC as discussed in [1].

2. TP for 37.340

## 13.X	QoE measurement in MR-DC

13.x.1	QoE Measurement Collection Activation and Reporting

When UE is configured with (NG)EN-DC and NE-DC, only MN can configure the QoE configuration. When the UE is configured with NR-DC, both MN and SN can configure the QoE configuration.

For the signalling based QoE measurement, it is MN to configure the QoE measurement.

For the management based QoE measurement received by MN from OAM, MN decides which node to configure the QoE measurement.

For the management based QoE measurement received by SN from OAM, SN can either configure the QoE measurement or request MN to configure.

MN sends the QoE reference info received from OAM to SN, while SN can configure other QoE measurements of different QoE reference info received from OAM.

The node which configures the QoE measurement can indicate and switch the path of QoE reporting.

13.x.2	RAN visible QoE Measurement

Only the node which sends the QoE measurement configuration container to the UE can configure the RAN visible QoE measurement corresponding to this QoE measurement, the UE only needs to send the RAN visible QoE results to this node.

If one node receives the RAN visible QoE report from UE and the services corresponding to this QoE measurement is served by the peer node, it can send the received RAN visible QoE report and the available RAN visible QoE metrics to the peer node, the peer node responds with the RAN visible metrics that it wants.

13.x.3	Alignment of MDT and QoE Measurements

The alignment between a signalling-based QoE measurement and a signalling-based MDT measurement is supported, the alignment of management based QoE measurement and management based MDT configured by the same node is also supported.

For the services that use the split bearers, the MCE/TCE performs the correlation of the immediate MDT measurement results from both of nodes and the QoE measurement results collected at the same UE.

3. Reference

- R3-226607, Further discussions on the support of QoE in NR-DC, Huawei