# 3GPP TSG-RAN WG3 Meeting #118	R3-226781

# Toulouse, France, 14th – 18th Nov 2022

Title:	LS on RB set configuration

Response to:

Release:	Rel-17

Work Item:	NR_IAB_enh-Core

Source:	RAN3

To:	RAN1

Cc:

Contact Person:	Ying Huang

huang.ying11@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

The F1AP (TS 38.473) and XnAP (TS 38.423) specifications state that “the start RB index of the first RB set for the RB set configuration is the lowest index of RB of the IAB-DU cell” as copied from TS 38.473 in the below. In RAN3’s understanding, there could be one or multiple numerologies in one NR cell. F1AP and XnAP presently allow defining a separate frequency offset for each numerology via the NR Carrier List IE as copied from TS 38.473 in the below. In this IE, each numerology is referred as “SCS-specific carrier”.

RAN3’s discussion considers the following RAN1 agreements:

Agreement (RAN1#106-e)
A Reference SCS is configured for frequency domain H/S/NA configuration.

Agreement (RAN1#108-e)
The start RB index of the first RB set for the Rel-17 IAB-DU HSNA resource configuration is the lowest index of RB of the IAB-DU cell

RAN3 has uncertainty on how to define the start RB index for the Rel-17 IAB-DU HSNA resource configuration of an IAB-DU cell.

RAN3 has two different understandings:

- Understanding 1: the start RB index for the Rel-17 IAB-DU HSNA resource configuration of the IAB-DU cell is point A plus the SCS-specific offset defined in the NR Carrier List IE based on the SCS included in the RB Set Configuration IE.

- Understanding 2: the start RB index for the Rel-17 IAB-DU HSNA resource configuration of the IAB-DU cell is point A plus the smallest SCS-specific frequency offset across all numerologies provided by the NR Carrier List IE for this cell. Note that in this understanding, the start RB index does not depend on the SCS value included in the RB Set Configuration IE in TS 38.473.

In both understandings, the start RB is aligned with point A in case the NR Carrier List IE is not present.

#### 9.3.1.230 RB Set Configuration This IE contains the RB Set Configuration. The IE is only applicable if the gNB-DU is an IAB-DU. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Subcarrier Spacing; M; ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …); Subcarrier spacing used as reference for the RB set configuration.. RB Set Size; M; ENUMERATED (rb2, rb4, rb8, rb16, rb32, rb64); Number of PRBs in each RB set. If the RB sets of IAB-DU H/S/NA resource configuration do not cover the entire carrier bandwidth, the remaining RBs not part of an RB set configuration are considered as included in the last RB set.. Number of RB Sets; M; INTEGER(1.. maxnoofRBsetsPerCell); Number of configured RB sets. The RB sets are contiguous and non-overlapping. The start RB index of the first RB set is the lowest index of RB of the IAB-DU cell.. Range bound; Explanation. maxnoofRBsetsPerCell; Maximum no. of RB sets per IAB-DU cell. Value is 8..

9.3.1.137	NR Carrier List

This IE indicates the SCS-specific carriers per TDD, per DL, per UL or per SUL of an NR cell.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR Carrier Item |  | 1..<maxnoofNRSCSs> |  |  |
| >NR SCS | M |  | ENUMERATED (scs15, scs30, scs60, scs120, …) | SCS for the corresponding carrier. |
| >Offset to Carrier | M |  | INTEGER (0.. 2199, ...) | Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the NR SCS IE defined for this carrier). The maximum value corresponds to 275×8−1. See TS 38.211 [33], clause 4.4.2. |
| >Carrier Bandwidth | M |  | INTEGER (1.. maxnoofPhysicalResourceBlocks, ...) | Width of this carrier in number of PRBs (using the NR SCS IE defined for this carrier). See TS 38.211 [33], clause 4.4.2. |

| Range bound | Explanation |
| --- | --- |
| maxnoofNRSCSs | Maximum no. of SCS-specific carriers per TDD, per DL, per UL or per SUL of an NR cell. Value is 5. |
| maxnoofPhysicalResourceBlocks | Maximum no. of Physical Resource Blocks. Value is 275. |

2	Actions

RAN3 kindly asks RAN1 to provide clarification on which of the two understandings is correct.

3. Date of Next RAN3 Meetings:

3GPP RAN3#119 		          27 Feb - 03 March 2023	Athens, Greece

3GPP RAN3#119-bis-e 		17 Apr - 26 April 2023		Electronic Meeting