3GPP TSG-RAN WG3 #118	R3-226797

# Toulouse, France, 14th - 18th November 2022

# Agenda Item:	9.2.7

# Source:	Huawei (moderator)

# Title:	Summary of discussion on Dataforwarding_MultipleTSNs

# Document for:	Approval

# 1. Introduction

In previous RAN3 meetings, indirect or direct data forwarding was discussed for the scenario in which there are multiple T-SNs prepared in SN initiated inter-SN CPC, and the following agreement was made:

For SN-initiated inter-SN CPC, SN CHANGE CONFIRM messages are enhanced to toss forwarding addresses of "multiple" candidate target SNs toward the source SN. To be continued on stage-3 encoding details within SN CHANGE CONFIRM messages.

In this meeting, there are multiple contributions submitted, and during online discussion, the following CB is assigned for further discussion:

CB: # 14_Dataforwarding_MultipleTSNs

- Check whether there is any issue on current mechanism

- Decide the solution if needed

(HW - moderator)

Summary of offline disc R3-226797

# 2. For the Chairman’s Notes

Agree the stage 3 CRs to provide multiple TNL data forwarding addresses:

- XnAP: R3-226894 (Rev of R3-226252)

- X2AP: R3-226894 (Rev of R3-226253)

Endorse the stage 2 draftCR on stop forwarding:

- 37.340: R3-226886 (Rev of R3-226244)

Agree revised stage 3 XnAP CR for split PDU Session:

- XnAP: R3-226891 (Rev of R3-226678)

# 3. Discussion

## 3.1 How to provide multiple TNL addresses in case multiple T-SNs are prepared?

Based on the online and some face to face offline discussion, there are three options to provide the multiple TNL address towards the source SN in the SN CHANCE CONFIRM message:

(assume 2 PDU session for the UE, and 3 T-SNs are prepared in the following options)

### Option 1: Reuse current PDU Session List, and include additional TNL address list for each PDU session [2][3]

### >PDU Session ID a

### >Data forwarding address a1

### >Data forwarding address a2

### >Data forwarding address a3

### >PDU Session ID b

### >Data forwarding address b1

### >Data forwarding address b2

### >Data forwarding address b3

### Option 2: Reuse current PDU session List, without new IE added, add clarification that the same PDU session ID will presents multiple times to carry multiple TNL addresses

### >PDU Session ID a

### >Data forwarding address a1

### >PDU Session ID a

### >Data forwarding address a2

### >PDU Session ID a

### >Data forwarding address a3

### >PDU Session ID b

### >Data forwarding address b1

### >PDU Session ID b

### >Data forwarding address b2

### >PDU Session ID b

### >Data forwarding address b3

### Option 3: Reuse current Target SN List, and include additional TNL address of List for each PDU session [5][6]

### >T-SN ID 1

### >PDU Session ID a

### >Data forwarding address 1a

### >PDU Session ID b

### >Data forwarding address 1b

### >T-SN ID 2

### >PDU Session ID a

### >Data forwarding address 2a

### >PDU Session ID b

### >Data forwarding address 2b

### >T-SN ID 3

### >PDU Session ID a

### >Data forwarding address 3a

### >PDU Session ID b

### >Data forwarding address 3b

### Comparison

Note that for MN initiated inter-SN CPC, the S-SN is not aware of which T-SN the data is forwarded to in case multiple T-SNs are prepared.

Based on moderator’s view, from the S-SN point of view, it does not and does not need to care of which T-SN a TNL address is used for, therefore in option 1, the MN provides multiple TNL addresses to the S-SN and then S-SN forward the data of a PDU session multiple times to the T-SN. For option 3, the S-SN will be informed with a list of TNL addresses for different PDU sessions for each T-SN, then the S-SN will forward the data according.

Based on the illustrate above, for theses three options, it is clearly that option 1 is more efficient from signalling point of view, especially when there are more PDU sessions, and more T-SNs, the efficiency difference will be even bigger comparing with option 2 and option 3.

For option 2, we do not think it is a good idea to repeat the same PDU session IDs multiple times in the same list, just to avoid new IE to be introduced, although it works but it is not the original design purpose for the PDU session List, and may not future proof in case of increasing max PDU sessions and max T-SNs.

Proposal 1: Use the IE structure of option 1 to provide multiple TNL address for each PDU sessions.

| Company | Acceptable/agreeable or not? | comment |
| --- | --- | --- |
| Huawei | Agree | If we have more prepared T-SNs, and more PDU sessions, we can easily found that option 1 is much more efficient than others, and the logical from S-SN side is much simpler, i.e. in case multiple TNL addresses are provided for a PDU session, the data of this session will be forwarded multiple times. |
| CATT | Agree | For option 2, the list limited should be one issue if big number PDU set up |

## 3.2 Stop Data Forwarding - stage 2 draftCR to TS 37.340

As a set of CR of [1], besides the XnAP and X2AP CRs to introduce multiple TNL addresses in [2] and [3], there is also a stage 2 proposal in [4], which clarifies that

EN-DC:

The Data Forwarding Address Indication procedure may further be invoked to indicate to the source SN to stop already initiated early data forwarding for some PDCP SDU if they are no longer subject to data forwarding due to the modification of the prepared conditional PSCell change.

MR-DC with 5GC:

The Xn-U Address Indication procedure may further be invoked to indicate to the source SN to stop already initiated early data forwarding for some PDCP SDU if they are no longer subject to data forwarding due to the modification of the prepared conditional PSCell change.

As moderator, we hear no comment/objections on this stage 2 draft CR, therefore we would like to propose to endorse it.

Proposal 2: agree on the clarification of using XnAP: Xn-U Address Indication procedure/ X2AP: Data Forwarding Address Indication to indicate early data forwarding stop.

| Company | Acceptable/agreeable or not? | comment |
| --- | --- | --- |
| Huawei | Agree |  |
| Qualcomm | Agree | Perhaps “for some PDCP SDU” is not needed. Early data forwarding is performed for a bearer. |
| CATT | Agree |  |

## 3.3 Split PDU Session support

Besides the proposals above, there is also another proposed change to include the Secondary Data Forwarding Info from target NG-RAN node List IE in the existing PDU Session Resource Change Confirm Info – SN terminated IE [5], and it was clarified that this is used to support the split PDU session scenario.

From moderator point of view, the Secondary Data Forwarding Info from target NG-RAN node List IE is used in case of Handover with DC, and source will need to get two data forwarding addresses for the same PDU session to forward data of two sets of QoS flows towards MN and SN separately.

The proposed change above not only impact SN initiated CPC, it also impacts the legacy SN Change procedure.

Our understanding of the reason why the Secondary Data Forwarding Info from target NG-RAN node List IE was not used in case of legacy SN change, is because from MN point of view, the data of the split PDU session is available already, and no need for the S-SN to forward redundantly in case of SN change. And similar logic should also be applicable for SN initiated CPC, therefore, moderator propose to note this proposed change.

Proposal 3: Noted the proposal on including the Secondary Data Forwarding Info from target NG-RAN node List IE in the existing PDU Session Resource Change Confirm Info – SN terminated IE.

| Company | Acceptable/agreeable or not? | comment |
| --- | --- | --- |
| Huawei | Agree |  |

# 4. Reference

|  | R3-226251 | Direct or indirect early data forwarding in SN initiated inter-SN CPC (Huawei, Deutsche Telekom, ZTE, CATT) | Discussion Intel: Additional List of PDU Session Resource Change Confirm Info – SN Terminated is added as optional while it is conditional in the semantic description HW: What’s usage for the existing IE in Intel’s CR SS: Same view as Intel, the only difference is that we do not have PDU session ID for each target SN E///: Current PDU session ID and procedure can be reused, no standard impact Nok: PScell on the single Target SN, the set of TNL address needs to be linked to a specific target SN, support HW’s CR HW: SS’s CR is imcomplete |
| --- | --- | --- | --- |
|  | R3-226252 | Direct early data forwarding in SN initiated inter-SN CPC (Huawei, Deutsche Telekom, ZTE, CATT, Nokia, Nokia Shanghai Bell) | CR0940r, TS 38.423 v17.2.0, Rel-17, Cat. F |
|  | R3-226253 | Direct early data forwarding in SN initiated inter-SN CPC (Huawei, Deutsche Telekom, ZTE, CATT, Nokia, Nokia Shanghai Bell) | CR1723r, TS 36.423 v17.2.0, Rel-17, Cat. F |
|  | R3-226244 | Clarification on direct data forwarding for SN initiated CPC to TS37.340 (ZTE, CATT, Huawei) | draftCR |
|  | R3-226678 | Rel-17 early data forwarding support in SN-initiated inter-SN CPC (Intel Corporation) | CR0954r, TS 38.423 v17.2.0, Rel-17, Cat. F |
|  | R3-226679 | Rel-17 early data forwarding support in SgNB-initiated inter-SN CPC (Intel Corporation) | CR1726r, TS 36.423 v17.2.0, Rel-17, Cat. F |
|  | R3-226724 | Including data forwarding addresses for multiple T-SNs in the SN Change Confirm message in the SN-initiated inter-SN CPC procedure (Samsung) | CR0842r2, TS 38.423 v17.2.0, Rel-17, Cat. F |