3GPP TSG-RAN WG3 Meeting #118	R3-226912

Toulouse, FR, 14-18 Nov, 2022

Title: 	(TP for SON BLCR for TS 38.423): RACH optimisation

Source: 	Huawei

Agenda item:	10.2.3

Document Type:	Other

- Introduction

This document contains the TP for SON BLCR for TS 38.423 for RACH indication over Xn in MR-DC case.

Annex –Text Proposals for SON BL CR for TS 38.423

Start of the next change

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| RAN Multicast Group Paging | RAN MULTICAST GROUP PAGING |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Retrieve UE Context Confirm | RETRIEVE UE CONTEXT CONFIRM |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| RACH Indication | RACH indication |

Start of the next change

8.3.x	RACH Indication

8.2.x.1		General

This message is sent by the S-NG-RAN node to the M-NG-RAN node to inform of one or more performed random access procedures at the S-NG-RAN, due to which one or more RACH reports are available at the UE.

8.2.x.2		Successful Operation

Figure 8.2.x.2-1: RACH Indication procedure.

The S-NG-RAN node initiates the procedure by sending the RACH Indication message to M-NG-RAN node.

The RACH INDICATION message contains information concerning one or more performed random access procedures and existence of one or more RACH report at the UE.

Upon reception of the RACH INDICATION message the M-NG-RAN node may fetch the RACH report from the UE.

Editor’s note: The procedure text can be updated further based on the agreements.

8.2.x.3 	Abnormal Conditions

Not applicable.

Start of the next change

9.1.2.x	RACH INDICATION

This message is sent by the S-NG-RAN node to inform the M-NG-RAN node that one or more RACH reports are available at the UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RACH Indication List |  | 1 |  |  | YES | ignore |
| >RACH Report List Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | ignore |
| >>Random access Indication | M |  | ENUMARATED (true, …) |  | YES | ignore |
| >>M-NG-RAN UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| >>S-NG-RAN UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |

Start of the next change

9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

HandoverRequestAcknowledge,

HandoverPreparationFailure,

SNStatusTransfer,

UEContextRelease,

HandoverCancel,

NotificationControlIndication,

RANPaging,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextConfirm,

RetrieveUEContextFailure,

XnUAddressIndication,

SecondaryRATDataUsageReport,

SNodeAdditionRequest,

SNodeAdditionRequestAcknowledge,

SNodeAdditionRequestReject,

SNodeReconfigurationComplete,

SNodeModificationRequest,

SNodeModificationRequestAcknowledge,

SNodeModificationRequestReject,

SNodeModificationRequired,

SNodeModificationConfirm,

SNodeModificationRefuse,

SNodeReleaseRequest,

SNodeReleaseRequestAcknowledge,

SNodeReleaseReject,

SNodeReleaseRequired,

SNodeReleaseConfirm,

SNodeCounterCheckRequest,

SNodeChangeRequired,

SNodeChangeConfirm,

SNodeChangeRefuse,

RRCTransfer,

XnRemovalRequest,

XnRemovalResponse,

XnRemovalFailure,

XnSetupRequest,

XnSetupResponse,

XnSetupFailure,

NGRANNodeConfigurationUpdate,

NGRANNodeConfigurationUpdateAcknowledge,

NGRANNodeConfigurationUpdateFailure,

E-UTRA-NR-CellResourceCoordinationRequest,

E-UTRA-NR-CellResourceCoordinationResponse,

ActivityNotification,

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ResetRequest,

ResetResponse,

ErrorIndication,

PrivateMessage,

DeactivateTrace,

TraceStart,

HandoverSuccess,

ConditionalHandoverCancel,

EarlyStatusTransfer,

FailureIndication,

HandoverReport,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

MobilityChangeRequest,

MobilityChangeAcknowledge,

MobilityChangeFailure,

AccessAndMobilityIndication,

CellTrafficTrace,

RANMulticastGroupPaging,

ScgFailureInformationReport,

ScgFailureTransfer,

F1CTrafficTransfer,

IABTransportMigrationManagementRequest,

IABTransportMigrationManagementResponse,

IABTransportMigrationManagementReject,

IABTransportMigrationModificationRequest,

IABTransportMigrationModificationResponse,

IABResourceCoordinationRequest,

IABResourceCoordinationResponse,

CPCCancel,

PartialUEContextTransfer,

PartialUEContextTransferAcknowledge,

PartialUEContextTransferFailure,

RachIndication

FROM XnAP-PDU-Contents

id-handoverPreparation,

id-sNStatusTransfer,

id-handoverCancel,

id-notificationControl,

id-retrieveUEContext,

id-rANPaging,

id-xnUAddressIndication,

id-uEContextRelease,

id-secondaryRATDataUsageReport,

id-sNGRANnodeAdditionPreparation,

id-sNGRANnodeReconfigurationCompletion,

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-mNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeCounterCheck,

id-sNGRANnodeChange,

id-activityNotification,

id-rRCTransfer,

id-xnRemoval,

id-xnSetup,

id-nGRANnodeConfigurationUpdate,

id-e-UTRA-NR-CellResourceCoordination,

id-cellActivation,

id-reset,

id-errorIndication,

id-privateMessage,

id-deactivateTrace,

id-traceStart,

id-handoverSuccess,

id-conditionalHandoverCancel,

id-earlyStatusTransfer,

id-failureIndication,

id-handoverReport,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-mobilitySettingsChange,

id-accessAndMobilityIndication,

id-cellTrafficTrace,

id-RANMulticastGroupPaging,

id-scgFailureInformationReport,

id-scgFailureTransfer,

id-f1CTrafficTransfer,

id-iABTransportMigrationManagement,

id-iABTransportMigrationModification,

id-iABResourceCoordination,

id-retrieveUEContextConfirm,

id-cPCCancel,

id-partialUEContextTransfer,

id-rachIndication

FROM XnAP-Constants;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome				OPTIONAL,

&UnsuccessfulOutcome				OPTIONAL,

&procedureCode			ProcedureCode	UNIQUE,

&criticality			Criticality		DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE		&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE			&procedureCode

[CRITICALITY			&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

XnAP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURES XNAP-ELEMENTARY-PROCEDURE ::= {

XNAP-ELEMENTARY-PROCEDURES-CLASS-1			|

XNAP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-1 XNAP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation										|

retrieveUEContext										|

sNGRANnodeAdditionPreparation							|

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

mNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeChange										|

xnRemoval												|

xnSetup													|

nGRANnodeConfigurationUpdate							|

e-UTRA-NR-CellResourceCoordination						|

cellActivation											|

reset													|

resourceStatusReportingInitiation						|

mobilitySettingsChange									|

iABTransportMigrationManagement							|

iABTransportMigrationModification						|

iABResourceCoordination									|

partialUEContextTransfer								,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

xnUAddressIndication					|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							|

notificationControl						|

activityNotification					|

secondaryRATDataUsageReport 			|

deactivateTrace							|

traceStart								|

handoverSuccess							|

conditionalHandoverCancel				|

earlyStatusTransfer						|

failureIndication						|

handoverReport							|

resourceStatusReporting					|

accessAndMobilityIndication				|

cellTrafficTrace							|

rANMulticastGroupPaging					|

scgFailureInformationReport				|

scgFailureTransfer						|

f1CTrafficTransfer						|

retrieveUEContextConfirm				|

cPCCancel								|

rachIndication							,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

handoverPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-handoverPreparation

CRITICALITY				reject

}

sNStatusTransfer	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNStatusTransfer

PROCEDURE CODE			id-sNStatusTransfer

CRITICALITY				ignore

}

handoverCancel	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverCancel

PROCEDURE CODE			id-handoverCancel

CRITICALITY				ignore

}

retrieveUEContext	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RetrieveUEContextRequest

SUCCESSFUL OUTCOME		RetrieveUEContextResponse

UNSUCCESSFUL OUTCOME	RetrieveUEContextFailure

PROCEDURE CODE			id-retrieveUEContext

CRITICALITY				reject

}

rANPaging	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RANPaging

PROCEDURE CODE			id-rANPaging

CRITICALITY				reject

}

xnUAddressIndication	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		XnUAddressIndication

PROCEDURE CODE			id-xnUAddressIndication

CRITICALITY				reject

}

uEContextRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextRelease

PROCEDURE CODE			id-uEContextRelease

CRITICALITY				reject

}

sNGRANnodeAdditionPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeAdditionRequest

SUCCESSFUL OUTCOME		SNodeAdditionRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeAdditionRequestReject

PROCEDURE CODE			id-sNGRANnodeAdditionPreparation

CRITICALITY				reject

}

sNGRANnodeReconfigurationCompletion	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReconfigurationComplete

PROCEDURE CODE			id-sNGRANnodeReconfigurationCompletion

CRITICALITY				reject

}

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeModificationRequest

SUCCESSFUL OUTCOME		SNodeModificationRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeModificationRequestReject

PROCEDURE CODE			id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation

CRITICALITY				reject

}

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeModificationRequired

SUCCESSFUL OUTCOME		SNodeModificationConfirm

UNSUCCESSFUL OUTCOME	SNodeModificationRefuse

PROCEDURE CODE			id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation

CRITICALITY				reject

}

mNGRANnodeinitiatedSNGRANnodeRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReleaseRequest

SUCCESSFUL OUTCOME		SNodeReleaseRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeReleaseReject

PROCEDURE CODE			id-mNGRANnodeinitiatedSNGRANnodeRelease

CRITICALITY				reject

}

sNGRANnodeinitiatedSNGRANnodeRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReleaseRequired

SUCCESSFUL OUTCOME		SNodeReleaseConfirm

PROCEDURE CODE			id-sNGRANnodeinitiatedSNGRANnodeRelease

CRITICALITY				reject

}

sNGRANnodeCounterCheck	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeCounterCheckRequest

PROCEDURE CODE			id-sNGRANnodeCounterCheck

CRITICALITY				reject

}

sNGRANnodeChange		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeChangeRequired

SUCCESSFUL OUTCOME		SNodeChangeConfirm

UNSUCCESSFUL OUTCOME	SNodeChangeRefuse

PROCEDURE CODE			id-sNGRANnodeChange

CRITICALITY				reject

}

rRCTransfer	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RRCTransfer

PROCEDURE CODE			id-rRCTransfer

CRITICALITY				reject

}

xnRemoval	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		XnRemovalRequest

SUCCESSFUL OUTCOME		XnRemovalResponse

UNSUCCESSFUL OUTCOME		XnRemovalFailure

PROCEDURE CODE			id-xnRemoval

CRITICALITY				reject

}

xnSetup	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		XnSetupRequest

SUCCESSFUL OUTCOME		XnSetupResponse

UNSUCCESSFUL OUTCOME		XnSetupFailure

PROCEDURE CODE			id-xnSetup

CRITICALITY				reject

}

nGRANnodeConfigurationUpdate	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGRANNodeConfigurationUpdate

SUCCESSFUL OUTCOME		NGRANNodeConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	NGRANNodeConfigurationUpdateFailure

PROCEDURE CODE			id-nGRANnodeConfigurationUpdate

CRITICALITY				reject

}

partialUEContextTransfer	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PartialUEContextTransfer

SUCCESSFUL OUTCOME		PartialUEContextTransferAcknowledge

UNSUCCESSFUL OUTCOME	PartialUEContextTransferFailure

PROCEDURE CODE			id-partialUEContextTransfer

CRITICALITY				reject

}

e-UTRA-NR-CellResourceCoordination	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-UTRA-NR-CellResourceCoordinationRequest

SUCCESSFUL OUTCOME		E-UTRA-NR-CellResourceCoordinationResponse

PROCEDURE CODE			id-e-UTRA-NR-CellResourceCoordination

CRITICALITY				reject

}

cellActivation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CellActivationRequest

SUCCESSFUL OUTCOME		CellActivationResponse

UNSUCCESSFUL OUTCOME	CellActivationFailure

PROCEDURE CODE			id-cellActivation

CRITICALITY				reject

}

reset	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResetRequest

SUCCESSFUL OUTCOME		ResetResponse

PROCEDURE CODE			id-reset

CRITICALITY				reject

}

errorIndication	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY				ignore

}

notificationControl			XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NotificationControlIndication

PROCEDURE CODE			id-notificationControl

CRITICALITY				ignore

}

activityNotification		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ActivityNotification

PROCEDURE CODE			id-activityNotification

CRITICALITY				ignore

}

privateMessage			XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

secondaryRATDataUsageReport	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SecondaryRATDataUsageReport

PROCEDURE CODE			id-secondaryRATDataUsageReport

CRITICALITY				reject

}

deactivateTrace XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-deactivateTrace

CRITICALITY				ignore

}

traceStart XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceStart

PROCEDURE CODE			id-traceStart

CRITICALITY				ignore

}

handoverSuccess			XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverSuccess

PROCEDURE CODE			id-handoverSuccess

CRITICALITY				ignore

}

conditionalHandoverCancel	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ConditionalHandoverCancel

PROCEDURE CODE			id-conditionalHandoverCancel

CRITICALITY				ignore

}

earlyStatusTransfer		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		EarlyStatusTransfer

PROCEDURE CODE			id-earlyStatusTransfer

CRITICALITY				ignore

}

failureIndication XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		FailureIndication

PROCEDURE CODE			id-failureIndication

CRITICALITY				ignore

}

handoverReport XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverReport

PROCEDURE CODE			id-handoverReport

CRITICALITY				ignore

}

resourceStatusReportingInitiation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				ResourceStatusRequest

SUCCESSFUL OUTCOME				ResourceStatusResponse

UNSUCCESSFUL OUTCOME			ResourceStatusFailure

PROCEDURE CODE					id-resourceStatusReportingInitiation

CRITICALITY						reject

}

resourceStatusReporting XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusUpdate

PROCEDURE CODE			id-resourceStatusReporting

CRITICALITY				ignore

}

mobilitySettingsChange	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				MobilityChangeRequest

SUCCESSFUL OUTCOME				MobilityChangeAcknowledge

UNSUCCESSFUL OUTCOME			MobilityChangeFailure

PROCEDURE CODE					id-mobilitySettingsChange

CRITICALITY						reject

}

accessAndMobilityIndication XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		AccessAndMobilityIndication

PROCEDURE CODE			id-accessAndMobilityIndication

CRITICALITY 				ignore

}

cellTrafficTrace XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CellTrafficTrace

PROCEDURE CODE			id-cellTrafficTrace

CRITICALITY				ignore

}

rANMulticastGroupPaging 	XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		RANMulticastGroupPaging

PROCEDURE CODE			id-RANMulticastGroupPaging

CRITICALITY 			reject

}

scgFailureInformationReport XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		ScgFailureInformationReport

PROCEDURE CODE			id-scgFailureInformationReport

CRITICALITY 				ignore

}

scgFailureTransfer XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		ScgFailureTransfer

PROCEDURE CODE			id-scgFailureTransfer

CRITICALITY 				ignore

}

f1CTrafficTransfer			XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		F1CTrafficTransfer

PROCEDURE CODE			id-f1CTrafficTransfer

CRITICALITY				reject

}

iABTransportMigrationManagement XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		IABTransportMigrationManagementRequest

SUCCESSFUL OUTCOME		IABTransportMigrationManagementResponse

UNSUCCESSFUL OUTCOME	IABTransportMigrationManagementReject

PROCEDURE CODE			id-iABTransportMigrationManagement

CRITICALITY				reject

}

iABTransportMigrationModification XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		IABTransportMigrationModificationRequest

SUCCESSFUL OUTCOME		IABTransportMigrationModificationResponse

PROCEDURE CODE			id-iABTransportMigrationModification

CRITICALITY				reject

}

iABResourceCoordination XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		IABResourceCoordinationRequest

SUCCESSFUL OUTCOME		IABResourceCoordinationResponse

PROCEDURE CODE			id-iABResourceCoordination

CRITICALITY				reject

}

retrieveUEContextConfirm	XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		RetrieveUEContextConfirm

PROCEDURE CODE			id-retrieveUEContextConfirm

CRITICALITY 				ignore

}

cPCCancel XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		CPCCancel

PROCEDURE CODE			id-cPCCancel

CRITICALITY 			ignore

}

rachIndication XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		RachIndication

PROCEDURE CODE			id-rachIndication

CRITICALITY 			ignore

}

END

-- ASN1STOP

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

AMF-UE-NGAP-ID,

AS-SecurityInformation,

AssistanceDataForRANPaging,

BitRate,

Cause,

CellAndCapacityAssistanceInfo-EUTRA,

CellAndCapacityAssistanceInfo-NR,

CellAssistanceInfo-EUTRA,

CellAssistanceInfo-NR,

CHOinformation-Req,

CHOinformation-Ack,

CHOinformation-AddReq,

CHOinformation-ModReq,

CHO-MRDC-EarlyDataForwarding,

CHO-MRDC-Indicator,

CPTransportLayerInformation,

TNLA-To-Add-List,

TNLA-To-Update-List,

TNLA-To-Remove-List,

TNLA-Setup-List,

TNLA-Failed-To-Setup-List,

CriticalityDiagnostics,

XnUAddressInfoperPDUSession-List,

DAPSResponseInfo-List,

DataTrafficResourceIndication,

DeliveryStatus,

DesiredActNotificationLevel,

DRB-ID,

DRB-List,

DRB-Number,

DRBsSubjectToDLDiscarding-List,

DRBsSubjectToEarlyStatusTransfer-List,

DRBsSubjectToStatusTransfer-List,

DRBToQoSFlowMapping-List,

E-UTRA-CGI,

ExpectedUEActivityBehaviour,

ExpectedUEBehaviour,

ExtendedUEIdentityIndexValue,

FiveGCMobilityRestrictionListContainer,

GlobalCell-ID,

GlobalNG-RANNode-ID,

GlobalNG-RANCell-ID,

GUAMI,

InterfaceInstanceIndication,

I-RNTI,

Local-NG-RAN-Node-Identifier,

LocationInformationSNReporting,

LocationReportingInformation,

LowerLayerPresenceStatusChange,

LTEUESidelinkAggregateMaximumBitRate,

LTEV2XServicesAuthorized,

MR-DC-ResourceCoordinationInfo,

ServedCells-E-UTRA,

ServedCells-NR,

ServedCellsToUpdate-E-UTRA,

ServedCellsToUpdate-NR,

MAC-I,

MaskedIMEISV,

MDT-Configuration,

MDTPLMNList,

MobilityRestrictionList,

Neighbour-NG-RAN-Node-List,

NG-RAN-Cell-Identity,

NG-RANnodeUEXnAPID,

NR-CGI,

NE-DC-TDM-Pattern,

NRUESidelinkAggregateMaximumBitRate,

NRV2XServicesAuthorized,

PagingDRX,

EUTRAPagingeDRXInformation,

PagingPriority,

PartialListIndicator,

PLMN-Identity,

PDCPChangeIndication,

PDUSessionAggregateMaximumBitRate,

PDUSession-ID,

PDUSession-List,

PDUSession-List-withCause,

PDUSession-List-withDataForwardingFromTarget,

PDUSession-List-withDataForwardingRequest,

PDUSessionResourcesAdmitted-List,

PDUSessionResourcesNotAdmitted-List,

PDUSessionResourcesToBeSetup-List,

PDUSessionResourceChangeRequiredInfo-SNterminated,

PDUSessionResourceChangeRequiredInfo-MNterminated,

PDUSessionResourceChangeConfirmInfo-SNterminated,

PDUSessionResourceChangeConfirmInfo-MNterminated,

PDUSessionResourceSecondaryRATUsageList,

PDUSessionResourceSetupInfo-SNterminated,

PDUSessionResourceSetupInfo-MNterminated,

PDUSessionResourceSetupResponseInfo-SNterminated,

PDUSessionResourceSetupResponseInfo-MNterminated,

PDUSessionResourceModificationInfo-SNterminated,

PDUSessionResourceModificationInfo-MNterminated,

PDUSessionResourceModificationResponseInfo-SNterminated,

PDUSessionResourceModificationResponseInfo-MNterminated,

PDUSessionResourceModConfirmInfo-SNterminated,

PDUSessionResourceModConfirmInfo-MNterminated,

PDUSessionResourceModRqdInfo-SNterminated,

PDUSessionResourceModRqdInfo-MNterminated,

PDUSessionType,

PC5QoSParameters,

QoSFlowIdentifier,

QoSFlowNotificationControlIndicationInfo,

QoSFlows-List,

RANPagingArea,

ResetRequestTypeInfo,

ResetResponseTypeInfo,

RFSP-Index,

RRCConfigIndication,

RRCResumeCause,

SCGConfigurationQuery,

SCGreconfigNotification,

SecurityIndication,

S-NG-RANnode-SecurityKey,

SpectrumSharingGroupID,

SplitSRBsTypes,

S-NG-RANnode-Addition-Trigger-Ind,

S-NSSAI,

TargetCellList,

TAISupport-List,

Target-CGI,

TimeToWait,

TraceActivation,

UEAggregateMaximumBitRate,

UEContextID,

UEContextInfoRetrUECtxtResp,

UEContextKeptIndicator,

UEHistoryInformation,

UEIdentityIndexValue,

UERadioCapabilityForPaging,

UERadioCapabilityID,

UERANPagingIdentity,

UESecurityCapabilities,

UPTransportLayerInformation,

UserPlaneTrafficActivityReport,

XnBenefitValue,

RANPagingFailure,

TNLConfigurationInfo,

MaximumCellListSize,

MessageOversizeNotification,

NG-RANTraceID,

MobilityInformation,

InitiatingCondition-FailureIndication,

HandoverReportType,

TargetCellinEUTRAN,

C-RNTI,

UERLFReportContainer,

Measurement-ID,

RegistrationRequest,

ReportCharacteristics,

CellToReport,

ReportingPeriodicity,

CellMeasurementResult,

UEHistoryInformationFromTheUE,

MobilityParametersInformation,

MobilityParametersModificationRange,

RACHReportInformation,

IABNodeIndication,

SNTriggered,

SCGIndicator,

UESpecificDRX,

DirectForwardingPathAvailability,

TransportLayerAddress,

PrivacyIndicator,

URIaddress,

MBS-Session-ID,

UEIdentityIndexList-MBSGroupPaging,

MBS-SessionInformation-List,

MBS-SessionInformationResponse-List,

SuccessfulHOReportInformation,

PSCellHistoryInformationRetrieve,

SSBOffsets-List,

NG-RANnode2SSBOffsetsModificationRange,

Coverage-Modification-List,

SCGFailureReportContainer,

SNMobilityInformation,

PSCellChangeHistory,

CHOConfiguration,

SCGUEHistoryInformation,

F1CTrafficContainer,

NoPDUSessionIndication,

IAB-TNL-Address-Request,

IAB-TNL-Address-Response,

TrafficIndex,

TrafficProfile,

TrafficToBeReleaseInformation,

F1-TerminatingTopologyBHInformation,

Non-F1-TerminatingTopologyBHInformation,

BHInfoList,

IABTNLAddress,

IABCellInformation,

IABTNLAddressException,

TimeSynchronizationAssistanceInformation,

SCGActivationRequest,

SCGActivationStatus,

CPAInformationRequest,

CPAInformationAck,

CPCInformationRequired,

CPCInformationConfirm,

CPAInformationModReq,

CPAInformationModReqAck,

CPC-DataForwarding-Indicator,

CPCInformationUpdate,

CPACInformationModRequired,

QMCConfigInfo,

FiveGProSeAuthorized,

FiveGProSePC5QoSParameters,

ServedCellSpecificInfoReq-NR,

NRPagingeDRXInformation,

NRPagingeDRXInformationforRRCINACTIVE,

SDTSupportRequest,

SDT-Termination-Request,

SDTPartialUEContextInfo,

SDTDataForwardingDRBList,

PEIPSassistanceInformation,

UESliceMaximumBitRateList,

PagingCause,

MDTPLMNModificationList,

F1-terminatingIAB-donorIndicator,

SRB-ID

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

id-ActivatedServedCells,

id-ActivationIDforCellActivation,

id-AdditionalDRBIDs,

id-AMF-Region-Information,

id-AMF-Region-Information-To-Add,

id-AMF-Region-Information-To-Delete,

id-AssistanceDataForRANPaging,

id-AvailableDRBIDs,

id-Cause,

id-cellAssistanceInfo-EUTRA,

id-cellAssistanceInfo-NR,

id-CellAndCapacityAssistanceInfo-EUTRA,

id-CellAndCapacityAssistanceInfo-NR,

id-ConfigurationUpdateInitiatingNodeChoice,

id-UEContextID,

id-CriticalityDiagnostics,

id-XnUAddressInfoperPDUSession-List,

id-DesiredActNotificationLevel,

id-DRBsSubjectToStatusTransfer-List,

id-ExpectedUEBehaviour,

id-ExtendedUEIdentityIndexValue,

id-FiveGCMobilityRestrictionListContainer,

id-GlobalNG-RAN-node-ID,

id-GUAMI,

id-indexToRatFrequSelectionPriority,

id-List-of-served-cells-E-UTRA,

id-List-of-served-cells-NR,

id-LocationInformationSN,

id-LocationInformationSNReporting,

id-LocationReportingInformation,

id-LTEUESidelinkAggregateMaximumBitRate,

id-LTEV2XServicesAuthorized,

id-MAC-I,

id-MaskedIMEISV,

id-MDT-Configuration,

id-MDTPLMNList,

id-MN-to-SN-Container,

id-MobilityRestrictionList,

id-M-NG-RANnodeUEXnAPID,

id-new-NG-RAN-Cell-Identity,

id-newNG-RANnodeUEXnAPID,

id-NRUESidelinkAggregateMaximumBitRate,

id-NRV2XServicesAuthorized,

id-oldNG-RANnodeUEXnAPID,

id-OldtoNewNG-RANnodeResumeContainer,

id-PagingCause,

id-PagingDRX,

id-EUTRAPagingeDRXInformation,

id-PagingPriority,

id-PartialListIndicator-EUTRA,

id-PartialListIndicator-NR,

id-PCellID,

id-PDUSessionResourceSecondaryRATUsageList,

id-PDUSessionResourcesActivityNotifyList,

id-PDUSessionResourcesAdmitted-List,

id-PDUSessionResourcesNotAdmitted-List,

id-PDUSessionResourcesNotifyList,

id-PDUSessionToBeAddedAddReq,

id-PDUSessionToBeReleased-RelReqAck,

id-procedureStage,

id-RANPagingArea,

id-requestedSplitSRB,

id-RequiredNumberOfDRBIDs,

id-ResetRequestTypeInfo,

id-ResetResponseTypeInfo,

id-RespondingNodeTypeConfigUpdateAck,

id-RRCResumeCause,

id-SCGreconfigNotification,

id-selectedPLMN,

id-ServedCellsToActivate,

id-servedCellsToUpdate-E-UTRA,

id-ServedCellsToUpdateInitiatingNodeChoice,

id-servedCellsToUpdate-NR,

id-sourceNG-RANnodeUEXnAPID,

id-SpareDRBIDs,

id-S-NG-RANnodeMaxIPDataRate-UL,

id-S-NG-RANnodeMaxIPDataRate-DL,

id-S-NG-RANnodeUEXnAPID,

id-TAISupport-list,

id-Target2SourceNG-RANnodeTranspContainer,

id-targetCellGlobalID,

id-targetNG-RANnodeUEXnAPID,

id-TimeToWait,

id-TNLA-To-Add-List,

id-TNLA-To-Update-List,

id-TNLA-To-Remove-List,

id-TNLA-Setup-List,

id-TNLA-Failed-To-Setup-List,

id-TraceActivation,

id-UEContextInfoHORequest,

id-UEContextInfoRetrUECtxtResp,

id-UEContextKeptIndicator,

id-UEContextRefAtSN-HORequest,

id-UEHistoryInformation,

id-UEIdentityIndexValue,

id-UERANPagingIdentity,

id-UESecurityCapabilities,

id-UserPlaneTrafficActivityReport,

id-XnRemovalThreshold,

id-PDUSessionAdmittedAddedAddReqAck,

id-PDUSessionNotAdmittedAddReqAck,

id-SN-to-MN-Container,

id-RRCConfigIndication,

id-SplitSRB-RRCTransfer,

id-UEReportRRCTransfer,

id-PDUSessionReleasedList-RelConf,

id-BearersSubjectToCounterCheck,

id-PDUSessionToBeReleasedList-RelRqd,

id-ResponseInfo-ReconfCompl,

id-initiatingNodeType-ResourceCoordRequest,

id-respondingNodeType-ResourceCoordResponse,

id-PDUSessionToBeReleased-RelReq,

id-PDUSession-SNChangeRequired-List,

id-PDUSession-SNChangeConfirm-List,

id-PDCPChangeIndication,

id-PC5QoSParameters,

id-SCGConfigurationQuery,

id-UEContextInfo-SNModRequest,

id-requestedSplitSRBrelease,

id-PDUSessionAdmitted-SNModResponse,

id-PDUSessionNotAdmitted-SNModResponse,

id-admittedSplitSRB,

id-admittedSplitSRBrelease,

id-PDUSessionAdmittedModSNModConfirm,

id-PDUSessionReleasedSNModConfirm,

id-s-ng-RANnode-SecurityKey,

id-PDUSessionToBeModifiedSNModRequired,

id-S-NG-RANnodeUE-AMBR,

id-PDUSessionToBeReleasedSNModRequired,

id-target-S-NG-RANnodeID,

id-S-NSSAI,

id-MR-DC-ResourceCoordinationInfo,

id-RANPagingFailure,

id-UERadioCapabilityForPaging,

id-PDUSessionDataForwarding-SNModResponse,

id-Secondary-MN-Xn-U-TNLInfoatM,

id-NE-DC-TDM-Pattern,

id-InterfaceInstanceIndication,

id-S-NG-RANnode-Addition-Trigger-Ind,

id-SNTriggered,

id-DRBs-transferred-to-MN,

id-TNLConfigurationInfo,

id-MessageOversizeNotification,

id-NG-RANTraceID,

id-FastMCGRecoveryRRCTransfer-SN-to-MN,

id-FastMCGRecoveryRRCTransfer-MN-to-SN,

id-RequestedFastMCGRecoveryViaSRB3,

id-AvailableFastMCGRecoveryViaSRB3,

id-RequestedFastMCGRecoveryViaSRB3Release,

id-ReleaseFastMCGRecoveryViaSRB3,

id-CHOinformation-Req,

id-CHOinformation-Ack,

id-targetCellsToCancel,

id-requestedTargetCellGlobalID,

id-DAPSResponseInfo-List,

id-CHO-MRDC-EarlyDataForwarding,

id-CHO-MRDC-Indicator,

id-MobilityInformation,

id-InitiatingCondition-FailureIndication,

id-UEHistoryInformationFromTheUE,

id-HandoverReportType,

id-HandoverCause,

id-SourceCellCGI,

id-TargetCellCGI,

id-ReEstablishmentCellCGI,

id-TargetCellinEUTRAN,

id-SourceCellCRNTI,

id-UERLFReportContainer,

id-NGRAN-Node1-Measurement-ID,

id-NGRAN-Node2-Measurement-ID,

id-RegistrationRequest,

id-ReportCharacteristics,

id-CellToReport,

id-ReportingPeriodicity,

id-CellMeasurementResult,

id-NG-RANnode1CellID,

id-NG-RANnode2CellID,

id-NG-RANnode1MobilityParameters,

id-NG-RANnode2ProposedMobilityParameters,

id-MobilityParametersModificationRange,

id-RACHReportInformation,

id-IABNodeIndication,

id-UERadioCapabilityID,

id-SCGIndicator,

id-UESpecificDRX,

id-PDUSessionExpectedUEActivityBehaviour,

id-DirectForwardingPathAvailability,

id-SourceNG-RAN-node-ID,

id-TargetNodeID,

id-ManagementBasedMDTPLMNList,

id-PrivacyIndicator,

id-TraceCollectionEntityIPAddress,

id-TraceCollectionEntityURI,

id-MBS-Session-ID,

id-UEIdentityIndexList-MBSGroupPaging,

id-MulticastRANPagingArea,

id-MBS-SessionInformation-List,

id-MBS-SessionInformationResponse-List,

id-SuccessfulHOReportInformation,

id-PSCellHistoryInformationRetrieve,

id-SSBOffsets-List,

id-NG-RANnode2SSBOffsetsModificationRange,

id-Coverage-Modification-List,

id-SourcePSCellCGI,

id-FailedPSCellCGI,

id-SCGFailureReportContainer,

id-SNMobilityInformation,

id-SourcePSCellID,

id-SuitablePSCellCGI,

id-PSCellChangeHistory,

id-CHOConfiguration,

id-SCGUEHistoryInformation,

id-F1CTrafficContainer,

id-NoPDUSessionIndication,

id-F1-Terminating-IAB-DonorUEXnAPID,

id-nonF1-Terminating-IAB-DonorUEXnAPID,

id-IAB-TNL-Address-Request,

id-IAB-TNL-Address-Response,

id-TrafficToBeAddedList,

id-TrafficToBeModifiedList,

id-TrafficToBeReleaseInformation,

id-TrafficAddedList,

id-TrafficModifiedList,

id-TrafficNotAddedList,

id-TrafficNotModifiedList,

id-TrafficRequiredToBeModifiedList,

id-TrafficRequiredModifiedList,

id-TrafficReleasedList,

id-IABTNLAddressToBeAdded,

id-IABTNLAddressToBeReleasedList,

id-BoundaryNodeCellsList,

id-ParentNodeCellsList,

id-IABTNLAddressException,

id-CHOinformation-AddReq,

id-CHOinformation-ModReq,

id-TimeSynchronizationAssistanceInformation,

id-SCGActivationRequest,

id-SCGActivationStatus,

id-CPAInformationRequest,

id-CPAInformationAck,

id-CPCInformationRequired,

id-CPCInformationConfirm,

id-CPAInformationModReq,

id-CPAInformationModReqAck,

id-CPC-DataForwarding-Indicator,

id-CPCInformationUpdate,

id-CPACInformationModRequired,

id-QMCConfigInfo,

id-Local-NG-RAN-Node-Identifier,

id-Neighbour-NG-RAN-Node-List,

id-Local-NG-RAN-Node-Identifier-Removal,

id-FiveGProSeAuthorized,

id-FiveGProSePC5QoSParameters,

id-FiveGProSeUEPC5AggregateMaximumBitRate,

id-ServedCellSpecificInfoReq-NR,

id-NRPagingeDRXInformation,

id-NRPagingeDRXInformationforRRCINACTIVE,

id-SDTSupportRequest,

id-SDT-SRB-between-NewNode-OldNode,

id-SDT-Termination-Request,

id-SDTPartialUEContextInfo,

id-SDTDataForwardingDRBList,

id-PEIPSassistanceInformation,

id-UESliceMaximumBitRateList,

id-S-NG-RANnodeUE-Slice-MBR,

id-ManagementBasedMDTPLMNModificationList,

id-F1-terminatingIAB-donorIndicator,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells

FROM XnAP-Constants;

-- **************************************************************

--

#### -- HANDOVER REQUEST

--

-- **************************************************************

HandoverRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverRequest-IEs}},

...

}

HandoverRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID			CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY reject	TYPE Cause										PRESENCE mandatory}|

{ ID id-targetCellGlobalID					CRITICALITY reject	TYPE Target-CGI									PRESENCE mandatory}|

{ ID id-GUAMI								CRITICALITY reject	TYPE GUAMI										PRESENCE mandatory}|

{ ID id-UEContextInfoHORequest				CRITICALITY reject	TYPE UEContextInfoHORequest						PRESENCE mandatory}|

{ ID id-TraceActivation						CRITICALITY ignore	TYPE TraceActivation							PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore	TYPE MaskedIMEISV								PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore	TYPE UEHistoryInformation						PRESENCE mandatory}|

{ ID id-UEContextRefAtSN-HORequest			CRITICALITY ignore	TYPE UEContextRefAtSN-HORequest				PRESENCE optional }|

{ ID id-CHOinformation-Req					CRITICALITY reject	TYPE CHOinformation-Req							PRESENCE optional }|

{ ID id-NRV2XServicesAuthorized				CRITICALITY ignore	TYPE NRV2XServicesAuthorized					PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized			CRITICALITY ignore	TYPE LTEV2XServicesAuthorized					PRESENCE optional }|

{ ID id-PC5QoSParameters					CRITICALITY ignore	TYPE PC5QoSParameters							PRESENCE optional }|

{ ID id-MobilityInformation					CRITICALITY ignore	TYPE MobilityInformation						PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE		CRITICALITY ignore	TYPE UEHistoryInformationFromTheUE			PRESENCE optional }|

{ ID id-IABNodeIndication					CRITICALITY reject	TYPE IABNodeIndication							PRESENCE optional }|

{ ID id-NoPDUSessionIndication				CRITICALITY ignore	TYPE NoPDUSessionIndication						PRESENCE optional }|

{ ID id-TimeSynchronizationAssistanceInformation	CRITICALITY ignore	TYPE TimeSynchronizationAssistanceInformation		PRESENCE optional }|

{ ID id-QMCConfigInfo						CRITICALITY ignore	TYPE QMCConfigInfo								PRESENCE optional }|

{ ID id-FiveGProSeAuthorized				CRITICALITY ignore	TYPE FiveGProSeAuthorized						PRESENCE optional }|

{ ID id-FiveGProSePC5QoSParameters			CRITICALITY ignore	TYPE FiveGProSePC5QoSParameters				PRESENCE optional },

...

}

UEContextInfoHORequest ::= SEQUENCE {

ng-c-UE-reference						AMF-UE-NGAP-ID,

cp-TNL-info-source						CPTransportLayerInformation,

ueSecurityCapabilities					UESecurityCapabilities,

securityInformation						AS-SecurityInformation,

indexToRatFrequencySelectionPriority	RFSP-Index															OPTIONAL,

ue-AMBR									UEAggregateMaximumBitRate,

pduSessionResourcesToBeSetup-List		PDUSessionResourcesToBeSetup-List,

rrc-Context								OCTET STRING,

locationReportingInformation			LocationReportingInformation										OPTIONAL,

mrl										MobilityRestrictionList												OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {UEContextInfoHORequest-ExtIEs} }	OPTIONAL,

...

}

UEContextInfoHORequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ ID id-FiveGCMobilityRestrictionListContainer	CRITICALITY ignore	EXTENSION FiveGCMobilityRestrictionListContainer		PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitRate		CRITICALITY ignore	EXTENSION NRUESidelinkAggregateMaximumBitRate			PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitRate	CRITICALITY ignore	EXTENSION LTEUESidelinkAggregateMaximumBitRate			PRESENCE optional }|

{ ID id-MDTPLMNList								CRITICALITY ignore	EXTENSION MDTPLMNList											PRESENCE optional }|

{ ID id-UERadioCapabilityID						CRITICALITY reject	EXTENSION UERadioCapabilityID								PRESENCE optional }|

{ ID id-MBS-SessionInformation-List				CRITICALITY ignore	EXTENSION MBS-SessionInformation-List						PRESENCE optional }|

{ ID id-FiveGProSeUEPC5AggregateMaximumBitRate	CRITICALITY ignore	EXTENSION NRUESidelinkAggregateMaximumBitRate			PRESENCE optional }|

{ ID id-UESliceMaximumBitRateList				CRITICALITY ignore	EXTENSION UESliceMaximumBitRateList						PRESENCE optional },

...

}

UEContextRefAtSN-HORequest ::= SEQUENCE {

globalNG-RANNode-ID				GlobalNG-RANNode-ID,

sN-NG-RANnodeUEXnAPID			NG-RANnodeUEXnAPID,

iE-Extensions					ProtocolExtensionContainer { {UEContextRefAtSN-HORequest-ExtIEs} }	OPTIONAL,

...

}

UEContextRefAtSN-HORequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

-- **************************************************************

--

#### -- HANDOVER REQUEST ACKNOWLEDGE

--

-- **************************************************************

HandoverRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverRequestAcknowledge-IEs}},

...

}

HandoverRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID				CRITICALITY ignore	TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID				CRITICALITY ignore	TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-PDUSessionResourcesAdmitted-List		CRITICALITY ignore	TYPE PDUSessionResourcesAdmitted-List			PRESENCE mandatory}|

{ ID id-PDUSessionResourcesNotAdmitted-List		CRITICALITY ignore	TYPE PDUSessionResourcesNotAdmitted-List		PRESENCE optional }|

{ ID id-Target2SourceNG-RANnodeTranspContainer	CRITICALITY ignore	TYPE OCTET STRING									PRESENCE mandatory}|

{ ID id-UEContextKeptIndicator					CRITICALITY ignore	TYPE UEContextKeptIndicator						PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-DRBs-transferred-to-MN					CRITICALITY ignore	TYPE DRB-List										PRESENCE optional }|

{ ID id-DAPSResponseInfo-List					CRITICALITY reject	TYPE DAPSResponseInfo-List						PRESENCE optional }|

{ ID id-CHOinformation-Ack						CRITICALITY reject	TYPE CHOinformation-Ack								PRESENCE optional }|

{ ID id-MBS-SessionInformationResponse-List		CRITICALITY ignore	TYPE MBS-SessionInformationResponse-List		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- HANDOVER PREPARATION FAILURE

--

-- **************************************************************

HandoverPreparationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverPreparationFailure-IEs}},

...

}

HandoverPreparationFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID				CRITICALITY ignore	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause										PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional }|

{ ID id-requestedTargetCellGlobalID				CRITICALITY reject	TYPE Target-CGI									PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SN STATUS TRANSFER

--

-- **************************************************************

SNStatusTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{SNStatusTransfer-IEs}},

...

}

SNStatusTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-DRBsSubjectToStatusTransfer-List		CRITICALITY ignore		TYPE DRBsSubjectToStatusTransfer-List		PRESENCE mandatory}|

{ ID id-CHOConfiguration							CRITICALITY ignore		TYPE CHOConfiguration								PRESENCE optional}|

{ ID id-MobilityInformation						CRITICALITY ignore		TYPE MobilityInformation							PRESENCE optional},

...

}

-- **************************************************************

--

#### -- UE CONTEXT RELEASE

--

-- **************************************************************

UEContextRelease ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{UEContextRelease-IEs}},

...

}

UEContextRelease-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- HANDOVER CANCEL

--

-- **************************************************************

HandoverCancel ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverCancel-IEs}},

...

}

HandoverCancel-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause										CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-targetCellsToCancel							CRITICALITY reject		TYPE TargetCellList								PRESENCE optional},

...

}

-- **************************************************************

--

#### -- HANDOVER SUCCESS

--

-- **************************************************************

HandoverSuccess ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverSuccess-IEs}},

...

}

HandoverSuccess-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-requestedTargetCellGlobalID					CRITICALITY reject		TYPE Target-CGI										PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- CONDITIONAL HANDOVER CANCEL

--

-- **************************************************************

ConditionalHandoverCancel ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ ConditionalHandoverCancel-IEs}},

...

}

ConditionalHandoverCancel-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause										CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-targetCellsToCancel							CRITICALITY reject		TYPE TargetCellList								PRESENCE optional},

...

}

-- **************************************************************

--

#### -- EARLY STATUS TRANSFER

--

-- **************************************************************

EarlyStatusTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ EarlyStatusTransfer-IEs}},

...

}

EarlyStatusTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-procedureStage								CRITICALITY reject		TYPE ProcedureStageChoice						PRESENCE mandatory},

...

}

ProcedureStageChoice ::= CHOICE {

first-dl-count						FirstDLCount,

dl-discarding						DLDiscarding,

choice-extension					ProtocolIE-Single-Container { {ProcedureStageChoice-ExtIEs} }

}

ProcedureStageChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

FirstDLCount ::= SEQUENCE {

dRBsSubjectToEarlyStatusTransfer			DRBsSubjectToEarlyStatusTransfer-List,

iE-Extension								ProtocolExtensionContainer { {FirstDLCount-ExtIEs} }	OPTIONAL,

...

}

FirstDLCount-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DLDiscarding ::= SEQUENCE {

dRBsSubjectToDLDiscarding					DRBsSubjectToDLDiscarding-List,

iE-Extension								ProtocolExtensionContainer { {DLDiscarding-ExtIEs} }	OPTIONAL,

...

}

DLDiscarding-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- RAN PAGING

--

-- **************************************************************

RANPaging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RANPaging-IEs}},

...

}

RANPaging-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue					CRITICALITY reject		TYPE UEIdentityIndexValue						PRESENCE mandatory}|

{ ID id-UERANPagingIdentity						CRITICALITY ignore		TYPE UERANPagingIdentity							PRESENCE mandatory}|

{ ID id-PagingDRX								CRITICALITY ignore		TYPE PagingDRX										PRESENCE mandatory}|

{ ID id-RANPagingArea							CRITICALITY reject		TYPE RANPagingArea									PRESENCE mandatory}|

{ ID id-PagingPriority							CRITICALITY ignore		TYPE PagingPriority									PRESENCE optional }|

{ ID id-AssistanceDataForRANPaging				CRITICALITY ignore		TYPE AssistanceDataForRANPaging				PRESENCE optional }|

{ ID id-UERadioCapabilityForPaging				CRITICALITY ignore		TYPE UERadioCapabilityForPaging				PRESENCE optional }|

{ ID id-ExtendedUEIdentityIndexValue			CRITICALITY ignore		TYPE ExtendedUEIdentityIndexValue				PRESENCE optional }|

{ ID id-EUTRAPagingeDRXInformation				CRITICALITY ignore		TYPE EUTRAPagingeDRXInformation				PRESENCE optional }|

{ ID id-UESpecificDRX							CRITICALITY ignore		TYPE UESpecificDRX									PRESENCE optional }|

{ ID id-NRPagingeDRXInformation					CRITICALITY ignore		TYPE NRPagingeDRXInformation					PRESENCE optional }|

{ ID id-NRPagingeDRXInformationforRRCINACTIVE	CRITICALITY ignore		TYPE NRPagingeDRXInformationforRRCINACTIVE	PRESENCE optional }|

{ ID id-PagingCause								CRITICALITY ignore		TYPE PagingCause										PRESENCE optional }|

{ ID id-PEIPSassistanceInformation				CRITICALITY ignore		TYPE PEIPSassistanceInformation				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT REQUEST

--

-- **************************************************************

RetrieveUEContextRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RetrieveUEContextRequest-IEs}},

...

}

RetrieveUEContextRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-UEContextID								CRITICALITY reject		TYPE UEContextID									PRESENCE mandatory}|

{ ID id-MAC-I									CRITICALITY reject		TYPE MAC-I											PRESENCE mandatory}|

{ ID id-new-NG-RAN-Cell-Identity				CRITICALITY reject		TYPE NG-RAN-Cell-Identity						PRESENCE mandatory}|

{ ID id-RRCResumeCause							CRITICALITY ignore		TYPE RRCResumeCause									PRESENCE optional }|

{ ID id-SDTSupportRequest						CRITICALITY ignore		TYPE SDTSupportRequest							PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT RESPONSE

--

-- **************************************************************

RetrieveUEContextResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RetrieveUEContextResponse-IEs}},

...

}

RetrieveUEContextResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID						CRITICALITY ignore	TYPE NG-RANnodeUEXnAPID									PRESENCE mandatory}|

{ ID id-oldNG-RANnodeUEXnAPID						CRITICALITY ignore	TYPE NG-RANnodeUEXnAPID									PRESENCE mandatory}|

{ ID id-GUAMI										CRITICALITY reject	TYPE GUAMI													PRESENCE mandatory}|

{ ID id-UEContextInfoRetrUECtxtResp					CRITICALITY reject	TYPE UEContextInfoRetrUECtxtResp						PRESENCE mandatory}|

{ ID id-TraceActivation								CRITICALITY ignore	TYPE TraceActivation										PRESENCE optional }|

{ ID id-MaskedIMEISV								CRITICALITY ignore	TYPE MaskedIMEISV											PRESENCE optional }|

{ ID id-LocationReportingInformation				CRITICALITY ignore	TYPE LocationReportingInformation						PRESENCE optional }|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional }|

{ ID id-NRV2XServicesAuthorized						CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized					CRITICALITY ignore	TYPE LTEV2XServicesAuthorized							PRESENCE optional }|

{ ID id-PC5QoSParameters							CRITICALITY ignore	TYPE PC5QoSParameters										PRESENCE optional }|

{ ID id-UEHistoryInformation						CRITICALITY ignore	TYPE UEHistoryInformation								PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE				CRITICALITY ignore	TYPE UEHistoryInformationFromTheUE					PRESENCE optional }|

{ ID id-MDTPLMNList									CRITICALITY ignore	TYPE MDTPLMNList											PRESENCE optional }|

{ ID id-IABNodeIndication							CRITICALITY reject	TYPE IABNodeIndication									PRESENCE optional }|

{ ID id-UEContextRefAtSN-HORequest					CRITICALITY ignore	TYPE UEContextRefAtSN-HORequest						PRESENCE optional }|

{ ID id-TimeSynchronizationAssistanceInformation	CRITICALITY ignore	TYPE TimeSynchronizationAssistanceInformation		PRESENCE optional }|

{ ID id-QMCConfigInfo								CRITICALITY ignore	TYPE QMCConfigInfo											PRESENCE optional }|

{ ID id-FiveGProSeAuthorized						CRITICALITY ignore	TYPE FiveGProSeAuthorized								PRESENCE optional }|

{ ID id-FiveGProSePC5QoSParameters					CRITICALITY ignore	TYPE FiveGProSePC5QoSParameters						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT CONFIRM

--

-- **************************************************************

RetrieveUEContextConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RetrieveUEContextConfirm-IEs}},

...

}

RetrieveUEContextConfirm-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-UEContextKeptIndicator					CRITICALITY ignore		TYPE UEContextKeptIndicator						PRESENCE optional }|

{ ID id-SDT-Termination-Request					CRITICALITY ignore		TYPE SDT-Termination-Request					PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT FAILURE

--

-- **************************************************************

RetrieveUEContextFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RetrieveUEContextFailure-IEs}},

...

}

RetrieveUEContextFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-OldtoNewNG-RANnodeResumeContainer		CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- XN-U ADDRESS INDICATION

--

-- **************************************************************

XnUAddressIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnUAddressIndication-IEs}},

...

}

XnUAddressIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-XnUAddressInfoperPDUSession-List		CRITICALITY reject		TYPE XnUAddressInfoperPDUSession-List		PRESENCE mandatory}|

{ ID id-CHO-MRDC-Indicator						CRITICALITY reject		TYPE CHO-MRDC-Indicator							PRESENCE optional }|

{ ID id-CHO-MRDC-EarlyDataForwarding			CRITICALITY ignore		TYPE CHO-MRDC-EarlyDataForwarding				PRESENCE optional }|

{ ID id-CPC-DataForwarding-Indicator			CRITICALITY reject		TYPE CPC-DataForwarding-Indicator				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST

--

-- **************************************************************

SNodeAdditionRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequest-IEs}},

...

}

SNodeAdditionRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-UESecurityCapabilities				CRITICALITY reject		TYPE UESecurityCapabilities						PRESENCE mandatory}|

{ ID id-s-ng-RANnode-SecurityKey			CRITICALITY reject		TYPE S-NG-RANnode-SecurityKey					PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUE-AMBR					CRITICALITY reject		TYPE UEAggregateMaximumBitRate					PRESENCE mandatory}|

{ ID id-selectedPLMN						CRITICALITY ignore		TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-MobilityRestrictionList				CRITICALITY ignore		TYPE MobilityRestrictionList						PRESENCE optional }|

{ ID id-indexToRatFrequSelectionPriority	CRITICALITY reject		TYPE RFSP-Index										PRESENCE optional }|

{ ID id-PDUSessionToBeAddedAddReq			CRITICALITY reject		TYPE PDUSessionToBeAddedAddReq					PRESENCE mandatory}|

{ ID id-MN-to-SN-Container					CRITICALITY reject		TYPE OCTET STRING									PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE optional }|

{ ID id-ExpectedUEBehaviour					CRITICALITY ignore		TYPE ExpectedUEBehaviour							PRESENCE optional }|

{ ID id-requestedSplitSRB					CRITICALITY reject		TYPE SplitSRBsTypes									PRESENCE optional }|

{ ID id-PCellID								CRITICALITY reject		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-DesiredActNotificationLevel			CRITICALITY ignore		TYPE DesiredActNotificationLevel				PRESENCE optional }|

{ ID id-AvailableDRBIDs						CRITICALITY reject		TYPE DRB-List										PRESENCE conditional}

-- The IE shall be present if there is at least one  PDUSessionResourceSetupInfo-SNterminated included --|

{ ID id-S-NG-RANnodeMaxIPDataRate-UL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-DL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-LocationInformationSNReporting		CRITICALITY ignore		TYPE LocationInformationSNReporting			PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo		CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore		TYPE MaskedIMEISV									PRESENCE optional }|

{ ID id-NE-DC-TDM-Pattern					CRITICALITY ignore		TYPE NE-DC-TDM-Pattern								PRESENCE optional }|

{ ID id-S-NG-RANnode-Addition-Trigger-Ind	CRITICALITY reject		TYPE S-NG-RANnode-Addition-Trigger-Ind		PRESENCE optional }|

{ ID id-TraceActivation						CRITICALITY ignore		TYPE TraceActivation								PRESENCE optional }|

{ ID id-RequestedFastMCGRecoveryViaSRB3		CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3			PRESENCE optional }|

{ ID id-UERadioCapabilityID					CRITICALITY reject		TYPE UERadioCapabilityID							PRESENCE optional }|

{ ID id-SourceNG-RAN-node-ID				CRITICALITY ignore		TYPE GlobalNG-RANNode-ID							PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNList			CRITICALITY ignore		TYPE MDTPLMNList									PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore		TYPE UEHistoryInformation							PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE		CRITICALITY ignore		TYPE UEHistoryInformationFromTheUE				PRESENCE optional }|

{ ID id-PSCellChangeHistory					CRITICALITY ignore		TYPE PSCellChangeHistory							PRESENCE optional }|

{ ID id-IABNodeIndication					CRITICALITY reject		TYPE IABNodeIndication								PRESENCE optional }|

{ ID id-NoPDUSessionIndication				CRITICALITY ignore		TYPE NoPDUSessionIndication						PRESENCE optional }|

{ ID id-CHOinformation-AddReq				CRITICALITY reject		TYPE CHOinformation-AddReq						PRESENCE optional }|

{ ID id-SCGActivationRequest				CRITICALITY ignore		TYPE SCGActivationRequest							PRESENCE optional }|

{ ID id-CPAInformationRequest				CRITICALITY reject		TYPE CPAInformationRequest						PRESENCE optional }|

{ ID id-S-NG-RANnodeUE-Slice-MBR				CRITICALITY reject		TYPE UESliceMaximumBitRateList					PRESENCE optional }|

{ ID id-F1-terminatingIAB-donorIndicator		CRITICALITY reject		TYPE F1-terminatingIAB-donorIndicator			PRESENCE optional },

...

}

PDUSessionToBeAddedAddReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionToBeAddedAddReq-Item

PDUSessionToBeAddedAddReq-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

s-NSSAI					S-NSSAI,

sN-PDUSessionAMBR		PDUSessionAggregateMaximumBitRate			OPTIONAL,

sn-terminated			PDUSessionResourceSetupInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceSetupInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE

-- nor the PDU Session Resource Setup Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.1.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionToBeAddedAddReq-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionToBeAddedAddReq-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RequestedFastMCGRecoveryViaSRB3 ::= ENUMERATED {true, ...}

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SNodeAdditionRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequestAcknowledge-IEs}},

...

}

SNodeAdditionRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionAdmittedAddedAddReqAck	CRITICALITY ignore		TYPE PDUSessionAdmittedAddedAddReqAck		PRESENCE mandatory}|

{ ID id-PDUSessionNotAdmittedAddReqAck		CRITICALITY ignore		TYPE PDUSessionNotAdmittedAddReqAck		PRESENCE optional }|

{ ID id-SN-to-MN-Container					CRITICALITY reject		TYPE OCTET STRING								PRESENCE mandatory}|

{ ID id-admittedSplitSRB					CRITICALITY reject		TYPE SplitSRBsTypes								PRESENCE optional }|

{ ID id-RRCConfigIndication					CRITICALITY reject		TYPE RRCConfigIndication						PRESENCE optional }|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore		TYPE CriticalityDiagnostics					PRESENCE optional }|

{ ID id-LocationInformationSN				CRITICALITY ignore		TYPE Target-CGI									PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo		CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo		PRESENCE optional }|

{ ID id-AvailableFastMCGRecoveryViaSRB3		CRITICALITY ignore		TYPE AvailableFastMCGRecoveryViaSRB3		PRESENCE optional }|

{ ID id-DirectForwardingPathAvailability	CRITICALITY ignore		TYPE DirectForwardingPathAvailability		PRESENCE optional }|

{ ID id-SCGActivationStatus					CRITICALITY ignore		TYPE SCGActivationStatus						PRESENCE optional	}|

{ ID id-CPAInformationAck					CRITICALITY ignore		TYPE CPAInformationAck							PRESENCE optional },

...

}

PDUSessionAdmittedAddedAddReqAck ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionAdmittedAddedAddReqAck-Item

PDUSessionAdmittedAddedAddReqAck-Item ::= SEQUENCE {

pduSessionId							PDUSession-ID,

sn-terminated			PDUSessionResourceSetupResponseInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceSetupResponseInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE

-- nor the PDU Session Resource Setup Response Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.1.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedAddedAddReqAck-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedAddedAddReqAck-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionNotAdmittedAddReqAck ::= SEQUENCE {

pduSessionResourcesNotAdmitted-SNterminated		PDUSessionResourcesNotAdmitted-List OPTIONAL,

pduSessionResourcesNotAdmitted-MNterminated		PDUSessionResourcesNotAdmitted-List OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionNotAdmittedAddReqAck-ExtIEs} }	OPTIONAL,

...

}

PDUSessionNotAdmittedAddReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AvailableFastMCGRecoveryViaSRB3 ::= ENUMERATED {true, ...}

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST REJECT

--

-- **************************************************************

SNodeAdditionRequestReject ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequestReject-IEs}},

...

}

SNodeAdditionRequestReject-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE RECONFIGURATION COMPLETE

--

-- **************************************************************

SNodeReconfigurationComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReconfigurationComplete-IEs}},

...

}

SNodeReconfigurationComplete-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-ResponseInfo-ReconfCompl				CRITICALITY ignore		TYPE ResponseInfo-ReconfCompl					PRESENCE mandatory},

...

}

ResponseInfo-ReconfCompl ::= SEQUENCE {

responseType-ReconfComplete		ResponseType-ReconfComplete,

iE-Extensions						ProtocolExtensionContainer { {ResponseInfo-ReconfCompl-ExtIEs} } OPTIONAL,

...

}

ResponseInfo-ReconfCompl-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResponseType-ReconfComplete ::= CHOICE {

configuration-successfully-applied			Configuration-successfully-applied,

configuration-rejected-by-M-NG-RANNode		Configuration-rejected-by-M-NG-RANNode,

choice-extension					ProtocolIE-Single-Container { {ResponseType-ReconfComplete-ExtIEs} }

}

ResponseType-ReconfComplete-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

Configuration-successfully-applied ::= SEQUENCE {

m-NG-RANNode-to-S-NG-RANNode-Container		OCTET STRING		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {Configuration-successfully-applied-ExtIEs} } OPTIONAL,

...

}

Configuration-successfully-applied-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Configuration-rejected-by-M-NG-RANNode ::= SEQUENCE {

cause											Cause,

m-NG-RANNode-to-S-NG-RANNode-Container		OCTET STRING		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {Configuration-rejected-by-M-NG-RANNode-ExtIEs} } OPTIONAL,

...

}

Configuration-rejected-by-M-NG-RANNode-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE MODIFICATION REQUEST

--

-- **************************************************************

SNodeModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequest-IEs}},

...

}

SNodeModificationRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-PDCPChangeIndication					CRITICALITY ignore		TYPE PDCPChangeIndication						PRESENCE optional }|

{ ID id-selectedPLMN							CRITICALITY ignore		TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-MobilityRestrictionList					CRITICALITY ignore		TYPE MobilityRestrictionList					PRESENCE optional }|

{ ID id-SCGConfigurationQuery					CRITICALITY ignore		TYPE SCGConfigurationQuery						PRESENCE optional }|

{ ID id-UEContextInfo-SNModRequest				CRITICALITY reject		TYPE UEContextInfo-SNModRequest				PRESENCE optional }|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-requestedSplitSRB						CRITICALITY ignore		TYPE SplitSRBsTypes									PRESENCE optional }|

{ ID id-requestedSplitSRBrelease				CRITICALITY ignore		TYPE SplitSRBsTypes									PRESENCE optional }|

{ ID id-DesiredActNotificationLevel				CRITICALITY ignore		TYPE DesiredActNotificationLevel				PRESENCE optional }|

{ ID id-AdditionalDRBIDs						CRITICALITY reject		TYPE DRB-List										PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-UL			CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-DL			CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-LocationInformationSNReporting			CRITICALITY ignore		TYPE LocationInformationSNReporting			PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo			CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-PCellID									CRITICALITY reject		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-NE-DC-TDM-Pattern						CRITICALITY ignore		TYPE NE-DC-TDM-Pattern							PRESENCE optional}|

{ ID id-RequestedFastMCGRecoveryViaSRB3			CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3			PRESENCE optional }|

{ ID id-RequestedFastMCGRecoveryViaSRB3Release	CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3Release	PRESENCE optional }|

{ ID id-SNTriggered								CRITICALITY ignore		TYPE SNTriggered									PRESENCE optional }|

{ ID id-TargetNodeID							CRITICALITY ignore		TYPE GlobalNG-RANNode-ID							PRESENCE optional }|

{ ID id-PSCellHistoryInformationRetrieve		CRITICALITY ignore		TYPE PSCellHistoryInformationRetrieve		PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE			CRITICALITY ignore		TYPE UEHistoryInformationFromTheUE			PRESENCE optional }|

{ ID id-CHOinformation-ModReq					CRITICALITY ignore		TYPE CHOinformation-ModReq						PRESENCE optional }|

{ ID id-SCGActivationRequest					CRITICALITY ignore		TYPE SCGActivationRequest						PRESENCE optional }|

{ ID id-CPAInformationModReq					CRITICALITY ignore		TYPE CPAInformationModReq						PRESENCE optional }|

{ ID id-CPCInformationUpdate					CRITICALITY ignore		TYPE CPCInformationUpdate						PRESENCE optional }|

{ ID id-S-NG-RANnodeUE-Slice-MBR					CRITICALITY ignore		TYPE UESliceMaximumBitRateList					PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNModificationList	CRITICALITY	ignore		TYPE MDTPLMNModificationList					PRESENCE optional },

...

}

UEContextInfo-SNModRequest ::= SEQUENCE {

ueSecurityCapabilities							UESecurityCapabilities								OPTIONAL,

s-ng-RANnode-SecurityKey						S-NG-RANnode-SecurityKey							OPTIONAL,

s-ng-RANnodeUE-AMBR								UEAggregateMaximumBitRate							OPTIONAL,

indexToRatFrequencySelectionPriority			RFSP-Index											OPTIONAL,

lowerLayerPresenceStatusChange					LowerLayerPresenceStatusChange						OPTIONAL,

pduSessionResourceToBeAdded						PDUSessionsToBeAdded-SNModRequest-List				OPTIONAL,

pduSessionResourceToBeModified					PDUSessionsToBeModified-SNModRequest-List			OPTIONAL,

pduSessionResourceToBeReleased					PDUSessionsToBeReleased-SNModRequest-List			OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {UEContextInfo-SNModRequest-ExtIEs} }			OPTIONAL,

...

}

UEContextInfo-SNModRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionsToBeAdded-SNModRequest-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionsToBeAdded-SNModRequest-Item

PDUSessionsToBeAdded-SNModRequest-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

s-NSSAI					S-NSSAI,

sN-PDUSessionAMBR		PDUSessionAggregateMaximumBitRate			OPTIONAL,

sn-terminated			PDUSessionResourceSetupInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceSetupInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE

-- nor the PDU Session Resource Setup Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.3.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionsToBeAdded-SNModRequest-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionsToBeAdded-SNModRequest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-PDUSessionExpectedUEActivityBehaviour		CRITICALITY ignore	EXTENSION ExpectedUEActivityBehaviour		PRESENCE optional},

...

}

PDUSessionsToBeModified-SNModRequest-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionsToBeModified-SNModRequest-Item

PDUSessionsToBeModified-SNModRequest-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

sN-PDUSessionAMBR		PDUSessionAggregateMaximumBitRate				OPTIONAL,

sn-terminated			PDUSessionResourceModificationInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceModificationInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Modification Info – SN terminated IE

-- nor the PDU Session Resource Modification Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.3.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionsToBeModified-SNModRequest-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionsToBeModified-SNModRequest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-S-NSSAI		CRITICALITY reject	EXTENSION S-NSSAI		PRESENCE optional}|

{ID id-PDUSessionExpectedUEActivityBehaviour		CRITICALITY ignore	EXTENSION ExpectedUEActivityBehaviour		PRESENCE optional},

...

}

PDUSessionsToBeReleased-SNModRequest-List ::= SEQUENCE {

pdu-session-list		PDUSession-List-withCause				OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionsToBeReleased-SNModRequest-List-ExtIEs} }	OPTIONAL,

...

}

PDUSessionsToBeReleased-SNModRequest-List-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RequestedFastMCGRecoveryViaSRB3Release ::= ENUMERATED {true, ...}

-- **************************************************************

--

#### -- S-NODE MODIFICATION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SNodeModificationRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequestAcknowledge-IEs}},

...

}

SNodeModificationRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-PDUSessionAdmitted-SNModResponse		CRITICALITY ignore		TYPE PDUSessionAdmitted-SNModResponse			PRESENCE optional }|

{ ID id-PDUSessionNotAdmitted-SNModResponse		CRITICALITY ignore		TYPE PDUSessionNotAdmitted-SNModResponse		PRESENCE optional }|

{ ID id-SN-to-MN-Container						CRITICALITY ignore		TYPE OCTET STRING										PRESENCE optional }|

{ ID id-admittedSplitSRB						CRITICALITY ignore		TYPE SplitSRBsTypes										PRESENCE optional }|

{ ID id-admittedSplitSRBrelease					CRITICALITY ignore		TYPE SplitSRBsTypes										PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics							PRESENCE optional }|

{ ID id-LocationInformationSN					CRITICALITY ignore		TYPE Target-CGI											PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo			CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo				PRESENCE optional }|

{ ID id-PDUSessionDataForwarding-SNModResponse	CRITICALITY ignore		TYPE PDUSessionDataForwarding-SNModResponse	PRESENCE optional }|

{ ID id-RRCConfigIndication						CRITICALITY reject		TYPE RRCConfigIndication								PRESENCE optional }|

{ ID id-AvailableFastMCGRecoveryViaSRB3			CRITICALITY ignore		TYPE AvailableFastMCGRecoveryViaSRB3				PRESENCE optional }|

{ ID id-ReleaseFastMCGRecoveryViaSRB3			CRITICALITY ignore		TYPE ReleaseFastMCGRecoveryViaSRB3				PRESENCE optional }|

{ ID id-DirectForwardingPathAvailability		CRITICALITY ignore		TYPE DirectForwardingPathAvailability			PRESENCE optional }|

{ ID id-SCGUEHistoryInformation					CRITICALITY ignore		TYPE SCGUEHistoryInformation						PRESENCE optional }|

{ ID id-SCGActivationStatus						CRITICALITY ignore		TYPE SCGActivationStatus								PRESENCE optional }|

{ ID id-CPAInformationModReqAck					CRITICALITY ignore		TYPE CPAInformationModReqAck						PRESENCE optional },

...

}

PDUSessionAdmitted-SNModResponse ::= SEQUENCE {

pduSessionResourcesAdmittedToBeAdded			PDUSessionAdmittedToBeAddedSNModResponse 			OPTIONAL,

pduSessionResourcesAdmittedToBeModified			PDUSessionAdmittedToBeModifiedSNModResponse 		OPTIONAL,

pduSessionResourcesAdmittedToBeReleased			PDUSessionAdmittedToBeReleasedSNModResponse			OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmitted-SNModResponse-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmitted-SNModResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionAdmittedToBeAddedSNModResponse ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionAdmittedToBeAddedSNModResponse-Item

PDUSessionAdmittedToBeAddedSNModResponse-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

sn-terminated			PDUSessionResourceSetupResponseInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceSetupResponseInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE

-- nor the PDU Session Resource Setup Response Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.3.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedToBeAddedSNModResponse-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedToBeAddedSNModResponse-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionAdmittedToBeModifiedSNModResponse::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionAdmittedToBeModifiedSNModResponse-Item

PDUSessionAdmittedToBeModifiedSNModResponse-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

sn-terminated			PDUSessionResourceModificationResponseInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceModificationResponseInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Modification Response Info – SN terminated IE

-- nor the PDU Session Resource Modification Response Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.3.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedToBeModifiedSNModResponse-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedToBeModifiedSNModResponse-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionAdmittedToBeReleasedSNModResponse ::= SEQUENCE {

sn-terminated			PDUSession-List-withDataForwardingRequest		OPTIONAL,

mn-terminated			PDUSession-List-withCause						OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedToBeReleasedSNModResponse-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedToBeReleasedSNModResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionNotAdmitted-SNModResponse ::= SEQUENCE {

pdu-Session-List		PDUSession-List OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionNotAdmitted-SNModResponse-ExtIEs} }	OPTIONAL,

...

}

PDUSessionNotAdmitted-SNModResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionDataForwarding-SNModResponse ::= SEQUENCE {

sn-terminated		PDUSession-List-withDataForwardingRequest,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionDataForwarding-SNModResponse-ExtIEs} }	OPTIONAL,

...

}

PDUSessionDataForwarding-SNModResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ReleaseFastMCGRecoveryViaSRB3 ::= ENUMERATED {true, ...}

-- **************************************************************

--

#### -- S-NODE MODIFICATION REQUEST REJECT

--

-- **************************************************************

SNodeModificationRequestReject ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequestReject-IEs}},

...

}

SNodeModificationRequestReject-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE MODIFICATION REQUIRED

--

-- **************************************************************

SNodeModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequired-IEs}},

...

}

SNodeModificationRequired-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-PDCPChangeIndication					CRITICALITY ignore		TYPE PDCPChangeIndication						PRESENCE optional }|

{ ID id-PDUSessionToBeModifiedSNModRequired		CRITICALITY ignore		TYPE PDUSessionToBeModifiedSNModRequired	PRESENCE optional }|

{ ID id-PDUSessionToBeReleasedSNModRequired		CRITICALITY ignore		TYPE PDUSessionToBeReleasedSNModRequired	PRESENCE optional }|

{ ID id-SN-to-MN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-SpareDRBIDs								CRITICALITY ignore		TYPE DRB-List										PRESENCE optional }|

{ ID id-RequiredNumberOfDRBIDs					CRITICALITY ignore		TYPE DRB-Number										PRESENCE optional }|

{ ID id-LocationInformationSN					CRITICALITY ignore		TYPE Target-CGI										PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo			CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-RRCConfigIndication						CRITICALITY reject		TYPE RRCConfigIndication							PRESENCE optional }|

{ ID id-AvailableFastMCGRecoveryViaSRB3			CRITICALITY ignore		TYPE AvailableFastMCGRecoveryViaSRB3			PRESENCE optional }|

{ ID id-ReleaseFastMCGRecoveryViaSRB3			CRITICALITY ignore		TYPE ReleaseFastMCGRecoveryViaSRB3			PRESENCE optional }|

{ ID id-SCGIndicator							CRITICALITY ignore		TYPE SCGIndicator									PRESENCE optional }|

{ ID id-SCGUEHistoryInformation					CRITICALITY ignore		TYPE SCGUEHistoryInformation					PRESENCE optional }|

{ ID id-SCGActivationRequest					CRITICALITY ignore		TYPE SCGActivationRequest						PRESENCE optional }|

{ ID id-CPACInformationModRequired				CRITICALITY ignore		TYPE CPACInformationModRequired				PRESENCE optional }|

{ ID id-SCGreconfigNotification					CRITICALITY ignore		TYPE SCGreconfigNotification					PRESENCE optional },

...

}

PDUSessionToBeModifiedSNModRequired::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF 	PDUSessionToBeModifiedSNModRequired-Item

PDUSessionToBeModifiedSNModRequired-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

sn-terminated			PDUSessionResourceModRqdInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceModRqdInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Modification Required Info – SN terminated IE

-- nor the PDU Session Resource Modification Required Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.4.4 apply.

iE-Extension		ProtocolExtensionContainer { {PDUSessionToBeModifiedSNModRequired-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionToBeModifiedSNModRequired-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionToBeReleasedSNModRequired ::= SEQUENCE {

sn-terminated			PDUSession-List-withDataForwardingRequest		OPTIONAL,

mn-terminated			PDUSession-List-withCause						OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionToBeReleasedSNModRequired-ExtIEs} } OPTIONAL,

...

}

PDUSessionToBeReleasedSNModRequired-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE MODIFICATION CONFIRM

--

-- **************************************************************

SNodeModificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationConfirm-IEs}},

...

}

SNodeModificationConfirm-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionAdmittedModSNModConfirm		CRITICALITY ignore		TYPE PDUSessionAdmittedModSNModConfirm		PRESENCE optional }|

{ ID id-PDUSessionReleasedSNModConfirm			CRITICALITY ignore		TYPE PDUSessionReleasedSNModConfirm			PRESENCE optional }|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-AdditionalDRBIDs						CRITICALITY reject		TYPE DRB-List										PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo			CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional },

...

}

PDUSessionAdmittedModSNModConfirm ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionAdmittedModSNModConfirm-Item

PDUSessionAdmittedModSNModConfirm-Item ::= SEQUENCE {

pduSessionId				PDUSession-ID,

sn-terminated			PDUSessionResourceModConfirmInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceModConfirmInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Modification Confirm Info – SN terminated IE

-- nor the PDU Session Resource Modification Confirm Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.4.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedModSNModConfirm-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedModSNModConfirm-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionReleasedSNModConfirm ::= SEQUENCE {

sn-terminated			PDUSession-List-withDataForwardingFromTarget						OPTIONAL,

mn-terminated			PDUSession-List														OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedToBeReleasedSNModConfirm-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedToBeReleasedSNModConfirm-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE MODIFICATION REFUSE

--

-- **************************************************************

SNodeModificationRefuse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRefuse-IEs}},

...

}

SNodeModificationRefuse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE RELEASE REQUEST

--

-- **************************************************************

SNodeReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseRequest-IEs}},

...

}

SNodeReleaseRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-PDUSessionToBeReleased-RelReq			CRITICALITY ignore		TYPE PDUSession-List-withCause					PRESENCE mandatory}|

{ ID id-UEContextKeptIndicator					CRITICALITY ignore		TYPE UEContextKeptIndicator						PRESENCE optional }|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-DRBs-transferred-to-MN					CRITICALITY ignore		TYPE DRB-List										PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE RELEASE REQUEST ACKNOWLEDGE

--

-- **************************************************************

SNodeReleaseRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseRequestAcknowledge-IEs}},

...

}

SNodeReleaseRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID						CRITICALITY reject		TYPE NG-RANnodeUEXnAPID									PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID						CRITICALITY reject		TYPE NG-RANnodeUEXnAPID									PRESENCE optional }|

{ ID id-PDUSessionToBeReleased-RelReqAck			CRITICALITY ignore		TYPE PDUSessionToBeReleasedList-RelReqAck			PRESENCE optional	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore		TYPE CriticalityDiagnostics							PRESENCE optional }|

{ ID id-SCGUEHistoryInformation						CRITICALITY ignore		TYPE SCGUEHistoryInformation							PRESENCE optional },

...

}

PDUSessionToBeReleasedList-RelReqAck ::= SEQUENCE {

pduSessionsToBeReleasedList-SNterminated		PDUSession-List-withDataForwardingRequest												OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { {PDUSessionToBeReleasedList-RelReqAck-ExtIEs} }	OPTIONAL,

...

}

PDUSessionToBeReleasedList-RelReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE RELEASE REJECT

--

-- **************************************************************

SNodeReleaseReject ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseReject-IEs}},

...

}

SNodeReleaseReject-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE RELEASE REQUIRED

--

-- **************************************************************

SNodeReleaseRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseRequired-IEs}},

...

}

SNodeReleaseRequired-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionToBeReleasedList-RelRqd		CRITICALITY ignore		TYPE PDUSessionToBeReleasedList-RelRqd		PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-SN-to-MN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-SCGUEHistoryInformation					CRITICALITY ignore		TYPE SCGUEHistoryInformation					PRESENCE optional },

...

}

PDUSessionToBeReleasedList-RelRqd ::= SEQUENCE {

pduSessionsToBeReleasedList-SNterminated		PDUSession-List-withDataForwardingRequest						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionToBeReleasedList-RelRqd-ExtIEs} }	OPTIONAL,

...

}

PDUSessionToBeReleasedList-RelRqd-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE RELEASE CONFIRM

--

-- **************************************************************

SNodeReleaseConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseConfirm-IEs}},

...

}

SNodeReleaseConfirm-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionReleasedList-RelConf			CRITICALITY ignore		TYPE PDUSessionReleasedList-RelConf			PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

PDUSessionReleasedList-RelConf ::= SEQUENCE {

pduSessionsReleasedList-SNterminated		PDUSession-List-withDataForwardingFromTarget					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionReleasedList-RelConf-ExtIEs} }	OPTIONAL,

...

}

PDUSessionReleasedList-RelConf-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE COUNTER CHECK REQUEST

--

-- **************************************************************

SNodeCounterCheckRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeCounterCheckRequest-IEs}},

...

}

SNodeCounterCheckRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-BearersSubjectToCounterCheck			CRITICALITY ignore		TYPE BearersSubjectToCounterCheck-List		PRESENCE mandatory},

...

}

BearersSubjectToCounterCheck-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF BearersSubjectToCounterCheck-Item

BearersSubjectToCounterCheck-Item ::= SEQUENCE {

drb-ID							DRB-ID,

ul-count						INTEGER (0.. 4294967295),

dl-count						INTEGER (0.. 4294967295),

iE-Extensions					ProtocolExtensionContainer { {BearersSubjectToCounterCheck-Item-ExtIEs} }	OPTIONAL,

...

}

BearersSubjectToCounterCheck-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE CHANGE REQUIRED

--

-- **************************************************************

SNodeChangeRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeChangeRequired-IEs}},

...

}

SNodeChangeRequired-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-target-S-NG-RANnodeID					CRITICALITY reject		TYPE GlobalNG-RANNode-ID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-PDUSession-SNChangeRequired-List		CRITICALITY ignore		TYPE PDUSession-SNChangeRequired-List		PRESENCE optional }|

{ ID id-SN-to-MN-Container						CRITICALITY reject		TYPE OCTET STRING									PRESENCE mandatory}|

{ ID id-SCGUEHistoryInformation					CRITICALITY ignore		TYPE SCGUEHistoryInformation					PRESENCE optional }|

{ ID id-SNMobilityInformation					CRITICALITY ignore		TYPE SNMobilityInformation						PRESENCE optional }|

{ ID id-SourcePSCellID							CRITICALITY ignore		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-CPCInformationRequired					CRITICALITY ignore		TYPE CPCInformationRequired						PRESENCE optional },

...

}

PDUSession-SNChangeRequired-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSession-SNChangeRequired-Item

PDUSession-SNChangeRequired-Item ::= SEQUENCE {

pduSessionId				PDUSession-ID,

sn-terminated			PDUSessionResourceChangeRequiredInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceChangeRequiredInfo-MNterminated	OPTIONAL,

-- NOTE: If the PDU Session Resource Change Required Info – SN terminated IE is not present,

-- abnormal conditions as specified in clause 8.3.5.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSession-SNChangeRequired-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-SNChangeRequired-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE CHANGE CONFIRM

--

-- **************************************************************

SNodeChangeConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeChangeConfirm-IEs}},

...

}

SNodeChangeConfirm-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSession-SNChangeConfirm-List			CRITICALITY ignore		TYPE PDUSession-SNChangeConfirm-List			PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-CPCInformationConfirm					CRITICALITY ignore		TYPE CPCInformationConfirm						PRESENCE optional }|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional },

...

}

PDUSession-SNChangeConfirm-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSession-SNChangeConfirm-Item

PDUSession-SNChangeConfirm-Item ::= SEQUENCE {

pduSessionId				PDUSession-ID,

sn-terminated			PDUSessionResourceChangeConfirmInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceChangeConfirmInfo-MNterminated	OPTIONAL,

-- NOTE: If the PDU Session Resource Change Confirm Info – SN terminated IE is not present,

-- abnormal conditions as specified in clause 8.3.5.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSession-SNChangeConfirm-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-SNChangeConfirm-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE CHANGE REFUSE

--

-- **************************************************************

SNodeChangeRefuse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeChangeRefuse-IEs}},

...

}

SNodeChangeRefuse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RRC TRANSFER

--

-- **************************************************************

RRCTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RRCTransfer-IEs}},

...

}

RRCTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-SplitSRB-RRCTransfer					CRITICALITY reject		TYPE SplitSRB-RRCTransfer						PRESENCE optional }|

{ ID id-UEReportRRCTransfer						CRITICALITY reject		TYPE UEReportRRCTransfer							PRESENCE optional }|

{ ID id-FastMCGRecoveryRRCTransfer-SN-to-MN		CRITICALITY ignore		TYPE FastMCGRecoveryRRCTransfer				PRESENCE optional }|

{ ID id-FastMCGRecoveryRRCTransfer-MN-to-SN		CRITICALITY ignore		TYPE FastMCGRecoveryRRCTransfer				PRESENCE optional }|

{ ID id-SDT-SRB-between-NewNode-OldNode			CRITICALITY ignore		TYPE SDT-SRB-between-NewNode-OldNode			PRESENCE optional },

...

}

SplitSRB-RRCTransfer ::= SEQUENCE {

rrcContainer					OCTET STRING						OPTIONAL,

srbType							ENUMERATED {srb1, srb2, ...},

deliveryStatus					DeliveryStatus						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {SplitSRB-RRCTransfer-ExtIEs} } OPTIONAL,

...

}

SplitSRB-RRCTransfer-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEReportRRCTransfer::= SEQUENCE {

rrcContainer					OCTET STRING,

iE-Extensions					ProtocolExtensionContainer { {UEReportRRCTransfer-ExtIEs} } OPTIONAL,

...

}

UEReportRRCTransfer-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

FastMCGRecoveryRRCTransfer::= SEQUENCE {

rrcContainer					OCTET STRING,

iE-Extensions					ProtocolExtensionContainer { { FastMCGRecoveryRRCTransfer-ExtIEs} } OPTIONAL,

...

}

FastMCGRecoveryRRCTransfer-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SDT-SRB-between-NewNode-OldNode::= SEQUENCE {

rrcContainer					OCTET STRING,

srb-ID							SRB-ID,

iE-Extensions					ProtocolExtensionContainer { { SDT-SRB-between-NewNode-OldNode-ExtIEs} } OPTIONAL,

...

}

SDT-SRB-between-NewNode-OldNode-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- NOTIFICATION CONTROL INDICATION

--

-- **************************************************************

NotificationControlIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{NotificationControlIndication-IEs}},

...

}

NotificationControlIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionResourcesNotifyList		CRITICALITY reject		TYPE PDUSessionResourcesNotifyList			PRESENCE optional },

...

}

PDUSessionResourcesNotifyList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourcesNotify-Item

PDUSessionResourcesNotify-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

qosFlowsNotificationContrIndInfo	QoSFlowNotificationControlIndicationInfo,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionResourcesNotify-Item-ExtIEs} } OPTIONAL,

...

}

PDUSessionResourcesNotify-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- ACTIVITY NOTIFICATION

--

-- **************************************************************

ActivityNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ActivityNotification-IEs}},

...

}

ActivityNotification-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-UserPlaneTrafficActivityReport			CRITICALITY ignore		TYPE UserPlaneTrafficActivityReport			PRESENCE optional }|

{ ID id-PDUSessionResourcesActivityNotifyList	CRITICALITY ignore		TYPE PDUSessionResourcesActivityNotifyList	PRESENCE optional }|

{ ID id-RANPagingFailure						CRITICALITY ignore		TYPE RANPagingFailure								PRESENCE optional },

...

}

PDUSessionResourcesActivityNotifyList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourcesActivityNotify-Item

PDUSessionResourcesActivityNotify-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

pduSessionLevelUPactivityreport		UserPlaneTrafficActivityReport											OPTIONAL,

qosFlowsActivityNotifyList			QoSFlowsActivityNotifyList												OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionResourcesActivityNotify-Item-ExtIEs} } OPTIONAL,

...

}

PDUSessionResourcesActivityNotify-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsActivityNotifyList ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsActivityNotifyItem

QoSFlowsActivityNotifyItem ::= SEQUENCE {

qosFlowIdentifier					QoSFlowIdentifier,

pduSessionLevelUPactivityreport		UserPlaneTrafficActivityReport,

iE-Extensions						ProtocolExtensionContainer { {QoSFlowsActivityNotifyItem-ExtIEs} } OPTIONAL,

...

}

QoSFlowsActivityNotifyItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- XN SETUP REQUEST

--

-- **************************************************************

XnSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnSetupRequest-IEs}},

...

}

XnSetupRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-GlobalNG-RAN-node-ID				CRITICALITY reject	TYPE GlobalNG-RANNode-ID						PRESENCE mandatory}|

{ ID id-TAISupport-list						CRITICALITY reject	TYPE TAISupport-List							PRESENCE mandatory}|

{ ID id-AMF-Region-Information				CRITICALITY reject	TYPE AMF-Region-Information						PRESENCE mandatory}|

{ ID id-List-of-served-cells-NR				CRITICALITY reject	TYPE ServedCells-NR								PRESENCE optional }|

{ ID id-List-of-served-cells-E-UTRA			CRITICALITY reject	TYPE ServedCells-E-UTRA							PRESENCE optional }|

{ ID id-InterfaceInstanceIndication			CRITICALITY reject	TYPE InterfaceInstanceIndication				PRESENCE optional }|

{ ID id-TNLConfigurationInfo				CRITICALITY ignore	TYPE TNLConfigurationInfo						PRESENCE optional }|

{ ID id-PartialListIndicator-NR				CRITICALITY ignore	TYPE PartialListIndicator						PRESENCE optional }|

{ ID id-CellAndCapacityAssistanceInfo-NR 	CRITICALITY ignore 	TYPE CellAndCapacityAssistanceInfo-NR		PRESENCE optional }|

{ ID id-PartialListIndicator-EUTRA			CRITICALITY ignore	TYPE PartialListIndicator						PRESENCE optional }|

{ ID id-CellAndCapacityAssistanceInfo-EUTRA	CRITICALITY ignore 	TYPE CellAndCapacityAssistanceInfo-EUTRA	PRESENCE optional }|

{ ID id-Local-NG-RAN-Node-Identifier		CRITICALITY ignore	TYPE Local-NG-RAN-Node-Identifier			PRESENCE optional	}|

{ ID id-Neighbour-NG-RAN-Node-List			CRITICALITY ignore	TYPE Neighbour-NG-RAN-Node-List				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- XN SETUP RESPONSE

--

-- **************************************************************

XnSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnSetupResponse-IEs}},

...

}

XnSetupResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-GlobalNG-RAN-node-ID				CRITICALITY reject	TYPE GlobalNG-RANNode-ID						PRESENCE mandatory}|

{ ID id-TAISupport-list						CRITICALITY reject	TYPE TAISupport-List							PRESENCE mandatory}|

{ ID id-List-of-served-cells-NR				CRITICALITY reject	TYPE ServedCells-NR								PRESENCE optional }|

{ ID id-List-of-served-cells-E-UTRA			CRITICALITY reject	TYPE ServedCells-E-UTRA							PRESENCE optional }|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-AMF-Region-Information				CRITICALITY reject	TYPE AMF-Region-Information						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication			CRITICALITY reject	TYPE InterfaceInstanceIndication				PRESENCE optional }|

{ ID id-TNLConfigurationInfo				CRITICALITY ignore	TYPE TNLConfigurationInfo						PRESENCE optional	}|

{ ID id-PartialListIndicator-NR				CRITICALITY ignore	TYPE PartialListIndicator						PRESENCE optional }|

{ ID id-CellAndCapacityAssistanceInfo-NR 	CRITICALITY ignore 	TYPE CellAndCapacityAssistanceInfo-NR		PRESENCE optional }|

{ ID id-PartialListIndicator-EUTRA			CRITICALITY ignore	TYPE PartialListIndicator						PRESENCE optional }|

{ ID id-CellAndCapacityAssistanceInfo-EUTRA	CRITICALITY ignore 	TYPE CellAndCapacityAssistanceInfo-EUTRA	PRESENCE optional }|

{ ID id-Local-NG-RAN-Node-Identifier		CRITICALITY ignore	TYPE Local-NG-RAN-Node-Identifier			PRESENCE optional	}|

{ ID id-Neighbour-NG-RAN-Node-List			CRITICALITY ignore	TYPE Neighbour-NG-RAN-Node-List				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- XN SETUP FAILURE

--

-- **************************************************************

XnSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnSetupFailure-IEs}},

...

}

XnSetupFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait						PRESENCE optional }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional }|

{ ID id-MessageOversizeNotification	CRITICALITY ignore	TYPE MessageOversizeNotification	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- NG-RAN NODE CONFIGURATION UPDATE

--

-- **************************************************************

NGRANNodeConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ NGRANNodeConfigurationUpdate-IEs}},

...

}

NGRANNodeConfigurationUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-TAISupport-list							CRITICALITY reject	TYPE TAISupport-List									PRESENCE optional }|

{ ID id-ConfigurationUpdateInitiatingNodeChoice	CRITICALITY ignore	TYPE ConfigurationUpdateInitiatingNodeChoice	PRESENCE mandatory}|

{ ID id-TNLA-To-Add-List						CRITICALITY ignore	TYPE TNLA-To-Add-List									PRESENCE optional }|

{ ID id-TNLA-To-Remove-List						CRITICALITY ignore	TYPE TNLA-To-Remove-List								PRESENCE optional }|

{ ID id-TNLA-To-Update-List						CRITICALITY ignore	TYPE TNLA-To-Update-List								PRESENCE optional }|

{ ID id-GlobalNG-RAN-node-ID					CRITICALITY reject	TYPE GlobalNG-RANNode-ID								PRESENCE optional }|

{ ID id-AMF-Region-Information-To-Add			CRITICALITY reject	TYPE AMF-Region-Information							PRESENCE optional }|

{ ID id-AMF-Region-Information-To-Delete		CRITICALITY reject	TYPE AMF-Region-Information							PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication					PRESENCE optional }|

{ ID id-TNLConfigurationInfo					CRITICALITY ignore	TYPE TNLConfigurationInfo								PRESENCE optional	}|

{ ID id-Coverage-Modification-List				CRITICALITY reject	TYPE Coverage-Modification-List						PRESENCE optional	}|

{ ID id-Local-NG-RAN-Node-Identifier			CRITICALITY ignore	TYPE Local-NG-RAN-Node-Identifier					PRESENCE optional	}|

{ ID id-Neighbour-NG-RAN-Node-List				CRITICALITY ignore	TYPE Neighbour-NG-RAN-Node-List						PRESENCE optional	}|

{ ID id-Local-NG-RAN-Node-Identifier-Removal	CRITICALITY ignore	TYPE Local-NG-RAN-Node-Identifier					PRESENCE optional	},

...

}

ConfigurationUpdateInitiatingNodeChoice ::= CHOICE {

gNB									ProtocolIE-Container	{ {ConfigurationUpdate-gNB} },

ng-eNB								ProtocolIE-Container	{ {ConfigurationUpdate-ng-eNB} },

choice-extension					ProtocolIE-Single-Container { {ServedCellsToUpdateInitiatingNodeChoice-ExtIEs} }

}

ServedCellsToUpdateInitiatingNodeChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ConfigurationUpdate-gNB XNAP-PROTOCOL-IES ::= {

{ ID id-servedCellsToUpdate-NR			CRITICALITY ignore TYPE	ServedCellsToUpdate-NR							PRESENCE optional }|

{ ID id-cellAssistanceInfo-NR			CRITICALITY ignore TYPE	CellAssistanceInfo-NR							PRESENCE optional }|

{ ID id-cellAssistanceInfo-EUTRA		CRITICALITY ignore TYPE	CellAssistanceInfo-EUTRA						PRESENCE optional }|

{ ID id-ServedCellSpecificInfoReq-NR	CRITICALITY ignore TYPE	ServedCellSpecificInfoReq-NR					PRESENCE optional},

...

}

ConfigurationUpdate-ng-eNB XNAP-PROTOCOL-IES ::= {

{ ID id-servedCellsToUpdate-E-UTRA	CRITICALITY ignore TYPE	ServedCellsToUpdate-E-UTRA				PRESENCE optional }|

{ ID id-cellAssistanceInfo-NR		CRITICALITY ignore TYPE	CellAssistanceInfo-NR			PRESENCE optional }|

{ ID id-cellAssistanceInfo-EUTRA			CRITICALITY ignore TYPE	CellAssistanceInfo-EUTRA					PRESENCE optional },

...

}

-- **************************************************************

--

#### -- NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

NGRANNodeConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ NGRANNodeConfigurationUpdateAcknowledge-IEs}},

...

}

NGRANNodeConfigurationUpdateAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-RespondingNodeTypeConfigUpdateAck			CRITICALITY ignore	TYPE RespondingNodeTypeConfigUpdateAck		PRESENCE mandatory}|

{ ID id-TNLA-Setup-List								CRITICALITY ignore	TYPE TNLA-Setup-List								PRESENCE optional }|

{ ID id-TNLA-Failed-To-Setup-List					CRITICALITY ignore	TYPE TNLA-Failed-To-Setup-List					PRESENCE optional }|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication					CRITICALITY reject	TYPE InterfaceInstanceIndication				PRESENCE optional }|

{ ID id-TNLConfigurationInfo						CRITICALITY ignore	TYPE TNLConfigurationInfo						PRESENCE optional	}|

{ ID id-Local-NG-RAN-Node-Identifier				CRITICALITY ignore	TYPE Local-NG-RAN-Node-Identifier				PRESENCE optional	}|

{ ID id-Neighbour-NG-RAN-Node-List					CRITICALITY ignore	TYPE Neighbour-NG-RAN-Node-List				PRESENCE optional	}|

{ ID id-Local-NG-RAN-Node-Identifier-Removal		CRITICALITY ignore	TYPE Local-NG-RAN-Node-Identifier				PRESENCE optional	},

...

}

RespondingNodeTypeConfigUpdateAck ::= CHOICE {

ng-eNB					RespondingNodeTypeConfigUpdateAck-ng-eNB,

gNB						RespondingNodeTypeConfigUpdateAck-gNB,

choice-extension		ProtocolIE-Single-Container { {RespondingNodeTypeConfigUpdateAck-ExtIEs} }

}

RespondingNodeTypeConfigUpdateAck-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

RespondingNodeTypeConfigUpdateAck-ng-eNB ::= SEQUENCE {

iE-Extension		ProtocolExtensionContainer { {RespondingNodeTypeConfigUpdateAck-ng-eNB-ExtIEs} }	OPTIONAL,

...

}

RespondingNodeTypeConfigUpdateAck-ng-eNB-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-List-of-served-cells-E-UTRA				CRITICALITY ignore	EXTENSION ServedCells-E-UTRA							PRESENCE optional }|

{ ID id-PartialListIndicator-EUTRA				CRITICALITY ignore	EXTENSION PartialListIndicator						PRESENCE optional }|

{ ID id-CellAndCapacityAssistanceInfo-EUTRA		CRITICALITY ignore	EXTENSION CellAndCapacityAssistanceInfo-EUTRA	PRESENCE optional },

...

}

RespondingNodeTypeConfigUpdateAck-gNB ::= SEQUENCE {

served-NR-Cells		ServedCells-NR																OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {RespondingNodeTypeConfigUpdateAck-gNB-ExtIEs} }	OPTIONAL,

...

}

RespondingNodeTypeConfigUpdateAck-gNB-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-PartialListIndicator-NR				CRITICALITY ignore	EXTENSION PartialListIndicator						PRESENCE optional }|

{ ID id-CellAndCapacityAssistanceInfo-NR 	CRITICALITY ignore 	EXTENSION CellAndCapacityAssistanceInfo-NR		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- NG-RAN NODE CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

NGRANNodeConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{NGRANNodeConfigurationUpdateFailure-IEs}},

...

}

NGRANNodeConfigurationUpdateFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait						PRESENCE optional }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- E-UTRA NR CELL RESOURCE COORDINATION REQUEST

--

-- **************************************************************

E-UTRA-NR-CellResourceCoordinationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{E-UTRA-NR-CellResourceCoordinationRequest-IEs}},

...

}

E-UTRA-NR-CellResourceCoordinationRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-initiatingNodeType-ResourceCoordRequest	CRITICALITY reject	TYPE InitiatingNodeType-ResourceCoordRequest		PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication						PRESENCE optional },

...

}

InitiatingNodeType-ResourceCoordRequest ::= CHOICE {

ng-eNB								ResourceCoordRequest-ng-eNB-initiated,

gNB									ResourceCoordRequest-gNB-initiated,

choice-extension					ProtocolIE-Single-Container { {InitiatingNodeType-ResourceCoordRequest-ExtIEs} }

}

InitiatingNodeType-ResourceCoordRequest-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ResourceCoordRequest-ng-eNB-initiated ::= SEQUENCE {

dataTrafficResourceIndication		DataTrafficResourceIndication,

spectrumSharingGroupID				SpectrumSharingGroupID,

listofE-UTRACells					SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI							OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ResourceCoordRequest-ng-eNB-initiated-ExtIEs} }	OPTIONAL,

...

}

ResourceCoordRequest-ng-eNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResourceCoordRequest-gNB-initiated ::= SEQUENCE {

dataTrafficResourceIndication		DataTrafficResourceIndication,

listofE-UTRACells					SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI						OPTIONAL,

spectrumSharingGroupID				SpectrumSharingGroupID,

listofNRCells						SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF NR-CGI								OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ResourceCoordRequest-gNB-initiated-ExtIEs} }	OPTIONAL,

...

}

ResourceCoordRequest-gNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- E-UTRA NR CELL RESOURCE COORDINATION RESPONSE

--

-- **************************************************************

E-UTRA-NR-CellResourceCoordinationResponse::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{E-UTRA-NR-CellResourceCoordinationResponse-IEs}},

...

}

E-UTRA-NR-CellResourceCoordinationResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-respondingNodeType-ResourceCoordResponse  	CRITICALITY reject	TYPE RespondingNodeType-ResourceCoordResponse 	PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication					CRITICALITY reject	TYPE InterfaceInstanceIndication						PRESENCE optional },

...

}

RespondingNodeType-ResourceCoordResponse ::= CHOICE {

ng-eNB								ResourceCoordResponse-ng-eNB-initiated,

gNB									ResourceCoordResponse-gNB-initiated,

choice-extension					ProtocolIE-Single-Container { {RespondingNodeType-ResourceCoordResponse-ExtIEs} }

}

RespondingNodeType-ResourceCoordResponse-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ResourceCoordResponse-ng-eNB-initiated ::= SEQUENCE {

dataTrafficResourceIndication		DataTrafficResourceIndication,

spectrumSharingGroupID				SpectrumSharingGroupID,

listofE-UTRACells					SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI								OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ResourceCoordResponse-ng-eNB-initiated-ExtIEs} }		OPTIONAL,

...

}

ResourceCoordResponse-ng-eNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResourceCoordResponse-gNB-initiated ::= SEQUENCE {

dataTrafficResourceIndication		DataTrafficResourceIndication,

spectrumSharingGroupID				SpectrumSharingGroupID,

listofNRCells						SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF NR-CGI									OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ResourceCoordResponse-gNB-initiated-ExtIEs} }		OPTIONAL,

...

}

ResourceCoordResponse-gNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SECONDARY RAT DATA USAGE REPORT

--

-- **************************************************************

SecondaryRATDataUsageReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SecondaryRATDataUsageReport-IEs}},

...

}

SecondaryRATDataUsageReport-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-PDUSessionResourceSecondaryRATUsageList	CRITICALITY reject		TYPE PDUSessionResourceSecondaryRATUsageList	PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- XN REMOVAL REQUEST

--

-- **************************************************************

XnRemovalRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnRemovalRequest-IEs}},

...

}

XnRemovalRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-GlobalNG-RAN-node-ID		CRITICALITY reject	TYPE GlobalNG-RANNode-ID			PRESENCE mandatory}|

{ ID id-XnRemovalThreshold			CRITICALITY reject	TYPE XnBenefitValue					PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- XN REMOVAL RESPONSE

--

-- **************************************************************

XnRemovalResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnRemovalResponse-IEs}},

...

}

XnRemovalResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-GlobalNG-RAN-node-ID		CRITICALITY reject	TYPE GlobalNG-RANNode-ID			PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- XN REMOVAL FAILURE

--

-- **************************************************************

XnRemovalFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnRemovalFailure-IEs}},

...

}

XnRemovalFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- CELL ACTIVATION REQUEST

--

-- **************************************************************

CellActivationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ CellActivationRequest-IEs}},

...

}

CellActivationRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ServedCellsToActivate					CRITICALITY reject		TYPE ServedCellsToActivate						PRESENCE mandatory}|

{ ID id-ActivationIDforCellActivation			CRITICALITY reject		TYPE ActivationIDforCellActivation			PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

ServedCellsToActivate ::= CHOICE {

nr-cells							SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF NR-CGI,

e-utra-cells						SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI,

choice-extension					ProtocolIE-Single-Container { {ServedCellsToActivate-ExtIEs} }

}

ServedCellsToActivate-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

#### -- CELL ACTIVATION RESPONSE

--

-- **************************************************************

CellActivationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{CellActivationResponse-IEs}},

...

}

CellActivationResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ActivatedServedCells					CRITICALITY reject		TYPE ActivatedServedCells						PRESENCE mandatory}|

{ ID id-ActivationIDforCellActivation			CRITICALITY reject		TYPE ActivationIDforCellActivation			PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

ActivatedServedCells ::= CHOICE {

nr-cells							SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF NR-CGI,

e-utra-cells						SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI,

choice-extension					ProtocolIE-Single-Container { {ActivatedServedCells-ExtIEs} }

}

ActivatedServedCells-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

#### -- CELL ACTIVATION FAILURE

--

-- **************************************************************

CellActivationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{CellActivationFailure-IEs}},

...

}

CellActivationFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ActivationIDforCellActivation			CRITICALITY reject		TYPE ActivationIDforCellActivation			PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RESET REQUEST

--

-- **************************************************************

ResetRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ResetRequest-IEs}},

...

}

ResetRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ResetRequestTypeInfo					CRITICALITY reject		TYPE ResetRequestTypeInfo						PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RESET RESPONSE

--

-- **************************************************************

ResetResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ResetResponse-IEs}},

...

}

ResetResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ResetResponseTypeInfo					CRITICALITY reject		TYPE ResetResponseTypeInfo						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- ERROR INDICATION

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ErrorIndication-IEs}},

...

}

ErrorIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

-- **************************************************************

--

-- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs XNAP-PRIVATE-IES ::= {

...

}

-- **************************************************************

--

#### -- TRACE START

--

-- **************************************************************

TraceStart ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TraceStartIEs} },

...

}

TraceStartIEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation							PRESENCE optional },

...

}

-- **************************************************************

--

#### -- DEACTIVATE TRACE

--

-- **************************************************************

DeactivateTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DeactivateTraceIEs} },

...

}

DeactivateTraceIEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-NG-RANTraceID							CRITICALITY ignore	TYPE NG-RANTraceID								PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- FAILURE INDICATION

--

-- **************************************************************

FailureIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{FailureIndication-IEs}},

...

}

FailureIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-InitiatingCondition-FailureIndication				CRITICALITY reject		TYPE InitiatingCondition-FailureIndication							PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- HANDOVER REPORT

--

-- **************************************************************

HandoverReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ HandoverReport-IEs}},

...

}

HandoverReport-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-HandoverReportType			CRITICALITY ignore		TYPE HandoverReportType				PRESENCE mandatory}|

{ ID id-HandoverCause				CRITICALITY ignore		TYPE Cause							PRESENCE mandatory}|

{ ID id-SourceCellCGI				CRITICALITY ignore		TYPE GlobalNG-RANCell-ID			PRESENCE mandatory }|

{ ID id-TargetCellCGI				CRITICALITY ignore		TYPE GlobalNG-RANCell-ID			PRESENCE mandatory }|

{ ID id-ReEstablishmentCellCGI		CRITICALITY ignore		TYPE GlobalCell-ID					PRESENCE conditional }|

-- This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell"

{ ID id-TargetCellinEUTRAN   		CRITICALITY ignore		TYPE TargetCellinEUTRAN				PRESENCE conditional }|

-- This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong"

{ ID id-SourceCellCRNTI				CRITICALITY ignore		TYPE C-RNTI							PRESENCE optional }|

{ ID id-MobilityInformation			CRITICALITY ignore		TYPE MobilityInformation			PRESENCE optional }|

{ ID id-UERLFReportContainer		CRITICALITY ignore		TYPE UERLFReportContainer			PRESENCE optional }|

{ ID id-CHOConfiguration			CRITICALITY ignore		TYPE CHOConfiguration				PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS REQUEST

--

-- **************************************************************

ResourceStatusRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusRequest-IEs}},

...

}

ResourceStatusRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID			CRITICALITY ignore	TYPE Measurement-ID					PRESENCE conditional}|

-- This IE shall be present if the Registration Request IE is set to the value "stop", "partial stop" or "add".

{ ID id-RegistrationRequest					CRITICALITY reject	TYPE RegistrationRequest			PRESENCE mandatory}|

{ ID id-ReportCharacteristics				CRITICALITY reject	TYPE ReportCharacteristics			PRESENCE conditional}|

-- This IE shall be present if the Registration Request IE is set to the value "start".

{ ID id-CellToReport						CRITICALITY ignore	TYPE CellToReport					PRESENCE optional}|

{ ID id-ReportingPeriodicity				CRITICALITY ignore	TYPE ReportingPeriodicity			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS RESPONSE

--

-- **************************************************************

ResourceStatusResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusResponse-IEs}},

...

}

ResourceStatusResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID							PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS FAILURE

--

-- **************************************************************

ResourceStatusFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusFailure-IEs}},

...

}

ResourceStatusFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID				CRITICALITY reject	TYPE Measurement-ID									PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID				CRITICALITY reject	TYPE Measurement-ID									PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS UPDATE

--

-- **************************************************************

ResourceStatusUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusUpdate-IEs}},

...

}

ResourceStatusUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-CellMeasurementResult			CRITICALITY ignore	TYPE CellMeasurementResult			PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- MOBILITY CHANGE REQUEST

--

-- **************************************************************

MobilityChangeRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MobilityChangeRequest-IEs}},

...

}

MobilityChangeRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NG-RANnode1CellID						CRITICALITY reject	TYPE GlobalNG-RANCell-ID							PRESENCE mandatory}|

{ ID id-NG-RANnode2CellID							CRITICALITY reject	TYPE GlobalNG-RANCell-ID							PRESENCE mandatory}|

{ ID id-NG-RANnode1MobilityParameters				CRITICALITY reject	TYPE MobilityParametersInformation				PRESENCE optional}|

{ ID id-NG-RANnode2ProposedMobilityParameters	CRITICALITY reject	TYPE MobilityParametersInformation				PRESENCE optional}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-SSBOffsets-List								CRITICALITY ignore	TYPE SSBOffsets-List								PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MOBILITY CHANGE ACKNOWLEDGE

--

-- **************************************************************

MobilityChangeAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MobilityChangeAcknowledge-IEs}},

...

}

MobilityChangeAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NG-RANnode1CellID					CRITICALITY reject	TYPE GlobalNG-RANCell-ID							PRESENCE mandatory}|

{ ID id-NG-RANnode2CellID					CRITICALITY reject	TYPE GlobalNG-RANCell-ID							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MOBILITY CHANGE FAILURE

--

-- **************************************************************

MobilityChangeFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MobilityChangeFailure-IEs}},

...

}

MobilityChangeFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NG-RANnode1CellID						CRITICALITY reject	TYPE GlobalNG-RANCell-ID									PRESENCE mandatory}|

{ ID id-NG-RANnode2CellID							CRITICALITY reject	TYPE GlobalNG-RANCell-ID									PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause													PRESENCE mandatory}|

{ ID id-MobilityParametersModificationRange 	CRITICALITY reject	TYPE MobilityParametersModificationRange				PRESENCE optional}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional}|

{ ID id-NG-RANnode2SSBOffsetsModificationRange	CRITICALITY ignore	TYPE NG-RANnode2SSBOffsetsModificationRange			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- ACCESS AND MOBILITY INDICATION

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ AccessAndMobilityIndication-IEs}},

...

}

AccessAndMobilityIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-RACHReportInformation			CRITICALITY ignore		TYPE RACHReportInformation					PRESENCE optional}|

{ ID id-SuccessfulHOReportInformation		CRITICALITY ignore		TYPE SuccessfulHOReportInformation			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- CELL TRAFFIC TRACE

--

-- **************************************************************

CellTrafficTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {CellTrafficTraceIEs} },

...

}

CellTrafficTraceIEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID			CRITICALITY reject	TYPE NG-RANnodeUEXnAPID				PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID			CRITICALITY reject	TYPE NG-RANnodeUEXnAPID				PRESENCE mandatory}|

{ ID id-NG-RANTraceID					CRITICALITY ignore	TYPE NG-RANTraceID				PRESENCE mandatory	}|

{ ID id-TraceCollectionEntityIPAddress	CRITICALITY ignore	TYPE TransportLayerAddress			PRESENCE mandatory	}|

{ ID id-PrivacyIndicator				CRITICALITY ignore	TYPE PrivacyIndicator				PRESENCE optional	}|

{ ID id-TraceCollectionEntityURI		CRITICALITY ignore	TYPE URIaddress						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RAN MULTICAST GROUP PAGING

--

-- **************************************************************

RANMulticastGroupPaging ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{RANMulticastGroupPaging-IEs}},

...

}

RANMulticastGroupPaging-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-MBS-Session-ID							CRITICALITY reject	TYPE MBS-Session-ID									PRESENCE mandatory}|

{ ID id-UEIdentityIndexList-MBSGroupPaging			CRITICALITY reject	TYPE UEIdentityIndexList-MBSGroupPaging		PRESENCE mandatory}|

{ ID id-MulticastRANPagingArea					CRITICALITY reject	TYPE RANPagingArea									PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- SCG FAILURE INFORMATION REPORT

--

-- **************************************************************

ScgFailureInformationReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ ScgFailureInformationReport-IEs}},

...

}

ScgFailureInformationReport-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory }|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory }|

{ ID id-SourcePSCellCGI								CRITICALITY ignore		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-FailedPSCellCGI								CRITICALITY ignore		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-SCGFailureReportContainer					CRITICALITY ignore		TYPE SCGFailureReportContainer					PRESENCE mandatory }|

{ ID id-SNMobilityInformation				CRITICALITY ignore		TYPE SNMobilityInformation						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- SCG FAILURE TRANSFER

--

-- **************************************************************

ScgFailureTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ ScgFailureTransfer-IEs}},

...

}

ScgFailureTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory }|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- F1-C TRAFFIC TRANSFER

--

-- **************************************************************

F1CTrafficTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{ F1CTrafficTransfer-IEs}},

...

}

F1CTrafficTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID						CRITICALITY reject	TYPE NG-RANnodeUEXnAPID			PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID						CRITICALITY reject	TYPE NG-RANnodeUEXnAPID			PRESENCE mandatory}|

{ ID id-F1CTrafficContainer							CRITICALITY reject	TYPE F1CTrafficContainer			PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- IAB TRANSPORT MIGRATION MANAGEMENT REQUEST

--

-- **************************************************************

IABTransportMigrationManagementRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ IABTransportMigrationManagementRequest-IEs}},

...

}

IABTransportMigrationManagementRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-F1-Terminating-IAB-DonorUEXnAPID		CRITICALITY reject		TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-nonF1-Terminating-IAB-DonorUEXnAPID		CRITICALITY reject		TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-TrafficToBeAddedList					CRITICALITY reject		TYPE TrafficToBeAddedList					PRESENCE optional }|

{ ID id-TrafficToBeModifiedList					CRITICALITY reject		TYPE TrafficToBeModifiedList				PRESENCE optional }|

{ ID id-TrafficToBeReleaseInformation			CRITICALITY reject		TYPE TrafficToBeReleaseInformation		PRESENCE optional }|

{ ID id-IAB-TNL-Address-Request					CRITICALITY reject		TYPE IAB-TNL-Address-Request				PRESENCE optional }|

{ ID id-IABTNLAddressException					CRITICALITY reject		TYPE IABTNLAddressException					PRESENCE optional },

...

}

TrafficToBeAddedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficToBeAdded-Item

TrafficToBeAdded-Item ::= SEQUENCE {

trafficIndex			TrafficIndex,

trafficProfile			TrafficProfile,

f1-TerminatingTopologyBHInformation		F1-TerminatingTopologyBHInformation		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {TrafficToBeAdded-Item-ExtIEs} }	OPTIONAL,

...

}

TrafficToBeAdded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TrafficToBeModifiedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficToBeModified-Item

TrafficToBeModified-Item ::= SEQUENCE {

trafficIndex			TrafficIndex,

trafficProfile			TrafficProfile		OPTIONAL,

f1-TerminatingTopologyBHInformation		F1-TerminatingTopologyBHInformation		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {TrafficToBeModified-Item-ExtIEs} }	OPTIONAL,

...

}

TrafficToBeModified-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE

--

-- **************************************************************

IABTransportMigrationManagementResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ IABTransportMigrationManagementResponse-IEs}},

...

}

IABTransportMigrationManagementResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-F1-Terminating-IAB-DonorUEXnAPID		CRITICALITY reject		TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-nonF1-Terminating-IAB-DonorUEXnAPID		CRITICALITY reject		TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-TrafficAddedList						CRITICALITY reject		TYPE TrafficAddedList							PRESENCE optional }|

{ ID id-TrafficModifiedList						CRITICALITY reject		TYPE TrafficModifiedList						PRESENCE optional }|

{ ID id-TrafficNotAddedList						CRITICALITY reject		TYPE TrafficNotAddedList						PRESENCE optional }|

{ ID id-TrafficNotModifiedList					CRITICALITY reject		TYPE TrafficNotModifiedList					PRESENCE optional }|

{ ID id-IAB-TNL-Address-Response				CRITICALITY reject		TYPE IAB-TNL-Address-Response				PRESENCE optional }|

{ ID id-TrafficReleasedList						CRITICALITY reject		TYPE TrafficReleasedList						PRESENCE optional },

...

}

TrafficAddedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficAdded-Item

TrafficAdded-Item ::= SEQUENCE {

trafficIndex			TrafficIndex,

non-F1-TerminatingTopologyBHInformation		Non-F1-TerminatingTopologyBHInformation,

iE-Extensions			ProtocolExtensionContainer { {TrafficAdded-Item-ExtIEs} }	OPTIONAL,

...

}

TrafficAdded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TrafficModifiedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficModified-Item

TrafficModified-Item ::= SEQUENCE {

trafficIndex			TrafficIndex,

non-F1-TerminatingTopologyBHInformation		Non-F1-TerminatingTopologyBHInformation,

iE-Extensions			ProtocolExtensionContainer { {TrafficModified-Item-ExtIEs} }	OPTIONAL,

...

}

TrafficModified-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TrafficNotAddedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficNotAdded-Item

TrafficNotAdded-Item ::= SEQUENCE {

trafficIndex			TrafficIndex,

casue					Cause			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {TrafficNotAdded-Item-ExtIEs} }	OPTIONAL,

...

}

TrafficNotAdded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TrafficNotModifiedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficNotModified-Item

TrafficNotModified-Item ::= SEQUENCE {

trafficIndex			TrafficIndex,

cause					Cause		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {TrafficNotModified-Item-ExtIEs} }	OPTIONAL,

...

}

TrafficNotModified-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TrafficReleasedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficReleased-Item

TrafficReleased-Item ::= SEQUENCE {

trafficIndex			TrafficIndex,

bHInfoList				BHInfoList		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { TrafficReleased-Item-ExtIEs} }	OPTIONAL,

...

}

TrafficReleased-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- IAB TRANSPORT MIGRATION MANAGEMENT REJECT

--

-- **************************************************************

IABTransportMigrationManagementReject ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ IABTransportMigrationManagementReject-IEs}},

...

}

IABTransportMigrationManagementReject-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-F1-Terminating-IAB-DonorUEXnAPID		CRITICALITY reject		TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-nonF1-Terminating-IAB-DonorUEXnAPID		CRITICALITY reject		TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause										PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics					PRESENCE optional },

...

}

-- **************************************************************

--

#### -- IAB TRANSPORT MIGRATION MODIFICATION REQUEST

--

-- **************************************************************

IABTransportMigrationModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ IABTransportMigrationModificationRequest-IEs}},

...

}

IABTransportMigrationModificationRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-F1-Terminating-IAB-DonorUEXnAPID			CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-nonF1-Terminating-IAB-DonorUEXnAPID			CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-TrafficRequiredToBeModifiedList				CRITICALITY reject	TYPE TrafficRequiredToBeModifiedList			PRESENCE optional }|

{ ID id-TrafficToBeReleaseInformation				CRITICALITY reject	TYPE TrafficToBeReleaseInformation			PRESENCE optional }|

{ ID id-IABTNLAddressToBeAdded						CRITICALITY reject	TYPE IAB-TNL-Address-Response					PRESENCE optional }|

{ ID id-IABTNLAddressToBeReleasedList				CRITICALITY reject	TYPE IABTNLAddressToBeReleasedList			PRESENCE optional },

...

}

TrafficRequiredToBeModifiedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficRequiredToBeModified-Item

TrafficRequiredToBeModified-Item ::= SEQUENCE {

trafficIndex			TrafficIndex,

non-f1-TerminatingTopologyBHInformation		Non-F1-TerminatingTopologyBHInformation,

iE-Extensions			ProtocolExtensionContainer{ { TrafficRequiredToBeModified-Item-ExtIEs} }		OPTIONAL,

...

}

TrafficRequiredToBeModified-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IABTNLAddressToBeReleasedList ::= SEQUENCE (SIZE(1..maxnoofTLAsIAB)) OF IABTNLAddressToBeReleased-Item

IABTNLAddressToBeReleased-Item ::= SEQUENCE {

iabTNLAddress			IABTNLAddress,

iE-Extensions			ProtocolExtensionContainer{ { IABTNLAddressToBeReleased-Item-ExtIEs} }	OPTIONAL,

...

}

IABTNLAddressToBeReleased-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- IAB TRANSPORT MIGRATION MODIFICATION RESPONSE

--

-- **************************************************************

IABTransportMigrationModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ IABTransportMigrationModificationResponse-IEs}},

...

}

IABTransportMigrationModificationResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-F1-Terminating-IAB-DonorUEXnAPID			CRITICALITY reject	TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-nonF1-Terminating-IAB-DonorUEXnAPID			CRITICALITY reject	TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-TrafficRequiredModifiedList					CRITICALITY reject	TYPE TrafficRequiredModifiedList			PRESENCE optional }|

{ ID id-TrafficReleasedList							CRITICALITY reject	TYPE TrafficReleasedList						PRESENCE optional },

...

}

TrafficRequiredModifiedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficRequiredModified-Item

TrafficRequiredModified-Item ::= SEQUENCE {

trafficIndex			TrafficIndex,

iE-Extensions			ProtocolExtensionContainer { { TrafficRequiredModified-Item-ExtIEs} }	OPTIONAL,

...

}

TrafficRequiredModified-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- IAB RESOURCE COORDINATION REQUEST

--

-- **************************************************************

IABResourceCoordinationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ IABResourceCoordinationRequest-IEs}},

...

}

IABResourceCoordinationRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-F1-Terminating-IAB-DonorUEXnAPID	CRITICALITY reject		TYPE NG-RANnodeUEXnAPID					PRESENCE mandatory}|

{ ID id-nonF1-Terminating-IAB-DonorUEXnAPID	CRITICALITY reject		TYPE NG-RANnodeUEXnAPID					PRESENCE mandatory}|

{ ID id-BoundaryNodeCellsList				CRITICALITY reject		TYPE BoundaryNodeCellsList			PRESENCE optional }|

{ ID id-ParentNodeCellsList					CRITICALITY reject		TYPE ParentNodeCellsList				PRESENCE optional },

...

}

BoundaryNodeCellsList ::= SEQUENCE (SIZE(1..maxnoofServedCellsIAB)) OF BoundaryNodeCellsList-Item

BoundaryNodeCellsList-Item ::= SEQUENCE {

boundaryNodeCellInformation			IABCellInformation,

iE-Extensions						ProtocolExtensionContainer { {BoundaryNodeCellsList-Item-ExtIEs} }		OPTIONAL,

...

}

BoundaryNodeCellsList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ParentNodeCellsList ::= SEQUENCE (SIZE(1..maxnoofServingCells)) OF ParentNodeCellsList-Item

ParentNodeCellsList-Item ::= SEQUENCE {

parentNodeCellInformation			IABCellInformation,

iE-Extensions						ProtocolExtensionContainer { {ParentNodeCellsList-Item-ExtIEs} }	OPTIONAL,

...

}

ParentNodeCellsList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- IAB RESOURCE COORDINATION RESPONSE

--

-- **************************************************************

IABResourceCoordinationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ IABResourceCoordinationResponse-IEs}},

...

}

IABResourceCoordinationResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-F1-Terminating-IAB-DonorUEXnAPID		CRITICALITY reject		TYPE NG-RANnodeUEXnAPID					PRESENCE mandatory}|

{ ID id-nonF1-Terminating-IAB-DonorUEXnAPID		CRITICALITY reject		TYPE NG-RANnodeUEXnAPID					PRESENCE mandatory}|

{ ID id-BoundaryNodeCellsList					CRITICALITY reject		TYPE BoundaryNodeCellsList				PRESENCE optional }|

{ ID id-ParentNodeCellsList						CRITICALITY reject		TYPE ParentNodeCellsList					PRESENCE optional },

...

}

-- **************************************************************

--

#### -- CONDITIONAL PSCELL CHANGE CANCEL

--

-- **************************************************************

CPCCancel ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ CPCCancel-IEs}},

...

}

CPCCancel-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore		TYPE Cause										PRESENCE optional }|

{ ID id-target-S-NG-RANnodeID				CRITICALITY reject		TYPE GlobalNG-RANNode-ID						PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- PARTIAL UE CONTEXT TRANSFER

--

-- **************************************************************

PartialUEContextTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ PartialUEContextTransfer-IEs}},

...

}

PartialUEContextTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-SDTPartialUEContextInfo					CRITICALITY ignore		TYPE SDTPartialUEContextInfo					PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE

--

-- **************************************************************

PartialUEContextTransferAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ PartialUEContextTransferAcknowledge-IEs}},

...

}

PartialUEContextTransferAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-SDTDataForwardingDRBList				CRITICALITY ignore		TYPE SDTDataForwardingDRBList					PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- PARTIAL UE CONTEXT TRANSFER FAILURE

--

-- **************************************************************

PartialUEContextTransferFailure::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ PartialUEContextTransferFailure-IEs}},

...

}

PartialUEContextTransferFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RACH INDICATION

--

-- **************************************************************

RachIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RachIndication-IEs}},

...

}

RachIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-RachIndicationList						CRITICALITY ignore		TYPE RachIndicaionList							PRESENCE mandatory },

...

}

END

-- ASN1STOP

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-DefaultDRB-Allowed,

id-DLCarrierList,

id-EndpointIPAddressAndPort,

id-ExtendedReportIntervalMDT,

id-ExtendedTAISliceSupportList,

id-FiveGCMobilityRestrictionListContainer,

id-SecondarydataForwardingInfoFromTarget-List,

id-LastE-UTRANPLMNIdentity,

id-IntendedTDD-DL-ULConfiguration-NR,

id-MaxIPrate-DL,

id-SecurityResult,

id-OldQoSFlowMap-ULendmarkerexpected,

id-PDUSessionCommonNetworkInstance,

id-PDUSession-PairID,

id-BPLMN-ID-Info-EUTRA,

id-BPLMN-ID-Info-NR,

id-DRBsNotAdmittedSetupModifyList,

id-Secondary-MN-Xn-U-TNLInfoatM,

id-ULForwardingProposal,

id-DRB-IDs-takenintouse,

id-SplitSessionIndicator,

id-NonGBRResources-Offered,

id-MDT-Configuration,

id-TraceCollectionEntityURI,

id-NPN-Broadcast-Information,

id-NPNPagingAssistanceInformation,

id-NPNMobilityInformation,

id-NPN-Support,

id-LTEUESidelinkAggregateMaximumBitRate,

id-NRUESidelinkAggregateMaximumBitRate,

id-ExtendedRATRestrictionInformation,

id-QoSMonitoringRequest,

id-QoSMonitoringDisabled,

id-QosMonitoringReportingFrequency,

id-DAPSRequestInfo,

id-OffsetOfNbiotChannelNumberToDL-EARFCN,

id-OffsetOfNbiotChannelNumberToUL-EARFCN,

id-NBIoT-UL-DL-AlignmentOffset,

id-TDDULDLConfigurationCommonNR,

id-CarrierList,

id-ULCarrierList,

id-FrequencyShift7p5khz,

id-SSB-PositionsInBurst,

id-NRCellPRACHConfig,

id-Redundant-UL-NG-U-TNLatUPF,

id-Redundant-DL-NG-U-TNLatNG-RAN,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-ExtendedPacketDelayBudget,

id-Additional-Redundant-UL-NG-U-TNLatUPF-List,

id-RedundantCommonNetworkInstance,

id-TSCTrafficCharacteristics,

id-RedundantQoSFlowIndicator,

id-Additional-PDCP-Duplication-TNL-List,

id-RedundantPDUSessionInformation,

id-UsedRSNInformation,

id-RLCDuplicationInformation,

id-CSI-RSTransmissionIndication,

id-UERadioCapabilityID,

id-secondary-SN-UL-PDCP-UP-TNLInfo,

id-pdcpDuplicationConfiguration,

id-duplicationActivation,

id-NPRACHConfiguration,

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated,

id-DL-scheduling-PDCCH-CCE-usage,

id-UL-scheduling-PDCCH-CCE-usage,

id-SFN-Offset,

id-QoS-Mapping-Information,

id-AdditionLocationInformation,

id-dataForwardingInfoFromTargetE-UTRANnode,

id-Cause,

id-SecurityIndication,

id-RRCConnReestab-Indicator,

id-SourceDLForwardingIPAddress,

id-SourceNodeDLForwardingIPAddress,

id-M4ReportAmount,

id-M5ReportAmount,

id-M6ReportAmount,

id-M7ReportAmount,

id-BeamMeasurementIndicationM1,

id-Supported-MBS-FSA-ID-List,

id-MBS-SessionAssociatedInformation,

id-MBS-SessionInformation-List,

id-SliceRadioResourceStatus-List,

id-CompositeAvailableCapacitySupplementaryUplink,

id-SSBOffsets-List,

id-NG-RANnode2SSBOffsetsModificationRange,

id-NR-U-Channel-List,

id-NR-U-ChannelInfo-List,

id-MIMOPRBusageInformation,

id-UEAssistantIdentifier,

id-IAB-MT-Cell-List,

id-NoPDUSessionIndication,

id-permutation,

id-UL-GNB-DU-Cell-Resource-Configuration,

id-DL-GNB-DU-Cell-Resource-Configuration,

id-tdd-GNB-DU-Cell-Resource-Configuration,

id-Additional-Measurement-Timing-Configuration-List,

id-SurvivalTime,

id-Local-NG-RAN-Node-Identifier,

id-Neighbour-NG-RAN-Node-List,

id-FiveGProSeUEPC5AggregateMaximumBitRate,

id-Redcap-Bcast-Information,

id-UESliceMaximumBitRateList,

id-PositioningInformation,

id-ServedCellSpecificInfoReq-NR,

id-TAINSAGSupportList,

id-earlyMeasurement,

id-BeamMeasurementsReportConfiguration,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

maxnoofCAGs,

maxnoofCAGsperPLMN,

maxnoofCellsinAoI,

maxnoofCellsinNG-RANnode,

maxnoofCellsinRNA,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEPLMNs,

maxnoofEPLMNsplus1,

maxnoofEUTRABands,

maxnoofEUTRABPLMNs,

maxnoofForbiddenTACs,

maxnoofMBSFNEUTRA,

maxnoofMultiConnectivityMinusOne,

maxnoofNeighbours,

maxnoofNIDs,

maxnoofNRCellBands,

maxnoofPDUSessions,

maxnoofPLMNs,

maxnoofProtectedResourcePatterns,

maxnoofQoSFlows,

maxnoofQoSParaSets,

maxnoofRANAreaCodes,

maxnoofRANAreasinRNA,

maxnoofSCellGroups,

maxnoofSCellGroupsplus1,

maxnoofSliceItems,

maxnoofExtSliceItems,

maxnoofSNPNIDs,

maxnoofsupportedTACs,

maxnoofsupportedPLMNs,

maxnoofTAI,

maxnoofTAIsinAoI,

maxnoofTNLAssociations,

maxnoofUEContexts,

maxNRARFCN,

maxNrOfErrors,

maxnoofRANNodesinAoI,

maxnooftimeperiods,

maxnoofslots,

maxnoofExtTLAs,

maxnoofGTPTLAs,

maxnoofCHOcells,

maxnoofPC5QoSFlows,

maxnoofSSBAreas,

maxnoofNRSCSs,

maxnoofPhysicalResourceBlocks,

maxnoofRACHReports,

maxnoofAdditionalPDCPDuplicationTNL,

maxnoofRLCDuplicationstate,

maxnoofBluetoothName,

maxnoofCellIDforMDT,

maxnoofMDTPLMNs,

maxnoofTAforMDT,

maxnoofWLANName,

maxnoofSensorName,

maxnoofNeighPCIforMDT,

maxnoofFreqforMDT,

maxnoofNonAnchorCarrierFreqConfig,

maxnoofDataForwardingTunneltoE-UTRAN,

maxnoofUEIDIndicesforMBSPaging,

maxnoofMBSFSAs,

maxnoofMBSQoSFlows,

maxnoofMRBs,

maxnoofCellsforMBS,

maxnoofMBSServiceAreaInformation,

maxnoofTAIforMBS,

maxnoofAssociatedMBSSessions,

maxnoofMBSSessions,

maxnoofSuccessfulHOReports,

maxnoofPSCellsPerSN,

maxnoofNR-UChannelIDs,

maxnoofCellsinCHO,

maxnoofCHOexecutioncond,

maxnoofServingCells,

maxnoofBHInfo,

maxnoofTLAsIAB,

maxnoofTrafficIndexEntries,

maxnoofBAPControlPDURLCCHs,

maxnoofServedCellsIAB,

maxnoofDUFSlots,

maxnoofSymbols,

maxnoofHSNASlots,

maxnoofRBsetsPerCell,

maxnoofChildIABNodes,

maxnoofIABSTCInfo,

maxnoofPSCellCandidates,

maxnoofTargetSNs,

maxnoofUEAppLayerMeas,

maxnoofSNSSAIforQMC,

maxnoofCellIDforQMC,

maxnoofPLMNforQMC,

maxnoofTAforQMC,

maxnoofMTCItems,

maxnoofCSIRSconfigurations,

maxnoofCSIRSneighbourCells,

maxnoofCSIRSneighbourCellsInMTC,

maxnoofNeighbour-NG-RAN-Nodes,

maxnoofSRBs,

maxnoofSMBR,

maxnoofNSAGs,

maxnoofRBsetsPerCell1

FROM XnAP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM XnAP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Single-Container{},

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES

FROM XnAP-Containers;

#### -- A

AdditionLocationInformation ::= ENUMERATED {

includePSCell,

...

}

Additional-PDCP-Duplication-TNL-List ::= SEQUENCE (SIZE(1..maxnoofAdditionalPDCPDuplicationTNL)) OF Additional-PDCP-Duplication-TNL-Item

Additional-PDCP-Duplication-TNL-Item ::= SEQUENCE {
	additional-PDCP-Duplication-UP-TNL-Information	UPTransportLayerInformation,
	iE-Extensions		ProtocolExtensionContainer { { Additional-PDCP-Duplication-TNL-ExtIEs} } 	OPTIONAL,
	...
}

Additional-PDCP-Duplication-TNL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
	...
}

Additional-UL-NG-U-TNLatUPF-Item ::= SEQUENCE {

additional-UL-NG-U-TNLatUPF				UPTransportLayerInformation,

iE-Extensions		ProtocolExtensionContainer { { Additional-UL-NG-U-TNLatUPF-Item-ExtIEs} }	OPTIONAL,

...

}

Additional-UL-NG-U-TNLatUPF-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-PDUSessionCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional},

...

}

Additional-UL-NG-U-TNLatUPF-List ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF Additional-UL-NG-U-TNLatUPF-Item

Additional-Measurement-Timing-Configuration-List ::= SEQUENCE (SIZE(1.. maxnoofMTCItems)) OF Additional-Measurement-Timing-Configuration-Item

Additional-Measurement-Timing-Configuration-Item ::= SEQUENCE {

additionalMeasurementTimingConfigurationIndex 		INTEGER (0..16),

csi-RS-MTC-Configuration-List						CSI-RS-MTC-Configuration-List,

iE-Extensions						ProtocolExtensionContainer { { Additional-Measurement-Timing-Configuration-Item-ExtIEs} }	OPTIONAL,

...

}

Additional-Measurement-Timing-Configuration-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ActivationIDforCellActivation	::= INTEGER (0..255)

Active-MBS-SessionInformation ::= SEQUENCE {

mBS-QoSFlowsToAdd-List								MBS-QoSFlowsToAdd-List,

mBS-ServiceArea										MBS-ServiceArea												OPTIONAL,

mBS-MappingandDataForwardingRequestInfofromSource	MBS-MappingandDataForwardingRequestInfofromSource		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Active-MBS-SessionInformation-ExtIEs} }		OPTIONAL,

...

}

Active-MBS-SessionInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AllocationandRetentionPriority ::= SEQUENCE {

priorityLevel					INTEGER (0..15,...),

pre-emption-capability			ENUMERATED {shall-not-trigger-preemption, may-trigger-preemption, ...},

pre-emption-vulnerability		ENUMERATED {not-preemptable, preemptable, ...},

iE-Extensions					ProtocolExtensionContainer { {AllocationandRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationandRetentionPriority-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ActivationSFN ::= INTEGER (0..1023)

AllowedCAG-ID-List-perPLMN ::= SEQUENCE (SIZE(1..maxnoofCAGsperPLMN)) OF CAG-Identifier

AllowedPNI-NPN-ID-List ::= SEQUENCE (SIZE(1..maxnoofEPLMNsplus1)) OF AllowedPNI-NPN-ID-Item

AllowedPNI-NPN-ID-Item ::= SEQUENCE {

plmn-id								PLMN-Identity,

pni-npn-restricted-information		PNI-NPN-Restricted-Information,

allowed-CAG-id-list-per-plmn		AllowedCAG-ID-List-perPLMN,

iE-Extensions						ProtocolExtensionContainer { {AllowedPNI-NPN-ID-Item-ExtIEs} } OPTIONAL,

...

}

AllowedPNI-NPN-ID-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AllTrafficIndication ::= ENUMERATED {true,...}

AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQoSParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParaSetIndex			QoSParaSetIndex,

guaranteedFlowBitRateDL				BitRate					OPTIONAL,

guaranteedFlowBitRateUL				BitRate					OPTIONAL,

packetDelayBudget					PacketDelayBudget		OPTIONAL,

packetErrorRate						PacketErrorRate			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AMF-Region-Information ::= SEQUENCE (SIZE (1..maxnoofAMFRegions)) OF GlobalAMF-Region-Information

GlobalAMF-Region-Information ::= SEQUENCE {

plmn-ID				PLMN-Identity,

amf-region-id		BIT STRING (SIZE (8)),

iE-Extensions					ProtocolExtensionContainer { {GlobalAMF-Region-Information-ExtIEs} } OPTIONAL,

...

}

GlobalAMF-Region-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AMF-UE-NGAP-ID ::= INTEGER (0..1099511627775)

AreaOfInterestInformation ::= SEQUENCE (SIZE(1..maxnoofAoIs)) OF AreaOfInterest-Item

AreaOfInterest-Item ::= SEQUENCE {

listOfTAIsinAoI					ListOfTAIsinAoI												OPTIONAL,

listOfCellsinAoI				ListOfCells													OPTIONAL,

listOfRANNodesinAoI				ListOfRANNodesinAoI											OPTIONAL,

requestReferenceID	RequestReferenceID,

iE-Extensions					ProtocolExtensionContainer { {AreaOfInterest-Item-ExtIEs} } OPTIONAL,

...

}

AreaOfInterest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AreaScopeOfMDT-NR ::= CHOICE {

cellBased					CellBasedMDT-NR,

tABased						TABasedMDT,

tAIBased					TAIBasedMDT,

...

}

AreaScopeOfMDT-EUTRA ::= CHOICE {

cellBased					CellBasedMDT-EUTRA,

tABased						TABasedMDT,

tAIBased					TAIBasedMDT,

...

}

AreaScopeOfNeighCellsList ::= SEQUENCE (SIZE(1..maxnoofFreqforMDT)) OF AreaScopeOfNeighCellsItem

AreaScopeOfNeighCellsItem ::= SEQUENCE {

nrFrequencyInfo				NRFrequencyInfo,

pciListForMDT				PCIListForMDT		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AreaScopeOfNeighCellsItem-ExtIEs} }	OPTIONAL,

...

}

AreaScopeOfNeighCellsItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AreaScopeOfQMC ::= CHOICE {

cellBased					CellBasedQMC,

tABased						TABasedQMC,

tAIBased					TAIBasedQMC,

pLMNAreaBased				PLMNAreaBasedQMC,

choice-extension			ProtocolIE-Single-Container { {AreaScopeOfQMC-ExtIEs} }

}

AreaScopeOfQMC-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

AS-SecurityInformation ::= SEQUENCE {

key-NG-RAN-Star					BIT STRING (SIZE(256)),

ncc								INTEGER (0..7),

iE-Extensions					ProtocolExtensionContainer { {AS-SecurityInformation-ExtIEs} } OPTIONAL,

...

}

AS-SecurityInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AssistanceDataForRANPaging ::= SEQUENCE {

ran-paging-attempt-info			RANPagingAttemptInfo	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {AssistanceDataForRANPaging-ExtIEs} } OPTIONAL,

...

}

AssistanceDataForRANPaging-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-NPNPagingAssistanceInformation	CRITICALITY ignore	EXTENSION NPNPagingAssistanceInformation	PRESENCE optional },

...

}

Associated-QoSFlowInfo-List ::= SEQUENCE (SIZE(1..maxnoofMBSQoSFlows)) OF Associated-QoSFlowInfo-Item

Associated-QoSFlowInfo-Item ::= SEQUENCE {

mBS-QoSFlowIdentifier				QoSFlowIdentifier,

associatedUnicastQoSFlowIdentifier	QoSFlowIdentifier,

iE-Extensions						ProtocolExtensionContainer { { Associated-QoSFlowInfo-Item-ExtIEs} }	OPTIONAL,

...

}

Associated-QoSFlowInfo-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AvailableCapacity ::= INTEGER (1.. 100,...)

AvailableRRCConnectionCapacityValue ::= INTEGER (0..100)

AvailableRVQoEMetrics ::= SEQUENCE {

bufferLevel						ENUMERATED {true, ...} 	OPTIONAL,

playoutDelayForMediaStartup		ENUMERATED {true, ...} 		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {AvailableRVQoEMetrics-ExtIEs} } OPTIONAL,

...

}

AvailableRVQoEMetrics-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AveragingWindow ::= INTEGER (0..4095, ...)

#### -- B

BAPAddress ::= BIT STRING (SIZE(10))

BAPPathID ::= BIT STRING (SIZE(10))

BAPRoutingID ::= SEQUENCE {

bAPAddress		BAPAddress,

bAPPathID		BAPPathID,

iE-Extensions	ProtocolExtensionContainer { {BAPRoutingID-ExtIEs} }	OPTIONAL,

...

}

BAPRoutingID-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

BeamMeasurementIndicationM1 ::= ENUMERATED {true, ...}

BeamMeasurementsReportConfiguration ::= SEQUENCE {

beamMeasurementsReportQuantity			BeamMeasurementsReportQuantity			OPTIONAL,

maxNrofRS-IndexesToReport				MaxNrofRS-IndexesToReport		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { BeamMeasurementsReportConfiguration-ExtIEs} } OPTIONAL,

...

}

BeamMeasurementsReportConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BeamMeasurementsReportQuantity ::= SEQUENCE {

rSRP						ENUMERATED {true, ...},

rSRQ						ENUMERATED {true, ...},

sINR						ENUMERATED {true, ...},

iE-Extensions			ProtocolExtensionContainer { { BeamMeasurementsReportQuantity-ExtIEs} } OPTIONAL,

...

}

BeamMeasurementsReportQuantity-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BHInfoIndex ::= INTEGER (1.. maxnoofBHInfo)

BHInfoList ::= SEQUENCE (SIZE(1.. maxnoofBHInfo)) OF BHInfo-Item

BHInfo-Item ::= SEQUENCE {

bHInfoIndex			BHInfoIndex,

iE-Extensions			ProtocolExtensionContainer { { BHInfo-Item-ExtIEs} }	OPTIONAL,

...

}

BHInfo-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BHRLCChannelID ::= BIT STRING (SIZE(16))

BAPControlPDURLCCH-List ::= SEQUENCE (SIZE(1.. maxnoofBAPControlPDURLCCHs)) OF BAPControlPDURLCCH-Item

BAPControlPDURLCCH-Item ::= SEQUENCE {

bHRLCCHID			BHRLCChannelID,

nexthopBAPAddress	BAPAddress,

iE-Extensions			ProtocolExtensionContainer { { BAPControlPDURLCCH-Item-ExtIEs} }	OPTIONAL,

...

}

BAPControlPDURLCCH-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BluetoothMeasurementConfiguration ::= SEQUENCE {

bluetoothMeasConfig             BluetoothMeasConfig,

bluetoothMeasConfigNameList		BluetoothMeasConfigNameList     OPTIONAL,

bt-rssi                         ENUMERATED {true, ...}          OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { BluetoothMeasurementConfiguration-ExtIEs } } OPTIONAL,

...

}

BluetoothMeasurementConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BluetoothMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofBluetoothName)) OF BluetoothName

BluetoothMeasConfig::= ENUMERATED {setup,...}

BluetoothName ::= OCTET STRING (SIZE (1..248))

BPLMN-ID-Info-EUTRA ::= SEQUENCE (SIZE(1..maxnoofEUTRABPLMNs)) OF BPLMN-ID-Info-EUTRA-Item

BPLMN-ID-Info-EUTRA-Item ::= SEQUENCE {

broadcastPLMNs					BroadcastEUTRAPLMNs,

tac								TAC,

e-utraCI						E-UTRA-Cell-Identity,

ranac							RANAC OPTIONAL,

iE-Extension					ProtocolExtensionContainer { {BPLMN-ID-Info-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

BPLMN-ID-Info-EUTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BPLMN-ID-Info-NR ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF BPLMN-ID-Info-NR-Item

BPLMN-ID-Info-NR-Item ::= SEQUENCE {

broadcastPLMNs					BroadcastPLMNs,

tac								TAC,

nr-CI							NR-Cell-Identity,

ranac							RANAC OPTIONAL,

iE-Extension					ProtocolExtensionContainer { {BPLMN-ID-Info-NR-Item-ExtIEs} } OPTIONAL,

...

}

BPLMN-ID-Info-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-ConfiguredTACIndication		CRITICALITY ignore	EXTENSION ConfiguredTACIndication		PRESENCE optional }|

{ ID id-NPN-Broadcast-Information	CRITICALITY reject	EXTENSION NPN-Broadcast-Information		PRESENCE optional },

...

}

BitRate	::= INTEGER (0..4000000000000,...)

BroadcastCAG-Identifier-List ::= SEQUENCE (SIZE(1..maxnoofCAGs)) OF BroadcastCAG-Identifier-Item

BroadcastCAG-Identifier-Item ::= SEQUENCE {

cag-Identifier					CAG-Identifier,

iE-Extension					ProtocolExtensionContainer { {BroadcastCAG-Identifier-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastCAG-Identifier-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastNID-List ::= SEQUENCE (SIZE(1..maxnoofNIDs)) OF BroadcastNID-Item

BroadcastNID-Item ::= SEQUENCE {

nid							NID,

iE-Extension				ProtocolExtensionContainer { {BroadcastNID-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastNID-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastPLMNs ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF PLMN-Identity

BroadcastEUTRAPLMNs ::= SEQUENCE (SIZE(1..maxnoofEUTRABPLMNs)) OF PLMN-Identity

BroadcastPLMNinTAISupport-Item ::= SEQUENCE {

plmn-id							PLMN-Identity,

tAISliceSupport-List			SliceSupport-List,

iE-Extension					ProtocolExtensionContainer { {BroadcastPLMNinTAISupport-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastPLMNinTAISupport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-NPN-Support						CRITICALITY reject	EXTENSION NPN-Support						PRESENCE optional}|

{ ID id-ExtendedTAISliceSupportList		CRITICALITY reject	EXTENSION ExtendedSliceSupportList		PRESENCE optional}|

{ ID id-TAINSAGSupportList				CRITICALITY ignore	EXTENSION TAINSAGSupportList				PRESENCE optional	},

...

}

BroadcastPNI-NPN-ID-Information ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF BroadcastPNI-NPN-ID-Information-Item

BroadcastPNI-NPN-ID-Information-Item ::= SEQUENCE {

plmn-id							PLMN-Identity,

broadcastCAG-Identifier-List	BroadcastCAG-Identifier-List,

iE-Extension					ProtocolExtensionContainer { {BroadcastPNI-NPN-ID-Information-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastPNI-NPN-ID-Information-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastSNPNID-List ::= SEQUENCE (SIZE(1..maxnoofSNPNIDs)) OF BroadcastSNPNID

BroadcastSNPNID ::= SEQUENCE {

plmn-id							PLMN-Identity,

broadcastNID-List								BroadcastNID-List,

iE-Extension					ProtocolExtensionContainer { {BroadcastSNPNID-ExtIEs} } OPTIONAL,

...

}

BroadcastSNPNID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- C

CAG-Identifier	::= BIT STRING (SIZE (32))

CapacityValue ::= INTEGER (0..100)

CapacityValueInfo ::= SEQUENCE {

capacityValue				CapacityValue,

ssbAreaCapacityValueList 	SSBAreaCapacityValue-List 	OPTIONAL,

iE-Extension 				ProtocolExtensionContainer { {CapacityValueInfo-ExtIEs} } OPTIONAL,

...

}

CapacityValueInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Cause ::= CHOICE {

radioNetwork		CauseRadioNetworkLayer,

transport			CauseTransportLayer,

protocol			CauseProtocol,

misc				CauseMisc,

choice-extension	ProtocolIE-Single-Container { {Cause-ExtIEs} }

}

Cause-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

CauseRadioNetworkLayer ::= ENUMERATED {

cell-not-available,

handover-desirable-for-radio-reasons,

handover-target-not-allowed,

invalid-AMF-Set-ID,

no-radio-resources-available-in-target-cell,

partial-handover,

reduce-load-in-serving-cell,

resource-optimisation-handover,

time-critical-handover,

tXnRELOCoverall-expiry,

tXnRELOCprep-expiry,

unknown-GUAMI-ID,

unknown-local-NG-RAN-node-UE-XnAP-ID,

inconsistent-remote-NG-RAN-node-UE-XnAP-ID,

encryption-and-or-integrity-protection-algorithms-not-supported,

not-used-causes-value-1,

multiple-PDU-session-ID-instances,

unknown-PDU-session-ID,

unknown-QoS-Flow-ID,

multiple-QoS-Flow-ID-instances,

switch-off-ongoing,

not-supported-5QI-value,

tXnDCoverall-expiry,

tXnDCprep-expiry,

action-desirable-for-radio-reasons,

reduce-load,

resource-optimisation,

time-critical-action,

target-not-allowed,

no-radio-resources-available,

invalid-QoS-combination,

encryption-algorithms-not-supported,

procedure-cancelled,

rRM-purpose,

improve-user-bit-rate,

user-inactivity,

radio-connection-with-UE-lost,

failure-in-the-radio-interface-procedure,

bearer-option-not-supported,

up-integrity-protection-not-possible,

up-confidentiality-protection-not-possible,

resources-not-available-for-the-slice-s,

ue-max-IP-data-rate-reason,

cP-integrity-protection-failure,

uP-integrity-protection-failure,

slice-not-supported-by-NG-RAN,

mN-Mobility,

sN-Mobility,

count-reaches-max-value,

unknown-old-NG-RAN-node-UE-XnAP-ID,

pDCP-Overload,

drb-id-not-available,

unspecified,

...,

ue-context-id-not-known,

non-relocation-of-context,

cho-cpc-resources-tobechanged,

rSN-not-available-for-the-UP,

npn-access-denied,

report-characteristics-empty,

existing-measurement-ID,

measurement-temporarily-not-available,

measurement-not-supported-for-the-object,

ue-power-saving,

unknown-NG-RAN-node2-Measurement-ID,

insufficient-ue-capabilities,

normal-release,

value-out-of-allowed-range,

scg-activation-deactivation-failure,

scg-deactivation-failure-due-to-data-transmission

}

CauseTransportLayer ::= ENUMERATED {

transport-resource-unavailable,

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

hardware-failure,

o-and-M-intervention,

not-enough-user-plane-processing-resources,

unspecified,

...

}

CellAssistanceInfo-NR	::= CHOICE {

limitedNR-List				SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF NR-CGI,

full-List					ENUMERATED {all-served-cells-NR, ...},

choice-extension			ProtocolIE-Single-Container { {CellAssistanceInfo-NR-ExtIEs} }

}

CellAssistanceInfo-NR-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

CellAndCapacityAssistanceInfo-NR	::= SEQUENCE {

maximumCellListSize					MaximumCellListSize								OPTIONAL,

cellAssistanceInfo-NR		CellAssistanceInfo-NR 				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CellAndCapacityAssistanceInfo-NR-ExtIEs} }	OPTIONAL,

...

}

CellAndCapacityAssistanceInfo-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellAndCapacityAssistanceInfo-EUTRA	::= SEQUENCE {

maximumCellListSize					MaximumCellListSize								OPTIONAL,

cellAssistanceInfo-EUTRA			CellAssistanceInfo-EUTRA 				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CellAndCapacityAssistanceInfo-EUTRA-ExtIEs} }	OPTIONAL,

...

}

CellAndCapacityAssistanceInfo-EUTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellAssistanceInfo-EUTRA	::= CHOICE {

limitedEUTRA-List			SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI,

full-List					ENUMERATED {all-served-cells-E-UTRA, ...},

choice-extension			ProtocolIE-Single-Container { {CellAssistanceInfo-EUTRA-ExtIEs} }

}

CellAssistanceInfo-EUTRA-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

CellBasedMDT-NR::= SEQUENCE {

cellIdListforMDT-NR	CellIdListforMDT-NR,

iE-Extensions		ProtocolExtensionContainer { {CellBasedMDT-NR-ExtIEs} } OPTIONAL,

...

}

CellBasedMDT-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellIdListforMDT-NR ::= SEQUENCE (SIZE(1..maxnoofCellIDforMDT)) OF NR-CGI

CellBasedQMC::= SEQUENCE {

cellIdListforQMC		CellIdListforQMC,

iE-Extensions		ProtocolExtensionContainer { {CellBasedQMC-ExtIEs} } OPTIONAL,

...

}

CellBasedQMC-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellIdListforQMC ::= SEQUENCE (SIZE(1..maxnoofCellIDforQMC)) OF GlobalNG-RANCell-ID

CellBasedMDT-EUTRA::= SEQUENCE {

cellIdListforMDT-EUTRA	CellIdListforMDT-EUTRA,

iE-Extensions		ProtocolExtensionContainer { {CellBasedMDT-EUTRA-ExtIEs} } OPTIONAL,

...

}

CellBasedMDT-EUTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellIdListforMDT-EUTRA ::= SEQUENCE (SIZE(1..maxnoofCellIDforMDT)) OF E-UTRA-CGI

CellCapacityClassValue ::= INTEGER (1..100,...)

CellDeploymentStatusIndicator ::= ENUMERATED {pre-change-notification, ...}

CellGroupID ::= INTEGER (0..maxnoofSCellGroups)

CellMeasurementResult ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellMeasurementResult-Item

CellMeasurementResult-Item	::= SEQUENCE {

cell-ID								GlobalNG-RANCell-ID,

radioResourceStatus					RadioResourceStatus				OPTIONAL,

tNLCapacityIndicator				TNLCapacityIndicator			OPTIONAL,

compositeAvailableCapacityGroup		CompositeAvailableCapacityGroup	OPTIONAL,

sliceAvailableCapacity				SliceAvailableCapacity			OPTIONAL,

numberofActiveUEs					NumberofActiveUEs				OPTIONAL,

rRCConnections						RRCConnections					OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CellMeasurementResult-Item-ExtIEs} }	OPTIONAL,

...

}

CellMeasurementResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-NR-U-Channel-List	CRITICALITY ignore	EXTENSION NR-U-Channel-List PRESENCE optional },

...

}

CellReplacingInfo ::= SEQUENCE {

replacingCells					ReplacingCells,

iE-Extensions					ProtocolExtensionContainer { {CellReplacingInfo-ExtIEs}}	OPTIONAL,

...

}

CellReplacingInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellToReport ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellToReport-Item

CellToReport-Item	::= SEQUENCE {

cell-ID									GlobalNG-RANCell-ID,

sSBToReport-List                        SSBToReport-List			OPTIONAL,

sliceToReport-List                      SliceToReport-List			OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CellToReport-Item-ExtIEs} }	OPTIONAL,

...

}

CellToReport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Cell-Type-Choice ::= CHOICE {

ng-ran-e-utra			E-UTRA-Cell-Identity,

ng-ran-nr				NR-Cell-Identity,

e-utran					E-UTRA-Cell-Identity,

choice-extension		ProtocolIE-Single-Container { { Cell-Type-Choice-ExtIEs} }

}

Cell-Type-Choice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

CHOConfiguration ::= SEQUENCE {

choCandidateCell-List				CHOCandidateCell-List,

iE-Extensions						ProtocolExtensionContainer { { CHOConfiguration-ExtIEs} }	OPTIONAL,

...

}

CHOConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CHOCandidateCell-List ::= SEQUENCE (SIZE(1..maxnoofCellsinCHO)) OF CHOCandidateCell-Item

CHOCandidateCell-Item ::= SEQUENCE {

choCandidateCellID					GlobalNG-RANCell-ID,

choExecutionCondition-List			CHOExecutionCondition-List,

iE-Extensions						ProtocolExtensionContainer { { CHOCandidateCell-Item-ExtIEs} }	OPTIONAL,

...

}

CHOCandidateCell-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CHOExecutionCondition-List ::= SEQUENCE (SIZE(1..maxnoofCHOexecutioncond)) OF CHOExecutionCondition-Item

CHOExecutionCondition-Item ::= SEQUENCE {	measObjectContainer					MeasObjectContainer,

reportConfigContainer				ReportConfigContainer,

iE-Extensions						ProtocolExtensionContainer { { CHOExecutionCondition-Item-ExtIEs} }	OPTIONAL,

...

}

CHOExecutionCondition-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CompositeAvailableCapacityGroup ::= SEQUENCE {

compositeAvailableCapacityDownlink		CompositeAvailableCapacity,

compositeAvailableCapacityUplink		CompositeAvailableCapacity,

iE-Extensions				ProtocolExtensionContainer { { CompositeAvailableCapacityGroup-ExtIEs} }	OPTIONAL,

...

}

CompositeAvailableCapacityGroup-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-CompositeAvailableCapacitySupplementaryUplink	CRITICALITY ignore	EXTENSION CompositeAvailableCapacity	PRESENCE optional	},

...

}

CompositeAvailableCapacity ::= SEQUENCE {

cellCapacityClassValue		CellCapacityClassValue				OPTIONAL,

capacityValueInfo			CapacityValueInfo, -- this IE represents the IE "CapacityValue" in 9.2.2.a, it’s used to distinguish the "CapacityValue"  in 9.2.2.c

iE-Extensions				ProtocolExtensionContainer { { CompositeAvailableCapacity-ExtIEs} }OPTIONAL,

...

}

CompositeAvailableCapacity-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ControlPlaneTrafficType ::= INTEGER (1..3, ...)

CHO-MRDC-EarlyDataForwarding ::= ENUMERATED {stop, ...}

CHO-MRDC-Indicator ::= ENUMERATED {true, ...}

CHOtrigger ::= ENUMERATED {

cho-initiation,

cho-replace,

...

}

CHOinformation-Req ::= SEQUENCE {

cho-trigger						CHOtrigger,

targetNG-RANnodeUEXnAPID		NG-RANnodeUEXnAPID										OPTIONAL

-- This IE shall be present if the cho-trigger IE is present and set to "CHO-replace" --,

cHO-EstimatedArrivalProbability	CHO-Probability											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CHOinformation-Req-ExtIEs} }	OPTIONAL,

...

}

CHOinformation-Req-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

CHOinformation-Ack ::= SEQUENCE {

requestedTargetCellGlobalID		Target-CGI,

maxCHOoperations				MaxCHOpreparations											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CHOinformation-Ack-ExtIEs} }	OPTIONAL,

...

}

CHOinformation-Ack-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

CHOinformation-AddReq ::= SEQUENCE {

source-M-NGRAN-node-ID				GlobalNG-RANNode-ID,

source-M-NGRAN-node-UE-XnAP-ID		NG-RANnodeUEXnAPID,

cHO-EstimatedArrivalProbability		CHO-Probability													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CHOinformation-AddReq-ExtIEs} }	OPTIONAL,

...

}

CHOinformation-AddReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

CHOinformation-ModReq ::= SEQUENCE {

conditionalReconfig					ENUMERATED {intra-mn-cho, ...},

cHO-EstimatedArrivalProbability		CHO-Probability													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CHOinformation-ModReq-ExtIEs} }	OPTIONAL,

...

}

CHOinformation-ModReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

CHO-Probability ::= INTEGER (1..100)

CNsubgroupID ::= INTEGER (0..7, ...)

ConfiguredTACIndication ::= ENUMERATED {

true,

...

}

Connectivity-Support		::= SEQUENCE {

eNDC-Support			ENUMERATED {supported, not-supported, ...},

iE-Extensions			ProtocolExtensionContainer { {Connectivity-Support-ExtIEs} }	OPTIONAL,

...

}

Connectivity-Support-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ContainerAppLayerMeasConfig ::= OCTET STRING (SIZE (1..8000))

COUNT-PDCP-SN12 ::= SEQUENCE {

pdcp-SN12						INTEGER (0..4095),

hfn-PDCP-SN12					INTEGER (0..1048575),

iE-Extensions					ProtocolExtensionContainer { {COUNT-PDCP-SN12-ExtIEs} }	OPTIONAL,

...

}

COUNT-PDCP-SN12-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

COUNT-PDCP-SN18 ::= SEQUENCE {

pdcp-SN18						INTEGER (0..262143),

hfn-PDCP-SN18					INTEGER (0..16383),

iE-Extensions					ProtocolExtensionContainer { {COUNT-PDCP-SN18-ExtIEs} }	OPTIONAL,

...

}

COUNT-PDCP-SN18-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Coverage-Modification-List ::= SEQUENCE (SIZE (0..maxnoofCellsinNG-RANnode)) OF Coverage-Modification-List-Item

Coverage-Modification-List-Item ::= SEQUENCE {

globalNG-RANCell-ID				GlobalCell-ID,

cellCoverageState				INTEGER (0..63, ...),

cellDeploymentStatusIndicator	CellDeploymentStatusIndicator			OPTIONAL,

cellReplacingInfo				CellReplacingInfo						OPTIONAL,

-- Included in case the Cell Deployment Status Indicator IE is present

sSB-Coverage-Modification-List  SSB-Coverage-Modification-List,

iE-Extension			ProtocolExtensionContainer { { Coverage-Modification-Item-ExtIEs} } OPTIONAL,

...

}

Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPTransportLayerInformation ::= CHOICE {

endpointIPAddress			TransportLayerAddress,

choice-extension			ProtocolIE-Single-Container { {CPTransportLayerInformation-ExtIEs} }

}

CPTransportLayerInformation-ExtIEs XNAP-PROTOCOL-IES ::= {

{ ID id-EndpointIPAddressAndPort		CRITICALITY reject	TYPE EndpointIPAddressAndPort		PRESENCE mandatory},

...

}

CPACcandidatePSCells-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CPACcandidatePSCells-item

CPACcandidatePSCells-item ::= SEQUENCE {

pscell-id						NR-CGI,

iE-Extensions					ProtocolExtensionContainer { {CPACcandidatePSCells-item-ExtIEs}}	OPTIONAL,

...

}

CPACcandidatePSCells-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPCindicator ::= ENUMERATED {cpc-initiation, cpc-modification, cpc-cancellation, ...}

CPAInformationRequest ::= SEQUENCE {

max-no-of-pscells					INTEGER (1..maxnoofPSCellCandidates, ...),

cpac-EstimatedArrivalProbability	CHO-Probability 	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CPAInformationRequest-ExtIEs} } OPTIONAL,

...

}

CPAInformationRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPAInformationAck ::= SEQUENCE {

candidate-pscells					CPACcandidatePSCells-list,

iE-Extensions		ProtocolExtensionContainer { { CPAInformationAck-ExtIEs} } OPTIONAL,

...

}

CPAInformationAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPCInformationRequired::= SEQUENCE {

cpc-target-sn-required-list		CPC-target-SN-required-list,

iE-Extensions					ProtocolExtensionContainer { {CPCInformationRequired-ExtIEs} } OPTIONAL,

...

}

CPCInformationRequired-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPC-target-SN-required-list ::= SEQUENCE (SIZE(1..maxnoofTargetSNs)) OF CPC-target-SN-required-list-Item

CPC-target-SN-required-list-Item	::= SEQUENCE {

target-S-NG-RANnodeID				GlobalNG-RANNode-ID,

cpc-indicator						CPCindicator,

max-no-of-pscells					INTEGER (1..maxnoofPSCellCandidates, ...),

cpac-EstimatedArrivalProbability	CHO-Probability 																	OPTIONAL,

sN-to-MN-Container					OCTET STRING,

iE-Extensions						ProtocolExtensionContainer { { CPC-target-SN-required-list-Item-ExtIEs} }	OPTIONAL,

...

}

CPC-target-SN-required-list-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPCInformationConfirm ::= SEQUENCE {

cpc-target-sn-confirm-list CPC-target-SN-confirm-list,

iE-Extensions		ProtocolExtensionContainer { { CPCInformationConfirm-ExtIEs} } OPTIONAL,

...

}

CPCInformationConfirm-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPC-target-SN-confirm-list ::= SEQUENCE (SIZE(1..maxnoofTargetSNs)) OF CPC-target-SN-confirm-list-Item

CPC-target-SN-confirm-list-Item ::= SEQUENCE {

target-S-NG-RANnodeID			GlobalNG-RANNode-ID,

candidate-pscells				CPACcandidatePSCells-list,

iE-Extensions					ProtocolExtensionContainer { { CPC-target-SN-confirm-list-Item-ExtIEs} } OPTIONAL,

...

}

CPC-target-SN-confirm-list-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPAInformationModReq ::=  SEQUENCE {

max-no-of-pscells					INTEGER (1..8, ...)	OPTIONAL,

cpac-EstimatedArrivalProbability	CHO-Probability 	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CPAInformationModReq-ExtIEs} } OPTIONAL,

...

}

CPAInformationModReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPAInformationModReqAck ::=  SEQUENCE {

candidate-pscells					CPACcandidatePSCells-list,

iE-Extensions						ProtocolExtensionContainer { { CPAInformationModReqAck-ExtIEs} } OPTIONAL,

...

}

CPAInformationModReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPC-DataForwarding-Indicator ::= ENUMERATED {triggered, early-data-transmission-stop, ...}

CPACInformationModRequired ::= SEQUENCE {

candidate-pscells	CPACcandidatePSCells-list,

iE-Extensions		ProtocolExtensionContainer { { CPACInformationModRequired-ExtIEs} } OPTIONAL,

...

}

CPACInformationModRequired-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPCInformationUpdate::= SEQUENCE {

cpc-target-sn-list					CPC-target-SN-mod-list,

iE-Extensions		ProtocolExtensionContainer { { CPCInformationUpdate-ExtIEs} } OPTIONAL,

...

}

CPCInformationUpdate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPC-target-SN-mod-list ::= SEQUENCE (SIZE(1..maxnoofTargetSNs)) OF CPC-target-SN-mod-item

CPC-target-SN-mod-item ::= SEQUENCE {

target-S-NG-RANnodeID				GlobalNG-RANNode-ID,

candidate-pscells					CPCInformationUpdatePSCells-list,

iE-Extensions		ProtocolExtensionContainer { {CPC-target-SN-mod-item-ExtIEs} } OPTIONAL,

...

}

CPC-target-SN-mod-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPCInformationUpdatePSCells-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CPCInformationUpdatePSCells-item

CPCInformationUpdatePSCells-item ::= SEQUENCE {

pscell-id						NR-CGI,

iE-Extensions					ProtocolExtensionContainer { {CPCInformationUpdatePSCells-item-ExtIEs}}	OPTIONAL,

...

}

CPCInformationUpdatePSCells-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode					OPTIONAL,

triggeringMessage				TriggeringMessage				OPTIONAL,

procedureCriticality			Criticality						OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CriticalityDiagnostics-ExtIEs} }	OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1..maxNrOfErrors)) OF

SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError				TypeOfError,

iE-Extensions			ProtocolExtensionContainer { {CriticalityDiagnostics-IE-List-ExtIEs} } OPTIONAL,

...

}

CriticalityDiagnostics-IE-List-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

C-RNTI ::= BIT STRING (SIZE(16))

CSI-RS-MTC-Configuration-List ::= SEQUENCE (SIZE(1.. maxnoofCSIRSconfigurations)) OF CSI-RS-MTC-Configuration-Item

CSI-RS-MTC-Configuration-Item ::= SEQUENCE {

csi-RS-Index			INTEGER(0..95),

csi-RS-Status			ENUMERATED {activated, deactivated, ...},

csi-RS-Neighbour-List	CSI-RS-Neighbour-List OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { CSI-RS-MTC-Configuration-Item-ExtIEs} }	OPTIONAL,

...

}

CSI-RS-MTC-Configuration-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CSI-RS-Neighbour-List ::= SEQUENCE (SIZE(1.. maxnoofCSIRSneighbourCells)) OF CSI-RS-Neighbour-Item

CSI-RS-Neighbour-Item ::= SEQUENCE {

nr-cgi							NR-CGI,

csi-RS-MTC-Neighbour-List 	CSI-RS-MTC-Neighbour-List OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { CSI-RS-Neighbour-Item-ExtIEs} }	OPTIONAL,

...

}

CSI-RS-Neighbour-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CSI-RS-MTC-Neighbour-List ::= SEQUENCE (SIZE(1.. maxnoofCSIRSneighbourCellsInMTC)) OF CSI-RS-MTC-Neighbour-Item

CSI-RS-MTC-Neighbour-Item ::= SEQUENCE {

csi-RS-Index		INTEGER(0..95),

iE-Extensions		ProtocolExtensionContainer { { CSI-RS-MTC-Neighbour-Item-ExtIEs} }	OPTIONAL,

...

}

CSI-RS-MTC-Neighbour-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CyclicPrefix-E-UTRA-DL ::= ENUMERATED {

normal,

extended,

...

}

CyclicPrefix-E-UTRA-UL ::= ENUMERATED {

normal,

extended,

...

}

CSI-RSTransmissionIndication ::= ENUMERATED {

activated,

deactivated,

...

}

#### -- D

XnUAddressInfoperPDUSession-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF XnUAddressInfoperPDUSession-Item

XnUAddressInfoperPDUSession-Item ::= SEQUENCE {

pduSession-ID			PDUSession-ID,

dataForwardingInfoFromTargetNGRANnode		DataForwardingInfoFromTargetNGRANnode										OPTIONAL,

pduSessionResourceSetupCompleteInfo-SNterm			PDUSessionResourceBearerSetupCompleteInfo-SNterminated		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { XnUAddressInfoperPDUSession-Item-ExtIEs} }					OPTIONAL,

...

}

XnUAddressInfoperPDUSession-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecondarydataForwardingInfoFromTarget-List	CRITICALITY ignore	EXTENSION SecondarydataForwardingInfoFromTarget-List	PRESENCE optional}|

{ ID id-DRB-IDs-takenintouse						CRITICALITY reject	EXTENSION DRB-List												PRESENCE optional}|

{ ID id-dataForwardingInfoFromTargetE-UTRANnode		CRITICALITY ignore	EXTENSION DataForwardingInfoFromTargetE-UTRANnode		PRESENCE optional},

...

}

DataForwardingInfoFromTargetE-UTRANnode ::= SEQUENCE {

dataForwardingInfoFromTargetE-UTRANnode-List			DataForwardingInfoFromTargetE-UTRANnode-List,

iE-Extension		ProtocolExtensionContainer { { DataForwardingInfoFromTargetE-UTRANnode-ExtIEs} }	OPTIONAL,

...

}

DataForwardingInfoFromTargetE-UTRANnode-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DataForwardingInfoFromTargetE-UTRANnode-List ::= SEQUENCE (SIZE(1.. maxnoofDataForwardingTunneltoE-UTRAN)) OF DataForwardingInfoFromTargetE-UTRANnode-Item

DataForwardingInfoFromTargetE-UTRANnode-Item ::= SEQUENCE {

dlForwardingUPTNLInformation	UPTransportLayerInformation,

qosFlowsToBeForwarded-List	QoSFlowsToBeForwarded-List,

iE-Extension		ProtocolExtensionContainer { { DataForwardingInfoFromTargetE-UTRANnode-Item-ExtIEs} }	OPTIONAL,

...

}

DataForwardingInfoFromTargetE-UTRANnode-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsToBeForwarded-List ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsToBeForwarded-Item

QoSFlowsToBeForwarded-Item ::= SEQUENCE {

qosFlowIdentifier			QoSFlowIdentifier,

iE-Extension		ProtocolExtensionContainer { { QoSFlowsToBeForwarded-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlowsToBeForwarded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DataForwardingInfoFromTargetNGRANnode ::= SEQUENCE {

qosFlowsAcceptedForDataForwarding-List			QoSFLowsAcceptedToBeForwarded-List,

pduSessionLevelDLDataForwardingInfo				UPTransportLayerInformation								OPTIONAL,

pduSessionLevelULDataForwardingInfo				UPTransportLayerInformation								OPTIONAL,

dataForwardingResponseDRBItemList				DataForwardingResponseDRBItemList						OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DataForwardingInfoFromTargetNGRANnode-ExtIEs} }	OPTIONAL,

...

}

DataForwardingInfoFromTargetNGRANnode-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFLowsAcceptedToBeForwarded-List ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoSFLowsAcceptedToBeForwarded-Item

QoSFLowsAcceptedToBeForwarded-Item ::= SEQUENCE {

qosFlowIdentifier			QoSFlowIdentifier,

iE-Extension				ProtocolExtensionContainer { {QoSFLowsAcceptedToBeForwarded-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFLowsAcceptedToBeForwarded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DataforwardingandOffloadingInfofromSource ::= SEQUENCE {

qosFlowsToBeForwarded			QoSFLowsToBeForwarded-List,

sourceDRBtoQoSFlowMapping		DRBToQoSFlowMapping-List													OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DataforwardingandOffloadingInfofromSource-ExtIEs} }	OPTIONAL,

...

}

DataforwardingandOffloadingInfofromSource-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFLowsToBeForwarded-List ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoSFLowsToBeForwarded-Item

QoSFLowsToBeForwarded-Item ::= SEQUENCE {

qosFlowIdentifier			QoSFlowIdentifier,

dl-dataforwarding			DLForwarding,

ul-dataforwarding			ULForwarding,

iE-Extension		ProtocolExtensionContainer { {QoSFLowsToBeForwarded-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFLowsToBeForwarded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-ULForwardingProposal					CRITICALITY ignore	EXTENSION ULForwardingProposal		PRESENCE optional }|

{ ID id-SourceDLForwardingIPAddress				CRITICALITY ignore	EXTENSION TransportLayerAddress		PRESENCE optional}|

{ ID id-SourceNodeDLForwardingIPAddress			CRITICALITY ignore	EXTENSION TransportLayerAddress		PRESENCE optional},

...

}

DataForwardingResponseDRBItemList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DataForwardingResponseDRBItem

DataForwardingResponseDRBItem ::= SEQUENCE {

drb-ID				DRB-ID,

dlForwardingUPTNL	UPTransportLayerInformation												OPTIONAL,

ulForwardingUPTNL	UPTransportLayerInformation												OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DataForwardingResponseDRBItem-ExtIEs} }	OPTIONAL,

...

}

DataForwardingResponseDRBItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DataTrafficResources ::= BIT STRING (SIZE(6..17600))

DataTrafficResourceIndication ::= SEQUENCE {

activationSFN					ActivationSFN,

sharedResourceType				SharedResourceType,

reservedSubframePattern			ReservedSubframePattern										OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DataTrafficResourceIndication-ExtIEs} }	OPTIONAL,

...

}

DataTrafficResourceIndication-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DAPSRequestInfo ::= SEQUENCE {

dapsIndicator				ENUMERATED {daps-HO-required, ...},

iE-Extensions				ProtocolExtensionContainer { {DAPSRequestInfo-ExtIEs} } OPTIONAL,

...

}

DAPSRequestInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DAPSResponseInfo-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DAPSResponseInfo-Item

DAPSResponseInfo-Item ::= SEQUENCE {

drbID						DRB-ID,

dapsResponseIndicator		ENUMERATED {daps-HO-accepted, daps-HO-not-accepted, ...},

iE-Extensions				ProtocolExtensionContainer { {DAPSResponseInfo-Item-ExtIEs} } OPTIONAL,

...

}

DAPSResponseInfo-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DeliveryStatus	::= INTEGER (0..4095, ...)

DesiredActNotificationLevel	::= ENUMERATED {none, qos-flow, pdu-session, ue-level, ...}

DefaultDRB-Allowed ::= ENUMERATED {true, false, ...}

DirectForwardingPathAvailability ::= ENUMERATED {direct-path-available, ...}

DLCountChoice ::= CHOICE {

count12bits				COUNT-PDCP-SN12,

count18bits				COUNT-PDCP-SN18,

choice-extension		ProtocolIE-Single-Container { {DLCountChoice-ExtIEs} }

}

DLCountChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

DLForwarding	::= ENUMERATED {dl-forwarding-proposed, ...}

DL-GBR-PRB-usage::= INTEGER (0..100)

DL-GBR-PRB-usage-for-MIMO::= INTEGER (0..100)

DL-non-GBR-PRB-usage::= INTEGER (0..100)

DL-non-GBR-PRB-usage-for-MIMO::= INTEGER (0..100)

DLF1Terminating-BHInfo ::= SEQUENCE {

egressBAPRoutingID		BAPRoutingID,

egressBHRLCCHID			BHRLCChannelID,

iE-Extensions			ProtocolExtensionContainer { { DLF1Terminating-BHInfo-ExtIEs} } OPTIONAL,

...

}

DLF1Terminating-BHInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DLNonF1Terminating-BHInfo ::= SEQUENCE {

ingressBAPRoutingID			BAPRoutingID,

ingressBHRLCCHID			BHRLCChannelID,

priorhopBAPAddress			BAPAddress,

iabqosMappingInformation	IAB-QoS-Mapping-Information,

iE-Extensions		ProtocolExtensionContainer { { DLNonF1Terminating-BHInfo-ExtIEs} } OPTIONAL,

...

}

DLNonF1Terminating-BHInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DL-Total-PRB-usage::= INTEGER (0..100)

DL-Total-PRB-usage-for-MIMO::= INTEGER (0..100)

DRB-ID	::= INTEGER (1..32, ...)

DRB-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRB-ID

DRB-List-withCause ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRB-List-withCause-Item

DRB-List-withCause-Item ::= SEQUENCE {

drb-id		DRB-ID,

cause		Cause,

rLC-Mode	RLCMode								OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DRB-List-withCause-Item-ExtIEs} }	OPTIONAL,

...

}

DRB-List-withCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Number ::= INTEGER (1..32, ...)

DRBsSubjectToDLDiscarding-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRBsSubjectToDLDiscarding-Item

DRBsSubjectToDLDiscarding-Item ::= SEQUENCE {

drbID				DRB-ID,

dlCount				DLCountChoice,

iE-Extension		ProtocolExtensionContainer { { DRBsSubjectToDLDiscarding-Item-ExtIEs} }	OPTIONAL,

...

}

DRBsSubjectToDLDiscarding-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsSubjectToEarlyStatusTransfer-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRBsSubjectToEarlyStatusTransfer-Item

DRBsSubjectToEarlyStatusTransfer-Item ::= SEQUENCE {

drbID				DRB-ID,

dlCount				DLCountChoice,

iE-Extension		ProtocolExtensionContainer { { DRBsSubjectToEarlyStatusTransfer-Item-ExtIEs} }	OPTIONAL,

...

}

DRBsSubjectToEarlyStatusTransfer-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsSubjectToStatusTransfer-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRBsSubjectToStatusTransfer-Item

DRBsSubjectToStatusTransfer-Item ::= SEQUENCE {

drbID				DRB-ID,

pdcpStatusTransfer-UL	DRBBStatusTransferChoice,

pdcpStatusTransfer-DL	DRBBStatusTransferChoice,

iE-Extension		ProtocolExtensionContainer { {DRBsSubjectToStatusTransfer-Item-ExtIEs} }	OPTIONAL,

...

}

DRBsSubjectToStatusTransfer-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-OldQoSFlowMap-ULendmarkerexpected	CRITICALITY reject		EXTENSION QoSFlows-List					PRESENCE optional },

...

}

DRBBStatusTransferChoice ::= CHOICE {

pdcp-sn-12bits		DRBBStatusTransfer12bitsSN,

pdcp-sn-18bits		DRBBStatusTransfer18bitsSN,

choice-extension		ProtocolIE-Single-Container { {DRBBStatusTransferChoice-ExtIEs} }

}

DRBBStatusTransferChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

DRBBStatusTransfer12bitsSN ::= SEQUENCE {

receiveStatusofPDCPSDU	BIT STRING (SIZE(1..2048))											OPTIONAL,

cOUNTValue				COUNT-PDCP-SN12,

iE-Extension			ProtocolExtensionContainer { {DRBBStatusTransfer12bitsSN-ExtIEs} }	OPTIONAL,

...

}

DRBBStatusTransfer12bitsSN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBBStatusTransfer18bitsSN ::= SEQUENCE {

receiveStatusofPDCPSDU	BIT STRING (SIZE(1..131072))										OPTIONAL,

cOUNTValue				COUNT-PDCP-SN18,

iE-Extension			ProtocolExtensionContainer { {DRBBStatusTransfer18bitsSN-ExtIEs} }	OPTIONAL,

...

}

DRBBStatusTransfer18bitsSN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBToQoSFlowMapping-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRBToQoSFlowMapping-Item

DRBToQoSFlowMapping-Item ::= SEQUENCE {

drb-ID							DRB-ID,

qosFlows-List					QoSFlows-List,

rLC-Mode						RLCMode								OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DRBToQoSFlowMapping-Item-ExtIEs} }	OPTIONAL,

...

}

DRBToQoSFlowMapping-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DAPSRequestInfo		CRITICALITY ignore		EXTENSION DAPSRequestInfo		PRESENCE optional  },

...

}

DUF-Slot-Config-List	::= SEQUENCE (SIZE(1..maxnoofDUFSlots)) OF DUF-Slot-Config-Item

DUF-Slot-Config-Item 	::=	CHOICE {

explicitFormat				ExplicitFormat,

implicitFormat				ImplicitFormat,

choice-extension				ProtocolIE-Single-Container { { DUF-Slot-Config-Item-ExtIEs} }

}

DUF-Slot-Config-Item-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

DUFSlotformatIndex ::= INTEGER(0..254)

DUFTransmissionPeriodicity ::= ENUMERATED { ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ...}

DU-RX-MT-RX ::= ENUMERATED {supported, not-supported, supported-FDM-required, ...}

DU-TX-MT-TX ::= ENUMERATED {supported, not-supported, supported-FDM-required, ...}

DU-RX-MT-TX ::= ENUMERATED {supported, not-supported, supported-FDM-required, ...}

DU-TX-MT-RX ::= ENUMERATED {supported, not-supported, supported-FDM-required, ...}

DuplicationActivation ::= ENUMERATED {active, inactive, ...}

Dynamic5QIDescriptor ::= SEQUENCE {

priorityLevelQoS			PriorityLevelQoS,

packetDelayBudget			PacketDelayBudget,

packetErrorRate				PacketErrorRate,

fiveQI						FiveQI													OPTIONAL,

delayCritical				ENUMERATED {delay-critical, non-delay-critical, ...}	OPTIONAL,

-- This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

averagingWindow				AveragingWindow											OPTIONAL,

-- This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

maximumDataBurstVolume		MaximumDataBurstVolume									OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {Dynamic5QIDescriptor-ExtIEs } }	OPTIONAL,

...

}

Dynamic5QIDescriptor-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedPacketDelayBudget		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional}|

{ ID id-CNPacketDelayBudgetDownlink		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional}|

{ ID id-CNPacketDelayBudgetUplink		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional},

...

}

#### -- E

EarlyMeasurement ::= ENUMERATED {true, ...}

E-RAB-ID		::= INTEGER (0..15, ...)

E-UTRAARFCN ::= INTEGER (0..maxEARFCN)

E-UTRA-Cell-Identity			::= BIT STRING (SIZE(28))

E-UTRA-CGI ::= SEQUENCE {

plmn-id				PLMN-Identity,

e-utra-CI			E-UTRA-Cell-Identity,

iE-Extension		ProtocolExtensionContainer { {E-UTRA-CGI-ExtIEs} } 	OPTIONAL,

...

}

E-UTRA-CGI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

E-UTRAFrequencyBandIndicator ::= INTEGER (1..256, ...)

E-UTRAMultibandInfoList ::= SEQUENCE (SIZE(1..maxnoofEUTRABands)) OF E-UTRAFrequencyBandIndicator

EUTRAPagingeDRXInformation ::= SEQUENCE {

eutrapaging-eDRX-Cycle		EUTRAPaging-eDRX-Cycle,

eutrapaging-Time-Window		EUTRAPaging-Time-Window											OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {EUTRAPagingeDRXInformation-ExtIEs} }	OPTIONAL,

...

}

EUTRAPagingeDRXInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

EUTRAPaging-eDRX-Cycle ::= ENUMERATED {

hfhalf, hf1, hf2, hf4, hf6,

hf8, hf10, hf12, hf14, hf16,

hf32, hf64, hf128, hf256,

...

}

EUTRAPaging-Time-Window ::= ENUMERATED {

s1, s2, s3, s4, s5,

s6, s7, s8, s9, s10,

s11, s12, s13, s14, s15, s16,

...

}

E-UTRAPCI ::= INTEGER (0..503, ...)

E-UTRAPRACHConfiguration ::= SEQUENCE {

rootSequenceIndex						INTEGER (0..837),

zeroCorrelationIndex					INTEGER (0..15),

highSpeedFlag							ENUMERATED {true, false, ...},

prach-FreqOffset						INTEGER (0..94),

prach-ConfigIndex						INTEGER (0..63)								OPTIONAL,

-- C-ifTDD: This IE shall be present if the EUTRA-Mode-Info IE in the Served Cell Information IE is set to the value "TDD" --

iE-Extensions							ProtocolExtensionContainer { {E-UTRAPRACHConfiguration-ExtIEs} }	OPTIONAL,

...

}

E-UTRAPRACHConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

E-UTRATransmissionBandwidth ::= ENUMERATED {bw6, bw15, bw25, bw50, bw75, bw100, ..., bw1}

EndpointIPAddressAndPort ::=SEQUENCE {

endpointIPAddress 				TransportLayerAddress,

portNumber						PortNumber,

iE-Extensions					ProtocolExtensionContainer { { EndpointIPAddressAndPort-ExtIEs} } OPTIONAL

}

EndpointIPAddressAndPort-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

EventTriggered ::= SEQUENCE {

loggedEventTriggeredConfig			LoggedEventTriggeredConfig,

iE-Extensions		ProtocolExtensionContainer { { EventTriggered-ExtIEs} } OPTIONAL,

...

}

EventTriggered-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

EventType ::= ENUMERATED {

report-upon-change-of-serving-cell,

report-UE-moving-presence-into-or-out-of-the-Area-of-Interest,

...,

report-upon-change-of-serving-cell-and-Area-of-Interest

}

EventTypeTrigger ::= CHOICE {

outOfCoverage					ENUMERATED {true, ...},

eventL1		EventL1,

choice-Extensions		ProtocolIE-Single-Container { {EventTypeTrigger-ExtIEs} }

}

EventTypeTrigger-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

EventL1 ::= SEQUENCE {

l1Threshold					MeasurementThresholdL1LoggedMDT,

hysteresis					Hysteresis,

timeToTrigger				TimeToTrigger,

iE-Extensions		ProtocolExtensionContainer { { EventL1-ExtIEs} } OPTIONAL,

...

}

EventL1-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MeasurementThresholdL1LoggedMDT ::= CHOICE {

threshold-RSRP				Threshold-RSRP,

threshold-RSRQ				Threshold-RSRQ,

...

}

ExpectedActivityPeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181, ...)

ExpectedHOInterval ::= ENUMERATED {

sec15, sec30, sec60, sec90, sec120, sec180, long-time,

...

}

ExpectedIdlePeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181, ...)

ExpectedUEActivityBehaviour ::= SEQUENCE {

expectedActivityPeriod						ExpectedActivityPeriod						OPTIONAL,

expectedIdlePeriod							ExpectedIdlePeriod							OPTIONAL,

sourceOfUEActivityBehaviourInformation		SourceOfUEActivityBehaviourInformation		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEActivityBehaviour-ExtIEs} }	OPTIONAL,

...

}

ExpectedUEActivityBehaviour-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedUEBehaviour	::= SEQUENCE {

expectedUEActivityBehaviour		ExpectedUEActivityBehaviour 					OPTIONAL,

expectedHOInterval				ExpectedHOInterval								OPTIONAL,

expectedUEMobility				ExpectedUEMobility								OPTIONAL,

expectedUEMovingTrajectory		ExpectedUEMovingTrajectory						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEBehaviour-ExtIEs} }	OPTIONAL,

...

}

ExpectedUEBehaviour-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedUEMobility ::= ENUMERATED {

stationary,

mobile,

...

}

ExpectedUEMovingTrajectory ::= SEQUENCE (SIZE(1..maxnoofCellsUEMovingTrajectory)) OF ExpectedUEMovingTrajectoryItem

ExpectedUEMovingTrajectoryItem ::= SEQUENCE {

nGRAN-CGI				GlobalNG-RANCell-ID,

timeStayedInCell		INTEGER (0..4095)													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEMovingTrajectoryItem-ExtIEs} }	OPTIONAL,

...

}

ExpectedUEMovingTrajectoryItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SourceOfUEActivityBehaviourInformation ::= ENUMERATED {

subscription-information,

statistics,

...

}

ExplicitFormat ::=	SEQUENCE {

permutation			Permutation,

noofDownlinkSymbols	INTEGER(0..14)		OPTIONAL,

noofUplinkSymbols	INTEGER(0..14)		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ExplicitFormat-ExtIEs} } OPTIONAL,

...

}

ExplicitFormat-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedRATRestrictionInformation ::= SEQUENCE {

primaryRATRestriction		BIT STRING (SIZE(8, ...)),

secondaryRATRestriction		BIT STRING (SIZE(8, ...)),

iE-Extensions		ProtocolExtensionContainer { {ExtendedRATRestrictionInformation-ExtIEs} }	OPTIONAL,

...

}

ExtendedRATRestrictionInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedPacketDelayBudget ::= INTEGER (0..65535, ...)

ExtendedSliceSupportList	::= SEQUENCE (SIZE(1..maxnoofExtSliceItems)) OF S-NSSAI

ExtendedUEIdentityIndexValue ::= BIT STRING (SIZE(16))

ExtTLAs ::= SEQUENCE (SIZE(1..maxnoofExtTLAs)) OF ExtTLA-Item

ExtTLA-Item ::= SEQUENCE {

iPsecTLA							TransportLayerAddress		OPTIONAL,

gTPTransportLayerAddresses			GTPTLAs							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExtTLA-Item-ExtIEs} } OPTIONAL,

...

}

ExtTLA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GTPTLAs	::= SEQUENCE (SIZE(1.. maxnoofGTPTLAs)) OF	GTPTLA-Item

GTPTLA-Item	::= SEQUENCE {

gTPTransportLayerAddresses				TransportLayerAddress,

iE-Extensions	ProtocolExtensionContainer { { GTPTLA-Item-ExtIEs } }		OPTIONAL,

...

}

GTPTLA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- F

F1CTrafficContainer ::= OCTET STRING

F1-terminatingIAB-donorIndicator ::= ENUMERATED {true, ...}

F1-TerminatingTopologyBHInformation	::= SEQUENCE {

f1TerminatingBHInformation-List				F1TerminatingBHInformation-List,

iE-Extensions							ProtocolExtensionContainer { {F1-TerminatingTopologyBHInformation-ExtIEs} }	OPTIONAL,

...

}

F1-TerminatingTopologyBHInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

F1TerminatingBHInformation-List ::= SEQUENCE (SIZE(1..maxnoofBHInfo)) OF F1TerminatingBHInformation-Item

F1TerminatingBHInformation-Item ::= SEQUENCE {

bHInfoIndex					BHInfoIndex,

dLTNLAddress				IABTNLAddress,

dlF1TerminatingBHInfo	DLF1Terminating-BHInfo		OPTIONAL,

ulF1TerminatingBHInfo	ULF1Terminating-BHInfo		OPTIONAL,

iE-Extension		ProtocolExtensionContainer { { F1TerminatingBHInformation-Item-ExtIEs} }	OPTIONAL,

...

}

F1TerminatingBHInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

FiveGCMobilityRestrictionListContainer ::= OCTET STRING

-- This octets of the OCTET STRING contain the Mobility Restriction List IE as specified in TS 38.413 [5]. --

FiveGProSeAuthorized ::= SEQUENCE {

fiveGproSeDirectDiscovery					FiveGProSeDirectDiscovery									OPTIONAL,

fiveGproSeDirectCommunication				FiveGProSeDirectCommunication								OPTIONAL,

fiveGnrProSeLayer2UEtoNetworkRelay		FiveGProSeLayer2UEtoNetworkRelay						OPTIONAL,

fiveGnrProSeLayer3UEtoNetworkRelay			FiveGProSeLayer3UEtoNetworkRelay							OPTIONAL,

fiveGnrProSeLayer2RemoteUE					FiveGProSeLayer2RemoteUE									OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { {FiveGProSeAuthorized-ExtIEs} }	OPTIONAL,

...

}

FiveGProSeAuthorized-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

FiveGProSeDirectDiscovery ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeDirectCommunication ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2UEtoNetworkRelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer3UEtoNetworkRelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2RemoteUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSePC5QoSParameters ::= SEQUENCE {

fiveGProSepc5QoSFlowList						FiveGProSePC5QoSFlowList,

fiveGproSepc5LinkAggregateBitRates				BitRate				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { FiveGProSePC5QoSParameters-ExtIEs} }	OPTIONAL,

...

}

FiveGProSePC5QoSParameters-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

FiveGProSePC5QoSFlowList::= SEQUENCE (SIZE(1..maxnoofPC5QoSFlows)) OF FiveGProSePC5QoSFlowItem

FiveGProSePC5QoSFlowItem::= SEQUENCE {

fiveGproSepQI				FiveQI,

fiveGproSepc5FlowBitRates	FiveGProSePC5FlowBitRates				OPTIONAL,

fiveGproSerange				Range						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { FiveGProSePC5QoSFlowItem-ExtIEs} }	OPTIONAL,

...

}

FiveGProSePC5QoSFlowItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

FiveGProSePC5FlowBitRates ::= SEQUENCE {

fiveGproSeguaranteedFlowBitRate		BitRate,

fiveGproSemaximumFlowBitRate		BitRate,

iE-Extensions		ProtocolExtensionContainer { { FiveGProSePC5FlowBitRates-ExtIEs} }	OPTIONAL,

...

}

FiveGProSePC5FlowBitRates-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

FiveQI ::= INTEGER (0..255, ...)

Flows-Mapped-To-DRB-List	::=	SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF Flows-Mapped-To-DRB-Item

Flows-Mapped-To-DRB-Item 	::= SEQUENCE {

qoSFlowIdentifier						QoSFlowIdentifier,

qoSFlowLevelQoSParameters				QoSFlowLevelQoSParameters,

qoSFlowMappingIndication				QoSFlowMappingIndication													OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Flows-Mapped-To-DRB-Item-ExtIEs} } 	OPTIONAL

}

Flows-Mapped-To-DRB-Item-ExtIEs 	XNAP-PROTOCOL-EXTENSION ::= {

...

}

FreqDomainHSNAconfiguration-List ::=  SEQUENCE (SIZE(1.. maxnoofHSNASlots)) OF FreqDomainHSNAconfiguration-List-Item

FreqDomainHSNAconfiguration-List-Item ::= SEQUENCE {

rBsetIndex 									INTEGER(0.. maxnoofRBsetsPerCell1, ...),

freqDomainSlotHSNAconfiguration-List 		FreqDomainSlotHSNAconfiguration-List,

iE-Extensions		ProtocolExtensionContainer { { FreqDomainHSNAconfiguration-List-Item-ExtIEs} }	OPTIONAL,

...

}

FreqDomainHSNAconfiguration-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

FreqDomainSlotHSNAconfiguration-List ::=  SEQUENCE (SIZE(1.. maxnoofHSNASlots)) OF FreqDomainSlotHSNAconfiguration-List-Item

FreqDomainSlotHSNAconfiguration-List-Item ::=	SEQUENCE {

slotIndex		INTEGER(1..maxnoofHSNASlots),

hSNADownlink	HSNADownlink		OPTIONAL,

hSNAUplink		HSNAUplink			OPTIONAL,

hSNAFlexible	HSNAFlexible		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { FreqDomainSlotHSNAconfiguration-List-Item-ExtIEs} }	OPTIONAL,

...

}

FreqDomainSlotHSNAconfiguration-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

FrequencyShift7p5khz ::= ENUMERATED {false, true, ...}

#### -- G

GBRQoSFlowInfo ::= SEQUENCE {

maxFlowBitRateDL			BitRate,

maxFlowBitRateUL			BitRate,

guaranteedFlowBitRateDL		BitRate,

guaranteedFlowBitRateUL		BitRate,

notificationControl			ENUMERATED {notification-requested, ...}				OPTIONAL,

maxPacketLossRateDL			PacketLossRate											OPTIONAL,

maxPacketLossRateUL			PacketLossRate											OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {GBRQoSFlowInfo-ExtIEs} }	OPTIONAL,

...

}

GBRQoSFlowInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-AlternativeQoSParaSetList	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetList	PRESENCE optional	},

...

}

GlobalgNB-ID	::= SEQUENCE {

plmn-id			PLMN-Identity,

gnb-id			GNB-ID-Choice,

iE-Extensions		ProtocolExtensionContainer { {GlobalgNB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalgNB-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GNB-DU-Cell-Resource-Configuration	::= SEQUENCE {

subcarrierSpacing				SSB-subcarrierSpacing,

dUFTransmissionPeriodicity		DUFTransmissionPeriodicity		OPTIONAL,

dUF-Slot-Config-List			DUF-Slot-Config-List			OPTIONAL,

hSNATransmissionPeriodicity		HSNATransmissionPeriodicity		OPTIONAL,

hNSASlotConfigList				HSNASlotConfigList				OPTIONAL,

rBsetConfiguration						RBsetConfiguration		OPTIONAL,

freqDomainHSNAconfiguration-List		FreqDomainHSNAconfiguration-List 	OPTIONAL,

nACellResourceConfigurationList			NACellResourceConfigurationList 		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { GNB-DU-Cell-Resource-Configuration-ExtIEs } } OPTIONAL,

...

}

GNB-DU-Cell-Resource-Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GNB-ID-Choice ::= CHOICE {

gnb-ID					BIT STRING (SIZE(22..32)),

choice-extension		ProtocolIE-Single-Container { {GNB-ID-Choice-ExtIEs} }

}

GNB-ID-Choice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

GNB-RadioResourceStatus	::= SEQUENCE {

ssbAreaRadioResourceStatus-List				SSBAreaRadioResourceStatus-List,

iE-Extensions							ProtocolExtensionContainer { { GNB-RadioResourceStatus-ExtIEs} }	OPTIONAL,

...

}

GNB-RadioResourceStatus-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SliceRadioResourceStatus-List	CRITICALITY ignore	EXTENSION SliceRadioResourceStatus-List	PRESENCE optional	}|

{ ID id-MIMOPRBusageInformation			CRITICALITY ignore	EXTENSION MIMOPRBusageInformation			PRESENCE optional	},

...

}

GlobalCell-ID	::= SEQUENCE {

plmn-id				PLMN-Identity,

cell-type			Cell-Type-Choice,

iE-Extensions		ProtocolExtensionContainer { { GlobalCell-ID-ExtIEs} } OPTIONAL,

...

}

GlobalCell-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalngeNB-ID	::= SEQUENCE {

plmn-id			PLMN-Identity,

enb-id			ENB-ID-Choice,

iE-Extensions		ProtocolExtensionContainer { {GlobaleNB-ID-ExtIEs} } OPTIONAL,

...

}

GlobaleNB-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ENB-ID-Choice ::= CHOICE {

enb-ID-macro			BIT STRING (SIZE(20)),

enb-ID-shortmacro		BIT STRING (SIZE(18)),

enb-ID-longmacro		BIT STRING (SIZE(21)),

choice-extension		ProtocolIE-Single-Container { {ENB-ID-Choice-ExtIEs} }

}

ENB-ID-Choice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

GlobalNG-RANCell-ID	::= SEQUENCE {

plmn-id					PLMN-Identity,

ng-RAN-Cell-id			NG-RAN-Cell-Identity,

iE-Extensions		ProtocolExtensionContainer { {GlobalNG-RANCell-ID-ExtIEs} } OPTIONAL,

...

}

GlobalNG-RANCell-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalNG-RANNode-ID ::= CHOICE {

gNB						GlobalgNB-ID,

ng-eNB					GlobalngeNB-ID,

choice-extension		ProtocolIE-Single-Container { {GlobalNG-RANNode-ID-ExtIEs} }

}

GlobalNG-RANNode-ID-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

GTP-TEID	::= OCTET STRING (SIZE(4))

GTPtunnelTransportLayerInformation ::= SEQUENCE {

tnl-address			TransportLayerAddress,

gtp-teid			GTP-TEID,

iE-Extensions		ProtocolExtensionContainer { {GTPtunnelTransportLayerInformation-ExtIEs} } OPTIONAL,

...

}

GTPtunnelTransportLayerInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GUAMI ::= SEQUENCE {

plmn-ID				PLMN-Identity,

amf-region-id		BIT STRING (SIZE (8)),

amf-set-id			BIT STRING (SIZE (10)),

amf-pointer			BIT STRING (SIZE (6)),

iE-Extensions		ProtocolExtensionContainer { {GUAMI-ExtIEs} } OPTIONAL,

...

}

GUAMI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- H

HandoverReportType ::= ENUMERATED {

hoTooEarly,

hoToWrongCell,

intersystempingpong,

...

}

HSNASlotConfigList ::= SEQUENCE (SIZE(1..maxnoofHSNASlots)) OF HSNASlotConfigItem

HSNASlotConfigItem 	::=	SEQUENCE {

hSNADownlink			HSNADownlink 		OPTIONAL,

hSNAUplink				HSNAUplink 			OPTIONAL,

hSNAFlexible			HSNAFlexible 		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { HSNASlotConfigItem-ExtIEs } } OPTIONAL,

...

}

HSNASlotConfigItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

HSNADownlink ::= ENUMERATED { hard, soft, notavailable }

HSNAFlexible ::= ENUMERATED { hard, soft, notavailable }

HSNAUplink ::= ENUMERATED { hard, soft, notavailable }

HSNATransmissionPeriodicity ::=	ENUMERATED { ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160, ...}

Hysteresis ::=		INTEGER (0..30)

#### -- I

IABCellInformation::=	SEQUENCE{

nRCGI 								NR-CGI,

iAB-DU-Cell-Resource-Configuration-Mode-Info 	IAB-DU-Cell-Resource-Configuration-Mode-Info	OPTIONAL,

iAB-STC-Info						IAB-STC-Info				OPTIONAL,

rACH-Config-Common					RACH-Config-Common			OPTIONAL,

rACH-Config-Common-IAB				RACH-Config-Common-IAB		OPTIONAL,

cSI-RS-Configuration				OCTET STRING	OPTIONAL,

sR-Configuration					OCTET STRING	OPTIONAL,

pDCCH-ConfigSIB1					OCTET STRING	OPTIONAL,

sCS-Common							OCTET STRING	OPTIONAL,

multiplexingInfo					MultiplexingInfo	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { IABCellInformation-ExtIEs} } OPTIONAL,

...

}

IABCellInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IAB-DU-Cell-Resource-Configuration-Mode-Info	::=	CHOICE {

tDD		IAB-DU-Cell-Resource-Configuration-TDD-Info,

fDD		IAB-DU-Cell-Resource-Configuration-FDD-Info,

choice-extension			ProtocolIE-Single-Container { { IAB-DU-Cell-Resource-Configuration-Mode-Info-ExtIEs} }

}

IAB-DU-Cell-Resource-Configuration-Mode-Info-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

IAB-DU-Cell-Resource-Configuration-FDD-Info ::= SEQUENCE {

gNB-DU-Cell-Resource-Configuration-FDD-UL				GNB-DU-Cell-Resource-Configuration,

gNB-DU-Cell-Resource-Configuration-FDD-DL				GNB-DU-Cell-Resource-Configuration,

uLFrequencyInfo 					NRFrequencyInfo			OPTIONAL,

dLFrequencyInfo 					NRFrequencyInfo			OPTIONAL,

uLTransmissionBandwidth 			NRTransmissionBandwidth	OPTIONAL,

dlTransmissionBandwidth 			NRTransmissionBandwidth	OPTIONAL,

uLCarrierList					NRCarrierList			OPTIONAL,

dlCarrierList					NRCarrierList			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {IAB-DU-Cell-Resource-Configuration-FDD-Info-ExtIEs} } OPTIONAL,

...

}

IAB-DU-Cell-Resource-Configuration-FDD-Info-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IAB-DU-Cell-Resource-Configuration-TDD-Info ::= SEQUENCE {

gNB-DU-Cell-Resource-Configuration-TDD				GNB-DU-Cell-Resource-Configuration,

frequencyInfo 					NRFrequencyInfo			OPTIONAL,

transmissionBandwidth 			NRTransmissionBandwidth	OPTIONAL,

carrierList						NRCarrierList			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {IAB-DU-Cell-Resource-Configuration-TDD-Info-ExtIEs} } OPTIONAL,

...

}

IAB-DU-Cell-Resource-Configuration-TDD-Info-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IAB-MT-Cell-List ::= SEQUENCE (SIZE(1..maxnoofServingCells)) OF IAB-MT-Cell-List-Item

IAB-MT-Cell-List-Item ::= 	SEQUENCE {

nRCellIdentity				NR-Cell-Identity,

dU-RX-MT-RX					DU-RX-MT-RX,

dU-TX-MT-TX					DU-TX-MT-TX,

dU-RX-MT-TX					DU-RX-MT-TX,

dU-TX-MT-RX					DU-TX-MT-RX,

iE-Extensions				ProtocolExtensionContainer { { IAB-MT-Cell-List-Item-ExtIEs } } OPTIONAL,

...

}

IAB-MT-Cell-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IABNodeIndication ::= ENUMERATED {true,...}

IAB-QoS-Mapping-Information ::= SEQUENCE {

dscp							BIT STRING (SIZE(6))			OPTIONAL,

flow-label						BIT STRING (SIZE(20))		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {IAB-QoS-Mapping-Information-ExtIEs} }	OPTIONAL,

...

}

IAB-QoS-Mapping-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IAB-STC-Info	::=	SEQUENCE{

iAB-STC-Info-List	IAB-STC-Info-List,

iE-Extensions		ProtocolExtensionContainer { { IAB-STC-Info-ExtIEs } } OPTIONAL,

...

}

IAB-STC-Info-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IAB-STC-Info-List ::= 	SEQUENCE (SIZE(1..maxnoofIABSTCInfo)) OF IAB-STC-Info-Item

IAB-STC-Info-Item::=	SEQUENCE {

sSB-freqInfo						SSB-freqInfo,

sSB-subcarrierSpacing				SSB-subcarrierSpacing,

sSB-transmissionPeriodicity			SSB-transmissionPeriodicity,

sSB-transmissionTimingOffset		SSB-transmissionTimingOffset,

sSB-transmissionBitmap				SSB-transmissionBitmap,

iE-Extensions		ProtocolExtensionContainer { { IAB-STC-Info-Item-ExtIEs } } OPTIONAL,

...

}

IAB-STC-Info-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IAB-TNL-Address-Request ::= SEQUENCE {

iABIPv4AddressesRequested				IABTNLAddressesRequested,

iABIPv6RequestType						IABIPv6RequestType,

iABTNLAddressToRemove-List				IABTNLAddressToRemove-List,

iE-Extensions							ProtocolExtensionContainer { {IAB-TNL-Address-Request-ExtIEs} }	OPTIONAL,

...

}

IAB-TNL-Address-Request-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IABIPv6RequestType	::= CHOICE {

iPv6Address						IABTNLAddressesRequested,

iPv6Prefix						IABTNLAddressesRequested,

choice-extension				ProtocolIE-Single-Container { {IABIPv6RequestType-ExtIEs} }

}

IABIPv6RequestType-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

IAB-TNL-Address-Response ::= SEQUENCE {

iABAllocatedTNLAddress-List		IABAllocatedTNLAddress-List,

iE-Extensions					ProtocolExtensionContainer { {IAB-TNL-Address-Response-ExtIEs} }	OPTIONAL,

...

}

IAB-TNL-Address-Response-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IABAllocatedTNLAddress-List	::= SEQUENCE (SIZE(1..maxnoofTLAsIAB))	OF IABAllocatedTNLAddress-Item

IABAllocatedTNLAddress-Item ::=	SEQUENCE {

iABTNLAddress				IABTNLAddress,

iABTNLAddressUsage			IABTNLAddressUsage		OPTIONAL,

associatedDonorDUAddress	BAPAddress				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {IABAllocatedTNLAddress-Item-ExtIEs} }	OPTIONAL,

...

}

IABAllocatedTNLAddress-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IABTNLAddress ::= CHOICE {

iPv4Address						BIT STRING (SIZE(32)),

iPv6Address						BIT STRING (SIZE(128)),

iPv6Prefix						BIT STRING (SIZE(64)),

choice-extension				ProtocolIE-Single-Container { {IABTNLAddress-ExtIEs} }

}

IABTNLAddress-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

IABTNLAddressesRequested ::= SEQUENCE {

tNLAddressesOrPrefixesRequestedAllTraffic	INTEGER (1..256) 	OPTIONAL,

tNLAddressesOrPrefixesRequestedF1-C			INTEGER (1..256) 	OPTIONAL,

tNLAddressesOrPrefixesRequestedF1-U			INTEGER (1..256) 	OPTIONAL,

tNLAddressesOrPrefixesRequestedNoNF1		INTEGER (1..256) 	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {IABTNLAddressesRequested-ExtIEs} } OPTIONAL

}

IABTNLAddressesRequested-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IABTNLAddressToRemove-List	::= SEQUENCE (SIZE(1..maxnoofTLAsIAB))	OF IABTNLAddressToRemove-Item

IABTNLAddressToRemove-Item ::=	SEQUENCE {

iABTNLAddress			IABTNLAddress,

iE-Extension			ProtocolExtensionContainer { {IABTNLAddressToRemove-Item-ExtIEs} }	OPTIONAL,

...

}

IABTNLAddressToRemove-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IABTNLAddressUsage ::= ENUMERATED {

f1-c,

f1-u,

non-f1,

...,

all

}

IABTNLAddressException ::= SEQUENCE (SIZE(1..maxnoofTLAsIAB)) OF IABTNLAddress-Item

IABTNLAddress-Item ::= SEQUENCE {

iABTNLAddress					IABTNLAddress,

iE-Extensions					ProtocolExtensionContainer { { IABTNLAddress-ItemExtIEs } }	OPTIONAL,

...}

IABTNLAddress-ItemExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ImmediateMDT-EUTRA ::= OCTET STRING

ImmediateMDT-NR ::= SEQUENCE {

measurementsToActivate		MeasurementsToActivate,

m1Configuration				M1Configuration				OPTIONAL,

m4Configuration				M4Configuration				OPTIONAL,

m5Configuration				M5Configuration				OPTIONAL,

mDT-Location-Info			MDT-Location-Info			OPTIONAL,

m6Configuration				M6Configuration				OPTIONAL,

m7Configuration				M7Configuration				OPTIONAL,

bluetoothMeasurementConfiguration			BluetoothMeasurementConfiguration			OPTIONAL,

wLANMeasurementConfiguration					WLANMeasurementConfiguration					OPTIONAL,

sensorMeasurementConfiguration				SensorMeasurementConfiguration				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { ImmediateMDT-NR-ExtIEs} } 	OPTIONAL,

...

}

ImmediateMDT-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ImplicitFormat	::= SEQUENCE	{

dUFSlotformatIndex 			DUFSlotformatIndex,

iE-Extensions		ProtocolExtensionContainer { { ImplicitFormat-ExtIEs } } OPTIONAL,

...

}

ImplicitFormat-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

InitiatingCondition-FailureIndication ::= CHOICE {

rRCReestab					RRCReestab-initiated,

rRCSetup					RRCSetup-initiated,

choice-extension				ProtocolIE-Single-Container { {InitiatingCondition-FailureIndication-ExtIEs} }

}

InitiatingCondition-FailureIndication-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

IntendedTDD-DL-ULConfiguration-NR ::= SEQUENCE {

nrscs							NRSCS,

nrCyclicPrefix					NRCyclicPrefix,

nrDL-ULTransmissionPeriodicity	NRDL-ULTransmissionPeriodicity,

slotConfiguration-List			SlotConfiguration-List,

iE-Extensions					ProtocolExtensionContainer { {IntendedTDD-DL-ULConfiguration-NR-ExtIEs} }	OPTIONAL,

...

}

IntendedTDD-DL-ULConfiguration-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

InterfaceInstanceIndication ::= INTEGER (0..255, ...)

I-RNTI ::= CHOICE {

i-RNTI-full			BIT STRING (SIZE(40)),

i-RNTI-short		BIT STRING (SIZE(24)),

choice-extension	ProtocolIE-Single-Container { {I-RNTI-ExtIEs} }

}

I-RNTI-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

#### -- J

#### -- K

#### -- L

Local-NG-RAN-Node-Identifier ::= CHOICE {

full-I-RNTI-Profile-List					Full-I-RNTI-Profile-List,

short-I-RNTI-Profile-List					Short-I-RNTI-Profile-List,

choice-extension							ProtocolIE-Single-Container { { Local-NG-RAN-Node-Identifier-ExtIEs} }

}

Local-NG-RAN-Node-Identifier-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

Full-I-RNTI-Profile-List ::= CHOICE {

full-I-RNTI-Profile-0	BIT STRING (SIZE (21)),

full-I-RNTI-Profile-1	BIT STRING (SIZE (18)),

full-I-RNTI-Profile-2	BIT STRING (SIZE (15)),

full-I-RNTI-Profile-3	BIT STRING (SIZE (12)),

choice-extension		ProtocolIE-Single-Container { { Full-I-RNTI-Profile-List-ExtIEs} }

}

Full-I-RNTI-Profile-List-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

Short-I-RNTI-Profile-List ::= CHOICE {

short-I-RNTI-Profile-0	BIT STRING (SIZE (8)),

short-I-RNTI-Profile-1	BIT STRING (SIZE (6)),

choice-extension		ProtocolIE-Single-Container { { Short-I-RNTI-Profile-List-ExtIEs} }

}

Short-I-RNTI-Profile-List-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

LastVisitedCell-Item ::= CHOICE {

nG-RAN-Cell						LastVisitedNGRANCellInformation,

e-UTRAN-Cell					LastVisitedEUTRANCellInformation,

uTRAN-Cell						LastVisitedUTRANCellInformation,

gERAN-Cell						LastVisitedGERANCellInformation,

choice-extension				ProtocolIE-Single-Container { { LastVisitedCell-Item-ExtIEs} }

}

LastVisitedCell-Item-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

LastVisitedEUTRANCellInformation ::= OCTET STRING

LastVisitedGERANCellInformation	::= OCTET STRING

LastVisitedNGRANCellInformation	::= OCTET STRING

LastVisitedUTRANCellInformation	::= OCTET STRING

LastVisitedPSCellInformation	::= OCTET STRING

LastVisitedPSCellList	::= SEQUENCE (SIZE(1..maxnoofPSCellsPerSN)) OF LastVisitedPSCellList-Item

LastVisitedPSCellList-Item	::= SEQUENCE {

lastVisitedPSCellInformation		LastVisitedPSCellInformation,

iE-Extensions		ProtocolExtensionContainer { { LastVisitedPSCellList-Item-ExtIEs} } OPTIONAL,

...

}

LastVisitedPSCellList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SCGUEHistoryInformation	::= SEQUENCE {

lastVisitedPSCellList			LastVisitedPSCellList		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { SCGUEHistoryInformation-ExtIEs} } OPTIONAL,

...

}

SCGUEHistoryInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LCID ::= INTEGER (1..32, ...)

Links-to-log ::= ENUMERATED {uplink, downlink, both-uplink-and-downlink, ...}

ListOfCells ::= SEQUENCE (SIZE(1..maxnoofCellsinAoI)) OF CellsinAoI-Item

CellsinAoI-Item ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

ng-ran-cell-id			NG-RAN-Cell-Identity,

iE-Extensions			ProtocolExtensionContainer { {CellsinAoI-Item-ExtIEs} } OPTIONAL,

...

}

CellsinAoI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ListOfRANNodesinAoI ::= SEQUENCE (SIZE(1.. maxnoofRANNodesinAoI)) OF GlobalNG-RANNodesinAoI-Item

GlobalNG-RANNodesinAoI-Item ::= SEQUENCE {

global-NG-RAN-Node-ID		GlobalNG-RANNode-ID,

iE-Extensions		ProtocolExtensionContainer { {GlobalNG-RANNodesinAoI-Item-ExtIEs} } OPTIONAL,

...

}

GlobalNG-RANNodesinAoI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ListOfTAIsinAoI ::= SEQUENCE (SIZE(1..maxnoofTAIsinAoI)) OF TAIsinAoI-Item

TAIsinAoI-Item ::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

tAC					TAC,

iE-Extensions		ProtocolExtensionContainer { {TAIsinAoI-Item-ExtIEs} } OPTIONAL,

...

}

TAIsinAoI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LocationInformationSNReporting ::= ENUMERATED {

pSCell,

...

}

LocationReportingInformation ::= SEQUENCE {

eventType			EventType,

reportArea			ReportArea,

areaOfInterest		AreaOfInterestInformation			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LocationReportingInformation-ExtIEs} } OPTIONAL,

...

}

LocationReportingInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ ID id-AdditionLocationInformation	CRITICALITY ignore	EXTENSION AdditionLocationInformation	PRESENCE optional},

...

}

LoggedMDT-EUTRA ::= SEQUENCE {

loggingInterval								LoggingInterval,

loggingDuration								LoggingDuration,

bluetoothMeasurementConfiguration			BluetoothMeasurementConfiguration		OPTIONAL,

wLANMeasurementConfiguration				WLANMeasurementConfiguration			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {LoggedMDT-EUTRA-ExtIEs} } OPTIONAL,

...

}

LoggedMDT-EUTRA-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

LoggedEventTriggeredConfig ::= SEQUENCE {

eventTypeTrigger					EventTypeTrigger,

iE-Extensions		ProtocolExtensionContainer { { LoggedEventTriggeredConfig-ExtIEs} } OPTIONAL,

...

}

LoggedEventTriggeredConfig-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LoggedMDT-NR ::= SEQUENCE {

loggingInterval						LoggingInterval,

loggingDuration						LoggingDuration,

reportType							ReportType,

bluetoothMeasurementConfiguration	BluetoothMeasurementConfiguration			OPTIONAL,

wLANMeasurementConfiguration			WLANMeasurementConfiguration				OPTIONAL,

sensorMeasurementConfiguration		SensorMeasurementConfiguration				OPTIONAL,

areaScopeOfNeighCellsList			AreaScopeOfNeighCellsList					OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {LoggedMDT-NR-ExtIEs} } OPTIONAL,

...

}

LoggedMDT-NR-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

{ID id-earlyMeasurement		CRITICALITY ignore	EXTENSION EarlyMeasurement		PRESENCE optional		},

...

}

LoggingInterval ::= ENUMERATED { ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960, ms61440}

LoggingDuration ::= ENUMERATED {m10, m20, m40, m60, m90, m120}

LowerLayerPresenceStatusChange ::= ENUMERATED {

release-lower-layers,

re-establish-lower-layers,

...,

suspend-lower-layers,

resume-lower-layers

}

LTEV2XServicesAuthorized ::= SEQUENCE {

vehicleUE			VehicleUE											OPTIONAL,

pedestrianUE 		PedestrianUE										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LTEV2XServicesAuthorized-ExtIEs} }	OPTIONAL,

...

}

LTEV2XServicesAuthorized-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTEUESidelinkAggregateMaximumBitRate ::= SEQUENCE {

uESidelinkAggregateMaximumBitRate		BitRate,

iE-Extensions					ProtocolExtensionContainer { {LTEUESidelinkAggregateMaximumBitRate-ExtIEs} } OPTIONAL,

...

}

LTEUESidelinkAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- M

MaxNrofRS-IndexesToReport::= INTEGER (1..64, ...)

MDTAlignmentInfo ::= CHOICE {

s-BasedMDT						S-BasedMDT,

choice-extension				ProtocolIE-Single-Container { {MDTAlignmentInfo-ExtIEs} }

}

MDTAlignmentInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

MeasCollectionEntityIPAddress ::= TransportLayerAddress

M1Configuration ::= SEQUENCE {

m1reportingTrigger			M1ReportingTrigger,

m1thresholdeventA2			M1ThresholdEventA2				OPTIONAL,

-- Included in case of event-triggered, or event-triggered periodic reporting for measurement M1

m1periodicReporting			M1PeriodicReporting				OPTIONAL,

-- Included in case of periodic or event-triggered periodic reporting

iE-Extensions				ProtocolExtensionContainer { { M1Configuration-ExtIEs} } OPTIONAL,

...

}

M1Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-BeamMeasurementIndicationM1				CRITICALITY ignore	EXTENSION BeamMeasurementIndicationM1					PRESENCE optional		}|

{ID id-BeamMeasurementsReportConfiguration		CRITICALITY ignore	EXTENSION BeamMeasurementsReportConfiguration		PRESENCE conditional		},

-- The above IE shall be present if the BeamMeasurementIndicationM1 IE is set to “true”

...

}

M1PeriodicReporting ::= SEQUENCE {

reportInterval				ReportIntervalMDT,

reportAmount				ReportAmountMDT,

iE-Extensions				ProtocolExtensionContainer { { M1PeriodicReporting-ExtIEs} } OPTIONAL,

...

}

M1PeriodicReporting-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-ExtendedReportIntervalMDT		CRITICALITY ignore	EXTENSION ExtendedReportIntervalMDT		PRESENCE optional},

...

}

M1ReportingTrigger ::= ENUMERATED{

periodic,

a2eventtriggered,

a2eventtriggered-periodic,

...

}

M1ThresholdEventA2 ::= SEQUENCE {

measurementThreshold	MeasurementThresholdA2,

iE-Extensions			ProtocolExtensionContainer { { M1ThresholdEventA2-ExtIEs} } OPTIONAL,

...

}

M1ThresholdEventA2-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

M4Configuration ::= SEQUENCE {

m4period			M4period,

m4-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M4Configuration-ExtIEs} } OPTIONAL,

...

}

M4Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-M4ReportAmount		CRITICALITY ignore	EXTENSION M4ReportAmountMDT 		PRESENCE optional		},

...

}

M4ReportAmountMDT ::= ENUMERATED{r1, r2, r4, r8, r16, r32, r64, infinity, ...}

M4period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M5Configuration ::= SEQUENCE {

m5period			M5period,

m5-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M5Configuration-ExtIEs} } OPTIONAL,

...

}

M5Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-M5ReportAmount		CRITICALITY ignore	EXTENSION M5ReportAmountMDT 		PRESENCE optional		},

...

}

M5ReportAmountMDT ::= ENUMERATED{r1, r2, r4, r8, r16, r32, r64, infinity, ...}

M5period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M6Configuration ::= SEQUENCE {

m6report-Interval	M6report-Interval,

m6-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M6Configuration-ExtIEs} } OPTIONAL,

...

}

M6Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-M6ReportAmount		CRITICALITY ignore	EXTENSION M6ReportAmountMDT 		PRESENCE optional		},

...

}

M6ReportAmountMDT ::= ENUMERATED{r1, r2, r4, r8, r16, r32, r64, infinity, ...}

M6report-Interval ::= ENUMERATED { ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30,... }

M7Configuration ::= SEQUENCE {

m7period			M7period,

m7-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M7Configuration-ExtIEs} } OPTIONAL,

...

}

M7Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-M7ReportAmount		CRITICALITY ignore	EXTENSION M7ReportAmountMDT 	PRESENCE optional		},

...

}

M7ReportAmountMDT ::= ENUMERATED{r1, r2, r4, r8, r16, r32, r64, infinity, ...}

M7period ::= INTEGER(1..60, ...)

MAC-I ::= BIT STRING (SIZE(16))

MaskedIMEISV	::= BIT STRING (SIZE(64))

MaxCHOpreparations ::= INTEGER (1..8, ...)

MaximumDataBurstVolume ::= INTEGER (0..4095, ..., 4096.. 2000000)

MaximumIPdatarate ::= SEQUENCE {

maxIPrate-UL			MaxIPrate,

iE-Extensions		ProtocolExtensionContainer { {MaximumIPdatarate-ExtIEs} }	OPTIONAL,

...

}

MaximumIPdatarate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-MaxIPrate-DL	CRITICALITY ignore	EXTENSION MaxIPrate	PRESENCE optional},

...

}

MaxIPrate ::= ENUMERATED {

bitrate64kbs,

max-UErate,

...

}

MBSFNControlRegionLength ::= INTEGER (0..3)

MBSFNSubframeAllocation-E-UTRA ::= CHOICE {

oneframe				BIT STRING (SIZE(6)),

fourframes				BIT STRING (SIZE(24)),

choice-extension		ProtocolIE-Single-Container { {MBSFNSubframeAllocation-E-UTRA-ExtIEs} }

}

MBSFNSubframeAllocation-E-UTRA-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

MBSFNSubframeInfo-E-UTRA ::= SEQUENCE (SIZE(1..maxnoofMBSFNEUTRA)) OF MBSFNSubframeInfo-E-UTRA-Item

MBSFNSubframeInfo-E-UTRA-Item ::= SEQUENCE {

radioframeAllocationPeriod		ENUMERATED{n1,n2,n4,n8,n16,n32,...},

radioframeAllocationOffset		INTEGER (0..7, ...),

subframeAllocation				MBSFNSubframeAllocation-E-UTRA,

iE-Extensions					ProtocolExtensionContainer { {MBSFNSubframeInfo-E-UTRA-Item-ExtIEs} } OPTIONAL,

...

}

MBSFNSubframeInfo-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

MBS-FrequencySelectionArea-Identity ::= OCTET STRING (SIZE(3))

MBS-Area-Session-ID  ::= INTEGER (0..65535, ...)

MBS-MappingandDataForwardingRequestInfofromSource ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MBS-MappingandDataForwardingRequestInfofromSource-Item

MBS-MappingandDataForwardingRequestInfofromSource-Item ::= SEQUENCE {

mRB-ID					MRB-ID,

mBS-QoSFlow-List			MBS-QoSFlow-List,

mRB-ProgressInformation	MRB-ProgressInformation				OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { MBS-MappingandDataForwardingRequestInfofromSource-Item-ExtIEs} } OPTIONAL,

...

}

MBS-MappingandDataForwardingRequestInfofromSource-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-DataForwardingResponseInfofromTarget ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MBS-DataForwardingResponseInfofromTarget-Item

MBS-DataForwardingResponseInfofromTarget-Item ::= SEQUENCE {

mRB-ID					MRB-ID,

dlForwardingUPTNL		UPTransportLayerInformation,

mRB-ProgressInformation	MRB-ProgressInformation					OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { MBS-DataForwardingResponseInfofromTarget-Item-ExtIEs} } OPTIONAL,

...

}

MBS-DataForwardingResponseInfofromTarget-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-QoSFlow-List ::= SEQUENCE (SIZE(1..maxnoofMBSQoSFlows)) OF QoSFlowIdentifier

MBS-QoSFlowsToAdd-List ::= SEQUENCE (SIZE(1..maxnoofMBSQoSFlows)) OF MBS-QoSFlowsToAdd-Item

MBS-QoSFlowsToAdd-Item ::= SEQUENCE {

mBS-QosFlowIdentifier				QoSFlowIdentifier,

mBS-QosFlowLevelQosParameters		QoSFlowLevelQoSParameters,

iE-Extensions					ProtocolExtensionContainer { { MBS-QoSFlowsToAdd-Item-ExtIEs} } OPTIONAL,

...

}

MBS-QoSFlowsToAdd-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceArea ::= CHOICE {

locationindependent		MBS-ServiceAreaInformation,

locationdependent		MBS-ServiceAreaInformationList,

choice-extension		ProtocolIE-Single-Container { {MBS-ServiceArea-ExtIEs} }

}

MBS-ServiceArea-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

MBS-ServiceAreaCell-List ::= SEQUENCE (SIZE(1.. maxnoofCellsforMBS)) OF NR-CGI

MBS-ServiceAreaInformation ::= SEQUENCE {

mBS-ServiceAreaCell-List			MBS-ServiceAreaCell-List														OPTIONAL,

mBS-ServiceAreaTAI-List			MBS-ServiceAreaTAI-List														OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformation-ExtIEs} }		OPTIONAL,

...

}

MBS-ServiceAreaInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaInformationList ::= SEQUENCE (SIZE(1..maxnoofMBSServiceAreaInformation)) OF MBS-ServiceAreaInformation-Item

MBS-ServiceAreaInformation-Item ::= SEQUENCE {	mBS-Area-Session-ID			MBS-Area-Session-ID,

mBS-ServiceAreaInformation	MBS-ServiceAreaInformation,

iE-Extensions			ProtocolExtensionContainer { { MBS-ServiceAreaInformation-Item-ExtIEs} } OPTIONAL,

...

}

MBS-ServiceAreaInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaTAI-List ::= SEQUENCE (SIZE(1.. maxnoofTAIforMBS)) OF MBS-ServiceAreaTAI-Item

MBS-ServiceAreaTAI-Item ::= SEQUENCE {

plmn-ID					PLMN-Identity,

tAC						TAC,

iE-Extensions			ProtocolExtensionContainer { {MBS-ServiceAreaTAI-Item-ExtIEs} } OPTIONAL,

...

}

MBS-ServiceAreaTAI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Session-ID ::= SEQUENCE {

tMGI								TMGI,

nID								NID																				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-Session-ID-ExtIEs} } 			OPTIONAL,

...

}

MBS-Session-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-SessionAssociatedInformation ::= SEQUENCE (SIZE(1..maxnoofAssociatedMBSSessions)) OF MBS-SessionAssociatedInformation-Item

MBS-SessionAssociatedInformation-Item ::= SEQUENCE {

mBS-Session-ID				MBS-Session-ID,

associated-QoSFlowInfo-List	Associated-QoSFlowInfo-List,

iE-Extensions				ProtocolExtensionContainer { { MBS-SessionAssociatedInformation-Item-ExtIEs} } OPTIONAL,

...

}

MBS-SessionAssociatedInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-SessionInformation-List ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBS-SessionInformation-Item

MBS-SessionInformation-Item ::= SEQUENCE {

mBS-Session-ID			MBS-Session-ID,

mBS-Area-Session-ID		MBS-Area-Session-ID														OPTIONAL,

active-MBS-SessioInformation			Active-MBS-SessionInformation							OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { MBS-SessionInformation-Item-ExtIEs} }	OPTIONAL,

...

}

MBS-SessionInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-SessionInformationResponse-List ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBS-SessionInformationResponse-Item

MBS-SessionInformationResponse-Item ::= SEQUENCE {

mBS-Session-ID						MBS-Session-ID,

mBS-DataForwardingResponseInfofromTarget		MBS-DataForwardingResponseInfofromTarget					OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { MBS-SessionInformationResponse-Item-ExtIEs} } 	OPTIONAL,

...

}

MBS-SessionInformationResponse-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MRB-ID ::= INTEGER (1..512, ...)

MRB-ProgressInformation ::= CHOICE {

pdcp-SN12			INTEGER (0..4095),

pdcp-SN18			INTEGER (0..262143),

choice-extension		ProtocolIE-Single-Container { { MRB-ProgressInformation-ExtIEs} }

}

MRB-ProgressInformation-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

MDT-Activation 	::= ENUMERATED {

immediate-MDT-only,

immediate-MDT-and-Trace,

logged-MDT-only,

...

}

MDT-Configuration ::= SEQUENCE {

mDT-Configuration-NR		MDT-Configuration-NR		OPTIONAL,

mDT-Configuration-EUTRA		MDT-Configuration-EUTRA		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MDT-Configuration-ExtIEs} } OPTIONAL,

...

}

MDT-Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MDT-Configuration-NR ::= SEQUENCE {

mdt-Activation				MDT-Activation,

areaScopeOfMDT-NR			AreaScopeOfMDT-NR	OPTIONAL,

mDTMode-NR					MDTMode-NR,

signallingBasedMDTPLMNList	MDTPLMNList,

iE-Extensions		ProtocolExtensionContainer { { MDT-Configuration-NR-ExtIEs} } OPTIONAL,

...

}

MDT-Configuration-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MDT-Configuration-EUTRA ::= SEQUENCE {

mdt-Activation				MDT-Activation,

areaScopeOfMDT-EUTRA		AreaScopeOfMDT-EUTRA	OPTIONAL,

mDTMode-EUTRA				MDTMode-EUTRA,

signallingBasedMDTPLMNList	MDTPLMNList,

iE-Extensions		ProtocolExtensionContainer { { MDT-Configuration-EUTRA-ExtIEs} } OPTIONAL,

...

}

MDT-Configuration-EUTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MDT-Location-Info ::= BIT STRING (SIZE (8))

MDTPLMNList ::= SEQUENCE (SIZE(1..maxnoofMDTPLMNs)) OF PLMN-Identity

MDTPLMNModificationList ::= SEQUENCE (SIZE(0..maxnoofMDTPLMNs)) OF PLMN-Identity

MDTMode-NR ::= CHOICE {

immediateMDT				ImmediateMDT-NR,

loggedMDT					LoggedMDT-NR,

...,

mDTMode-NR-Extension			MDTMode-NR-Extension

}

MDTMode-NR-Extension ::= ProtocolIE-Single-Container {{ MDTMode-NR-ExtensionIE }}

MDTMode-NR-ExtensionIE XNAP-PROTOCOL-IES ::= {

...

}

MDTMode-EUTRA ::= CHOICE {

immediateMDT				ImmediateMDT-EUTRA,

loggedMDT					LoggedMDT-EUTRA,

...,

mDTMode-EUTRA-Extension			MDTMode-EUTRA-Extension

}

MDTMode-EUTRA-Extension ::= ProtocolIE-Single-Container {{ MDTMode-EUTRA-ExtensionIE }}

MDTMode-EUTRA-ExtensionIE XNAP-PROTOCOL-IES ::= {

...

}

MeasObjectContainer ::= OCTET STRING

MeasurementsToActivate ::= BIT STRING (SIZE (8))

MeasurementThresholdA2 ::= CHOICE {

threshold-RSRP				Threshold-RSRP,

threshold-RSRQ				Threshold-RSRQ,

threshold-SINR				Threshold-SINR,

choice-extension	ProtocolIE-Single-Container { { MeasurementThresholdA2-ExtIEs} }

}

MeasurementThresholdA2-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

Measurement-ID	::= INTEGER (1..4095,...)

MIMOPRBusageInformation ::= SEQUENCE {

dl-GBR-PRB-usage-for-MIMO					DL-GBR-PRB-usage-for-MIMO,

ul-GBR-PRB-usage-for-MIMO					UL-GBR-PRB-usage-for-MIMO,

dl-non-GBR-PRB-usage-for-MIMO				DL-non-GBR-PRB-usage-for-MIMO,

ul-non-GBR-PRB-usage-for-MIMO				UL-non-GBR-PRB-usage-for-MIMO,

dl-Total-PRB-usage-for-MIMO					DL-Total-PRB-usage-for-MIMO,

ul-Total-PRB-usage-for-MIMO					UL-Total-PRB-usage-for-MIMO,

iE-Extensions							ProtocolExtensionContainer { { MIMOPRBusageInformation-ExtIEs} }	OPTIONAL,

...

}

MIMOPRBusageInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MobilityInformation	::= BIT STRING (SIZE(32))

MobilityParametersModificationRange ::= SEQUENCE {

handoverTriggerChangeLowerLimit		INTEGER (-20..20),

handoverTriggerChangeUpperLimit		INTEGER (-20..20),

...

}

MobilityParametersInformation ::= SEQUENCE {

handoverTriggerChange			INTEGER (-20..20),

...

}

MobilityRestrictionList ::= SEQUENCE {

serving-PLMN						PLMN-Identity,

equivalent-PLMNs					SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF PLMN-Identity			OPTIONAL,

rat-Restrictions					RAT-RestrictionsList										OPTIONAL,

forbiddenAreaInformation			ForbiddenAreaList											OPTIONAL,

serviceAreaInformation				ServiceAreaList												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MobilityRestrictionList-ExtIEs} }				OPTIONAL,

...

}

MobilityRestrictionList-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ ID id-LastE-UTRANPLMNIdentity					CRITICALITY ignore	EXTENSION PLMN-Identity									PRESENCE optional		}|

{ ID id-CNTypeRestrictionsForServing			CRITICALITY ignore	EXTENSION CNTypeRestrictionsForServing				PRESENCE optional		}|

{ ID id-CNTypeRestrictionsForEquivalent			CRITICALITY ignore	EXTENSION CNTypeRestrictionsForEquivalent			PRESENCE optional		}|

{ ID id-NPNMobilityInformation					CRITICALITY reject	EXTENSION NPNMobilityInformation						PRESENCE optional		},

...

}

CNTypeRestrictionsForEquivalent ::= SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF CNTypeRestrictionsForEquivalentItem

CNTypeRestrictionsForEquivalentItem ::= SEQUENCE {

plmn-Identity						PLMN-Identity,

cn-Type								ENUMERATED {epc-forbidden, fiveGC-forbidden, ...},

iE-Extensions						ProtocolExtensionContainer { {CNTypeRestrictionsForEquivalentItem-ExtIEs} }				OPTIONAL,

...

}

CNTypeRestrictionsForEquivalentItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

CNTypeRestrictionsForServing ::= ENUMERATED {

epc-forbidden,

...

}

RAT-RestrictionsList ::= SEQUENCE (SIZE(1..maxnoofPLMNs)) OF RAT-RestrictionsItem

RAT-RestrictionsItem ::= SEQUENCE {

plmn-Identity					PLMN-Identity,

rat-RestrictionInformation		RAT-RestrictionInformation,

iE-Extensions		ProtocolExtensionContainer { {RAT-RestrictionsItem-ExtIEs} } OPTIONAL,

...

}

RAT-RestrictionsItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ ID id-ExtendedRATRestrictionInformation	CRITICALITY ignore	EXTENSION ExtendedRATRestrictionInformation		PRESENCE optional},

...

}

RAT-RestrictionInformation ::= BIT STRING {e-UTRA (0),nR (1), nR-unlicensed (2), nR-LEO (3), nR-MEO (4), nR-GEO (5), nR-OTHERSAT (6)} (SIZE(8, ...))

ForbiddenAreaList ::= SEQUENCE (SIZE(1..maxnoofPLMNs)) OF ForbiddenAreaItem

ForbiddenAreaItem ::= SEQUENCE {

plmn-Identity		PLMN-Identity,

forbidden-TACs		SEQUENCE (SIZE(1..maxnoofForbiddenTACs)) OF TAC,

iE-Extensions		ProtocolExtensionContainer { {ForbiddenAreaItem-ExtIEs} } OPTIONAL,

...

}

ForbiddenAreaItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

ServiceAreaList ::= SEQUENCE (SIZE(1..maxnoofPLMNs)) OF ServiceAreaItem

ServiceAreaItem ::= SEQUENCE {

plmn-Identity						PLMN-Identity,

allowed-TACs-ServiceArea			SEQUENCE (SIZE(1..maxnoofAllowedAreas)) OF TAC		OPTIONAL,

not-allowed-TACs-ServiceArea		SEQUENCE (SIZE(1..maxnoofAllowedAreas)) OF TAC		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServiceAreaItem-ExtIEs} }				OPTIONAL,

...

}

ServiceAreaItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

MR-DC-ResourceCoordinationInfo ::= SEQUENCE {

ng-RAN-Node-ResourceCoordinationInfo			NG-RAN-Node-ResourceCoordinationInfo,

iE-Extension											ProtocolExtensionContainer { {MR-DC-ResourceCoordinationInfo-ExtIEs}}	OPTIONAL,

...

}

MR-DC-ResourceCoordinationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NG-RAN-Node-ResourceCoordinationInfo ::= CHOICE {

eutra-resource-coordination-info					E-UTRA-ResourceCoordinationInfo,

nr-resource-coordination-info						NR-ResourceCoordinationInfo

}

E-UTRA-ResourceCoordinationInfo ::= SEQUENCE {

e-utra-cell												E-UTRA-CGI,

ul-coordination-info									BIT STRING (SIZE (6..4400)),

dl-coordination-info									BIT STRING (SIZE (6..4400))	OPTIONAL,

nr-cell													NR-CGI	OPTIONAL,

e-utra-coordination-assistance-info				E-UTRA-CoordinationAssistanceInfo	OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {E-UTRA-ResourceCoordinationInfo-ExtIEs} }	OPTIONAL,

...

}

E-UTRA-ResourceCoordinationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

E-UTRA-CoordinationAssistanceInfo ::= ENUMERATED {coordination-not-required, ...}

NR-ResourceCoordinationInfo ::= SEQUENCE {

nr-cell													NR-CGI,

ul-coordination-info									BIT STRING (SIZE (6..4400)),

dl-coordination-info									BIT STRING (SIZE (6..4400))	OPTIONAL,

e-utra-cell												E-UTRA-CGI	OPTIONAL,

nr-coordination-assistance-info					NR-CoordinationAssistanceInfo		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {NR-ResourceCoordinationInfo-ExtIEs} } 	OPTIONAL,

...

}

NR-ResourceCoordinationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NR-CoordinationAssistanceInfo ::= ENUMERATED {coordination-not-required, ...}

MessageOversizeNotification ::= SEQUENCE {

maximumCellListSize								MaximumCellListSize,

iE-Extension				ProtocolExtensionContainer { {MessageOversizeNotification-ExtIEs}}	OPTIONAL,

...

}

MessageOversizeNotification-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MaximumCellListSize ::= INTEGER(1..16384, ...)

MultiplexingInfo 	::=	SEQUENCE{

iAB-MT-Cell-List 	IAB-MT-Cell-List,

iE-Extensions		ProtocolExtensionContainer { {MultiplexingInfo-ExtIEs} } OPTIONAL,

...

}

MultiplexingInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- N

NACellResourceConfigurationList  ::=  SEQUENCE (SIZE(1..maxnoofHSNASlots)) OF NACellResourceConfiguration-Item

NACellResourceConfiguration-Item  ::=  SEQUENCE {

nAdownlin			ENUMERATED {true, false, ...}  OPTIONAL,

nAuplink 			ENUMERATED {true, false, ...}  OPTIONAL,

nAflexible			ENUMERATED {true, false, ...}  OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { NACellResourceConfiguration-Item-ExtIEs} } OPTIONAL,

...

}

NACellResourceConfiguration-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NBIoT-UL-DL-AlignmentOffset ::= ENUMERATED {

khz-7dot5,

khz0,

khz7dot5,

...

}

NE-DC-TDM-Pattern ::= SEQUENCE {

subframeAssignment			ENUMERATED {sa0,sa1,sa2,sa3,sa4,sa5,sa6},

harqOffset					INTEGER (0..9),

iE-Extension				ProtocolExtensionContainer { {NE-DC-TDM-Pattern-ExtIEs}}	OPTIONAL,

...

}

NE-DC-TDM-Pattern-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NeighbourInformation-E-UTRA ::= SEQUENCE (SIZE(1..maxnoofNeighbours)) OF NeighbourInformation-E-UTRA-Item

NeighbourInformation-E-UTRA-Item ::= SEQUENCE {

e-utra-PCI			E-UTRAPCI,

e-utra-cgi			E-UTRA-CGI,

earfcn				E-UTRAARFCN,

tac					TAC,

ranac				RANAC																		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NeighbourInformation-E-UTRA-Item-ExtIEs} } 	OPTIONAL,

...

}

NeighbourInformation-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

NeighbourInformation-NR ::= SEQUENCE (SIZE(1..maxnoofNeighbours)) OF NeighbourInformation-NR-Item

NeighbourInformation-NR-Item ::= SEQUENCE {

nr-PCI								NRPCI,

nr-cgi								NR-CGI,

tac									TAC,

ranac								RANAC														OPTIONAL,

nr-mode-info						NeighbourInformation-NR-ModeInfo,

connectivitySupport					Connectivity-Support,

measurementTimingConfiguration		OCTET STRING,

iE-Extensions			ProtocolExtensionContainer { {NeighbourInformation-NR-Item-ExtIEs} } 	OPTIONAL,

...

}

NeighbourInformation-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

NeighbourInformation-NR-ModeInfo ::= CHOICE {

fdd-info				NeighbourInformation-NR-ModeFDDInfo,

tdd-info				NeighbourInformation-NR-ModeTDDInfo,

choice-extension		ProtocolIE-Single-Container { {NeighbourInformation-NR-ModeInfo-ExtIEs} }

}

NeighbourInformation-NR-ModeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NeighbourInformation-NR-ModeFDDInfo ::= SEQUENCE {

ul-NR-FreqInfo		NRFrequencyInfo,

dl-NR-FequInfo		NRFrequencyInfo,

ie-Extensions		ProtocolExtensionContainer { {NeighbourInformation-NR-ModeFDDInfo-ExtIEs} } OPTIONAL,

...

}

NeighbourInformation-NR-ModeFDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NeighbourInformation-NR-ModeTDDInfo ::= SEQUENCE {

nr-FreqInfo			NRFrequencyInfo,

ie-Extensions		ProtocolExtensionContainer { {NeighbourInformation-NR-ModeTDDInfo-ExtIEs} } OPTIONAL,

...

}

NeighbourInformation-NR-ModeTDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Neighbour-NG-RAN-Node-List ::= SEQUENCE (SIZE(0..maxnoofNeighbour-NG-RAN-Nodes)) OF Neighbour-NG-RAN-Node-Item

Neighbour-NG-RAN-Node-Item ::= SEQUENCE {

globalNG-RANNodeID				GlobalNG-RANNode-ID,

local-NG-RAN-Node-Identifier	Local-NG-RAN-Node-Identifier,

ie-Extensions		ProtocolExtensionContainer { {Neighbour-NG-RAN-Node-Item-ExtIEs} } OPTIONAL,

...

}

Neighbour-NG-RAN-Node-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NID	::= BIT STRING (SIZE(44))

NRCarrierList ::= SEQUENCE (SIZE(1..maxnoofNRSCSs)) OF NRCarrierItem

NRCarrierItem ::= SEQUENCE {

carrierSCS					NRSCS,

offsetToCarrier				INTEGER (0..2199, ...),

carrierBandwidth			INTEGER (0..maxnoofPhysicalResourceBlocks, ...),

iE-Extension		ProtocolExtensionContainer { {NRCarrierItem-ExtIEs} } 		OPTIONAL,

...

}

NRCarrierItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRCellPRACHConfig ::= OCTET STRING

NG-RAN-Cell-Identity ::= CHOICE {

nr						NR-Cell-Identity,

e-utra					E-UTRA-Cell-Identity,

choice-extension		ProtocolIE-Single-Container { {NG-RAN-Cell-Identity-ExtIEs} }

}

NG-RAN-Cell-Identity-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NG-RAN-CellPCI ::= CHOICE {

nr					NRPCI,

e-utra				E-UTRAPCI,

choice-extension	ProtocolIE-Single-Container { {NG-RAN-CellPCI-ExtIEs} }

}

NG-RAN-CellPCI-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NG-RANnode2SSBOffsetsModificationRange ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBOffsetModificationRange

NG-RANnodeUEXnAPID ::= INTEGER (0.. 4294967295)

NumberofActiveUEs::= INTEGER(0..16777215, ...)

NoofRRCConnections ::= INTEGER (1..65536,...)

NonDynamic5QIDescriptor ::= SEQUENCE {

fiveQI						FiveQI,

priorityLevelQoS			PriorityLevelQoS													OPTIONAL,

averagingWindow				AveragingWindow														OPTIONAL,

maximumDataBurstVolume		MaximumDataBurstVolume 												OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {NonDynamic5QIDescriptor-ExtIEs } }	OPTIONAL,

...

}

NonDynamic5QIDescriptor-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-CNPacketDelayBudgetDownlink		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget	PRESENCE optional}|

{ ID id-CNPacketDelayBudgetUplink		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget	PRESENCE optional},

...

}

NRARFCN	::= INTEGER (0.. maxNRARFCN)

NG-eNB-RadioResourceStatus	::= SEQUENCE {

dL-GBR-PRB-usage							DL-GBR-PRB-usage,

uL-GBR-PRB-usage							UL-GBR-PRB-usage,

dL-non-GBR-PRB-usage						DL-non-GBR-PRB-usage,

uL-non-GBR-PRB-usage						UL-non-GBR-PRB-usage,

dL-Total-PRB-usage						DL-Total-PRB-usage,

uL-Total-PRB-usage						UL-Total-PRB-usage,

iE-Extensions							ProtocolExtensionContainer { { NG-eNB-RadioResourceStatus-ExtIEs} } OPTIONAL,

...

}

NG-eNB-RadioResourceStatus-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DL-scheduling-PDCCH-CCE-usage		CRITICALITY ignore	EXTENSION DL-scheduling-PDCCH-CCE-usage	PRESENCE optional}|

{ ID id-UL-scheduling-PDCCH-CCE-usage		CRITICALITY ignore	EXTENSION UL-scheduling-PDCCH-CCE-usage	PRESENCE optional},

...

}

DL-scheduling-PDCCH-CCE-usage ::= INTEGER (0.. 100)

UL-scheduling-PDCCH-CCE-usage ::= INTEGER (0.. 100)

TNLCapacityIndicator ::= SEQUENCE {

dLTNLOfferedCapacity					OfferedCapacity,

dLTNLAvailableCapacity					AvailableCapacity,

uLTNLOfferedCapacity					OfferedCapacity,

uLTNLAvailableCapacity					AvailableCapacity,

iE-Extensions							ProtocolExtensionContainer { { TNLCapacityIndicator-ExtIEs} } OPTIONAL,

...

}

TNLCapacityIndicator-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Non-F1-TerminatingTopologyBHInformation	::= SEQUENCE {

nonF1TerminatingBHInformation-List		NonF1TerminatingBHInformation-List,

bAPControlPDURLCCH-List					BAPControlPDURLCCH-List			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {Non-F1-TerminatingTopologyBHInformation-ExtIEs} }	OPTIONAL,

...

}

Non-F1-TerminatingTopologyBHInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NonF1TerminatingBHInformation-List ::= SEQUENCE (SIZE(1..maxnoofBHInfo)) OF NonF1TerminatingBHInformation-Item

NonF1TerminatingBHInformation-Item ::= SEQUENCE {

bHInfoIndex						BHInfoIndex,

dlNon-F1TerminatingBHInfo		DLNonF1Terminating-BHInfo		OPTIONAL,

ulNon-F1TerminatingBHInfo		ULNonF1Terminating-BHInfo		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { NonF1TerminatingBHInformation-Item-ExtIEs} }	OPTIONAL,

...

}

NonF1TerminatingBHInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NonUPTraffic ::= CHOICE {

nonUPTrafficType				NonUPTrafficType,

controlPlaneTrafficType			ControlPlaneTrafficType,

choice-extension				ProtocolIE-Single-Container { { NonUPTraffic-ExtIEs} }

}

NonUPTraffic-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NonUPTrafficType ::= ENUMERATED {ueassociatedf1ap, nonueassociatedf1ap, nonf1, ...}

NoPDUSessionIndication	::= ENUMERATED {true, ...}

NPN-Broadcast-Information ::= CHOICE {

snpn-Information					NPN-Broadcast-Information-SNPN,

pni-npn-Information					NPN-Broadcast-Information-PNI-NPN,

choice-extension					ProtocolIE-Single-Container { {NPN-Broadcast-Information-ExtIEs} }

}

NPN-Broadcast-Information-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NPN-Broadcast-Information-SNPN ::= SEQUENCE {

broadcastSNPNID-List		BroadcastSNPNID-List,

iE-Extension				ProtocolExtensionContainer { {NPN-Broadcast-Information-SNPN-ExtIEs} }	OPTIONAL,

...

}

NPN-Broadcast-Information-SNPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPN-Broadcast-Information-PNI-NPN ::= SEQUENCE {

broadcastPNI-NPN-ID-Information		BroadcastPNI-NPN-ID-Information,

iE-Extension						ProtocolExtensionContainer { {NPN-Broadcast-Information-PNI-NPN-ExtIEs} }	OPTIONAL,

...

}

NPN-Broadcast-Information-PNI-NPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPNMobilityInformation::= CHOICE {

snpn-mobility-information			NPNMobilityInformation-SNPN,

pni-npn-mobility-information		NPNMobilityInformation-PNI-NPN,

choice-extension					ProtocolIE-Single-Container { {NPNMobilityInformation-ExtIEs} }

}

NPNMobilityInformation-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NPNMobilityInformation-SNPN ::= SEQUENCE {

serving-NID					NID,

iE-Extension				ProtocolExtensionContainer { {NPNMobilityInformation-SNPN-ExtIEs} }	OPTIONAL,

...

}

NPNMobilityInformation-SNPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPNMobilityInformation-PNI-NPN ::= SEQUENCE {

allowedPNI-NPN-ID-List		AllowedPNI-NPN-ID-List,

iE-Extension				ProtocolExtensionContainer { {NPNMobilityInformation-PNI-NPN-ExtIEs} }	OPTIONAL,

...

}

NPNMobilityInformation-PNI-NPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPNPagingAssistanceInformation ::= CHOICE {

pni-npn-Information					NPNPagingAssistanceInformation-PNI-NPN,

choice-extension					ProtocolIE-Single-Container { {NPNPagingAssistanceInformation-ExtIEs} }

}

NPNPagingAssistanceInformation-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NPNPagingAssistanceInformation-PNI-NPN ::= SEQUENCE {

allowedPNI-NPN-ID-List			AllowedPNI-NPN-ID-List,

iE-Extension					ProtocolExtensionContainer { {NPNPagingAssistanceInformation-PNI-NPN-ExtIEs} }	OPTIONAL,

...

}

NPNPagingAssistanceInformation-PNI-NPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPN-Support ::= CHOICE {

sNPN					NPN-Support-SNPN,

choice-Extensions		ProtocolIE-Single-Container { {NPN-Support-ExtIEs} }

}

NPN-Support-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NPN-Support-SNPN ::= SEQUENCE {

nid					NID,

ie-Extension		ProtocolExtensionContainer { {NPN-Support-SNPN-ExtIEs} }	OPTIONAL,

...

}

NPN-Support-SNPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPRACHConfiguration::= SEQUENCE {

fdd-or-tdd				CHOICE {

fdd			NPRACHConfiguration-FDD,

tdd			NPRACHConfiguration-TDD,

choice-extension		ProtocolIE-Single-Container { { FDD-or-TDD-in-NPRACHConfiguration-Choice-ExtIEs} }

},

iE-Extensions						ProtocolExtensionContainer { { NPRACHConfiguration-ExtIEs} }	OPTIONAL,

...

}

NPRACHConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

FDD-or-TDD-in-NPRACHConfiguration-Choice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NPRACHConfiguration-FDD::= SEQUENCE {

nprach-CP-length						NPRACH-CP-Length,

anchorCarrier-NPRACHConfig						OCTET STRING,

anchorCarrier-EDT-NPRACHConfig 					OCTET STRING 						OPTIONAL,

anchorCarrier-Format2-NPRACHConfig				OCTET STRING						OPTIONAL,

anchorCarrier-Format2-EDT-NPRACHConfig			OCTET STRING 						OPTIONAL,

non-anchorCarrier-NPRACHConfig					OCTET STRING 						OPTIONAL,

non-anchorCarrier-Format2-NPRACHConfig			OCTET STRING 						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { NPRACHConfiguration-FDD-ExtIEs} }	OPTIONAL,

...

}

NPRACHConfiguration-FDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPRACHConfiguration-TDD::= SEQUENCE {

nprach-preambleFormat							NPRACH-preambleFormat,

anchorCarrier-NPRACHConfigTDD					OCTET STRING,

non-anchorCarrierFequencyConfiglist				Non-AnchorCarrierFrequencylist 		OPTIONAL,

non-anchorCarrier-NPRACHConfigTDD				OCTET STRING 						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { NPRACHConfiguration-TDD-ExtIEs} }	OPTIONAL,

...

}

NPRACHConfiguration-TDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPRACH-CP-Length::=		ENUMERATED {

us66dot7,

us266dot7,

...

}

NPRACH-preambleFormat::= 	ENUMERATED {fmt0,fmt1,fmt2,fmt0a,fmt1a,...}

Non-AnchorCarrierFrequencylist ::= SEQUENCE (SIZE(1..maxnoofNonAnchorCarrierFreqConfig)) OF

SEQUENCE {

non-anchorCarrierFrquency		OCTET STRING,

iE-Extensions					ProtocolExtensionContainer { { Non-AnchorCarrierFrequencylist-ExtIEs} } OPTIONAL,

...

}

Non-AnchorCarrierFrequencylist-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NR-Cell-Identity		::= BIT STRING (SIZE (36))

NG-RAN-Cell-Identity-ListinRANPagingArea ::= SEQUENCE (SIZE (1..maxnoofCellsinRNA)) OF NG-RAN-Cell-Identity

NR-CGI ::= SEQUENCE {

plmn-id				PLMN-Identity,

nr-CI				NR-Cell-Identity,

iE-Extension		ProtocolExtensionContainer { {NR-CGI-ExtIEs} } 	OPTIONAL,

...

}

NR-CGI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NR-U-Channel-List ::= SEQUENCE (SIZE (1..maxnoofNR-UChannelIDs)) OF NR-U-Channel-Item

NR-U-Channel-Item ::= SEQUENCE {

nR-U-ChannelID						NR-U-ChannelID,

channelOccupancyTimePercentageDL	ChannelOccupancyTimePercentage,

energyDetectionThreshold			EnergyDetectionThreshold,

iE-Extension		ProtocolExtensionContainer { {NR-U-Channel-Item-ExtIEs} } 	OPTIONAL,

...

}

NR-U-Channel-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NR-U-ChannelID ::= INTEGER (1..maxnoofNR-UChannelIDs, ...)

ChannelOccupancyTimePercentage ::= INTEGER (0..100,...)

EnergyDetectionThreshold ::= INTEGER (-100..-50, ...)

NR-U-ChannelInfo-List ::= SEQUENCE (SIZE (1..maxnoofNR-UChannelIDs)) OF NR-U-ChannelInfo-Item

NR-U-ChannelInfo-Item ::= SEQUENCE {

nR-U-ChannelID	NR-U-ChannelID,

nRARFCN			NRARFCN,

bandwidth		Bandwidth,

iE-Extension		ProtocolExtensionContainer { {NR-U-ChannelInfo-Item-ExtIEs} } 	OPTIONAL,

...

}

NR-U-ChannelInfo-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Bandwidth ::= ENUMERATED{mhz10, mhz20, mhz40, mhz60, mhz80, ...}

NRCyclicPrefix ::= ENUMERATED {normal, extended, ...}

NRDL-ULTransmissionPeriodicity ::= ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms3, ms4, ms5, ms10, ms20, ms40, ms60, ms80, ms100, ms120, ms140, ms160, ...}

NRFrequencyBand ::= INTEGER (1..1024, ...)

NRFrequencyBand-List ::= SEQUENCE (SIZE(1..maxnoofNRCellBands)) OF NRFrequencyBandItem

NRFrequencyBandItem ::= SEQUENCE {

nr-frequency-band			NRFrequencyBand,

supported-SUL-Band-List		SupportedSULBandList											OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {NRFrequencyBandItem-ExtIEs} } 	OPTIONAL,

...

}

NRFrequencyBandItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRFrequencyInfo ::= SEQUENCE {

nrARFCN				NRARFCN,

sul-information		SUL-Information												OPTIONAL,

frequencyBand-List		NRFrequencyBand-List,

iE-Extension		ProtocolExtensionContainer { {NRFrequencyInfo-ExtIEs} }		OPTIONAL,

...

}

NRFrequencyInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-FrequencyShift7p5khz	CRITICALITY ignore	EXTENSION FrequencyShift7p5khz	PRESENCE optional },...

}

NRMobilityHistoryReport ::= OCTET STRING

NRModeInfo ::= CHOICE {

fdd							NRModeInfoFDD,

tdd							NRModeInfoTDD,

choice-extension			ProtocolIE-Single-Container { {NRModeInfo-ExtIEs} }

}

NRModeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NRModeInfoFDD ::= SEQUENCE {

ulNRFrequencyInfo			NRFrequencyInfo,

dlNRFrequencyInfo			NRFrequencyInfo,

ulNRTransmissonBandwidth	NRTransmissionBandwidth,

dlNRTransmissonBandwidth	NRTransmissionBandwidth,

iE-Extension		ProtocolExtensionContainer { {NRModeInfoFDD-ExtIEs} } 	OPTIONAL,

...

}

NRModeInfoFDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-ULCarrierList			CRITICALITY ignore	EXTENSION NRCarrierList			PRESENCE optional }|

{ ID id-DLCarrierList			CRITICALITY ignore	EXTENSION NRCarrierList			PRESENCE optional }|

{ ID id-UL-GNB-DU-Cell-Resource-Configuration	CRITICALITY ignore	EXTENSION GNB-DU-Cell-Resource-Configuration		PRESENCE optional }|

{ ID id-DL-GNB-DU-Cell-Resource-Configuration	CRITICALITY ignore	EXTENSION GNB-DU-Cell-Resource-Configuration		PRESENCE optional },

...

}

NRModeInfoTDD ::= SEQUENCE {

nrFrequencyInfo			NRFrequencyInfo,

nrTransmissonBandwidth	NRTransmissionBandwidth,

iE-Extension			ProtocolExtensionContainer { {NRModeInfoTDD-ExtIEs} } 	OPTIONAL,

...

}

NRModeInfoTDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-IntendedTDD-DL-ULConfiguration-NR	CRITICALITY ignore	EXTENSION IntendedTDD-DL-ULConfiguration-NR	PRESENCE optional }|

{ID id-TDDULDLConfigurationCommonNR			CRITICALITY ignore	EXTENSION TDDULDLConfigurationCommonNR			PRESENCE optional }|

{ ID id-CarrierList				CRITICALITY ignore	EXTENSION NRCarrierList			PRESENCE optional }|

{ID id-tdd-GNB-DU-Cell-Resource-Configuration	CRITICALITY ignore	EXTENSION GNB-DU-Cell-Resource-Configuration	PRESENCE optional },

...

}

NRNRB ::= ENUMERATED { nrb11, nrb18, nrb24, nrb25, nrb31, nrb32, nrb38, nrb51, nrb52, nrb65, nrb66, nrb78, nrb79, nrb93, nrb106, nrb107, nrb121, nrb132, nrb133, nrb135, nrb160, nrb162, nrb189, nrb216, nrb217, nrb245, nrb264, nrb270, nrb273, ..., nrb33, nrb62, nrb124, nrb148, nrb248}

NRPagingeDRXInformation ::= SEQUENCE {

nRPaging-eDRX-Cycle		NRPaging-eDRX-Cycle,

nRPaging-Time-Window	NRPaging-Time-Window					OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {NRPagingeDRXInformation-ExtIEs} }	OPTIONAL,

...

}

NRPagingeDRXInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRPaging-eDRX-Cycle ::= ENUMERATED {

hfquarter, hfhalf, hf1, hf2, hf4,

hf8, hf16,

hf32, hf64, hf128, hf256,

hf512, hf1024,

...

}

NRPaging-Time-Window ::= ENUMERATED {

s1, s2, s3, s4, s5,

s6, s7, s8, s9, s10,

s11, s12, s13, s14, s15, s16,

...,s17, s18, s19, s20, s21, s22,

s23, s24, s25, s26, s27, s28, s29,

s30, s31, s32

}

NRPagingeDRXInformationforRRCINACTIVE ::= SEQUENCE {

nRPaging-eDRX-Cycle-Inactive		NRPaging-eDRX-Cycle-Inactive,

iE-Extensions						ProtocolExtensionContainer { { NRPagingeDRXInformationforRRCINACTIVE-ExtIEs} }	OPTIONAL,

...

}

NRPagingeDRXInformationforRRCINACTIVE-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRPaging-eDRX-Cycle-Inactive ::= ENUMERATED {

hfquarter, hfhalf, hf1,

...

}

NRPCI ::= INTEGER (0..1007, ...)

NRSCS ::= ENUMERATED { scs15, scs30, scs60, scs120, ..., scs480, scs960}

NRTransmissionBandwidth	::= SEQUENCE {

nRSCS	NRSCS,

nRNRB	NRNRB,

iE-Extensions				ProtocolExtensionContainer { {NRTransmissionBandwidth-ExtIEs} } OPTIONAL,

...

}

NRTransmissionBandwidth-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NumberOfAntennaPorts-E-UTRA ::= ENUMERATED {an1, an2, an4, ...}

NG-RANTraceID 				::=OCTET STRING (SIZE (8))

NonGBRResources-Offered ::= ENUMERATED {true, ...}

NRV2XServicesAuthorized ::= SEQUENCE {

vehicleUE			VehicleUE											OPTIONAL,

pedestrianUE 		PedestrianUE						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NRV2XServicesAuthorized-ExtIEs} }	OPTIONAL,

...

}

NRV2XServicesAuthorized-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRUESidelinkAggregateMaximumBitRate ::= SEQUENCE {

uESidelinkAggregateMaximumBitRate		BitRate,

iE-Extensions					ProtocolExtensionContainer { {NRUESidelinkAggregateMaximumBitRate-ExtIEs} } OPTIONAL,

...

}

NRUESidelinkAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NSAG-ID ::= INTEGER (0..255, ...)

#### -- O

OfferedCapacity ::= INTEGER (1.. 16777216,...)

OffsetOfNbiotChannelNumberToEARFCN ::= ENUMERATED {

minusTen,

minusNine,

minusEightDotFive,

minusEight,

minusSeven,

minusSix,

minusFive,

minusFourDotFive,

minusFour,

minusThree,

minusTwo,

minusOne,

minusZeroDotFive,

zero,

one,

two,

three,

threeDotFive,

four,

five,

six,

seven,

sevenDotFive,

eight,

nine,

...

}

#### -- P

PositioningInformation ::= SEQUENCE {

requestedSRSTransmissionCharacteristics				RequestedSRSTransmissionCharacteristics,

routingID											RoutingID,

nRPPaTransactionID									INTEGER (0..32767),

iE-Extension 				ProtocolExtensionContainer { { PositioningInformation-ExtIEs} } OPTIONAL,

...

}

PositioningInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PacketDelayBudget ::= INTEGER (0..1023, ...)

PacketErrorRate ::= SEQUENCE {

pER-Scalar			PER-Scalar,

pER-Exponent		PER-Exponent,

iE-Extensions		ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} }	OPTIONAL,

...

}

PacketErrorRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PagingCause ::= ENUMERATED {

voice,

...

}

PedestrianUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

PER-Scalar ::= INTEGER (0..9, ...)

PER-Exponent ::= INTEGER (0..9, ...)

PEIPSassistanceInformation ::= SEQUENCE {

cNsubgroupID			CNsubgroupID,

iE-Extensions		ProtocolExtensionContainer { {PEIPSassistanceInformation-ExtIEs} } OPTIONAL,

...

}

PEIPSassistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PacketLossRate ::= INTEGER (0..1000, ...)

PagingDRX	::= ENUMERATED {

v32,

v64,

v128,

v256,

... ,

v512,

v1024

}

PagingPriority ::= ENUMERATED {

priolevel1,

priolevel2,

priolevel3,

priolevel4,

priolevel5,

priolevel6,

priolevel7,

priolevel8,

...

}

PartialListIndicator ::= ENUMERATED {partial, ...}

PC5QoSParameters ::= SEQUENCE {

pc5QoSFlowList				PC5QoSFlowList,

pc5LinkAggregateBitRates	BitRate				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { PC5QoSParameters-ExtIEs} }	OPTIONAL,

...

}

PC5QoSParameters-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PC5QoSFlowList ::= SEQUENCE (SIZE(1..maxnoofPC5QoSFlows)) OF PC5QoSFlowItem

-- The size of the PC5 QoS Flow List shall not exceed 2048 items.

PC5QoSFlowItem::= SEQUENCE {

pQI							FiveQI,

pc5FlowBitRates				PC5FlowBitRates				OPTIONAL,

range						Range						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { PC5QoSFlowItem-ExtIEs} }	OPTIONAL,

...

}

PC5QoSFlowItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PC5FlowBitRates ::= SEQUENCE {

guaranteedFlowBitRate		BitRate,

maximumFlowBitRate			BitRate,

iE-Extensions		ProtocolExtensionContainer { { PC5FlowBitRates-ExtIEs} }	OPTIONAL,

...

}

PC5FlowBitRates-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDCPChangeIndication ::= CHOICE {

from-S-NG-RAN-node				ENUMERATED {s-ng-ran-node-key-update-required, pdcp-data-recovery-required, ...},

from-M-NG-RAN-node				ENUMERATED {pdcp-data-recovery-required, ...},

choice-extension				ProtocolIE-Single-Container { {PDCPChangeIndication-ExtIEs} }

}

PDCPChangeIndication-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

PDCPDuplicationConfiguration ::= ENUMERATED {

configured,

de-configured,

...

}

PDCPSNLength ::= SEQUENCE {

ulPDCPSNLength			ENUMERATED {v12bits, v18bits, ...},

dlPDCPSNLength			ENUMERATED {v12bits, v18bits, ...},

iE-Extension			ProtocolExtensionContainer { {PDCPSNLength-ExtIEs} }		OPTIONAL,

...

}

PDCPSNLength-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionAggregateMaximumBitRate ::= SEQUENCE {

downlink-session-AMBR				BitRate,

uplink-session-AMBR					BitRate,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionAggregateMaximumBitRate-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSession-List ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF PDUSession-ID

PDUSession-List-withCause ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF PDUSession-List-withCause-Item

PDUSession-List-withCause-Item ::= SEQUENCE {

pduSessionId		PDUSession-ID,

cause				Cause				OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {PDUSession-List-withCause-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-List-withCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSession-List-withDataForwardingFromTarget ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF

PDUSession-List-withDataForwardingFromTarget-Item

PDUSession-List-withDataForwardingFromTarget-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

dataforwardinginfoTarget			DataForwardingInfoFromTargetNGRANnode,

iE-Extension		ProtocolExtensionContainer { {PDUSession-List-withDataForwardingFromTarget-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-List-withDataForwardingFromTarget-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DRB-IDs-takenintouse		CRITICALITY reject	EXTENSION DRB-List	PRESENCE optional},

...

}

PDUSession-List-withDataForwardingRequest ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF

PDUSession-List-withDataForwardingRequest-Item

PDUSession-List-withDataForwardingRequest-Item ::= SEQUENCE {

pduSessionId							PDUSession-ID,

dataforwardingInfofromSource			DataforwardingandOffloadingInfofromSource					OPTIONAL,

dRBtoBeReleasedList						DRBToQoSFlowMapping-List									OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {PDUSession-List-withDataForwardingRequest-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-List-withDataForwardingRequest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-Cause				CRITICALITY ignore EXTENSION Cause				PRESENCE optional},

...

}

-- **************************************************************

--

##### -- PDU Session related message level IEs BEGIN

--

-- **************************************************************

-- **************************************************************

--

###### -- PDU Session Resources Admitted List

--

-- **************************************************************

PDUSessionResourcesAdmitted-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourcesAdmitted-Item

PDUSessionResourcesAdmitted-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

pduSessionResourceAdmittedInfo		PDUSessionResourceAdmittedInfo,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionResourcesAdmitted-Item-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourcesAdmitted-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceAdmittedInfo ::= SEQUENCE {

dL-NG-U-TNL-Information-Unchanged		ENUMERATED {true, ...}														OPTIONAL,

qosFlowsAdmitted-List					QoSFlowsAdmitted-List,

qosFlowsNotAdmitted-List				QoSFlows-List-withCause														OPTIONAL,

dataForwardingInfoFromTarget			DataForwardingInfoFromTargetNGRANnode										OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionResourceAdmittedInfo-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceAdmittedInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecondarydataForwardingInfoFromTarget-List	CRITICALITY ignore	EXTENSION SecondarydataForwardingInfoFromTarget-List	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resources Not Admitted List

--

-- **************************************************************

PDUSessionResourcesNotAdmitted-List ::= SEQUENCE (SIZE (1..maxnoofPDUSessions)) OF PDUSessionResourcesNotAdmitted-Item

PDUSessionResourcesNotAdmitted-Item ::= SEQUENCE {

pduSessionId				PDUSession-ID,

cause						Cause				OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {PDUSessionResourcesNotAdmitted-Item-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourcesNotAdmitted-Item-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resources To Be Setup List

--

-- **************************************************************

PDUSessionResourcesToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourcesToBeSetup-Item

PDUSessionResourcesToBeSetup-Item ::= SEQUENCE {

pduSessionId					PDUSession-ID,

s-NSSAI							S-NSSAI,

pduSessionAMBR					PDUSessionAggregateMaximumBitRate													OPTIONAL,

uL-NG-U-TNLatUPF				UPTransportLayerInformation,

source-DL-NG-U-TNL-Information  UPTransportLayerInformation															OPTIONAL,

securityIndication				SecurityIndication																	OPTIONAL,

pduSessionType					PDUSessionType,

pduSessionNetworkInstance		PDUSessionNetworkInstance															OPTIONAL,

qosFlowsToBeSetup-List			QoSFlowsToBeSetup-List,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourcesToBeSetup-Item-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourcesToBeSetup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-UL-NG-U-TNLatUPF-List			CRITICALITY ignore	EXTENSION Additional-UL-NG-U-TNLatUPF-List 		PRESENCE optional}|

{ ID id-PDUSessionCommonNetworkInstance				CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ ID id-Redundant-UL-NG-U-TNLatUPF					CRITICALITY	ignore	EXTENSION UPTransportLayerInformation				PRESENCE optional}|

{ ID id-Additional-Redundant-UL-NG-U-TNLatUPF-List	CRITICALITY ignore	EXTENSION Additional-UL-NG-U-TNLatUPF-List  	PRESENCE optional}|

{ ID id-RedundantCommonNetworkInstance				CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ ID id-RedundantPDUSessionInformation				CRITICALITY ignore	EXTENSION RedundantPDUSessionInformation			PRESENCE optional}|

{ ID id-MBS-SessionAssociatedInformation			CRITICALITY ignore	EXTENSION MBS-SessionAssociatedInformation		PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Setup Info - SN terminated

--

-- **************************************************************

PDUSessionResourceSetupInfo-SNterminated ::= SEQUENCE {

uL-NG-U-TNLatUPF				UPTransportLayerInformation,

pduSessionType					PDUSessionType,

pduSessionNetworkInstance		PDUSessionNetworkInstance																	OPTIONAL,

qosFlowsToBeSetup-List			QoSFlowsToBeSetup-List-Setup-SNterminated,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource													OPTIONAL,

securityIndication				SecurityIndication																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupInfo-SNterminated-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecurityResult						CRITICALITY reject	EXTENSION SecurityResult							PRESENCE optional}|

{ ID id-PDUSessionCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ID id-DefaultDRB-Allowed					CRITICALITY ignore	EXTENSION DefaultDRB-Allowed						PRESENCE optional}|

{ ID id-SplitSessionIndicator				CRITICALITY reject	EXTENSION SplitSessionIndicator					PRESENCE optional}|

{ID id-NonGBRResources-Offered				CRITICALITY ignore	EXTENSION NonGBRResources-Offered				PRESENCE optional}|

{ ID id-Redundant-UL-NG-U-TNLatUPF			CRITICALITY	ignore	EXTENSION UPTransportLayerInformation			PRESENCE optional}|

{ ID id-RedundantCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ ID id-RedundantPDUSessionInformation		CRITICALITY ignore	EXTENSION RedundantPDUSessionInformation		PRESENCE optional},

...

}

QoSFlowsToBeSetup-List-Setup-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsToBeSetup-List-Setup-SNterminated-Item

QoSFlowsToBeSetup-List-Setup-SNterminated-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

qosFlowLevelQoSParameters		QoSFlowLevelQoSParameters,

offeredGBRQoSFlowInfo			GBRQoSFlowInfo																						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {QoSFlowsToBeSetup-List-Setup-SNterminated-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlowsToBeSetup-List-Setup-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-TSCTrafficCharacteristics		CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics 	PRESENCE optional}|

{ ID id-RedundantQoSFlowIndicator		CRITICALITY ignore	EXTENSION RedundantQoSFlowIndicator	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Setup Response Info - SN terminated

--

-- **************************************************************

PDUSessionResourceSetupResponseInfo-SNterminated ::= SEQUENCE {

dL-NG-U-TNLatNG-RAN				UPTransportLayerInformation,

dRBsToBeSetup					DRBsToBeSetupList-SetupResponse-SNterminated 	OPTIONAL,

dataforwardinginfoTarget		DataForwardingInfoFromTargetNGRANnode			OPTIONAL,

qosFlowsNotAdmittedList			QoSFlows-List-withCause							OPTIONAL,

securityResult					SecurityResult									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupResponseInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceSetupResponseInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DRB-IDs-takenintouse						CRITICALITY reject	EXTENSION DRB-List	PRESENCE optional}|

{ ID id-Redundant-DL-NG-U-TNLatNG-RAN				CRITICALITY ignore	EXTENSION UPTransportLayerInformation		PRESENCE optional}|

{ ID id-UsedRSNInformation							CRITICALITY ignore	EXTENSION RedundantPDUSessionInformation	PRESENCE optional},

...

}

DRBsToBeSetupList-SetupResponse-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetupList-SetupResponse-SNterminated-Item

DRBsToBeSetupList-SetupResponse-SNterminated-Item ::= SEQUENCE {

drb-ID													DRB-ID,

sN-UL-PDCP-UP-TNLInfo									UPTransportParameters,

dRB-QoS													QoSFlowLevelQoSParameters,

pDCP-SNLength											PDCPSNLength						OPTIONAL,

rLC-Mode												RLCMode,

uL-Configuration									ULConfiguration						OPTIONAL,

secondary-SN-UL-PDCP-UP-TNLInfo							UPTransportParameters				OPTIONAL,

duplicationActivation									DuplicationActivation				OPTIONAL,

qoSFlowsMappedtoDRB-SetupResponse-SNterminated			QoSFlowsMappedtoDRB-SetupResponse-SNterminated,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeSetupList-SetupResponse-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeSetupList-SetupResponse-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-RLCDuplicationInformation					CRITICALITY ignore	EXTENSION RLCDuplicationInformation	PRESENCE optional},

...

}

QoSFlowsMappedtoDRB-SetupResponse-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF

QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item

QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier				QoSFlowIdentifier,

mCGRequestedGBRQoSFlowInfo		GBRQoSFlowInfo														OPTIONAL,

qosFlowMappingIndication		QoSFlowMappingIndication											OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex				CRITICALITY ignore	EXTENSION QoSParaSetIndex				PRESENCE optional }|

{ ID id-SourceDLForwardingIPAddress			CRITICALITY ignore	EXTENSION TransportLayerAddress		PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Setup Info - MN terminated

--

-- **************************************************************

PDUSessionResourceSetupInfo-MNterminated ::= SEQUENCE {

pduSessionType					PDUSessionType,

dRBsToBeSetup					DRBsToBeSetupList-Setup-MNterminated,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceSetupInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeSetupList-Setup-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetupList-Setup-MNterminated-Item

DRBsToBeSetupList-Setup-MNterminated-Item ::= SEQUENCE {

drb-ID													DRB-ID,

mN-UL-PDCP-UP-TNLInfo									UPTransportParameters,

rLC-Mode												RLCMode,

uL-Configuration										ULConfiguration						OPTIONAL,

dRB-QoS													QoSFlowLevelQoSParameters,

pDCP-SNLength											PDCPSNLength						OPTIONAL,

secondary-MN-UL-PDCP-UP-TNLInfo							UPTransportParameters				OPTIONAL,

duplicationActivation									DuplicationActivation				OPTIONAL,

qoSFlowsMappedtoDRB-Setup-MNterminated		QoSFlowsMappedtoDRB-Setup-MNterminated,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeSetupList-Setup-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeSetupList-Setup-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-RLCDuplicationInformation					CRITICALITY ignore	EXTENSION RLCDuplicationInformation	PRESENCE optional},

...

}

QoSFlowsMappedtoDRB-Setup-MNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsMappedtoDRB-Setup-MNterminated-Item

QoSFlowsMappedtoDRB-Setup-MNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier				QoSFlowIdentifier,

qoSFlowLevelQoSParameters		QoSFlowLevelQoSParameters,

qosFlowMappingIndication		QoSFlowMappingIndication		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowsMappedtoDRB-Setup-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsMappedtoDRB-Setup-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-TSCTrafficCharacteristics		CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics 	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Setup Response Info - MN terminated

--

-- **************************************************************

PDUSessionResourceSetupResponseInfo-MNterminated ::= SEQUENCE {

dRBsAdmittedList				DRBsAdmittedList-SetupResponse-MNterminated,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupResponseInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceSetupResponseInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-DRBsNotAdmittedSetupModifyList	CRITICALITY ignore	EXTENSION DRB-List-withCause		PRESENCE optional},

...

}

DRBsAdmittedList-SetupResponse-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsAdmittedList-SetupResponse-MNterminated-Item

DRBsAdmittedList-SetupResponse-MNterminated-Item ::= SEQUENCE {

drb-ID									DRB-ID,

sN-DL-SCG-UP-TNLInfo					UPTransportParameters,

secondary-SN-DL-SCG-UP-TNLInfo			UPTransportParameters				OPTIONAL,

lCID									LCID								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsAdmittedList-SetupResponse-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsAdmittedList-SetupResponse-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated	CRITICALITY ignore	EXTENSION	QoSFlowsMappedtoDRB-SetupResponse-MNterminated	PRESENCE optional},

...

}

QoSFlowsMappedtoDRB-SetupResponse-MNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsMappedtoDRB-SetupResponse-MNterminated-Item

QoSFlowsMappedtoDRB-SetupResponse-MNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier				QoSFlowIdentifier,

currentQoSParaSetIndex			QoSParaSetIndex,

iE-Extensions					ProtocolExtensionContainer { {QoSFlowsMappedtoDRB-SetupResponse-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsMappedtoDRB-SetupResponse-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Info - SN terminated

--

-- **************************************************************

PDUSessionResourceModificationInfo-SNterminated ::= SEQUENCE {

uL-NG-U-TNLatUPF				UPTransportLayerInformation						OPTIONAL,

pduSessionNetworkInstance		PDUSessionNetworkInstance						OPTIONAL,

qosFlowsToBeSetup-List			QoSFlowsToBeSetup-List-Setup-SNterminated		OPTIONAL,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource		OPTIONAL,

qosFlowsToBeModified-List		QoSFlowsToBeSetup-List-Modified-SNterminated	OPTIONAL,

qoSFlowsToBeReleased-List		QoSFlows-List-withCause							OPTIONAL,

drbsToBeModifiedList			DRBsToBeModified-List-Modified-SNterminated		OPTIONAL,

dRBsToBeReleased				DRB-List-withCause								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModificationInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModificationInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-PDUSessionCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ID id-DefaultDRB-Allowed					CRITICALITY ignore	EXTENSION DefaultDRB-Allowed						PRESENCE optional}|

{ID id-NonGBRResources-Offered				CRITICALITY ignore	EXTENSION NonGBRResources-Offered				PRESENCE optional}|

{ID id-Redundant-UL-NG-U-TNLatUPF			CRITICALITY ignore	EXTENSION UPTransportLayerInformation			PRESENCE optional}|

{ID id-RedundantCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ID id-SecurityIndication					CRITICALITY ignore	EXTENSION SecurityIndication						PRESENCE optional},

...

}

QoSFlowsToBeSetup-List-Modified-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsToBeSetup-List-Modified-SNterminated-Item

QoSFlowsToBeSetup-List-Modified-SNterminated-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

qosFlowLevelQoSParameters		QoSFlowLevelQoSParameters							OPTIONAL,

offeredGBRQoSFlowInfo			GBRQoSFlowInfo										OPTIONAL,

qosFlowMappingIndication		QoSFlowMappingIndication 							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {QoSFlowsToBeSetup-List-Modified-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsToBeSetup-List-Modified-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-TSCTrafficCharacteristics		CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics 	PRESENCE optional}|

{ ID id-RedundantQoSFlowIndicator		CRITICALITY ignore	EXTENSION RedundantQoSFlowIndicator	PRESENCE optional},

...

}

DRBsToBeModified-List-Modified-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeModified-List-Modified-SNterminated-Item

DRBsToBeModified-List-Modified-SNterminated-Item ::= SEQUENCE {

drb-ID									DRB-ID,

mN-DL-SCG-UP-TNLInfo					UPTransportParameters		OPTIONAL,

secondary-MN-DL-SCG-UP-TNLInfo			UPTransportParameters		OPTIONAL,

lCID									LCID						OPTIONAL,

rlc-status								RLC-Status					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeModified-List-Modified-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModified-List-Modified-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Response Info - SN terminated

--

-- **************************************************************

PDUSessionResourceModificationResponseInfo-SNterminated ::= SEQUENCE {

dL-NG-U-TNLatNG-RAN				UPTransportLayerInformation								OPTIONAL,

dRBsToBeSetup					DRBsToBeSetupList-SetupResponse-SNterminated			OPTIONAL,

dataforwardinginfoTarget		DataForwardingInfoFromTargetNGRANnode					OPTIONAL,

dRBsToBeModified				DRBsToBeModifiedList-ModificationResponse-SNterminated	OPTIONAL,

dRBsToBeReleased				DRB-List-withCause										OPTIONAL,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource				OPTIONAL,

qosFlowsNotAdmittedTBAdded		QoSFlows-List-withCause									OPTIONAL,

qosFlowsReleased				QoSFlows-List-withCause									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModificationResponseInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModificationResponseInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DRB-IDs-takenintouse			CRITICALITY reject	EXTENSION DRB-List								PRESENCE optional}|

{ ID id-Redundant-DL-NG-U-TNLatNG-RAN	CRITICALITY ignore	EXTENSION UPTransportLayerInformation		PRESENCE optional}|

{ ID id-SecurityResult					CRITICALITY ignore	EXTENSION SecurityResult						PRESENCE optional},

...

}

DRBsToBeModifiedList-ModificationResponse-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF

DRBsToBeModifiedList-ModificationResponse-SNterminated-Item

DRBsToBeModifiedList-ModificationResponse-SNterminated-Item ::= SEQUENCE {

drb-ID													DRB-ID,

sN-UL-PDCP-UP-TNLInfo									UPTransportParameters									OPTIONAL,

dRB-QoS													QoSFlowLevelQoSParameters								OPTIONAL,

qoSFlowsMappedtoDRB-SetupResponse-SNterminated			QoSFlowsMappedtoDRB-SetupResponse-SNterminated		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {DRBsToBeModifiedList-ModificationResponse-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModifiedList-ModificationResponse-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-RLCDuplicationInformation					CRITICALITY ignore	EXTENSION RLCDuplicationInformation					PRESENCE optional}|

{ ID id-secondary-SN-UL-PDCP-UP-TNLInfo				CRITICALITY ignore	EXTENSION UPTransportParameters						PRESENCE optional}|

{ ID id-pdcpDuplicationConfiguration				CRITICALITY ignore	EXTENSION PDCPDuplicationConfiguration				PRESENCE optional}|

{ ID id-duplicationActivation						CRITICALITY ignore	EXTENSION DuplicationActivation						PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Info - MN terminated

--

-- **************************************************************

PDUSessionResourceModificationInfo-MNterminated ::= SEQUENCE {

pduSessionType					PDUSessionType,

dRBsToBeSetup					DRBsToBeSetupList-Setup-MNterminated						OPTIONAL,

dRBsToBeModified				DRBsToBeModifiedList-Modification-MNterminated				OPTIONAL,

dRBsToBeReleased				DRB-List-withCause											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModificationInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModificationInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeModifiedList-Modification-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF

DRBsToBeModifiedList-Modification-MNterminated-Item

DRBsToBeModifiedList-Modification-MNterminated-Item ::= SEQUENCE {

drb-ID													DRB-ID,

mN-UL-PDCP-UP-TNLInfo									UPTransportParameters						OPTIONAL,

dRB-QoS													QoSFlowLevelQoSParameters					OPTIONAL,

secondary-MN-UL-PDCP-UP-TNLInfo							UPTransportParameters						OPTIONAL,

uL-Configuration										ULConfiguration								OPTIONAL,

pdcpDuplicationConfiguration							PDCPDuplicationConfiguration 				OPTIONAL,

duplicationActivation									DuplicationActivation						OPTIONAL,

qoSFlowsMappedtoDRB-Setup-MNterminated					QoSFlowsMappedtoDRB-Setup-MNterminated	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeModifiedList-Modification-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModifiedList-Modification-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-RLCDuplicationInformation					CRITICALITY ignore	EXTENSION RLCDuplicationInformation	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Response Info - MN terminated

--

-- **************************************************************

PDUSessionResourceModificationResponseInfo-MNterminated ::= SEQUENCE {

dRBsAdmittedList						DRBsAdmittedList-ModificationResponse-MNterminated,

dRBsReleasedList						DRB-List																		OPTIONAL,

dRBsNotAdmittedSetupModifyList			DRB-List-withCause																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModificationResponseInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModificationResponseInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsAdmittedList-ModificationResponse-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsAdmittedList-ModificationResponse-MNterminated-Item

DRBsAdmittedList-ModificationResponse-MNterminated-Item ::= SEQUENCE {

drb-ID									DRB-ID,

sN-DL-SCG-UP-TNLInfo					UPTransportParameters				OPTIONAL,

secondary-SN-DL-SCG-UP-TNLInfo			UPTransportParameters				OPTIONAL,

lCID									LCID								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsAdmittedList-ModificationResponse-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsAdmittedList-ModificationResponse-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated	CRITICALITY ignore	EXTENSION	QoSFlowsMappedtoDRB-SetupResponse-MNterminated	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Change Required Info - SN terminated

--

-- **************************************************************

PDUSessionResourceChangeRequiredInfo-SNterminated ::= SEQUENCE {

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceChangeRequiredInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceChangeRequiredInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Change Confirm Info - SN terminated

--

-- **************************************************************

PDUSessionResourceChangeConfirmInfo-SNterminated ::= SEQUENCE {

dataforwardinginfoTarget		DataForwardingInfoFromTargetNGRANnode				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceChangeConfirmInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceChangeConfirmInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DRB-IDs-takenintouse	CRITICALITY reject	EXTENSION DRB-List	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Change Required Info - MN terminated

--

-- **************************************************************

PDUSessionResourceChangeRequiredInfo-MNterminated ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceChangeRequiredInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceChangeRequiredInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Change Confirm Info - MN terminated

--

-- **************************************************************

PDUSessionResourceChangeConfirmInfo-MNterminated ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceChangeConfirmInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceChangeConfirmInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Required Info - SN terminated

--

-- **************************************************************

PDUSessionResourceModRqdInfo-SNterminated ::= SEQUENCE {

dL-NG-U-TNLatNG-RAN				UPTransportLayerInformation						OPTIONAL,

qoSFlowsToBeReleased-List		QoSFlows-List-withCause							OPTIONAL,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource		OPTIONAL,

drbsToBeSetupList				DRBsToBeSetup-List-ModRqd-SNterminated			OPTIONAL,

drbsToBeModifiedList			DRBsToBeModified-List-ModRqd-SNterminated		OPTIONAL,

dRBsToBeReleased				DRB-List-withCause								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModRqdInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModRqdInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeSetup-List-ModRqd-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetup-List-ModRqd-SNterminated-Item

DRBsToBeSetup-List-ModRqd-SNterminated-Item ::= SEQUENCE {

drb-ID											DRB-ID,

pDCP-SNLength									PDCPSNLength											OPTIONAL,

sn-UL-PDCP-UPTNLinfo							UPTransportParameters,

dRB-QoS											QoSFlowLevelQoSParameters,

secondary-SN-UL-PDCP-UP-TNLInfo					UPTransportParameters								OPTIONAL,

duplicationActivation							DuplicationActivation								OPTIONAL,

uL-Configuration								ULConfiguration										OPTIONAL,

qoSFlowsMappedtoDRB-ModRqd-SNterminated			QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated,

rLC-Mode										RLCMode,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeSetup-List-ModRqd-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeSetup-List-ModRqd-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-RLCDuplicationInformation					CRITICALITY ignore	EXTENSION RLCDuplicationInformation					PRESENCE optional},

...

}

QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF

QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated-Item

QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier				QoSFlowIdentifier,

mCGRequestedGBRQoSFlowInfo		GBRQoSFlowInfo														OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeModified-List-ModRqd-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeModified-List-ModRqd-SNterminated-Item

DRBsToBeModified-List-ModRqd-SNterminated-Item ::= SEQUENCE {

drb-ID											DRB-ID,

sN-UL-PDCP-UP-TNLInfo							UPTransportParameters									OPTIONAL,

dRB-QoS											QoSFlowLevelQoSParameters								OPTIONAL,

secondary-SN-UL-PDCP-UP-TNLInfo					UPTransportParameters									OPTIONAL,

uL-Configuration								ULConfiguration											OPTIONAL,

pdcpDuplicationConfiguration					PDCPDuplicationConfiguration							OPTIONAL,

duplicationActivation							DuplicationActivation									OPTIONAL,

qoSFlowsMappedtoDRB-ModRqd-SNterminated		QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeModified-List-ModRqd-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModified-List-ModRqd-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-RLCDuplicationInformation					CRITICALITY ignore	EXTENSION RLCDuplicationInformation					PRESENCE optional},

...

}

QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF

QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated-Item

QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier					QoSFlowIdentifier,

mCGRequestedGBRQoSFlowInfo			GBRQoSFlowInfo														OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Confirm Info - SN terminated

--

-- **************************************************************

PDUSessionResourceModConfirmInfo-SNterminated ::= SEQUENCE {

uL-NG-U-TNLatUPF						UPTransportLayerInformation								OPTIONAL,

dRBsAdmittedList						DRBsAdmittedList-ModConfirm-SNterminated,

dRBsNotAdmittedSetupModifyList			DRB-List-withCause										OPTIONAL,

dataforwardinginfoTarget				DataForwardingInfoFromTargetNGRANnode					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModConfirmInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModConfirmInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DRB-IDs-takenintouse		CRITICALITY reject	EXTENSION DRB-List	PRESENCE optional},

...

}

DRBsAdmittedList-ModConfirm-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF

DRBsAdmittedList-ModConfirm-SNterminated-Item

DRBsAdmittedList-ModConfirm-SNterminated-Item ::= SEQUENCE {

drb-ID											DRB-ID,

mN-DL-CG-UP-TNLInfo								UPTransportParameters							OPTIONAL,

secondary-MN-DL-CG-UP-TNLInfo					UPTransportParameters							OPTIONAL,

lCID											LCID											OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {DRBsAdmittedList-ModConfirm-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsAdmittedList-ModConfirm-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Required Info - MN terminated

--

-- **************************************************************

PDUSessionResourceModRqdInfo-MNterminated ::= SEQUENCE {

dRBsToBeModified				DRBsToBeModified-List-ModRqd-MNterminated				OPTIONAL,

dRBsToBeReleased				DRB-List-withCause											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModRqdInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModRqdInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeModified-List-ModRqd-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeModified-List-ModRqd-MNterminated-Item

DRBsToBeModified-List-ModRqd-MNterminated-Item ::= SEQUENCE {

drb-ID								DRB-ID,

sN-DL-SCG-UP-TNLInfo				UPTransportLayerInformation,

secondary-SN-DL-SCG-UP-TNLInfo		UPTransportLayerInformation		OPTIONAL,

lCID								LCID							OPTIONAL,

rlc-status							RLC-Status						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeModified-List-ModRqd-MNterminated-Item-ExtIEs} } OPTIONAL,

...

}

DRBsToBeModified-List-ModRqd-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Confirm Info - MN terminated

--

-- **************************************************************

PDUSessionResourceModConfirmInfo-MNterminated ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModConfirmInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModConfirmInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- PDU Session Resource Setup Complete Info - SN terminated

--

-- **************************************************************

PDUSessionResourceBearerSetupCompleteInfo-SNterminated ::= SEQUENCE {

dRBsToBeSetupList 			SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetupList-BearerSetupComplete-SNterminated-Item,

iE-Extensions				ProtocolExtensionContainer { {PDUSessionResourceBearerSetupCompleteInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceBearerSetupCompleteInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeSetupList-BearerSetupComplete-SNterminated-Item ::= SEQUENCE {

dRB-ID						DRB-ID,

mN-Xn-U-TNLInfoatM			UPTransportLayerInformation,

iE-Extensions				ProtocolExtensionContainer { {DRBsToBeSetupList-BearerSetupComplete-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeSetupList-BearerSetupComplete-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-Secondary-MN-Xn-U-TNLInfoatM	CRITICALITY ignore	EXTENSION UPTransportLayerInformation		PRESENCE optional},

...

}

-- **************************************************************

--

##### -- PDU Session related message level IEs END

--

-- **************************************************************

PDUSessionResourceSecondaryRATUsageList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSecondaryRATUsageItem

PDUSessionResourceSecondaryRATUsageItem ::= SEQUENCE {

pDUSessionID								PDUSession-ID,

secondaryRATUsageInformation						SecondaryRATUsageInformation,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSecondaryRATUsageItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSecondaryRATUsageItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionUsageReport ::= SEQUENCE {

rATType								ENUMERATED {nr, eutra, ..., nr-unlicensed, e-utra-unlicensed},

pDUSessionTimedReportList			VolumeTimedReportList,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionUsageReport-ExtIEs} } OPTIONAL,

...

}

PDUSessionUsageReport-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionType ::= ENUMERATED {ipv4, ipv6, ipv4v6, ethernet, unstructured, ...}

PDUSession-ID	::= INTEGER (0..255)

PDUSessionNetworkInstance	::= INTEGER (1..256, ...)

PDUSessionCommonNetworkInstance	::= OCTET STRING

PDUSession-PairID	::= INTEGER (0..255, ...)

Periodical ::= SEQUENCE {

iE-Extensions		ProtocolExtensionContainer { { Periodical-ExtIEs} } OPTIONAL,

...

}

Periodical-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Permutation ::= ENUMERATED {dfu, ufd, ...}

PLMN-Identity ::= OCTET STRING (SIZE(3))

PLMNAreaBasedQMC ::= SEQUENCE {

plmnListforQMC		PLMNListforQMC,

iE-Extensions		ProtocolExtensionContainer { {PLMNAreaBasedQMC-ExtIEs} } OPTIONAL,

...

}

PLMNAreaBasedQMC-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PLMNListforQMC ::= SEQUENCE (SIZE(1..maxnoofPLMNforQMC)) OF PLMN-Identity

PCIListForMDT ::= SEQUENCE (SIZE(1.. maxnoofNeighPCIforMDT)) OF NRPCI

PNI-NPN-Restricted-Information ::= ENUMERATED { restriced, not-restricted, ...}

PortNumber ::= BIT STRING (SIZE (16))

PriorityLevelQoS ::= INTEGER (1..127, ...)

ProtectedE-UTRAResourceIndication ::= SEQUENCE {

activationSFN					ActivationSFN,

protectedResourceList			ProtectedE-UTRAResourceList,

mbsfnControlRegionLength		MBSFNControlRegionLength					OPTIONAL,

pDCCHRegionLength				INTEGER (1..3),

iE-Extensions					ProtocolExtensionContainer { {ProtectedE-UTRAResourceIndication-ExtIEs} } 	OPTIONAL,

...

}

ProtectedE-UTRAResourceIndication-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ProtectedE-UTRAResourceList ::= SEQUENCE (SIZE (1.. maxnoofProtectedResourcePatterns)) OF ProtectedE-UTRAResource-Item

ProtectedE-UTRAResource-Item ::= SEQUENCE {

resourceType							ENUMERATED {downlinknonCRS, cRS, uplink, ...},

intra-PRBProtectedResourceFootprint		BIT STRING (SIZE(84, ...)),

protectedFootprintFrequencyPattern		BIT STRING (SIZE(6..110, ...)),

protectedFootprintTimePattern			ProtectedE-UTRAFootprintTimePattern,

iE-Extensions					ProtocolExtensionContainer { {ProtectedE-UTRAResource-Item-ExtIEs} } 	OPTIONAL,

...

}

ProtectedE-UTRAResource-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ProtectedE-UTRAFootprintTimePattern ::= SEQUENCE {

protectedFootprintTimeperiodicity			INTEGER (1..320, ...),

protectedFootrpintStartTime					INTEGER (1..20, ...),

iE-Extensions					ProtocolExtensionContainer { {ProtectedE-UTRAFootprintTimePattern-ExtIEs} } 	OPTIONAL,

...

}

ProtectedE-UTRAFootprintTimePattern-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PrivacyIndicator ::= ENUMERATED {

immediate-MDT,

logged-MDT,

...

}

PSCellChangeHistory ::= ENUMERATED {reporting-full-history, ...}

PSCellHistoryInformationRetrieve ::= ENUMERATED {query, ...}

#### -- Q

QMCConfigInfo ::= SEQUENCE {

uEAppLayerMeasInfoList			UEAppLayerMeasInfoList,

iE-Extensions					ProtocolExtensionContainer { {QMCConfigInfo-ExtIEs} }	OPTIONAL,

...

}

QMCConfigInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEAppLayerMeasInfoList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF UEAppLayerMeasInfo-Item

UEAppLayerMeasInfo-Item ::= SEQUENCE {

uEAppLayerMeasConfigInfo	UEAppLayerMeasConfigInfo,

iE-Extensions				ProtocolExtensionContainer { { UEAppLayerMeasInfo-Item-ExtIEs} } 	OPTIONAL,

...

}

UEAppLayerMeasInfo-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QOEMeasConfAppLayerID ::= INTEGER (0..15, ...)

QOEMeasStatus ::= ENUMERATED {ongoing, ...}

QOEReference ::= OCTET STRING (SIZE (6))

QoSCharacteristics ::= CHOICE {

non-dynamic						NonDynamic5QIDescriptor,

dynamic							Dynamic5QIDescriptor,

choice-extension				ProtocolIE-Single-Container { {QoSCharacteristics-ExtIEs} }

}

QoSCharacteristics-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

QoSFlowIdentifier	::= INTEGER (0..63, ...)

QoSFlowLevelQoSParameters ::= SEQUENCE {

qos-characteristics			QoSCharacteristics,

allocationAndRetentionPrio	AllocationandRetentionPriority,

gBRQoSFlowInfo				GBRQoSFlowInfo														OPTIONAL,

reflectiveQoS				ReflectiveQoSAttribute												OPTIONAL,

additionalQoSflowInfo		ENUMERATED {more-likely, ...}										OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {QoSFlowLevelQoSParameters-ExtIEs} } OPTIONAL,

...

}

QoSFlowLevelQoSParameters-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-QoSMonitoringRequest				CRITICALITY ignore	EXTENSION QosMonitoringRequest					PRESENCE optional}|

{ID id-QosMonitoringReportingFrequency	CRITICALITY ignore	EXTENSION QosMonitoringReportingFrequency	PRESENCE optional}|

{ID id-QoSMonitoringDisabled				CRITICALITY ignore	EXTENSION QoSMonitoringDisabled					PRESENCE optional},

...

}

QoSFlowMappingIndication ::= ENUMERATED {

ul,

dl,

...

}

QoSFlowNotificationControlIndicationInfo ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlowNotify-Item

QoSFlowNotify-Item ::= SEQUENCE {

qosFlowIdentifier			QoSFlowIdentifier,

notificationInformation		ENUMERATED {fulfilled, not-fulfilled, ...},

iE-Extensions				ProtocolExtensionContainer { {QoSFlowNotificationControlIndicationInfo-ExtIEs} } OPTIONAL,

...

}

QoSFlowNotificationControlIndicationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{	ID id-CurrentQoSParaSetIndex	CRITICALITY ignore	EXTENSION QoSParaSetNotifyIndex		PRESENCE optional },

...

}

QoSFlows-List ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlow-Item

QoSFlow-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

qosFlowMappingIndication		QoSFlowMappingIndication					OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {QoSFlow-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlow-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlows-List-withCause ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlowwithCause-Item

QoSFlowwithCause-Item ::= SEQUENCE {

qfi					QoSFlowIdentifier,

cause				Cause					OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {QoSFlowwithCause-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlowwithCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Mapping-Information ::= SEQUENCE {

dscp							BIT STRING (SIZE(6))			OPTIONAL,

flow-label						BIT STRING (SIZE(20))		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {QoS-Mapping-Information-ExtIEs} }	OPTIONAL,

...

}

QoS-Mapping-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSParaSetIndex ::= INTEGER (1..8,...)

QoSParaSetNotifyIndex ::= INTEGER (0..8,...)

QoSFlowsAdmitted-List ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlowsAdmitted-Item

QoSFlowsAdmitted-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

iE-Extension		ProtocolExtensionContainer { {QoSFlowsAdmitted-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlowsAdmitted-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex	CRITICALITY ignore	EXTENSION QoSParaSetIndex	PRESENCE optional	},

...

}

QoSFlowsToBeSetup-List ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlowsToBeSetup-Item

QoSFlowsToBeSetup-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

qosFlowLevelQoSParameters		QoSFlowLevelQoSParameters,

e-RAB-ID						E-RAB-ID											OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {QoSFlowsToBeSetup-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlowsToBeSetup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-TSCTrafficCharacteristics		CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics 	PRESENCE optional}|

{ ID id-RedundantQoSFlowIndicator		CRITICALITY ignore	EXTENSION RedundantQoSFlowIndicator	PRESENCE optional},

...

}

QoSFlowsUsageReportList ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsUsageReport-Item

QoSFlowsUsageReport-Item ::= SEQUENCE {

qosFlowIdentifier					QoSFlowIdentifier,

rATType								ENUMERATED {nr, eutra, ..., nr-unlicensed, e-utra-unlicensed},

qoSFlowsTimedReportList				VolumeTimedReportList,

iE-Extensions						ProtocolExtensionContainer { {QoSFlowsUsageReport-Item-ExtIEs} } OPTIONAL,

...

}

QoSFlowsUsageReport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QosMonitoringRequest ::= ENUMERATED {ul, dl, both}

QoSMonitoringDisabled ::= ENUMERATED {true, ...}

QosMonitoringReportingFrequency ::= INTEGER (1..1800, ...)

#### -- R

RACH-Config-Common	::= OCTET STRING

RACH-Config-Common-IAB	::= OCTET STRING

RACHReportInformation	::= SEQUENCE (SIZE(1.. maxnoofRACHReports)) OF RACHReportList-Item

RACHReportList-Item	::= SEQUENCE {

rACHReport				RACHReportContainer,

iE-Extensions						ProtocolExtensionContainer { { RACHReportList-Item-ExtIEs} }	OPTIONAL,

...

}

RACHReportList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-UEAssistantIdentifier	CRITICALITY ignore	EXTENSION NG-RANnodeUEXnAPID 	PRESENCE optional},

...

}

RACHReportContainer	::= OCTET STRING

RadioResourceStatus	::= CHOICE {

ng-eNB-RadioResourceStatus	NG-eNB-RadioResourceStatus,

gNB-RadioResourceStatus		GNB-RadioResourceStatus,

choice-extension			ProtocolIE-Single-Container { { RadioResourceStatus-ExtIEs} }

}

RadioResourceStatus-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

RANAC ::= INTEGER (0..255)

RANAreaID ::= SEQUENCE {

tAC					TAC,

rANAC				RANAC												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {RANAreaID-ExtIEs} } 	OPTIONAL,

...

}

RANAreaID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RANAreaID-List ::= SEQUENCE (SIZE(1..maxnoofRANAreasinRNA)) OF RANAreaID

Range ::= ENUMERATED {m50, m80, m180, m200, m350, m400, m500, m700, m1000, ...}

RANPagingArea ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

rANPagingAreaChoice		RANPagingAreaChoice,

iE-Extensions			ProtocolExtensionContainer { {RANPagingArea-ExtIEs} } OPTIONAL,

...

}

RANPagingArea-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RANPagingAreaChoice ::= CHOICE {

cell-List			NG-RAN-Cell-Identity-ListinRANPagingArea,

rANAreaID-List		RANAreaID-List,

choice-extension	ProtocolIE-Single-Container { {RANPagingAreaChoice-ExtIEs} }

}

RANPagingAreaChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

RANPagingAttemptInfo ::= SEQUENCE {

pagingAttemptCount					INTEGER (1..16, ...),

intendedNumberOfPagingAttempts		INTEGER (1..16, ...),

nextPagingAreaScope					ENUMERATED {same, changed, ...}	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {RANPagingAttemptInfo-ExtIEs} } OPTIONAL,

...

}

RANPagingAttemptInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RANPagingFailure		::= 	ENUMERATED {

true,

...

}

RBsetConfiguration ::= SEQUENCE {

subcarrierSpacing 		SSB-subcarrierSpacing,

rBsetSize				ENUMERATED {rb2, rb4, rb8, rb16, rb32, rb64},

numberofRBSets			INTEGER(1.. maxnoofRBsetsPerCell),

iE-Extensions				ProtocolExtensionContainer { { RBsetConfiguration-ExtIEs} } OPTIONAL,

...

}

RBsetConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Redcap-Bcast-Information ::=  BIT STRING(SIZE(8))

RedundantQoSFlowIndicator ::= ENUMERATED {true, false}

RedundantPDUSessionInformation ::= SEQUENCE {

rSN					RSN,

iE-Extensions		ProtocolExtensionContainer { {RedundantPDUSessionInformation-ExtIEs} }	OPTIONAL,

...

}

RedundantPDUSessionInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-PDUSession-PairID	CRITICALITY ignore	EXTENSION PDUSession-PairID	PRESENCE optional	},

...

}

RSN ::= ENUMERATED {v1, v2, ...}

ReflectiveQoSAttribute ::= ENUMERATED {subject-to-reflective-QoS, ...}

RequestedSRSTransmissionCharacteristics ::= OCTET STRING

RoutingID ::= OCTET STRING

ReplacingCells ::= SEQUENCE (SIZE(0.. maxnoofCellsinNG-RANnode)) OF ReplacingCells-Item

ReplacingCells-Item ::= SEQUENCE {

globalNG-RANCell-ID				GlobalCell-ID,

iE-Extensions		ProtocolExtensionContainer { {ReplacingCells-Item-ExtIEs} }	OPTIONAL,

...

}

ReplacingCells-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ReportAmountMDT ::= ENUMERATED{r1, r2, r4, r8, r16, r32, r64, infinity, ...}

ReportArea ::= ENUMERATED {

cell,

...

}

ReportConfigContainer ::= OCTET STRING

ReportIntervalMDT ::= ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60, ...}

ReportType ::= CHOICE {

periodical					Periodical,

eventTriggered				EventTriggered,

...

}

ExtendedReportIntervalMDT ::= ENUMERATED {

ms20480,

ms40960,

...

}

ReportCharacteristics ::=  BIT STRING(SIZE(32))

ReportingPeriodicity ::= ENUMERATED {

half-thousand-ms,

one-thousand-ms,

two-thousand-ms,

five-thousand-ms,

ten-thousand-ms,

...

}

RegistrationRequest ::= ENUMERATED {start, stop, add, ... }

RequestReferenceID ::= INTEGER (1..64, ...)

ReservedSubframePattern ::= SEQUENCE {

subframeType					ENUMERATED {mbsfn, non-mbsfn, ...},

reservedSubframePattern			BIT STRING (SIZE(10..160)),

mbsfnControlRegionLength		MBSFNControlRegionLength					OPTIONAL,

iE-Extension					ProtocolExtensionContainer { {ReservedSubframePattern-ExtIEs} } OPTIONAL,

...

}

ReservedSubframePattern-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetRequestTypeInfo ::= CHOICE {

fullReset			ResetRequestTypeInfo-Full,

partialReset		ResetRequestTypeInfo-Partial,

choice-extension	ProtocolIE-Single-Container { {ResetRequestTypeInfo-ExtIEs} }

}

ResetRequestTypeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ResetRequestTypeInfo-Full ::= SEQUENCE {

iE-Extension					ProtocolExtensionContainer { {ResetRequestTypeInfo-Full-ExtIEs} } OPTIONAL,

...

}

ResetRequestTypeInfo-Full-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetRequestTypeInfo-Partial ::= SEQUENCE {

ue-contexts-ToBeReleasedList	ResetRequestPartialReleaseList,

iE-Extension					ProtocolExtensionContainer { {ResetRequestTypeInfo-Partial-ExtIEs} } OPTIONAL,

...

}

ResetRequestTypeInfo-Partial-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetRequestPartialReleaseList ::= SEQUENCE (SIZE(1..maxnoofUEContexts)) OF ResetRequestPartialReleaseItem

ResetRequestPartialReleaseItem ::= SEQUENCE {

ng-ran-node1UEXnAPID						NG-RANnodeUEXnAPID			OPTIONAL,

ng-ran-node2UEXnAPID						NG-RANnodeUEXnAPID			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {ResetRequestPartialReleaseItem-ExtIEs} } OPTIONAL,

...

}

ResetRequestPartialReleaseItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetResponseTypeInfo ::= CHOICE {

fullReset			ResetResponseTypeInfo-Full,

partialReset		ResetResponseTypeInfo-Partial,

choice-extension	ProtocolIE-Single-Container { {ResetResponseTypeInfo-ExtIEs} }

}

ResetResponseTypeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ResetResponseTypeInfo-Full ::= SEQUENCE {

iE-Extension					ProtocolExtensionContainer { {ResetResponseTypeInfo-Full-ExtIEs} } OPTIONAL,

...

}

ResetResponseTypeInfo-Full-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetResponseTypeInfo-Partial ::= SEQUENCE {

ue-contexts-AdmittedToBeReleasedList	ResetResponsePartialReleaseList,

iE-Extension					ProtocolExtensionContainer { {ResetResponseTypeInfo-Partial-ExtIEs} } OPTIONAL,

...

}

ResetResponseTypeInfo-Partial-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetResponsePartialReleaseList ::= SEQUENCE (SIZE(1..maxnoofUEContexts)) OF ResetResponsePartialReleaseItem

ResetResponsePartialReleaseItem ::= SEQUENCE {

ng-ran-node1UEXnAPID						NG-RANnodeUEXnAPID		OPTIONAL,

ng-ran-node2UEXnAPID						NG-RANnodeUEXnAPID		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {ResetResponsePartialReleaseItem-ExtIEs} } OPTIONAL,

...

}

ResetResponsePartialReleaseItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RLCMode	::= ENUMERATED {

rlc-am,

rlc-um-bidirectional,

rlc-um-unidirectional-ul,

rlc-um-unidirectional-dl,

...

}

RLC-Status ::= SEQUENCE {

reestablishment-Indication 	Reestablishment-Indication,

iE-Extensions					ProtocolExtensionContainer { {RLC-Status-ExtIEs} } OPTIONAL,

...

}

RLC-Status-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RLCDuplicationInformation ::= 		SEQUENCE {

rLCDuplicationStateList 		RLCDuplicationStateList,

rLC-PrimaryIndicator 		ENUMERATED {true, false}		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {RLCDuplicationInformation-ItemExtIEs} }	OPTIONAL

}

RLCDuplicationInformation-ItemExtIEs 	XNAP-PROTOCOL-EXTENSION ::= {

...

}

RLCDuplicationStateList	::= 	SEQUENCE (SIZE(1..maxnoofRLCDuplicationstate)) OF RLCDuplicationState-Item

RLCDuplicationState-Item ::=	SEQUENCE {

duplicationState			ENUMERATED {active,inactive, ...},

iE-Extensions	ProtocolExtensionContainer { {RLCDuplicationState-ItemExtIEs } }	OPTIONAL,

...

}

RLCDuplicationState-ItemExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Reestablishment-Indication ::= ENUMERATED {

reestablished,

...

}

RFSP-Index ::= INTEGER (1..256)

RRCConfigIndication ::= ENUMERATED {

full-config,

delta-config,

...

}

RRCConnections::= SEQUENCE {

noofRRCConnections							NoofRRCConnections,

availableRRCConnectionCapacityValue		AvailableRRCConnectionCapacityValue,

iE-Extensions				ProtocolExtensionContainer { { RRCConnections-ExtIEs} }	OPTIONAL,

...

}

RRCConnections-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RRCConnReestab-Indicator ::= ENUMERATED { reconfigurationFailure, handoverFailure, otherFailure, ...}

RRCReestab-initiated ::= SEQUENCE {

rRRCReestab-initiated-reporting	RRCReestab-Initiated-Reporting,

iE-Extensions			ProtocolExtensionContainer { { RRCReestab-initiated-ExtIEs} } OPTIONAL,

...

}

RRCReestab-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RRCReestab-Initiated-Reporting ::= CHOICE {

rRCReestab-reporting-wo-UERLFReport					RRCReestab-Initiated-Reporting-wo-UERLFReport,

rRCReestab-reporting-with-UERLFReport				RRCReestab-Initiated-Reporting-with-UERLFReport,

choice-extension				ProtocolIE-Single-Container { {RRCReestab-Initiated-Reporting-ExtIEs} }

}

RRCReestab-Initiated-Reporting-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

RRCReestab-Initiated-Reporting-wo-UERLFReport ::= SEQUENCE {

failureCellPCI		NG-RAN-CellPCI,

reestabCellCGI		GlobalNG-RANCell-ID,

c-RNTI				C-RNTI,

shortMAC-I			MAC-I,

iE-Extensions		ProtocolExtensionContainer { { RRCReestab-Initiated-Reporting-wo-UERLFReport-ExtIEs} } OPTIONAL,

...

}

RRCReestab-Initiated-Reporting-wo-UERLFReport-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-RRCConnReestab-Indicator 	CRITICALITY ignore	EXTENSION RRCConnReestab-Indicator		PRESENCE optional },

...

}

RRCReestab-Initiated-Reporting-with-UERLFReport ::= SEQUENCE {

uERLFReportContainer	UERLFReportContainer,

iE-Extensions			ProtocolExtensionContainer { {RRCReestab-Initiated-Reporting-with-UERLFReport-ExtIEs} } OPTIONAL,

...

}

RRCReestab-Initiated-Reporting-with-UERLFReport-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RRCSetup-initiated ::= SEQUENCE {

rRRCSetup-Initiated-Reporting	RRCSetup-Initiated-Reporting,

uERLFReportContainer			UERLFReportContainer			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { RRCSetup-initiated-ExtIEs} } OPTIONAL,

...

}

RRCSetup-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RRCSetup-Initiated-Reporting ::= CHOICE {

rRCSetup-reporting-with-UERLFReport				RRCSetup-Initiated-Reporting-with-UERLFReport,

choice-extension				ProtocolIE-Single-Container { {RRCSetup-Initiated-Reporting-ExtIEs} }

}

RRCSetup-Initiated-Reporting-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

RRCSetup-Initiated-Reporting-with-UERLFReport ::= SEQUENCE {

uERLFReportContainer	UERLFReportContainer,

iE-Extensions			ProtocolExtensionContainer { {RRCSetup-Initiated-Reporting-with-UERLFReport-ExtIEs} } OPTIONAL,

...

}

RRCSetup-Initiated-Reporting-with-UERLFReport-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RRCResumeCause ::= ENUMERATED {

rna-Update,

...

}

RachIndicationList ::= SEQUENCE (SIZE(1..maxnoofRachReports)) OF RachIndication-Item

RachIndication-Item ::= SEQUENCE {

randomAccessIndciation 				ENUMERATED { true,...}	PRESENCE mandatory },

m-NG-RANAssistantIdentifier			NG-RANnodeUEXnAPID		PRESENCE mandatory },

s-NG-RANAssistantIdentifier			NG-RANnodeUEXnAPID		PRESENCE mandatory },

iE-Extensions						ProtocolExtensionContainer { { RachIndication-Item-ExtIEs} }	OPTIONAL,

...

}

RachIndication-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- S

SCGreconfigNotification ::= ENUMERATED {executed, ...}

S-NSSAIListQoE ::= SEQUENCE (SIZE(1..maxnoofSNSSAIforQMC)) OF S-NSSAI

S-BasedMDT ::= SEQUENCE {

ng-ran-TraceID				NG-RANTraceID,

iE-Extension				ProtocolExtensionContainer { {S-BasedMDT-ExtIEs} }	OPTIONAL,

...

}

S-BasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServiceType ::= ENUMERATED{

qMC-for-streaming-service,

qMC-for-MTSI-service,

qMC-for-VR-service,

...

}

SecondarydataForwardingInfoFromTarget-Item::= SEQUENCE {

secondarydataForwardingInfoFromTarget				DataForwardingInfoFromTargetNGRANnode,

iE-Extensions		ProtocolExtensionContainer { { SecondarydataForwardingInfoFromTarget-Item-ExtIEs} }	OPTIONAL,

...

}

SecondarydataForwardingInfoFromTarget-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SecondarydataForwardingInfoFromTarget-List ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF SecondarydataForwardingInfoFromTarget-Item

SCGActivationRequest ::= ENUMERATED {activate-scg, deactivate-scg, ...}

SCGActivationStatus ::= ENUMERATED {scg-activated, scg-deactivated, ...}

SCGConfigurationQuery	::= ENUMERATED {true, ...}

SCGIndicator	::=	ENUMERATED{released, ...}

SCGFailureReportContainer	::=	OCTET STRING

SDTSupportRequest ::= SEQUENCE {

sdtindicator				SDTIndicator,

sdtAssistantInfo			SDTAssistantInfo		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { SDTSupportRequest-ExtIEs} } OPTIONAL,

...

}

SDTSupportRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SDTIndicator ::= ENUMERATED {true, ...}

SDTAssistantInfo ::= ENUMERATED {single-packet, multiple-packets, ...}

SDT-Termination-Request	::= ENUMERATED {radio-link-problem, normal, ...}

SDTPartialUEContextInfo ::= SEQUENCE {

dRBsToBeSetup					SDT-DRBsToBeSetupList		OPTIONAL,

sRBsToBeSetup					SDT-SRBsToBeSetupList,

iE-Extensions					ProtocolExtensionContainer { { SDTPartialUEContextInfo-ExtIEs} } 	OPTIONAL,

...

}

SDTPartialUEContextInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SDT-DRBsToBeSetupList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF SDT-DRBsToBeSetupList-Item

SDT-DRBsToBeSetupList-Item	::= SEQUENCE {

drb-ID							DRB-ID,

uL-TNLInfo						UPTransportParameters,

dRB-RLC-Bearer-Configuration	OCTET STRING,

dRB-QoS							QoSFlowLevelQoSParameters,

rLC-Mode						RLCMode,

s-nssai							S-NSSAI,

pDCP-SNLength					PDCPSNLength,

flows-Mapped-To-DRB-List		Flows-Mapped-To-DRB-List,

iE-Extensions		ProtocolExtensionContainer { { SDT-DRBsToBeSetupList-Item-ExtIEs} } OPTIONAL,

...

}

SDT-DRBsToBeSetupList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SDT-SRBsToBeSetupList ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF SDT-SRBsToBeSetupList-Item

SDT-SRBsToBeSetupList-Item	::= SEQUENCE {

srb-ID							SRB-ID,

sRB-RLC-Bearer-Configuration	OCTET STRING,

iE-Extensions					ProtocolExtensionContainer { { SDT-SRBsToBeSetupList-Item-ExtIEs} } OPTIONAL,

...

}

SDT-SRBsToBeSetupList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SRB-ID ::= INTEGER (0..4, ...)

SDTDataForwardingDRBList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF SDTDataForwardingDRBList-Item

SDTDataForwardingDRBList-Item ::= SEQUENCE {

drb-ID							DRB-ID,

dL-TNLInfo						UPTransportParameters		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SDTDataForwardingDRBList-Item-ExtIEs} } OPTIONAL,

...

}

SDTDataForwardingDRBList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SecondaryRATUsageInformation ::= SEQUENCE {

pDUSessionUsageReport		PDUSessionUsageReport				OPTIONAL,

qosFlowsUsageReportList		QoSFlowsUsageReportList				OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {SecondaryRATUsageInformation-ExtIEs} }	OPTIONAL,

...

}

SecondaryRATUsageInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SecurityIndication ::= SEQUENCE {

integrityProtectionIndication			ENUMERATED {required, preferred, not-needed, ...},

confidentialityProtectionIndication		ENUMERATED {required, preferred, not-needed, ...},

maximumIPdatarate						MaximumIPdatarate												OPTIONAL,

-- This IE shall be present if the Integrity Protection IE within the Security Indication IE is present and set to "required" or "preferred". --

iE-Extensions							ProtocolExtensionContainer { {SecurityIndication-ExtIEs} } OPTIONAL,

...

}

SecurityIndication-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SecurityResult ::= SEQUENCE {

integrityProtectionResult				ENUMERATED {performed, not-performed, ...},

confidentialityProtectionResult			ENUMERATED {performed, not-performed, ...},

iE-Extensions							ProtocolExtensionContainer { {SecurityResult-ExtIEs} } OPTIONAL,

...

}

SecurityResult-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SensorMeasurementConfiguration ::= SEQUENCE {

sensorMeasConfig 				SensorMeasConfig,

sensorMeasConfigNameList		SensorMeasConfigNameList			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { SensorMeasurementConfiguration-ExtIEs } } OPTIONAL,

...

}

SensorMeasurementConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SensorMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofSensorName)) OF SensorName

SensorMeasConfig::= ENUMERATED {setup,...}

SensorName ::= SEQUENCE {

uncompensatedBarometricConfig	ENUMERATED {true, ...}         OPTIONAL,

ueSpeedConfig					ENUMERATED {true, ...}         OPTIONAL,

ueOrientationConfig				ENUMERATED {true, ...}         OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {SensorNameConfig-ExtIEs} } OPTIONAL,

...

}

SensorNameConfig-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

##### -- Served Cells E-UTRA IEs

ServedCellInformation-E-UTRA ::= SEQUENCE {

e-utra-pci								E-UTRAPCI,

e-utra-cgi								E-UTRA-CGI,

tac										TAC,

ranac									RANAC																					OPTIONAL,

broadcastPLMNs							SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF ServedCellInformation-E-UTRA-perBPLMN,

e-utra-mode-info						ServedCellInformation-E-UTRA-ModeInfo,

numberofAntennaPorts					NumberOfAntennaPorts-E-UTRA																OPTIONAL,

prach-configuration						E-UTRAPRACHConfiguration																OPTIONAL,

mBSFNsubframeInfo						MBSFNSubframeInfo-E-UTRA																OPTIONAL,

multibandInfo							E-UTRAMultibandInfoList																	OPTIONAL,

freqBandIndicatorPriority				ENUMERATED {not-broadcast, broadcast, ...} 												OPTIONAL,

bandwidthReducedSI						ENUMERATED {scheduled, ...}																OPTIONAL,

protectedE-UTRAResourceIndication		ProtectedE-UTRAResourceIndication														OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-ExtIEs} }					OPTIONAL,

...

}

ServedCellInformation-E-UTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-BPLMN-ID-Info-EUTRA		CRITICALITY ignore	EXTENSION BPLMN-ID-Info-EUTRA		PRESENCE optional }|

{ ID id-NPRACHConfiguration		CRITICALITY ignore	EXTENSION	NPRACHConfiguration		PRESENCE optional},

...

}

ServedCellInformation-E-UTRA-perBPLMN ::= SEQUENCE {

plmn-id					PLMN-Identity,

iE-Extensions			ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-perBPLMN-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-E-UTRA-perBPLMN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCellInformation-E-UTRA-ModeInfo ::= CHOICE {

fdd					ServedCellInformation-E-UTRA-FDDInfo,

tdd					ServedCellInformation-E-UTRA-TDDInfo,

choice-extension	ProtocolIE-Single-Container{ {ServedCellInformation-E-UTRA-ModeInfo-ExtIEs} }

}

ServedCellInformation-E-UTRA-ModeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ServedCellInformation-E-UTRA-FDDInfo ::= SEQUENCE {

ul-earfcn			E-UTRAARFCN,

dl-earfcn			E-UTRAARFCN,

ul-e-utraTxBW		E-UTRATransmissionBandwidth,

dl-e-utraTxBW		E-UTRATransmissionBandwidth,

iE-Extensions		ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-FDDInfo-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-E-UTRA-FDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-OffsetOfNbiotChannelNumberToDL-EARFCN	CRITICALITY reject	EXTENSION OffsetOfNbiotChannelNumberToEARFCN		PRESENCE optional}|

{ ID id-OffsetOfNbiotChannelNumberToUL-EARFCN	CRITICALITY reject	EXTENSION OffsetOfNbiotChannelNumberToEARFCN		PRESENCE optional},

...

}

ServedCellInformation-E-UTRA-TDDInfo ::= SEQUENCE {

earfcn					E-UTRAARFCN,

e-utraTxBW				E-UTRATransmissionBandwidth,

subframeAssignmnet		ENUMERATED {sa0,sa1,sa2,sa3,sa4,sa5,sa6,...},

specialSubframeInfo		SpecialSubframeInfo-E-UTRA,

iE-Extensions			ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-TDDInfo-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-E-UTRA-TDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-OffsetOfNbiotChannelNumberToDL-EARFCN	CRITICALITY reject	EXTENSION OffsetOfNbiotChannelNumberToEARFCN		PRESENCE optional}|

{ ID id-NBIoT-UL-DL-AlignmentOffset				CRITICALITY reject	EXTENSION NBIoT-UL-DL-AlignmentOffset					PRESENCE optional},

...

}

ServedCells-E-UTRA ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-E-UTRA-Item

ServedCells-E-UTRA-Item ::= SEQUENCE {

served-cell-info-E-UTRA		ServedCellInformation-E-UTRA,

neighbour-info-NR			NeighbourInformation-NR									OPTIONAL,

neighbour-info-E-UTRA		NeighbourInformation-E-UTRA								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServedCells-E-UTRA-Item-ExtIEs} } OPTIONAL,

...

}

ServedCells-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SFN-Offset				CRITICALITY ignore EXTENSION SFN-Offset					PRESENCE optional },

...

}

ServedCellsToUpdate-E-UTRA ::= SEQUENCE {

served-Cells-ToAdd-E-UTRA		ServedCells-E-UTRA																OPTIONAL,

served-Cells-ToModify-E-UTRA	ServedCells-ToModify-E-UTRA														OPTIONAL,

served-Cells-ToDelete-E-UTRA	SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI 					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ServedCellsToUpdate-E-UTRA-ExtIEs} }	OPTIONAL,

...

}

ServedCellsToUpdate-E-UTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCells-ToModify-E-UTRA ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-ToModify-E-UTRA-Item

ServedCells-ToModify-E-UTRA-Item ::= SEQUENCE {

old-ECGI					E-UTRA-CGI,

served-cell-info-E-UTRA		ServedCellInformation-E-UTRA,

neighbour-info-NR			NeighbourInformation-NR											OPTIONAL,

neighbour-info-E-UTRA		NeighbourInformation-E-UTRA										OPTIONAL,

deactivation-indication		ENUMERATED {deactivated, ...}									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {Served-cells-ToModify-E-UTRA-Item-ExtIEs} }	OPTIONAL,

...

}

Served-cells-ToModify-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

##### -- Served Cells NR IEs

ServedCellInformation-NR ::= SEQUENCE {

nrPCI								NRPCI,

cellID								NR-CGI,

tac									TAC,

ranac								RANAC						OPTIONAL,

broadcastPLMN						BroadcastPLMNs,

nrModeInfo							NRModeInfo,

measurementTimingConfiguration		OCTET STRING,

connectivitySupport					Connectivity-Support,

iE-Extensions						ProtocolExtensionContainer { {ServedCellInformation-NR-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-BPLMN-ID-Info-NR				CRITICALITY ignore	EXTENSION BPLMN-ID-Info-NR						PRESENCE optional }|

{ ID id-ConfiguredTACIndication			CRITICALITY ignore	EXTENSION ConfiguredTACIndication				PRESENCE optional }|

{ ID id-SSB-PositionsInBurst			CRITICALITY ignore	EXTENSION SSB-PositionsInBurst					PRESENCE optional }|

{ ID id-NRCellPRACHConfig				CRITICALITY ignore	EXTENSION NRCellPRACHConfig						PRESENCE optional }|

{ ID id-NPN-Broadcast-Information		CRITICALITY reject	EXTENSION NPN-Broadcast-Information			PRESENCE optional }|

{ ID id-CSI-RSTransmissionIndication	CRITICALITY ignore	EXTENSION CSI-RSTransmissionIndication		PRESENCE optional } |

{ ID id-SFN-Offset						CRITICALITY ignore	EXTENSION SFN-Offset							PRESENCE optional }|

{ ID id-Supported-MBS-FSA-ID-List		CRITICALITY ignore	EXTENSION Supported-MBS-FSA-ID-List			PRESENCE optional }|

{ ID id-NR-U-ChannelInfo-List	CRITICALITY ignore	EXTENSION NR-U-ChannelInfo-List	PRESENCE optional }|

{ ID id-Additional-Measurement-Timing-Configuration-List		CRITICALITY ignore	EXTENSION Additional-Measurement-Timing-Configuration-List		PRESENCE optional }|

{ ID id-Redcap-Bcast-Information		CRITICALITY ignore	EXTENSION Redcap-Bcast-Information			PRESENCE optional },

...

}

SFN-Offset ::= SEQUENCE {

sFN-Time-Offset					BIT STRING (SIZE(24)),

iE-Extensions		ProtocolExtensionContainer { {SFN-Offset-ExtIEs} } OPTIONAL,

...

}

SFN-Offset-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCells-NR ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-NR-Item

ServedCells-NR-Item ::= SEQUENCE {

served-cell-info-NR			ServedCellInformation-NR,

neighbour-info-NR			NeighbourInformation-NR				OPTIONAL,

neighbour-info-E-UTRA		NeighbourInformation-E-UTRA			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServedCells-NR-Item-ExtIEs} } OPTIONAL,

...

}

ServedCells-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-ServedCellSpecificInfoReq-NR	CRITICALITY ignore EXTENSION	ServedCellSpecificInfoReq-NR	PRESENCE optional },

...

}

ServedCells-ToModify-NR ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-ToModify-NR-Item

ServedCells-ToModify-NR-Item ::= SEQUENCE {

old-NR-CGI					NR-CGI,

served-cell-info-NR			ServedCellInformation-NR,

neighbour-info-NR			NeighbourInformation-NR											OPTIONAL,

neighbour-info-E-UTRA		NeighbourInformation-E-UTRA										OPTIONAL,

deactivation-indication		ENUMERATED {deactivated, ...}									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {Served-cells-ToModify-NR-Item-ExtIEs} } 	OPTIONAL,

...

}

Served-cells-ToModify-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCellSpecificInfoReq-NR	::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF ServedCellSpecificInfoReq-NR-Item

ServedCellSpecificInfoReq-NR-Item	::= SEQUENCE {

nRCGI									NR-CGI,

additionalMTCListRequestIndicator		ENUMERATED {additionalMTCListRequested, ...}			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { ServedCellSpecificInfoReq-NR-Item-ExtIEs} }	OPTIONAL,

...

}

ServedCellSpecificInfoReq-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCellsToUpdate-NR ::= SEQUENCE {

served-Cells-ToAdd-NR		ServedCells-NR															OPTIONAL,

served-Cells-ToModify-NR	ServedCells-ToModify-NR													OPTIONAL,

served-Cells-ToDelete-NR	SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF NR-CGI 				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServedCellsToUpdate-NR-ExtIEs} } OPTIONAL,

...

}

ServedCellsToUpdate-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SharedResourceType ::= CHOICE {

ul-onlySharing				SharedResourceType-UL-OnlySharing,

ul-and-dl-Sharing			SharedResourceType-ULDL-Sharing,

choice-extension			ProtocolIE-Single-Container { {SharedResourceType-ExtIEs} }

}

SharedResourceType-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SharedResourceType-UL-OnlySharing ::= SEQUENCE {

ul-resourceBitmap			DataTrafficResources,

iE-Extensions			ProtocolExtensionContainer { {SharedResourceType-UL-OnlySharing-ExtIEs} } OPTIONAL,

...

}

SharedResourceType-UL-OnlySharing-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SharedResourceType-ULDL-Sharing ::= CHOICE {

ul-resources				SharedResourceType-ULDL-Sharing-UL-Resources,

dl-resources				SharedResourceType-ULDL-Sharing-DL-Resources,

choice-extension			ProtocolIE-Single-Container { {SharedResourceType-ULDL-Sharing-ExtIEs} }

}

SharedResourceType-ULDL-Sharing-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SharedResourceType-ULDL-Sharing-UL-Resources ::= CHOICE {

unchanged					NULL,

changed						SharedResourceType-ULDL-Sharing-UL-ResourcesChanged,

choice-extension			ProtocolIE-Single-Container { {SharedResourceType-ULDL-Sharing-UL-Resources-ExtIEs} }

}

SharedResourceType-ULDL-Sharing-UL-Resources-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SharedResourceType-ULDL-Sharing-UL-ResourcesChanged ::= SEQUENCE {

ul-resourceBitmap			DataTrafficResources,

iE-Extensions			ProtocolExtensionContainer { {SharedResourceType-ULDL-Sharing-UL-ResourcesChanged-ExtIEs} } OPTIONAL,

...

}

SharedResourceType-ULDL-Sharing-UL-ResourcesChanged-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SharedResourceType-ULDL-Sharing-DL-Resources ::= CHOICE {

unchanged					NULL,

changed						SharedResourceType-ULDL-Sharing-DL-ResourcesChanged,

choice-extension			ProtocolIE-Single-Container { {SharedResourceType-ULDL-Sharing-DL-Resources-ExtIEs} }

}

SharedResourceType-ULDL-Sharing-DL-Resources-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SharedResourceType-ULDL-Sharing-DL-ResourcesChanged ::= SEQUENCE {

dl-resourceBitmap			DataTrafficResources,

iE-Extensions			ProtocolExtensionContainer { {SharedResourceType-ULDL-Sharing-DL-ResourcesChanged-ExtIEs} } OPTIONAL,

...

}

SharedResourceType-ULDL-Sharing-DL-ResourcesChanged-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceAvailableCapacity ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceAvailableCapacity-Item

SliceAvailableCapacity-Item	::= SEQUENCE {

pLMNIdentity						PLMN-Identity,

sNSSAIAvailableCapacity-List		SNSSAIAvailableCapacity-List,

iE-Extensions						ProtocolExtensionContainer { { SliceAvailableCapacity-Item-ExtIEs} }	OPTIONAL,

...

}

SliceAvailableCapacity-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAIAvailableCapacity-List ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SNSSAIAvailableCapacity-Item

SNSSAIAvailableCapacity-Item ::= SEQUENCE {

sNSSAI		S-NSSAI,

sliceAvailableCapacityValueDownlink	INTEGER (0..100),

sliceAvailableCapacityValueUplink	INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { SNSSAIAvailableCapacity-Item-ExtIEs } }	OPTIONAL

}

SNSSAIAvailableCapacity-Item-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceRadioResourceStatus-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceRadioResourceStatus-Item

SliceRadioResourceStatus-Item	::= SEQUENCE {

plmn-Identity						PLMN-Identity,

sNSSAIRadioResourceStatus-List		SNSSAIRadioResourceStatus-List,

iE-Extensions						ProtocolExtensionContainer { { SliceRadioResourceStatus-Item-ExtIEs} }	OPTIONAL,

...

}

SliceRadioResourceStatus-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAIRadioResourceStatus-List ::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF SNSSAIRadioResourceStatus-Item

SNSSAIRadioResourceStatus-Item	::= SEQUENCE {

sNSSAI								S-NSSAI,

slice-DL-GBR-PRB-Usage				Slice-DL-GBR-PRB-Usage,

slice-UL-GBR-PRB-Usage				Slice-UL-GBR-PRB-Usage,

slice-DL-non-GBR-PRB-Usage			Slice-DL-non-GBR-PRB-Usage,

slice-UL-non-GBR-PRB-Usage			Slice-UL-non-GBR-PRB-Usage,

slice-DL-Total-PRB-Allocation		Slice-DL-Total-PRB-Allocation,

slice-UL-Total-PRB-Allocation		Slice-UL-Total-PRB-Allocation,

iE-Extensions						ProtocolExtensionContainer { { SNSSAIRadioResourceStatus-Item-ExtIEs} }	OPTIONAL,

...

}

SNSSAIRadioResourceStatus-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Slice-DL-GBR-PRB-Usage 			::= INTEGER (0..100)

Slice-UL-GBR-PRB-Usage 			::= INTEGER (0..100)

Slice-DL-non-GBR-PRB-Usage 		::= INTEGER (0..100)

Slice-UL-non-GBR-PRB-Usage 		::= INTEGER (0..100)

Slice-DL-Total-PRB-Allocation 	::= INTEGER (0..100)

Slice-UL-Total-PRB-Allocation 	::= INTEGER (0..100)

SliceSupport-List	::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF S-NSSAI

SliceToReport-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceToReport-List-Item

SliceToReport-List-Item	::= SEQUENCE {

pLMNIdentity				PLMN-Identity,

sNSSAIlist					SNSSAI-list,

iE-Extensions						ProtocolExtensionContainer { { SliceToReport-List-Item-ExtIEs} }	OPTIONAL,

...

}

SliceToReport-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAI-list ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SNSSAI-Item

SNSSAI-Item ::= SEQUENCE {

sNSSAI		S-NSSAI,

iE-Extensions				ProtocolExtensionContainer { { SNSSAI-Item-ExtIEs } }	OPTIONAL

}

SNSSAI-Item-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

SlotConfiguration-List ::= SEQUENCE (SIZE (1..maxnoofslots)) OF SlotConfiguration-List-Item

SlotConfiguration-List-Item ::= SEQUENCE {

slotIndex						INTEGER (0..5119),

symbolAllocation-in-Slot		SymbolAllocation-in-Slot,

iE-Extensions				ProtocolExtensionContainer { {SlotConfiguration-List-Item-ExtIEs} }	OPTIONAL,

...

}

SlotConfiguration-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

S-NG-RANnode-SecurityKey ::= BIT STRING (SIZE(256))

S-NG-RANnode-Addition-Trigger-Ind ::= ENUMERATED {

sn-change,

inter-MN-HO,

intra-MN-HO,

...

}

S-NSSAI ::= SEQUENCE {

sst						OCTET STRING (SIZE(1)),

sd						OCTET STRING (SIZE(3))							OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {S-NSSAI-ExtIEs} } OPTIONAL,

...

}

S-NSSAI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNMobilityInformation ::= BIT STRING (SIZE(32))

SNTriggered ::=ENUMERATED{

true,

...

}

SpecialSubframeInfo-E-UTRA ::= SEQUENCE {

specialSubframePattern	SpecialSubframePatterns-E-UTRA,

cyclicPrefixDL			CyclicPrefix-E-UTRA-DL,

cyclicPrefixUL			CyclicPrefix-E-UTRA-UL,

iE-Extensions			ProtocolExtensionContainer { {SpecialSubframeInfo-E-UTRA-ExtIEs} } OPTIONAL,

...

}

SpecialSubframeInfo-E-UTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SpecialSubframePatterns-E-UTRA ::= ENUMERATED {

ssp0,

ssp1,

ssp2,

ssp3,

ssp4,

ssp5,

ssp6,

ssp7,

ssp8,

ssp9,

ssp10,

...

}

SpectrumSharingGroupID ::= INTEGER (1..maxnoofCellsinNG-RANnode)

SplitSessionIndicator ::= ENUMERATED {

split,

...

}

SplitSRBsTypes ::= ENUMERATED {srb1, srb2, srb1and2, ...}

SSBAreaCapacityValue-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBAreaCapacityValue-List-Item

SSBAreaCapacityValue-List-Item	::= SEQUENCE {

sSBIndex				INTEGER(0..63),

ssbAreaCapacityValue	INTEGER (0..100),

iE-Extensions						ProtocolExtensionContainer { { SSBAreaCapacityValue-List-Item-ExtIEs} }	OPTIONAL,

...

}

SSBAreaCapacityValue-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SSBAreaRadioResourceStatus-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBAreaRadioResourceStatus-List-Item

SSBAreaRadioResourceStatus-List-Item	::= SEQUENCE {

sSBIndex								INTEGER(0..63),

ssb-Area-DL-GBR-PRB-usage	DL-GBR-PRB-usage,

ssb-Area-UL-GBR-PRB-usage	UL-GBR-PRB-usage,

ssb-Area-dL-non-GBR-PRB-usage		DL-non-GBR-PRB-usage,

ssb-Area-uL-non-GBR-PRB-usage		UL-non-GBR-PRB-usage,

ssb-Area-dL-Total-PRB-usage			DL-Total-PRB-usage,

ssb-Area-uL-Total-PRB-usage			UL-Total-PRB-usage,

iE-Extensions						ProtocolExtensionContainer { { SSBAreaRadioResourceStatus-List-Item-ExtIEs} }	OPTIONAL,

...

}

SSBAreaRadioResourceStatus-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DL-scheduling-PDCCH-CCE-usage		CRITICALITY ignore	EXTENSION DL-scheduling-PDCCH-CCE-usage	PRESENCE optional}|

{ ID id-UL-scheduling-PDCCH-CCE-usage		CRITICALITY ignore	EXTENSION UL-scheduling-PDCCH-CCE-usage	PRESENCE optional},

...

}

SSB-Coverage-Modification-List ::= SEQUENCE (SIZE (0..maxnoofSSBAreas)) OF SSB-Coverage-Modification-List-Item

SSB-Coverage-Modification-List-Item ::= SEQUENCE {

sSBIndex						INTEGER(0..63),

sSBCoverageState				INTEGER (0..15, ...),

iE-Extension			ProtocolExtensionContainer { { SSB-Coverage-Modification-Item-ExtIEs} } OPTIONAL,

...

}

SSB-Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SSB-PositionsInBurst ::= CHOICE {

shortBitmap						BIT STRING (SIZE (4)),

mediumBitmap					BIT STRING (SIZE (8)),

longBitmap						BIT STRING (SIZE (64)),

choice-extension				ProtocolIE-Single-Container { {SSB-PositionsInBurst-ExtIEs} }

}

SSB-PositionsInBurst-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SSB-freqInfo ::= INTEGER (0..maxNRARFCN)

SSB-subcarrierSpacing ::=  ENUMERATED {kHz15, kHz30, kHz120, kHz240, spare3, spare2, spare1, ...}

SSBOffsets-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBOffsets-Item

SSBOffsets-Item ::= SEQUENCE {

nG-RANnode1SSBOffsets			SSBOffsetInformation										OPTIONAL,

nG-RANnode2ProposedSSBOffsets	SSBOffsetInformation,

iE-Extensions					ProtocolExtensionContainer { { SSBOffsets-Item-ExtIEs} }	OPTIONAL,

...

}

SSBOffsets-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SSBOffsetInformation	::= SEQUENCE {

sSBIndex			INTEGER(0..63),

sSBTriggeringOffset			MobilityParametersInformation,

iE-Extensions				ProtocolExtensionContainer { { SSBOffsetInformation-ExtIEs} }	OPTIONAL,

...

}

SSBOffsetInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SSBOffsetModificationRange	::= SEQUENCE {

sSBIndex				INTEGER(0..63),

sSBobilityParametersModificationRange		MobilityParametersModificationRange,

iE-Extensions				ProtocolExtensionContainer { { SSBOffsetModificationRange-ExtIEs} }	OPTIONAL,

...

}

SSBOffsetModificationRange-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SSBToReport-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBToReport-List-Item

SSBToReport-List-Item	::= SEQUENCE {

sSBIndex				INTEGER(0..63),

iE-Extensions						ProtocolExtensionContainer { { SSBToReport-List-Item-ExtIEs} }	OPTIONAL,

...

}

SSBToReport-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SSB-transmissionPeriodicity	::= ENUMERATED {sf10, sf20, sf40, sf80, sf160, sf320, sf640, ...}

SSB-transmissionTimingOffset ::= INTEGER (0..127, ...)

SSB-transmissionBitmap ::= CHOICE {

shortBitmap			BIT STRING (SIZE (4)),

mediumBitmap		BIT STRING (SIZE (8)),

longBitmap			BIT STRING (SIZE (64)),

choice-extension	ProtocolIE-Single-Container { { SSB-transmisisonBitmap-ExtIEs} }

}

SSB-transmisisonBitmap-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SuccessfulHOReportInformation	::= SEQUENCE (SIZE(1.. maxnoofSuccessfulHOReports)) OF SuccessfulHOReportList-Item

SuccessfulHOReportList-Item	::= SEQUENCE {

successfulHOReport					SuccessfulHOReportContainer,

iE-Extensions						ProtocolExtensionContainer { { SuccessfulHOReportList-Item-ExtIEs} }	OPTIONAL,

...

}

SuccessfulHOReportList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SuccessfulHOReportContainer	::= OCTET STRING

SUL-FrequencyBand ::= INTEGER (1..1024)

SUL-Information ::= SEQUENCE {

sulFrequencyInfo			NRARFCN,

sulTransmissionBandwidth	NRTransmissionBandwidth,

iE-Extensions				ProtocolExtensionContainer { {SUL-Information-ExtIEs} } OPTIONAL,

...

}

SUL-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-CarrierList				CRITICALITY ignore	EXTENSION NRCarrierList			PRESENCE optional }|

{ ID id-FrequencyShift7p5khz	CRITICALITY ignore	EXTENSION FrequencyShift7p5khz	PRESENCE optional },

...

}

Supported-MBS-FSA-ID-List ::= SEQUENCE (SIZE(1..maxnoofMBSFSAs)) OF MBS-FrequencySelectionArea-Identity

SupportedSULBandList ::= SEQUENCE (SIZE(1..maxnoofNRCellBands)) OF SupportedSULBandItem

SupportedSULBandItem ::= SEQUENCE {

sulBandItem					SUL-FrequencyBand,

iE-Extensions				ProtocolExtensionContainer { {SupportedSULBandItem-ExtIEs} } OPTIONAL,

...

}

SupportedSULBandItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SurvivalTime ::= INTEGER (0..1920000, ...)

SymbolAllocation-in-Slot ::= CHOICE {

allDL				SymbolAllocation-in-Slot-AllDL,

allUL				SymbolAllocation-in-Slot-AllUL,

bothDLandUL			SymbolAllocation-in-Slot-BothDLandUL,

choice-extension	ProtocolIE-Single-Container { {SymbolAllocation-in-Slot-ExtIEs} }

}

SymbolAllocation-in-Slot-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SymbolAllocation-in-Slot-AllDL ::= SEQUENCE {

iE-Extension		ProtocolExtensionContainer { {SymbolAllocation-in-Slot-AllDL-ExtIEs} }	OPTIONAL,

...

}

SymbolAllocation-in-Slot-AllDL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SymbolAllocation-in-Slot-AllUL ::= SEQUENCE {

iE-Extension		ProtocolExtensionContainer { {SymbolAllocation-in-Slot-AllUL-ExtIEs} }	OPTIONAL,

...

}

SymbolAllocation-in-Slot-AllUL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SymbolAllocation-in-Slot-BothDLandUL ::= SEQUENCE {

numberofDLSymbols	INTEGER (0..13),

numberofULSymbols	INTEGER (0..13),

iE-Extension		ProtocolExtensionContainer { {SymbolAllocation-in-Slot-BothDLandUL-ExtIEs} }	OPTIONAL,

...

}

SymbolAllocation-in-Slot-BothDLandUL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-permutation		CRITICALITY ignore	EXTENSION Permutation	PRESENCE optional },

...

}

#### -- T

TABasedMDT ::= SEQUENCE {

tAListforMDT		TAListforMDT,

iE-Extensions		ProtocolExtensionContainer { {TABasedMDT-ExtIEs} } OPTIONAL,

...

}

TABasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAIBasedMDT ::= SEQUENCE {

tAIListforMDT			TAIListforMDT,

iE-Extensions			ProtocolExtensionContainer { {TAIBasedMDT-ExtIEs} } OPTIONAL,

...

}

TAIBasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAIforMDT-Item

TAIforMDT-Item ::= SEQUENCE {

plmn-ID				PLMN-Identity,

tAC						TAC,

iE-Extensions			ProtocolExtensionContainer { {TAIforMDT-Item-ExtIEs} } OPTIONAL,

...

}

TAIforMDT-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAC ::= OCTET STRING (SIZE (3))

TAINSAGSupportList ::= SEQUENCE (SIZE(1..maxnoofNSAGs)) OF TAINSAGSupportItem

TAINSAGSupportItem ::= SEQUENCE {

nSAG-ID								NSAG-ID,

nSAGSliceSupportList			ExtendedSliceSupportList,

iE-Extensions		ProtocolExtensionContainer { {TAINSAGSupportItem-ExtIEs} } OPTIONAL,

...

}

TAINSAGSupportItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAISupport-List	::= SEQUENCE (SIZE(1..maxnoofsupportedTACs)) OF TAISupport-Item

TAISupport-Item ::= SEQUENCE {

tac								TAC,

broadcastPLMNs					SEQUENCE (SIZE(1..maxnoofsupportedPLMNs)) OF BroadcastPLMNinTAISupport-Item,

iE-Extensions					ProtocolExtensionContainer { {TAISupport-Item-ExtIEs} } OPTIONAL,

...

}

TAISupport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAC

TABasedQMC ::= SEQUENCE {

tAListforQMC		TAListforQMC,

iE-Extensions		ProtocolExtensionContainer { {TABasedQMC-ExtIEs} } OPTIONAL,

...

}

TABasedQMC-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAListforQMC ::= SEQUENCE (SIZE(1..maxnoofTAforQMC)) OF TAC

TAIBasedQMC ::= SEQUENCE {

tAIListforQMC		TAIListforQMC,

iE-Extensions		ProtocolExtensionContainer { {TAIBasedQMC-ExtIEs} } OPTIONAL,

...

}

TAIBasedQMC-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListforQMC ::= SEQUENCE (SIZE(1..maxnoofTAforQMC)) OF TAI-Item

TAI-Item ::= SEQUENCE {

tAC					TAC,

pLMN-Identity		PLMN-Identity,

iE-Extensions		ProtocolExtensionContainer { {TAI-Item-ExtIEs} } OPTIONAL,

...

}

TAI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TargetCellinEUTRAN  ::= OCTET STRING -- This IE is to be encoded according to Global Cell ID in the Last Visited E-UTRAN Cell Information IE, as defined in in TS 36.413 [31]

Target-CGI ::= CHOICE {

nr							NR-CGI,

e-utra						E-UTRA-CGI,

choice-extension			ProtocolIE-Single-Container { {TargetCGI-ExtIEs} }

}

TargetCGI-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

TDDULDLConfigurationCommonNR ::= OCTET STRING

TargetCellList ::= SEQUENCE (SIZE(1..maxnoofCHOcells)) OF TargetCellList-Item

TargetCellList-Item ::= SEQUENCE {

target-cell								Target-CGI,

iE-Extensions							ProtocolExtensionContainer { { TargetCellList-Item-ExtIEs} } OPTIONAL

}

TargetCellList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Threshold-RSRQ ::= INTEGER(0..34)

Threshold-RSRP ::= INTEGER(0..97)

Threshold-SINR ::= INTEGER(0..127)

TimeSynchronizationAssistanceInformation ::= SEQUENCE {

timeDistributionIndication					ENUMERATED {enabled, disabled, ...},

uuTimeSynchronizationErrorBudget			INTEGER (0..1000000, ...)				OPTIONAL,

-- The above IE shall be present if the Time Distribution Indication IE is set to the value “enabled”

ie-Extension								ProtocolExtensionContainer { { TimeSynchronizationAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

TimeSynchronizationAssistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TimeToTrigger ::= ENUMERATED {ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256, ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560, ms5120}

TimeToWait ::= ENUMERATED {

v1s,

v2s,

v5s,

v10s,

v20s,

v60s,

...

}

TMGI ::=  OCTET STRING (SIZE(6))

TNLConfigurationInfo ::= SEQUENCE {

extendedUPTransportLayerAddressesToAdd			ExtTLAs										OPTIONAL,

extendedUPTransportLayerAddressesToRemove		ExtTLAs										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TNLConfigurationInfo-ExtIEs} }	OPTIONAL,

...

}

TNLConfigurationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-To-Add-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-To-Add-Item

TNLA-To-Add-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

tNLAssociationUsage						TNLAssociationUsage,

iE-Extensions							ProtocolExtensionContainer { { TNLA-To-Add-Item-ExtIEs} } OPTIONAL

}

TNLA-To-Add-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-To-Update-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-To-Update-Item

TNLA-To-Update-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

tNLAssociationUsage						TNLAssociationUsage 	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { TNLA-To-Update-Item-ExtIEs} } OPTIONAL

}

TNLA-To-Update-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-To-Remove-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-To-Remove-Item

TNLA-To-Remove-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

iE-Extensions							ProtocolExtensionContainer { { TNLA-To-Remove-Item-ExtIEs} } OPTIONAL

}

TNLA-To-Remove-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-Setup-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-Setup-Item

TNLA-Setup-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

iE-Extensions							ProtocolExtensionContainer { { TNLA-Setup-Item-ExtIEs} } OPTIONAL,

...

}

TNLA-Setup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-Failed-To-Setup-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-Failed-To-Setup-Item

TNLA-Failed-To-Setup-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { TNLA-Failed-To-Setup-Item-ExtIEs} } OPTIONAL

}

TNLA-Failed-To-Setup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLAssociationUsage ::= ENUMERATED {

ue,

non-ue,

both,

...

}

TransportLayerAddress ::= BIT STRING (SIZE(1..160, ...))

TraceActivation ::= SEQUENCE {

ng-ran-TraceID			NG-RANTraceID,

interfaces-to-trace 	BIT STRING { ng-c (0), x-nc (1), uu (2), f1-c (3), e1 (4)} (SIZE(8)),

trace-depth 			Trace-Depth,

trace-coll-address		TransportLayerAddress,

ie-Extension 			ProtocolExtensionContainer { {TraceActivation-ExtIEs} } OPTIONAL,

...

}

TraceActivation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

-- Extension to support MDT –

{ ID id-TraceCollectionEntityURI	CRITICALITY ignore	EXTENSION URIaddress				PRESENCE optional}|

{ ID id-MDT-Configuration			CRITICALITY ignore	EXTENSION MDT-Configuration			PRESENCE optional},

...

}

Trace-Depth ::= ENUMERATED {

minimum,

medium,

maximum,

minimumWithoutVendorSpecificExtension,

mediumWithoutVendorSpecificExtension,

maximumWithoutVendorSpecificExtension,

...

}

TrafficIndex ::= INTEGER (1..1024, ...)

TrafficProfile ::= CHOICE {

uPTraffic					QoSFlowLevelQoSParameters,

nonUPTraffic				NonUPTraffic,

choice-extension			ProtocolIE-Single-Container { {TrafficProfile-ExtIEs} }

}

TrafficProfile-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

TrafficReleaseType ::= CHOICE {

fullRelease				AllTrafficIndication,

partialRelease			TrafficToBeRelease-List,

choice-extension		ProtocolIE-Single-Container { {TrafficReleaseType-ExtIEs} }

}

TrafficReleaseType-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

TrafficToBeReleaseInformation ::= SEQUENCE {

releaseType			TrafficReleaseType,

ie-Extensions 			ProtocolExtensionContainer { {TrafficToBeReleaseInformation-ExtIEs} } OPTIONAL,

...

}

TrafficToBeReleaseInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TrafficToBeRelease-List ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficToBeRelease-Item

TrafficToBeRelease-Item ::= SEQUENCE {

trafficIndex			TrafficIndex,

bHInfoList				BHInfoList		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {TrafficToBeRelease-Item-ExtIEs} }	OPTIONAL,

...

}

TrafficToBeRelease-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TSCTrafficCharacteristics ::= SEQUENCE {

tSCAssistanceInformationDownlink	TSCAssistanceInformation OPTIONAL,

tSCAssistanceInformationUplink		TSCAssistanceInformation OPTIONAL,

ie-Extension 			ProtocolExtensionContainer { {TSCTrafficCharacteristics-ExtIEs} } OPTIONAL,

...

}

TSCTrafficCharacteristics-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TSCAssistanceInformation ::= SEQUENCE {

periodicity			INTEGER (0.. 640000, ...),

burstArrivalTime	OCTET STRING				OPTIONAL,

ie-Extension 			ProtocolExtensionContainer { { TSCAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

TSCAssistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{	ID id-SurvivalTime			CRITICALITY ignore	EXTENSION SurvivalTime			PRESENCE optional},

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

#### -- U

UEAggregateMaximumBitRate ::= SEQUENCE {

dl-UE-AMBR				BitRate,

ul-UE-AMBR				BitRate,

iE-Extension			ProtocolExtensionContainer { {UEAggregateMaximumBitRate-ExtIEs} } OPTIONAL,

...

}

UEAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEAppLayerMeasConfigInfo ::= SEQUENCE {

qOEReference					QOEReference,

qOEMeasConfigAppLayerID			QOEMeasConfAppLayerID				OPTIONAL,

serviceType						ServiceType,

qOEMeasStatus					QOEMeasStatus						OPTIONAL,

containerAppLayerMeasConfig	ContainerAppLayerMeasConfig				OPTIONAL,

mDTAlignmentInfo				MDTAlignmentInfo					OPTIONAL,

measCollectionEntityIPAddress	MeasCollectionEntityIPAddress		OPTIONAL,

areaScopeOfQMC					AreaScopeOfQMC						OPTIONAL,

s-NSSAIListQoE					S-NSSAIListQoE						OPTIONAL,

availableRVQoEMetrics			AvailableRVQoEMetrics				OPTIONAL,

iE-Extension					ProtocolExtensionContainer { {UEAppLayerMeasConfigInfo-ExtIEs} } OPTIONAL,

...

}

UEAppLayerMeasConfigInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEContextKeptIndicator ::= ENUMERATED {true, ...}

UEContextID ::= CHOICE {

rRCResume				UEContextIDforRRCResume,

rRRCReestablishment		UEContextIDforRRCReestablishment,

choice-extension		ProtocolIE-Single-Container { {UEContextID-ExtIEs} }

}

UEContextID-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UEContextIDforRRCResume ::= SEQUENCE {

i-rnti					I-RNTI,

allocated-c-rnti			C-RNTI,

accessPCI				NG-RAN-CellPCI,

iE-Extension			ProtocolExtensionContainer { {UEContextIDforRRCResume-ExtIEs} } OPTIONAL,

...

}

UEContextIDforRRCResume-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEContextIDforRRCReestablishment ::= SEQUENCE {

c-rnti					C-RNTI,

failureCellPCI			NG-RAN-CellPCI,

iE-Extension			ProtocolExtensionContainer { {UEContextIDforRRCReestablishment-ExtIEs} } OPTIONAL,

...

}

UEContextIDforRRCReestablishment-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEContextInfoRetrUECtxtResp ::= SEQUENCE {

ng-c-UE-signalling-ref					AMF-UE-NGAP-ID,

signalling-TNL-at-source				CPTransportLayerInformation,

ueSecurityCapabilities					UESecurityCapabilities,

securityInformation						AS-SecurityInformation,

ue-AMBR									UEAggregateMaximumBitRate,

pduSessionResourcesToBeSetup-List		PDUSessionResourcesToBeSetup-List,

rrc-Context								OCTET STRING,

mobilityRestrictionList					MobilityRestrictionList									OPTIONAL,

indexToRatFrequencySelectionPriority	RFSP-Index												OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {UEContextInfoRetrUECtxtResp-ExtIEs} } 	OPTIONAL,

...

}

UEContextInfoRetrUECtxtResp-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-FiveGCMobilityRestrictionListContainer 	CRITICALITY ignore	EXTENSION FiveGCMobilityRestrictionListContainer		PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitRate		CRITICALITY ignore	EXTENSION NRUESidelinkAggregateMaximumBitRate			PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitRate	CRITICALITY ignore	EXTENSION LTEUESidelinkAggregateMaximumBitRate			PRESENCE optional }|

{ ID id-UERadioCapabilityID						CRITICALITY reject	EXTENSION UERadioCapabilityID								PRESENCE optional }|

{ ID id-MBS-SessionInformation-List				CRITICALITY ignore	EXTENSION MBS-SessionInformation-List						PRESENCE optional }|

{ ID id-NoPDUSessionIndication					CRITICALITY ignore	EXTENSION NoPDUSessionIndication							PRESENCE optional }|

{ ID id-FiveGProSeUEPC5AggregateMaximumBitRate	CRITICALITY ignore	EXTENSION NRUESidelinkAggregateMaximumBitRate			PRESENCE optional }|

{ ID id-UESliceMaximumBitRateList				CRITICALITY ignore	EXTENSION UESliceMaximumBitRateList						PRESENCE optional }|

{ ID id-PositioningInformation					CRITICALITY ignore	EXTENSION PositioningInformation							PRESENCE optional },

...

}

UEHistoryInformation ::= SEQUENCE (SIZE(1..maxnoofCellsinUEHistoryInfo)) OF LastVisitedCell-Item

UEHistoryInformationFromTheUE ::= CHOICE {

nR						NRMobilityHistoryReport,

choice-extension			ProtocolIE-Single-Container { {UEHistoryInformationFromTheUE-ExtIEs} }

}

UEHistoryInformationFromTheUE-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UEIdentityIndexValue ::= CHOICE {

indexLength10				BIT STRING (SIZE(10)),

choice-extension			ProtocolIE-Single-Container { {UEIdentityIndexValue-ExtIEs} }

}

UEIdentityIndexValue-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UEIdentityIndexList-MBSGroupPaging ::= SEQUENCE (SIZE(1..maxnoofUEIDIndicesforMBSPaging)) OF UEIdentityIndexList-MBSGroupPaging-Item

UEIdentityIndexList-MBSGroupPaging-Item ::= SEQUENCE {

ueIdentityIndexList-MBSGroupPagingValue		UEIdentityIndexList-MBSGroupPagingValue,

pagingDRX									UESpecificDRX		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {UEIdentityIndexList-MBSGroupPaging-Item-ExtIEs} } 	OPTIONAL,

...

}

UEIdentityIndexList-MBSGroupPaging-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEIdentityIndexList-MBSGroupPagingValue ::= CHOICE {

uEIdentityIndexValueMBSGroupPaging		BIT STRING (SIZE(10)),

choice-extension			ProtocolIE-Single-Container { {UEIdentityIndexValueMBSGroupPaging-ExtIEs} }

}

UEIdentityIndexValueMBSGroupPaging-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UERadioCapabilityForPaging ::= SEQUENCE {

uERadioCapabilityForPagingOfNR			UERadioCapabilityForPagingOfNR			OPTIONAL,

uERadioCapabilityForPagingOfEUTRA		UERadioCapabilityForPagingOfEUTRA		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UERadioCapabilityForPaging-ExtIEs} }	OPTIONAL,

...

}

UERadioCapabilityForPaging-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UERadioCapabilityForPagingOfNR ::= OCTET STRING

UERadioCapabilityForPagingOfEUTRA ::= OCTET STRING

UERadioCapabilityID ::= OCTET STRING

UERANPagingIdentity ::= CHOICE {

i-RNTI-full			BIT STRING ( SIZE (40)),

choice-extension	ProtocolIE-Single-Container { {UERANPagingIdentity-ExtIEs} }

}

UERANPagingIdentity-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UERLFReportContainer ::= CHOICE {

nR-UERLFReportContainer			UERLFReportContainerNR,

lTE-UERLFReportContainer		UERLFReportContainerLTE,

choice-Extension		ProtocolIE-Single-Container { {UERLFReportContainer-ExtIEs} }

}

UERLFReportContainer-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UERLFReportContainerLTE ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the RLF-Report-r9 IE contained in the UEInformationResponse message (TS 36.331 [14])

UERLFReportContainerNR ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the nr-RLF-Report-r16 IE contained in the UEInformationResponse message (TS 38.331 [10])

UESliceMaximumBitRateList ::= SEQUENCE (SIZE(1.. maxnoofSMBR)) OF UESliceMaximumBitRate-Item

UESliceMaximumBitRate-Item ::= SEQUENCE {

s-NSSAI						S-NSSAI,

dl-UE-Slice-MBR				BitRate,

ul-UE-Slice-MBR				BitRate,

iE-Extensions						ProtocolExtensionContainer { { UESliceMaximumBitRate-Item-ExtIEs} } OPTIONAL,

...

}

UESliceMaximumBitRate-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UESecurityCapabilities ::= SEQUENCE {

nr-EncyptionAlgorithms					BIT STRING {nea1-128(1),

nea2-128(2),

nea3-128(3)} (SIZE(16, ...)),

nr-IntegrityProtectionAlgorithms		BIT STRING {nia1-128(1),

nia2-128(2),

nia3-128(3)} (SIZE(16, ...)),

e-utra-EncyptionAlgorithms				BIT STRING {eea1-128(1),

eea2-128(2),

eea3-128(3)} (SIZE(16, ...)),

e-utra-IntegrityProtectionAlgorithms	BIT STRING {eia1-128(1),

eia2-128(2),

eia3-128(3)} (SIZE(16, ...)),

iE-Extension			ProtocolExtensionContainer { {UESecurityCapabilities-ExtIEs} } OPTIONAL,

...

}

UESecurityCapabilities-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UESpecificDRX ::= ENUMERATED {

v32,

v64,

v128,

v256,

...

}

ULConfiguration::= SEQUENCE {

uL-PDCP							UL-UE-Configuration,

iE-Extensions					ProtocolExtensionContainer { {ULConfiguration-ExtIEs} } OPTIONAL,

...

}

ULConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UL-UE-Configuration::= ENUMERATED {no-data, shared, only, ...}

ULF1Terminating-BHInfo ::= SEQUENCE {

ingressBAPRoutingID				BAPRoutingID,

ingressBHRLCCHID				BHRLCChannelID,

iE-Extensions		ProtocolExtensionContainer { { ULF1Terminating-BHInfo-ExtIEs} } OPTIONAL,

...

}

ULF1Terminating-BHInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ULNonF1Terminating-BHInfo ::= SEQUENCE {

egressBAPRoutingID			BAPRoutingID,

egressBHRLCCHID				BHRLCChannelID,

nexthopBAPAddress			BAPAddress,

iE-Extensions		ProtocolExtensionContainer { { ULNonF1Terminating-BHInfo-ExtIEs} } OPTIONAL,

...

}

ULNonF1Terminating-BHInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ULForwarding	::= ENUMERATED {ul-forwarding-proposed, ...}

ULForwardingProposal	::= ENUMERATED {ul-forwarding-proposed, ...}

UL-GBR-PRB-usage::= INTEGER (0..100)

UL-GBR-PRB-usage-for-MIMO::= INTEGER (0..100)

UL-non-GBR-PRB-usage::= INTEGER (0..100)

UL-non-GBR-PRB-usage-for-MIMO::= INTEGER (0..100)

UL-Total-PRB-usage::= INTEGER (0..100)

UL-Total-PRB-usage-for-MIMO::= INTEGER (0..100)

UPTransportLayerInformation ::= CHOICE {

gtpTunnel					GTPtunnelTransportLayerInformation,

choice-extension			ProtocolIE-Single-Container { {UPTransportLayerInformation-ExtIEs} }

}

UPTransportLayerInformation-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UPTransportParameters ::= SEQUENCE (SIZE(1..maxnoofSCellGroupsplus1)) OF UPTransportParametersItem

UPTransportParametersItem ::= SEQUENCE {

upTNLInfo		UPTransportLayerInformation,

cellGroupID		CellGroupID,

iE-Extension	ProtocolExtensionContainer { {UPTransportParametersItem-ExtIEs} } OPTIONAL,

...

}

UPTransportParametersItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-QoS-Mapping-Information	CRITICALITY reject	EXTENSION QoS-Mapping-Information	PRESENCE optional},

...

}

UserPlaneTrafficActivityReport ::= ENUMERATED {inactive, re-activated, ...}

URIaddress ::= VisibleString

#### -- V

VehicleUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

VolumeTimedReportList ::= SEQUENCE (SIZE(1..maxnooftimeperiods)) OF VolumeTimedReport-Item

VolumeTimedReport-Item ::= SEQUENCE {

startTimeStamp				OCTET STRING (SIZE(4)),

endTimeStamp					OCTET STRING (SIZE(4)),

usageCountUL					INTEGER (0..18446744073709551615),

usageCountDL					INTEGER (0..18446744073709551615),

iE-Extensions				ProtocolExtensionContainer { {VolumeTimedReport-Item-ExtIEs} } OPTIONAL,

...

}

VolumeTimedReport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- W

WLANMeasurementConfiguration ::= SEQUENCE {

wlanMeasConfig				WLANMeasConfig,

wlanMeasConfigNameList		WLANMeasConfigNameList				OPTIONAL,

wlan-rssi					ENUMERATED {true, ...}				OPTIONAL,

wlan-rtt					ENUMERATED {true, ...}				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { WLANMeasurementConfiguration-ExtIEs } } OPTIONAL,

...

}

WLANMeasurementConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

WLANMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofWLANName)) OF WLANName

WLANMeasConfig::= ENUMERATED {setup,...}

WLANName ::= OCTET STRING (SIZE (1..32))

#### -- X

XnBenefitValue ::= INTEGER (1..8, ...)

#### -- Y

#### -- Z

END

-- ASN1STOP

9.3.6	Common definitions

-- ASN1START

-- **************************************************************

--

-- Common definitions

--

-- **************************************************************

XnAP-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-CommonDataTypes (3) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs 									INTEGER ::= 65535

maxProtocolExtensions 							INTEGER ::= 65535

maxProtocolIEs									INTEGER ::= 65535

-- **************************************************************

--

#### -- Common Data Types

--

-- **************************************************************

Criticality		::= ENUMERATED { reject, ignore, notify }

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local				INTEGER (0.. maxPrivateIEs),

global				OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolIE-ID		::= INTEGER (0..maxProtocolIEs)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessful-outcome}

END

-- ASN1STOP

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= 27

id-traceStart													ProcedureCode ::= 28

id-handoverSuccess												ProcedureCode ::= 29

id-conditionalHandoverCancel									ProcedureCode ::= 30

id-earlyStatusTransfer											ProcedureCode ::= 31

id-failureIndication												ProcedureCode ::= 32

id-handoverReport												ProcedureCode ::= 33

id-resourceStatusReportingInitiation							ProcedureCode ::= 34

id-resourceStatusReporting										ProcedureCode ::= 35

id-mobilitySettingsChange										ProcedureCode ::= 36

id-accessAndMobilityIndication									ProcedureCode ::= 37

id-cellTrafficTrace 											ProcedureCode ::= 38

id-RANMulticastGroupPaging										ProcedureCode ::= 39

id-scgFailureInformationReport									ProcedureCode ::= 40

id-ProcedureCode41-NotToBeUsed									ProcedureCode ::= 41

id-scgFailureTransfer											ProcedureCode ::= 42

id-f1CTrafficTransfer											ProcedureCode ::= 43

id-iABTransportMigrationManagement								ProcedureCode ::= 44

id-iABTransportMigrationModification							ProcedureCode ::= 45

id-iABResourceCoordination										ProcedureCode ::= 46

id-retrieveUEContextConfirm										ProcedureCode ::= 47

id-cPCCancel													ProcedureCode ::= 48

id-partialUEContextTransfer										ProcedureCode ::= 49

id-rachIndicaion												ProcedureCode ::= xx

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 262143

maxnoofAllowedAreas							INTEGER ::= 16

maxnoofAMFRegions							INTEGER ::= 16

maxnoofAoIs									INTEGER ::= 64

maxnoofBluetoothName						INTEGER ::= 4

maxnoofBPLMNs								INTEGER ::= 12

maxnoofCAGs									INTEGER ::= 12

maxnoofCAGsperPLMN							INTEGER ::= 256

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofCellsinAoI							INTEGER ::= 256

maxnoofCellsinUEHistoryInfo					INTEGER ::= 16

maxnoofCellsinNG-RANnode					INTEGER ::= 16384

maxnoofCellsinRNA							INTEGER ::= 32

maxnoofCellsUEMovingTrajectory				INTEGER ::= 16

maxnoofDRBs									INTEGER ::= 32

maxnoofEUTRABands							INTEGER ::= 16

maxnoofEUTRABPLMNs							INTEGER ::= 6

maxnoofEPLMNs								INTEGER ::= 15

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofEPLMNsplus1							INTEGER ::= 16

maxnoofForbiddenTACs							INTEGER ::= 4096

maxnoofFreqforMDT							INTEGER ::= 8

maxnoofMBSFNEUTRA							INTEGER ::= 8

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofMultiConnectivityMinusOne			INTEGER ::= 3

maxnoofNeighbours							INTEGER ::= 1024

maxnoofNeighPCIforMDT						INTEGER ::= 32

maxnoofNIDs									INTEGER ::= 12

maxnoofNRCellBands							INTEGER ::= 32

maxnoofPLMNs								INTEGER ::= 16

maxnoofPDUSessions							INTEGER ::= 256

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoofQoSFlows								INTEGER ::= 64

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofRANAreaCodes							INTEGER ::= 32

maxnoofRANAreasinRNA						INTEGER ::= 16

maxnoofRANNodesinAoI						INTEGER ::= 64

maxnoofSCellGroups							INTEGER ::= 3

maxnoofSCellGroupsplus1						INTEGER ::= 4

maxnoofSensorName							INTEGER ::= 3

maxnoofSliceItems							INTEGER ::= 1024

maxnoofSNPNIDs								INTEGER ::= 12

maxnoofsupportedPLMNs						INTEGER ::= 12

maxnoofsupportedTACs						INTEGER ::= 256

maxnoofTAforMDT								INTEGER ::= 8

maxnoofTAI									INTEGER ::= 16

maxnoofTAIsinAoI 							INTEGER ::= 16

maxnooftimeperiods							INTEGER ::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofUEContexts							INTEGER ::= 8192

maxNRARFCN									INTEGER ::= 3279165

maxNrOfErrors								INTEGER ::= 256

maxnoofslots								INTEGER ::= 5120

maxnoofExtTLAs								INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofCHOcells								INTEGER ::= 8

maxnoofPC5QoSFlows 							INTEGER ::= 2064

maxnoofSSBAreas								INTEGER ::= 64

maxnoofRACHReports							INTEGER ::= 64

maxnoofNRSCSs								INTEGER ::= 5

maxnoofPhysicalResourceBlocks				INTEGER ::= 275

maxnoofAdditionalPDCPDuplicationTNL			INTEGER ::= 2

maxnoofRLCDuplicationstate					INTEGER ::= 3

maxnoofWLANName								INTEGER ::= 4

maxnoofNonAnchorCarrierFreqConfig			INTEGER ::= 15

maxnoofDataForwardingTunneltoE-UTRAN		INTEGER ::= 256

maxnoofMBSFSAs								INTEGER ::= 256

maxnoofUEIDIndicesforMBSPaging				INTEGER ::= 4096

maxnoofMBSQoSFlows							INTEGER ::= 64

maxnoofMRBs									INTEGER ::= 32

maxnoofCellsforMBS							INTEGER ::= 8192

maxnoofMBSServiceAreaInformation				INTEGER ::= 256

maxnoofTAIforMBS 							INTEGER ::= 1024

maxnoofAssociatedMBSSessions				INTEGER ::= 32

maxnoofMBSSessions							INTEGER ::= 256

maxnoofSuccessfulHOReports					INTEGER ::= 64

maxnoofPSCellsPerSN 						INTEGER ::= 8

maxnoofNR-UChannelIDs						INTEGER ::= 16

maxnoofCellsinCHO							INTEGER ::= 8

maxnoofCHOexecutioncond						INTEGER ::= 2

maxnoofServedCellsIAB						INTEGER ::= 512

maxnoofServingCells							INTEGER ::= 32

maxnoofBHInfo								INTEGER ::= 1024

maxnoofTrafficIndexEntries					INTEGER	::=	1024

maxnoofTLAsIAB								INTEGER	::=	1024

maxnoofBAPControlPDURLCCHs					INTEGER ::= 2

maxnoofIABSTCInfo							INTEGER ::= 45

maxnoofSymbols								INTEGER ::= 14

maxnoofDUFSlots								INTEGER ::= 320

maxnoofHSNASlots							INTEGER ::= 5120

maxnoofRBsetsPerCell						INTEGER ::= 8

maxnoofRBsetsPerCell1						INTEGER ::= 7

maxnoofChildIABNodes						INTEGER ::= 1024

maxnoofPSCellCandidates						INTEGER ::= 8

maxnoofTargetSNs 							INTEGER ::= 8

maxnoofUEAppLayerMeas						INTEGER ::= 16

maxnoofSNSSAIforQMC							INTEGER ::= 16

maxnoofCellIDforQMC							INTEGER ::= 32

maxnoofPLMNforQMC							INTEGER ::= 16

maxnoofTAforQMC								INTEGER ::= 8

maxnoofMTCItems								INTEGER ::= 16

maxnoofCSIRSconfigurations					INTEGER ::= 96

maxnoofCSIRSneighbourCells					INTEGER ::= 16

maxnoofCSIRSneighbourCellsInMTC				INTEGER ::= 16

maxnoofNeighbour-NG-RAN-Nodes				INTEGER ::= 256

maxnoofSRBs									INTEGER ::= 5

maxnoofSMBR									INTEGER ::= 8

maxnoofNSAGs								INTEGER ::= 256

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation																	ProtocolIE-ID ::= 1

id-admittedSplitSRB																					ProtocolIE-ID ::= 2

id-admittedSplitSRBrelease																			ProtocolIE-ID ::= 3

id-AMF-Region-Information																			ProtocolIE-ID ::= 4

id-AssistanceDataForRANPaging																		ProtocolIE-ID ::= 5

id-BearersSubjectToCounterCheck																		ProtocolIE-ID ::= 6

id-Cause																							ProtocolIE-ID ::= 7

id-cellAssistanceInfo-NR																			ProtocolIE-ID ::= 8

id-ConfigurationUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 9

id-CriticalityDiagnostics																			ProtocolIE-ID ::= 10

id-XnUAddressInfoperPDUSession-List																	ProtocolIE-ID ::= 11

id-DRBsSubjectToStatusTransfer-List																	ProtocolIE-ID ::= 12

id-ExpectedUEBehaviour																				ProtocolIE-ID ::= 13

id-GlobalNG-RAN-node-ID																				ProtocolIE-ID ::= 14

id-GUAMI																							ProtocolIE-ID ::= 15

id-indexToRatFrequSelectionPriority																	ProtocolIE-ID ::= 16

id-initiatingNodeType-ResourceCoordRequest															ProtocolIE-ID ::= 17

id-List-of-served-cells-E-UTRA																		ProtocolIE-ID ::= 18

id-List-of-served-cells-NR																			ProtocolIE-ID ::= 19

id-LocationReportingInformation																		ProtocolIE-ID ::= 20

id-MAC-I																							ProtocolIE-ID ::= 21

id-MaskedIMEISV																						ProtocolIE-ID ::= 22

id-M-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 23

id-MN-to-SN-Container																				ProtocolIE-ID ::= 24

id-MobilityRestrictionList																			ProtocolIE-ID ::= 25

id-new-NG-RAN-Cell-Identity																			ProtocolIE-ID ::= 26

id-newNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 27

id-UEReportRRCTransfer																				ProtocolIE-ID ::= 28

id-oldNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 29

id-OldtoNewNG-RANnodeResumeContainer																ProtocolIE-ID ::= 30

id-PagingDRX																						ProtocolIE-ID ::= 31

id-PCellID																							ProtocolIE-ID ::= 32

id-PDCPChangeIndication																				ProtocolIE-ID ::= 33

id-PDUSessionAdmittedAddedAddReqAck																	ProtocolIE-ID ::= 34

id-PDUSessionAdmittedModSNModConfirm																ProtocolIE-ID ::= 35

id-PDUSessionAdmitted-SNModResponse																	ProtocolIE-ID ::= 36

id-PDUSessionNotAdmittedAddReqAck																	ProtocolIE-ID ::= 37

id-PDUSessionNotAdmitted-SNModResponse																ProtocolIE-ID ::= 38

id-PDUSessionReleasedList-RelConf																	ProtocolIE-ID ::= 39

id-PDUSessionReleasedSNModConfirm																	ProtocolIE-ID ::= 40

id-PDUSessionResourcesActivityNotifyList															ProtocolIE-ID ::= 41

id-PDUSessionResourcesAdmitted-List																	ProtocolIE-ID ::= 42

id-PDUSessionResourcesNotAdmitted-List																ProtocolIE-ID ::= 43

id-PDUSessionResourcesNotifyList																	ProtocolIE-ID ::= 44

id-PDUSession-SNChangeConfirm-List																	ProtocolIE-ID ::= 45

id-PDUSession-SNChangeRequired-List																	ProtocolIE-ID ::= 46

id-PDUSessionToBeAddedAddReq																		ProtocolIE-ID ::= 47

id-PDUSessionToBeModifiedSNModRequired																ProtocolIE-ID ::= 48

id-PDUSessionToBeReleasedList-RelRqd																ProtocolIE-ID ::= 49

id-PDUSessionToBeReleased-RelReq																	ProtocolIE-ID ::= 50

id-PDUSessionToBeReleasedSNModRequired																ProtocolIE-ID ::= 51

id-RANPagingArea																					ProtocolIE-ID ::= 52

id-PagingPriority																					ProtocolIE-ID ::= 53

id-requestedSplitSRB																				ProtocolIE-ID ::= 54

id-requestedSplitSRBrelease																			ProtocolIE-ID ::= 55

id-ResetRequestTypeInfo																				ProtocolIE-ID ::= 56

id-ResetResponseTypeInfo																			ProtocolIE-ID ::= 57

id-RespondingNodeTypeConfigUpdateAck																ProtocolIE-ID ::= 58

id-respondingNodeType-ResourceCoordResponse															ProtocolIE-ID ::= 59

id-ResponseInfo-ReconfCompl																			ProtocolIE-ID ::= 60

id-RRCConfigIndication																				ProtocolIE-ID ::= 61

id-RRCResumeCause																					ProtocolIE-ID ::= 62

id-SCGConfigurationQuery																			ProtocolIE-ID ::= 63

id-selectedPLMN																						ProtocolIE-ID ::= 64

id-ServedCellsToActivate																			ProtocolIE-ID ::= 65

id-servedCellsToUpdate-E-UTRA																		ProtocolIE-ID ::= 66

id-ServedCellsToUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 67

id-servedCellsToUpdate-NR																			ProtocolIE-ID ::= 68

id-s-ng-RANnode-SecurityKey																			ProtocolIE-ID ::= 69

id-S-NG-RANnodeUE-AMBR																				ProtocolIE-ID ::= 70

id-S-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 71

id-SN-to-MN-Container																				ProtocolIE-ID ::= 72

id-sourceNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 73

id-SplitSRB-RRCTransfer																				ProtocolIE-ID ::= 74

id-TAISupport-list																					ProtocolIE-ID ::= 75

id-TimeToWait																						ProtocolIE-ID ::= 76

id-Target2SourceNG-RANnodeTranspContainer															ProtocolIE-ID ::= 77

id-targetCellGlobalID																				ProtocolIE-ID ::= 78

id-targetNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 79

id-target-S-NG-RANnodeID																			ProtocolIE-ID ::= 80

id-TraceActivation																					ProtocolIE-ID ::= 81

id-UEContextID																						ProtocolIE-ID ::= 82

id-UEContextInfoHORequest																			ProtocolIE-ID ::= 83

id-UEContextInfoRetrUECtxtResp																		ProtocolIE-ID ::= 84

id-UEContextInfo-SNModRequest																		ProtocolIE-ID ::= 85

id-UEContextKeptIndicator																			ProtocolIE-ID ::= 86

id-UEContextRefAtSN-HORequest																		ProtocolIE-ID ::= 87

id-UEHistoryInformation																				ProtocolIE-ID ::= 88

id-UEIdentityIndexValue																				ProtocolIE-ID ::= 89

id-UERANPagingIdentity																				ProtocolIE-ID ::= 90

id-UESecurityCapabilities																			ProtocolIE-ID ::= 91

id-UserPlaneTrafficActivityReport																	ProtocolIE-ID ::= 92

id-XnRemovalThreshold 																				ProtocolIE-ID ::= 93

id-DesiredActNotificationLevel																		ProtocolIE-ID ::= 94

id-AvailableDRBIDs 																					ProtocolIE-ID ::= 95

id-AdditionalDRBIDs 																				ProtocolIE-ID ::= 96

id-SpareDRBIDs 																						ProtocolIE-ID ::= 97

id-RequiredNumberOfDRBIDs 																			ProtocolIE-ID ::= 98

id-TNLA-To-Add-List																					ProtocolIE-ID ::= 99

id-TNLA-To-Update-List																				ProtocolIE-ID ::= 100

id-TNLA-To-Remove-List																				ProtocolIE-ID ::= 101

id-TNLA-Setup-List																					ProtocolIE-ID ::= 102

id-TNLA-Failed-To-Setup-List																		ProtocolIE-ID ::= 103

id-PDUSessionToBeReleased-RelReqAck																	ProtocolIE-ID ::= 104

id-S-NG-RANnodeMaxIPDataRate-UL																		ProtocolIE-ID ::= 105

id-PDUSessionResourceSecondaryRATUsageList															ProtocolIE-ID ::= 107

id-Additional-UL-NG-U-TNLatUPF-List																	ProtocolIE-ID ::= 108

id-SecondarydataForwardingInfoFromTarget-List														ProtocolIE-ID ::= 109

id-LocationInformationSNReporting																	ProtocolIE-ID ::= 110

id-LocationInformationSN																			ProtocolIE-ID ::= 111

id-LastE-UTRANPLMNIdentity																			ProtocolIE-ID ::= 112

id-S-NG-RANnodeMaxIPDataRate-DL																		ProtocolIE-ID ::= 113

id-MaxIPrate-DL																						ProtocolIE-ID ::= 114

id-SecurityResult																					ProtocolIE-ID ::= 115

id-S-NSSAI																							ProtocolIE-ID ::= 116

id-MR-DC-ResourceCoordinationInfo																	ProtocolIE-ID ::= 117

id-AMF-Region-Information-To-Add																	ProtocolIE-ID ::= 118

id-AMF-Region-Information-To-Delete																	ProtocolIE-ID ::= 119

id-OldQoSFlowMap-ULendmarkerexpected																ProtocolIE-ID ::= 120

id-RANPagingFailure																					ProtocolIE-ID ::= 121

id-UERadioCapabilityForPaging																		ProtocolIE-ID ::= 122

id-PDUSessionDataForwarding-SNModResponse															ProtocolIE-ID ::= 123

id-DRBsNotAdmittedSetupModifyList																	ProtocolIE-ID ::= 124

id-Secondary-MN-Xn-U-TNLInfoatM																		ProtocolIE-ID ::= 125

id-NE-DC-TDM-Pattern																				ProtocolIE-ID ::= 126

id-PDUSessionCommonNetworkInstance																	ProtocolIE-ID ::= 127

id-BPLMN-ID-Info-EUTRA																				ProtocolIE-ID ::= 128

id-BPLMN-ID-Info-NR																					ProtocolIE-ID ::= 129

id-InterfaceInstanceIndication																		ProtocolIE-ID ::= 130

id-S-NG-RANnode-Addition-Trigger-Ind																ProtocolIE-ID ::= 131

id-DefaultDRB-Allowed																				ProtocolIE-ID ::= 132

id-DRB-IDs-takenintouse																				ProtocolIE-ID ::= 133

id-SplitSessionIndicator																			ProtocolIE-ID ::= 134

id-CNTypeRestrictionsForEquivalent																	ProtocolIE-ID ::= 135

id-CNTypeRestrictionsForServing																		ProtocolIE-ID ::= 136

id-DRBs-transferred-to-MN																			ProtocolIE-ID ::= 137

id-ULForwardingProposal																				ProtocolIE-ID ::= 138

id-EndpointIPAddressAndPort 																		ProtocolIE-ID ::= 139

id-IntendedTDD-DL-ULConfiguration-NR																ProtocolIE-ID ::= 140

id-TNLConfigurationInfo																				ProtocolIE-ID ::= 141

id-PartialListIndicator-NR																			ProtocolIE-ID ::= 142

id-MessageOversizeNotification																		ProtocolIE-ID ::= 143

id-CellAndCapacityAssistanceInfo-NR																	ProtocolIE-ID ::= 144

id-NG-RANTraceID																					ProtocolIE-ID ::= 145

id-NonGBRResources-Offered																			ProtocolIE-ID ::= 146

id-FastMCGRecoveryRRCTransfer-SN-to-MN																ProtocolIE-ID ::= 147

id-RequestedFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 148

id-AvailableFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 149

id-RequestedFastMCGRecoveryViaSRB3Release															ProtocolIE-ID ::= 150

id-ReleaseFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 151

id-FastMCGRecoveryRRCTransfer-MN-to-SN																ProtocolIE-ID ::= 152

id-ExtendedRATRestrictionInformation																ProtocolIE-ID ::= 153

id-QoSMonitoringRequest																				ProtocolIE-ID ::= 154

id-FiveGCMobilityRestrictionListContainer															ProtocolIE-ID ::= 155

id-PartialListIndicator-EUTRA																		ProtocolIE-ID ::= 156

id-CellAndCapacityAssistanceInfo-EUTRA																ProtocolIE-ID ::= 157

id-CHOinformation-Req																				ProtocolIE-ID ::= 158

id-CHOinformation-Ack																				ProtocolIE-ID ::= 159

id-targetCellsToCancel																				ProtocolIE-ID ::= 160

id-requestedTargetCellGlobalID																		ProtocolIE-ID ::= 161

id-procedureStage																					ProtocolIE-ID ::= 162

id-DAPSRequestInfo																					ProtocolIE-ID ::= 163

id-DAPSResponseInfo-List																			ProtocolIE-ID ::= 164

id-CHO-MRDC-Indicator																				ProtocolIE-ID ::= 165

id-OffsetOfNbiotChannelNumberToDL-EARFCN															ProtocolIE-ID ::= 166

id-OffsetOfNbiotChannelNumberToUL-EARFCN															ProtocolIE-ID ::= 167

id-NBIoT-UL-DL-AlignmentOffset																		ProtocolIE-ID ::= 168

id-LTEV2XServicesAuthorized																			ProtocolIE-ID ::= 169

id-NRV2XServicesAuthorized																			ProtocolIE-ID ::= 170

id-LTEUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 171

id-NRUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 172

id-PC5QoSParameters																					ProtocolIE-ID ::= 173

id-AlternativeQoSParaSetList																		ProtocolIE-ID ::= 174

id-CurrentQoSParaSetIndex																			ProtocolIE-ID ::= 175

id-MobilityInformation 																				ProtocolIE-ID ::= 176

id-InitiatingCondition-FailureIndication															ProtocolIE-ID ::= 177

id-UEHistoryInformationFromTheUE																	ProtocolIE-ID ::= 178

id-HandoverReportType																				ProtocolIE-ID ::= 179

id-HandoverCause																					ProtocolIE-ID ::= 180

id-SourceCellCGI																					ProtocolIE-ID ::= 181

id-TargetCellCGI																					ProtocolIE-ID ::= 182

id-ReEstablishmentCellCGI																			ProtocolIE-ID ::= 183

id-TargetCellinEUTRAN																				ProtocolIE-ID ::= 184

id-SourceCellCRNTI																					ProtocolIE-ID ::= 185

id-UERLFReportContainer																				ProtocolIE-ID ::= 186

id-NGRAN-Node1-Measurement-ID																		ProtocolIE-ID ::= 187

id-NGRAN-Node2-Measurement-ID																		ProtocolIE-ID ::= 188

id-RegistrationRequest																				ProtocolIE-ID ::= 189

id-ReportCharacteristics																				ProtocolIE-ID ::= 190

id-CellToReport																								ProtocolIE-ID ::= 191

id-ReportingPeriodicity																					ProtocolIE-ID ::= 192

id-CellMeasurementResult																				ProtocolIE-ID ::= 193

id-NG-RANnode1CellID																						ProtocolIE-ID ::= 194

id-NG-RANnode2CellID																				ProtocolIE-ID ::= 195

id-NG-RANnode1MobilityParameters																	ProtocolIE-ID ::= 196

id-NG-RANnode2ProposedMobilityParameters															ProtocolIE-ID ::= 197

id-MobilityParametersModificationRange																ProtocolIE-ID ::= 198

id-TDDULDLConfigurationCommonNR																		ProtocolIE-ID ::= 199

id-CarrierList																						ProtocolIE-ID ::= 200

id-ULCarrierList																					ProtocolIE-ID ::= 201

id-FrequencyShift7p5khz																				ProtocolIE-ID ::= 202

id-SSB-PositionsInBurst																				ProtocolIE-ID ::= 203

id-NRCellPRACHConfig																				ProtocolIE-ID ::= 204

id-RACHReportInformation																			ProtocolIE-ID ::= 205

id-IABNodeIndication																				ProtocolIE-ID ::= 206

id-Redundant-UL-NG-U-TNLatUPF																		ProtocolIE-ID ::= 207

id-CNPacketDelayBudgetDownlink																		ProtocolIE-ID ::= 208

id-CNPacketDelayBudgetUplink																		ProtocolIE-ID ::= 209

id-Additional-Redundant-UL-NG-U-TNLatUPF-List														ProtocolIE-ID ::= 210

id-RedundantCommonNetworkInstance																	ProtocolIE-ID ::= 211

id-TSCTrafficCharacteristics																		ProtocolIE-ID ::= 212

id-RedundantQoSFlowIndicator																		ProtocolIE-ID ::= 213

id-Redundant-DL-NG-U-TNLatNG-RAN																	ProtocolIE-ID ::= 214

id-ExtendedPacketDelayBudget																		ProtocolIE-ID ::= 215

id-Additional-PDCP-Duplication-TNL-List																ProtocolIE-ID ::= 216

id-RedundantPDUSessionInformation																	ProtocolIE-ID ::= 217

id-UsedRSNInformation																				ProtocolIE-ID ::= 218

id-RLCDuplicationInformation																		ProtocolIE-ID ::= 219

id-NPN-Broadcast-Information																		ProtocolIE-ID ::= 220

id-NPNPagingAssistanceInformation																	ProtocolIE-ID ::= 221

id-NPNMobilityInformation																			ProtocolIE-ID ::= 222

id-NPN-Support																						ProtocolIE-ID ::= 223

id-MDT-Configuration																				ProtocolIE-ID ::= 224

id-MDTPLMNList																						ProtocolIE-ID ::= 225

id-TraceCollectionEntityURI																			ProtocolIE-ID ::= 226

id-UERadioCapabilityID																				ProtocolIE-ID ::= 227

id-CSI-RSTransmissionIndication																		ProtocolIE-ID ::= 228

id-SNTriggered																						ProtocolIE-ID ::= 229

id-DLCarrierList																					ProtocolIE-ID ::= 230

id-ExtendedTAISliceSupportList																		ProtocolIE-ID ::= 231

id-cellAssistanceInfo-EUTRA																			ProtocolIE-ID ::= 232

id-ConfiguredTACIndication																			ProtocolIE-ID ::= 233

id-secondary-SN-UL-PDCP-UP-TNLInfo																	ProtocolIE-ID ::= 234

id-pdcpDuplicationConfiguration																		ProtocolIE-ID ::= 235

id-duplicationActivation																			ProtocolIE-ID ::= 236

id-NPRACHConfiguration																				ProtocolIE-ID ::= 237

id-QosMonitoringReportingFrequency																	ProtocolIE-ID ::= 238

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated													ProtocolIE-ID ::= 239

id-DL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 240

id-UL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 241

id-SFN-Offset																						ProtocolIE-ID ::= 242

id-QoSMonitoringDisabled																				ProtocolIE-ID ::= 243

id-ExtendedUEIdentityIndexValue																		ProtocolIE-ID ::= 244

id-EUTRAPagingeDRXInformation																		ProtocolIE-ID ::= 245

id-CHO-MRDC-EarlyDataForwarding																		ProtocolIE-ID ::= 246

id-SCGIndicator																						ProtocolIE-ID ::= 247

id-UESpecificDRX																					ProtocolIE-ID ::= 248

id-PDUSessionExpectedUEActivityBehaviour															ProtocolIE-ID ::= 249

id-QoS-Mapping-Information																			ProtocolIE-ID ::= 250

id-AdditionLocationInformation																		ProtocolIE-ID ::= 251

id-dataForwardingInfoFromTargetE-UTRANnode															ProtocolIE-ID ::= 252

id-DirectForwardingPathAvailability																	ProtocolIE-ID ::= 253

id-SourceNG-RAN-node-ID																				ProtocolIE-ID ::= 254

id-SourceDLForwardingIPAddress																		ProtocolIE-ID ::= 255

id-SourceNodeDLForwardingIPAddress																	ProtocolIE-ID ::= 256

id-ExtendedReportIntervalMDT																			ProtocolIE-ID ::= 257

id-SecurityIndication																				ProtocolIE-ID ::= 258

id-RRCConnReestab-Indicator																			ProtocolIE-ID ::= 259

id-TargetNodeID																						ProtocolIE-ID ::= 260

id-ManagementBasedMDTPLMNList																		ProtocolIE-ID ::= 261

id-PrivacyIndicator																					ProtocolIE-ID ::= 262

id-TraceCollectionEntityIPAddress																	ProtocolIE-ID ::= 263

id-M4ReportAmount																					ProtocolIE-ID ::= 264

id-M5ReportAmount																					ProtocolIE-ID ::= 265

id-M6ReportAmount																					ProtocolIE-ID ::= 266

id-M7ReportAmount																					ProtocolIE-ID ::= 267

id-BeamMeasurementIndicationM1																		ProtocolIE-ID ::= 268

id-MBS-Session-ID																					ProtocolIE-ID ::= 269

id-UEIdentityIndexList-MBSGroupPaging																	ProtocolIE-ID ::= 270

id-MulticastRANPagingArea																			ProtocolIE-ID ::= 271

id-Supported-MBS-FSA-ID-List																		ProtocolIE-ID ::= 272

id-MBS-SessionInformation-List																		ProtocolIE-ID ::= 273

id-MBS-SessionInformationResponse-List																ProtocolIE-ID ::= 274

id-MBS-SessionAssociatedInformation																	ProtocolIE-ID ::= 275

id-SuccessfulHOReportInformation																	ProtocolIE-ID ::= 276

id-SliceRadioResourceStatus-List																	ProtocolIE-ID ::= 277

id-CompositeAvailableCapacitySupplementaryUplink													ProtocolIE-ID ::= 278

id-SCGUEHistoryInformation																			ProtocolIE-ID ::= 279

id-SSBOffsets-List																					ProtocolIE-ID ::= 280

id-NG-RANnode2SSBOffsetModificationRange															ProtocolIE-ID ::= 281

id-Coverage-Modification-List																		ProtocolIE-ID ::= 282

id-NR-U-Channel-List																				ProtocolIE-ID ::= 283

id-SourcePSCellCGI																					ProtocolIE-ID ::= 284

id-FailedPSCellCGI																					ProtocolIE-ID ::= 285

id-SCGFailureReportContainer																		ProtocolIE-ID ::= 286

id-SNMobilityInformation																			ProtocolIE-ID ::= 287

id-SourcePSCellID																					ProtocolIE-ID ::= 288

id-SuitablePSCellCGI																				ProtocolIE-ID ::= 289

id-PSCellChangeHistory																				ProtocolIE-ID ::= 290

id-CHOConfiguration																					ProtocolIE-ID ::= 291

id-NR-U-ChannelInfo-List																			ProtocolIE-ID ::= 292

id-PSCellHistoryInformationRetrieve																	ProtocolIE-ID ::= 293

id-NG-RANnode2SSBOffsetsModificationRange															ProtocolIE-ID ::= 294

id-MIMOPRBusageInformation																			ProtocolIE-ID ::= 295

id-F1CTrafficContainer																				ProtocolIE-ID ::= 296

id-IAB-MT-Cell-List																					ProtocolIE-ID ::= 297

id-NoPDUSessionIndication																			ProtocolIE-ID ::= 298

id-IAB-TNL-Address-Request																			ProtocolIE-ID ::= 299

id-IAB-TNL-Address-Response																			ProtocolIE-ID ::= 300

id-TrafficToBeAddedList																				ProtocolIE-ID ::= 301

id-TrafficToBeModifiedList																			ProtocolIE-ID ::= 302

id-TrafficToBeReleaseInformation																	ProtocolIE-ID ::= 303

id-TrafficAddedList																					ProtocolIE-ID ::= 304

id-TrafficModifiedList																				ProtocolIE-ID ::= 305

id-TrafficNotAddedList																				ProtocolIE-ID ::= 306

id-TrafficNotModifiedList																			ProtocolIE-ID ::= 307

id-TrafficRequiredToBeModifiedList																	ProtocolIE-ID ::= 308

id-TrafficRequiredModifiedList																		ProtocolIE-ID ::= 309

id-TrafficReleasedList																				ProtocolIE-ID ::= 310

id-IABTNLAddressToBeAdded																			ProtocolIE-ID ::= 311

id-IABTNLAddressToBeReleasedList																	ProtocolIE-ID ::= 312

id-nonF1-Terminating-IAB-DonorUEXnAPID																ProtocolIE-ID ::= 313

id-F1-Terminating-IAB-DonorUEXnAPID																	ProtocolIE-ID ::= 314

id-BoundaryNodeCellsList																			ProtocolIE-ID ::= 315

id-ParentNodeCellsList																				ProtocolIE-ID ::= 316

id-tdd-GNB-DU-Cell-Resource-Configuration															ProtocolIE-ID ::= 317

id-UL-GNB-DU-Cell-Resource-Configuration															ProtocolIE-ID ::= 318

id-DL-GNB-DU-Cell-Resource-Configuration															ProtocolIE-ID ::= 319

id-permutation																						ProtocolIE-ID ::= 320

id-IABTNLAddressException																			ProtocolIE-ID ::= 321

id-CHOinformation-AddReq																			ProtocolIE-ID ::= 322

id-CHOinformation-ModReq																			ProtocolIE-ID ::= 323

id-SurvivalTime																						ProtocolIE-ID ::= 324

id-TimeSynchronizationAssistanceInformation															ProtocolIE-ID ::= 325

id-SCGActivationRequest																				ProtocolIE-ID ::= 326

id-SCGActivationStatus																				ProtocolIE-ID ::= 327

id-CPAInformationRequest																			ProtocolIE-ID ::= 328

id-CPAInformationAck																				ProtocolIE-ID ::= 329

id-CPCInformationRequired																			ProtocolIE-ID ::= 330

id-CPCInformationConfirm																			ProtocolIE-ID ::= 331

id-CPAInformationModReq																				ProtocolIE-ID ::= 332

id-CPAInformationModReqAck																			ProtocolIE-ID ::= 333

id-CPC-DataForwarding-Indicator																		ProtocolIE-ID ::= 334

id-CPCInformationUpdate																				ProtocolIE-ID ::= 335

id-CPACInformationModRequired																		ProtocolIE-ID ::= 336

id-QMCConfigInfo																					ProtocolIE-ID ::= 337

id-ProtocolIE-ID338-NotToBeUsed																		ProtocolIE-ID ::= 338

id-Additional-Measurement-Timing-Configuration-List													ProtocolIE-ID ::= 339

id-PDUSession-PairID																					ProtocolIE-ID ::= 340

id-Local-NG-RAN-Node-Identifier																		ProtocolIE-ID ::= 341

id-Neighbour-NG-RAN-Node-List																		ProtocolIE-ID ::= 342

id-Local-NG-RAN-Node-Identifier-Removal																ProtocolIE-ID ::= 343

id-FiveGProSeAuthorized																				ProtocolIE-ID ::= 344

id-FiveGProSePC5QoSParameters																		ProtocolIE-ID ::= 345

id-FiveGProSeUEPC5AggregateMaximumBitRate															ProtocolIE-ID ::= 346

id-ServedCellSpecificInfoReq-NR																		ProtocolIE-ID ::= 347

id-NRPagingeDRXInformation																			ProtocolIE-ID ::= 348

id-NRPagingeDRXInformationforRRCINACTIVE																ProtocolIE-ID ::= 349

id-Redcap-Bcast-Information																			ProtocolIE-ID ::= 350

id-SDTSupportRequest																				ProtocolIE-ID ::= 351

id-SDT-SRB-between-NewNode-OldNode																	ProtocolIE-ID ::= 352

id-SDT-Termination-Request																			ProtocolIE-ID ::= 353

id-SDTPartialUEContextInfo																			ProtocolIE-ID ::= 354

id-SDTDataForwardingDRBList																			ProtocolIE-ID ::= 355

id-PagingCause																						ProtocolIE-ID ::= 356

id-PEIPSassistanceInformation																		ProtocolIE-ID ::= 357

id-UESliceMaximumBitRateList																			ProtocolIE-ID ::= 358

id-S-NG-RANnodeUE-Slice-MBR																			ProtocolIE-ID ::= 359

id-PositioningInformation																			ProtocolIE-ID ::= 360

id-UEAssistantIdentifier																			ProtocolIE-ID ::= 361

id-ManagementBasedMDTPLMNModificationList															ProtocolIE-ID ::= 362

id-F1-terminatingIAB-donorIndicator																	ProtocolIE-ID ::= 363

id-TAINSAGSupportList																				ProtocolIE-ID ::= 364

id-SCGreconfigNotification																			ProtocolIE-ID ::= 365

id-earlyMeasurement																					ProtocolIE-ID ::= 366

id-BeamMeasurementsReportConfiguration																ProtocolIE-ID ::= 367

END

-- ASN1STOP

9.3.8	Container definitions

-- ASN1START

-- **************************************************************

--

-- Container definitions

--

-- **************************************************************

XnAP-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-Containers (5) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs,

Criticality,

Presence,

PrivateIE-ID,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Class Definition for Protocol IEs

--

-- **************************************************************

XNAP-PROTOCOL-IES ::= CLASS {

&id				ProtocolIE-ID 			UNIQUE,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol IE pairs

--

-- **************************************************************

XNAP-PROTOCOL-IES-PAIR ::= CLASS {

&id						ProtocolIE-ID 		UNIQUE,

&firstCriticality		Criticality,

&FirstValue,

&secondCriticality		Criticality,

&SecondValue,

&presence				Presence

}

WITH SYNTAX {

ID						&id

FIRST CRITICALITY 		&firstCriticality

FIRST TYPE				&FirstValue

SECOND CRITICALITY 		&secondCriticality

SECOND TYPE				&SecondValue

PRESENCE				&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol Extensions

--

-- **************************************************************

XNAP-PROTOCOL-EXTENSION ::= CLASS {

&id					ProtocolIE-ID 		UNIQUE,

&criticality		Criticality,

&Extension,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

EXTENSION			&Extension

PRESENCE			&presence

}

-- **************************************************************

--

-- Class Definition for Private IEs

--

-- **************************************************************

XNAP-PRIVATE-IES ::= CLASS {

&id					PrivateIE-ID,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container {XNAP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Single-Container {XNAP-PROTOCOL-IES : IEsSetParam} ::= 	ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field {XNAP-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				XNAP-PROTOCOL-IES.&id					({IEsSetParam}),

criticality		XNAP-PROTOCOL-IES.&criticality			({IEsSetParam}{@id}),

value			XNAP-PROTOCOL-IES.&Value				({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container for Protocol IE Pairs

--

-- **************************************************************

ProtocolIE-ContainerPair {XNAP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-FieldPair {{IEsSetParam}}

ProtocolIE-FieldPair {XNAP-PROTOCOL-IES-PAIR : IEsSetParam} ::= SEQUENCE {

id					XNAP-PROTOCOL-IES-PAIR.&id					({IEsSetParam}),

firstCriticality	XNAP-PROTOCOL-IES-PAIR.&firstCriticality	({IEsSetParam}{@id}),

firstValue			XNAP-PROTOCOL-IES-PAIR.&FirstValue			({IEsSetParam}{@id}),

secondCriticality	XNAP-PROTOCOL-IES-PAIR.&secondCriticality	({IEsSetParam}{@id}),

secondValue			XNAP-PROTOCOL-IES-PAIR.&SecondValue			({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container Lists for Protocol IE Containers

--

-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, XNAP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-Container {{IEsSetParam}}

ProtocolIE-ContainerPairList {INTEGER : lowerBound, INTEGER : upperBound, XNAP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-ContainerPair {{IEsSetParam}}

-- **************************************************************

--

#### -- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer {XNAP-PROTOCOL-EXTENSION : ExtensionSetParam} ::=	SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField {XNAP-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					XNAP-PROTOCOL-EXTENSION.&id				({ExtensionSetParam}),

criticality			XNAP-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		XNAP-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

-- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container {XNAP-PRIVATE-IES : IEsSetParam} ::=

SEQUENCE (SIZE (1..maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field {XNAP-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id				XNAP-PRIVATE-IES.&id			({IEsSetParam}),

criticality		XNAP-PRIVATE-IES.&criticality	({IEsSetParam}{@id}),

value			XNAP-PRIVATE-IES.&Value			({IEsSetParam}{@id})

}

END

-- ASN1STOP

Start of the next change