3GPP TSG RAN WG3 Meeting #119	R3-230078

Athens, Greece

Feb. 27th – Mar. 3rd, 2023

Title: 	Sharing processing for both unicast reception and broadcast reception

Source: 	TD Tech, Chengdu TD Tech

Agenda item:	15.2

Document Type:	Discussion

1. Introduction

In order to reduce the power consumption in a UE for receiving a multicast session in RRC_CONNECTED state and support the sharing processing for both unicast reception and broadcast reception, the 3GPP R18 WI “Enhancements of NR Multicast and Broadcast services” [1] was approved. In the contribution, the sharing processing for both unicast reception and broadcast reception is discussed with the related proposals suggested.

2. Sharing processing for both unicast reception and broadcast reception

In order to support the sharing processing for both unicast reception and broadcast reception, the following questions shall be solved.

- Which solution among solutions 2, 7, 24 and 29 is selected over the NG interface for MOCN network sharing?

- How many GTP-U tunnels over F1 are needed for a broadcast session for the scenario of MOCN network sharing?

For question (1), many companies support solutions 2 and 7 in the last RAN3 e-meeting. Solution 2 has the following features:

- Set up the user plane over the NG interface for a same broadcast session from each CN

- Only send the data from the first CN over Uu

Solution 7 has the following features:

- Only set up the user plane over the NG interface for a same broadcast session from the first CN

- Only send the data from the first CN over Uu

- If there’s a failure in the established user plane over the NG interface, establish another user plane over the NG interface for the same broadcast session from a second CN

Based on the above analysis, when a failure happens in the current user plane over the NG interface, it takes a long time for solution 7 to restore the transmission of the same broadcast session over Uu. It seems solution 2 is better and thus the following proposal is suggested.

Proposal 1: Solution 2 is selected over the NG interface for the MOCN network sharing.

For question (2), if solution 2 is selected, each CN under the MOCN network sharing scenario has its own user plane over the NG interface. Correspondingly there are two options over F1:

Option 1: One GTP-U tunnel over F1 per CN

Option 2: One GTP-U tunnel over F1 for the first CN and no GTP-U tunnel over F1 for the other CNs

For option 1, the user plane data from each CN is sent on a GTP-U tunnel over F1 to gNB-DU and thus gNB-DU receives many copies. Therefore option 1 needs more buffer space in gNB-DU and consumes more resource over F1.

For option 2, only the user plane data from the first CN is sent on a GTP-U tunnel over F1 to gNB-DU and thus gNB-DU receives only one cope. Compared with option 1, option 2 needs less buffer space in gNB-DU and consumes less resource over F1. If the failure happens in the user plane of the first CN, option 2 only needs to establish a new GTP-U tunnel over F1 for a same broadcast session from a second CN. The delay for the new GTP-U tunnel over F1 is acceptable.

Based on the above discussion, the following proposals are suggested.

Proposal 2: A GTP-U tunnel over F1 is set up for sending the user plane data from the first CN. The other CNs have no GTP-U tunnel over F1.

Proposal 3: If the failure happens in the user plane of the first CN, a new GTP-U tunnel over F1 is set up for the same broadcast session from a second CN.

3. Conclusions

Based on the discussion in the above section, the following proposals are suggested.

Proposal 1: Solution 2 is selected over the NG interface for the MOCN network sharing.

Proposal 2: A GTP-U tunnel over F1 is set up for sending the user plane data from the first CN. The other CNs have no GTP-U tunnel over F1.

Proposal 3: If the failure happens in the user plane of the first CN, a new GTP-U tunnel over F1 is set up for the same broadcast session from a second CN.

4. References

- RP-221458, Revised WID: Enhancements of NR Multicast and Broadcast Services, RAN Meeting#96-e