3GPP TSG-RAN WG3 Meeting #119	R3-230108

Athens, Greece, Feb 27th – Mar 3rd, 2023

Agenda item:	14.4

Source:	Qualcomm Incorporated

Title:	SCG Selective Activation in NR-DC

WID/SID:	NR_Mob_enh2-Core – Release 18

Document for:	Discussion and Decision

# Introduction

In a previous RAN3 meeting (RAN3 #117-bis-e [1], [2]), good progress was made on the scenarios to work on and there was a good discussion on some RAN3 aspects of this feature. Listed below is the set of agreements and some FFS issues identified during the meeting.

RAN3 #117-bis-e agreements:

RAN3 considers the Inter-CU and Intra-CU cases with equal priority, and studies both the F1 and Xn signalling aspects. It can be revisited based on RAN2 progress.

WA: RAN3 will work to enable both indirect and direct early data forwarding in Selective Activation. At this moment, RAN3 does not foresee any scenarios where direct forwarding is not feasible/desired.

WA (up to RAN2’s discussion): RAN3 assumes the last serving (source) PSCell may remain prepared within the prepared cells for Selective Activation.

WA: Enhance signalling for Selective Activation.

At this moment, RAN3 does not exclude any scenario for Selective Activation (for example CPA, MN-initiated CPC, SN-initiated CPC). RAN3 discussed following two sub-scenarios for Selective Activation and will further check their applicability/feasibility (the progress is also up to RAN2):

- Keeping PSCells prepared for CPA after UE accesses one of them;

- Keeping PSCells prepared for Selective Activation after DC operation is released so that they can be used for subsequent CPA (the cells prepared for Selective Activation may be prepared by the MN or the SN).

Any optimizations to the signalling are FFS. Proposals made so far:

- activation/deactivation of groups of PSCells prepared for Selective Activation;

- parallel data transmission from the UPF to all prepared PSCells.

In this contribution, we discuss the procedures for SCG selective activation and provide some proposals for RAN3 discussion.

# Procedures and signalling aspects

We consider below a basic procedure for SCG selective activation and its signalling flow, considering the Rel-16/Rel-17 CPAC procedures as the starting point, as has been agreed in RAN3 in a previous meeting (RAN3 #117-e [3]).

RAN3 #117-e agreement:

From RAN3 point of view, Rel-16/Rel-17 CPAC procedures are considered as start point for the Rel-18 work.

We assume that the UE is in MR-DC operation when the preparation procedure is initiated. The discussion would proceed along similar lines in case the UE is in standalone mode when the preparation procedure is initiated and is thus omitted.

Assumption. We assume that the UE is in MR-DC operation when the configuration preparation procedure is initiated.

As in the case of Rel-17 CPC, source MN or source SN can initiate the procedure.

Proposal 1. Assuming the UE is in MR-DC operation, either the source MN or the source SN can initiate the procedure for SCG selective activation (similar as in Rel-17 CPC).

Source MN initiated procedure for SCG selective activation

We discuss the source MN initiated procedure since it seems to be simpler. Figure 1 shows the signalling flow for the procedure. The signalling flow and the discussion below consider the example where only a first PSCell change and a subsequent PSCell change are configured. In the example, the candidate PSCells for the first PSCell change belong to target SN1 and the candidate PSCells for the subsequent PSCell change belong to target SN2.

Steps 1-6: MN initiates the preparation procedure based on UE measurement reports and transmits SN Addition Request to a target SN including a set of suggested candidate PSCells and associated UE measurements.

A target SN may not have the capability to support the SCG selective activation feature, e.g., it may not be able to keep candidate target PSCells and conditional configurations after a PSCell change, which seems to be a key enabling aspect of the feature and which has been agreed in a previous RAN2 meeting (RAN2 #119-bis-e [4]).

Observation 1. In SN Addition Request, source MN includes an SCG Selective Activation request indication so that a target SN that does not support the SCG selective activation feature may indicate accordingly.

Proposal 1. In SN Addition Request to a target SN, source MN includes an SCG Selective Activation request indication.

For the signalling flow in Figure 1 we consider the case where the target SN supports the feature and in SN Addition Request Acknowledge it provides a list of the candidate target PSCells, their associated SCG configurations, and data forwarding addresses.

Proposal 2. In SN Addition Request Ack, a target SN includes the data forwarding addresses.

MN prepares the RRC reconfiguration message containing the configuration for SCG selective activation, which consists of conditional configurations for the first PSCell change and a subsequent PSCell change (the example in the figure is for the case of two PSCell changes).

Proposal 3. Upon receiving the RRCReconfigurationComplete from the UE, using Xn-U Address Indication messages MN provides the data forwarding addresses needed for the procedure to the source SN and the target SNs.

Proposal 4. Upon receiving the Xn-U Address Indication message, source SN may initiate Early Data Forwarding corresponding to the first PSCell change.

Steps 7-12: Upon CPC triggering for a candidate target PSCell, UE may keep the source configuration as well as the conditional configurations. This seems to be a basic mechanism to achieve the goal of this objective in the WI and was agreed as a Working Assumption in a previous RAN3 meeting (RAN3 #117-e [3]) and as an agreement in a previous RAN2 meeting (RAN2 #119-bis-e [4]).

RAN2 #119-bis-e agreements:

Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs):

a. Step 1: When the execution condition of a CPC candidate PSCell is met, a UE performs the execution of CPC towards this candidate PSCell.

b. Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PSCells.

RAN3 #117-e agreement:

WA: A primary focus of the objective is to enable subsequent cell changes by keeping conditional reconfigurations after a cell change. RAN3 to pursue study of the Xn/F1 signaling changes required to support this objective.

Upon receiving SN Release Request from the MN:

- Source SN stops transmitting user data to the UE.

- Source SN keeps the data forwarding addresses provided earlier by the source MN, since the UE may come back to it later. The Source SN may also keep the data forwarding addresses based on a received network indication.

Proposal 5. After a cell change is triggered, the source MN notifies the selected SN, the serving SN, and the other target SNs of the cell change.

RAN2 #119-bis-e agreement:

Baseline procedure to support subsequent secondary cell group change:

b. Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PSCells.

Step 14: After accessing the candidate target PSCell, UE initiates CPC evaluation for the subsequent PSCell change. This was also agreed in a previous RAN2 meeting (RAN2 #119-bis-e [4]).

Figure 1: MN initiated procedure for SCG selective activation

# 3 Conclusions

Based on the above discussions, we recommend that RAN3 discuss the following observations and proposals.

Proposal 1. Assuming the UE is in MR-DC operation, either the source MN or the source SN can initiate the procedure for SCG selective activation (similar as in Rel-17 CPC).

Source MN initiated procedure for SCG selective activation

Observation 1. In SN Addition Request, source MN includes an SCG Selective Activation request indication so that a target SN that does not support the SCG selective activation feature may indicate accordingly.

Proposal 1. In SN Addition Request to a target SN, source MN includes an SCG Selective Activation request indication.

Proposal 2. In SN Addition Request Ack, a target SN includes the data forwarding addresses.

Proposal 3. Upon receiving the RRCReconfigurationComplete from the UE, using Xn-U Address Indication messages MN provides the data forwarding addresses needed for the procedure to the source SN and the target SNs.

Proposal 4. Upon receiving the Xn-U Address Indication message, source SN may initiate Early Data Forwarding corresponding to the first PSCell change.

Proposal 5. After a cell change is triggered, the source MN notifies the selected SN, the serving SN, and the other target SNs of the cell change.

# References

[1] Chair notes, RAN3 #117-bis-e meeting.

[2] R3-225959	Summary of Offline Discussion on Selective Activation (CB #MobilityEnh3_Others), Nokia.

[3] Chair notes, RAN3 #117-e meeting.

[4] Chair notes, RAN2 #119-bis-e meeting.