3GPP TSG-RAN WG3 #119																			  R3-230112

27th Feb – 3rd Mar 2023

Athens, Greece

Title: 	Stage 3 issues on MT-SDT

Source: 	ZTE

Agenda item:	20.2

Document for:	Discussion and Decision

- Introduction

We have already finished MO-SDT including RA-SDT and CG-SDT in Rel-17. In Rel-18, we will design MT-SDT as SDT enhancement, the scope of the Rel-18 MT-SDT WID (RP-213583) is as below.

### 4.1 Objective of SI or Core part WI or Testing part WI Specify the support for paging-triggered SDT (MT-SDT) [RAN2, RAN3] - MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RA-SDT and CG-SDT as the UL response; - MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE. Note: Data transmission in DL within paging message is not in scope of this WI.

Before RAN3 starts this WID, RAN2 has got some progress in the last RAN2 #120 meeting, which can be seen as useful input for RAN3 discussion.

Agreements

- For RAN paging, MT-SDT indication (at least one bit) is explicitly included per UE via a paging message.  FFS if more information for MT-SDT are needed FFS what the indication will be called.  FFS signalling details

- Rel-18 MT-SDT after the MT-SDT paging trigger is detected, RA-SDT and CG SDT solutions/procedures specified in Rel-17 is re-used as a baseline.  The detailed triggers will be discussed on case by case.  FFS on resources used for access

- UE can use non-SDT random access resources for accessing the network for an MT-SDT transfer.  The UE can also use the configured grant resources and/or MO-RA resources.

- The network should be able to differentiate why the UL access was triggered, i.e. implicit or explicit indication by the UE.

- MT-SDT is data that belongs to bearers that are configured for SDT.    FFS whether the configuration is MO-SDT or MT-SDT specific.  The network can only trigger MT-SDT if the data belongs to those bearers.

- It is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT.  Subsequent UL/DL data belonging to SDT bearers while in INACTIVE is allowed like MO-SDT procedure.  FFS stage 3 details

- New Resume cause in RRC resume will be introduced, one code point MT-SDT indication

This contribution addresses to analyse RAN3 impact on the Stage 3 issues for this Rel-18 MT-SDT WID.

- Discussion

In our other paper (R3-230112 Discussion on MT-SDT stage 2 issues), we have analyzed the Stage 2 issues and provided the following proposals.

Observation 1: It is the anchor node to receive DL data from 5GC. In case of split gNB, it is the anchor node’s CU-UP to receive the DL user data. Proposal 1: MT-SDT includes both DL user data transmission and DL NAS signalling transmission. Observation 2: Only the anchor gNB has acknowledged of MT-SDT bearer information, other than other gNBs within the RAN area. Proposal 2: If DL data/signalling is received and its related MT-SDT is configured, the anchor gNB can send MT-SDT indication within Xn RAN paging message to other gNBs within the RAN area, to indicate MT-SDT is allowed/expected. Proposal 3: Upon reception of MT-SDT data from UPF, the anchor gNB sends Xn paging message including an optional MT-SDT assistant information (e.g., MT-SDT data size, "single/multiple packets" indicator) as well as MT-SDT indication to other gNBs within in the RAN area. In case of split anchor gNB, the optional MT-SDT assistant information is sent from CU-UP to CU-CP. Observation 3: According to RAN2’s agreement, RAN3 thinks it is the UE to initiate MT-SDT procedure.

TS 38.423

In MO-SDT, after receiving MO-SDT indication, the anchor node decides whether to relocate UE context based on the optional SDT assistant information included in the RETRIEVE UE CONTEXT REQUEST message.

9.2.3.163 SDT Support Request This IE indicates that the UE requested for SDT and may include additional assistance information. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. SDT Indicator; M; ENUMERATED (true,…). SDT assistant information; O; ENUMERATED (single packet, multiple packets, …); “Single packet” indicates no subsequent SDT transmission is expected; “Multiple packets” indicates subsequent SDT transmission is expected..

Based on the above observation 4 and proposal 2 and 3, we suggest to design MT-SDT IE structure similar to SDT Support Request IE used for MO-SDT, and add it into the RAN Paging message.

9.2.3.XXX MT-SDT Support Request This IE indicates that the anchor gNB requests for MT-SDT and may include additional assistance information. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. MT-SDT Indicator; M; ENUMERATED (true, …). MT-SDT data size; O; FFS (pending to RAN2); Indicates size of DL data packet. Subsequent MT-SDT Indicator; O; ENUMERATED (single packet, multiple packets, …); “Single packet” indicates no subsequent MT-SDT transmission is expected; “Multiple packets” indicates subsequent MT-SDT transmission in excess of MT-SDT data size is expected..

TS 38.473

5.    MT-SDT is data that belongs to bearers that are configured for SDT.    FFS whether the configuration is MO-SDT or MT-SDT specific.  The network can only trigger MT-SDT if the data belongs to those bearers.

7.    New Resume cause in RRC resume will be introduced, one code point MT-SDT indication

In order to align with RAN2 agreement as above, we suggest to enhance UE CONTEXT RELEASE COMMAND message, INITIAL UL RRC MESSAGE TRANSFER message and PAGING message.

TS 37.483

Based on the above agreement 4, i.e., upon reception of MT-SDT data from UPF, the anchor gNB sends Xn paging message including an optional MT-SDT assistant information (e.g., MT-SDT data size ,"single/multiple packets" indicator) as well as MT-SDT indication to other gNBs within in the RAN area. In case of split anchor gNB, the optional MT-SDT assistant information is sent from CU-UP to CU-CP.

#### 9.2.2.13 DL DATA NOTIFICATION This message is sent by the gNB-CU-UP to provide information about the DL data detection to the gNB-CU-CP. Direction: gNB-CU-UP gNB-CU-CP IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. gNB-CU-CP UE E1AP ID; M; 9.3.1.4; YES; reject. gNB-CU-UP UE E1AP ID; M; 9.3.1.5; YES; reject. Paging Priority Indicator (PPI); O; 9.3.1.55; YES; ignore. PDU Session To Notify List; O; YES; ignore. >PDU Session To Notify Item; 1..<maxnoofPDUSessionResource>; -; -. >>PDU Session ID; M; 9.3.1.21; -; -. >>QoS Flow List; M; 9.3.1.12; -; -. MT-SDT data size; O; FFS (pending to RAN2); Indicates size of DL packet; -. Subsequent MT-SDT Indicator; O; ENUMERATED (single packet, multiple packets, …); “Single packet” indicates no subsequent MT-SDT transmission is expected; “Multiple packets” indicates subsequent MT-SDT transmission in excess of MT-SDT data size is expected.; -. Range bound; Explanation. maxnoofPDUSessionResource; Maximum no. of PDU Sessions for a UE. Value is 256..

- Text proposal

  - TS 38.423

=====================================================

9.1.1.7	RAN PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page a UE.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE UE Identity Index Value | M |  |  |  | YES | reject |
| >Length-10 |  |  |  |  |  |  |
| >>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33] and TS 36.304 [34]. | – |  |
| UE RAN Paging Identity | M |  | 9.2.3.43 |  | YES | ignore |
| Paging DRX | M |  | 9.2.3.66 | Includes the RAN paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| RAN Paging Area | M |  | 9.2.3.38 |  | YES | reject |
| Paging Priority | O |  | 9.2.3.44 |  | YES | ignore |
| Assistance Data for RAN Paging | O |  | 9.2.3.41 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.3.91 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.2.3.141 | Coded as specified in TS 36.304 [34]. | YES | ignore |
| E-UTRA Paging eDRX Information | O |  | 9.2.3.142 |  | YES | ignore |
| UE specific DRX | O |  | 9.2.3.143 | Includes the UE specific paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| NR Paging eDRX Information | O |  | 9.2.3.161 |  | YES | ignore |
| NR Paging eDRX Information for RRC INACTIVE | O |  | 9.2.3.162 |  | YES | ignore |
| Paging Cause | O |  | ENUMERATED (voice, …) |  | YES | ignore |
| PEIPS Assistance Information | O |  | 9.2.3.166 |  | YES | ignore |
| MT-SDT Support Request | O |  | 9.2.3.XXX |  | YES | ignore |

9.2.3.XXX	MT-SDT Support Request

This IE indicates that the anchor gNB requests for MT-SDT and may include additional assistance information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MT-SDT Indicator | M |  | ENUMERATED (true, …) |  |
| MT-SDT data size | O |  | FFS (pending to RAN2) | Indicates size of DL data packet |
| Subsequent MT-SDT Indicator | O |  | ENUMERATED (single packet, multiple packets, …) | “Single packet” indicates no subsequent MT-SDT transmission is expected; “Multiple packets” indicates subsequent MT-SDT transmission in excess of MT-SDT data size is expected. |

==================================================================================

  - TS 38.473

9.2.2.5	UE CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU to request the gNB-DU to release the UE-associated logical F1 connection or candidate cells in conditional handover or conditional PSCell addition or conditional PSCell change.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8] encapsulated in a PDCP PDU, or the DL-CCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8]. | YES | ignore |
| SRB ID | C- ifRRCContainer |  | 9.3.1.7 | The gNB-DU sends the RRC message on the indicated SRB. | YES | ignore |
| old gNB-DU UE F1AP ID | O |  | 9.3.1.5 | Include it if RRCReestablishmentRequest is not accepted | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| Positioning Context Reservation Indication | O |  | ENUMERATED (True,...) |  | YES | ignore |
| CG-SDT Kept Indicator | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| MT-CG-SDT Kept Indicator | O |  | ENUMERATED (true, ...) | FFS: Pending to RAN2 | YES | ignore |

9.2.3.1	INITIAL UL RRC MESSAGE TRANSFER

This message is sent by the gNB-DU to transfer the initial layer 3 message to the gNB-CU over the F1 interface.

Direction: gNB-DU gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 | NG-RAN Cell Global Identifier (NR CGI) | YES | reject |
| C-RNTI | M |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | reject |
| RRC-Container | M |  | 9.3.1.6 | Includes the UL-CCCH-Message IE or UL-CCCH1-Message IE as defined in subclause 6.2 of TS 38.331 [8]. | YES | reject |
| DU to CU RRC Container | O |  | OCTET STRING | CellGroupConfig IE as defined in subclause 6.3.2 in TS 38.331 [8]. Required at least to carry SRB1 configuration. The ReconfigurationWithSync field is not included in the CellGroupConfig IE. | YES | reject |
| SUL Access Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | ignore |
| RAN UE ID | O |  | OCTET STRING (SIZE (8)) |  | YES | ignore |
| RRC-Container-RRCSetupComplete | O |  | 9.3.1.6 | Includes the UL-DCCH-Message IE including the RRCSetupComplete message, as defined in subclause 6.2 of TS 38.331 [8]. | YES | ignore |
| NR RedCap UE Indication | O |  | ENUMERATED(true, …) |  | YES | ignore |
| SDT Information | O |  | 9.3.1.262 |  | YES | ignore |
| Sidelink Relay Configuration | O |  | 9.3.1.264 |  | YES | ignore |
| MT-SDT indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |

9.2.6.1	PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| UE Identity Index value | M |  | 9.3.1.39 |  | YES | reject |
| CHOICE Paging Identity | M |  |  |  | YES | reject |
| >RAN UE Paging identity |  |  |  |  |  |  |
| >>RAN UE Paging identity | M |  | 9.3.1.43 |  | - |  |
| >CN UE paging identity |  |  |  |  |  |  |
| >>CN UE paging identity | M |  | 9.3.1.44 |  | - |  |
| Paging DRX | O |  | 9.3.1.40 | It is defined as the minimum between the RAN UE Paging DRX and CN UE Paging DRX | YES | ignore |
| Paging Priority | O |  | 9.3.1.41 |  | YES | ignore |
| Paging Cell List |  | 1 |  |  | YES | ignore |
| >Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Last Used Cell Indication | O |  | ENUMERATED(true, …) |  | YES | ignore |
| >>PEI Subgrouping Support Indication | O |  | ENUMERATED(true, …) |  | YES | ignore |
| Paging Origin | O |  | 9.3.1.79 |  | YES | ignore |  |
| RAN UE Paging DRX | O |  | Paging DRX 9.3.1.40 | This IE indicates the RAN paging cycle as defined in TS 38.304 [24]. | YES | ignore |  |
| CN UE Paging DRX | O |  | Paging DRX 9.3.1.40 | This IE indicates the UE specific paging cycle as defined in TS 38.304 [24]. | YES | ignore |  |
| NR Paging eDRX Information | O |  | 9.3.1.258 |  | YES | ignore |  |
| NR Paging eDRX Information for RRC INACTIVE | O |  | 9.3.1.259 |  | YES | ignore |  |
| Paging Cause | O |  | ENUMERATED(voice, …, MT-SDT) | This IE indicates either the paging cause is IMS voice, refer to TS 23.501[21], or the paging cause is to trigger MT-SDT. | YES | ignore |  |
| PEIPS Assistance Information | O |  | 9.3.1.269 |  | YES | ignore |  |
| UE Paging Capability | O |  | 9.3.1.270 |  | YES | ignore |  |
| MT-SDT Support Request | O |  | 9.3.1.XXX |  | YES | ignore |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |

9.3.1.XXX	MT-SDT Support Request

This IE indicates that the gNB-CU requests for MT-SDT and may include additional assistance information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MT-SDT Indicator | M |  | ENUMERATED (true, …) |  |
| MT-SDT data size | O |  | FFS (pending to RAN2) | Indicates size of DL data packet |
| Subsequent MT-SDT Indicator | O |  | ENUMERATED (single packet, multiple packets, …) | “Single packet” indicates no subsequent MT-SDT transmission is expected; “Multiple packets” indicates subsequent MT-SDT transmission in excess of MT-SDT data size is expected. |

===================================================================================

  - TS 37.483

9.2.2.13	DL DATA NOTIFICATION

This message is sent by the gNB-CU-UP to provide information about the DL data detection to the gNB-CU-CP.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Paging Priority Indicator (PPI) | O |  | 9.3.1.55 |  | YES | ignore |
| PDU Session To Notify List | O |  |  |  | YES | ignore |
| >PDU Session To Notify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>QoS Flow List | M |  | 9.3.1.12 |  | - | - |
| MT-SDT data size | O |  | FFS (pending to RAN2) | Indicates size of DL data packet | YES | ignore |
| Subsequent MT-SDT Indicator | O |  | ENUMERATED (single packet, multiple packets, …) | “Single packet” indicates no subsequent MT-SDT transmission is expected; “Multiple packets” indicates subsequent MT-SDT transmission is expected. | YES | ignore |

===================================================================================