3GPP TSG-RAN WG3 Meeting #119	R3-230130

Athens, Greece, EU, 27 Feb – 3 Mar 2023

Agenda item:	10.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	[TP to TS38423, SON] SN support for MRO for fast MCG recovery

Document for:	Endorsement

# 1	Introduction

In RAN3#117e the following agreements with regard to fast MCG recovery scenarios was made:

Following problems will be addressed in RAN3:

MRO for the fast MCG recovery:

- SCG fails or is deactivated when the UE attempts MCG recovery (i.e. a SCG failure/deactivation while T316 is running after MCG failure)

- the signalling delay is longer than the time the UE waits for the response (T316 expired);

- other problem are not precluded if legacy MRO mechanism cannot cope with it.

The discussion continued at RAN3 #118 and was concluded with a following agreement:

It is beneficial for the UE to report at least the cause of the fast MCG recovery failure (at least T316 expiry, SCG failure) and also, if the problem is SCG failure, the SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx).

Here, we remind some arguments made before [1] and propose a solution that matches this agreement.

# 2	Discussion

The information agreed to be beneficial shall not be considered independently. We have presented two sub-scenarios leading to a fast MCG recovery failure:

Sub-scenario b1): T316 runs out on the UE side while the SN is trying to deliver the MN message. in this case the maximum number of retransmissions at the SN side has not been reached.

Sub-scenario b2): The SN reaches the maximum number of retransmissions while T316 has not expired on the UE side. In this case the SN can not make any further attempts to deliver the MN message but the UE will continue to wait for it for the remainder of the T316 time. This will only increase the interruption time experienced by the UE as after this point the fast MCG recovery can no longer be successful. Such situation should be especially avoided.

In order to avoid the situation presented in sub-scenarios b1) and b2), the network would need additional information in order to determine the root case of the fast MCG recovery failure and appropriate corrective actions. In sub-scenario b1) T316 may be set to a too small value which only accommodates few/no re-transmission from the SN. By contrast, in sub-scenario b2) T316 may be set to a too large value (by comparison to the maximum number of re-transmission in the SN), which means the UE needlessly awaits for a message that can no longer be delivered.

Proposal 1: The MRO shall be provided with information enabling it to optimize the T316 to ensure fast MCG recovery succeeds and interruption time on the UE side is minimized.

In sub-scenario b2), even though it is pretty clear the value of T316 is too large, it is hard to optimize it based on the information alone that the max number of retransmissions was reached. The information that the MN or a central entity currently has access to does not reveal the whole situation and makes the optimization more difficult.

In order to optimize the T316 duration, in addition to the cause of failure (T316 expiry or or maximum number of re-transmission reached) the values of both counters should be provided:

- Elapsed value of T316 when maximum number of retransmissions was reached

- Number of retransmissions that SN performed before T316 ran out.

This information will allow the MN to further tune the T316 value so that the UE can fail early and decrease interruption time introduced by a procedure that will fail.

Proposal 2: In addition to the already agreed cause of the failure (T316 ran out or maximum number of re-transmission reached), the SN can transfer the following additional information to MN in the case of fast MCG recovery failure to aid with T316 optimization:

- Elapsed value of T316 when maximum number of retransmissions was reached

- Number of retransmissions that SN performed before T316 ran out.

In order for the SN to be able to compile the information above, it would need a way of determining or estimating on its own the current value of the T316 in the UE. This could be done by the SN by computing the time between receiving the MCG failure information until the time the DC operation was released (in case the UE switched to idle mode). The SN may compare this value to the T316 configure value for the UE. The UE may report it explicitly to the SN outside of the failure container. Also, the MN may provide the information to the SN in, e.g. Addition procedure (and update it in the MN-initiated modification).

Proposal 3: RAN3 shall consider if the value of the T316 can be estimated based on SN’s implementation or shall be reported from the UE or from the MN.

# 3	Conclusions

In this paper, we take the outcomes of the discussion at RAN3 #118 and remind some additional arguments:

Proposal 1: The MRO shall be provided with information enabling it to optimize the T316 to ensure fast MCG recovery succeeds and interruption time on the UE side is minimized.

Proposal 2: In addition to the already agreed cause of the failure (T316 ran out or maximum number of re-transmission reached), the SN can transfer the following additional information to MN in the case of fast MCG recovery failure to aid with T316 optimization:

- Elapsed value of T316 when maximum number of retransmissions was reached

- Number of retransmissions that SN performed before T316 ran out.

Proposal 3: RAN3 shall consider if the value of the T316 can be estimated based on SN’s implementation or shall be reported from the UE or from the MN.

# References

- R3-226202, RAN3 #118

# Text proposal to XnAP

The below changes are proposed to be included in the BL CR to TS 38.423.

First change, ommited text not changed

#### 9.1.2.20	RRC TRANSFER

This message is sent by the M-NG-RAN-NODE to the S-NG-RAN-NODE to transfer an RRC message or from the S-NG-RAN-NODE to the M-NG-RAN-NODE to report the DL RRC message delivery status.

This message is also sent by the new NG-RAN-NODE to the old NG-RAN-NODE or from the old NG-RAN-NODE to the new NG-RAN-NODE to transfer an RRC message containing the SDT SRB in case of RACH based SDT without UE context relocation.

Direction: M-NG-RAN node ® S-NG-RAN node or S-NG-RAN node ® M-NG-RAN node (Dual Connectivity).

Direction: new NG-RAN node ® old NG-RAN node or old NG-RAN node ® new NG-RAN node (SDT).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Split SRB |  | 0..1 |  |  | YES | reject |
| >RRC Container | O |  | OCTET STRING | Contains a PDCP-C PDU encapsulating an RRC message as defined in subclause 6.2.1 of TS 38.331 [10] or TS 36.331 [14] and ciphered with the key of the M-NG-RAN node | – |  |
| >SRB Type | M |  | ENUMERATED (srb1, srb2, ...) | The SRB type to be used | – |  |
| >Delivery Status | O |  | 9.2.3.45 | DL RRC delivery status of split SRB | – |  |
| UE Report |  | 0..1 |  |  | YES | reject |
| >RRC Container | M |  | OCTET STRING | For NGEN-DC and NR-DC, includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [10] containing the MeasurementReport message or the RRCReconfigurationComplete message or the FailureInformation message or the UEAssistanceInformation message. For NE-DC, includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 36.331 [14] containing the MeasurementReport message. | – |  |
| Fast MCG Recovery via SRB3 from SN to MN |  | 0..1 |  |  | YES | ignore |
| >RRC Container | M |  | OCTET STRING | For NR-DC, includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [10] containing the MCGFailureInformation, message. For NGEN-DC, includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 36.331 [14] containing the MCGFailureInformation message. | – |  |
| >MRO support information |  | 0..1 |  |  | YES | ignore |
| >>Cause of recovery | M |  | ENUMERATED (T316-run-out, max-retransmissions, ...) |  |  |  |
| >>T316 At Max Retransmissions | O |  | FFS |  |  |  |
| >>Retransmission At T316 | O |  | FFS |  |  |  |
| Fast MCG Recovery via SRB3 from MN to SN |  | 0..1 |  |  | YES | ignore |
| >RRC Container | M |  | OCTET STRING | For NR-DC, includes the DL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [10] containing the RRCReconfiguration message, or the RRCRelease message, or the MobilityFromNRCommand message. For NGEN-DC, includes the DL-DCCH-Message as defined in subclause 6.2.1 of TS 36.331 [14] containing the RRCConnectionReconfiguration message, or the RRCConnectionRelease message, or the MobilityFromEUTRACommand message. | – |  |
| >MRO support information |  | 0..1 |  | This IE shall be ignored if received. | YES | ignore |
| >>Cause of recovery | M |  | ENUMERATED (T316-run-out, max-retransmissions, ...) |  |  |  |
| >>T316 At Max Retransmissions | O |  | FFS |  |  |  |
| >>Retransmission At T316 | O |  | FFS |  |  |  |
| SDT SRB between New NG-RAN node and Old NG-RAN node |  | 0..1 |  |  | YES | ignore |
| >RRC Container | M |  | OCTET STRING | Contains a PDCP-C PDU encapsulating an RRC message as defined in subclause 6.2.1 of TS 38.331 [10]. | – |  |
| >SRB ID | M |  | 9.2.3.165 | In this version of the specification, values "0", "1", "3", and "4" are not set by the sender and ignored by the receiver. | – |  |

Remaining text not changed