3GPP TSG-RAN WG3 Meeting #119	R3-230140

# Athens, Greece, 27 February - 3 March 2023

Agenda Item:	16.4

Source:	Ericsson

Title:	Multi-path for Sidelink Relay

Document for:	Discussion, Decision

# 1	Introduction

In RAN3#117bis-e meeting, the following progress has been made for the support of Multipath for Sidelink Relay.

For Scenario 1, the responsibility of gNB-CU and gNB-DU in Rel-17 SL relay can be reused as a baseline. Whether to enhance the responsibility of gNB-CU and gNB-DU in Rel-18 is FFS.

For the responsibility of gNB-CU and gNB-DU in Scenario 2, the RAN3 waits for RAN2’s progress on protocol stack for Scenario 2.

For the multi-path support, the gNB-CU takes the responsibility to decide the addition/modification/release of the path.

For intra-DU and inter-DU cases, the UE Context Setup / Modification procedure can be reused to configure the 2nd path with possible enhancements. The details will be discussed based on RAN2 progress.

The RAN3 waits for the RAN2 progress on whether the gNB-DU knows the path information of each configured path.

WA: The direct path and indirect path cannot be configured for a remote UE simultaneously in this release, depending on RAN2 decision.

The gNB-CU is responsible to determine the data split among two paths for a DRB for both intra-DU and inter-DU cases.

For intra-DU case, two F1-U tunnels are setup between CU and DU for a split DRB. FFS on how to support the multi-path delivery of split SRB.

WA: The RAN3 will specify the details of the path change procedure after introducing the procedure of the direct/indirect path addition.

Previous RAN3 agreement is updated as follows:

For Scenario 1, addition of direct/indirect path are supported as follows:

Add direct path, after the establishment of the indirect path.

Add indirect path, after the establishment of the direct path.

For Scenario 2, addition of direct/indirect path are supported as follows:

Add indirect path, after the establishment of the direct path.

Whether to add direct path, after the establishment of the indirect path is pending to RAN2 decision.

For Scenario 2, interface between UEs are non-3GPP defined. Therefore, in the UE context setup/modification procedure, the PC5 Relay RLC channel configurations are not needed for remote UE and relay UE..

In RAN3#118 meeting, it has been concluded that the study is completed and the normative work will start.

RAN3#118:

From RAN3 point of view, the Multi-path study phase is completed, and the Multi-path can move to normative work phase.

In this contribution, we would provide anlaysis and give the proposals.

# 2	Discussion

The following figure depicts CU-DU split architecture supporting multipath scenario for intra-gNB case, including both intra-DU and inter-DU.

Figure 1: CU-DU split architecture supporting the MP scenario in the same gNB, where the two paths are associated with different cells

## 2.1 Inter-DU case

During the discussion on Scenario 1 in RAN2, one issue that was raised pertained to inter-DU communication, i.e., two paths are connected to different DUs. RAN2 has agreed that it needs to be considered when having mode 1 RA scheduling. No details have been discussed yet.

Considering inter-DU is within RAN3’s realm, we need to check the possible impacts. In this case, for example, the remote UE has a direct path with DU1, also has an indirect path with DU2. However, the remote UE cannot send/receive the scheduling info to/from DU2. Thus, DU1 needs to be able to provide scheduling information to DU2.

RAN3 should check whether need to transfer info over F1 to support the scheduling coordination between DU1 and DU2.

- RAN3 to consider inter-DU case for MP transmission.

## 2.2 Direct/indirect path addition/release

Proposal 1-1A (modified): The following cases are to be supported for Scenario 1.

A. The remote UE operating only on the direct path adds the indirect path under the same gNB;

B. The remote UE operating only on the indirect path adds the direct path under the same gNB;

C. The remote UE operating in multi-path releases the indirect path;

D. The remote UE operating in multi-path releases the direct path;

G. The remote UE operating in multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.  FFS if this case would be supported via separate release-and-add (A+C in separate reconfigurations) or a single switch procedure (e.g. similar to i2i service continuity).

The following case can be supported via separate release-and-add for scenario 1 (B+D in separate reconfigurations):

E. The remote UE operating in multi-path changes the direct path to a different cell of the same gNB while using the serving relay UE for the indirect path under the same gNB. FFS if a single procedure for this case would be supported.

RAN2 reached some agreements on the above cases and put some into low priority to close the SI. They will be further discussed in the normative phase. RAN2 is going to discuss which cases to be supported for Scenario 1. For example, in case E, considering the direct path is changed to a different cell, this can mostly be attributed to its own mobility and as a result, the remote UE will not be able to keep the indirect path via the same U2N relay UE. In addition, based on the measurement configuration and results, the gNB can configure:

- Either the change of the direct path to a different cell of the same gNB and keep the indirect path.

- Or the change of the direct path to a different cell in a different gNB and release the indirect path. As Re-18 does not support the case where the remote UE is now allowed to be connected to two cells from different gNBs.

Given that this is the first release for this item, we should specify basic operations first, hence, the network behavior should be consistent when the direct path changes to a different cell of the same/different gNB. We foresee F1AP impacts when support the gNB-CU to inform the gNB-DU about the path addition/release.

- The gNB-CU indicates to the gNB-DU whether to add/release at least an indirect path. F1AP can be enhanced by adding an indicator and/or the channels mapping to be added/released.

## 2.3 UL/DL transmission

RAN3 has the following agreement and FFS on the data aspects.

The gNB-CU is responsible to determine the data split among two paths for a DRB for both intra-DU and inter-DU cases.

For intra-DU case, two F1-U tunnels are setup between CU and DU for a split DRB. FFS on how to support the multi-path delivery of split SRB.

In RAN2, it also has been discussed for Scenario 1 how the split bearer will be configured. Considering the case when a multipath split bearer is configured, SRB1/SRB2 can be configured only on direct or only on indirect path or on both paths. In DL, the selection of the transmission path for SRB1/SRB2, as in legacy, should be up to network implementation. For UL transmissions, the CP signaling can be performed on the best or most robust path available, i.e., only on the direct path or only on the indirect path or duplicated on both paths. RAN3 would wait for the conclusion from RAN2 on the MP split bearer. Regarding how to support the multi-path delivery of split SRB, we don’t see major difference as the legacy DC. Therefore, PDCP duplication can be reused when split bearer is configured.

- In general, the legacy DC mechanism can be reused to support split bearer. Further details rely on RAN2’s progress.

For UP signaling over DRBs, after two F1-U tunnels have been established, the data split can be left to network’s implementation, that means whether the CU would prefer to transmit data over the primary path (terminology to be decided by RAN2) or choose to split the data or duplicate over two tunnels according to its evaluation of network performance.

- How to split the data over different paths for DRBs is up to network’s implementation.

# 3	Conclusion

In this paper we propose.

Proposal 1	RAN3 to consider inter-DU case for MP transmission.

Proposal 2	The gNB-CU indicates to the gNB-DU whether to add/release at least an indirect path. F1AP can be enhanced by adding an indicator and/or the channels mapping to be added/released.

Proposal 3	In general, the legacy DC mechanism can be reused to support split bearer. Further details rely on RAN2’s progress.

Proposal 4	How to split the data over different paths for DRBs is up to network’s implementation.