3GPP TSG-RAN WG3 Meeting #119	R3-230262

Athens, GR, 27 Feb – 03 Mar, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1732 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Missing Criticality Diagnostics [Add_Crit_Diag] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, BT, Deutsche Telekom, Orange, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Criticality Diagnostics is used to inform the peer node that parts of a received message have not been comprehended or were missing, or if the message contained logical errors and indicate which IEs were not comprehended or were missing. Therefore for Class1 procedure, in RAN3 interfaces, we include the Criticality Diagnostics in the response messages and the failure messages. But in X2AP, the Criticality Diagnostics IE is missing in some response messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the Criticality Diagnostics IE in the following X2AP messages: - EN-DC X2 SETUP RESPONSE - PARTIAL RESET CONFIRM - E-UTRA – NR CELL RESOURCE COORDINATION RESPONSE Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it only adds the missing Criticality Diagnostics as optional IE with criticality ignore in some response messages.The impact can be considered isolated because the change only adding the Criticality Diagnostics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Criticality Diagnostics IE is missing in many response messages, the receiving node is not able to inform the related errors to the sendng node. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.32, 9.1.4.23, 9.1.4.25, 9.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.473 CR1112 TS 37.473 CR0019 TS 38.423 CR0973 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------Start of the First Change-----------

#### 9.1.2.32	EN-DC X2 SETUP RESPONSE

This message is sent by a neighbouring node to an initiating node, both nodes able to interact for EN-DC, to transfer the initialization information for a TNL association.

Direction: eNB  en-gNB, en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| CHOICE Responding NodeType | M |  |  |  | YES | reject |
| >eNB |  |  |  |  |  |  |
| >>Global eNB ID | M |  | 9.2.22 |  | YES | reject |
| >>List of Served E-UTRA Cells |  | 1 .. <maxCellineNB> |  | Complete list of cells served by the eNB | YES | reject |
| >>>Served E-UTRA Cell Information | M |  | Served Cell Information 9.2.8 |  | – |  |
| >>>NR Neighbour Information | O |  | 9.2.98 | NR neighbours | – |  |
| >>Cell and Capacity Assistance Information | O |  | 9.2.146 |  | YES | ignore |
| >en-gNB |  |  |  |  |  |  |
| >>Global en-gNB ID | M |  | 9.2.112 |  | YES | reject |
| >>List of Served NR Cells |  | 1 .. <maxCellinengNB> |  | List of cells served by the en-gNB. If a partial list of cells is signalled, it contains at least one cell per carrier configured at the gNB. | YES | reject |
| >>>Served NR Cell Information | M |  | 9.2.110 |  | – |  |
| >>>NR Neighbour Information | O |  | 9.2.98 | NR neighbours | – |  |
| >>Partial List Indicator | O |  | ENUMERATED (partial, ...) | Value "partial" indicates that a partial list of cells is included in the List of Served NR Cells IE | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |
| TNL Transport Layer Address info | O |  | 9.2.149 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxCellinengNB | Maximum no. cells that can be served by an en-gNB. Value is 16384. |

-----------Start of the Next Change-----------

#### 9.1.4.23	PARTIAL RESET CONFIRM

This message is sent by an initiating node to a neighbouring node, both nodes able to interact for EN-DC, to confirm the release all the resources for selected UEs.

Direction: en-gNB  MeNB, MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality |  | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES |  | reject |
| UEs Admitted to be Reset List |  | 1 |  |  |  | YES | reject |
| >UEs Admitted To Be Reset Item |  | 1 .. <maxnoof UEsinengNBDU> |  |  |  |  |  |
| >>MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. |  |  |  |
| >>SgNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. |  |  |  |
| >>MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. |  |  |  |
| Interface Instance Indication | O |  | 9.2.143 |  | YES |  | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES |  | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEsinengNBDU | Maximum no. of UEs. Value is 8192. |

-----------Start of the Next Change-----------

#### 9.1.4.25	E-UTRA – NR CELL RESOURCE COORDINATION RESPONSE

This message is sent by a neighbouring eNB to a peer en-gNB or by a neighbouring en-gNB to a peer eNB, both nodes able to interact for EN-DC, as a response to the E-UTRA – NR CELL RESOURCE COORDINATION REQUEST.

Direction: eNB  en-gNB, en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| CHOICE Responding NodeType | M |  |  |  | - |  |
| >eNB |  |  |  |  |  |  |
| >>Data Traffic Resource Indication | M |  | 9.2.126 | Indicates resource allocations for data traffic. | YES | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | YES | reject |
| >>List of E-UTRA Cells in E-UTRA Coordination Response |  | 0 .. <maxCellineNB> |  | List of applicable E-UTRA cells | YES | reject |
| >>>EUTRA Cell ID | M |  | ECGI 9.2.14 |  | - |  |
| >en-gNB |  |  |  |  |  |  |
| >>Data Traffic Resource Indication | M |  | 9.2.126 | Indicates resource allocations for data traffic. | YES | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the NR cells involved in resource coordination with the E-UTRA cells affiliated with the same Spectrum Sharing Group ID. | YES | reject |
| >>List of NR Cells in NR Coordination Response |  | 0 .. < maxnoNRcellsSpectrumSharingwithE-UTRA > |  | List of applicable NR cells | YES | reject |
| >>>NR Cell ID | M |  | NR-CGI 9.2.111 |  | - |  |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. of E-UTRA cells in eNB. Value is 256. |
| maxnoNRcellsSpectrumSharingwithE-UTRA | Maximum no. of NR cells affiliated to a Spectrum Sharing Group ID involved in cell resource coordination with a number of E-UTRA cells affiliated with the same Spectrum Sharing Group ID. Value is 64. |

-----------Start of the Next Change-----------

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for X2AP.

--

-- **************************************************************

X2AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ABSInformation,

ABS-Status,

AS-SecurityInformation,

BearerType,

//skip unchanged part

-- **************************************************************

--

#### -- EN-DC X2 SETUP RESPONSE

--

-- **************************************************************

ENDCX2SetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCX2SetupResponse-IEs}},

...

}

ENDCX2SetupResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-RespondingNodeType-EndcX2Setup		CRITICALITY reject	TYPE RespondingNodeType-EndcX2Setup		PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication			CRITICALITY reject 	TYPE InterfaceInstanceIndication			PRESENCE optional }|

{ ID id-TNLConfigurationInfo					CRITICALITY ignore	TYPE TNLConfigurationInfo					PRESENCE optional }|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional	},

...

}

RespondingNodeType-EndcX2Setup ::= CHOICE {

respond-eNB			ProtocolIE-Container	{{ENB-ENDCX2SetupReqAckIEs}},

respond-en-gNB		ProtocolIE-Container	{{En-gNB-ENDCX2SetupReqAckIEs}},

...

}

ENB-ENDCX2SetupReqAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID								CRITICALITY reject	TYPE GlobalENB-ID										PRESENCE mandatory}|

{ ID id-ServedEUTRAcellsENDCX2ManagementList		CRITICALITY reject	TYPE ServedEUTRAcellsENDCX2ManagementList		PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject 	TYPE InterfaceInstanceIndication					PRESENCE optional }|

-- NOTE: In the current version of this specification the Interface Instance Indication IE is not included in the Responding NodeType IE --

{ ID id-CellandCapacityAssistInfo				CRITICALITY ignore	TYPE CellandCapacityAssistInfo						PRESENCE optional },

...

}

En-gNB-ENDCX2SetupReqAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-Globalen-gNB-ID							CRITICALITY reject	TYPE GlobalGNB-ID										PRESENCE mandatory}|

{ ID id-ServedNRcellsENDCX2ManagementList		CRITICALITY reject	TYPE ServedNRcellsENDCX2ManagementList			PRESENCE mandatory}|

{ ID id-PartialListIndicator						CRITICALITY ignore	TYPE PartialListIndicator								PRESENCE optional },

...

}

//skip unchanged part

-- **************************************************************

--

#### -- EN-DC PARTIAL RESET REQUIRED

--

-- **************************************************************

ENDCPartialResetRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCPartialResetRequired-IEs}},

...

}

ENDCPartialResetRequired-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-UEs-ToBeReset				CRITICALITY reject	TYPE UEsToBeResetList					PRESENCE mandatory}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause								PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject 	TYPE InterfaceInstanceIndication			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- EN-DC PARTIAL RESET CONFIRM

--

-- **************************************************************

ENDCPartialResetConfirm ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCPartialResetConfirm-IEs}},

...

}

ENDCPartialResetConfirm-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-UEs-Admitted-ToBeReset				CRITICALITY reject	TYPE UEsToBeResetList					PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication			CRITICALITY reject 	TYPE InterfaceInstanceIndication		PRESENCE optional	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

//skip unchanged part

-- **************************************************************

--

#### -- E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE

--

-- **************************************************************

EUTRANRCellResourceCoordinationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{EUTRANRCellResourceCoordinationResponse-IEs}},

...

}

EUTRANRCellResourceCoordinationResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-RespondingNodeType-EutranrCellResourceCoordination		CRITICALITY reject	TYPE RespondingNodeType-EutranrCellResourceCoordination			PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject 	TYPE InterfaceInstanceIndication	PRESENCE optional }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional },

...

}

RespondingNodeType-EutranrCellResourceCoordination ::= CHOICE {

respond-eNB			ProtocolIE-Container	{{ENB-EUTRA-NRCellResourceCoordinationReqAckIEs}},

respond-en-gNB		ProtocolIE-Container	{{En-gNB-EUTRA-NRCellResourceCoordinationReqAckIEs}},

...

}

ENB-EUTRA-NRCellResourceCoordinationReqAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-DataTrafficResourceIndication				CRITICALITY reject	TYPE DataTrafficResourceIndication				PRESENCE mandatory}|

{ ID id-SpectrumSharingGroupID						CRITICALITY reject	TYPE SpectrumSharingGroupID							PRESENCE mandatory}|

{ ID id-ListofEUTRACellsinEUTRACoordinationResp		CRITICALITY reject	TYPE ListofEUTRACellsinEUTRACoordinationResp	PRESENCE mandatory},

...

}

En-gNB-EUTRA-NRCellResourceCoordinationReqAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-DataTrafficResourceIndication		CRITICALITY reject	TYPE DataTrafficResourceIndication			PRESENCE mandatory}|

{ ID id-SpectrumSharingGroupID				CRITICALITY reject	TYPE SpectrumSharingGroupID						PRESENCE mandatory}|

{ ID id-ListofNRCellsinNRCoordinationResp	CRITICALITY reject	TYPE ListofNRCellsinNRCoordinationResp		PRESENCE mandatory},

...

}

ListofEUTRACellsinEUTRACoordinationResp ::= SEQUENCE (SIZE (0..maxCellineNB)) OF ECGI

ListofNRCellsinNRCoordinationResp ::= SEQUENCE (SIZE (0..maxnoNRcellsSpectrumSharingWithE-UTRA)) OF NRCGI

-----------End of the Changes-----------