3GPP TSG-RAN WG3 Meeting #119                                                      	R3-230369

Athens, Greece, Feb 27th – Mar 3rd, 2023

Agenda item:	11.2

Source:	Qualcomm Incorporated

Title:	QoE enhancements for high mobility scenarios

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss further on QoE enhancements for high mobility scenarios based on the agreements last meeting.

# Discussion

The following was agreed last meeting:

OAM should have the flexibility to collect QoE only in high mobility scenarios and/or in HSDN cells instead of collecting blindly. FFS whether enhancements are needed to support the requirement

The following aspects on high-speed scenario shall be discussed in next meeting:

Whether a “HSDN wide indication” can be included in the Area Scope of QoE configuration (from OAM to gNB), instead of OAM being required to provide the whole list of HSDN cells.

Whether the ‘high UE velocity’ indication can be added into the QoE configuration.

  - HSDN-wide indication in Area Scope of QoE configuration

We propose to discuss the HSDN cells scenario first i.e., the potential enhancements needed for collecting QoE from only HSDN cells.

Observation 1: OAM knows whether a cell is a HSDN cell or non-HSDN cell

Observation 2: If OAM is interested in collecting QoE from all HSDN cells, currently it needs to provide the list of all HSDN cells in the Area Scope of QMC

In case of m-based QoE, OAM should be able to indicate that it is interested in collecting QoE only from only HSDN cells by providing a “HSDN only” indicator in Area Scope of QMC. This is similar to a discussion we are having currently in Rel-18 SON/MDT for NPN where the CAG ID can be included in Area Scope of MDT to collect MDT only specific CAG cells.

Proposal 1: In case of m-based QoE, OAM should be able to indicate that it is interested in collecting QoE only from HSDN cells by providing a “HSDN-wide” indicator in Area Scope of QMC

Proposal 2: Upon UE mobility outside the area scope (from a HSDN cell to non-HSDN cell), gNB can release the QoE config (if there is no ongoing session) or pause the QoE config to ensure QoE is collected only in HSDN cells

  - QoE enhancements for UEs in high mobility state

Let’s now discuss the potential enhancements needed for collecting QoE from only UEs in high mobility state.

Observation 3: The gNB knows a UE’s mobility state after receiving the mobilityState in RRC Complete messages

Observation 4: OAM doesn’t know a UE’s mobility state directly and it might only know if provided in SON/MDT reports

Proposal 3: If OAM is interested in collecting QoE from only UEs in high mobility state, it can provide a “High Mobility State Only” indicator in m-based QoE configuration sent to gNB

Proposal 4: Upon receiving a “High Mobility State Only” indicator in m-based QoE configuration, a gNB can select only those UEs which are in high mobility state

Proposal 5: Upon UE changing mobility states (from a High Mobility state to Low/Medium mobility state), gNB can release/pause the QoE config to ensure QoE is collected only in high mobility state

Proposal 6: Use the following table as reference to add the “HSDN-wide” and “High Mobility State only” indicator in Area Scope of QMC:

| CHOICE Area Scope of QMC | M |  |  |  |
| --- | --- | --- | --- | --- |
| >Cell based |  |  |  |  |
| >>Cell ID List for QMC |  | 1..<maxnoofCellIDforQMC> |  |  |
| >>>NG-RAN CGI | M |  | 9.3.1.73 | This IE can only indicate the NR CGI. |
| >TA based |  |  |  |  |
| >>TA List for QMC |  | 1..<maxnoofTAforQMC> |  |  |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. |
| >TAI based |  |  |  |  |
| >>TAI List for QMC |  | 1..<maxnoofTAforQMC> |  |  |
| >>>TAI | M |  | 9.3.3.11 |  |
| >PLMN area based |  |  |  |  |
| >>PLMN List for QMC |  | 1..<maxnoofPLMNforQMC> |  |  |
| >>>PLMN Identity | M |  | 9.3.3.5 |  |
| >HSDN-wide |  |  |  |  |
| >High Mobility State Only |  |  |  |  |

# Conclusion

Observation 1: OAM knows whether a cell is a HSDN cell or non-HSDN cell

Observation 2: If OAM is interested in collecting QoE from all HSDN cells, currently it needs to provide the list of all HSDN cells in the Area Scope of QMC

Proposal 1: In case of m-based QoE, OAM should be able to indicate that it is interested in collecting QoE only from HSDN cells by providing a “HSDN only” indicator in Area Scope of QMC

Proposal 2: Upon UE mobility outside the area scope (from a HSDN cell to non-HSDN cell), gNB can release the QoE config (if there is no ongoing session) or pause the QoE config to ensure QoE is collected only in HSDN cells

Observation 3: The gNB knows a UE’s mobility state after receiving the mobilityState in RRC Complete messages

Observation 4: OAM doesn’t know a UE’s mobility state directly and it might only know if provided in SON/MDT reports

Proposal 3: If OAM is interested in collecting QoE from only UEs in high mobility state, it can provide a “High Mobility State Only” indicator in m-based QoE configuration sent to gNB

Proposal 4: Upon receiving a “High Mobility State Only” indicator in m-based QoE configuration, a gNB can select only those UEs which are in high mobility state

Proposal 5: Upon UE changing mobility states (from a High Mobility state to Low/Medium mobility state), gNB can release/pause the QoE config to ensure QoE is collected only in high mobility state

Proposal 6: Use the following table as reference to add the “HSDN-wide” and “High Mobility State only” indicator in Area Scope of QMC:

| CHOICE Area Scope of QMC | M |  |  |  |
| --- | --- | --- | --- | --- |
| >Cell based |  |  |  |  |
| >>Cell ID List for QMC |  | 1..<maxnoofCellIDforQMC> |  |  |
| >>>NG-RAN CGI | M |  | 9.3.1.73 | This IE can only indicate the NR CGI. |
| >TA based |  |  |  |  |
| >>TA List for QMC |  | 1..<maxnoofTAforQMC> |  |  |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. |
| >TAI based |  |  |  |  |
| >>TAI List for QMC |  | 1..<maxnoofTAforQMC> |  |  |
| >>>TAI | M |  | 9.3.3.11 |  |
| >PLMN area based |  |  |  |  |
| >>PLMN List for QMC |  | 1..<maxnoofPLMNforQMC> |  |  |
| >>>PLMN Identity | M |  | 9.3.3.5 |  |
| >HSDN-wide |  |  |  |  |
| >High Mobility State Only |  |  |  |  |

# 4. References