3GPP RAN3 Meeting #119	R3-230453

E-meeting, February 27th - March 3rd 2023

# Title:	Reply LS on Partially allowed/rejected NSSAI

# Response to:	S2-2301467

# Release:	Rel-18

# Work Item:	eNS_Ph3

Source:	RAN3

To:	SA2

Cc:	RAN2

Contact Person:

Name:	Angelo Centonza

E-mail Address:	angelo (dot) centonza (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: none

1. Overall Description:

RAN3 would like to thank SA2 for their LS on Partially allowed/rejected NSSAI.

RAN3 has discussed the questions posed by SA2 and it has concluded on the following answers.

Question 1: SA2 asks RAN3 if the Partially Allowed NSSAI is useful for NG-RAN to get over NG-AP in all messages where the Allowed NSSAI is sent?

Answer 1: The introduction of signalling of S-NSSAIs forming the Partially Allowed NSSAI to the RAN may either be part of the existing Allowed NSSAI or it may be a separate IE. Including Partially Allowed NSSAI information in the Allowed NSSAI has the advantage of not impacting the RAN.

Question 2: if the answer to Question 1 is "yes", whether it is needed to send also the Partially Allowed NSSAI with or without the associated TA-list for each S-NSSAI in the Partially Allowed NSSAI

Answer 2: Signalling a Partially Allowed NSSAI with the associated TA-list for each S-NSSAI in the Partially Allowed NSSAI is not necessary to enable the RAN to optimise RRM processes. For the purpose of mobility, the Target NSSAI and Target RFSP provide more useful information that help the RAN deciding where the UE should be moved.

Question 3: is it possible and feasible in rel-18 in the RAN to leverage the received Partially Allowed NSSAI to e.g. deactivate the PDU session, or trigger reporting of entering or exiting a S-NSSAI area of support to AMF?

Answer 3:

The RAN is responsible for admission/rejection/removal of PDU Session resources. Current specifications already enable the RAN to notify that PDU session resource(s) for a given UE are released. Furthermore, the AMF can request the NG-RAN to release already established PDU session resources for a given UE.
For PDU session resources associated to S-NSSAIs not supported by a serving/target cell in an NG-RAN, the serving/target NG-RAN node shall reject admission of such PDU session resources.

With respect to the triggering of entering or exiting a S-NSSAI area of support to AMF, it is already possible for the AMF to configure Location Information Reporting at the RAN, where the Area of Interest may be identified by a set of TAs where one or more partially allowed network slice is supported.

Question 4: Should the S-NSSAIs of the Rejected S-NSSAI for part of the RA be made available to NG-RAN for RRM purposes e.g. so that the RAN can consider this information for RRM purposes e.g. to steer the UE to bands supporting also this S-NSSAI?

Answer 4: There is no need to make the S-NSSAIs of the Rejected S-NSSAI for part of the RA available to the NG-RAN. If steering of the UE towards other frequency bands is desired, the Target NSSAI may be used to provide information to the RAN that can be used to enable such steering.

2. Actions:

To SA2

ACTION: 	
RAN3 kindly asks SA2 to take the above into account.

3. Date of Next RAN3 Meetings:

Please check the RAN3 calendar here