3GPP TSG-RAN3 #119	R3-230458

27th February – 3rd March 2023

Athens, Greece

Title: 	(TP for AI/ML BLCR to TS38.423) AI/ML Network Energy Saving Procedures

Source: 	Ericsson

Agenda item: 	12.2.2.3

Document Type:	Discussion and Decision

# Introduction

In this paper a TP is presented reflecting the proposals in R3-230480

# TP for TS 38.423

Below, a draft TP is presented where the proposals made in this paper are reflected.

<<<<<< Start of CHANGEs >>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5QI	5G QoS Identifier

AI	Artificial Intelligence

AMF	Access and Mobility Management Function

BH	Backhaul

CAG	Closed Access Group

CGI	Cell Global Identifier

CHO	Conditional Handover

CP	Control Plane

CPA	Conditional PSCell Addition

CPAC	Conditional PSCell Addition or Change

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DL	Downlink

EN-DC	E-UTRA-NR Dual Connectivity

E-RAB	E-UTRAN Radio Access Bearer

GUAMI	Globally Unique AMF Identifier

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

MBS	Multicast/Broadcast Service

MCG	Master Cell Group

ML	Machine Learning

M-NG-RAN node	Master NG-RAN node

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NSSAI	Network Slice Selection Assistance Information

PEIPS	Paging Early Indication with Paging Subgrouping

PNI-NPN	Public Network Integrated Non-Public Network

ProSe 	Proximity Services

RANAC	RAN Area Code

RedCap	Reduced Capability

RSN	Redundancy Sequence Number

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SNPN	Stand-alone Non-Public Network

S-NG-RAN node	Secondary NG-RAN node

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

SDT	Small Data Transmission

TAC	Tracking Area Code

TAI	Tracking Area Identity

UL	Uplink

UPF	User Plane Function

V2X	Vehicle-to-Everything

<<<<<< NEXT CHANGE >>>>>>

# 8	XnAP procedures

## 8.1	Elementary procedures

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| IAB Transport Migration Management | IAB TRANSPORT MIGRATION MANAGEMENT REQUEST | IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE | IAB TRANSPORT MIGRATION MANAGEMENT REJECT |
| IAB Transport Migration Modification | IAB TRANSPORT MIGRATION MODIFICATION REQUEST | IAB TRANSPORT MIGRATION MODIFICATION RESPONSE |  |
| IAB Resource Coordination | IAB RESOURCE COORDINATION REQUEST | IAB RESOURCE COORDINATION RESPONSE |  |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE | PARTIAL UE CONTEXT TRANSFER FAILURE |
| AI-ML Action Evaluation | AI-ML Action Evaluation Request | AI-ML Action Evaluation Response | AI-ML Action Evaluation Failure |

<<<<<< NEXT CHANGE >>>>>>

## 8.4	Global procedures

<<<<<< UNCHANGED TEXT OMITTED >>>>>>

### 8.4.xx	AI-ML Action Evaluation

#### 8.4.xx.1	General

This message is sent by NG-RAN node1 to NG-RAN node2 to request AI-ML assistance data for a planned AI-ML event by NG-RAN node1.

Direction: NG-RAN node1  NG-RAN node2.

The procedure uses non-UE-associated signalling.

#### 8.4.xx.2	Successful Operation

Figure 8.4.xx.2-1: AI-ML Assistance Data Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the AI-ML ACTION EVALUATION REQUEST message to NG-RAN node2 to request assistance data in the context of a planned AI-ML event/action by NG-RAN node1. Upon receipt, NG-RAN node2:

-	shall respond with the requested assistance data reporting according to the parameters given in the Requested DATA IE

If NG-RAN node2 is capable of providing all of the requested AI/ML assistance data, it shall respond with the AI-ML ACTION EVALUATION RESPONSE message.

If NG-RAN node2 cannot provide all the requested AI/ML assistance data, NG-RAN node2 shall respond with the AI-ML ACTION EVALUATION FAILURE message.

When providing AI/ML action evaluation assistance, the Requested Data IE in the AI-ML ACTION EVALUATION REQUEST indicates the type of objects NG-RAN node2 shall report assistance data on.

NG-RAN node2 shall include in the AI-ML ACTION EVALUATION RESPONSE message:

-	the Predicted Energy Efficiency IE considering the proposed action, if the first bit, "Predicted Energy Efficiency " of the Requested Data IE included in the AI-ML ACTION EVALUATION REQUEST message is set to "1".

#### 8.4.xx.3	Unsuccessful Operation

Figure 8.4.xx.3-1: AI-ML Assistance Data Reporting Initiation, unsuccessful operation

If any of the requested measurements cannot be initiated, NG-RAN node2 shall send the AI-ML ACTION EVALUATION FAILURE message

#### 8.4.xx.4	Abnormal Conditions

If the initiating NG-RAN node1 does not receive either the AI-ML ACTION EVALUATION RESPONSE message or the AI-ML ACTION EVALUATION FAILURE message, the NG-RAN node1 may reinitiate the AI-ML Action Evaluation Request procedure towards the same NG-RAN node, provided that the content of the new AI-ML ACTION EVALUATION REQUEST message is identical to the content of the previously unacknowledged AI-ML ACTION EVALUATION REQUEST message.

If the Requested Data IE bitmap is set to "0" (all bits are set to "0") in the AI-ML ACTION EVALUATION REQUEST message, then NG-RAN node2 shall initiate an AI-ML ACTION EVALUATION FAILURE message with an appropriate cause value.

If the initiating NG-RAN node1 receives the AI-ML ACTION EVALUATION RESPONSE message including the Per Cell Measurement Result IE containing no measurements, the NG-RAN node1 shall consider the procedure as failed.

<<<<<< NEXT CHANGE >>>>>>

### 9.1.3	Messages for Global Procedures

<<<<<< UNCHANGED TEXT OMITTED >>>>>>

#### 9.1.3.xx	 AI/ML ACTION EVALUATION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to request AI-ML assistance data in the context of a planned AI-ML event by NG-RAN node1.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Requested Data | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the NG-RAN node2 is requested to report. First Bit = Predicted energy efficiency, Other bits shall be ignored by the NG-RAN node2. | YES | reject |
| Proposed Action | M |  | 9.2.2.xx | Proposed Action Information | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | Reject |

#### 9.1.3.ww	AI-ML ACTION EVALUATION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to provide the requested information concerning planned actions

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Predicted Energy Efficiency | O |  | INTEGER (0..100) | Value 1 indicates the minimum Predicted Energy Efficiency and 100 indicates the maximum Predicted Energy Efficiency. Predicted Energy Efficiency should be measured on a linear scale |  |  |

#### 9.1.3.zz	AI-ML ACTION EVALUATION FAILURE

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for any of the requested assistance data the measurement cannot be initiated.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

<<<<<< NEXT CHANGE >>>>>>

9.2.2 NG-RAN Node and Cell Configuration related IE definitions

<<<<<< UNCHANGED TEXT OMITTED >>>>>>

#### 9.2.2.xx Proposed Action

This IE provides description of the planned action.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Proposed Action | M |  |  |  | YES | reject |
| >Offloading Request |  |  |  |  |  |  |
| >>Offloading Request Information | M |  | 9.2.2.yy |  | - |  |

#### 9.2.2.yy Offloading Request Information

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Source Cell Global ID | M |  | 9.2.3.25 | CGI of the source cell from which offloading is triggered. Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | CGI of the target cell to which offloading is triggered. Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| Offloading Ratio | O |  | INTEGER (0..100) | The percentage of the source cell load to be offloaded. 0 indicates no load and 100 indicates offloading all the source cell load. Load should be measured on a linear scale. |  |  |
| Number of UEs | O |  |  | Number of UEs to be handed over |  |  |

<<<<<< End of CHANGES >>>>>