3GPP TSG-RAN3 Meeting #119	R3-230515

Athens, Greece, 27th February - 3rd March 2023

# Title:	Draft Reply LS on Partially allowed/rejected NSSAI

Response to:        S2-2301467/R3-230040

# Release:	Rel-18

# Work Item:	eNS_Ph3

Source:                  Qualcomm Incorporated [to be RAN3]

To:                          SA2

Cc:                          -

Contact Person:

Name:	Shankar Krishnan

E-mail Address:	shakrish@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: None

1. Overall Description:

RAN3 would like to thank SA2 for their LS on Partially allowed/rejected NSSAI. RAN3 would like to provide the following feedback.

Question 1: SA2 asks RAN3 if the Partially Allowed NSSAI is useful for NG-RAN to get over NG-AP in all messages where the Allowed NSSAI is sent?

RAN3’s answer: Yes, RAN3 agrees that it is beneficial if AMF can signal the Partially Allowed NSSAI in all NGAP messages where Allowed NSSAI is present.

Question 2: if the answer to Question 1 is "yes", whether it is needed to send also the Partially Allowed NSSAI with or without the associated TA-list for each S-NSSAI in the Partially Allowed NSSAI

RAN3’s answer: Yes, it is beneficial for the NG-RAN node to know the Partially allowed NSSAI with the associated TA-list for each NSSAI, so that the NG-RAN node can make appropriate handover decisions.

Question 3: is it possible and feasible in rel-18 in the RAN to leverage the received Partially Allowed NSSAI to e.g., deactivate the PDU session, or trigger reporting of entering or exiting a S-NSSAI area of support to AMF?

RAN3’s answer: Yes, it is feasible for the NG-RAN node to leverage the Partially Allowed NSSAI (if received from AMF) to make appropriate handover decisions. NG-RAN can also take the Partially Allowed NSSAI into account and use location reporting procedures to inform AMF (which informs the SMF) upon entering or exiting a S-NSSAI area of interest. SMF can then deactivate the PDU session if needed.

Question 4: Should the S-NSSAIs of the Rejected S-NSSAI for part of the RA be made available to NG-RAN for RRM purposes e.g., so that the RAN can consider this information for RRM purposes e.g., to steer the UE to bands supporting also this S-NSSAI?

RAN3’s answer: Yes. Similar to Partially Allowed NSSAI, it can be useful if the S-NSSAIs of the Rejected S-NSSAI for part of the RA are made available to NG-RAN.

2. Actions:

To SA2

ACTION: 	RAN3 kindly asks SA2 to take the above information into account and provide feedback if any.

3. Date of Next TSG RAN WG3 Meetings:

RAN3#119bis-e           April 17 – April 26, 2023                                E-Meeting

RAN3#120                   May 22 - May 26, 2023                                 Incheon, South Korea