3GPP TSG-RAN WG3 Meeting #119	R3-230519

# Athens, Greece, 27 February-3 March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1735 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | X2AP corrections of references to RRC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NewRAT-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2023-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the agreed guidelines in RAN3#118 meeting, the references to RRC specifications need to be corrected in X2AP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the references to RRC specifications according to the agreed rules. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The CR has no impact from an ASN.1 point of view and does not introduce functional changes for implementations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity remains in X2AP about RRC references. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.7.2, 9.1.1.1. 9.1.1.2, 9.1.2.18, 9.2.1.19, 9.1.2.28, 9.1.2.29, 9.1.2.50, 9.1.4.1, 9.1.4.4, 9.1.4.5, 9.1.4.6, 9.1.4.8, 9.1.4.9, 9.1.4.14, 9.1.4.18, 9.1.4.21, 9.1.5.1, 9.2.8, 9.2.56, 9.2.76, 9.2.91, 9.2.98, 9.2.110, 9.2.138, 9.2.144 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.7.7	SgNB initiated SgNB Modification

#### 8.7.7.1	General

This procedure is used by the en-gNB to modify the UE context in the en-gNB.

The procedure uses UE-associated signalling.

#### 8.7.7.2	Successful Operation

Figure 8.7.7.2-1: SgNB initiated SgNB Modification, successful operation.

The en-gNB initiates the procedure by sending the SGNB MODIFICATION REQUIRED message to the MeNB. When the en-gNB sends the SGNB MODIFICATION REQUIRED message, it shall start the timer TDCoverall.

The SGNB MODIFICATION REQUIRED message may contain

-	the PDCP Change Indication IE;

-	the SgNB to MeNB Container IE.

-	E-RABs to be modified within the E-RABs To Be Modified Item IE;

-	E-RABs to be released within the E-RABs To Be Released Item IE;

-	the SgNB Resource Coordination Information IE.

For the SN terminated split bearers, the en-gNB may include in the SGNB MODIFICATION REQUIRED message the UL Configuration IE to indicate that the MCG UL configuration of the UE has changed.

The en-gNB may include for each bearer in the E-RABs to Be Modified List IE in the SGNB MODIFICATION REQUIRED message the New DRB ID Request IE to request the MeNB to assign a new DRB ID for that bearer.

If the MeNB is able to perform the change requested by the en-gNB, the MeNB shall send the SGNB MODIFICATION CONFIRM message to the en-gNB. The SGNB MODIFICATION CONFIRM message may contain the MeNB to SgNB Container IE.

If the SGNB MODIFICATION REQUIRED message contains the SgNB Resource Coordination Information IE, the MeNB may use it for the purpose of resource coordination with the en-gNB. The MeNB shall consider the received UL Coordination Information IE value valid until reception of a new update of the IE for the same UE. The MeNB shall consider the received DL Coordination Information IE value valid until reception of a new update of the IE for the same UE. If the SgNB Coordination Assistance Information IE is contained in the SgNB Resource Coordination Information IE, the MeNB shall, if supported, use the information to determine further coordination of resource utilisation between the en-gNB and the MeNB.

If the en-gNB applied a full configuration or delta configuration, e.g. as part of a mobility procedure involving a change of DU, the en-gNB shall inform the MeNB by including the RRC config indication IE in the SGNB MODIFICATION REQUIRED message.

For each E-RAB successfully modified as requested by the en-gNB, the MeNB shall inform the en-gNB, in the SGNB MODIFICATION CONFIRM message, the same value in the EN-DC Resource Configuration IE as received in the SGNB MODIFICATION REQUIRED message.

If the SCG resources IE in the EN-DC Resource Configuration IE in the SGNB MODIFICATION REQUIRED message for all the E-RABs of the UE are set to “not present”, the MeNB shall, if supported, deduce that the SCG resources are removed.

Upon reception of the SGNB MODIFICATION CONFIRM message the en-gNB shall stop the timer TDCoverall.

If the SGNB MODIFICATION CONFIRM message contains the MeNB Resource Coordination Information IE, the en-gNB should forward it to lower layers and it may use it for the purpose of resource coordination with the MeNB, or to coordinate with sidelink resources used in the MeNB. The en-gNB shall consider the received UL Coordination Information IE value valid until reception of a new update of the IE for the same UE. The en-gNB shall consider the received DL Coordination Information IE value valid until reception of a new update of the IE for the same UE. If the MeNB Coordination Assistance Information IE is contained in the MeNB Resource Coordination Information IE, the en-gNB shall, if supported, use the information to determine further coordination of resource utilisation between the en-gNB and the MeNB.

If the MeNB receives for an E-RAB for which the PDCP entiy is allocated at the MeNB the Secondary SgNB DL GTP Tunnel Endpoint at SCG IE in the SGNB MODIFICATION REQUIRED message, it shall provide the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE to the en-gNB in the SGNB MODIFICATION CONFIRM message. If the LCID IE is included in the SGNB MODIFICATION REQUIRED message, the MeNB should take it into account.

If the SGNB MODIFICATION REQUIRED message contains the RLC Status IE, the MeNB shall assume that RLC has been reestablished at the en-gNB and may trigger PDCP data recovery.

If the RLC Mode IE is included for an E-RAB within the E-RABs To Be Released List IE (for E-RABs hosted at the en-gNB) in the SGNB MODIFICATION REQUIRED message, it indicates the mode that the en-gNB used for the E-RAB when it was hosted at the en-gNB.

The MeNB shall include only E-RABs with the following IE in E-RABs Admitted To Be Modified List IE:

-	the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE.

If the Location Information at SgNB IE is included in the SGNB MODIFICATION REQUIRED, the MeNB shall store the included information so that it may be transferred towards the MME.

For each requested E-RAB configured as MN-terminated split bearer/SCG bearer, if the QoS Mapping Information IE is included in the GTP Tunnel Endpoint IE in the SGNB MODIFICATION REQUIRED message, the MeNB shall, if supported, use it to set DSCP and/or flow label fields for the downlink IP packets which are transmitted from MeNB to SgNB through the GTP tunnels indicated by the GTP Tunnel Endpoint IE.

If the SGNB MODIFICATION REQUIRED message includes the SCG UE History Information IE, the MeNB shall, if supported, use the information to update the UE History Information with PSCell history.

If the SCG Activation Request IE is included in the SGNB MODIFICATION REQUIRED message, the MeNB shall, if supported, consider that the en-gNB node is about to reconfigure the SCG resources as specified in TS 37.340 [32].

If the CPAC Information Required IE is included in the SGNB MODIFICATION REQUIRED message, the MeNB shall, if supported, consider that the request provides the configuration update for the list of PSCells prepared at the target en-gNB, as described in TS 37.340 [32].

If the CG-CandidateList is included in the SgNB to MeNB Container IE in the SGNB MODIFICATION REQUIRED message, the MeNB shall, if supported, use it for the purpose of CPAC.

##### If the SCG Reconfiguration Notification IE is included in the SGNB MODIFICATION REQUIRED message and set to "executed", the MeNB shall, if supported, consider that a prepared SN-initiated intra-SN CPC procedure or a reconfiguration with sync of the SCG using SRB3 has been executed, as specificed in TS 37.340 [32]. If the SgNB to MeNB Container IE is also included in the SGNB MODIFICATION REQUIRED message, the MeNB shall, if supported, consider that the received SCG configuration has already been applied in the UE and should not be forwarded to the UE.

##### Interaction with the MeNB initiated SgNB Modification Preparation procedure:

If applicable, as specified in TS 37.340 [32], the en-gNB may receive, after having initiated the SgNB initiated SgNB Modification procedure, the SGNB MODIFICATION REQUEST message including the DL Forwarding GTP Tunnel Endpoint IE and the UL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released List IE.

If applicable, as specified in TS 37.340 [32], the en-gNB may receive, after having initiated the SgNB initiated SgNB Modification procedure, the SGNB MODIFICATION REQUEST message including the SgNB Security Key IE within the UE Context Information IE.

If applicable, as specified in TS 37.340 [32], the en-gNB may receive, after having initiated the SgNB initiated SgNB Modification procedure, the SGNB MODIFICATION REQUEST message including the measGapConfig contained in the CG-ConfigInfo message as defined in TS 38.331 [31] within the MeNB to SgNB Container IE.

The en-gNB may receive, after having initiated the SgNB initiated SgNB modification procedure including the New DRB ID Request IE for an SN terminated bearer within the E-RABs To Be Modified List IE, the SGNB MODIFICATION REQUEST message to release and add the same bearer with a new DRB ID or with the same DRB ID but together with the SgNB Security Key IE within the UE Context Information IE.

The en-gNB may receive, after having initiated the SgNB initiated SgNB modification procedure, the SGNB MODIFICATION REQUEST message including the SN triggered IE.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

# 9	Elements for X2AP Communication

## 9.0	General

Sub clauses 9.1 and 9.2 describe the structure of the messages and information elements required for the X2AP protocol in tabular format. Sub clause 9.3 provides the corresponding ASN.1 definition.

The following attributes are used for the tabular description of the messages and information elements: Presence, Range Criticality and Assigned Criticality. Their definition and use can be found in TS 36.413 [4].

NOTE:	The messages have been defined in accordance to the guidelines specified in TR 25.921 [30].

## 9.1	Message Functional Definition and Content

### 9.1.1	Messages for Basic Mobility Procedures

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source eNB to the target eNB to request the preparation of resources for a handover.

Direction: source eNB  target eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| GUMMEI | M |  | 9.2.16 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >MME UE S1AP ID | M |  | INTEGER (0..232 -1) | MME UE S1AP ID allocated at the MME | – |  |
| >UE Security Capabilities | M |  | 9.2.29 |  | – |  |
| >AS Security Information | M |  | 9.2.30 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.12 |  | – |  |
| >Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.25 |  | – |  |
| >E-RABs To Be Setup List |  | 1 |  |  | – |  |
| >>E-RABs To Be Setup Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – |  |
| >>>DL Forwarding | O |  | 9.2.5 |  | – |  |
| >>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – |  |
| >>>Bearer Type | O |  | 9.2.92 |  | YES | reject |
| >>>DAPS Request Information | O |  | 9.2.154 |  | YES | ignore |
| >>>Ethernet Type | O |  | 9.2.157 |  | YES | ignore |
| >>>Source DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >>>Security Indication | O |  | 9.2.181 |  | YES | reject |
| >RRC Context | M |  | OCTET STRING | Includes the HandoverPreparationInformation message as defined in subclause 10.2.2 of TS 36.331 [9], or the HandoverPreparationInformation-NB message as defined in 10.6.2 of TS 36.331 [9]. | – |  |
| >Handover Restriction List | O |  | 9.2.3 |  | – |  |
| >Location Reporting Information | O |  | 9.2.21 | Includes the necessary parameters for location reporting | – |  |
| >Management Based MDT Allowed | O |  | 9.2.59 |  | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.64 |  | YES | ignore |
| >UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.97 | This IE applies only if the UE is authorized for V2X services. | YES | ignore |
| >EPC Handover Restriction List Container | O |  | 9.2.153 |  | YES | ignore |
| >Additional RRM Policy Index | O |  | 9.2.25a |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.159 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.171 |  | YES | reject |
| >IMS voice EPS fallback from 5G | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| UE History Information | M |  | 9.2.38 | Same definition as in TS 36.413 [4] | YES | ignore |
| Trace Activation | O |  | 9.2.2 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.33 |  | YES | ignore |
| CSG Membership Status | O |  | 9.2.52 |  | YES | reject |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source eNB provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| Masked IMEISV | O |  | 9.2.69 |  | YES | ignore |
| UE History Information from the UE | O |  | OCTET STRING | Includes the VisitedCellInfoList IE contained in the UEInformationResponse message (TS 36.331 [9]) | YES | ignore |
| Expected UE Behaviour | O |  | 9.2.70 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.78 |  | YES | ignore |
| UE Context Reference at the SeNB | O |  |  |  | YES | ignore |
| >Global SeNB ID | M |  | Global eNB ID 9.2.22 |  | – |  |
| >SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | – |  |
| >SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | – |  |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | reject |
| V2X Services Authorized | O |  | 9.2.93 |  | YES | ignore |
| UE Context Reference at the WT | O |  |  |  | YES | ignore |
| >WT ID | M |  | 9.2.95 |  | – |  |
| >WT UE XwAP ID | M |  | 9.2.96 |  | – |  |
| NR UE Security Capabilities | O |  | 9.2.107 |  | YES | ignore |
| UE Context Reference at the SgNB | O |  |  |  | YES | ignore |
| >Global en-gNB ID | M |  | 9.2.112 |  | – |  |
| >SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the SgNB. | – |  |
| Aerial UE subscription information | O |  | 9.2.129 |  | YES | ignore |
| Subscription Based UE Differentiation Information | O |  | 9.2.136 |  | YES | ignore |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >New eNB UE X2AP ID | C-ifCHOmod |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | – |  |
| >New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.158 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.160 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

#### 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target eNB to inform the source eNB about the prepared resources at the target.

Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | ignore |
| E-RABs Admitted List |  | 1 |  |  | YES | ignore |
| >E-RABs Admitted Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>UL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – |  |
| >>DL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer. used for forwarding of DL PDUs | – |  |
| >>DAPS Response Information | O |  | 9.2.155 |  | YES | reject |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |
| Target eNB To Source eNB Transparent Container | M |  | OCTET STRING | Includes the HandoverCommand message as defined in subclause 10.2.2 in TS 36.331 [9] | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.85 |  | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | YES | reject |
| WT UE Context Kept Indicator | O |  | UE Context Kept Indicator 9.2.85 | Indicates that the WT has acknowledged to keep the UE context | YES | ignore |
| E-RABs transferred to MeNB | O |  | E-RAB List 9.2.28 | In case of EN-DC, indicates that SN Status is needed for the listed E-RABs from the SgNB.. | YES | ignore |
| Conditional Handover Information Acknowledge | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | ECGI 9.2.14 | Target cell indicated in the corresponding HANDOVER REQUEST message | – |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.156 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

#### 9.1.1.3	HANDOVER PREPARATION FAILURE

This message is sent by the target eNB to inform the source eNB that the Handover Preparation has failed.

Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | ignore |
| Requested Target Cell ID | O |  | ECGI 9.2.14 | Target cell indicated in the corresponding HANDOVER REQUEST message | YES | reject |

#### 9.1.1.4	SN STATUS TRANSFER

This message is sent by the source eNB to the target eNB to transfer the uplink/downlink PDCP SN and HFN status during a handover or for EN-DC.

Direction: source eNB  target eNB (handover), eNB from which the E-RAB context is transferred  eNB to which the E-RAB context is transferred (RRC connection re-establishment or dual connectivity), MeNB/en-gNB from which the E-RAB context is transferred  en-gNB/MeNB to which the E-RAB context is transferred (EN-DC).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated for handover at the source eNB and for dual connectivity/EN-DC at the eNB from which the E-RAB context is transferred | YES | reject |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated for handover at the target eNB and for dual connectivity/EN-DC at the eNB to which the E-RAB context is transferred | YES | reject |
| E-RABs Subject To Status Transfer List |  | 1 |  |  | YES | ignore |
| >E-RABs Subject To Status Transfer Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>Receive Status Of UL PDCP SDUs | O |  | BIT STRING (4096) | PDCP Sequence Number = (First Missing SDU Number + bit position) modulo 4096 0: PDCP SDU has not been received. 1: PDCP SDU has been received correctly. | – |  |
| >>UL COUNT Value | M |  | COUNT Value 9.2.15 | PDCP-SN and Hyper Frame Number of the first missing UL SDU in case of 12 bit long PDCP-SN | – |  |
| >>DL COUNT Value | M |  | COUNT Value 9.2.15 | PDCP-SN and Hyper frame number that the target eNB/en-gNB should assign for the next DL SDU not having an SN yet in case of 12 bit long PDCP-SN | – |  |
| >>Receive Status Of UL PDCP SDUs Extended | O |  | BIT STRING (1..16384) | The IE is used in case of 15 bit long PDCP-SN in this release. The first bit indicates the status of the SDU after the First Missing UL PDCP SDU. The Nth bit indicates the status of the UL PDCP SDU in position (N + First Missing SDU Number) modulo (1 + the maximum value of the PDCP-SN). 0: PDCP SDU has not been received. 1: PDCP SDU has been received correctly. | YES | ignore |
| >>UL COUNT Value Extended | O |  | COUNT Value Extended 9.2.66 | PDCP-SN and Hyper Frame Number of the first missing UL SDU in case of 15 bit long PDCP-SN | YES | ignore |
| >>DL COUNT Value Extended | O |  | COUNT Value Extended 9.2.66 | PDCP-SN and Hyper Frame Number that the target eNB/en-gNB should assign for the next DL SDU not having an SN yet in case of 15 bit long PDCP-SN | YES | ignore |
| >>Receive Status Of UL PDCP SDUs for PDCP SN Length 18 | O |  | BIT STRING (1..131072) | The IE is used in case of 18 bit long PDCP-SN. The first bit indicates the status of the SDU after the First Missing UL PDCP SDU. The Nth bit indicates the status of the UL PDCP SDU in position (N + First Missing SDU Number) modulo (1 + the maximum value of the PDCP-SN). 0: PDCP SDU has not been received. 1: PDCP SDU has been received correctly. | YES | ignore |
| >>UL COUNT Value for PDCP SN Length 18 | O |  | COUNT Value for PDCP SN Length 18 9.2.82 | PDCP-SN and Hyper Frame Number of the first missing UL SDU in case of 18 bit long PDCP-SN | YES | ignore |
| >>DL COUNT Value for PDCP SN Length 18 | O |  | COUNT Value for PDCP SN Length 18 9.2.82 | PDCP-SN and Hyper Frame Number that the target eNB/en-gNB should assign for the next DL SDU not having an SN yet in case of 18 bit long PDCP-SN | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated for handover at the source eNB and for dual connectivity/EN-DC at the eNB from which the E-RAB context is transferred. | YES | reject |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated for handover at the target eNB and for dual connectivity/EN-DC at the eNB to which the E-RAB context is transferred. | YES | reject |
| SgNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated for EN-DC at the SgNB. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256. |

#### 9.1.1.5	UE CONTEXT RELEASE

This message is sent by the target eNB to the source eNB to indicate that resources can be released.

Direction: target eNB  source eNB (handover), MeNB  SeNB (dual connectivity), MeNB  en-gNB (EN-DC).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated for handover at the source eNB and for dual connectivity at the SeNB. | YES | reject |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated for handover at the target eNB and for dual connectivity/EN-DC at the MeNB. | YES | reject |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated for handover at the source eNB and for dual connectivity at the SeNB. | YES | reject |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated for handover at the source eNB and for dual connectivity/EN-DC at the MeNB. | YES | reject |
| SIPTO Bearer Deactivation Indication | O |  | ENUMERATED (True, …) | Indicates that SIPTO@LN PDN connection deactivation is needed. | YES | ignore |
| SgNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated for EN-DC at the SgNB. | YES | ignore |

#### 9.1.1.6	HANDOVER CANCEL

This message is sent by the source eNB to the target eNB to cancel an ongoing handover.

Direction: source eNB  target eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | reject |
| New eNB UE X2AP ID | O |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | reject |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | YES | ignore |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | ECGI 9.2.14 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional handover. Value is 8. |

#### 9.1.1.7	HANDOVER SUCCESS

This message is sent by the target eNB to the source eNB to indicate the successful access of the UE toward the target eNB.

Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | reject |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | reject |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | YES | ignore |
| Target Cell ID | M |  | ECGI 9.2.14 | Target cell indicated in the corresponding Handover Preparation procedure | YES | reject |

#### 9.1.1.8	CONDITIONAL HANDOVER CANCEL

This message is sent by the target eNB to the source eNB to cancel an ongoing conditional handover.

Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | YES | reject |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | ECGI 9.2.14 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional handover. Value is 8. |

#### 9.1.1.9	EARLY STATUS TRANSFER

This message is sent by the source eNB to the target eNB to transfer the COUNT value related to the forwarded downlink SDUs during DAPS Handover or Conditional Handover.

During a Conditional Handover with EN-DC or Dual Connectivity, or CPAC in EN-DC, this message is also used to transfer the COUNT value related to the forwarded downlink SDUs. In case of EN-DC, the COUNT value is transferred from the en-gNB to the eNB, while in case of Dual Connectivity, the COUNT value is transferred from the SeNB to the MeNB. In case of Conditional PSCell Addition, the COUNT value is transferred from the eNB to the en-gNB. In case of Conditional PSCell Change, the COUNT value is transferred from the source en-gNB to the eNB, and from eNB to the target en-gNB.

Direction: source eNB  target eNB (DAPS Handover or Conditional Handover).

Direction: en-gNB  MeNB (Conditional Handover with EN-DC), SeNB  MeNB (Conditional Handover with Dual Connectivity)

Direction: eNB  en-gNB (Conditional PSCell Addition)

Direction: source en-gNB  eNB, eNB  target en-gNB (Conditional PSCell Change)

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated for DAPS handover or Conditional handover at the source eNB | YES | reject |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated for DAPS handover or Conditional handover at the target eNB | YES | reject |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated for DAPS handover or Conditional handover at the source eNB | YES | reject |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated for DAPS handover or Conditional handover at the target eNB | YES | reject |
| CHOICE Procedure Stage | M |  |  |  | YES | reject |
| >First DL COUNT |  |  |  |  |  |  |
| >>E-RABs Subject To Early Status Transfer List |  | 1 .. <maxnoofBearers> |  |  | – |  |
| >>>E-RABs Subject To Early Status Transfer Item |  |  |  |  | – |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>FIRST DL COUNT Value | M |  | COUNT Value 9.2.15 | PDCP-SN and Hyper frame number of the first DL SDU that the source eNB/SeNB/en-gNB forwards to the target eNB/MeNB in case of 12 bit long PDCP-SN | – |  |
| >>>>FIRST DL COUNT Value Extended | O |  | COUNT Value Extended 9.2.66 | PDCP-SN and Hyper frame number of the first DL SDU that the source eNB/SeNB/en-gNB forwards to the target eNB/MeNB in case of 15 bit long PDCP-SN | – |  |
| >>>>FIRST DL COUNT Value for PDCP SN Length 18 | O |  | COUNT Value for PDCP SN Length 18 9.2.82 | PDCP-SN and Hyper frame number of the first DL SDU that the source eNB/SeNB/en-gNB forwards to the target eNB/MeNB in case of 18 bit long PDCP-SN | – |  |
| >DL Discarding |  |  |  |  |  |  |
| >>E-RABs Subject To DL Discarding List | M | 1 |  |  | – |  |
| >>>E-RABs Subject To DL Discarding Item |  | 1 .. <maxnoofBearers> |  |  | – |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>DISCARD DL COUNT Value | M |  | COUNT Value 9.2.15 | PDCP-SN and Hyper frame number for which the target eNB/MeNB should discard forwarded DL SDUs associated with lower values in case of 12 bit long PDCP-SN | – |  |
| >>>>DISCARD DL COUNT Value Extended | O |  | COUNT Value Extended 9.2.66 | PDCP-SN and Hyper frame number for which the target eNB/MeNB should discard forwarded DL SDUs associated with lower values in case of 15 bit long PDCP-SN | – |  |
| >>>>DISCARD DL COUNT Value for PDCP SN Length 18 | O |  | COUNT Value for PDCP SN Length 18 9.2.82 | PDCP-SN and Hyper frame number for which the target eNB/MeNB should discard forwarded DL SDUs associated with lower values in case of 18 bit long PDCP-SN | – |  |
| SgNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated for EN-DC at the en-gNB. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256. |

### 9.1.2	Messages for global procedures

#### 9.1.2.1	LOAD INFORMATION

This message is sent by an eNB to neighbouring eNBs to transfer load and interference co-ordination information.

Direction: eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Cell Information | M |  |  |  | YES | ignore |
| >Cell Information Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 | Id of the source cell | – |  |
| >>UL Interference Overload Indication | O |  | 9.2.17 |  | – |  |
| >>UL High Interference Information |  | 0 .. <maxCellineNB> |  |  | – |  |
| >>>Target Cell ID | M |  | ECGI 9.2.14 | Id of the cell for which the HII is meant | – |  |
| >>>UL High Interference Indication | M |  | 9.2.18 |  | – |  |
| >>Relative Narrowband Tx Power (RNTP) | O |  | 9.2.19 |  | – |  |
| >>ABS Information | O |  | 9.2.54 |  | YES | ignore |
| >>Invoke Indication | O |  | 9.2.55 |  | YES | ignore |
| >>Intended UL-DL Configuration | O |  | ENUMERATED(sa0, sa1, sa2, sa3, sa4, sa5, sa6,…) | One of the UL-DL configurations defined in TS 36.211 [10]. The UL subframe(s) in the indicated configuration is subset of those in SIB1 UL-DL configuration. This IE applies to TDD only. | YES | ignore |
| >>Extended UL Interference Overload Info | O |  | 9.2.67 | This IE applies to TDD only. | YES | ignore |
| >>CoMP Information | O |  | 9.2.74 |  | YES | ignore |
| >>Dynamic DL transmission information | O |  | 9.2.77 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

#### 9.1.2.2	ERROR INDICATION

This message is used to indicate that some error has been detected in the eNB/en-gNB.

Direction: eNB1  eNB2 or eNB  en-gNB or en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | O |  | eNB UE X2AP ID 9.2.24 | Allocated for handover at the source eNB and for dual connectivity at the SeNB or for a SN Status Transfer procedure at the eNB from which the E-RAB context is transferred. | YES | ignore |
| New eNB UE X2AP ID | O |  | eNB UE X2AP ID 9.2.24 | Allocated for handover at the target eNB and for dual connectivity/EN-DC at the MeNB or for a SN Status Transfer procedure at the eNB to which the E-RAB context is transferred. | YES | ignore |
| Cause | O |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated for handover at the source eNB and for dual connectivity at the SeNB or for a SN Status Transfer procedure at the eNB from which the E-RAB context is transferred. | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated for handover at the target eNB and for dual connectivity/EN-DC at the MeNB or for a SN Status Transfer procedure at the eNB to which the E-RAB context is transferred. | YES | ignore |
| Old en-gNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated for EN-DC at the en-gNB. | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

#### 9.1.2.3	X2 SETUP REQUEST

This message is sent by an eNB to a neighbouring eNB to transfer the initialization information for a TNL association.

Direction: eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Global eNB ID | M |  | 9.2.22 |  | YES | reject |
| Served Cells |  | 1 .. <maxCellineNB> |  | Complete list of cells served by the eNB | YES | reject |
| >Served Cell Information | M |  | 9.2.8 |  | – |  |
| >Neighbour Information |  | 0 .. <maxnoofNeighbours> |  |  | – |  |
| >>ECGI | M |  | ECGI 9.2.14 | E-UTRAN Cell Global Identifier of the neighbour cell | – |  |
| >>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the neighbour cell | – |  |
| >>EARFCN | M |  | 9.2.26 | DL EARFCN for FDD or EARFCN for TDD | – |  |
| >>TAC | O |  | OCTET STRING (2) | Tracking Area Code | YES | ignore |
| >>EARFCN Extension | O |  | 9.2.65 | DL EARFCN for FDD or EARFCN for TDD. If this IE is present, the value signalled in the EARFCN IE is ignored. | YES | reject |
| >NR Neighbour Information | O |  | 9.2.98 | NR neighbour, capable of performing EN-DC with the served E-UTRA cell | YES | ignore |
| >Served Cell Specific Info Request | O |  | 9.2.180 |  | YES | ignore |
| GU Group Id List |  | 0 .. <maxfPools> |  | List of all the pools to which the eNB belongs | GLOBAL | reject |
| >GU Group Id | M |  | 9.2.20 |  | - |  |
| LHN ID | O |  | 9.2.83 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofNeighbours | Maximum no. of neighbour cells associated to a given served cell. Value is 512. |
| maxPools | Maximum no. of pools an eNB can belong to. Value is 16. |

#### 9.1.2.4	X2 SETUP RESPONSE

This message is sent by an eNB to a neighbouring eNB to transfer the initialization information for a TNL association.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Global eNB ID | M |  | 9.2.22 |  | YES | reject |
| Served Cells |  | 1 .. <maxCellineNB> |  | Complete list of cells served by the eNB | GLOBAL | reject |
| >Served Cell Information | M |  | 9.2.8 |  | – |  |
| >Neighbour Information |  | 0 .. <maxnoofNeighbours> |  |  | – |  |
| >>ECGI | M |  | ECGI 9.2.14 | E-UTRAN Cell Global Identifier of the neighbour cell | – |  |
| >>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the neighbour cell | – |  |
| >>EARFCN | M |  | 9.2.26 | DL EARFCN for FDD or EARFCN for TDD | – |  |
| >>TAC | O |  | OCTET STRING (2) | Tracking Area Code | YES | ignore |
| >>EARFCN Extension | O |  | 9.2.65 | DL EARFCN for FDD or EARFCN for TDD. If this IE is present, the value signalled in the EARFCN IE is ignored. | YES | reject |
| >NR Neighbour Information | O |  | 9.2.98 | NR neighbour, capable of performing EN-DC with the served E-UTRA cell | YES | ignore |
| GU Group Id List |  | 0 .. <maxPools> |  | List of all the pools to which the eNB belongs | GLOBAL | reject |
| >GU Group Id | M |  | 9.2.20 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| LHN ID | O |  | 9.2.83 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofNeighbours | Maximum no. of neighbour cells associated to a given served cell. Value is 512. |
| maxPools | Maximum no. of pools an eNB can belong to. Value is 16. |

#### 9.1.2.5	X2 SETUP FAILURE

This message is sent by the eNB to indicate X2 Setup failure.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Time To Wait | O |  | 9.2.32 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

#### 9.1.2.6	RESET REQUEST

This message is sent from one eNB to another eNB/en-gNB or from en-gNB to an eNB and is used to request the X2 interface between the two eNB or between an eNB and an en-gNB to be reset.

Direction: eNB1  eNB2, eNB  en-gNB, en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

#### 9.1.2.7	RESET RESPONSE

This message is sent by a eNB/en-gNB as a response to a RESET REQUEST message.

Direction: eNB2  eNB1, eNB  en-gNB, en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

#### 9.1.2.8	ENB CONFIGURATION UPDATE

This message is sent by an eNB to a peer eNB to transfer updated information for a TNL association.

Direction: eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Served Cells To Add |  | 0 .. <maxCellineNB> |  | Complete list of added cells served by the eNB | GLOBAL | reject |
| >Served Cell Information | M |  | 9.2.8 |  | – |  |
| >Neighbour Information |  | 0 .. <maxnoofNeighbours> |  |  | – |  |
| >>ECGI | M |  | ECGI 9.2.14 | E-UTRAN Cell Global Identifier of the neighbour cell | – |  |
| >>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the neighbour cell | – |  |
| >>EARFCN | M |  | 9.2.26 | DL EARFCN for FDD or EARFCN for TDD | – |  |
| >>TAC | O |  | OCTET STRING (2) | Tracking Area Code | YES | ignore |
| >>EARFCN Extension | O |  | 9.2.65 | DL EARFCN for FDD or EARFCN for TDD. If this IE is present, the value signalled in the EARFCN IE is ignored. | YES | reject |
| >NR Neighbour Information | O |  | 9.2.98 | NR neighbour, capable of performing EN-DC with the served E-UTRA cell | YES | ignore |
| Served Cells To Modify |  | 0 .. <maxCellineNB> |  | Complete list of modified cells served by the eNB | GLOBAL | reject |
| >Old ECGI | M |  | ECGI 9.2.14 | Old E-UTRAN Cell Global Identifier | - |  |
| >Served Cell Information | M |  | 9.2.8 |  | – |  |
| >Neighbour Information |  | 0 .. <maxnoofNeighbours> |  |  | – |  |
| >>ECGI | M |  | ECGI 9.2.14 | E-UTRAN Cell Global Identifier of the neighbour cell | – |  |
| >>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the neighbour cell | – |  |
| >>EARFCN | M |  | 9.2.26 | DL EARFCN for FDD or EARFCN for TDD | – |  |
| >>TAC | O |  | OCTET STRING (2) | Tracking Area Code | YES | ignore |
| >>EARFCN Extension | O |  | 9.2.65 | DL EARFCN for FDD or EARFCN for TDD. If this IE is present, the value signalled in the EARFCN IE is ignored. | YES | reject |
| >NR Neighbour Information | O |  | 9.2.98 | NR neighbour, capable of performing EN-DC with the served E-UTRA cell | YES | ignore |
| >Deactivation Indication | O |  | ENUMERATED(deactivated, …) | Indicates that the concerned cell is switched off for energy saving reasons | YES | ignore |
| Served Cells To Delete |  | 0 .. <maxCellineNB> |  | Complete list of deleted cells served by the eNB | GLOBAL | reject |
| >Old ECGI | M |  | ECGI 9.2.14 | Old E-UTRAN Cell Global Identifier of the cell to be deleted | - |  |
| GU Group Id To Add List |  | 0 .. <maxPools> |  |  | GLOBAL | reject |
| >GU Group Id | M |  | 9.2.20 |  | - |  |
| GU Group Id To Delete List |  | 0 .. <maxPools> |  |  | GLOBAL | reject |
| >GU Group Id | M |  | 9.2.20 |  | - |  |
| Coverage Modification List |  | 0 .. <maxCellineNB> |  | List of cells with modified coverage | GLOBAL | reject |
| >ECGI | M |  | ECGI 9.2.14 | E-UTRAN Cell Global Identifier of the cell to be modified | - |  |
| >Cell Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell | - |  |
| >Cell Deployment Status Indicator | O |  | ENUMERATED(pre-change-notification, ...) | Indicates the Cell Coverage State is planned to be used at the next reconfiguration |  |  |
| >Cell Replacing Info | C-ifCellDeploymentStatusIndicatorPresent |  |  |  |  |  |
| >>Replacing Cells |  | 0 .. <maxCellineNB> |  |  |  |  |
| >>>ECGI |  |  | ECGI 9.2.14 | E-UTRAN Cell Global Identifier of a cell that may replace all or part of the coverage of the cell to be modified |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofNeighbours | Maximum no. of neighbour cells associated to a given served cell. Value is 512. |
| maxPools | Maximum no. of pools an eNB can belong to. Value is 16. |

| Condition | Explanation |
| --- | --- |
| ifCellDeploymentStatusIndicatorPresent | This IE shall be present if the Cell Deployment Status Indicator IE is present. |

#### 9.1.2.9	ENB CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by an eNB to a peer eNB to acknowledge update of information for a TNL association.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

#### 9.1.2.10	ENB CONFIGURATION UPDATE FAILURE

This message is sent by an eNB to a peer eNB to indicate eNB Configuration Update Failure.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Time To Wait | O |  | 9.2.32 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

#### 9.1.2.11	RESOURCE STATUS REQUEST

This message is sent by an eNB1 to neighbouring eNB2 to initiate the requested measurement according to the parameters given in the message.

Direction: eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| eNB1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | C-ifRegistrationRequestStoporPartialStoporAdd |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …, partial stop, add) | Type of request for which the resource status is required. | YES | reject |
| Report Characteristics | O |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the eNB2 is requested to report. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic, this bit should be set to 1 if at least one of the First, Second or Third bits is set to 1, Fifth Bit = ABS Status Periodic, Sixth Bit = RSRP Measurement Report Periodic, Seventh Bit = CSI Report Periodic, Eighth Bit = CAC for Possibly Aggregated Cells Periodic. Other bits shall be ignored by the eNB2. | YES | reject |
| Cell To Report |  | 1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of PRB Periodic, TNL Load Ind Periodic, HW Load Ind Periodic, Composite Available Capacity Periodic or ABS Status Periodic. | YES | ignore |
| Partial Success Indicator | O |  | ENUMERATED(partial success allowed, ...) | Included if partial success is allowed | YES | ignore |
| Reporting Periodicity of RSRP Measurement Report | O |  | ENUMERATED(120ms, 240ms, 480ms, 640ms, ...) | Periodicity that can be used for the reporting of RSRP Measurement Report Periodic. | YES | ignore |
| Reporting Periodicity of CSI Report | O |  | ENUMERATED(5ms, 10ms, 20ms, 40ms, 80ms, ...) | Periodicity that can be used for the reporting of CSI Report Periodic. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporPartialStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop", "partial stop" or "add". |

#### 9.1.2.12	RESOURCE STATUS RESPONSE

This message is sent by the eNB2 to indicate that the requested measurement, for all or for a subset of the measurement objects included in the measurement is successfully initiated.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| eNB1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Measurement Initiation Result |  | 0..1 |  | List of all cells in which measurement objects were requested, included when indicating partial success | YES | ignore |
| >Measurement Initiation Result Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 |  | – |  |
| >>Measurement Failure Cause List |  | 0..1 |  | Indicates that eNB2 could not initiate the measurement for at least one of the requested measurement objects in the cell | – |  |
| >>>Measurement Failure Cause Item |  | 1 .. <maxFailedMeasObjects> |  |  | EACH | ignore |
| >>>>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object that failed to be initiated in the eNB2. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic, Fifth Bit = ABS Status Periodic, Sixth Bit = RSRP Measurement Report Periodic, Seventh Bit = CSI Report Periodic, Eighth Bit = CAC for Possibly Aggregated Cells Periodic. Other bits shall be ignored by the eNB1. | – |  |
| >>>>Cause | M |  | 9.2.6 | Failure cause for measurement objects for which the measurement cannot be initiated | – |  |

| Range bound | Explanation |
| --- | --- |
| maxFailedMeasObjects | Maximum number of measurement objects that can fail per measurement. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

#### 9.1.2.13	RESOURCE STATUS FAILURE

This message is sent by the eNB2 to indicate that for none of the requested measurement objects the measurement can be initiated.

Direction: eNB2  eNB1.

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.2.13 |  | YES | reject |
| eNB1 Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | reject |
| Cause | M |  |  | 9.2.6 | Ignored by the receiver when the Complete Failure Cause Information IE is included | YES | ignore |
| Criticality Diagnostics |  | O |  | 9.2.7 |  | YES | ignore |
| Complete Failure Cause Information |  |  | 0..1 |  | Complete list of failure causes for all requested cells | YES | ignore |
| >Complete Failure Cause Information Item |  |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  |  | ECGI 9.2.14 |  | – |  |
| >>Measurement Failure Cause List |  |  | 1 |  |  | – |  |
| >>>Measurement Failure Cause Item |  |  | 1 .. <maxFailedMeasObjects> |  |  | EACH | ignore |
| >>>>Measurement Failed Report Characteristics | M |  |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object that failed to be initiated in the eNB2. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic, Fifth Bit = ABS Status Periodic, Sixth Bit = RSRP Measurement Report Periodic, Seventh Bit = CSI Report Periodic, Eighth Bit = CAC for Possibly Aggregated Cells Periodic. Other bits shall be ignored by the eNB1. | – |  |
| >>>>Cause | M |  |  | 9.2.6 | Failure cause for measurements that cannot be initiated | – |  |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxFailedMeasObjects | Max number of measurement objects that can fail per measurement. Value is 32. |

#### 9.1.2.14	RESOURCE STATUS UPDATE

This message is sent by eNB2 to neighbouring eNB1 to report the results of the requested measurements.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| eNB1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 |  | – |  |
| >>Hardware Load Indicator | O |  | 9.2.34 |  | – |  |
| >>S1 TNL Load Indicator | O |  | 9.2.35 |  | – |  |
| >>Radio Resource Status | O |  | 9.2.37 |  | – |  |
| >>Composite Available Capacity Group | O |  | 9.2.44 |  | YES | ignore |
| >>ABS Status | O |  | 9.2.58 |  | YES | ignore |
| >>RSRP Measurement Report List | O |  | 9.2.76 |  | YES | ignore |
| >>CSI Report | O |  | 9.2.79 |  | YES | ignore |
| >>Cell Reporting Indicator | O |  | ENUMERATED(stop request, ...) |  | YES | ignore |
| >>Measurement Result for NR Cells Possibly Aggregated |  | 0..1 |  |  | YES | ignore |
| >>>Measurement Result for NR Cells Possibly Aggregated Item |  | 1 .. < maxnoofReportedNRCellsPossiblyAggregated > |  |  | – |  |
| >>>>Cell ID | M |  | NR CGI 9.2.111 |  | – |  |
| >>>>NR Composite Available Capacity Group | O |  | 9.2.163 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofReportedNRCellsPossiblyAggregated | Maximum no. of reported NR cells that may be aggregated with the serving cell. Value is 16. |

#### 9.1.2.15	MOBILITY CHANGE REQUEST

This message is sent by an eNB1 to neighbouring eNB2 to initiate adaptation of mobility parameters.

Direction: eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| eNB1 Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| eNB2 Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| eNB1 Mobility Parameters | O |  | Mobility Parameters Information 9.2.48 | Configuration change in eNB1 cell | YES | ignore |
| eNB2 Proposed Mobility Parameters | M |  | Mobility Parameters Information 9.2.48 | Proposed configuration change in eNB2 cell | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | reject |

#### 9.1.2.16	MOBILITY CHANGE ACKNOWLEDGE

This message is sent by the eNB2 to indicate that the eNB2 Proposed Mobility Parameter proposed by eNB1 was accepted.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| eNB1 Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| eNB2 Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

#### 9.1.2.17	MOBILITY CHANGE FAILURE

This message is sent by the eNB2 to indicate that the eNB2 Proposed Mobility Parameter proposed by eNB1 was refused.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| eNB1 Cell ID | M |  | ECGI 9.2.14 |  | YES | ignore |
| eNB2 Cell ID | M |  | ECGI 9.2.14 |  | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Mobility Parameters Modification Range | O |  | 9.2.49 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

#### 9.1.2.18	RLF INDICATION

This message is sent by the eNB2 to indicate an RRC re-establishment attempt or a reception of an RLF Report from a UE that suffered a connection failure at eNB1.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Failure cell PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier | YES | ignore |
| Re-establishment cell ECGI | M |  | ECGI 9.2.14 |  | YES | ignore |
| C-RNTI | M |  | BIT STRING (SIZE (16)) | Corresponds to information provided in the c-RNTI contained in the RRCConnectionReestablishmentRequest message (TS 36.331 [9]) | YES | ignore |
| ShortMAC-I | O |  | BIT STRING (SIZE (16)) | Corresponds to information provided in the shortMAC-I contained in the RRCConnectionReestablishmentRequest message (TS 36.331 [9]) | YES | ignore |
| UE RLF Report Container | O |  | OCTET STRING | Corresponds to information provided in the rLF-Report-r9 contained in the UEInformationResponse message (TS 36.331 [9]) | YES | ignore |
| RRC Conn Setup Indicator | O |  | ENUMERATED(RRC Conn Setup, ...) | Included if the RLF Report within the UE RLF Report Container IE is retrieved after an RRC connection setup or an incoming successful handover | YES | reject |
| RRC Conn Reestab Indicator | O |  | ENUMERATED(reconfigurationFailure, handoverFailure, otherFailure, ...) | Corresponds to information provided in the reestablishmentCause in the RRCConnectionReestablishmentRequest message (TS 36.331 [9]) | YES | ignore |
| UE RLF Report Container for extended bands | O |  | OCTET STRING | Includes the rlf-Report-v9e0 contained in the UEInformationResponse message (TS 36.331 [9]) | YES | ignore |
| NB-IoT RLF Report Container | O |  | OCTET STRING | Includes the rlf-Report-NB IE contained in the UEInformationResponse-NB message (TS 36.331 [9]) | YES | ignore |

#### 9.1.2.19	HANDOVER REPORT

This message is sent by the eNB1 to report a handover failure event or other critical mobility problem.

Direction: eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, …, InterRAT ping-pong, Inter System ping-pong) |  | YES | ignore |
| Handover Cause | M |  | Cause 9.2.6 | Indicates handover cause employed for handover from eNB2 | YES | ignore |
| Source cell ECGI | M |  | ECGI 9.2.14 | ECGI of source cell for handover procedure (in eNB2) | YES | ignore |
| Failure cell ECGI | M |  | ECGI 9.2.14 | ECGI of target cell for handover procedure (in eNB1) | YES | ignore |
| Re-establishment cell ECGI | C- ifHandoverReportType HoToWrongCell |  | ECGI 9.2.14 | ECGI of cell where UE attempted re-establishment | YES | ignore |
| Target cell in UTRAN | C- ifHandoverReportType InterRATpingpong |  | OCTET STRING | Encoded according to UTRAN Cell ID in the Last Visited UTRAN Cell Information IE, as defined in in TS 25.413 [24] | YES | ignore |
| Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | Includes the sourceUE-Identity (a C-RNTI) allocated at the source eNB (in eNB2) contained in the AS-Config IE as specified in TS 36.331 [9]. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message from eNB2. | YES | ignore |
| UE RLF Report Container | O |  | OCTET STRING | Includes the UE RLF Report Container IE received in the RLF INDICATION message. | YES | ignore |
| UE RLF Report Container for extended bands | O |  | OCTET STRING | Includes the UE RLF Report Container for extended bands IE received in the RLF INDICATION message. | YES | ignore |
| Target cell in NG-RAN | C- ifHandoverReportType Inter-system pingpong |  | OCTET STRING | Encoded according to NG-RAN CGI IE in TS 38.413 [39]. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifHandoverReportType HoToWrongCell | This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell" |
| ifHandoverReportType InterRATpingpong | This IE shall be present if the Handover Report Type IE is set to the value "InterRAT ping-pong" |
| ifHandoverReportType Inter-system pingpong | This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong" |

#### 9.1.2.20	CELL ACTIVATION REQUEST

This message is sent by an eNB to a peer eNB to request a previously switched-off cell(s) to be re-activated.

Direction: eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Served Cells To Activate |  | 1 .. <maxCellineNB> |  |  | GLOBAL | reject |
| >ECGI | M |  | 9.2.14 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

#### 9.1.2.21	CELL ACTIVATION RESPONSE

This message is sent by an eNB to a peer eNB to indicate that one or more cell(s) previously switched-off has(have) been activated.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Activated Cell List |  | 1 .. <maxCellineNB> |  |  | GLOBAL | ignore |
| >ECGI | M |  | 9.2.14 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

#### 9.1.2.22	CELL ACTIVATION FAILURE

This message is sent by an eNB to a peer eNB to indicate cell activation failure.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

#### 9.1.2.23	X2 RELEASE

This message is used to indicate that the signalling connection to an eNB is unavailable.

Direction: eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Global eNB ID | M |  | 9.2.22 |  | YES | reject |

#### 9.1.2.24	X2AP MESSAGE TRANSFER

This message is used for indirect transport of an X2AP message (except the X2AP MESSAGE TRANSFER message) between two eNBs, and to allow an eNB to perform registration.

Direction: eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| RNL Header | M |  | 9.2.68 |  | YES | reject |
| X2AP Message | O |  | OCTET STRING | Includes any X2AP message except the X2AP MESSAGE TRANSFER message | YES | reject |

#### 9.1.2.25	X2 REMOVAL REQUEST

This message is sent by an eNB to a neighbouring eNB to initiate the removal of the signaling connection.

Direction: eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Global eNB ID | M |  | 9.2.22 |  | YES | reject |
| X2 Removal Threshold | O |  | X2 Benefit Value 9.2.90 |  | YES | reject |

#### 9.1.2.26	X2 REMOVAL RESPONSE

This message is sent by an eNB to a neighbouring eNB to acknowledge the initiation of removal of the signaling connection.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Global eNB ID | M |  | 9.2.22 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

#### 9.1.2.27	X2 REMOVAL FAILURE

This message is sent by the eNB to indicate that removing the signaling connection cannot be accepted.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

#### 9.1.2.28	RETRIEVE UE CONTEXT REQUEST

This message is sent by the new eNB to request the old eNB to transfer the UE Context to the new eNB.

Direction: new eNB  old eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the new eNB | YES | reject |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the new eNB See Note 1) | YES | reject |
| Resume ID | M |  | 9.2.91 |  | YES | reject |
| ShortMAC-I | M |  | BIT STRING (SIZE (16)) | RRC Resume: Corresponds to information provided in the shortResumeMAC-I in the RRCConnection ResumeRequest message as defined in TS 36.331 [9] RRC Reestablishment: Corresponds to information provided in the shortMAC-I in the RRCConnectionReestablishmentRequestmessage as defined in TS 36.331 [9]. | YES | reject |
| New E-UTRAN Cell Identifier | M |  | BIT STRING (SIZE (28)) | RRC Resume: Corresponds to information provided in the cellIdentity within the VarShortResumeMAC-Input as specified in TS 36.331 [9]. RRC Reestablishment: Corresponds to information provided in the cellIdentity within the VarShortMAC-Input as specified in TS 36.331 [9]. | YES | reject |
| C-RNTI | O |  | BIT STRING (SIZE (16)) | Corresponds to information provided in the c-RNTI contained in the RRCReestablishmentRequest message as specified in TS 36.331 [9]. If this IE is present, the Resume ID IE is ignored | YES | reject |
| Failure cell PCI | O |  | INTEGER (0..503, …) | Physical Cell Identifier | YES | reject |

NOTE 1:	The ASN.1 definition of the RETRIEVE UE CONTEXT REQUEST message contains the a wrong IE-Id, which references the SeNB UE X2AP ID Extension IE instead of the New eNB UE X2AP ID Extension IE. The old eNB interprets the content of this IE as the Extended eNB UE X2AP ID, which, together with the New eNB UE X2AP ID IE represents the eNB UE X2AP ID allocated at the new eNB.

#### 9.1.2.29	RETRIEVE UE CONTEXT RESPONSE

This message is sent by the old eNB to transfer the UE context to the new eNB.

Direction: old eNB  new eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the new eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the new eNB | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the old eNB | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the old eNB | YES | ignore |
| GUMMEI | M |  | 9.2.16 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >MME UE S1AP ID | M |  | INTEGER (0..232 -1) | MME UE S1AP ID allocated at the MME | – |  |
| >UE Security Capabilities | M |  | 9.2.29 |  | – |  |
| >AS Security Information | M |  | 9.2.30 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.12 |  | – |  |
| >Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.25 |  | – |  |
| >E-RABs To Be Setup List |  | 1 |  |  | – |  |
| >>E-RABs To Be Setup Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – |  |
| >>>Bearer Type | O |  | 9.2.92 |  | – |  |
| >>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | YES | reject |
| >>>DL Forwarding | O |  | 9.2.5 |  | YES | ignore |
| >>>Ethernet Type | O |  | 9.2.157 |  | YES | ignore |
| >>>Security Indication | O |  | 9.2.181 |  | YES | reject |
| >RRC Context | M |  | OCTET STRING | Includes either the HandoverPreparation nformation message as defined in subclause 10.2.2 of TS 36.331 [9], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [9]. | – |  |
| >Handover Restriction List | O |  | 9.2.3 |  | – |  |
| >Location Reporting Information | O |  | 9.2.21 | Includes the necessary parameters for location reporting | – |  |
| >Management Based MDT Allowed | O |  | 9.2.59 |  | – |  |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.64 |  | – |  |
| >UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.97 | This IE applies only if the UE is authorized for V2X services. | YES | ignore |
| >Additional RRM Policy Index | O |  | 9.2.25a |  | YES | ignore |
| >EPC Handover Restriction List Container | O |  | 9.2.153 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.159 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.171 |  | YES | reject |
| >IMS voice EPS fallback from 5G | O |  | ENUMERATED (true, …) |  | YES | ignore |
| Trace Activation | O |  | 9.2.2 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.33 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.69 |  | YES | ignore |
| Expected UE Behaviour | O |  | 9.2.70 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.78 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| V2X Services Authorized | O |  | 9.2.93 |  | YES | ignore |
| Aerial UE subscription information | O |  | 9.2.129 |  | YES | ignore |
| Subscription Based UE Differentiation Information | O |  | 9.2.136 |  | YES | ignore |
| NR V2X Services Authorized | O |  | 9.2.158 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.160 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

#### 9.1.2.30	RETRIEVE UE CONTEXT FAILURE

This message is sent by the old eNB to inform the new eNB that the Retrieve UE Context procedure has failed.

Direction: old eNB  new eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the new eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the new eNB | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

#### 9.1.2.31	EN-DC X2 SETUP REQUEST

This message is sent by an initiating node to a neighbouring node, both nodes able to interact for EN-DC, to transfer the initialization information for a TNL association.

Direction: eNB  en-gNB, en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| CHOICE Initiating NodeType | M |  |  |  | YES | reject |
| >eNB |  |  |  |  |  |  |
| >>Global eNB ID | M |  | 9.2.22 |  | YES | reject |
| >>List of Served E-UTRA Cells |  | 1 .. <maxCellineNB> |  | Complete list of cells served by the eNB | YES | reject |
| >>>Served E-UTRA Cell Information | M |  | Served Cell Information 9.2.8 |  | – |  |
| >>>NR Neighbour Information | O |  | 9.2.98 | NR neighbours | – |  |
| >>Interface Instance Indication | O |  | 9.2.143 | NOTE: In the current version of this specification this IE is not included in the Initiating Node Type IE. | YES | reject |
| >>Cell and Capacity Assistance Information | O |  | 9.2.146 |  | YES | ignore |
| >en-gNB |  |  |  |  |  |  |
| >>Global en-gNB ID | M |  | 9.2.112 |  | YES | reject |
| >>List of Served NR Cells |  | 1 .. <maxCellinengNB> |  | List of cells served by the en-gNB. If a partial list of cells is signalled, it contains at least one cell per carrier configured at the gNB. | YES | reject |
| >>>Served NR Cell Information | M |  | 9.2.110 |  | – |  |
| >>>NR Neighbour Information | O |  | 9.2.98 | NR neighbours. | – |  |
| >>>Served Cell Specific Info Request | O |  | 9.2.180 |  | YES | ignore |
| >>Partial List Indicator | O |  | ENUMERATED (partial, ...) | Value "partial" indicates that a partial list of cells is included in the List of Served NR Cells IE | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |
| TNL Transport Layer Address info | O |  | 9.2.149 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxCellinengNB | Maximum no. cells that can be served by an en-gNB. Value is 16384. |

#### 9.1.2.32	EN-DC X2 SETUP RESPONSE

This message is sent by a neighbouring node to an initiating node, both nodes able to interact for EN-DC, to transfer the initialization information for a TNL association.

Direction: eNB  en-gNB, en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| CHOICE Responding NodeType | M |  |  |  | YES | reject |
| >eNB |  |  |  |  |  |  |
| >>Global eNB ID | M |  | 9.2.22 |  | YES | reject |
| >>List of Served E-UTRA Cells |  | 1 .. <maxCellineNB> |  | Complete list of cells served by the eNB | YES | reject |
| >>>Served E-UTRA Cell Information | M |  | Served Cell Information 9.2.8 |  | – |  |
| >>>NR Neighbour Information | O |  | 9.2.98 | NR neighbours | – |  |
| >>Cell and Capacity Assistance Information | O |  | 9.2.146 |  | YES | ignore |
| >en-gNB |  |  |  |  |  |  |
| >>Global en-gNB ID | M |  | 9.2.112 |  | YES | reject |
| >>List of Served NR Cells |  | 1 .. <maxCellinengNB> |  | List of cells served by the en-gNB. If a partial list of cells is signalled, it contains at least one cell per carrier configured at the gNB. | YES | reject |
| >>>Served NR Cell Information | M |  | 9.2.110 |  | – |  |
| >>>NR Neighbour Information | O |  | 9.2.98 | NR neighbours | – |  |
| >>Partial List Indicator | O |  | ENUMERATED (partial, ...) | Value "partial" indicates that a partial list of cells is included in the List of Served NR Cells IE | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |
| TNL Transport Layer Address info | O |  | 9.2.149 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxCellinengNB | Maximum no. cells that can be served by an en-gNB. Value is 16384. |

#### 9.1.2.33	EN-DC X2 SETUP FAILURE

This message is sent by the neighbouring node to indicate EN-DC X2 Setup failure.

Direction: eNB  en-gNB, en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Time To Wait | O |  | 9.2.32 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |
| Message Oversize Notification | O |  | 9.2.148 |  | YES | ignore |

#### 9.1.2.34	EN-DC CONFIGURATION UPDATE

This message is sent by an initiating node to a peer neighbouring node, both nodes able to interact for EN-DC, to transfer updated information for a TNL association.

Direction: eNB  en-gNB, en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| CHOICE Initiating NodeType | M |  |  |  | YES | reject |
| >eNB |  |  |  |  |  |  |
| >>Cell Assistance Information | O |  | 9.2.115 |  | YES | reject |
| >>Served E-UTRA Cells To Add |  | 0 .. <maxCellineNB> |  |  | GLOBAL | reject |
| >>>Served E-UTRA Cell Information | M |  | Served Cell Information 9.2.8 |  | – |  |
| >>>NR Neighbour Information | O |  | 9.2.98 | NR neighbours | – |  |
| >>Served E-UTRA Cells To Modify |  | 0 .. <maxCellineNB> |  | Complete list of modified cells served by the eNB | GLOBAL | reject |
| >>>Old ECGI | M |  | ECGI 9.2.14 | Old E-UTRAN Cell Global Identifier | – |  |
| >>>Served E-UTRA Cell Information | M |  | Served Cell Information 9.2.8 |  | – |  |
| >>>NR Neighbour Information | O |  | 9.2.98 | NR neighbours | – |  |
| >>Served E-UTRA Cells To Delete |  | 0 .. <maxCellineNB> |  | Complete list of deleted cells served by the eNB | GLOBAL | reject |
| >>>Old ECGI | M |  | ECGI 9.2.14 | Old E-UTRAN Cell Global Identifier of the cell to be deleted | - |  |
| >en-gNB |  |  |  |  |  |  |
| >>Served NR Cells To Add |  | 0 .. <maxCellinengNB> |  |  | GLOBAL | reject |
| >>>Served NR Cell Information | M |  | 9.2.110 |  | – |  |
| >>>NR Neighbour Information | O |  | 9.2.98 | NR neighbours | – |  |
| >>>Served Cell Specific Info Request | O |  | 9.2.180 |  | YES | ignore |
| >>Served NR Cells To Modify |  | 0 .. <maxCellinengNB> |  |  | GLOBAL | reject |
| >>>Old NR-CGI | M |  | NR CGI 9.2.111 |  | - |  |
| >>>Served NR Cell Information | M |  | 9.2.110 |  | – |  |
| >>>NR Neighbour Information | O |  | 9.2.98 | NR neighbours | – |  |
| >>>NR Deactivation Indication | O |  | ENUMERATED(deactivated, …) | Indicates that the concerned NR cell is switched off for energy saving reasons. If this IE is not included, indicates that the concerned cell is activated. | YES | ignore |
| >>Served NR Cells To Delete |  | 0 .. <maxCellinengNB> |  |  | GLOBAL | reject |
| >>>Old NR-CGI | M |  | NR CGI 9.2.111 |  | - |  |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |
| TNL Transport Layer Address info | O |  | 9.2.149 |  | YES | ignore |
| TNLA To Add List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | 9.2.150 | CP Transport Layer Information of the en-gNB | - | - |
| >>TNLA Usage | M |  | 9.2.151 |  | - | - |
| TNLA To Update List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | 9.2.150 | CP Transport Layer Information of the en-gNB | - | - |
| >>TNLA Usage | O |  | 9.2.151 |  | - | - |
| TNLA To Remove List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Remove Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | 9.2.150 | CP Transport Layer Information of the en-gNB | - | - |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxCellinengNB | Maximum no. cells that can be served by an en-gNB. Value is 16384. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the eNB and the en-gNB. Value is 32. |

#### 9.1.2.35	EN-DC CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a neighbouring node to a peer node, both nodes able to interact for EN-DC, to acknowledge update of information for a TNL association.

Direction: en-gNB  eNB, eNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| CHOICE Responding NodeType | M |  |  |  | YES | reject |
| >eNB |  |  |  |  |  |  |
| >en-gNB |  |  |  |  |  |  |
| >>List of Served NR Cells |  | 0 .. <maxCellinengNB> |  | Complete or limited list of cells served by the en-gNB, if requested by the eNB. | – |  |
| >>>Served NR Cell Information | M |  | 9.2.110 |  | – |  |
| >>>NR Neighbour Information | O |  | 9.2.98 | NR neighbours. | – |  |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| TNL Transport Layer Address info | O |  | 9.2.149 |  | YES | ignore |
| TNLA Setup List |  | 0..1 |  |  | YES | ignore |
| >TNLA Setup Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Address | M |  | 9.2.150 | CP Transport Layer Information of the en-gNB | – |  |
| TNLA Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >TNLA Failed To Setup Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Address | M |  | 9.2.150 | CP Transport Layer Information of the en-gNB | – |  |
| >>Cause | M |  | 9.2.6 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxCellinengNB | Maximum no. cells that can be served by an en-gNB. Value is 16384. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the eNB and the en-gNB. Value is 32. |

#### 9.1.2.36	EN-DC CONFIGURATION UPDATE FAILURE

This message is sent by a neighbouring node to a peer node to indicate EN-DC eNB Configuration Update Failure.

Direction: en-gNB  eNB, eNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Time To Wait | O |  | 9.2.32 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

#### 9.1.2.37	EN-DC CELL ACTIVATION REQUEST

This message is sent by an eNB to a peer en-gNB to request a previously switched-off cell(s) to be re-activated.

Direction: eNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Served NR Cells To Activate |  | 1 |  |  | GLOBAL | reject |
| >Served NR Cells To Activate Item |  | 1 .. < maxCellinengNB > |  |  |  |  |
| >>NR CGI | M |  | 9.2.111 |  | - |  |
| Activation ID | M |  | INTEGER (0..255) | Allocated by the eNB | YES | reject |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxCellinengNB | Maximum no. cells that can be served by an en-gNB. Value is 16384. |

#### 9.1.2.38	EN-DC CELL ACTIVATION RESPONSE

This message is sent by an en-gNB to a peer eNB to indicate that one or more cell(s) previously switched-off has (have) been activated.

Direction: en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Activated NR Cell List |  | 1 |  |  | GLOBAL | ignore |
| >Activated NR Cell Item |  | 1 .. < maxCellinengNB > |  |  |  |  |
| >>NR CGI | M |  | 9.2.111 |  | - |  |
| Activation ID | M |  | INTEGER (0..255) | Allocated by the eNB | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxCellinengNB | Maximum no. cells that can be served by an en-gNB. Value is 16384. |

#### 9.1.2.39	EN-DC CELL ACTIVATION FAILURE

This message is sent by an en-gNB to a peer eNB to indicate cell activation failure.

Direction: en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Activation ID | M |  | INTEGER (0..255) | Allocated by the eNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

#### 9.1.2.40	EN-DC X2 REMOVAL REQUEST

This message is sent by an initiating node to a neighbouring node to initiate the removal of the interface instance.

Direction: eNB  en-gNB, en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| CHOICE Initiating Node Type | M |  |  |  |  |  |
| >eNB |  |  |  |  |  |  |
| >>Global eNB ID | M |  | 9.2.22 |  | YES | reject |
| >en-gNB |  |  |  |  |  |  |
| >>Global en-gNB ID | M |  | 9.2.112 |  |  |  |
| X2 Removal Threshold | O |  | X2 Benefit Value 9.2.90 |  | YES | reject |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

#### 9.1.2.41	EN-DC X2 REMOVAL RESPONSE

This message is sent by an initiating node to a neighbouring node to acknowledge the initiation of removal of the interface instance.

Direction: eNB  en-gNB, en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| CHOICE Initiating Node Type | M |  |  |  |  |  |
| >eNB |  |  |  |  |  |  |
| >>Global eNB ID | M |  | 9.2.22 |  | YES | reject |
| >en-gNB |  |  |  |  |  |  |
| >>Global en-gNB ID | M |  | 9.2.112 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

#### 9.1.2.42	EN-DC X2 REMOVAL FAILURE

This message is sent by the initiating node to indicate that removing the interface instance cannot be accepted.

Direction: eNB  en-gNB, en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

#### 9.1.2.43	DATA FORWARDING ADDRESS INDICATION

This message is sent by the new eNB to indicate to the old eNB forwarding addresses for each E-RAB for which it admits data forwarding.

During a Conditional Handover with EN-DC or Dual Connectivity or Conditional PSCell Change, this message is also used to provide data forwarding related information. In case of EN-DC, the data forwarding related information is transferred from the eNB to the en-gNB, while in case of Dual Connectivity, the data forwarding related information is transferred from the MeNB to the SeNB.

Direction: new eNB  old eNB.

Direction: MeNB  en-gNB (Conditional Handover with EN-DC, Conditional PSCell Change), MeNB  SeNB (Conditional Handover with Dual Connectivity)

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the new eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the new eNB | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the old eNB | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the old eNB | YES | ignore |
| E-RABs Data Forwarding Address List |  | 1 |  | This IE is ignored if the CHO DC Indicator IE is included and set to "coordination-only" or if the CPC Data Forwarding Indicator IE is included and set to "coordination-only". | YES | ignore |
| > E-RABs Data Forwarding Address Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| CHO DC Indicator | O |  | ENUMERATED (true, ..., coordination-only) | Indicating that the DATA FORWARDING ADDRESS INDICATION message is for a Conditional Handover. | YES | reject |
| CHO DC Early Data Forwarding Indicator | O |  | ENUMERATED (stop, ...) |  | YES | ignore |
| SgNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated for EN-DC at the en-gNB. | YES | ignore |
| CPC Data Forwarding Indicator | O |  | ENUMERATED (triggered, early data transmission stop, ..., coordination-only) | Indicates that the DATA FORWARDING ADDRESS INDICATION message is for a Conditional PSCell Change. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

#### 9.1.2.44	EN-DC CONFIGURATION TRANSFER

This message is sent by an eNB in order to transfer the EN-DC SON Configuration container to an en-gNB, or it is sent by an en-gNB in order to transfer the EN-DC SON Configuration container to an eNB.

Direction: eNB  en-gNB or en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| EN-DC SON Configuration Transfer | O |  | OCTET STRING | Contains the EN-DC SON Configuration Transfer IE as defined in TS 36.413 [4]. | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

#### 9.1.2.45	EN-DC RESOURCE STATUS REQUEST

This message is sent by the eNB to the en-gNB or by the en-gNB to the eNB to initiate the requested measurement according to the parameters given in the message.

Direction: E-UTRAN node1  E-UTRAN node2 (eNB  en-gNB, en-gNB  eNB).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| E-UTRAN node1 Measurement ID | M |  | INTEGER (1..4095, ...) | Allocated by the E-UTRAN node1. | YES | reject |
| E-UTRAN node2 Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095, ...) | Allocated by the E-UTRAN node2. | YES | ignore |
| Registration Request EN-DC | M |  | ENUMERATED (start, stop, add, …) | Type of request for which the resource status is required. | YES | reject |
| Reporting Periodicity EN-DC | O |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of PRB Periodic, TNL Capacity Ind Periodic, Composite Available Capacity Periodic. Also used as the averaging window length for all measurement object if supported. | YES | ignore |
| Report Characteristics EN-DC | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | When sent by the eNB, each position in the bitmap indicates measurement object the en-gNB is requested to report. First Bit = PRB Periodic, Second Bit = TNL Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit = Number of Active UEs. Other bits shall be ignored by the en-gNB. When sent by the en-gNB, each position in the bitmap indicates measurement object the eNB is requested to report. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic, this bit should be set to 1 if at least one of the First, Second or Third bits is set to 1. Other bits shall be ignored by the eNB. | YES | ignore |
| NR Cell To Report EN-DC List |  | 0..1 |  | NR cell ID list to which the request applies. | YES | ignore |
| >NR Cell To Report EN-DC Item |  | 1 .. <maxCellinengNB> |  |  | EACH | ignore |
| >>NR Cell ID | M |  | NR CGI 9.2.111 |  | – |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | YES | ignore |
| >>>SSB To Report Item |  | 1 .. <maxnoofSSBAreas> |  |  | EACH | ignore |
| >>>>SSB Index | M |  | 9.2.167 |  | – |  |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |
| E-UTRA Cell To Report EN-DC List |  | 0..1 |  | E-UTRA cell ID list to which the request applies. | YES | ignore |
| >E-UTRA Cell To Report EN-DC Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>E-UTRA Cell ID | M |  | ECGI 9.2.14 |  | – |  |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request EN-DC IE is set to the value "stop", or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request EN-DC IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxCellinengNB | Maximum no. cells that can be served by an en-gNB. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

#### 9.1.2.46	EN-DC RESOURCE STATUS RESPONSE

This message is sent by the en-gNB or by the eNB to indicate that the requested measurement, for all of the measurement objects included in the measurement is successfully initiated.

Direction: E-UTRAN node2  E-UTRAN node1 (en-gNB  eNB, eNB  en-gNB).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| E-UTRAN node1 Measurement ID | M |  | INTEGER (1..4095, ...) | Allocated by the E-UTRAN node1. | YES | reject |
| E-UTRAN node2 Measurement ID | M |  | INTEGER (1..4095, ...) | Allocated by the E-UTRAN node2. | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

#### 9.1.2.47	EN-DC RESOURCE STATUS FAILURE

This message is sent by the en-gNB or by the eNB to indicate that for any of the requested measurement objects the measurement cannot be initiated.

Direction: E-UTRAN node2  E-UTRAN node1 (en-gNB  eNB, eNB  en-gNB).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| E-UTRAN node1 Measurement ID | M |  | INTEGER (1..4095, ...) | Allocated by the E-UTRAN node1. | YES | reject |
| E-UTRAN node2 Measurement ID | M |  | INTEGER (1..4095, ...) | Allocated by the E-UTRAN node2. | YES | reject |
| Cause | M |  | 9.2.6 | Ignored by the receiver when the Complete Failure Cause Information IE is included | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

#### 9.1.2.48	EN-DC RESOURCE STATUS UPDATE

This message is sent by the en-gNB or by the eNB to the en-gNB to the eNB to report the results of the requested measurements.

Direction: E-UTRAN node2  E-UTRAN node1 (en-gNB  eNB, eNB  en-gNB).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| E-UTRAN node1 Measurement ID | M |  | INTEGER (1..4095, ...) | Allocated by the E-UTRAN node1. | YES | reject |
| E-UTRAN node2 Measurement ID | M |  | INTEGER (1..4095, ...) | Allocated by the E-UTRAN node2. | YES | reject |
| NR Cell Measurement Result |  | 0..1 |  | Concerned NR cells in the en-gNB. | YES | ignore |
| >NR Cell Measurement Result Item |  | 1 .. <maxCellinengNB> |  |  | EACH | ignore |
| >>NR Cell ID | M |  | NR CGI 9.2.111 |  | – |  |
| >>NR Radio Resource Status | O |  | 9.2.162 |  | – |  |
| >>TNL Capacity Indicator | O |  | 9.2.161 |  | – |  |
| >>NR Composite Available Capacity Group | O |  | 9.2.163 |  | – |  |
| >>Number of Active UEs | O |  | INTEGER (0..16777215, ...) | As defined in TS 38.314 [45]. Value "1" is equivalent to 0.1 Active UEs, value "2" is equivalent to 0.2 Active UEs, value n is equivalent to n/10 Active UEs. | – |  |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |
| E-UTRA Cell Measurement Result |  | 0..1 |  | Concerned E-UTRA cells in the eNB. | YES | ignore |
| >E-UTRA Cell Measurement Result Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>E-UTRA Cell ID | M |  | ECGI 9.2.14 |  | – |  |
| >>Hardware Load Indicator | O |  | 9.2.34 |  | – |  |
| >>S1 TNL Load Indicator | O |  | 9.2.35 |  | – |  |
| >>Radio Resource Status | O |  | 9.2.37 |  | – |  |
| >>Composite Available Capacity Group | O |  | 9.2.44 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxCellinengNB | Maximum no. cells that can be served by an en-gNB. Value is 16384. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

#### 9.1.2.49	CELL TRAFFIC TRACE

This message is sent by en-gNB to transfer the trace information to the MeNB.

Direction: en-gNB  MeNB

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  | 9.2.1.1 |  | YES | ignore |
| MeNB UE X2AP ID |  | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID |  | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| E-UTRAN Trace ID |  | M |  | OCTET STRING (SIZE(8)) | As per E-UTRAN Trace ID in Trace Activation IE | YES | ignore |
| Trace Collection Entity IP Address |  | M |  | BIT STRING (1..160,…) | Defined in TS 32.422 [6] | YES | ignore |
| Privacy Indicator |  | O |  | ENUMERATED (Immediate MDT, ...) |  | YES | ignore |
| MeNB UE X2AP ID Extension |  | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |

#### 9.1.2.50	ACCESS AND MOBILITY INDICATION

This message is sent by the eNB to the en-gNB to transfer access and mobility related information.

Direction: eNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NR RACH Report List |  | 0..1 |  |  | YES | ignore |
| >NR RACH Report List Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | ignore |
| >>NR RACH Report Container | M |  | OCTET STRING | Includes the ra-ReportList contained in the UEInformationResponse messageas defined in subclause 6.2.2 in TS 38.331 [10]. | – |  |
| >>UE Assistant Identifier | O |  | en-gNB UE X2AP ID 9.2.100 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |

### 9.1.3	Messages for Dual Connectivity Procedures

#### 9.1.3.1	SENB ADDITION REQUEST

This message is sent by the MeNB to the SeNB to request the preparation of resources for dual connectivity operation for a specific UE

Direction: MeNB  SeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| UE Security Capabilities | C- ifSCGBearerOption |  | 9.2.29 |  | YES | reject |
| SeNB Security Key | C- ifSCGBearerOption |  | 9.2.72 | The S-KeNB which is provided by the MeNB, see TS 33.401 [18]. | YES | reject |
| SeNB UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.12 | The UE Aggregate Maximum Bit Rate is split into MeNB UE Aggregate Maximum Bit Rate and SeNB UE Aggregate Maximum Bit Rate which are enforced by MeNB and SeNB respectively. | YES | reject |
| Serving PLMN | O |  | PLMN Identity 9.2.4 | The serving PLMN of the SCG in the SeNB. | YES | ignore |
| E-RABs To Be Added List |  | 1 |  |  | YES | reject |
| >E-RABs To Be Added Item |  | 1 .. <maxnoofBearers> |  |  | EACH | reject |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>SCG Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – |  |
| >>>>DL Forwarding | O |  | 9.2.5 |  | – |  |
| >>>>S1 UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – |  |
| >>>>Correlation ID | O |  | Correlation ID 9.2.84 |  | – |  |
| >>>>SIPTO Correlation ID | O |  | Correlation ID 9.2.84 |  | – |  |
| >>>>Bearer Type | O |  | 9.2.92 |  | YES | ignore |
| >>>>Ethernet Type | O |  | 9.2.157 |  | YES | ignore |
| >>>>Source DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >>>Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – |  |
| >>>>MeNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2 transport bearer. For delivery of UL PDUs. | – |  |
| >>>>Source DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| MeNB to SeNB Container | M |  | OCTET STRING | Includes the SCG-ConfigInfo message as defined in TS 36.331 [9] | YES | reject |
| CSG Membership Status | O |  | 9.2.52 |  | YES | reject |
| SeNB UE X2AP ID | O |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | reject |
| SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | YES | reject |
| Expected UE Behaviour | O |  | 9.2.70 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSCGBearerOption | This IE shall be present if the Bearer Option IE is set to the value "SCG bearer". |

#### 9.1.3.2	SENB ADDITION REQUEST ACKNOWLEDGE

This message is sent by the SeNB to confirm the MeNB about the SeNB addition preparation.

Direction: SeNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | reject |
| E-RABs Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Added Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>SCG Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>S1 DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SeNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – |  |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – |  |
| >>>>Source DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >>>Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>SeNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | Endpoint of the X2 transport bearer at the SeNB. | – |  |
| >>>>Source DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |
| SeNB to MeNB Container | M |  | OCTET STRING | Includes the SCG-Config message as defined in TS 36.331 [9] | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| GW Transport Layer Address | O |  | BIT STRING (1..160, ...) | Indicating GW Transport Layer Address. | YES | ignore |
| SIPTO L-GW Transport Layer Address | O |  | BIT STRING (1..160, ...) | Indicating SIPTO L-GW Transport Layer Address. | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | YES | reject |
| Tunnel Information for BBF | O |  | Tunnel Information 9.2.89 | Indicating eNB’s Local IP Address assigned by the broadband access provider, UDP port Number. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

#### 9.1.3.3	SENB ADDITION REQUEST REJECT

This message is sent by the SeNB to inform the MeNB that the SeNB Addition Preparation has failed.

Direction: SeNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | YES | reject |

#### 9.1.3.4	SENB RECONFIGURATION COMPLETE

This message is sent by the MeNB to the SeNB to indicate whether the configuration requested by the SeNB was applied by the UE.

Direction: MeNB  SeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | reject |
| Response Information | M |  |  |  | YES | ignore |
| >CHOICE Response Type | M |  |  |  |  |  |
| >>Configuration successfully applied |  |  |  |  |  |  |
| >>>MeNB to SeNB Container | O |  | OCTET STRING | Includes the SCG-ConfigInfo message as defined in TS 36.331 [9] | - |  |
| >>Configuration rejected by the MeNB |  |  |  |  |  |  |
| >>>Cause | M |  | 9.2.6 |  | - |  |
| >>>MeNB to SeNB Container | O |  | OCTET STRING | Includes the SCG-ConfigInfo message as defined in TS 36.331 [9] | - |  |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | YES | reject |

#### 9.1.3.5	SENB MODIFICATION REQUEST

This message is sent by the MeNB to the SeNB to request the preparation to modify SeNB resources for a specific UE.

Direction: MeNB  SeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| SCG Change Indication | O |  | 9.2.73 |  | YES | ignore |
| Serving PLMN | O |  | PLMN Identity 9.2.4 | The serving PLMN of the SCG in the SeNB. | YES | ignore |
| UE Context Information |  | 0..1 |  |  | YES | reject |
| >UE Security Capabilities | O |  | 9.2.29 |  | – |  |
| >SeNB Security Key | O |  | 9.2.72 |  | – |  |
| >SeNB UE Aggregate Maximum Bit Rate | O |  | UE Aggregate Maximum Bit Rate 9.2.12 |  | – |  |
| >E-RABs To Be Added List |  | 0..1 |  |  | – |  |
| >>E-RABs To Be Added Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – |  |
| >>>>>DL Forwarding | O |  | 9.2.5 |  | – |  |
| >>>>>S1 UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – |  |
| >>>>>Correlation ID | O |  | Correlation ID 9.2.84 |  | – |  |
| >>>>>SIPTO Correlation ID | O |  | Correlation ID 9.2.84 |  | – |  |
| >>>>>Bearer Type | O |  | 9.2.92 |  | YES | ignore |
| >>>>>Ethernet Type | O |  | 9.2.157 |  | YES | ignore |
| >>>>>Source DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – |  |
| >>>>>MeNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2 transport bearer. For delivery of UL PDUs. | – |  |
| >>>>>Source DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >E-RABs To Be Modified List |  | 0..1 |  |  | – |  |
| >>E-RABs To Be Modified Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>>E-RAB Level QoS Parameters | O |  | 9.2.9 | Includes QoS parameters to be modified | – |  |
| >>>>>S1 UL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – |  |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>>E-RAB Level QoS Parameters | O |  | 9.2.9 | Includes QoS parameters to be modified | – |  |
| >>>>>MeNB GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2 transport bearer. For delivery of UL PDUs. | – |  |
| >E-RABs To Be Released List |  | 0..1 |  |  | – |  |
| >>E-RABs To Be Released Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer. used for forwarding of UL PDUs | – |  |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| MeNB to SeNB Container | O |  | OCTET STRING | Includes the SCG-ConfigInfo message as defined in TS 36.331 [9] | YES | ignore |
| CSG Membership Status | O |  | 9.2.52 |  | YES | reject |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

#### 9.1.3.6	SENB MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the SeNB to confirm the MeNB’s request to modify the SeNB resources for a specific UE.

Direction: SeNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | ignore |
| E-RABs Admitted List |  | 0..1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Added List |  | 1 |  |  | – |  |
| >>E-RABs Admitted To Be Added Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>>S1 DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SeNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – |  |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – |  |
| >>>>>Source DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>>SeNB GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | Endpoint of the X2 transport bearer at the SeNB. | – |  |
| >>>>>Source DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >E-RABs Admitted To Be Modified List |  | 0..1 |  |  | – |  |
| >>E-RABs Admitted To Be Modified Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>>S1 DL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | SeNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – |  |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>>SeNB GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Endpoint of the X2 transport bearer at the SeNB. | – |  |
| >E-RABs Admitted To Be Released List |  | 0..1 |  |  | – |  |
| >>E-RABs Admitted To Be Released Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |
| SeNB to MeNB Container | O |  | OCTET STRING | Includes the SCG-Config message as defined in TS 36.331 [9] | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

#### 9.1.3.7	SENB MODIFICATION REQUEST REJECT

This message is sent by the SeNB to inform the MeNB that the MeNB initiated SeNB Modification Preparation has failed.

Direction: SeNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | YES | ignore |

#### 9.1.3.8	SENB MODIFICATION REQUIRED

This message is sent by the SeNB to the MeNB to request the modification of SeNB resources for a specific UE.

Direction: SeNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| SCG Change Indication | O |  | 9.2.73 |  | YES | ignore |
| E-RABs To Be Released List |  | 0..1 |  |  | YES | ignore |
| >E-RABs To Be Released Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>Cause | M |  | 9.2.6 |  | – |  |
| SeNB to MeNB Container | O |  | OCTET STRING | Includes the SCG-Config message as defined in TS 36.331 [9] | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

#### 9.1.3.9	SENB MODIFICATION CONFIRM

This message is sent by the MeNB to inform the SeNB about the successful modification.

Direction: MeNB  SeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | ignore |
| MeNB to SeNB Container | O |  | OCTET STRING | Includes the SCG-ConfigInfo message as defined in TS 36.331 [9] | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | YES | ignore |

#### 9.1.3.10	SENB MODIFICATION REFUSE

This message is sent by the MeNB to inform the SeNB that the SeNB initiated SeNB Modification has failed.

Direction: MeNB  SeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| MeNB to SeNB Container | O |  | OCTET STRING | Includes the SCG-ConfigInfo message as defined in TS 36.331 [9] | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | YES | ignore |

#### 9.1.3.11	SENB RELEASE REQUEST

This message is sent by the MeNB to the SeNB to request the release of resources.

Direction: MeNB  SeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID | O |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | reject |
| Cause | O |  | 9.2.6 |  | YES | ignore |
| E-RABs To Be Released List |  | 0..1 |  |  | YES | ignore |
| > E-RABs To Be Released Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>SCG Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – |  |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer. used for forwarding of DL PDUs | – |  |
| >>>Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer. used for forwarding of DL PDUs | – |  |
| UE Context Kept Indicator | O |  | 9.2.85 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | YES | reject |
| MakeBeforeBreak Indicator | O |  | ENUMERATED (True, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

#### 9.1.3.12	SENB RELEASE REQUIRED

This message is sent by the SeNB to request the release of all resources for a specific UE at the SeNB.

Direction: SeNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | YES | reject |

#### 9.1.3.13	SENB RELEASE CONFIRM

This message is sent by the MeNB to confirm the release of all resources for a specific UE at the SeNB.

Direction: MeNB  SeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | ignore |
| E-RABs to be Released List |  | 0..1 |  |  | YES | ignore |
| >E-RABs To Be Released Item |  | 1 .. <maxnoofBearers> |  |  | – |  |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>SCG Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – |  |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| >>>Split Bearer |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

#### 9.1.3.14	SENB COUNTER CHECK REQUEST

This message is sent by the SeNB to request the verification of the value of the PDCP COUNTs associated with SCG bearers established in the SeNB.

Direction: SeNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | ignore |
| E-RABs Subject to Counter Check List |  | 1 |  |  | YES | ignore |
| >E-RABs Subject to Counter Check Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | - |  |
| >>UL COUNT | M | INTEGER(0.. 4294967295) |  | Indicates the value of uplink COUNT associated to this E-RAB. | - |  |
| >>DL COUNT | M | INTEGER(0.. 4294967295) |  | Indicates the value of downlink COUNT associated to this E-RAB. | - |  |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

### 9.1.4	Messages for E-UTRAN-NR Dual Connectivity Procedures

#### 9.1.4.1	SGNB ADDITION REQUEST

This message is sent by the MeNB to the en-gNB to request the preparation of resources for EN-DC operation for a specific UE

Direction: MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| NR UE Security Capabilities | M |  | 9.2.107 |  | YES | reject |
| SgNB Security Key | M |  | 9.2.101 | The S-KgNB which is provided by the MeNB, see TS 33.401 [18]. | YES | reject |
| SgNB UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.12 | The UE Aggregate Maximum Bit Rate is split into MeNB UE Aggregate Maximum Bit Rate and SgNB UE Aggregate Maximum Bit Rate which are enforced by MeNB and en-gNB respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.4 | The selected PLMN of the SCG in the en-gNB. | YES | ignore |
| Handover Restriction List | O |  | 9.2.3 |  | YES | ignore |
| E-RABs To Be Added List |  | 1 |  |  | YES | reject |
| >E-RABs To Be Added Item |  | 1 .. <maxnoofBearers> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>DRB ID | M |  | 9.2.122 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>Full E-RAB Level QoS Parameters | M |  | E-RAB Level QoS Parameters 9.2.9 | Includes the E-RAB level QoS parameters as received on S1-MME. | – |  |
| >>>>Maximum MCG admittable E-RAB Level QoS Parameters | C-ifMCGandSCGpresent_GBR |  | GBR QoS Information 9.2.10 | Includes the GBR QoS Information admittable by the MCG. | – |  |
| >>>>DL Forwarding | O |  | 9.2.5 |  | – |  |
| >>>>MeNB DL GTP Tunnel Endpoint at MCG | C-ifMCGpresent |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer at MCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>S1 UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1-U transport bearer. For delivery of UL PDUs from the en-gNB. | – |  |
| >>>>RLC Mode | O |  | RLC Mode 9.2.119 | Indicates the RLC mode at the MeNB for PDCP transfer to en-gNB. | YES | ignore |
| >>>>Bearer Type | O |  | 9.2.92 |  | YES | ignore |
| >>>>Ethernet Type | O |  | 9.2.157 |  | YES | ignore |
| >>>>Source DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | Identifies the TNL address used by the source MN for data forwarding. | YES | ignore |
| >>>>Source Node DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | This IE is present only for the case of SA to EN-DC handover and it is used to identify the TNL address allocated by the source NG-RAN node for data forwarding. | YES | ignore |
| >>>>Security Indication | O |  | 9.2.181 |  | YES | reject |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>Requested SCG E-RAB Level QoS Parameters | M |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters requested to be provided by the SCG. | – |  |
| >>>>MeNB UL GTP Tunnel Endpoint at PDCP | M |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer. For delivery of UL PDCP PDUs. | – |  |
| >>>>Secondary MeNB UL GTP Tunnel Endpoint at PDCP | O |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer. For delivery of UL PDCP PDUs in case of PDCP duplication. | – |  |
| >>>>RLC Mode | M |  | RLC Mode 9.2.119 | Indicates the RLC mode to be used in the assisting node. | – |  |
| >>>>UL Configuration | C-ifMCGandSCGpresent |  | 9.2.118 | Information about UL usage in the en-gNB. | – |  |
| >>>>UL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the UL. | YES | ignore |
| >>>>DL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the DL. | YES | ignore |
| >>>>Duplication activation | O |  | 9.2.137 | Indicated the initial staus of PDCP duplication. | YES | ignore |
| MeNB to SgNB Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in TS 38.331 [31]. | YES | reject |
| SgNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Expected UE Behaviour | O |  | 9.2.70 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |
| Requested split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRB are requested. | YES | reject |
| MeNB Resource Coordination Information | O |  | 9.2.116 | Information used to coordinate resources utilisation between MeNB and en-gNB. | YES | ignore |
| SGNB Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-eNB HO, intra-eNB HO, ...) | This IE indicates the trigger for SGNB Addition procedure. | YES | reject |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.25 |  | YES | ignore |
| MeNB Cell ID | M |  | ECGI 9.2.14 | Indicates the cell ID for PCell in MeNB. | YES | reject |
| Desired Activity Notification Level | O |  | 9.2.141 |  | YES | ignore |
| Trace Activation | O |  | 9.2.2 |  | YES | ignore |
| Location Information at SgNB reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s location information is to be provided. | YES | ignore |
| Masked IMEISV | O |  | 9.2.69 |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.2.25a |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| UE Context Reference at Source NG-RAN | O |  | RAN UE NGAP ID 9.2.152 |  | YES | ignore |
| Management Based MDT Allowed | O |  | 9.2.59 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.64 |  | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.171 |  | YES | reject |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| Source NG-RAN Node ID | O |  | Global RAN Node ID 9.2.176 |  | YES | ignore |
| UE History Information | O |  | 9.2.38 |  | YES | ignore |
| UE History Information from the UE | O |  | OCTET STRING | Includes the VisitedCellInfoList contained in the UEInformationResponse message as defined TS 36.331 [9]. | YES | ignore |
| PScell Change History | O |  | ENUMERATED (reporting full history, ...) |  | YES | ignore |
| CHO Information SN Addition | O |  |  |  | YES | reject |
| >Source eNB ID | M |  | Global eNB ID 9.2.22 |  |  |  |
| >Source eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source MeNB |  |  |
| >Source eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source MeNB |  |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  |  |  |
| SCG Activation Request | O |  | 9.2.179 |  | YES | ignore |
| Conditional PSCell Addition Information Request | O |  |  |  | YES | reject |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the target may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target en-gNB. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifMCGandSCGpresent | This IE shall be present if, for the E-RAB requested to be added, the MCG resources and SCG resources IEs in the EN-DC Resource Configuration IE are set to the value "present". |
| ifMCGpresent | This IE shall be present if, for the E-RAB requested to be added, the MCG resources IE in the EN-DC Resource Configuration IE is set to the value "present". |
| C-ifMCGandSCGpresent_GBR | This IE shall be present if, for the E-RAB requested to be added, the MCG resources and SCG resources IEs in the EN-DC Resource Configuration IE are set to the value "present", and GBR QoS Information IE is present in Full E-RAB Level QoS Parameters IE. |

#### 9.1.4.2	SGNB ADDITION REQUEST ACKNOWLEDGE

This message is sent by the en-gNB to confirm the MeNB about the SgNB addition preparation.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| E-RABs Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Added Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>S1 DL GTP Tunnel Endpoint at the SgNB | M |  | GTP Tunnel Endpoint 9.2.1 | en-gNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – |  |
| >>>>SgNB UL GTP Tunnel Endpoint at PDCP | C-ifMCGpresent |  | GTP Tunnel Endpoint 9.2.1 | en-gNB endpoint of the X2-U transport bearer at PDCP. For delivery of UL PDCP PDUs. | – |  |
| >>>>RLC Mode | C-ifMCGpresent |  | RLC Mode 9.2.119 | Indicates the RLC mode. | – |  |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the transport bearer used for forwarding of DL PDUs | – |  |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the transport bearer used for forwarding of UL PDUs | – |  |
| >>>>Requested MCG E-RAB Level QoS Parameters | C-ifMCGandSCGpresent_GBRpresent |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters requested to be provided by the MCG. | – |  |
| >>>>UL Configuration | C-ifMCGandSCGpresent |  | 9.2.118 | Information about UL usage in the MeNB. | – |  |
| >>>>UL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the UL. | YES | ignore |
| >>>>DL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the DL. | YES | ignore |
| >>>>Source DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >>>>Security Result | O |  | 9.2.182 |  | YES | ignore |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>SgNB DL GTP Tunnel Endpoint at SCG | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>Secondary SgNB DL GTP Tunnel Endpoint at SCG | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs in case of PDCP duplication | – |  |
| >>>>LCID | O |  | 9.2.138 | LCID for the primary path in case of PDCP duplication | YES | ignore |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |
| SgNB to MeNB Container | M |  | OCTET STRING | Includes the CG-Config message or the CG-CandidateList message, as defined in TS 38.331[31]. | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| Admitted split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |
| SgNB Resource Coordination Information | O |  | 9.2.117 | Information used to coordinate resources utilisation between en-gNB and MeNB. | YES | ignore |
| RRC config indication | O |  | 9.2.132 | Indicates the type of RRC configuration used at the en-gNB. | YES | reject |
| Location Information at SgNB | O |  | 9.2.142 | Contains information to support localisation of the UE | YES | ignore |
| Available fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 is enabled. | YES | ignore |
| Direct Forwarding Path Availability | O |  | ENUMERATED (direct path available, …) | Indicates direct forwarding path is available between the target en-gNB and source NG-RAN node for SA to EN-DC handover. | YES | ignore |
| SCG Activation Status | O |  | 9.2.178 |  | YES | ignore |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  | YES | ignore |
| >Candidate PSCell List |  | 1 |  |  | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.111 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCells for CPAC. Value is 8. |

| Condition | Explanation |
| --- | --- |
| ifMCGpresent | This IE shall be present if, for the E-RAB admitted to be added, the MCG resources IE in the EN-DC Resource Configuration IE is set to the value "present". |
| ifMCGandSCGpresent | This IE shall be present if, for the E-RAB admitted to be added, the MCG resources and SCG resources IEs in the EN-DC Resource Configuration IE are set to the value "present". |
| C-ifMCGandSCGpresent_GBRpresent | This IE shall be present if, for the E-RAB admitted to be added, the MCG resources and SCG resources IEs in the EN-DC Resource Configuration IE are set to the value "present", and the GBR QoS Information IE is present in the Requested MCG E-RAB Level QoS Parameters IE. |

#### 9.1.4.3	SGNB ADDITION REQUEST REJECT

This message is sent by the en-gNB to inform the MeNB that the SgNB Addition Preparation has failed.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |

#### 9.1.4.4	SGNB RECONFIGURATION COMPLETE

This message is sent by the MeNB to the en-gNB to indicate whether the configuration requested by the en-gNB was applied by the UE.

Direction: MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Response Information | M |  |  |  | YES | ignore |
| >CHOICE Response Type | M |  |  |  |  |  |
| >>Configuration successfully applied |  |  |  |  |  |  |
| >>>MeNB to SgNB Container | O |  | OCTET STRING | Includes the RRCReconfigurationComplete message as defined in TS 38.331 [31]. | - |  |
| >>Configuration rejected |  |  |  |  |  |  |
| >>>Cause | M |  | 9.2.6 |  | - |  |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |

#### 9.1.4.5	SGNB MODIFICATION REQUEST

This message is sent by the MeNB to the en-gNB to request the preparation to modify en-gNB resources for a specific UE, to query for the current SCG configuration, or to provide the S-RLF-related information to the en-gNB.

Direction: MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Selected PLMN | O |  | PLMN Identity 9.2.4 | The selected PLMN of the SCG in the en-gNB. | YES | ignore |
| Handover Restriction List | O |  | 9.2.3 |  | YES | ignore |
| SCG Configuration Query | O |  | 9.2.103 |  | YES | ignore |
| UE Context Information |  | 0..1 |  |  | YES | reject |
| >NR UE Security Capabilities | O |  | 9.2.107 |  | – |  |
| >SgNB Security Key | O |  | 9.2.101 |  | – |  |
| >SgNB UE Aggregate Maximum Bit Rate | O |  | UE Aggregate Maximum Bit Rate 9.2.12 |  | – |  |
| >Lower Layer presence status change | O |  | 9.2.145 |  | – |  |
| >E-RABs To Be Added List |  | 0..1 |  |  | – |  |
| >>E-RABs To Be Added Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>DRB ID | M |  | 9.2.122 |  | – |  |
| >>>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". | – |  |
| >>>>>Full E-RAB Level QoS Parameters | M |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters as received on S1-MME. | – |  |
| >>>>>Maximum MCG admittable E-RAB Level QoS Parameters | C-ifMCGandSCGpresent_GBR |  | GBR QoS Information 9.2.10 | Includes the GBR QoS Information admittable by the MCG. | – |  |
| >>>>>DL Forwarding | O |  | 9.2.5 |  | – |  |
| >>>>>MeNB DL GTP Tunnel Endpoint at MCG | C-ifMCGpresent |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer at MCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>>S1 UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1-U transport bearer. For delivery of UL PDUs from the en-gNB. | – |  |
| >>>>>RLC Mode | O |  | RLC Mode 9.2.119 | Indicates the RLC mode at the MeNB for PDCP transfer to en-gNB. | YES | ignore |
| >>>>>Bearer Type | O |  | 9.2.92 |  | YES | ignore |
| >>>>>Ethernet Type | O |  | 9.2.157 |  | YES | ignore |
| >>>>>Security Indication | O |  | 9.2.181 |  | YES | reject |
| >>>>>Source DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >>>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>>Requested SCG E-RAB Level QoS Parameters | M |  | E-RAB Level QoS Parameters 9.2.9 | Includes necessary E-RAB level QoS parameters requested to be provided by the SCG. | – |  |
| >>>>>MeNB UL GTP Tunnel Endpoint at PDCP | M |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer. For delivery of UL PDCP PDUs. | – |  |
| >>>>>Secondary MeNB UL GTP Tunnel Endpoint at PDCP | O |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer. For delivery of UL PDCP PDUs in case of PDCP duplication. | – |  |
| >>>>>RLC Mode | M |  | RLC Mode 9.2.119 | Indicates the RLC mode to be used in the assisting node. | – |  |
| >>>>>UL Configuration | C-ifMCGandSCGpresent |  | 9.2.118 | Information about UL usage in the en-gNB. | – |  |
| >>>>>UL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the UL. | YES | ignore |
| >>>>>DL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the DL. | YES | ignore |
| >>>>>Duplication activation | O |  | 9.2.137 | Indicated the initial staus of PDCP duplication. | YES | ignore |
| >E-RABs To Be Modified List |  | 0..1 |  |  | – |  |
| >>E-RABs To Be Modified Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>>Full E-RAB Level QoS Parameters | O |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters to be modified as received on S1-MME | – |  |
| >>>>>Maximum MCG admittable E-RAB Level QoS Parameters | O |  | GBR QoS Information 9.2.10 | Includes the GBR QoS information admittable by the MCG | – |  |
| >>>>>MeNB GTP Tunnel Endpoint at MCG | O |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer at MCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>>S1 UL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1-U transport bearer. For delivery of UL PDUs from the en-gNB. | – |  |
| >>>>>RLC Status | O |  | 9.2.131 | Indicates the RLC has been re-established.. |  |  |
| >>>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>>Requested SCG E-RAB Level QoS Parameters | O |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters requested to be provided by the SCG. | – |  |
| >>>>>MeNB UL GTP Tunnel Endpoint at PDCP | O |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer. For delivery of UL PDCP PDUs. | – |  |
| >>>>>UL Configuration | O |  | 9.2.118 | Information about UL usage in the en-gNB. | – |  |
| >>>>>UL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Shall be ignored by the en-gNB if received. | YES | ignore |
| >>>>>DL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Shall be ignored by the en-gNB if received. | YES | ignore |
| >>>>>Secondary MeNB UL GTP Tunnel Endpoint at PDCP | O |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer. For delivery of UL PDCP PDUs in case of PDCP duplication. | YES | ignore |
| >E-RABs To Be Released List |  | 0..1 |  |  | – |  |
| >>E-RABs To Be Released Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer. used for forwarding of UL PDUs | – |  |
| >>>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.25 |  | YES | ignore |
| >Additional RRM Policy Index | O |  | 9.2.25a |  | YES | ignore |
| MeNB to SgNB Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in TS 38.331 [31]. | YES | reject |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| MeNB Resource Coordination Information | O |  | 9.2.116 | Information used to coordinate resources utilisation between MeNB and en-gNB. | YES | ignore |
| Requested split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRB are requested. | YES | ignore |
| Requested split SRBs release | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRB are requested to be released. | YES | ignore |
| Desired Activity Notification Level | O |  | 9.2.141 |  | YES | ignore |
| Location Information at SgNB reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s location information is to be provided. | YES | ignore |
| MeNB Cell ID | O |  | ECGI 9.2.14 | Indicates the cell ID for PCell in MeNB. | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| Requested Fast MCG recovery via SRB3 Release | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested to be released. | YES | ignore |
| SN triggered | O |  | ENUMERATED (True, ...) |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| PSCell History Information Retrieve | O |  | ENUMERATED (query, ...) | Indicates that the SN UE history information is requested. | YES | ignore |
| UE History Information from the UE | O |  | OCTET STRING | Includes the VisitedCellInfoList contained in the UEInformationResponse message as defined in TS 36.331 [9]. | YES | ignore |
| CHO Information SN Modification | O |  |  |  | YES | ignore |
| >Conditional Reconfiguration | M |  | ENUMERATED (intra-MN-CHO, ...) |  | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | - | - |
| SCG Activation Request | O |  | 9.2.179 |  | YES | ignore |
| Conditional PSCell Addition Information Modification Request | O |  |  | This IE may be sent to the target en-gNB. | YES | ignore |
| >Maximum Number of PSCells To Prepare | O |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target en-gNB may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target en-gNB. | – |  |
| Conditional PSCell Change Information Update | O |  |  | This IE may be sent to the source en-gNB. | YES | ignore |
| >Multiple Target SgNB Node List |  | 1 |  |  | – |  |
| >>Multiple Target SgNB Node Item |  | 1 .. <maxnoofTargetSgNBs> |  |  | – |  |
| >>>Target SgNB ID | M |  | Global en-gNB ID 9.2.112 |  | – |  |
| >>>Candidate PSCell List |  | 1 |  |  | – |  |
| >>>>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>>>PSCell ID | M |  | NR CGI 9.2.111 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCells for CPAC. Value is 8. |
| maxnoofTargetSgNBs | Maximum no. of the target SgNB nodes. Value is 8 |

| Condition | Explanation |
| --- | --- |
| ifMCGandSCGpresent | This IE shall be present if, for the E-RAB requested to be added, the MCG resources and SCG resources IEs in the EN-DC Resource Configuration IE are set to the value "present". |
| ifMCGpresent | This IE shall be present if, for the E-RAB requested to be added, the MCG resources IE in the EN-DC Resource Configuration IE is set to the value "present". |
| C-ifMCGandSCGpresent_GBR | This IE shall be present if, for the E-RAB requested to be added, the MCG resources and SCG resources IEs in the EN-DC Resource Configuration IE are set to the value "present", and GBR QoS Information IE is present in Full E-RAB Level QoS Parameters IE. |

#### 9.1.4.6	SGNB MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the en-gNB to confirm the MeNB’s request to modify the en-gNB resources for a specific UE.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| E-RABs Admitted To Be Added List |  | 0..1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Added Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>S1 DL GTP Tunnel Endpoint at the SgNB | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – |  |
| >>>>SgNB UL GTP Tunnel Endpoint at PDCP | C-ifMCGpresent |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at PDCP. For delivery of UL PDCP PDUs. | – |  |
| >>>>RLC Mode | C-ifMCGpresent |  | RLC Mode 9.2.119 | Indicates the RLC mode to be used at the assisting node. | – |  |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – |  |
| >>>>Requested MCG E-RAB Level QoS Parameters | C-ifMCGandSCGpresent_GBRpresent |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters requested to be provided by the MCG. | – |  |
| >>>>UL Configuration | C-ifMCGandSCGpresent |  | 9.2.118 | Information about UL usage in the MeNB. | – |  |
| >>>>UL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the UL. | YES | ignore |
| >>>>DL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the DL. | YES | ignore |
| >>>>Security Result | O |  | 9.2.182 |  | YES | ignore |
| >>>>Source DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>SgNB DL GTP Tunnel Endpoint at SCG | M |  | GTP Tunnel Endpoint 9.2.1 | Endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>Secondary SgNB DL GTP Tunnel Endpoint at SCG | O |  | GTP Tunnel Endpoint 9.2.1 | Endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs in case of PDCP duplication. | – |  |
| >>>>LCID | O |  | 9.2.138 | LCID for the primary path in case of PDCP duplication configured. | YES | ignore |
| E-RABs Admitted To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Modified Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>S1 DL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – |  |
| >>>>SgNB UL GTP Tunnel Endpoint at PDCP | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at PDCP. For delivery of UL PDCP PDUs. | – |  |
| >>>>Requested MCG E-RAB Level QoS Parameters | O |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters requested to be provided by the MCG. | – |  |
| >>>>UL Configuration | O |  | 9.2.118 | Information about UL usage in the MeNB. | – |  |
| >>>>UL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Shall be ignored by the MeNB if received. | YES | ignore |
| >>>>DL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Shall be ignored by the MeNB if received. | YES | ignore |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>SgNB DL GTP Tunnel Endpoint at SCG | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>Secondary SgNB DL GTP Tunnel Endpoint at SCG | O |  | GTP Tunnel Endpoint 9.2.1 | Endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs in case of PDCP duplication. | YES | ignore |
| >>>>RLC Status | O |  | 9.2.131 | Indicates the RLC has been re-established. | YES | ignore |
| E-RABs Admitted To Be Released List |  | 0..1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Released Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  | Note: no further information contained in the IE container |  |  |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |
| SgNB to MeNB Container | O |  | OCTET STRING | Includes the CG-Config message or the CG-CandidateList message, as defined in TS 38.331 [31]. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SgNB Resource Coordination Information | O |  | 9.2.117 | Information used to coordinate resources utilisation between en-gNB and MeNB. | YES | ignore |
| Admitted split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | ignore |
| Admitted split SRBs release | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs release | YES | ignore |
| RRC config indication | O |  | 9.2.132 | Indicates the type of RRC configuration used at the en-gNB. | YES | reject |
| Location Information at SgNB | O |  | 9.2.142 | Contains information to support localisation of the UE | YES | ignore |
| Available fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 isenabled. | YES | ignore |
| Release fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 is released. | YES | ignore |
| SCG UE History Information | O |  | 9.2.177 |  | YES | ignore |
| SCG Activation Status | O |  | 9.2.178 |  | YES | ignore |
| Conditional PSCell Addition Information Modification Acknowledge | O |  |  | This IE may be sent from the target en-gNB. | YES | reject |
| >Candidate PSCell List |  | 1 |  |  | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.111 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCells for CPAC. Value is 8. |

| Condition | Explanation |
| --- | --- |
| ifMCGandSCGpresent | This IE shall be present if, for the E-RAB admitted to be added, the MCG resources and SCG resources IEs in the EN-DC Resource Configuration IE are set to the value "present". |
| ifMCGpresent | This IE shall be present if, for the E-RAB admitted to be added, the MCG resources IE in the EN-DC Resource Configuration IE is set to the value "present". |
| C-ifMCGandSCGpresent_GBRpresent | This IE shall be present if, for the E-RAB admitted to be added, the MCG resources and SCG resources IEs in the EN-DC Resource Configuration IE are set to the value "present", and the GBR QoS Information IE is present in the Requested MCG E-RAB Level QoS Parameters IE. |

#### 9.1.4.7	SGNB MODIFICATION REQUEST REJECT

This message is sent by the en-gNB to inform the MeNB that the MeNB initiated SgNB Modification Preparation has failed.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | ignore |

#### 9.1.4.8	SGNB MODIFICATION REQUIRED

This message is sent by the en-gNB to the MeNB to request the modification of en-gNB resources for a specific UE.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| PDCP Change Indication | O |  | 9.2.109 |  | YES | ignore |
| E-RABs To Be Released List |  | 0..1 |  |  | YES | ignore |
| >E-RABs To Be Released Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>Cause | M |  | 9.2.6 |  | – |  |
| >>RLC Mode | O |  | RLC Mode 9.2.119 | Indicates the RLC mode at the en-gNB for PDCP transfer to MeNB. | YES | ignore |
| SgNB to MeNB Container | O |  | OCTET STRING | Includes the CG-Config message or the CG-CandidateList message, as defined in TS 38.331 [31]. | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| E-RABs To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >E-RABs To Be Modified Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>Requested MCG E-RAB Level QoS Parameters | O |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters requested to be provided by the MCG. | – |  |
| >>>>UL Configuration | O |  | 9.2.118 | Information about UL usage in the MeNB. | – |  |
| >>>>UL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Shall be ignored by the MeNB if received. | YES | ignore |
| >>>>DL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Shall be ignored by the MeNB if received. | YES | ignore |
| >>>>SgNB UL GTP Tunnel Endpoint at PDCP | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at PDCP. For delivery of UL PDCP PDUs. | – |  |
| >>>>S1 DL GTP Tunnel Endpoint at the SgNB | O |  | GTP Tunnel Endpoint 9.2.1 | en-gNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – |  |
| >>>>New DRB ID Request | O |  | ENUMERATED (True, …) |  | YES | ignore |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>SgNB DL GTP Tunnel Endpoint at SCG | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>Secondary SgNB DL GTP Tunnel Endpoint at SCG | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs for PDCP duplication. | – |  |
| >>>>RLC Status | O |  | 9.2.131 | Indicates the RLC has been re-established.. |  |  |
| >>>>LCID | O |  | 9.2.138 | Indicate the LCID of the primary path in case of PDCP duplication | YES | ignore |
| SgNB Resource Coordination Information | O |  | 9.2.117 | Information used to coordinate resources utilisation between the en-gNB and the MeNB. | YES | ignore |
| RRC config indication | O |  | 9.2.132 | Indicates the type of RRC configuration used at the en-gNB. | YES | reject |
| Location Information at SgNB | O |  | 9.2.142 | Contains information to support localisation of the UE | YES | ignore |
| SCG UE History Information | O |  | 9.2.177 |  | YES | ignore |
| SCG Activation Request | O |  | 9.2.179 |  | YES | ignore |
| CPAC Information Required | O |  |  | This IE may be sent from the target en-gNB. | YES | ignore |
| >Candidate PSCell List |  | 1 |  | Indicates the full list of candidate PSCells prepared at the target en-gNB. | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.111 |  | – |  |
| SCG Reconfiguration Notification | O |  | ENUMERATED (executed, ...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCells for CPAC. Value is 8. |

#### 9.1.4.9	SGNB MODIFICATION CONFIRM

This message is sent by the MeNB to inform the en-gNB about the successful modification.

Direction: MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| E-RABs Admitted To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Modified Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>Secondary MeNB UL GTP Tunnel Endpoint at PDCP | O |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer at the PDCP. For delivery of UL PDCP PDUs for PDCP duplication. | – |  |
| >>>>UL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Shall be ignored by the en-gNB if received. | YES | ignore |
| >>>>DL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Shall be ignored by the en-gNB if received. | YES | ignore |
| MeNB to SgNB Container | O |  | OCTET STRING | Includes the RRCReconfigurationComplete message as defined in TS 38.331 [31]. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | ignore |
| MeNB Resource Coordination Information | O |  | 9.2.116 | Information used to coordinate resources utilisation between the MeNB and the en-gNB. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

#### 9.1.4.10	SGNB MODIFICATION REFUSE

This message is sent by the MeNB to inform the en-gNB that the SgNB initiated SgNB Modification has failed.

Direction: MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| MeNB to SgNB Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in TS 38.331 [31]. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | ignore |

#### 9.1.4.11	SGNB RELEASE REQUEST

This message is sent by the MeNB to the en-gNB to request the release of resources.

Direction: MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| E-RABs To Be Released List |  | 0..1 |  |  | YES | ignore |
| >E-RABs To Be Released Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the transport bearer used for forwarding of UL PDUs | – |  |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the transport bearer used for forwarding of DL PDUs | – |  |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| UE Context Kept Indicator | O |  | 9.2.85 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| MeNB to SgNB Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in TS 38.331 [31]. | YES | reject |
| E-RABs transferred to MeNB | O |  | E-RAB List 9.2.28 | Indicates the target MeNB reconfigured the listed E-RABs as MN-terminated bearers. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

#### 9.1.4.12	SGNB RELEASE REQUEST ACKNOWLEDGE

This message is sent by the en-gNB to the MeNB to confirme the request to release en-gNB resources.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |
| E-RABs Admitted To Be Released List |  | 0..1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Released Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>RLC Mode | M |  | RLC Mode 9.2.119 | Indicates the RLC mode at the en-gNB for PDCP transfer to MeNB. | – |  |
| SCG UE History Information | O |  | 9.2.177 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

#### 9.1.4.13	SGNB RELEASE REQUEST REJECT

This message is sent by the en-gNB to the MeNB to reject the request to release en-gNB resources.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |

#### 9.1.4.14	SGNB RELEASE REQUIRED

This message is sent by the en-gNB to request the release of all resources for a specific UE at the en-gNB.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |
| E-RABs To Be Released List |  | 0..1 |  |  | YES | ignore |
| >E-RABs To Be Released Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>RLC Mode | M |  | RLC Mode 9.2.119 | Indicates the RLC mode at the en-gNB for PDCP transfer to MeNB. | – |  |
| SgNB to MeNB Container | O |  | OCTET STRING | Includes the CG-Config message as defined in TS 38.331 [31]. | YES | ignore |
| SCG UE History Information | O |  | 9.2.177 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

#### 9.1.4.15	SGNB RELEASE CONFIRM

This message is sent by the MeNB to confirm the release of all resources for a specific UE at the en-gNB.

Direction: MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| E-RABs to be Released List |  | 0..1 |  |  | YES | ignore |
| >E-RABs To Be Released Item |  | 1 .. <maxnoofBearers> |  |  | – |  |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – |  |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

#### 9.1.4.16	SGNB COUNTER CHECK REQUEST

This message is sent by the en-gNB to request the verification of the value of the PDCP COUNTs associated with the bearers established in the en-gNB.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| E-RABs Subject to Counter Check List |  | 1 |  |  | YES | ignore |
| >E-RABs Subject to Counter Check Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | - |  |
| >>UL COUNT | M | INTEGER(0.. 4294967295) |  | Indicates the value of uplink COUNT associated to this E-RAB. | - |  |
| >>DL COUNT | M | INTEGER(0.. 4294967295) |  | Indicates the value of downlink COUNT associated to this E-RAB. | - |  |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

#### 9.1.4.17	SGNB CHANGE REQUIRED

This message is sent by the en-gNB to the MeNB to request the change of en-gNB for a specific UE.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Target SgNB ID Information | M |  | 9.2.102 | This IE shall be ignored if the Conditional PSCell Change Information Required IE is present. | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| SgNB to MeNB Container | O |  | OCTET STRING | Includes the CG-Config message as defined in TS 38.331 [31]. This IE shall be ignored if the Conditional PSCell Change Information Required IE is present. | YES | reject |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |
| SCG UE History Information | O |  | 9.2.177 |  | YES | ignore |
| Conditional PSCell Change Information Required | O |  |  |  | YES | reject |
| >Multiple Target SgNB Node List |  | 1 |  |  | – |  |
| >>Multiple Target SgNB Node Item |  | 1 .. <maxnoofTargetSgNBs> |  |  | – |  |
| >>>Target SgNB ID Information | M |  | 9.2.102 |  | – |  |
| >>>CPC Indicator | M |  | ENUMERATED (CPC-initiation, CPC-modification, CPC-cancel,...) |  | – |  |
| >>>Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the target may prepare. | – |  |
| >>>Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target en-gNB. | – |  |
| >>>SgNB to MeNB Container | M |  | OCTET STRING | Includes the CG-Config message as defined in TS 38.331 [31]. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTargetSgNBs | Maximum no. of the target SgNB nodes. Value is 8 |

#### 9.1.4.18	SGNB CHANGE CONFIRM

This message is sent by the MeNB to inform the en-gNB about the successful change.

Direction: MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| E-RABs to be Released List |  | 0..1 |  |  | YES | ignore |
| >E-RABs To Be Released Item |  | 1 .. <maxnoofBearers> |  |  | – |  |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – |  |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| >>>>Additional List of Forwarding GTP Tunnel Endpoint |  | 0..1 |  | This IE would be present only if multiple candidate target en-gNBs are prepared in case of SN initiated inter-SN CPC. | YES | ignore |
| >>>>>Additional List of Forwarding GTP Tunnel Endpoint-Item |  | 1 .. <maxnoofTargetSgNBsMinusOne> |  |  | - |  |
| >>>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs. | - |  |
| >>>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs. | - |  |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| Conditional PSCell Change Information Confirm | O |  |  |  | YES | ignore |
| >Multiple Target SgNB Node List |  | 1 |  |  | – |  |
| >>Multiple Target SgNB Node Item |  | 1 .. <maxnoofTargetSgNBs> |  |  | – |  |
| >>>Target SgNB ID | M |  | Global en-gNB ID 9.2.112 |  | – |  |
| >>>Candidate PSCell List |  | 1 |  |  | – |  |
| >>>>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>>>PSCell ID | M |  | NR CGI 9.2.111 |  | – |  |
| MeNB to SgNB Container | O |  | OCTET STRING | Includes the RRCReconfigurationComplete message as defined in subclause 6.2.2 of TS 38.331 [10]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |
| maxnoofTargetSgNBs | Maximum no. of the target SgNB nodes. Value is 8 |
| maxnoofPSCellCandidate | Maximum no. of PSCells for CPAC. Value is 8. |

#### 9.1.4.19	SGNB CHANGE REFUSE

This message is sent by the MeNB to inform the en-gNB that the SgNB initiated SgNB Change has failed.

Direction: MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |

#### 9.1.4.20	SECONDARY RAT DATA USAGE REPORT

This message is sent by the en-gNB to report data volumes for secondary RAT.

Direction: en-gNB  MeNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Secondary RAT Usage Report List | M |  | 9.2.120 |  | YES | reject |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |

#### 9.1.4.21	RRC TRANSFER

This message is sent by the MeNB to the en-gNB or by the en-gNB to the MeNB to transfer an RRC message.

Direction: MeNB  en-gNB or en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Split SRB |  | 0..1 |  |  | YES | reject |
| >RRC Container | O |  | OCTET STRING | Contains a PDCP-C PDU encapsulating an RRC message as defined in subclause 6.2.1 of TS 36.331 [9] and ciphered with the key of the MeNB | – |  |
| >SRB Type | M |  | ENUMERATED (srb1, srb2, ...) | The SRB type | – |  |
| >Delivery Status | O |  | 9.2.104 | DL RRC delivery status of split SRB | – |  |
| NR UE Report |  | 0..1 |  |  | YES | reject |
| >RRC Container | M |  | OCTET STRING | Includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [31] containing the MeasurementReport message, or the FailureInformation message, or the RRCReconfigurationComplete message, or the UEAssistanceInformation message, or the IABOtherInformation message. | – |  |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |
| Fast MCG Recovery via SRB3 from SgNB to MeNB |  | 0..1 |  |  | YES | ignore |
| >RRC Container | O |  | OCTET STRING | Includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 36.331 [9] containing MCGFailureInformation message. | – |  |
| Fast MCG Recovery via SRB3 from MeNB to SgNB |  | 0..1 |  |  | YES | ignore |
| >RRC Container | O |  | OCTET STRING | Includes the DL-DCCH-Message as defined in subclause 6.2.1 of TS 36.331 [9] containing the RRCConnectionReconfiguration message, or the RRCConnectionRelease message, or the MobilityfromEUTRACommand message. | – |  |

#### 9.1.4.22	PARTIAL RESET REQUIRED

This message is sent by an initiating node to a neighbouring node, both nodes able to interact for EN-DC, to release all the resources for selected UEs.

Direction: en-gNB  MeNB, MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| UEs to be Reset List |  | 1 |  |  | YES | reject |
| >UEs To Be Reset Item |  | 1 .. <maxnoof UEs> |  |  |  |  |
| >>MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. |  |  |
| >>SgNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. |  |  |
| >>MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. |  |  |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEs | Maximum no. of UEs. Value is 8192. |

#### 9.1.4.23	PARTIAL RESET CONFIRM

This message is sent by an initiating node to a neighbouring node, both nodes able to interact for EN-DC, to confirm the release all the resources for selected UEs.

Direction: en-gNB  MeNB, MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality |  | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES |  | reject |
| UEs Admitted to be Reset List |  | 1 |  |  |  | YES | reject |
| >UEs Admitted To Be Reset Item |  | 1 .. <maxnoof UEsinengNBDU> |  |  |  |  |  |
| >>MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. |  |  |  |
| >>SgNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. |  |  |  |
| >>MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. |  |  |  |
| Interface Instance Indication | O |  | 9.2.143 |  | YES |  | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEsinengNBDU | Maximum no. of UEs. Value is 8192. |

#### 9.1.4.24	E-UTRA – NR CELL RESOURCE COORDINATION REQUEST

Direction: eNB  en-gNB, en-gNB  eNB.

This message is sent by a neighbouring eNB to a peer en-gNB or by a neighbouring en-gNB to a peer eNB, both nodes able to interact for EN-DC, to express the desired resource allocation for data traffic, for the sake of E-UTRA - NR Cell Resource Coordination.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| CHOICE Initiating Node Type | M |  |  |  | - |  |
| >eNB |  |  |  |  |  |  |
| >>Data Traffic Resource Indication | M |  | 9.2.126 | Indicates resource allocations for data traffic. | YES | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | YES | reject |
| >>List of E-UTRA Cells in E-UTRA Coordination Request |  | 0 .. <maxCellineNB> |  | List of applicable E-UTRA cells. | YES | reject |
| >>>EUTRA Cell ID | M |  | ECGI 9.2.14 |  | - |  |
| >en-gNB |  |  |  |  |  |  |
| >>Data Traffic Resource Indication | M |  | 9.2.126 | Indicates resource allocations for data traffic. | YES | reject |
| >>List of E-UTRA Cells in NR Coordination Request |  | 1 .. <maxCellineNB> |  | List of applicable E-UTRA cells | YES | reject |
| >>>EUTRA Cell ID | M |  | ECGI 9.2.14 |  | – |  |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the NR cells involved in resource coordination with the E-UTRA cells affiliated with the same Spectrum Sharing Group ID. | YES | reject |
| >>List of NR Cells in NR Coordination Request |  | 0 .. < maxnoNRcellsSpectrumSharingwithE-UTRA > |  | List of applicable NR cells | YES | reject |
| >>>NR-Cell ID | M |  | NR-CGI 9.2.111 |  | - |  |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. of E-UTRA cells in eNB. Value is 256. |
| maxnoNRcellsSpectrumSharingwithE-UTRA | Maximum no. of NR cells affiliated to a Spectrum Sharing Group ID involved in cell resource coordination with a number of E-UTRA cells affiliated with the same Spectrum Sharing Group ID. Value is 64. |

#### 9.1.4.25	E-UTRA – NR CELL RESOURCE COORDINATION RESPONSE

This message is sent by a neighbouring eNB to a peer en-gNB or by a neighbouring en-gNB to a peer eNB, both nodes able to interact for EN-DC, as a response to the E-UTRA – NR CELL RESOURCE COORDINATION REQUEST.

Direction: eNB  en-gNB, en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| CHOICE Responding NodeType | M |  |  |  | - |  |
| >eNB |  |  |  |  |  |  |
| >>Data Traffic Resource Indication | M |  | 9.2.126 | Indicates resource allocations for data traffic. | YES | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | YES | reject |
| >>List of E-UTRA Cells in E-UTRA Coordination Response |  | 0 .. <maxCellineNB> |  | List of applicable E-UTRA cells | YES | reject |
| >>>EUTRA Cell ID | M |  | ECGI 9.2.14 |  | - |  |
| >en-gNB |  |  |  |  |  |  |
| >>Data Traffic Resource Indication | M |  | 9.2.126 | Indicates resource allocations for data traffic. | YES | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the NR cells involved in resource coordination with the E-UTRA cells affiliated with the same Spectrum Sharing Group ID. | YES | reject |
| >>List of NR Cells in NR Coordination Response |  | 0 .. < maxnoNRcellsSpectrumSharingwithE-UTRA > |  | List of applicable NR cells | YES | reject |
| >>>NR Cell ID | M |  | NR-CGI 9.2.111 |  | - |  |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. of E-UTRA cells in eNB. Value is 256. |
| maxnoNRcellsSpectrumSharingwithE-UTRA | Maximum no. of NR cells affiliated to a Spectrum Sharing Group ID involved in cell resource coordination with a number of E-UTRA cells affiliated with the same Spectrum Sharing Group ID. Value is 64. |

#### 9.1.4.26	SGNB ACTIVITY NOTIFICATION

This message is sent by the en-gNB to inform the MeNB that resources for E-RABs controlled by the en-gNB have not been used or are in use again.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| UE Context level user plane activity report | O |  | User plane traffic activity report 9.2.130 |  | YES | ignore |
| E-RAB Activity Notify Item List |  | 0..<maxnoofBearers> |  |  | EACH | ignore |
| >E-RAB ID | M |  | 9.2.23 |  | – |  |
| >User plane traffic activity report | M |  | 9.2.130 |  | – |  |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

#### 9.1.4.27	GNB STATUS INDICATION

This message is sent by the en-gNB to indicate to the eNB its status of overload.

Direction: en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| gNB Overload Information | M |  | ENUMERATED (overloaded, not-overloaded, ...) |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

#### 9.1.4.28	TRACE START

This message is sent by the MeNB to initiate a trace session for a UE.

Direction: MeNB  en-gNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Trace Activation | M |  | 9.2.2 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |

#### 9.1.4.29	DEACTIVATE TRACE

This message is sent by the MeNB to deactivate a trace session.

Direction: MeNB  en-gNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| E-UTRAN Trace ID | M |  | OCTET STRING (SIZE(8)) | As per E-UTRAN Trace ID in Trace Activation IE | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |

#### 9.1.4.30	UE Radio Capability ID Mapping Request

This message is sent by the en-gNB and is used to request the UE Radio Capability information that maps to a specific UE Radio Capability ID.

Direction: en-gNB  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| UE Radio Capability ID | M |  | 9.2.171 |  | YES | reject |

#### 9.1.4.31	UE Radio Capability ID Mapping Response

This message is sent by the eNB and is used to provide the UE Radio Capability information that maps to a specific UE Radio Capability ID indicated in the UE RADIO CAPABILITY ID MAPPING REQUEST message.

Direction: eNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| UE Radio Capability ID | M |  | 9.2.171 |  | YES | reject |
| UE Radio Capability | M |  | 9.2.173 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

#### 9.1.4.32	CONDITIONAL PSCELL CHANGE CANCEL

This message is sent by the MeNB to the source en-gNB to inform the cancellation of all of the prepared PSCells in the target en-gNB during a Conditional PSCell Change.

Direction: MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |
| Cause | O |  | 9.2.6 |  | YES | ignore |
| Target en-gNB ID Information | M |  | 9.2.102 |  | YES | reject |

### 9.1.5	Messages for IAB Procedures

#### 9.1.5.1	F1-C TRAFFIC TRANSFER

This message is sent by the en-gNB to the MeNB or by the MeNB to the en-gNB to transfer the F1-C traffic to and from an IAB-node.

Direction: MeNB  en-gNB or en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| F1-C Traffic Container | M |  | OCTET STRING | Contains an F1-C interface SCTP CHUNK and IP header, or an IP packet to protect the traffic on the F1-C interface as defined in TS 33.501 [50]. This IE corresponds to the dedicatedInfoF1c defined in TS 36.331 [9]. | YES | reject |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |

## 9.2	Information Element definitions

### 9.2.0	General

When specifying information elements which are to be represented by bit strings, if not otherwise specifically stated in the semantics description of the concerned IE or elsewhere, the following principle applies with regards to the ordering of bits:

-	The first bit (leftmost bit) contains the most significant bit (MSB);

-	The last bit (rightmost bit) contains the least significant bit (LSB);

-	When importing bit strings from other specifications, the first bit of the bit string contains the first bit of the concerned information.

### 9.2.1	GTP Tunnel Endpoint

The GTP Tunnel Endpoint IE identifies an X2 transport bearer or the S-GW endpoint of the S1 transport bearer associated to an E-RAB. It contains a Transport Layer Address and a GTP Tunnel Endpoint Identifier. The Transport Layer Address is an IP address to be used for the X2 user plane transport (see TS 36.424 [8]) or for the S1 user plane transport (see TS 36.414 [19]). The GTP Tunnel Endpoint Identifier is to be used for the user plane transport. The QoS Mapping Information is used to set the IP header of packets in case that the en-gNB serves the IAB, and the packets belonging to MN-terminated split bearer/SCG bearer are transmitted from MeNB to en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Transport Layer Address | M |  | BIT STRING (1..160, ...) | For details on the Transport Layer Address, see TS 36.424 [8], TS 36.414 [19] | – |  |
| GTP TEID | M |  | OCTET STRING (4) | For details and range, see TS 29.281 [26] | – |  |
| QoS Mapping Information | O |  | 9.2.172 |  | YES | reject |

### 9.2.2	Trace Activation

Defines parameters related to trace activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRAN Trace ID | M |  | OCTET STRING (8) | The E-UTRAN Trace ID IE is composed of the following: Trace Reference defined in TS 32.422 [6] (leftmost 6 octets, with PLMN information coded as in 9.2.4), and Trace Recording Session Reference defined in TS 32.422 [6] (last 2 octets) | – |  |
| Interfaces To Trace | M |  | BIT STRING (8) | Each position in the bitmap represents a eNB interface: first bit =S1-MME, second bit =X2, third bit =Uu, fourth bit = F1-C, fifth bit = E1. Other bits reserved for future use. Value ‘1’ indicates ‘should be traced’. Value ‘0’ indicates ‘should not be traced’. | – |  |
| Trace Depth | M |  | ENUMERATED( minimum, medium, maximum, MinimumWithoutVendorSpecificExtension, MediumWithoutVendorSpecificExtension, MaximumWithoutVendorSpecificExtension, ...) | Defined in TS 32.421 [7] | – |  |
| Trace Collection Entity IP Address | M |  | BIT STRING (1..160,…) | For File based Reporting. Defined in TS 32.422 [6]. For details on the Transport Layer Address, see TS 36.424 [8], TS 36.414 [19]. This IE is ignored if the Trace Collection Entity URI IE is present. | _ |  |
| MDT Configuration | O |  | 9.2.56 |  | YES | ignore |
| UE Application layer measurement configuration | O |  | 9.2.121 |  | YES | ignore |
| MDT Configuration NR | O |  | OCTET STRING | Defined in TS 38.413 [39]. | YES | ignore |
| Trace Collection Entity URI | O |  | URI 9.2.174 | For Streaming based Reporting. Defined in TS 32.422 [11]. Replaces Trace Collection Entity IP Address if present. | YES | ignore |

### 9.2.3	Handover Restriction List

This IE defines roaming or access restrictions for subsequent mobility action for which the eNB provides information about the target of the mobility action towards the UE, e.g., handover and CCO, or for SCG selection during dual connectivity operation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Serving PLMN | M |  | PLMN Identity 9.2.4 |  | – |  |
| Equivalent PLMNs |  | 0..<maxnoofEPLMNs> |  | Allowed PLMNs in addition to Serving PLMN. This list corresponds to the list of "equivalent PLMNs list" as defined in TS 24.301 [14]. This list is part of the roaming restriction information. Roaming restrictions apply to PLMNs other than the serving PLMN and Equivalent PLMNs. | – |  |
| >PLMN Identity | M |  | 9.2.4 |  | – |  |
| Forbidden TAs |  | 0..<maxnoofEPLMNsPlusOne> |  | intra E-UTRAN roaming restrictions | – |  |
| >PLMN Identity | M |  | 9.2.4 | The PLMN of forbidden TACs | – |  |
| >Forbidden TACs |  | 1..<maxnoofForbTACs> |  |  | – |  |
| >>TAC | M |  | OCTET STRING(2) | The forbidden TAC | – |  |
| Forbidden LAs |  | 0..<maxnoofEPLMNsPlusOne> |  | inter-3GPP RAT roaming restrictions | – |  |
| >PLMN Identity | M |  | 9.2.4 |  | – |  |
| >Forbidden LACs |  | 1..<maxnoofForbLACs> |  |  | – |  |
| >>LAC | M |  | OCTET STRING(2) |  | – |  |
| Forbidden inter RATs | O |  | ENUMERATED(ALL, GERAN, UTRAN, CDMA2000, …,GERAN and UTRAN, CDMA2000 and UTRAN) | inter-3GPP and 3GPP2 RAT access restrictions. "ALL" means that all RATs mentioned in the enumeration of this IE are restricted. | – |  |
| NR restriction in EPS as secondary RAT | O |  | ENUMERATED(NRrestrictedinEPSasSecondaryRAT, …) | Restriction to use NR when the NR is used as secondary RAT in EN-DC. | YES | ignore |
| Core Network Type Restrictions |  | 0..<maxnoofEPLMNsPlusOne> |  | Includes any of the Serving PLMN or any PLMN of the Equivalent PLMNs listed in the Mobility Restriction List IE for which core network type restriction applies as specified in TS 23.501 [38]. | YES | ignore |
| >PLMN Identity | M |  | 9.2.4 |  | – |  |
| >Core Network Type | M |  | ENUMERATED (5GCForbidden, …, EPCForbidden) | The indication indicates whether the UE is restricted to connect to 5GC or to EPC for this PLMN. | – |  |
| NR Restriction in 5GS | O |  | ENUMERATED(NRrestrictedin5GS, …) | Restriction to use NR when the NR connects to 5GS. | YES | ignore |
| Last NG-RAN PLMN Identity | O |  | 9.2.4 | Indicates the NG-RAN PLMN from where the UE formerly handed over to EPS and which is preferred in case of subsequent mobility to 5GS. | YES | ignore |
| Unlicensed Spectrum Restriction | O |  | ENUMERATED(UnlicensedRestricted, …) | Restriction to use unlicensed spectrum in the form of LAA or LWA/LWIP or NR-U as described in TS 23.401 [11]. | YES | ignore |
| RAT Restrictions |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains RAT restriction related information as specified in TS 23.401 [11]. | – |  |
| >PLMN Identity | M |  | 9.2.4 |  | – |  |
| >RAT Restriction Information | M |  | BIT STRING { LEO (0), MEO (1), GEO (2), OTHERSAT (3)} (SIZE(8, …)) | Each position in the bitmap represents a RAT. If a bit is set to "1", the respective RAT is restricted for the UE. If a bit is set to "0", the respective RAT is not restricted for the UE. Bits 4-7 reserved for future use. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofEPLMNs | Maximum no. of equivalent PLMN Ids. Value is 15. |
| maxnoofEPLMNsPlusOne | Maximum no. of equivalent PLMN Ids plus one. Value is 16. |
| maxnoofForbTACs | Maximum no. of forbidden Tracking Area Codes. Value is 4096. |
| maxnoofForbLACs | Maximum no. of forbidden Location Area Codes. Value is 4096. |

### 9.2.4	PLMN Identity

This information element indicates the PLMN Identity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | OCTET STRING (3) | - digits 0 to 9, encoded 0000 to 1001, - 1111 used as filler digit, two digits per octet, - bits 4 to 1 of octet n encoding digit 2n-1 - bits 8 to 5 of octet n encoding digit 2n -The PLMN identity consists of 3 digits from MCC followed by either -a filler digit plus 2 digits from MNC (in case of 2 digit MNC) or -3 digits from MNC (in case of a 3 digit MNC). |

### 9.2.5	DL Forwarding

This element indicates that the E-RAB is proposed for forwarding of downlink packets.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL Forwarding | M |  | ENUMERATED (DL forwarding proposed, …) |  |

### 9.2.6	Cause

The purpose of the cause information element is to indicate the reason for a particular event for the whole protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Handover Desirable for Radio Reasons, Time Critical Handover, Resource Optimisation Handover, Reduce Load in Serving Cell, Partial Handover, Unknown New eNB UE X2AP ID, Unknown Old eNB UE X2AP ID, Unknown Pair of UE X2AP ID, HO Target not Allowed, TX2RELOCoverall Expiry, TRELOCprep Expiry, Cell not Available, No Radio Resources Available in Target Cell, Invalid MME Group ID, Unknown MME Code, Encryption And/Or Integrity Protection Algorithms Not Supported, ReportCharacteristicsEmpty, NoReportPeriodicity, ExistingMeasurementID, Unknown eNB Measurement ID, Measurement Temporarily not Available, Unspecified,...,Load Balancing, Handover Optimisation, Value out of allowed range, Multiple E-RAB ID instances, Switch Off Ongoing, Not supported QCI value, Measurement not supported for the object,TDCoverall Expiry, TDCprep Expiry, Action Desirable for Radio Reasons, Reduce Load, Resource Optimisation, Time Critical action, Target not Allowed, No Radio Resources Available, Invalid QoS combination, Encryption Algorithms Not Supported, Procedure cancelled, RRM purpose, Improve user bit rate, User Inactivity, Radio Connection With UE Lost, Failure in the Radio Interface Procedure, Bearer Option not Supported, MCG Mobility, SCG Mobility, Count reaches max value, Unknown Old en-gNB UE X2AP ID, PDCP Overload, CHO-CPC resources to be changed, UE Power Saving, Insufficient UE Capabilities, Normal Release, Unknown E-UTRAN node Measurement ID, SCG activation deactivation failure, SCG deactivation failure due to data transmission, UP integrity protection not possible) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Transport Resource Unavailable, Unspecified,...) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error,Abstract Syntax Error (Reject),Abstract Syntax Error (Ignore and Notify),Message not Compatible with Receiver State,Semantic Error,Unspecified,Abstract Syntax Error (Falsely Constructed Message),...) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Hardware Failure,O&M Intervention,Not enough User Plane Processing Resources,Unspecified,...) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the concerned capability is missing. On the other hand, "not available" cause values indicate that the concerned capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Cell not Available | The concerned cell is not available. |
| Handover Desirable for Radio Reasons | The reason for requesting handover is radio related. |
| Handover Target not Allowed | Handover to the indicated target cell is not allowed for the UE in question |
| Invalid MME Group ID | The target eNB doesn’t belong to the same pool area of the source eNB i.e. S1 handovers should be attempted instead. |
| No Radio Resources Available in Target Cell | The target cell doesn’t have sufficient radio resources available. |
| Partial Handover | Provides a reason for the handover cancellation. The target eNB did not admit all E-RABs included in the HANDOVER REQUEST and the source eNB estimated service continuity for the UE would be better by not proceeding with handover towards this particular target eNB. |
| Reduce Load in Serving Cell | Load in serving cell needs to be reduced. When applied to handover preparation, it indicates the handover is triggered due to load balancing. |
| Resource Optimisation Handover | The reason for requesting handover is to improve the load distribution with the neighbour cells. |
| Time Critical Handover | Handover is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where the connection is likely to be dropped if handover is not performed. |
| TX2RELOCoverall Expiry | The reason for the action is expiry of timer TX2RELOCoverall. |
| TRELOCprep Expiry | Handover Preparation procedure is cancelled when timer TRELOCprep expires. |
| Unknown MME Code | The target eNB belongs to the same pool area of the source eNB and recognizes the MME Group ID. However, the MME Code is unknown to the target eNB. |
| Unknown New eNB UE X2AP ID | The action failed because the New eNB UE X2AP ID or the MeNB UE X2AP ID is unknown. |
| Unknown Old eNB UE X2AP ID | The action failed because the Old eNB UE X2AP ID or the SeNB UE X2AP ID is unknown. |
| Unknown Pair of UE X2AP ID | The action failed because the pair of UE X2 AP IDs is unknown. |
| Encryption And/Or Integrity Protection Algorithms Not Supported | The target eNB is unable to support any of the encryption and/or integrity protection algorithms supported by the UE, or the en-gNB is unable to support any of the NR encryption and/or integrity protection algorithms supported by the UE for EN-DC operation. |
| ReportCharacteristicsEmpty | The action failed because there is no characteristic reported. |
| NoReportPeriodicity | The action failed because the periodicity is not defined. |
| ExistingMeasurementID | The action failed because measurement-ID is already used. |
| Unknown eNB Measurement ID | The action failed because some eNB Measurement-ID is unknown. |
| Measurement Temporarily not Available | The eNB can temporarily not provide the requested measurement object. |
| Load Balancing | The reason for mobility settings change is load balancing. |
| Handover Optimisation | The reason for mobility settings change is handover optimisation. |
| Value out of allowed range | The action failed because the proposed Handover Trigger parameter change in the eNB2 Proposed Mobility Parameters IE is too low or too high. |
| Multiple E-RAB ID Instances | The action failed because multiple instances of the same E-RAB had been provided to the eNB. |
| Switch Off Ongoing | The reason for the action is an ongoing switch off i.e. the concerned cell will be switched off after offloading and not be available. It aides the receiving eNB in taking subsequent actions, e.g. selecting the target cell for subsequent handovers. |
| Not supported QCI value | The action failed because the requested QCI is not supported. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Radio Network Layer related. |
| Measurement not Supported For The Object | At least one of the concerned cell(s) does not support the requested measurement. |
| TDCoverall Expiry | The reason for the action is expiry of timer TDCoverall. |
| TDCprep Expiry | The reason for the action is expiry of timer TDCprep. |
| Action Desirable for Radio Reasons | The reason for requesting the action is radio related. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Reduce Load | Load in the cell(group) served by the requesting node needs to be reduced. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Resource Optimisation | The reason for requesting this action is to improve the load distribution with the neighbour cells. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Time Critical action | The action is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where radio resources are likely to be dropped if the requested action is not performed. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Target not Allowed | Requested action towards the indicated target cell is not allowed for the UE in question. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| No Radio Resources Available | The cell(s) in the requested node don’t have sufficient radio resources available. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Invalid QoS combination | The action was failed because of invalid QoS combination. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Encryption Algorithms Not Supported | The requested eNB is unable to support any of the encryption algorithms supported by the UE. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| RRM purpose | The procedure is initiated due to node internal RRM purposes. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Improve User Bit Rate | The reason for requesting this action is to improve the user bit rate. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| User Inactivity | The action is requested due to user inactivity on all E-RABs, e.g., S1 is requested to be released in order to optimise the radio resources; or SeNB/en-gNB didn’t see activity on the DRB recently. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Radio Connection With UE Lost | The action is requested due to losing the radio connection to the UE. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Failure in the Radio Interface Procedure | Radio interface procedure has failed. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Bearer Option not Supported | The requested bearer option is not supported by the sending node. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| MCG Mobility | The procedure is initiated due to mobility related at MCG radio resource. |
| SCG Mobility | The procedure is initiated due to mobility related at SCG radio resource. |
| Count reaches max value | Indicates the PDCP COUNT for UL or DL reached the max value and the bearer may be released. |
| Unknown Old en-gNB UE X2AP ID | The action failed because the Old en-gNB UE X2AP ID or the SgNB UE X2AP ID is unknown. |
| PDCP Overload | The procedure is initiated due to PDCP resource limitation. |
| CHO-CPC resources to be changed | The prepared resources for CHO or CPC for a UE are to be changed. |
| UE Power Saving | The procedure is initiated to accommodate the preference indicated by UE to release the SCG for UE power saving purpose. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Insufficient UE Capabilities | The procedure can’t proceed due to insufficient UE capabilities. |
| Normal Release | The release is due to normal reasons. |
| Unknown E-UTRAN node Measurement ID | The action failed because some E-UTRAN node Measurement-ID is unknown. |
| SCG activation deactivation failure | The action failed due to rejection of the SCG activation deactivation request. |
| SCG deactivation failure due to data transmission | The SCG deactivation failure due to ongoing or arriving data transmission. |
| UP integrity protection not possible | The E-RAB can’t be accepted according to the required user plane integrity protection policy. |

| Transport Network Layer cause | Meaning |
| --- | --- |
| Transport resource unavailable | The required transport resources are not available. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Transport Network Layer related |

| Protocol cause | Meaning |
| --- | --- |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerned criticality indicated "reject" (see sub clause 10.3 of TS 36.413 [4]). |
| Abstract Syntax Error (Ignore and Notify) | The received message included an abstract syntax error and the concerned criticality indicated "ignore and notify" (see sub clause 10.3 of TS 36.413 [4]). |
| Abstract syntax error (falsely constructed message) | The received message contained IEs or IE groups in wrong order or with too many occurrences (see sub clause 10.3 of TS 36.413 [4]). |
| Message not Compatible with Receiver State | The received message was not compatible with the receiver state (see sub clause 10.4 of TS 36.413 [4]). |
| Semantic Error | The received message included a semantic error (see sub clause 10.4 of TS 36.413 [4]). |
| Transfer Syntax Error | The received message included a transfer syntax error (see sub clause 10.2 of TS 36.413 [4]). |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related |

| Miscellaneous cause | Meaning |
| --- | --- |
| Control Processing Overload | eNB control processing overload |
| Hardware Failure | eNB hardware failure |
| Not enough User Plane Processing Resources | eNB has insufficient user plane processing resources available. |
| O&M Intervention | Operation and Maintenance intervention related to eNB equipment |
| Unspecified | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer or Protocol |

### 9.2.7	Criticality Diagnostics

The Criticality Diagnostics IE is sent by the eNB/en-gNB when parts of a received message have not been comprehended or were missing, or if the message contained logical errors. When applicable, it contains information about which IEs were not comprehended or were missing.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Procedure Code | O |  | INTEGER (0..255) | Procedure Code is to be used if Criticality Diagnostics is part of Error Indication procedure, and not within the response message of the same procedure that caused the error. |
| Triggering Message | O |  | ENUMERATED(initiating message, successful outcome, unsuccessful outcome) | The Triggering Message is used only if the Criticality Diagnostics is part of Error Indication procedure. |
| Procedure Criticality | O |  | ENUMERATED(reject, ignore, notify) | This Procedure Criticality is used for reporting the Criticality of the Triggering message (Procedure). |
| Information Element Criticality Diagnostics |  | 0..<maxNrOfErrors> |  |  |
| >IE Criticality | M |  | ENUMERATED(reject, ignore, notify) | The IE Criticality is used for reporting the criticality of the triggering IE. The value "ignore'" shall not be used. |
| >IE ID | M |  | INTEGER (0..65535) | The IE ID of the not understood or missing IE |
| >Type Of Error | M |  | ENUMERATED(not understood, missing, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxNrOfErrors | Maximum no. of IE errors allowed to be reported with a single message. The value for maxnooferrors is 256. |

### 9.2.8	Served Cell Information

This IE contains cell configuration information of a cell that a neighbour eNB may need for the X2 AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PCI | M |  | INTEGER (0..503, …) | Physical Cell ID | – |  |
| Cell ID | M |  | ECGI 9.2.14 |  | – |  |
| TAC | M |  | OCTET STRING(2) | Tracking Area Code | – |  |
| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB1 associated to the E-UTRA Cell Identity in the Cell ID IE. | – |  |
| >PLMN Identity | M |  | 9.2.4 |  | – |  |
| CHOICE EUTRA-Mode-Info | M |  |  |  | – |  |
| >FDD |  |  |  |  |  |  |
| >>FDD Info |  | 1 |  |  | – |  |
| >>>UL EARFCN | M |  | EARFCN 9.2.26 | Corresponds to NUL in TS 36.104 [16] for E-UTRA operating bands for which it is defined; ignored for E-UTRA operating bands for which NUL is not defined | – |  |
| >>>DL EARFCN | M |  | EARFCN 9.2.26 | Corresponds to NDL in TS 36.104 [16] | – |  |
| >>>UL Transmission Bandwidth | M |  | Transmission Bandwidth 9.2.27 | Same as DL Transmission Bandwidth in this release; ignored in case UL EARFCN value is ignored | – |  |
| >>>DL Transmission Bandwidth | M |  | Transmission Bandwidth 9.2.27 |  | – |  |
| >>>UL EARFCN Extension | O |  | EARFCN Extension 9.2.65 | If this IE is present, the value signalled in the UL EARFCN IE is ignored. | YES | reject |
| >>>DL EARFCN Extension | O |  | EARFCN Extension 9.2.65 | If this IE is present, the value signalled in the DL EARFCN IE is ignored. | YES | reject |
| >>>Offset of NB-IoT Channel Number to DL EARFCN | O |  | Offset of NB-IoT Channel Number to EARFCN 9.2.94 | Corresponds to MDL in TS 36.104 [16] | YES | reject |
| >>>Offset of NB-IoT Channel Number to UL EARFCN | O |  | Offset of NB-IoT Channel Number to EARFCN 9.2.94 | Corresponds to MUL in TS 36.104 [16] | YES | reject |
| >>>NRS-NSSS-PowerOffset | O |  | ENUMERATED (-3, 0, 3, …) | NRS to NSSS power ratio, as defined in TS6.213 [11]. | YES | Ignore |
| >>>NSSS-NumOccasionDifferentPrecoder | O |  | ENUMERATED (2, 4, 8, …) | The number of consecutive NSSS occasions that use different precoders for NSSS transmission, as defined in TS6.213 [11]. | YES | ignore |
| >TDD |  |  |  |  | – |  |
| >>TDD Info |  | 1 |  |  | – |  |
| >>>EARFCN | M |  | 9.2.26 | Corresponds to NDL/NUL in TS 36.104 [16] | – |  |
| >>>Transmission Bandwidth | M |  | Transmission Bandwidth 9.2.27 |  | – |  |
| >>>Subframe Assignment | M |  | ENUMERATED(sa0, sa1, sa2, sa3, sa4, sa5, sa6,…) | Uplink-downlink subframe configuration information defined in TS 36.211 [10]. In NB-IOT, sa0 and sa6 are not applicable. | – |  |
| >>>Special Subframe Info |  | 1 |  | Special subframe configuration information defined in TS 36.211 [10] | – |  |
| >>>>Special Subframe Patterns | M |  | ENUMERATED(ssp0, ssp1, ssp2, ssp3, ssp4, ssp5, ssp6, ssp7, ssp8, …) |  | – |  |
| >>>>Cyclic Prefix DL | M |  | ENUMERATED(Normal, Extended,…) |  | – |  |
| >>>>Cyclic Prefix UL | M |  | ENUMERATED(Normal, Extended,…) |  | – |  |
| >>>Additional Special Subframe Info | O |  |  | Special subframe configuration information defined in TS 36.211 [10]. Only for newly defined configuration of special subframe from Release 11. | YES | ignore |
| >>>>Additional Special Subframe Patterns | M |  | ENUMERATED(ssp0, ssp1, ssp2, ssp3, ssp4, ssp5, ssp6, ssp7, ssp8, ssp9, …) |  | – |  |
| >>>>Cyclic Prefix DL | M |  | ENUMERATED(Normal, Extended,…) |  | – |  |
| >>>>Cyclic Prefix UL | M |  | ENUMERATED(Normal, Extended,…) |  | – |  |
| >>>EARFCN Extension | O |  | 9.2.65 | If this IE is present, the value signalled in the EARFCN IE is ignored. | YES | reject |
| >>>Additional Special Subframe Extension Info | O |  |  | Special subframe configuration information defined in TS 36.211 [10]. Only for newly defined configuration of special subframe from Release 14. | YES | ignore |
| >>>>Additional Special Subframe Patterns Extension | M |  | ENUMERATED(ssp10, …) |  | – |  |
| >>>>Cyclic Prefix DL | M |  | ENUMERATED(Normal, Extended,…) |  | – |  |
| >>>>Cyclic Prefix UL | M |  | ENUMERATED(Normal, Extended,…) |  | – |  |
| >>>Offset of NB-IoT Channel Number to DL EARFCN | O |  | Offset of NB-IoT Channel Number to EARFCN 9.2.94 | Corresponds to MDL in TS 36.104 [16] | YES | reject |
| >>>NB-IoT UL DL Alignment Offset | O |  | NB-IoT UL DL Alignment Offset 9.2.144 |  | YES | reject |
| Number of Antenna Ports | O |  | 9.2.43 |  | YES | ignore |
| PRACH Configuration | O |  | PRACH Configuration 9.2.50 |  | YES | ignore |
| MBSFN Subframe Info |  | 0..<maxnoofMBSFN> |  | Contains information provided in the MBSFN-SubframeConfig IE as defined in TS 36.331 [9] | GLOBAL | ignore |
| >Radioframe Allocation Period | M |  | ENUMERATED(n1, n2, n4, n8, n16, n32, …) |  | – |  |
| >Radioframe Allocation Offset | M |  | INTEGER (0..7, ...) |  | – |  |
| >Subframe Allocation | M |  | 9.2.51 |  | – |  |
| CSG ID | O |  | 9.2.53 |  | YES | ignore |
| MBMS Service Area Identity List |  | 0..<maxnoofMBMSServiceAreaIdentities > |  | Supported MBMS Service Area Identities in the cell | GLOBAL | ignore |
| >MBMS Service Area Identity |  |  | OCTET STRING(2) | MBMS Service Area Identities as defined in TS 23.003 [29] |  |  |
| MultibandInfoList | O |  | 9.2.60 |  | YES | ignore |
| FreqBandIndicatorPriority | O |  | ENUMERATED (not-broadcasted, broadcasted, ...) | This IE indicates that the eNodeB supports FreqBandIndicationPriority, and whether FreqBandIndicatorPriority is broadcasted in SIB 1 (see TS 36.331 [9]) | YES | ignore |
| BandwidthReducedSI | O |  | ENUMERATED (scheduled, ...) | This IE indicates that the SystemInformationBlockType1-BR is scheduled in the cell (see TS 36.331 [9]) | YES | ignore |
| Protected E-UTRA Resource Indication | O |  | 9.2.125 | This IE indicates which E-UTRA control/reference signal resources are protected and are not subject to E-UTRA - NR Cell Resource Coordination. | YES | ignore |
| Broadcast PLMN Identity Info List E-UTRA |  | 0..<maxnoofBPLMNs> |  | This IE corresponds to information provided in the cellAccessRelatedInfo in the SystemInformationBlockType1 message as specified in TS 36.331 [9]. All PLMN Identities and associated information contained in the cellAccessRelatedInfo IE are included and provided in the same order as broadcast in the SystemInformationBlockType1 message. | YES | ignore |
| >Broadcast PLMNs |  | 1..<maxnoof BPLMNs> |  | Broadcast PLMN IDs in the SystemInformationBlockType1 message (SIB1) associated to the E-UTRA Cell Identity IE. | – |  |
| >>PLMN Identity | M |  | 9.2.4 |  | – |  |
| >TAC | M |  | OCTET STRING(2) |  | – |  |
| >E-UTRA Cell Identity | M |  | BIT STRING (28) |  | – |  |
| NPRACH Configuration | O |  | NPRACH Configuration 9.2.170 |  | YES | ignore |
| SFN Offset | O |  | 9.2.175 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofMBSFN | Maximum no. of MBSFN frame allocation with different offset. Value is 8. |
| maxnoofMBMSServiceAreaIdentities | Maximum no. of MBMS Service Area Identities. Value is 256. |

### 9.2.9	E-RAB Level QoS Parameters

This IE defines the QoS to be applied to an E-RAB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QCI | M |  | INTEGER (0..255) | QoS Class Identifier defined in TS 23.401 [12]. Logical range and coding specified in TS 23.203 [13]. | – |  |
| Allocation and Retention Priority | M |  | 9.2.31 |  | – |  |
| GBR QoS Information | O |  | 9.2.10 | This IE applies to GBR bearers only and shall be ignored otherwise. | – |  |
| Downlink Maximum Packet Loss Rate | O |  | Packet Loss Rate 9.2.124 | This IE applies only to bearers with specific QCI (see TS 23.401 [12]) and indicates the maximum allowed packet loss rate for downlink as specified in TS 23.401 [12]. | YES | ignore |
| Uplink Maximum Packet Loss Rate | O |  | Packet Loss Rate 9.2.124 | This IE applies only to bearers with specific QCI (see TS 23.401 [12]) and indicates the maximum allowed packet loss rate for uplink as specified in TS 23.401 [12]. | YES | ignore |

### 9.2.10	GBR QoS Information

This IE indicates the maximum and guaranteed bit rates of a GBR E-RAB for downlink and uplink.

NOTE:	For LTE DC, the SeNB regards the GBR QoS Information IE as an E-RAB level parameter also for E-RABs configured with the split bearer option, although for the split bearer option the bitrates signalled by the MeNB are typically not equal to the bitrates signalled by the MME for that E-RAB (see TS 36.300 [15]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-RAB Maximum Bit Rate Downlink | M |  | Bit Rate 9.2.11 | Maximum Bit Rate in DL (i.e. from EPC to E-UTRAN) for the bearer. Details in TS 23.401 [12]. If the Extended E-RAB Maximum Bit Rate Downlink IE is included, the E-RAB Maximum Bit Rate Downlink IE shall be ignored. | – |  |
| E-RAB Maximum Bit Rate Uplink | M |  | Bit Rate 9.2.11 | Maximum Bit Rate in UL (i.e. from E-UTRAN to EPC) for the bearer. Details in TS 23.401 [12]. If the Extended E-RAB Maximum Bit Rate Uplink IE is included, the E-RAB Maximum Bit Rate Uplink IE shall be ignored. | – |  |
| E-RAB Guaranteed Bit Rate Downlink | M |  | Bit Rate 9.2.11 | Guaranteed Bit Rate (provided that there is data to deliver) in DL (i.e. from EPC to E-UTRAN) for the bearer. Details in TS 23.401 [12]. If the Extended E-RAB Guaranteed Bit Rate Downlink IE is included, the E-RAB Guaranteed Bit Rate Downlink IE shall be ignored. | – |  |
| E-RAB Guaranteed Bit Rate Uplink | M |  | Bit Rate 9.2.11 | Guaranteed Bit Rate (provided that there is data to deliver) in UL (i.e. from E-UTRAN to EPC) for the bearer. Details in TS 23.401 [12]. If the Extended E-RAB Guaranteed Bit Rate Uplink IE is included, the E-RAB Guaranteed Bit Rate Uplink IE shall be ignored. | – |  |
| Extended E-RAB Maximum Bit Rate Downlink | O |  | Extended Bit Rate 9.2.99 | Maximum Bit Rate in DL (i.e. from EPC to E-UTRAN) for the bearer. Details in TS 23.401 [12]. | – |  |
| Extended E-RAB Maximum Bit Rate Uplink | O |  | Extended Bit Rate 9.2.99 | Maximum Bit Rate in UL (i.e. from E-UTRAN to EPC) for the bearer. Details in TS 23.401 [12]. | – |  |
| Extended E-RAB Guaranteed Bit Rate Downlink | O |  | Extended Bit Rate 9.2.99 | Guaranteed Bit Rate (provided that there is data to deliver) in DL (i.e. from EPC to E-UTRAN) for the bearer. Details in TS 23.401 [12]. | – |  |
| Extended E-RAB Guaranteed Bit Rate Uplink | O |  | Extended Bit Rate 9.2.99 | Guaranteed Bit Rate (provided that there is data to deliver) in UL (i.e. from E-UTRAN to EPC) for the bearer. Details in TS 23.401 [12]. | – |  |

### 9.2.11	Bit Rate

This IE indicates the number of bits delivered by E-UTRAN in UL or to E-UTRAN in DL or by UE in sidelink within a period of time, divided by the duration of the period. It is used, for example, to indicate the maximum or guaranteed bit rate for a GBR E-RAB, or an aggregated maximum bit rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bit Rate | M |  | INTEGER (0..10,000,000,000) | The unit is: bit/s |

### 9.2.12	UE Aggregate Maximum Bit Rate

On Handover Aggregate Maximum Bitrate is transferred to the target eNB. In Dual Connectivity, UE Aggregate Maximum Bit Rate is split into MeNB UE Aggregate Maximum Bit Rate and SeNB UE Aggregate Maximum Bit Rate which are enforced by MeNB and SeNB respectively as specified in TS 36.300 [15]. The UE Aggregate Maximum Bitrate is applicable for all Non-GBR bearers per UE which is defined for the Downlink and the Uplink direction and provided by the MME to the eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UE Aggregate Maximum Bit Rate Downlink | M |  | Bit Rate 9.2.11 | If the Extended UE Aggregate Maximum Bit Rate Downlink IE is included, the UE Aggregate Maximum Bit Rate Downlink IE shall be ignored. | – |  |
| UE Aggregate Maximum Bit Rate Uplink | M |  | Bit Rate 9.2.11 | If the Extended UE Aggregate Maximum Bit Rate Uplink IE is included, the UE Aggregate Maximum Bit Rate Uplink IE shall be ignored. | – |  |
| Extended UE Aggregate Maximum Bit Rate Downlink | O |  | Extended Bit Rate 9.2.99 | UE Aggregate Maximum Bit Rate in DL. Details in TS 23.401 [12]. | – |  |
| Extended UE Aggregate Maximum Bit Rate Uplink | O |  | Extended Bit Rate 9.2.99 | UE Aggregate Maximum Bit Rate in UL. Details in TS 23.401 [12]. | – |  |

### 9.2.13	Message Type

The Message Type IE uniquely identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Procedure Code | M |  | INTEGER (0..255) |  |
| Type of Message | M |  | CHOICE (Initiating Message, Successful Outcome, Unsuccessful Outcome, …) |  |

### 9.2.14	ECGI

The E-UTRAN Cell Global Identifier (ECGI) is used to globally identify a cell (see TS 36.401 [2]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.4 |  | – |  |
| E-UTRAN Cell Identifier | M |  | BIT STRING (28) | The leftmost bits of the E-UTRAN Cell Identifier IE value correspond to the value of the eNB ID IE contained in the Global eNB ID IE (defined in section 9.2.22) identifying the eNB that controls the cell. | – |  |

### 9.2.15	COUNT Value

This information element indicates the 12 bit PDCP sequence number and the corresponding 20 bit Hyper frame number.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP-SN | M |  | INTEGER (0..4095) |  | – |  |
| HFN | M |  | INTEGER (0..1048575) |  | – |  |

### 9.2.16	GUMMEI

This information element indicates the globally unique MME identity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| GU Group Id | M |  | 9.2.20 |  | – |  |
| MME code | M |  | OCTET STRING (1) |  | – |  |

### 9.2.17	UL Interference Overload Indication

This IE provides, per PRB, a report on interference overload. The interaction between the indication of UL Interference Overload and UL High Interference is implementation specific.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UL Interference Overload Indication List |  | 1..<maxnoofPRBs> |  |  |
| >UL Interference Overload Indication | M |  | ENUMERATED (high interference, medium interference, low interference, …) | Each PRB is identified by its position in the list: the first element in the list corresponds to PRB 0, the second to PRB 1, etc. |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRBs | Maximum no. Physical Resource Blocks. Value is 110. |

### 9.2.18	UL High Interference Indication

This IE provides, per PRB, a 2 level report on interference sensitivity. The interaction between the indication of UL Overload and UL High Interference is implementation specific.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| HII | M |  | BIT STRING (1..110, …) | Each position in the bitmap represents a PRB (first bit=PRB 0 and so on), for which value ‘"1" indicates ‘high interference sensitivity’ and value "0" indicates ’low interference sensitivity’. The maximum number of Physical Resource Blocks is 110. |

### 9.2.19	Relative Narrowband Tx Power (RNTP)

This IE provides an indication on DL power restriction per PRB or per subframe per PRB (Enhanced RNTP) in a cell and other information needed by a neighbour eNB for interference aware scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RNTP Per PRB | M |  | BIT STRING (6..110, …) | Each position in the bitmap represents a nPRB value (i.e. first bit=PRB 0 and so on), for which the bit value represents RNTP (nPRB), defined in TS 36.213 [11]. Value 0 indicates "Tx not exceeding RNTP threshold". Value 1 indicates "no promise on the Tx power is given". The IE is ignored if the Enhanced RNTP IE is included. | – |  |
| RNTP Threshold | M |  | ENUMERATED (-∞, -11, -10, -9, -8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, …) | RNTPthreshold is defined in TS 36.213 [11]. | – |  |
| Number Of Cell-specific Antenna Ports | M |  | ENUMERATED (1, 2, 4, …) | P (number of antenna ports for cell-specific reference signals) defined in TS 36.211 [10] | – |  |
| P_B | M |  | INTEGER (0..3, …) | PB is defined in TS 36.213 [11]. | – |  |
| PDCCH Interference Impact | M |  | INTEGER (0..4, …) | Measured by Predicted Number Of Occupied PDCCH OFDM Symbols (see TS 36.211 [10]). Value 0 means "no prediction is available". | – |  |
| Enhanced RNTP | O |  |  |  | YES | ignore |
| >Enhanced RNTP Bitmap | M |  | BIT STRING (12..8800, …) | Each position in the bitmap represents a PRB in a subframe; value "00" indicates "Tx not exceeding RNTP Threshold", value "01" indicates "Tx not exceeding RNTP High Power Threshold", value "11" indicates that "no promise on the Tx power is given". Value "10" is ignored by the receiver". Each position is applicable only in positions corresponding to DL subframes. The first 2 bits correspond to PRB 0 of the first subframe for which the IE is valid, the following 2 bits correspond to PRB 1 of the first subframe for which the IE is valid, and so on. The bit string may span across multiple contiguous subframes (maximum 40). The length of the bit string is an integer multiple of . is defined in TS 36.211 [10]. The Enhanced RNTP pattern is continuously repeated. |  |  |
| >RNTP High Power Threshold | M |  | ENUMERATED (-∞, -11, -10, -9, -8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, …) | Defined as the RNTPthreshold in TS 36.213 [11]. |  |  |
| >Enhanced RNTP Start Time |  | 0..1 |  |  |  |  |
| >>Start SFN | M |  | INTEGER (0..1023, …) | SFN of the radio frame containing the first subframe when the Enhanced RNTP IE is valid. |  |  |
| >>Start Subframe Number | M |  | INTEGER (0..9, …) | Subframe number, within the radio frame indicated by the Start SFN IE, of the first subframe when the Enhanced RNTP IE is valid. |  |  |

### 9.2.20	GU Group Id

The GU Group Id IE is the globally unique group id corresponding to a pool area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PLMN Id | M |  | PLMN Identity 9.2.4 |  | – |  |
| MME Group Id | M |  | OCTET STRING(2) |  | – |  |

### 9.2.21	Location Reporting Information

This information element indicates how the location information should be reported.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Event | M |  | ENUMERATED (Change of serving cell, …) |  | – |  |
| Report Area | M |  | ENUMERATED (ECGI, …) |  | – |  |
| Additional Location Information | O |  | ENUMERATED (Include PSCell, ...) |  | YES | ignore |

### 9.2.22	Global eNB ID

This IE is used to globally identify an eNB (see TS 36.401 [2]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.4 |  | – |  |
| CHOICE eNB ID | M |  |  |  | – |  |
| >Macro eNB ID | M |  | BIT STRING (20) | Equal to the 20 leftmost bits of the value of the E-UTRAN Cell Identifier IE contained in the ECGI IE (see section 9.2.14) identifying each cell controlled by the eNB | – |  |
| >Home eNB ID | M |  | BIT STRING (28) | Equal to the value of the E-UTRAN Cell Identifier IE contained in the ECGI IE (see section 9.2.14) identifying the cell controlled by the eNB | – |  |
| >Short Macro eNB ID | M |  | BIT STRING (SIZE(18)) | Equal to the 18 leftmost bits of the value of the E-UTRAN Cell Identifier IE contained in the ECGI IE (see section 9.2.14) identifying each cell controlled by the eNB. | – |  |
| >Long Macro eNB ID | M |  | BIT STRING (SIZE(21)) | Equal to the 21 leftmost bits of the value of the E-UTRAN Cell Identifier IE contained in the ECGI IE (see section 9.2.14) identifying each cell controlled by the eNB. | – |  |

### 9.2.23	E-RAB ID

This IE uniquely identifies an E-RAB for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E-RAB ID | M |  | INTEGER (0..15, ...) |  |

### 9.2.24	eNB UE X2AP ID

This information element, combined with the eNB UE X2AP ID Extension when present regardless its value, uniquely identifies an UE over the X2 interface within an eNB.

The usage of this IE is defined in TS 36.401 [2].

NOTE: 	If X2-C signalling transport is shared among multiple interface instances, the value of the eNB UE X2AP ID, combined with the eNB UE X2AP ID Extension, if applicable, is allocated so that it can be associated with an X2-C interface instance.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| eNB UE X2AP ID | M |  | INTEGER (0..4095) |  |

### 9.2.25	Subscriber Profile ID for RAT/Frequency priority

The Subscriber Profile ID IE for RAT/Frequency Selection Priority is used to define camp priorities in Idle mode and to control inter-RAT/inter-frequency handover in Active mode (TS 36.300 [15]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subscriber Profile ID for RAT/Frequency Priority | M |  | INTEGER (1..256) |  |

### 9.2.25a	Additional RRM Policy Index

The Additional RRM Policy Index IE is used to provide additional information independent from the Subscriber Profile ID for RAT/Frequency priority as specified in TS 36.300 [15].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Additional RRM Policy Index | M |  | BIT STRING (32) |  |

### 9.2.26	EARFCN

The E-UTRA Absolute Radio Frequency Channel Number defines the carrier frequency used in a cell for a given direction (UL or DL) in FDD or for both UL and DL directions in TDD.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| EARFCN | M |  | INTEGER (0..maxEARFCN) | The relation between EARFCN and carrier frequency (in MHz) are defined in TS 36.104 [16]. |

| Range bound | Explanation |
| --- | --- |
| maxEARFCN | Maximum value of EARFCNs. Value is 65535. |

### 9.2.27	Transmission Bandwidth

The Transmission Bandwidth IE is used to indicate the UL or DL transmission bandwidth expressed in units of resource blocks " NRB " (TS 36.104 [16]). The values bw1, bw6, bw15, bw25, bw50, bw75, bw100 correspond to the number of resource blocks "NRB" 6, 15, 25, 50, 75, 100.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Transmission Bandwidth | M |  | ENUMERATED (bw6, bw15, bw25, bw50, bw75, bw100,... , bw1) |  |

### 9.2.28	E-RAB List

The IE contains a list of E-RAB identities with a cause value. It is used for example to indicate not admitted bearers.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-RAB List Item |  | 1..<maxnoofBearers> |  |  | EACH | ignore |
| >E-RAB ID | M |  | 9.2.23 |  | – |  |
| >Cause | M |  | 9.2.6 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256. |

### 9.2.29	UE Security Capabilities

The UE Security Capabilities IE defines the supported algorithms for encryption and integrity protection in the UE. The eNBs store and send the complete bitmaps without modification or truncation as specified in TS 36.300 [15].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Encryption Algorithms | M |  | BIT STRING (16, ...) | Each position in the bitmap represents an encryption algorithm: "all bits equal to 0" - UE supports no other algorithm than EEA0 "first bit" - 128-EEA1, "second bit" - 128-EEA2, "third bit" - 128-EEA3, “fourth to seventh bit” correspond to bit 4 to bit 1 of octet 3 in the UE Security Capability IE defined in TS 24.301 [14], other bits reserved for future use. Value ‘1’ indicates support and value "0" indicates no support of the algorithm. Algorithms are defined in TS 33.401 [18]. |
| Integrity Protection Algorithms | M |  | BIT STRING (16, ...) | Each position in the bitmap represents an integrity protection algorithm: all bits equal to 0" - UE supports no other algorithm than EIA0 (TS 33.401 [18]) "first bit" - 128-EIA1, "second bit" - 128-EIA2, "third bit" - 128-EIA3, “fourth to seventh bit” correspond to bit 4 to bit 1 of octet 4 in the UE Security Capability IE defined in TS 24.301 [14], other bits reserved for future use. Value '1' indicates support and value "0" indicates no support of the algorithm. Algorithms are defined in TS 33.401 [18]. |

### 9.2.30	AS Security Information

The AS Security Information IE is used to generate the key material to be used for AS security with the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Key eNodeB Star | M |  | BIT STRING (256) | KeNB* defined in TS 33.401 [18]. If the target cell belongs to multiple frequency bands, the source eNB selects the DL-EARFCN for KeNB* calculation as specified in section 10.3 of TS 36.331 [9]. |
| Next Hop Chaining Count | M |  | INTEGER (0..7) | Next Hop Chaining Count (NCC) defined in TS 33.401 [18] |

### 9.2.31	Allocation and Retention Priority

This IE specifies the relative importance compared to other E-RABs for allocation and retention of the E-UTRAN Radio Access Bearer.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Priority Level | M |  | INTEGER (0..15) | Desc.: This IE should be understood as "priority of allocation and retention" (see TS 23.401 [12]). Usage: Value 15 means "no priority". Values between 1 and 14 are ordered in decreasing order of priority, i.e. 1 is the highest and 14 the lowest. Value 0 shall be treated as a logical error if received. |
| Pre-emption Capability | M |  | ENUMERATED(shall not trigger pre-emption, may trigger pre-emption) | Descr.: This IE indicates the pre-emption capability of the request on other E-RABs Usage: The E-RAB shall not pre-empt other E-RABs or, the E-RAB may pre-empt other E-RABs The Pre-emption Capability indicator applies to the allocation of resources for an E-RAB and as such it provides the trigger to the pre-emption procedures/processes of the eNB. |
| Pre-emption Vulnerability | M |  | ENUMERATED(not pre-emptable, pre-emptable) | Desc.: This IE indicates the vulnerability of the E-RAB to pre-emption of other E-RABs. Usage: The E-RAB shall not be pre-empted by other E-RABs or the E-RAB may be pre-empted by other RABs. Pre-emption Vulnerability indicator applies for the entire duration of the E-RAB, unless modified, and as such indicates whether the E-RAB is a target of the pre-emption procedures/processes of the eNB. |

### 9.2.32	Time To Wait

This IE defines the minimum allowed waiting times.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Time To Wait | M |  | ENUMERATED(1s, 2s, 5s, 10s, 20s, 60s, …) |  |

### 9.2.33	SRVCC Operation Possible

The IE indicates that both the UE and the MME are SRVCC-capable. E-UTRAN behaviour on reception of this is specified in TS 23.216 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRVCC Operation Possible | M |  | ENUMERATED(Possible, …) |  |

### 9.2.34	Hardware Load Indicator

The Hardware Load Indicator IE indicates the status of the Hardware Load experienced by the cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL Hardware Load Indicator | M |  | Load Indicator 9.2.36 |  |
| UL Hardware Load Indicator | M |  | Load Indicator 9.2.36 |  |

### 9.2.35	S1 TNL Load Indicator

The S1 TNL Load Indicator IE indicates the status of the S1 Transport Network Load experienced by the cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL S1TNL Load Indicator | M |  | Load Indicator 9.2.36 |  |
| UL S1TNL Load Indicator | M |  | Load Indicator 9.2.36 |  |

### 9.2.36	Load Indicator

The Load Indicator IE indicates the status of Load.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Load Indicator | M |  | ENUMERATED (LowLoad, MediumLoad, HighLoad, Overload, ...) |  |

### 9.2.37	Radio Resource Status

The Radio Resource Status IE indicates the usage of the PRBs for all traffic in Downlink and Uplink (TS 36.314 [22], TS 23.203 [13]) and the usage of PDCCH CCEs for Downlink and Uplink scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL GBR PRB usage | M |  | INTEGER (0..100) |  |
| UL GBR PRB usage | M |  | INTEGER (0..100) |  |
| DL non-GBR PRB usage | M |  | INTEGER (0..100) |  |
| UL non-GBR PRB usage | M |  | INTEGER (0..100) |  |
| DL Total PRB usage | M |  | INTEGER (0..100) |  |
| UL Total PRB usage | M |  | INTEGER (0..100) |  |
| DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |

### 9.2.38	UE History Information

The UE History Information IE contains information about cells that a UE has been served by in active state prior to the target cell. The overall mechanism is described in TS 36.300 [15].

NOTE:	The definition of this IE is aligned with the definition of the UE History Information IE in TS 36.413 [4].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Last Visited Cell List |  | 1..<maxnoofCells> |  | Most recent information is added to the top of this list | – |  |
| >Last Visited Cell Information | M |  | 9.2.39 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCells | Maximum number of last visited cell information records that can be reported in the IE. Value is 16. |

### 9.2.39	Last Visited Cell Information

The Last Visited Cell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Last Visited Cell Information | M |  |  |  | - |  |
| >E-UTRAN Cell |  |  |  |  | - |  |
| >>Last Visited E-UTRAN Cell Information | M |  | 9.2.40 |  | - |  |
| >UTRAN Cell |  |  |  |  | - |  |
| >>Last Visited UTRAN Cell Information | M |  | OCTET STRING | Defined in TS 25.413 [24] |  |  |
| >GERAN Cell |  |  |  |  | - |  |
| >>Last Visited GERAN Cell Information | M |  | 9.2.41 |  | - |  |
| >NG-RAN Cell |  |  |  |  | - |  |
| >>Last Visited NG-RAN Cell Information | M |  | OCTET STRING | Defined in TS 38.413 [39]. (see subclause 9.3.1.97). |  |  |

### 9.2.40	Last Visited E-UTRAN Cell Information

The Last Visited E-UTRAN Cell Information contains information about a cell that is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | ECGI 9.2.14 |  | - |  |
| Cell Type | M |  | 9.2.42 |  | - |  |
| Time UE stayed in Cell | M |  | INTEGER (0..4095) | The duration of the time the UE stayed in the cell in seconds. If the UE stays in a cell more than 4095s, this IE is set to 4095. | - |  |
| Time UE stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. | YES | ignore |
| HO Cause Value | O |  | Cause 9.2.6 | The cause for the handover from the E-UTRAN cell. | YES | ignore |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | OCTET STRING | The PSCell related information. Defined in TS 36.413 [4]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited PScell information records that can be reported in the IE. Value is 8. |

### 9.2.41	Last Visited GERAN Cell Information

The Last Visited Cell Information for GERAN is currently undefined.

NOTE:	If in later Releases this is defined, the choice type may be extended with the actual GERAN specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Last Visited GERAN Cell Information | M |  |  |  | - |  |
| >Undefined | M |  | NULL |  | - |  |

### 9.2.42	Cell Type

The cell type provides the cell coverage area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell Size | M |  | ENUMERATED (verysmall, small, medium, large, …) |  | - |  |

### 9.2.43	Number of Antenna Ports

The Number of Antenna Ports IE is used to indicate the number of cell specific antenna ports.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Number of Antenna Ports |  |  | ENUMERATED (an1, an2, an4,...) | an1 = One antenna port an2 = Two antenna ports an4 = Four antenna ports |

### 9.2.44	Composite Available Capacity Group

The Composite Available Capacity Group IE indicates the overall available resource level in the cell in Downlink and Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Composite Available Capacity Downlink | M |  | Composite Available Capacity 9.2.45 | For the Downlink | - |  |
| Composite Available Capacity Uplink | M |  | Composite Available Capacity 9.2.45 | For the Uplink | - |  |

### 9.2.45	Composite Available Capacity

The Composite Available Capacity IE indicates the overall available resource level in the cell in either Downlink or Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell Capacity Class Value | O |  | 9.2.46 |  | - |  |
| Capacity Value | M |  | 9.2.47 | ‘0’ indicates no resource is available, Measured on a linear scale. | - |  |

### 9.2.46	Cell Capacity Class Value

The Cell Capacity Class Value IE indicates the value that classifies the cell capacity with regards to the other cells. The Cell Capacity Class Value IE only indicates resources that are configured for traffic purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell Capacity Class Value | M |  | INTEGER (1..100,...) | Value 1 shall indicate the minimum cell capacity, and 100 shall indicate the maximum cell capacity. There should be a linear relation between cell capacity and Cell Capacity Class Value. | - |  |

### 9.2.47	Capacity Value

The Capacity Value IE indicates the amount of resources that are available relative to the total E-UTRAN resources. The capacity value should be measured and reported so that the minimum E-UTRAN resource usage of existing services is reserved according to implementation. The Capacity Value IE can be weighted according to the ratio of cell capacity class values, if available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity . Capacity Value should be measured on a linear scale. | - |  |

### 9.2.48	Mobility Parameters Information

The Mobility Parameters Information IE contains the change of the Handover Trigger as compared to its current value. The Handover Trigger corresponds to the threshold at which a cell initialises the handover preparation procedure towards a specific neighbour cell. Positive value of the change means the handover is proposed to take place later.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Handover Trigger Change | M |  | INTEGER (-20..20) | The actual value is IE value * 0.5 dB. |

### 9.2.49	Mobility Parameters Modification Range

The Mobility Parameters Modification Range IE contains the range of Handover Trigger Change values permitted by the eNB2 at the moment the MOBILITY CHANGE FAILURE message is sent.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Handover Trigger Change Lower Limit | M |  | INTEGER (-20..20) | The actual value is IE value * 0.5 dB. |
| Handover Trigger Change Upper Limit | M |  | INTEGER (-20..20) | The actual value is IE value * 0.5 dB. |

### 9.2.50	PRACH Configuration

This IE indicates the PRACH resources used in neighbor cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RootSequenceIndex | M |  | INTEGER (0..837) | See section 5.7.2. in TS 36.211 [10] | – |  |
| ZeroCorrelationZoneConfiguration | M |  | INTEGER (0..15) | See section 5.7.2. in TS 36.211 [10] | – |  |
| HighSpeedFlag | M |  | BOOLEAN | TRUE corresponds to Restricted set and FALSE to Unrestricted set. See section 5.7.2 in TS 36.211 [10] | – |  |
| PRACH-FrequencyOffset | M |  | INTEGER (0..94) | See section 5.7.1 of TS 36.211 [10] | – |  |
| PRACH-ConfigurationIndex | O |  | INTEGER (0..63) | Mandatory for TDD, shall not be present for FDD. See section 5.7.1. in TS 36.211 [10] | – |  |

### 9.2.51	Subframe Allocation

The Subframe Allocation IE is used to indicate the subframes that are allocated for MBSFN within the radio frame allocation period as defined in TS 36.331 [9].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Subframe Allocation | M |  |  |  |
| >Oneframe | M |  | BITSTRING (SIZE(6)) |  |
| >Fourframes | M |  | BITSTRING (SIZE(24)) |  |

### 9.2.52	CSG Membership Status

This element indicates the membership status of the UE to a particular CSG.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CSG Membership Status | M |  | ENUMERATED (member, not-member) |  | - |  |

### 9.2.53	CSG ID

This element indicates the identifier of the Closed Subscriber Group.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CSG ID | M |  | BIT STRING (SIZE (27)) |  | - |  |

### 9.2.54	ABS Information

This IE provides information about which sub frames the sending eNB is configuring as almost blank subframes and which subset of almost blank subframes are recommended for configuring measurements towards the UE. Almost blank subframes are subframes with reduced power on some physical channels and/or reduced activity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE ABS Information | M |  |  |  |
| >FDD |  |  |  |  |
| >>ABS Pattern Info | M |  | BIT STRING (SIZE(40)) | Each position in the bitmap represents a DL subframe, for which value "1" indicates ‘ABS’ and value "0" indicates ’non ABS’. The first position of the ABS pattern corresponds to subframe 0 in a radio frame where SFN = 0. The ABS pattern is continuously repeated in all radio frames. The maximum number of subframes is 40. |
| >>Number Of Cell-specific Antenna Ports | M |  | ENUMERATED (1, 2, 4, …) | P (number of antenna ports for cell-specific reference signals) defined in TS 36.211 [10] |
| >>Measurement Subset | M |  | BIT STRING (SIZE(40)) | Indicates a subset of the ABS Pattern Info above, and is used to configure specific measurements towards the UE. |
| >TDD |  |  |  |  |
| >>ABS Pattern Info | M |  | BIT STRING (1..70, ...) | Each position in the bitmap represents a subframe. Value "1" indicates ‘ABS’ and value "0" indicates ’non ABS’ which is applicable only in positions corresponding to the DL direction. The maximum number of subframes depends on UL/DL subframe configuration. The maximum number of subframes is 20 for UL/DL subframe configuration 1~5; 60 for UL/DL subframe configuration 6; 70 for UL/DL subframe configuration 0. UL/DL subframe configuration defined in TS 36.211 [10]. The first position of the ABS pattern corresponds to subframe 0 in a radio frame where SFN = 0. The ABS pattern is continuously repeated in all radio frames, and restarted each time SFN = 0. |
| >>Number Of Cell-specific Antenna Ports | M |  | ENUMERATED (1, 2, 4, …) | P (number of antenna ports for cell-specific reference signals) defined in TS 36.211 [10] |
| >>Measurement Subset | M |  | BIT STRING (1..70, ...) | Indicates a subset of the ABS Pattern Info above, and is used to configure specific measurements towards the UE |
| >ABS Inactive | M |  | NULL | Indicates that interference coordination by means of almost blank sub frames is not active |

### 9.2.55	Invoke Indication

This IE provides an indication about which type of information the sending eNB would like the receiving eNB to send back.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Invoke Indication | M |  | ENUMERATED (ABS Information, …, Start NAICS Information, Stop NAICS Information) |  |

### 9.2.56	MDT Configuration

The IE defines the MDT configuration parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED(Immediate MDT only, Immediate MDT and Trace, …) |  | – |  |
| CHOICE Area Scope of MDT | M |  |  |  | – |  |
| >Cell Based |  |  |  |  | – |  |
| >>Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | – |  |
| >>>ECGI | M |  | 9.2.14 |  | – |  |
| >TA Based |  |  |  |  | – |  |
| >>TA List for MDT |  | 1..<maxnoofTAforMDT> |  |  | – |  |
| >>>TAC | M |  | OCTET STRING (2) | Tracking Area Code. The TAI is derived using the current serving PLMN. | – |  |
| >PLMN Wide |  |  | NULL |  | – |  |
| >TAI based |  |  |  |  |  |  |
| >>TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  |  |  |
| >>>TAC | M |  | OCTET STRING (2) | Tracking Area Code |  |  |
| >>>PLMN Identity | M |  | 9.2.4 |  |  |  |
| Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [25]. First Bit = M1, Second Bit = M2, Third Bit = M3, Fourth Bit = M4, Fifth Bit = M5, Sixth Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration. Seventh Bit = M6, Eighth Bit = M7. Value "1" indicates "activate" and value "0" indicates "do not activate". | – |  |
| M1 Reporting Trigger | M |  | ENUMERATED (periodic, A2event-triggered, …, A2event-triggered periodic) | This IE shall be ignored if the Measurements to Activate IE has the first bit set to "0". | – |  |
| M1 Threshold Event A2 | C-ifM1A2trigger |  |  | Included in case of event-triggered or event-triggered periodic reporting for measurement M1 | – |  |
| >CHOICE Threshold | M |  |  |  | – |  |
| >>RSRP |  |  |  |  | – |  |
| >>>Threshold RSRP | M |  | INTEGER (0..97) | Corresponds to information provided in the RSRP-Range IE as defined in TS 36.331 [9]. | – |  |
| >>RSRQ |  |  |  |  | – |  |
| >>>Threshold RSRQ | M |  | INTEGER (0..34) | Corresponds to information provided in the RSRQ-Range IE as defined in TS 36.331 [9]. | – |  |
| M1 Periodic reporting | C-ifperiodicMDT |  |  | Included in case of periodic or event-triggered periodic reporting for measurement M1 | – |  |
| >Report interval | M |  | ENUMERATED ( ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60) | Corresponds to information provided in the ReportInterval IE as defined in TS 36.331 [9]. | – |  |
| >Report amount | M |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity) | Number of reports | – |  |
| M3 Configuration | C-ifM3 |  | 9.2.61 |  | YES | ignore |
| M4 Configuration | C-ifM4 |  | 9.2.62 |  | YES | ignore |
| M5 Configuration | C-ifM5 |  | 9.2.63 |  | YES | ignore |
| MDT Location Information | O |  | BITSTRING(SIZE(8)) | Each position in the bitmap represents requested location information as defined in TS 37.320 [31]. First Bit = GNSS Second Bit = E-CID information. Other bits are reserved for future use and are ignored if received. Value "1" indicates "activate" and value "0" indicates "do not activate". The eNB shall ignore the first bit unless the Measurements to Activate IE has the first bit or the sixth bit set to "1". | YES | ignore |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.2.64 |  | YES | ignore |
| M6 Configuration | C-ifM6 |  | 9.2.87 |  | YES | ignore |
| M7 Configuration | C-ifM7 |  | 9.2.88 |  | YES | ignore |
| Bluetooth Measurement Configuration | O |  | 9.2.134 |  | YES | ignore |
| WLAN Measurement Configuration | O |  | 9.2.135 |  | YES | ignore |
| Sensor Measurement Configuration | O |  | 9.2.183 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |

| Condition | Explanation |
| --- | --- |
| ifM1A2trigger | This IE shall be present if the Measurements to Activate IE has the first bit set to "1" and the M1 Reporting Trigger IE is set to "A2event-triggered" or to "A2event-triggered periodic". |
| ifperiodicMDT | This IE shall be present if the M1 Reporting Trigger IE is set to "periodic" or to "A2event-triggered periodic". |
| ifM3 | This IE shall be present if the Measurements to Activate IE has the third bit set to "1". |
| ifM4 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to "1". |
| ifM5 | This IE shall be present if the Measurements to Activate IE has the fifth bit set to "1". |
| ifM6 | This IE shall be present if the Measurements to Activate IE has the seventh bit set to "1". |
| ifM7 | This IE shall be present if the Measurements to Activate IE has the eighth bit set to "1". |

### 9.2.57	Void

### 9.2.58	ABS Status

The ABS Status IE is used to aid the eNB designating ABS to evaluate the need for modification of the ABS pattern.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL ABS status | M |  | INTEGER (0..100) | Percentage of used ABS resources. The numerator of the percentage calculation consists of resource blocks within the ABS indicated in the Usable ABS Pattern Info IE allocated by the eNB2 for DL traffic needing protection by ABS from inter-cell interference for DL scheduling, or allocated by the eNB2 for other reasons (e.g. some control channels). The denominator of the percentage calculation is the total quantity of resource blocks within the ABS indicated in the Usable ABS Pattern Info IE. |
| CHOICE Usable ABS Information | M |  | – | – |
| >FDD |  |  | – | – |
| >>Usable ABS Pattern Info | M |  | BIT STRING (SIZE(40)) | Each position in the bitmap represents a subframe, for which value "1" indicates ‘ABS that has been designated as protected from inter-cell interference by the eNB1, and available to serve this purpose for DL scheduling in the eNB2’ and value "0" is used for all other subframes. The pattern represented by the bitmap is a subset of, or the same as, the corresponding ABS Pattern Info IE conveyed in the LOAD INFORMATION message from the eNB1. |
| >TDD |  |  | – | – |
| >>Usable ABS Pattern Info | M |  | BIT STRING (1..70) | Each position in the bitmap represents a subframe, for which value "1" indicates ‘ABS that has been designated as protected from inter-cell interference by the eNB1, and available to serve this purpose for DL scheduling in the eNB2’ and value "0" is used for all other subframes. The pattern represented by the bitmap is a subset of, or the same as, the corresponding ABS Pattern Info IE conveyed in the LOAD INFORMATION message from the eNB1. |

### 9.2.59	Management Based MDT Allowed

This information element is used by the eNB to allow selection of the UE for management based MDT as described in TS 32.422 [6].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Management Based MDT Allowed | M |  | ENUMERATED (Allowed, ...) |  |

### 9.2.60	MultibandInfoList

The MultibandInfoList IE contains the additional frequency band indicators that a cell belongs to listed in decreasing order of preference, see TS 36.331 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| BandInfo |  | 1..<maxnoofBands> |  |  | – |  |
| >FrequencyBandIndicator | M |  | INTEGER (1.. 256, ...) | E-UTRA operating band as defined in TS 36.101 [42, table 5.5-1] | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBands | Maximum number of frequency bands that a cell belongs to. The value is 16. |

### 9.2.61	M3 Configuration

This IE defines the parameters for M3 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M3 Collection Period | M |  | ENUMERATED (ms100, ms1000, ms10000, …) |  |

### 9.2.62	M4 Configuration

This IE defines the parameters for M4 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M4 Collection Period | M |  | ENUMERATED (ms1024, ms2048, ms5120, ms10240, min1, …) |  |
| M4 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

### 9.2.63	M5 Configuration

This IE defines the parameters for M5 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M5 Collection Period | M |  | ENUMERATED (ms1024, ms2048, ms5120, ms10240, min1, …) |  |
| M5 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

### 9.2.64	MDT PLMN List

The purpose of the MDT PLMN List IE is to provide the list of PLMNs allowed for MDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MDT PLMN List |  | 1..<maxnoofMDTPLMNs> |  |  |
| >PLMN Identity | M |  | 9.2.4 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | Maximum no. of PLMNs in the MDT PLMN list. Value is 16. |

### 9.2.65	EARFCN Extension

The E-UTRA Absolute Radio Frequency Channel Number Extension defines the carrier frequency used in a cell for a given direction (UL or DL) in FDD or for both UL and DL directions in TDD.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| EARFCN Extension | M |  | INTEGER (maxEARFCN+1 .. newmaxEARFCN, ...) | The relation between EARFCN and carrier frequency (in MHz) are defined in TS 36.104 [16]. |

| Range bound | Explanation |
| --- | --- |
| maxEARFCN | Maximum value of EARFCNs. Value is 65535. |
| newmaxEARFCN | New maximum value of EARFCNs. Value is 262143. |

### 9.2.66	COUNT Value Extended

This information element indicates the 15 bit long PDCP SN and the corresponding 17 bit long Hyper Frame Number.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP-SN Extended | M |  | INTEGER (0..32767) |  | - |  |
| HFN Modified | M |  | INTEGER (0..131071) |  | - |  |

### 9.2.67	Extended UL Interference Overload Info

This IE provides report on interference overload for the set of subframes that are subject to UL-DL subframe reconfiguration. This IE applies to TDD only.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Associated Subframes | M |  | BITSTRING (SIZE(5)) | The set of subframe(s) to which the Extended UL interference overload indication is applicable. The bitmap from the least significant bit position to the most significant bit position represents subframes #{3, 4, 7, 8, 9} in a radio frame. Value "1" in a bit position indicates that the Extended UL interference overload indication is applicable to the corresponding subframe; and value "0" indicates otherwise. |
| Extended UL Interference Overload Indication | M |  | UL Interference Overload Indication 9.2.17 |  |

### 9.2.68	RNL Header

The RNL Header IE indicates the target eNB ID and source eNB ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Source eNB ID | M |  | Global eNB ID 9.2.22 |  | - |  |
| Target eNB ID | O |  | Global eNB ID 9.2.22 |  | - |  |

### 9.2.69	Masked IMEISV

This information element contains the IMEISV value with a mask, to identify a terminal model without identifying an individual Mobile Equipment.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Masked IMEISV | M |  | BIT STRING (SIZE(64)) | Coded as the International Mobile station Equipment Identity and Software Version Number (IMEISV) defined in TS 23.003 [29] with the last 4 digits of the SNR masked by setting the corresponding bits to 1. |

### 9.2.70	Expected UE Behaviour

This IE defines the behaviour of a UE with predictable activity and/or mobility behaviour, to assist the eNB/en-gNB in determining the optimum RRC connection time.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Expected UE Activity Behaviour | M |  | 9.2.71 |  |
| Expected HO Interval | O |  | ENUMERATED (sec15, sec30, sec60, sec90, sec120, sec180, long-time, …) | Indicates the expected time interval between inter-eNB handovers. If "long-time" is included, the interval between inter-eNB handovers is expected to be longer than 180 seconds. |

### 9.2.71	Expected UE Activity Behaviour

Indicates information about the expected "UE activity behaviour" as defined in TS 23.401 [12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Expected Activity Period | O |  | INTEGER (1..30¦40¦50¦60¦80¦ 100¦120¦150¦180¦ 181, ...) | If this IE is set to "181" the expected activity time is longer than 180 seconds. The remaining values indicate the expected activity time in [seconds]. |
| Expected Idle Period | O |  | INTEGER (1..30¦40¦50¦60¦80¦ 100¦120¦150¦180¦ 181, ...) | If this IE is set to "181" the expected idle time is longer than 180 seconds. The remaining values indicate the expected idle time in [seconds]. |
| Source of UE Activity Behaviour Information | O |  | ENUMERATED (subscription information, statistics, ...) | If "subscription information" is indicated, the information contained in the Expected Activity Period IE and the Expected Idle Period IE, if present, is derived from subscription information. If "statistics" is indicated, the information contained in the Expected Activity Period IE and the Expected Idle Period IE, if present, is derived from statistical information. |

### 9.2.72	SeNB Security Key

The SeNB Security Key IE is used to apply security in the SeNB as defined in TS 33.401 [18].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SeNB Security Key | M |  | BIT STRING (SIZE(256)) | The S-KeNB which is provided by the MeNB, see TS 33.401 [18]. |

### 9.2.73	SCG Change Indication

The SCG Change Indication IE is either used to request the SeNB to prepare the SCG Change in the SeNB or to request the MeNB to initiate the SCG Change towards the UE (see TS 36.300 [15]).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SCG Change Indication | M |  | ENUMERATED (PDCPCountWrapAround, PSCellChange, other, …) |  |

### 9.2.74	CoMP Information

This IE provides the list of CoMP hypothesis sets, where each CoMP hypothesis set is the collection of CoMP hypothesis(es) of one or multiple cells and each CoMP hypothesis set is associated with a benefit metric.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CoMP Information Item |  | 1 .. <maxnoofCoMPHypothesisSet> |  |  |
| >CoMP Hypothesis Set | M |  | 9.2.75 |  |
| >Benefit Metric | M |  | INTEGER (-101..100, …) | Value -100 indicates the maximum cost, and 100 indicates the maximum benefit. Value -101 indicates unknown benefit. Values from -100 to 100 should be calculated on a linear scale. |
| CoMP Information Start Time |  | 0..1 |  |  |
| >Start SFN | M |  | INTEGER (0..1023, …) | SFN of the radio frame containing the first subframe when the CoMP Information IE is valid. |
| >Start Subframe Number | M |  | INTEGER (0..9, …) | Subframe number, within the radio frame indicated by the Start SFN IE, of the first subframe when the CoMP Information IE is valid. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCoMPHypothesisSet | Maximum number of CoMP Hypothesis sets. The value is 256. |

### 9.2.75	CoMP Hypothesis Set

This IE provides a set of CoMP hypotheses. A CoMP hypothesis is hypothetical PRB-specific resource allocation information for a cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CoMP Hypothesis Set Item |  | 1..<maxnoofCoMPCells> |  |  |
| >Cell ID | M |  | ECGI 9.2.14 | ID of the cell for which the CoMP Hypothesis IE is applied. |
| >CoMP Hypothesis | M |  | BIT STRING (6..4400, …) | Each position in the bitmap represents a PRB in a subframe, for which value "1" indicates ‘interference protected resource’ and value "0" indicates ‘resource with no utilization constraints,’ which is applicable only in positions corresponding to the DL direction. The first bit corresponds to PRB 0 of the first subframe for which the IE is valid, the second bit corresponds to PRB 1 of the first subframe for which the IE is valid, and so on. The bit string may span across multiple contiguous subframes. The length of the bit string is an integer (maximum 40) multiple of . is defined in TS 36.211 [10]. The CoMP hypothesis pattern is continuously repeated. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCoMPCells | Maximum number of cells in a CoMP hypothesis set. Value is 32. |

### 9.2.76	RSRP Measurement Report List

This IE provides RSRP measurement reports of UEs served by the sending eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RSRP Measurement Report Item |  | 1 .. <maxUEReport> |  |  |
| >RSRP Measurement Result |  | 1 .. <maxCellReport> |  |  |
| >>RSRP Cell ID | M |  | ECGI 9.2.14 | ID of the cell on which the RSRP is measured. |
| >>RSRP Measured | M |  | INTEGER (0..97, ...) | Corresponds to information provided in the RSRP-Range IE as defined in TS 36.331 [9]. |
| >UE ID | O |  | BIT STRING (SIZE(16)) | ID assigned by eNB2 for the UE. |

| Range bound | Explanation |
| --- | --- |
| maxUEReport | Maximum number of UE measurement reports. Value is 128. |
| maxCellReport | Maximum number of reported cells. The value is 9. |

### 9.2.77	Dynamic DL transmission information

This IE contains assistance information for DL interference mitigation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE NAICS Information | M |  |  |  |
| >NAICS Active |  |  |  |  |
| >>Transmission Modes | O |  | BIT STRING (SIZE(8)) | The set bits indicate some or all transmission modes: 1, 2, 3, 4, 6, 8, 9, 10, as defined in TS 36.213 [23, 7.1]. The first/ leftmost bit is for transmission mode 1, the second bit is for transmission mode 2, and so on. |
| >>P_B | O |  | INTEGER (0..3) | See TS 36.213 [23, Table 5.2-1] |
| >>P_A_list |  | 0 .. <maxnoofPA> |  |  |
| >>>P_A | M |  | ENUMERATED (dB-6, dB-4dot77, dB-3, dB-1dot77, dB0, dB1, dB2, dB3,...) | See PA TS 36.213 [23, 5.2]. Value dB-6 corresponds to -6 dB, dB-4dot77 corresponds to -4.77 dB etc. |
| >NAICS Inactive |  |  | NULL |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPA | Maximum no of PA values that can be configured. Value is 3. |

### 9.2.78	ProSe Authorized

This IE provides information on the authorization status of the UE for ProSe service(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| ProSe Direct Discovery | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for ProSe Direct Discovery | - |  |
| ProSe Direct Communication | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for ProSe Direct Communication | - |  |
| ProSe UE-to-Network Relaying | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized to act as ProSe UE-to-Network Relay | YES | ignore |

### 9.2.79	CSI Report

This IE provides CSI reports of UEs served by the cell for which the information is provided.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSI Report per Cell |  | 1 .. <maxUEReport> |  |  |
| >UE ID | M |  | BIT STRING (SIZE(16)) | ID assigned by eNB2 for the UE. |
| >CSI Report per CSI Process |  | 1 .. <maxCSIProcess> |  |  |
| >>CSI Process Configuration Index | M |  | INTEGER (1..7, …) | Indicates one of the possible CSI Process configurations in the serving cell. |
| >>CSI Report per CSI Process Item |  | 1.. <maxCSIReport> |  |  |
| >>>RI | M |  | INTEGER (1..8, …) | The RI corresponding to the CQI being reported for this CSI process item. Value defined in TS 36.213 [11]. |
| >>>Wideband CQI | M |  | 9.2.80 |  |
| >>>Subband Size | M |  | ENUMERATED (2, 3, 4, 6, 8, …) | Corresponds to a value of subband size k defined in TS 36.213 [11] for the system bandwidth. |
| >>>Subband CQI List |  | 0 .. <maxSubband> |  |  |
| >>>>Subband CQI | M |  | 9.2.81 |  |
| >>>>Subband Index | M |  | INTEGER (0..27, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxUEReport | Maximum number of UE. Value is 128. |
| maxCSIProcess | Maximum number of CSI processes per UE. The value is 4. |
| maxCSIReport | Maximum number of CSI Reports per CSI Process. The value is 2. |
| maxSubband | Maximum number of subbands. The value is 14. |

### 9.2.80	Wideband CQI

This IE indicates the Wideband CQI as defined in TS 36.213 [11].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Wideband CQI Codeword 0 | M |  | INTEGER (0..15, …) | Value defined in TS 36.213 [11]. |
| CHOICE Wideband CQI Codeword 1 | O |  |  |  |
| >4-bit CQI | M |  | INTEGER (0..15, …) | Value defined in TS 36.213 [11]. |
| >3-bit spatial differential CQI | M |  | INTEGER (0..7, …) | Value defined in TS 36.213 [11]. |

### 9.2.81	Subband CQI

This IE indicates the Subband CQI as defined in TS 36.213 [11].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Subband CQI Codeword 0 | M |  |  |  |
| >4-bit CQI | M |  | INTEGER (0..15, …) | Value defined in TS 36.213 [11]. |
| >2-bit Subband differential CQI | M |  | INTEGER (0..3, …) | Value defined in TS 36.213 [11]. |
| >2-bit differential CQI | M |  | INTEGER (0..3, …) | Value defined in TS 36.213 [11]. |
| CHOICE Subband CQI Codeword 1 | O |  |  |  |
| >4-bit CQI | M |  | INTEGER (0..15, …) | Value defined in TS 36.213 [11]. |
| >3-bit spatial differential CQI | M |  | INTEGER (0..7, …) | Value defined in TS 36.213 [11]. |
| >2-bit Subband differential CQI | M |  | INTEGER (0..3, …) | Value defined in TS 36.213 [11]. |
| >2-bit differential CQI | M |  | INTEGER (0..3, …) | Value defined in TS 36.213 [11]. |

### 9.2.82	COUNT Value for PDCP SN Length 18

This information element indicates the 18 bit long PDCP SN and the corresponding 14 bit long Hyper Frame Number.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP-SN Length 18 | M |  | INTEGER (0..262143) |  | – |  |
| HFN for PDCP-SN Length 18 | M |  | INTEGER (0..16383) |  | – |  |

### 9.2.83	LHN ID

The LHN ID IE is used to indicate the LHN ID of the eNB, as defined in TS 23.003 [29].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Local Home Network ID | M |  | OCTET STRING (SIZE (32..256)) | Identifies the Local Home Network. |

### 9.2.84	Correlation ID

This information element is the GTP Tunnel Endpoint Identifier or GRE key to be used for the user plane transport between eNB and the L-GW described in TS 23.401 [12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation ID | M |  | OCTET STRING (SIZE(4)) |  |

### 9.2.85	UE Context Kept Indicator

This IE indicates that the UE Context at the SeNB is kept in case of inter-MeNB handover without SeNB/SgNB Change procedure, as specified in TS 36.300 [15] or TS37.340 [32].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Context Kept Indicator | M |  | ENUMERATED (True, …) |  |

### 9.2.86	eNB UE X2AP ID Extension

This information element combined with the eNB UE X2AP ID uniquely identifies an UE over the X2 interface within an eNB. If the setup of an UE associcated signalling connection was initiated including the eNB UE X2AP ID Extension, the eNB UE X2AP ID Extension shall be used by both peers for the life-time of the respective UE-associated signalling connection.

The usage of this IE is defined in TS 36.401 [2].

NOTE: 	If X2-C signalling transport is shared among multiple interface instances, the value of the eNB UE X2AP ID, combined with the eNB UE X2AP ID Extension, is allocated so that it can be associated with an X2-C interface instance.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| eNB UE X2AP ID Extension | M |  | INTEGER (0..4095,…) |  |

### 9.2.87	M6 Configuration

This IE defines the parameters for M6 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M6 Report Interval | M |  | ENUMERATED (ms1024, ms2048, ms5120, ms10240, …) |  |
| M6 Delay Threshold | C-ifUL |  | ENUMERATED (ms30, ms40, ms50, ms60, ms70, ms80, ms90, ms100, ms150, ms300, ms500, ms750, …) |  |
| M6 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

| Condition | Explanation |
| --- | --- |
| ifUL | This IE shall be present if the M6 Links to log IE is set to "uplink" or to "both-uplink-and-downlink". |

### 9.2.88	M7 Configuration

This IE defines the parameters for M7 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M7 Collection Period | M |  | INTEGER (1..60, …) | Unit: minutes |
| M7 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

### 9.2.89	Tunnel Information

The Tunnel Information IE indicates the transport layer address and UDP port number.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transport Layer Address | M |  | BIT STRING (1..160, ...) | eNB’s Transport Layer Address. |
| UDP Port Numbers | O |  | OCTET STRING (SIZE(2)) | UDP Port Numbers if NAT/NAPT is deployed in the BBF access network. |

### 9.2.90	X2 Benefit Value

The X2 Benefit Value IE indicates the quantified benefit of the signalling connection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| X2 Benefit Value | M |  | INTEGER (1..8, …) | Value 1 indicates low benefit, and 8 indicates high benefit. |

### 9.2.91	Resume ID

The Resume ID IE is used to address a suspended UE Context within an eNB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Resume ID | M |  |  |  |
| >Resume ID not truncated |  |  |  |  |
| >>Resume ID not truncated | M |  | BIT STRING (SIZE (40)) | 40 bit Resume Resume Identity contained in the RRCConnectionResumeRequest message (TS 36.331 [9]). The 20 most significant bits refer to the eNB ID of the eNB that allocated the Resume ID, the 20 least significant bits identify the UE Context stored at the eNB that allocated the Resume ID. |
| >Resume ID truncated |  |  |  |  |
| >>Resume ID truncated | M |  | BIT STRING (SIZE (24)) | 24 bit Resume Identity contained in the RRCConnectionResumeRequest message (TS 36.331 [9]). The 12 most significant bits refer to the 12 least significant bits of the eNB ID of the eNB that allocated the Resume ID. The 12 least significant bits refer to the 12 least significant bits that identify the UE Context stored at the eNB that allocated the Resume ID. |

### 9.2.92	Bearer Type

This IE is used to support Non-IP data as specified in TS 23.401 [11].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bearer Type | M |  | ENUMERATED (non IP, …) |  |

### 9.2.93	V2X Services Authorized

This IE provides information on the authorization status of the UE to use the sidelink for V2X services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Vehicle UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized as Vehicle UE | - |  |
| Pedestrian UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized as Pedestrian UE | - |  |

### 9.2.94	Offset of NB-IoT Channel Number to EARFCN

This IE is used to indicate the offset of the NB-IoT Channel Number to the EARFCN (TS 36.104 [16]).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Offset of NB-IoT Channel Number to EARFCN | M |  | ENUMERATED (-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,-0.5,0,1,2,3,4,5,6,7,8,9,... , -8.5, -4.5, 3.5, 7.5) |  |

### 9.2.95	WT ID

This IE is used to identify a WT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE WT ID Type | M |  |  |  |
| >WT ID Type 1 |  |  |  |  |
| >>PLMN ID | M |  | PLMN Identity 9.2.4 |  |
| >>Short WT ID | M |  | BIT STRING (24) |  |
| >WT ID Type 2 |  |  |  |  |
| >>Long WT ID | M |  | BIT STRING (48) |  |

### 9.2.96	WT UE XwAP ID

The WT UE XwAP ID is allocated by the WT and uniquely identifies a UE over the Xw interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| WT UE XwAP ID | M |  | OCTET STRING (SIZE(3)) |  |

### 9.2.97	UE Sidelink Aggregate Maximum Bit Rate

This IE indicates the aggregate maximum bit rate for all radio bearers per UE in the sidelink for V2X services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Sidelink Aggregate Maximum Bit Rate | M |  | Bit Rate 9.2.11 | Value 0 shall be considered as a logical error by the receiving eNB. |

### 9.2.98	NR Neighbour Information

This IE contains cell configuration information of NR cells that a neighbour node may need for the X2 AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR Neighbour Information |  | 1 .. <maxnoofNRNeighbours> |  |  | – |  |
| >NR Neighbour Information Item |  |  |  |  | – |  |
| >>NRPCI | M |  | INTEGER (0..1007) | NR Physical Cell ID | – |  |
| >>NR CGI | M |  | 9.2.111 |  | – |  |
| >>5GS-TAC | O |  | OCTET STRING (3) | Broadcast 5GS Tracking Area Code | – |  |
| >>Configured TAC | O |  | OCTET STRING (2) | This is the TAC configured in the en-gNB, different from the 5GS TAC broadcast in the NR cell and enables application of Roaming and Access Restrictions for EN-DC as specified in TS 37.340 [32]. | – |  |
| >>Measurement Timing Configuration | M |  | OCTET STRING | Includes the MeasurementTimingConfiguration inter-node message for the neighbour cell, as defined in TS 38.331 [31]. | – |  |
| >>CHOICE NR-Neighbour-Mode-Info | M |  |  |  | – |  |
| >>>FDD |  |  |  |  |  |  |
| >>>>FDD Info |  | 1 |  |  | – |  |
| >>>>>UL ARFCNFreqInfo | M |  | NR ARFCN Frequency Info 9.2.106 | This IE is ignored for NR operating bands for which uplink range of NREF is not defined in section 5.4.2.3 of TS 38.104 [37]. | – |  |
| >>>>>DL ARFCNFreqInfo | M |  | NR ARFCN Frequency Info 9.2.106 |  | – |  |
| >>>>>UL Carrier List | O |  | NR Carrier List 9.2.168 |  | YES | ignore |
| >>>TDD |  |  |  |  |  |  |
| >>>>TDD Info |  | 1 |  |  | – |  |
| >>>>>ARFCNNRFreqInfo | M |  | NR ARFCN Frequency Info 9.2.106 |  | – |  |
| >>>>>Intended TDD DL-UL Configuration NR | O |  | OCTET STRING | Contains the Intended TDD DL-UL Configuration NR IE as defined in TS 38.423 [49]. | YES | ignore |
| >>>>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | Includes the TDD-UL-DL-ConfigCommon IE as defined in TS 38.331 [31]. | YES | ignore |
| >>>>>Carrier List | O |  | NR Carrier List 9.2.168 |  | YES | ignore |
| >>CSI-RS Transmission Indication | O |  | ENUMERATED {activated, deactivated, ...} | This IE indicates the CSI-RS transmission status of the given cell. If the Additional Measurement Timing Configuration List IE is present in the NR Neighbour Information IE, this IE is ignored. | YES | ignore |
| >>SSB Positions In Burst | O |  | 9.2.169 |  | YES | ignore |
| >>NR Cell PRACH Configuration | O |  | OCTET STRING | Contains the NR Cell PRACH Configuration IE as defined in TS 38.473 [44]. | YES | ignore |
| >>Additional Measurement Timing Configuration List | O | 1 .. <maxnoofMTCItems> |  |  | YES | ignore |
| >>>Measurement Timing Configuration Index | M |  | INTEGER (0..16) | “0” refers to the configuration contained in the Measurement Timing Configuration IE. Any value between “1” and “16” refers to a configuration within the Additional Measurement Timing Configuration List IE. | – |  |
| >>>CSI-RS MTC Configuration List | M | 1 .. <maxnoofCSIRSconfigurations> |  | This list explicitly expresses the CSI-RS configurations contained in the MTC. | – |  |
| >>>>CSI-RS Index | M |  | INTEGER (0..95) | Index of CSI-RS as in MTC | – |  |
| >>>>CSI-RS Status | M |  | ENUMERATED (activated, deactivated, …) | This IE indicates the CSI-RS transmission status of the configuration. | – |  |
| >>>>CSI-RS Neighbour List | O | 1 .. <maxnoofCSIRSneighbourCells> |  | This list expresses the cells and CSI-RSs neighbouring the CSI-RS in the CSI-RS Index IE. | – |  |
| >>>>>NR CGI | M |  | 9.2.111 |  | – |  |
| >>>>>CSI-RS MTC Neighbour List | O | 1 .. <maxnoofCSIRSneighbourCellsInMTC> |  | This list expresses the CSI-RSs served by the CGI, which are neighbouring the CSI-RS of the neighbour cell. | – |  |
| >>>>>>CSI-RS Index | M |  | INTEGER (0..95) |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofNRNeighbours | Maximum no. of neighbour NR cells associated to a given served cell. Value is 1024. |
| maxnoofMTCItems | Maximum no. of measurement timing configurations associated with the neighbour cell. Value is 16. |
| maxnoofCSIRSconfigurations | Maximum number of CSI-RS configurations reported in the MTC. Value is 96 |
| maxnoofCSIRSneighbourCells | Maximum number of cells neighbouring a CSI-RS coverage area. Value is 16 |
| maxnoofCSIRSneighbourCellsInMTC | Maximum number of CSI-RS coverage areas neighbouring a specific CSI-RS coverage area. Value is 16 |

### 9.2.99	Extended Bit Rate

This IE indicates the number of bits delivered by E-UTRAN in UL or to E-UTRAN in DL within a period of time, divided by the duration of the period. It is used, for example, to indicate the maximum or guaranteed bit rate for a GBR bearer, or an aggregated maximum bit rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended Bit Rate |  |  | INTEGER (10,000,000,001..4,000,000,000,000,...) | The unit is: bit/s |

### 9.2.100	en-gNB UE X2AP ID

This information element uniquely identifies an UE over the X2 interface within an en-gNB.

The usage of this IE is defined in TS 36.401 [2].

NOTE: 	If X2-C signalling transport is shared among multiple interface instances, the value of the en-gNB UE X2AP ID is allocated so that it can be associated with an X2-C interface instance.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| en-gNB UE X2AP ID | M |  | INTEGER (0.. 232 -1) |  |

### 9.2.101	SgNB Security Key

The SgNB Security Key IE is used to apply security in the en-gNB as defined in TS 33.401 [18].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SgNB Security Key | M |  | BIT STRING (SIZE(256)) | The S-KgNB which is provided by the MeNB, see TS 33.401 [18]. |

### 9.2.102	Target SgNB ID Information

This IE contains the target SgNB ID used by MeNB to find the target en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Target SgNB ID | M |  | Global en-gNB ID 9.2.112 |  |

### 9.2.103	SCG Configuration Query

The SCG Configuration Query IE is used to request the en-gNB to provide current SCG configuration.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SCG Configuration Query | M |  | ENUMERATED (True, …) |  |

### 9.2.104	Delivery Status

This IE defines the Delivery Status IE of RRC Transfer message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Highest successfully delivered NR PDCP Sequence Number | M | 0..212-1 | INTEGER (0..212-1) | Highest successfully delivered NR PDCP SN, as defined in 38.323 [33]. | – |  |

### 9.2.105	Void

Void

### 9.2.106	NR Frequency Info

The NR Frequency Info defines the carrier frequency and bands used in a cell for a given direction (UL or DL) in FDD or for both UL and DL directions in TDD.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NRARFCN | M |  | INTEGER (0..maxNRARFCN) | RF Reference Frequency as defined in TS 38.104 [37] section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. | – |  |
| Frequency Band List |  | 1 |  |  | – |  |
| >Frequency Band Item |  | 1..<maxnoofNrCellBands> |  |  |  |  |
| >>NR Frequency Band | M |  | INTEGER (1.. 1024, ...) | Primary NR Operating Band as defined in TS38.104 [37] section 5.4.2.3. The value 1 corresponds e n1, value 2 corresponds to NR operating band n2, etc. | – |  |
| >>Supported SUL band List |  | 0..<maxnoofNrCellBands> |  |  | – |  |
| >>>Supported SUL band Item | M |  | INTEGER (1.. 1024, ...) | Supplementary NR Operating Band as defined in TS 38.104 [37] section 5.4.2.3 that can be used for SUL duplex mode as per TS 38.101-1 table 5.2.-1. The value 80 corresponds to NR operating band n80, value 81 corresponds to NR operating band n81, etc. | – |  |
| SUL Information | O |  | 9.2.123 |  | – |  |
| Frequency Shift 7p5khz | O |  | ENUMERATED (false, true, ...) | Indicate whether the value of Δshift is 0kHz or 7.5kHz when calculating FREF,shift as defined in Section 5.4.2.1 of TS 38.104 [37]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxNRARFCN | Maximum value of NRARFCNs. Value is 3279165. |
| maxnoofNrCellBands | Maximum no. of frequency bands supported for a NR cell. Value is 32. |

### 9.2.107	NR UE Security Capabilities

This IE defines the supported algorithms for encryption and integrity protection in NR as defined in TS 33.401 [18]. The eNBs store and send the complete bitmaps without modification or truncation as specified in TS 36.300 [15].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR Encryption Algorithms | M |  | BIT STRING (SIZE(16, …)) | Each position in the bitmap represents an encryption algorithm: "all bits equal to 0" – UE supports no other NR algorithm than NEA0, "first bit" – 128-NEA1, "second bit" – 128-NEA2, "third bit" – 128-NEA3, “fourth to seventh bit” correspond to bit 4 to bit 1 of octet 3 in the UE Additional Security Capability IE defined in TS 24.301 [14], “eighth to fifteenth bit” correspond to bit 8 to bit 1 of octet 4 in the UE Additional Security Capability IE defined in TS 24.301 [14], other bits reserved for future use. Value '1' indicates support and value '0' indicates no support of the algorithm. Algorithms are defined in TS 33.401 [18]. |
| NR Integrity Protection Algorithms | M |  | BIT STRING (SIZE(16, …)) | Each position in the bitmap represents an integrity protection algorithm: "all bits equal to 0" – UE supports no other NR algorithm than NIA0, "first bit" – 128-NIA1, "second bit" – 128-NIA2, "third bit" – 128-NIA3, “fourth to seventh bit” correspond to bit 4 to bit 1 of octet 5 in the UE Additional Security Capability IE defined in TS 24.301 [14], “eighth to fifteenth bit” correspond to bit 8 to bit 1 of octet 6 in the UE Additional Security Capability IE defined in TS 24.301 [14], other bits reserved for future use. Value '1' indicates support and value '0' indicates no support of the algorithm. Algorithms are defined in TS 33.401 [18]. |

### 9.2.108	EN-DC Resource Configuration

This IE contains the EN-DC resource configuration for an E-RAB, indicating the presence of PDCP at the en-gNB and Lower Layers at MCG and SCG.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP at SgNB | M |  | ENUMERATED (present, not present) |  | – |  |
| MCG resources | M |  | ENUMERATED (present, not present) |  | – |  |
| SCG resources | M |  | ENUMERATED (present, not present) |  | – |  |

### 9.2.109	PDCP Change Indication

The PDCP Change Indication IE is used to require the MeNB to either initiate the security key update or to perform PDCP data recovery towards the UE (see TS 37.340 [15]).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PDCP Change Indication | M |  | ENUMERATED (S-KgNB update required, PDCP data recovery required, …) | The value of S-KgNB update required indicates that the security key in en-gNB needs to be updated. The value of PDCP data recovery required indicates that MeNB needs to perform PDCP data recovery. |

### 9.2.110	Served NR Cell Information

This IE contains cell configuration information of an NR cell that a neighbour eNB may need for the X2 AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID | – |  |
| Cell ID | M |  | NR CGI 9.2.111 |  | – |  |
| 5GS-TAC | O |  | OCTET STRING (3) | Broadcast 5GS Tracking Area Code. If this IE is included, the receiving node may assume that the NR cell provides 5GS service and is eligible as inter-system HO target candidate. | – |  |
| Configured TAC | O |  | OCTET STRING (2) | This is the TAC configured in the en-gNB, different from the 5GS TAC broadcast in the NR cell and enables application of Roaming and Access Restrictions for EN-DC as specified in TS 37.340 [32]. | – |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB1 associated to the NR Cell Identity in the Cell ID IE. If more than maxnoofBPLMNs are needed for NR, they are provided by the Additional PLMNs IE. | – |  |
| >PLMN Identity | M |  | 9.2.4 |  | – |  |
| CHOICE NR-Mode-Info | M |  |  |  | – |  |
| >FDD |  |  |  |  |  |  |
| >>FDD Info |  | 1 |  |  | – |  |
| >>>UL FreqInfo | M |  | NR Frequency Info 9.2.106 | This IE is ignored for NR operating bands for which uplink range of NREF is not defined in section 5.4.2.3 of TS 38.104 [37]. | – |  |
| >>>DL FreqInfo | M |  | NR Frequency Info 9.2.106 |  | – |  |
| >>>UL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.114 | This IE is ignored for NR operating bands for which uplink range of NREF is not defined in section 5.4.2.3 of TS 38.104 [37]. | – |  |
| >>>DL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.114 |  | – |  |
| >>>UL Carrier List | O |  | NR Carrier List 9.2.168 | If included, the UL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>DL Carrier List | O |  | NR Carrier List 9.2.168 | If included, the DL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >TDD |  |  |  |  |  |  |
| >>TDD Info |  | 1 |  |  | – |  |
| >>>NRFreqInfo | M |  | NR Frequency Info 9.2.106 |  | – |  |
| >>>Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.114 |  | – |  |
| >>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | Includes the TDD-UL-DL-ConfigCommon IE as defined in TS 38.331 [31] | YES | ignore |
| >>>Carrier List | O |  | NR Carrier List 9.2.168 | If included, the Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>Intended TDD DL-UL Configuration NR | O |  | OCTET STRING | Contains the Intended TDD DL-UL Configuration NR IE as defined in TS 38.423 [49]. | YES | ignore |
| Measurement Timing Configuration | M |  | OCTET STRING | Includes the MeasurementTimingConfiguration inter-node message for the served cell, as defined in TS 38.331 [31]. | – |  |
| Additional PLMNs |  | 0..<maxnoofAdditionalPLMNs> |  | Additional PLMNs in addition to the Served PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.2.4 |  | – |  |
| Broadcast PLMN Identity Info List NR |  | 0..<maxnoofextBPLMNs> |  | Corresponds to information provided in the PLMN-IdentityInfoList IE in the SIB1 message as specified in TS 38.331 [31]. All PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE are included and provided in the same order as broadcast in the SIB1 message. | YES | ignore |
| >Broadcast PLMNs |  | 1..<maxnoofextBPLMNs> |  | Broadcast PLMN IDs in SIB1 associated to the NR Cell Identity IE | – |  |
| >>PLMN Identity | M |  | 9.2.4 |  | – |  |
| >5GS-TAC | O |  | OCTET STRING (3) |  | – |  |
| >NR Cell Identity | M |  | BIT STRING (SIZE(36)) |  | – |  |
| SSB Positions In Burst | O |  | 9.2.169 |  | YES | ignore |
| NR Cell PRACH Configuration | O |  | OCTET STRING | Containing 9.3.1.139 NR Cell PRACH Configuration as of TS 38.473 [44]. | YES | ignore |
| CSI-RS Transmission Indication | O |  | ENUMERATED {activated, deactivated, ...} | This IE indicates the CSI-RS transmission status of the given cell. If the Additional Measurement Timing Configuration List IE is present, this IE is ignored. | YES | ignore |
| SFN Offset | O |  | 9.2.175 |  | YES | ignore |
| Additional Measurement Timing Configuration List | O | 1 .. <maxnoofMTCItems> |  |  | YES | ignore |
| >Measurement Timing Configuration Index | M |  | INTEGER (0..16) | “0” refers to the configuration contained in the Measurement Timing Configuration IE. Any value between “1” and “16” refers to a configuration within the Additional Measurement Timing Configuration List IE. | – |  |
| >CSI-RS MTC Configuration List | M | 1 .. <maxnoofCSIRSconfigurations> |  | This list explicitly expresses the CSI-RS configurations contained in the MTC. | – |  |
| >>CSI-RS Index | M |  | INTEGER (0..95) | Index of CSI-RS as in MTC | – |  |
| >>CSI-RS Status | M |  | ENUMERATED (activated, deactivated, …) | This IE indicates the CSI-RS transmission status of the configuration. | – |  |
| >>CSI-RS Neighbour List | O | 1 .. <maxnoofCSIRSneighbourCells> |  | This list expresses the cells and CSI-RSs neighbouring the CSI-RS in the CSI-RS Index IE. | – |  |
| >>>NR CGI | M |  | 9.2.111 |  | – |  |
| >>>CSI-RS MTC Neighbour List | O | 1 .. <maxnoofCSIRSneighbourCellsInMTC> |  | This list expresses the CSI-RSs served by the CGI, which are neighbouring the CSI-RS of the neighbour cell. | – |  |
| >>>>CSI-RS Index | M |  | INTEGER (0..95) |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMN Ids. Value is 6. |
| maxnoofAdditionalPLMNs | Maximum no. additional PLMN Ids. Value is 6. |
| maxnoofextBPLMNs | Maximum no. of extended broadcast PLMN Ids. Value is 12. |
| maxnoofMTCItems | Maximum no. of measurement timing configurations associated with the neighbour cell. Value is 16. |
| maxnoofCSIRSconfigurations | Maximum number of CSI-RS configurations reported in the MTC. Value is 96 |
| maxnoofCSIRSneighbourCells | Maximum number of cells neighbouring a CSI-RS coverage area. Value is 16 |
| maxnoofCSIRSneighbourCellsInMTC | Maximum number of CSI-RS coverage areas neighbouring a specific CSI-RS coverage area. Value is 16 |

### 9.2.111	NR CGI

The NR Cell Global Identifier (NR CGI) is used to globally identify an NR cell (see TS 38.401 [34]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.4 |  |
| NR Cell Identity | M |  | BIT STRING (36) | The leftmost bits of the NR Cell Identity IE value correspond to the value of the en-gNB ID IE contained in the Global en-gNB ID IE (defined in section 9.2.112) identifying the en-gNB that controls the cell. |

### 9.2.112	Global en-gNB ID

This IE is used to globally identify an en-gNB (see TS 37.340 [32]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.4 |  |
| CHOICE en-gNB ID | M |  |  |  |
| >en-gNB ID |  |  |  |  |
| >>en-gNB ID | M |  | BIT STRING (SIZE(22..32)) | Equal to the leftmost bits of the NR Cell Identity IE contained in the NR CGI IE of each cell served by the en-gNB. |

### 9.2.113	Void

### 9.2.114	NR Transmission Bandwidth

The NR Transmission Bandwidth IE is used to indicate the UL or DL transmission bandwidth.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR SCS | M |  | ENUMERATED (scs15, scs30, scs60, scs120, …) | The values scs15, scs30, scs60 and scs120 corresponds to the sub carrier spacing in TS 38.104 [37]. |
| NR NRB | M |  | ENUMERATED (nrb11, nrb18, nrb24, nrb25, nrb31, nrb32, nrb38, nrb51, nrb52, nrb65, nrb66, nrb78, nrb79, nrb93, nrb106, nrb107, nrb121, nrb132, nrb133, nrb135, nrb160, nrb162, nrb189, nrb216, nrb217, nrb245, nrb264, nrb270, nrb273, ...) | This IE is used to indicate the UL or DL transmission bandwidth expressed in units of resource blocks "NRB" (TS 38.104 [37]). The values nrb11, nrb18, etc. correspond to the number of resource blocks "NRB" 11, 18, etc. |

### 9.2.115	Cell Assistance Information

The Cell Assistance Information IE is used by the eNB to request information about NR cells.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cell Assistance Type | M |  |  | This IE may be refined. |
| >Limited List |  |  |  |  |
| >>List of Requested NR Cells |  | 1 .. < maxCellinengNB > |  | Included when the eNB requests a limited list of served NR cells. |
| >>>NR CGI | M |  | 9.2.111 | NR cell for which served NR cell information is requested. |
| >Full List |  |  |  |  |
| >>Complete Information Request Indicator | M |  | ENUMERATED (allServedNRCells, …) | Included when the eNB requests the complete list of served NR cells. |

| Range bound | Explanation |
| --- | --- |
| maxCellinengNB | Maximum no. cells that can be served by an en-gNB. Value is 16384. |

### 9.2.116	MeNB Resource Coordination Information

The MeNB Resource Coordination Information IE is LTE resource allocation at MeNB and used at the en-gNB to coordinate resource or sidelink resource utilisation between the MeNB and the en-gNB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| EUTRA Cell ID | M |  | ECGI 9.2.14 | This IE indicates the PCell. | – |  |
| UL Coordination Information | M |  | BIT STRING (6..4400, …) | Each position in the bitmap represents a PRB pair in a subframe; value "0" indicates "PCell resource not intended to be used for transmission by the sending node", value "1" indicates "PCell resource intended to be used for transmission by the sending node". The bit string spans from the first PRB pair of the first represented subframe to the last PRB pair of the same subframe and then moves to the following PRBs in the following subframes in the same order. Each position is applicable only in positions corresponding to UL subframes or SL subframes for sidelink transmission. The bit string may span across multiple contiguous subframes (maximum 40). The first position of the UL Coordination Information corresponds to subframe 0 in a radio frame where SFN = 0. The length of the bit string is an integer multiple of . is defined in TS 36.211 [10]. The UL Coordination Information is continuously repeated. | – |  |
| DL Coordination Information | O |  | BIT STRING (6..4400, …) | Each position in the bitmap represents a PRB pair in a subframe; value "0" indicates "PCell resource not intended to be used for transmission by the sending node", value "1" indicates "PCell resource intended to be used for transmission by the sending node". The bit string spans from the first PRB pair of the first represented subframe to the last PRB pair of the same subframe and then moves to the following PRBs in the following subframes in the same order. Each position is applicable only in positions corresponding to DL subframes. The bit string may span across multiple contiguous subframes (maximum 40). The first position of the DL Coordination Information corresponds to the receiving node’s subframe 0 in a receiving node’s radio frame where SFN = 0. The length of the bit string is an integer multiple of . is defined in TS 36.211 [10]. The DL Coordination Information is continuously repeated. | – |  |
| NR CGI | O |  | 9.2.111 | This IE indicates the assumed PSCell. | YES | ignore |
| MeNB Coordination Assistance Information | O |  | 9.2.139 |  | YES | reject |

### 9.2.117	SgNB Resource Coordination Information

The SgNB Resource Coordination Information IE indicates resources within the bandwidth of the PCell which are not available for use by the MeNB and is used at the MeNB to coordinate resource utilisation between the en-gNB and the MeNB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.2.111 | This IE indicates the PSCell. | – |  |
| UL Coordination Information | M |  | BIT STRING (6..4400, …) | Each position in the bitmap represents a PRB pair in a subframe; value "0" indicates "PCell resource not intended to be used for transmission by the sending node", value "1" indicates "PCell resource intended to be used for transmission by the sending node". The bit string spans from the first PRB pair of the first represented subframe to the last PRB pair of the same subframe and then moves to the following PRBs in the following subframes in the same order. Each position is applicable only in positions corresponding to UL subframes. The bit string may span across multiple contiguous subframes (maximum 40). The first position of the UL Coordination Information corresponds to the receiving node’s subframe 0 in a receiving node’s radio frame where SFN = 0. The length of the bit string is an integer multiple of . is defined in TS 36.211 [10]. The UL Coordination Information is continuously repeated. | – |  |
| DL Coordination Information | O |  | BIT STRING (6..4400, …) | Each position in the bitmap represents a PRB pair in a subframe; value "0" indicates "PCell resource not intended to be used for transmission by the sending node", value "1" indicates "PCell resource intended to be used for transmission by the sending node". The bit string spans from the first PRB pair of the first represented subframe to the last PRB pair of the same subframe and then moves to the following PRBs in the following subframes in the same order. Each position is applicable only in positions corresponding to DL subframes. The bit string may span across multiple contiguous subframes (maximum 40). The first position of the DL Coordination Information corresponds to the receiving node’s subframe 0 in a receiving node’s radio frame where SFN = 0. The length of the bit string is an integer multiple of . is defined in TS 36.211 [10]. The DL Coordination Information is continuously repeated. | – |  |
| EUTRA Cell ID | O |  | ECGI 9.2.14 | Reference cell for UL Coordination Information IE and DL Coordination Information IE. | YES | ignore |
| SgNB Coordination Assistance Information | O |  | 9.2.140 |  | YES | reject |

### 9.2.118	UL Configuration

This IE indicates how the UL PDCP is configured for the assisting node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UL UE Configuration | M |  | ENUMERATED (no-data, shared, only, ...) | Indicates how the UE uses the UL at the assisting node. |

### 9.2.119	RLC Mode

The RLC Mode IE indicates the RLC Mode used for an E-RAB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| RLC Mode | M |  | ENUMERATED ( RLC-AM, RLC-UM-Bidirectional, RLC-UM-Unidirectional-UL, RLC-UM-Unidirectionall-DL, …) |  |

### 9.2.120	Secondary RAT Usage Report List

This IE provides information on the NR resources used with EN-DC as specified in TS 37.340 [32].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Secondary RAT usage report Item |  | 1 .. < maxnoofbearers > |  |  | EACH | reject |
| >E-RAB ID | M |  | 9.2.23 |  | - |  |
| >Secondary RAT Type | M |  | ENUMERATED (nR, …, nR-unlicensed) |  | - |  |
| >E-RAB Usage Report List |  | 1 |  |  | - |  |
| >>E-RAB Usage Report Item |  | 1.. <maxnooftimeperiods> |  |  | EACH | ignore |
| >>>Start timestamp | M |  | OCTET STRING (SIZE(4)) | UTC time encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [35]. It indicates the start time of the collecting period of the included Usage Count UL IE and Usage Count DL IE. | - |  |
| >>>End timestamp | M |  | OCTET STRING (SIZE(4)) | UTC time encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [35]. It indicates the end time of the collecting period of the included Usage Count UL IE and Usage Count DL IE. | - |  |
| >>>Usage count UL | M |  | INTEGER (0..264-1) | The unit is: octets. | - |  |
| >>>Usage count DL | M |  | INTEGER (0..264-1) | The unit is: octets. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofbearers | Maximum no. of E-RABs. Value is 256. |
| maxnooftimeperiods | Maximum no. of time reporting periods. Value is 2. |

### 9.2.121	UE Application layer measurement configuration

The IE defines configuration information for the QoE Measurement Collection (QMC) function.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Container for application layer measurement configuration | M |  | OCTET STRING (1..1000) | Indicates application layer measurement configuration, see Annex L in [36]. | - |  |
| CHOICE Area Scope of QMC | M |  |  |  | - |  |
| >Cell based |  |  |  |  |  |  |
| >>Cell ID List for QMC |  | 1 .. <maxnoofCellIDforQMC> |  |  |  |  |
| >>>E-CGI | M |  | 9.2.1.38 |  | - |  |
| >TA based |  |  |  |  |  |  |
| >>TA List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  |  |  |
| >>>TAC | M |  | 9.2.3.7 | The TAI is derived using the current serving PLMN. | - |  |
| >TAI based |  |  |  |  | - |  |
| >>TAI List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  | - |  |
| >>>TAI | M |  | 9.2.3.16 |  | - |  |
| >PLMN area based |  |  |  |  |  |  |
| >>PLMN List for QMC |  | 1 .. <maxnoofPLMNforQMC> |  |  |  |  |
| >>>PLMN Identity | M |  | 9.2.3.8 |  | - |  |
| Service Type | M |  | ENUMERATED (QMC for streaming service, QMC for MTSI service, ...) | This IE indicates the service type of UE application layer measurements. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforQMC | Maximum no. of Cell ID subject for QMC scope. Value is 32. |
| maxnoofTAforQMC | Maximum no. of TA subject for QMC scope. Value is 8. |
| maxnoofPLMNforQMC | Maximum no. of PLMNs in the PLMN list for QMC scope. Value is 16. |

### 9.2.122	DRB ID

This information element uniquely identifies a DRB over the X2 interface within an en-gNB.

The usage of this IE is defined in TS 36.331 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB ID | M |  | INTEGER (1.. 32) |  |

### 9.2.123	SUL Information

This IE provides information about the SUL carrier.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SUL ARFCN | M |  | INTEGER (0..maxNRARFCN) | RF Reference Frequency as defined in TS 38.104 [37] section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the SUL carrier. Its lowest subcarrier is also known as Point A. | – |  |
| SUL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.114 |  | – |  |
| Carrier List | O |  | NR Carrier List 9.2.168 | If included, the SUL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| Frequency Shift 7p5khz | O |  | ENUMERATED (false, true, ...) | Indicate whether the value of Δshift is 0kHz or 7.5kHz when calculating FREF,shift as defined in Section 5.4.2.1 of TS 38.104 [37]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxNRARFCN | Maximum value of NRARFCNs. Value is 3279165. |

### 9.2.124	Packet Loss Rate

This IE indicates the packet loss rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Packet Loss Rate | M |  | INTEGER(0..1000) | Ratio of lost packets per number of packets sent, expressed in tenth of percent. | - | - |

### 9.2.125	Protected E-UTRA Resource Indication

This IE indicates the resources allocated for E-UTRA DL and UL reference and control signals (hereby referred to as protected resources). This information is used in the process of E-UTRA – NR Cell Resource Coordination.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Activation SFN | M |  | INTEGER (0..1023) | Indicates from which SFN of the receiving node the resource allocation is valid. |  |  |
| Protected Resource List |  | 1 |  | The protected resource pattern is continuously repeated, and it is valid until stated otherwise or until replaced by a new pattern. The pattern does not apply in reserved subframes. | YES | ignore |
| >Protected Resource List Item |  | 1..<maxnoofProtectedResourcePatterns> |  | Each item describes one transmission pattern. A pattern may comprise several control signals. | - |  |
| >>Resource Type | M |  | ENUMERATED (downlinknonCRS,CRS,uplink…) | Indicates whether the protected resource is E-UTRA DL non-CRS, E-UTRA CRS or E-UTRA UL. | - |  |
| >>Intra-PRB Protected Resource Footprint | M |  | BIT STRING (84, ...) | The bitmap of REs occupied by the protected signal within one PRB. Each position in the bitmap represents an RE in one PRB; value "0" indicates "resource not protected", value "1" indicates "resource protected". The first bit of the string corresponds to the RE with the smallest time and frequency index in the PRB, where the indexing first goes into the frequency domain. The length of the bit string equals the product of NRBSC and the length of PRB in time dimension, measured in REs. NRBSC is defined in TS 36.211 [10]. The intra-PRB pattern consisting of all "1"s is equivalent to PRB-level granularity. |  |  |
| >>Protected Footprint Frequency Pattern | M |  | BIT STRING(6..110, ...) | The bit string indicates in which PRBs inside carrier bandwidth the Intra-PRB Protected Resource Footprint applies. How often in time dimension this frequency pattern applies, depends on time periodicity of Intra-PRB Protected Resource Footprint. The first bit of the bit string corresponds to the PRB occupying the lowest subcarrier frequencies of the carrier bandwidth, where the indexing first goes into the frequency domain. Each position in the string represents a PRB; value "0" indicates " Intra-PRB Protected Resource Footprint does not appear in PRB", value "1" indicates "Intra-PRB Protected Resource Footprint appears in PRB". The length of the bit string equals the number of PRBs in the carrier bandwidth. | - |  |
| >>Protected Footprint Time Pattern | M |  |  | The description of time periodicity of the Intra-PRB Protected Resource Footprint. |  |  |
| >>>Protected Footprint Time-periodicity | M |  | INTEGER(1..320, ...) | Periodicity with which the periodic Intra-PRB Protected Resource Footprint repeats in time-dimension (1= every PRB (i.e. slot), 2=every other PRB (i.e. slot) etc. | - |  |
| >>>Protected Footprint Start Time | M |  | INTEGER(1..20, ...) | The time-position of the PRB inside the frame in which the periodic Intra-PRB Protected Resource Footprint appears for the first time. The value "1" corresponds to the receiving node’s slot 0 in subframe 0 in the receiving node’s radio frame where SFN = Activation SFN. | - |  |
| MBSFN Control Region Length | O |  | INTEGER(0..3) | Length of control region in MBSFN subframes. Expressed in REs, in the time dimension. |  |  |
| PDCCH Region Length | M |  | INTEGER(1..3) | Length of PDCCH region in regular subframes. Expressed in REs, in the time dimension. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofProtectedResourcePatterns | Maximum no. protected resource patterns. Value is 16. |

### 9.2.126	Data Traffic Resource Indication

This IE indicates the intended data traffic resource allocation for E-UTRA - NR Cell Resource Coordination.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Activation SFN | M |  | INTEGER (0..1023) | Indicates from which SFN of the receiving node the agreement is valid. |  |  |
| CHOICE Shared Resource Type | M |  |  |  | - |  |
| >UL Only Sharing |  |  |  |  |  |  |
| >>UL Resource Bitmap | M |  | Data Traffic Resources 9.2.127 |  | - |  |
| >UL and DL Sharing |  |  |  |  |  |  |
| >>CHOICE UL Resources | M |  |  |  |  |  |
| >>>Unchanged |  |  | NULL |  |  |  |
| >>>Changed |  |  |  |  |  |  |
| >>>>UL Resource Bitmap | M |  | Data Traffic Resources 9.2.127 |  |  |  |
| >>CHOICE DL Resources | M |  |  |  |  |  |
| >>>Unchanged |  |  | NULL |  |  |  |
| >>>Changed |  |  |  |  |  |  |
| >>>>DL Resource Bitmap | M |  | Data Traffic Resources 9.2.127 |  |  |  |
| Reserved Subframe Pattern | O |  | 9.2.128 | Indicates subframes in which the resource allocation does not hold. |  |  |

### 9.2.127	Data Traffic Resources

The Data Traffic Resources IE indicates the intended data traffic resource allocation for E-UTRA - NR Cell Resource Coordination.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Traffic Resources | M |  | BIT STRING (6..17600) | The indication of resources allocated to E-UTRA PDSCH/PUSCH. Each position in the bit string represents a PRB pair in a subframe; value "0" indicates "resource not intended to be used for transmission", value "1" indicates "resource intended to be used for transmission ". The first bit of the bit string corresponds to the PRB pair occupying the lowest subcarrier frequencies of the carrier, where the indexing first goes into the frequency domain. The bit string may span across multiple contiguous subframes. The first position of the Data Traffic Resources corresponds to the receiving node’s subframe 0 in a receiving node’s radio frame where SFN = Activation SFN. The length of the bit string is an integer multiple of NRBDL or NRBUL, defined in TS 36.211 [10]. |

### 9.2.128	Reserved Subframe Pattern

The Reserved Subframe Pattern IE indicates the pattern of subframes in which the Protected E-UTRA Resource Indication and Data Traffic Resource Indication do not hold.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subframe Type | M |  | ENUMERATED(MBSFN,non-MBSFN, …) | Indicates what type of non-regular subframes the Reserved Subframe Pattern refers to (e.g. MBSFN). |
| Reserved Subframe Pattern | M |  | BIT STRING (10..160) | Each position in the bitmap represents a subframe. Value ‘0’ indicates "regular subframe". Value '1' indicates "reserved subframe". For MBSFN subframes, the exception refers only to the non-control region of the subframe. The bit string may span across multiple contiguous subframes. The first position of the Subframe Configuration IE corresponds to the receiving node's subframe 0 in a receiving node's radio frame where SFN = Activation SFN. The IE is ignored if received by the eNB. |
| MBSFN Control Region Length | O |  | INTEGER(0..3) | Length of control region in MBSFN subframes. Expressed in REs, in the time dimension. |

### 9.2.129	Aerial UE subscription information

This information element is used by the eNB to know if the UE is allowed to use aerial UE function, refer to TS 23.401[12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Aerial UE subscription information | M |  | ENUMERATED (allowed, not allowed, ...) |  |

### 9.2.130	User plane traffic activity report

This IE is used to indicate user plane traffic activity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| User plane traffic activity report | M |  | ENUMERATED (inactive, re-activated, …) | "re-activated" shall be only set after "inactive" has been reported for the concerned reporting object |

### 9.2.131	RLC Status

This IE indicates about the RLC configuration change included in the container towards the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reestablishment Indication | O |  | ENUMERATED (reestablished, ...) | Indicates that following the change of the radio status, the RLC has been re-established. |

### 9.2.132	RRC config indication

This IE is used to indicate the type of RRC configuration used at the en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RRC config indication | M |  | ENUMERATED (full config, delta config, ...) |  |

### 9.2.133	PDCP SN Length

The PDCP SN Length IE is used to indicate the PDCP SN length configuration of the bearer.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PDCP SN Length | M |  | ENUMERATED (12bits, 18bits, …) | This IE indicates the PDCP sequence number size. |

### 9.2.134	Bluetooth Measurement Configuration

This IE defines the parameters for Bluetooth measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bluetooth Measurement Configuration | M |  | ENUMERATED (Setup, …) |  |
| Bluetooth Measurement Configuration Name List |  | 0..1 |  | This IE is present if the Bluetooth Measurement Configuration IE is set to "Setup". |
| >Bluetooth Measurement Configuration Name Item |  | 1 .. <maxnoofBluetoothName> |  |  |
| >>Bluetooth Measurement Configuration Name | M |  | OCTET STRING (SIZE (1..248)) |  |
| BT RSSI | O |  | ENUMERATED (True, …) | In case of Immediate MDT, it corresponds to M8 measurement as defined in 37.320 [31]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofBluetoothname | Maximum no. of Bluetooth local name used for Bluetooth measurement collection, the maximum value is 4. |

### 9.2.135	WLAN Measurement Configuration

This IE defines the parameters for WLAN measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| WLAN Measurement Configuration | M |  | ENUMERATED (Setup, …) |  |
| WLAN Measurement Configuration Name List |  | 0..1 |  | This IE is present if the WLAN Measurement Configuration IE is set to "Setup". |
| >WLAN Measurement Configuration Name Item |  | 1 .. <maxnoofWLANName> |  |  |
| >>WLAN Measurement Configuration Name | M |  | OCTET STRING (SIZE (1..32)) |  |
| WLAN RSSI | O |  | ENUMERATED (True, …) | In case of Immediate MDT, it corresponds to M8 as defined in 37.320 [31]. |
| WLAN RTT | O |  | ENUMERATED (True, …) | For Immediate MDT, it corresponds to M9 as defined in 37.320 [31]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofWLANname | Maximum no. of WLAN SSID used for WLAN measurement collection, the maximum value is 4. |

### 9.2.136	Subscription Based UE Differentiation Information

This IE is generated by the MME based on the UE subscription information, it provides the Subscription Based UE differentiation Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodic Communication Indicator | O |  | ENUMERATED(periodically, on demand, …) | This IE indicates whether the UE communicates periodically or not, e.g. only on demand. |
| Periodic Time | O |  | INTEGER (1..3600, …) | This IE indicates the interval time of periodic communication, the unit is: second |
| Scheduled Communication Time |  | 0..1 |  | This IE indicates the time zone and day of the week when the UE is available for communication. |
| >Day of Week | O |  | BIT STRING (SIZE(7)) | If Day-Of-Week is not provided this shall be interpreted as every day of the week. Each position in the bitmap represents a day of the week: first bit = Mon, second bit =Tue, third bit =Wed, and so on. Value ‘1’ indicates ‘scheduled. Value ‘0’ indicates ‘not scheduled’. |
| >Time of Day Start | O |  | INTEGER (0..86399, …) | This IE indicates the time to start of the day, each value represent the corresponding second since 00:00 of the day. If Time-Of-Day-Start is not provided, starting time shall be set to start of the day(s) indicated by Day-Of-Week. |
| >Time of Day End | O |  | INTEGER (0..86399, …) | This IE indicates the time to start of the day, each value represent the corresponding second since 00:00 of the day. The value of this IE should be bigger than the value of Time of Day Start IE. If Time-Of-Day-End is not provided, ending time is end of the day(s) indicated by Day-Of-Week. |
| Stationary Indication | O |  | ENUMERATED(stationary, mobile, …) |  |
| Traffic Profile | O |  | ENUMERATED(single packet, dual packets, multiple packets, …) | "single packet" indicates single packet transmission (UL or DL), "dual packets" indicates dual packet transmission (UL with subsequent DL, or DL with subsequent UL), "multiple packets" indicates multiple packets transmission. |
| Battery Indication | O |  | ENUMERATED(battery powered, battery powered not rechargeable or replaceable, not battery powered, …) | "battery powered" indicates that the UE is battery powered and the battery is rechargeable/replaceable, "battery powered not rechargeable or replaceable" indicates that the UE is battery powered but the battery is not rechargeable/replaceable, "not battery powered" indicates that the UE is not battery powered. |

### 9.2.137	Duplication activation

The Duplication Activation IE indicates whether UL PDCP Duplication is activated or not.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Duplication Activation | M |  | ENUMERATED ( Active, Inactive, …) |  |

### 9.2.138	LCID

This IE uniquely identifies a LCID for the associated DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LCID | M |  | INTEGER (1..32, ...) | Corresponds to information provided in the LogicalChannelIdentity IE as defined in TS 38.331 [8]. |

### 9.2.139	MeNB Coordination Assistance Information

The MeNB Coordination Assistance Information IE is provided by the MeNB and used by the SgNB to determine further coordination of resource utilisation between the en-gNB and the MeNB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MeNB Coordination Assistance Information | M |  | ENUMERATED(Coordination Not Required, …) | The absence of this IE indicates that the resource coordination is required. |

### 9.2.140	SgNB Coordination Assistance Information

The SgNB Coordination Assistance Information IE is provided by the SgNB and used by the MeNB to determine further coordination of resource utilisation between the en-gNB and the MeNB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SgNB Coordination Assistance Information | M |  | ENUMERATED(Coordination Not Required, …) | The absence of this IE indicates that the resource coordination is required. |

### 9.2.141	Desired Activity Notification Level

This IE contains information on which level activity notification shall be performed.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Desired Activity Notification Level | O |  | ENUMERATED (None, E-RAB, UE-level, …) |  |

### 9.2.142	Location Information at SgNB

The Location Information at SgNB IE enables the SgNB to provide the MeNB with information that supports localisation of the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PSCell ID | M |  | NR CGI 9.2.111 | PSCell of the UE | – |  |

### 9.2.143	Interface Instance Indication

The Interface Instance Indication identifies the interface instance the X2AP message is destined for.

NOTE:	The Interface Instance Indication is allocated so that it can be associated with an X2-C interface instance. The Interface Instance Indication may identify more than one interface instance.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Interface Instance Indication | M |  | INTEGER (0..255, ...) |  |

### 9.2.144	NB-IoT UL DL Alignment Offset

This IE is used to indicate the offset between the UL carrier frequency center with respect to DL carrier frequency center.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NB-IoT UL DL Alignment Offset | M |  | ENUMERATED (-7.5, 0, 7.5, …) | Unit: kHz Corresponds to information provided in the TDD-UL-DL-AlignmentOffset-NB as specified in TS 36.331 [9]. |

### 9.2.145	Lower Layer presence status change

This IE is used to indicate that lower layer resources’ presence status shall be changed. If the presence status is set to "release lower layers" or "suspend lower layers", PDCP entities, X2-U bearer resources, S1-U bearer resources and UE context information shall be kept.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Lower Layer presence status change | M |  | ENUMERATED (release lower layers, re-establish lower layers, suspend lower layers, resume lower layers ...) | "re-establish lower layers" shall be only set after "release lower layers" has been indicated. "resume lower layers" shall restore SCG. "resume lower layers" shall be only set after "suspend lower layers" has been indicated. |

### 9.2.146	Cell and Capacity Assistance Information

The Cell and Capacity Assistance Information IE is used by the eNB to request information about NR or E-UTRA cells and it includes information about cell list size capacity.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Maximum Cell List Size | O |  | 9.2.147 |  |
| Cell Assistance Information | O |  | 9.2.115 |  |

### 9.2.147	Maximum Cell List Size

This IE indicates the maximum size the sending node can handle for a given list.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Maximum Cell List Size | M |  | INTEGER (0..16384, …) |  |

### 9.2.148	Message Oversize Notification

This IE indicates the maximum number of cells that can be received in the List of Served Cells NR IE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum Cell List Size | M |  | 9.2.147 |  |  |  |

### 9.2.149	TNL Transport Layer Address Info

This IE is used for signalling IP addresses of IP-Sec endpoints used for establishment of IP-Sec tunnels.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transport UP Layer Addresses Info to Add List |  | 0..1 |  |  |
| >Transport UP Layer Addresses Info to Add Item |  | 1..<maxnoofTLAs> |  |  |
| >>IP-Sec Transport Layer Address | M |  | BIT STRING(1..160, ...) | Transport Layer Addresses for IP-Sec endpoint. |
| >>GTP Transport Layer Addresses To Add List |  | 0..1 |  |  |
| >>>GTP Transport Layer Addresses To Add Item |  | 1..<maxnoofGTPTLAs> |  |  |
| >>>>GTP Transport Layer Address Info | M |  | BIT STRING (1..160, ...) | GTP Transport Layer Addresses for GTP end-points. |
| Transport UP Layer Addresses Info to Remove List |  | 0..1 |  |  |
| >Transport UP Layer Addresses Info to Remove Item |  | 1..<maxnoofTLAs> |  |  |
| >>IP-Sec Transport Layer Address | M |  | BIT STRING (1..160, ...) | Transport Layer Addresses for IP-Sec endpoint. |
| >>GTP Transport Layer Addresses To Remove List |  | 0..1 |  |  |
| >>>GTP Transport Layer Addresses To Remove Item |  | 1..<maxnoofGTPTLAs> |  |  |
| >>>>GTP Transport Layer Address Info | M |  | BIT STRING (1..160, ...) | GTP Transport Layer Addresses for GTP end-points. |

| Range bound | Explanation |
| --- | --- |
| maxnoofTLAs | Maximum no. of Transport Layer Addresses in the message. Value is 16 |
| maxnoofGTPTLAs | Maximum no. of GTP Transport Layer Addresses for a GTP end-point in the message. Value is 16. |

### 9.2.150	CP Transport Layer Information

This element is used to provide the transport layer information associated with EN-DC X2 control plane transport.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE CP Transport Layer Information |  |  |  |  |
| >Endpoint-IP-address |  |  |  |  |
| >>Endpoint IP Address | M |  | BIT STRING (1..160, ...) |  |
| >Endpoint-IP-address-and-port |  |  |  |  |
| >>Endpoint IP Address | M |  | BIT STRING (1..160, ...) |  |
| >>Port Number | M |  | OCTET STRING (SIZE(2)) |  |

### 9.2.151	TNL Association Usage

This IE indicates the usage of the TNL association.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TNL Association Usage | O |  | ENUMERATED (ue, non-ue, both, …) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. |

### 9.2.152	RAN UE NGAP ID

This IE uniquely identifies the UE association over the NG interface within the NG-RAN node, as specified in TS 38.413 [39].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAN UE NGAP ID | M |  | INTEGER (0..232 -1) |  |

### 9.2.153	EPC Handover Restriction List Container

This IE contains the Handover Restriction List IE specified in TS 36.413 [4] as received by the E-UTRAN from the EPC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| EPC Handover Restriction List Container | M |  | OCTET STRING | The octets of the OCTET STRING are encoded according to the specifications of the Handover Restriction List IE specified in TS 36.413 [4]. |

### 9.2.154	DAPS Request Information

The DAPS Indicator IE indicates that the source eNB requests a DAPS HO for the concered E-RAB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DAPS Indicator | M |  | ENUMERATED (DAPS HO required, …) | Indicates that DAPS HO is requested |

### 9.2.155	DAPS Response Information

The DAPS Response Indicator IE indicates the response to a requested DAPS Handover for the concerned E-RAB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DAPS Response Indicator | M |  | ENUMERATED (DAPS HO accepted, DAPS HO not accepted, …) | Indicates that DAPS Handover is accepted or not. |

### 9.2.156	Maximum Number of CHO Preparations

This IE indicates the maximum number of concurrently prepared CHO candidate cells for a UE at a candidate target eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Maximum Number of CHO Preparations | M |  | INTEGER (1..8, ...) |  |

### 9.2.157	Ethernet Type

This IE is used to indicate that Ethernet data is expected.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Ethernet Type | M |  | ENUMERATED (True, …) |  |

### 9.2.158	NR V2X Services Authorized

This IE provides information on the authorization status of the UE to use the NR sidelink for V2X services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Vehicle UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized as Vehicle UE. |
| Pedestrian UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized as Pedestrian UE. |

### 9.2.159	NR UE Sidelink Aggregate Maximum Bit Rate

This IE provides information on the Aggregate Maximum Bitrate of the UE’s sidelink communication for NR V2X services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR UE Sidelink Aggregate Maximum Bit Rate | M |  | Bit Rate 9.2.97 | Value 0 shall be considered as a logical error by the receiving eNB. |

### 9.2.160	PC5 QoS Parameters

This IE provides information on the PC5 QoS parameters of the UE’s sidelink communication for NR PC5.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PC5 QoS Flow List |  | 1 |  |  |
| >PC5 QoS Flow Item |  | 1..<maxnoofPC5QoSFlows> |  |  |
| >>PQI | M |  | INTEGER (0..255, …) | PQI is a special 5QI as specified in TS 23.501 [9]. |
| >>PC5 Flow Bit Rates | O |  |  | Only applies for GBR QoS Flows. |
| >>>Guaranteed Flow Bit Rate | M |  | Bit Rate 9.2.11 | Guaranteed Bit Rate for the PC5 QoS flow. Details in TS 23.501 [9]. |
| >>>Maximum Flow Bit Rate | M |  | Bit Rate 9.2.11 | Maximum Bit Rate for the PC5 QoS flow. Details in TS 23.501 [9]. |
| >>Range | O |  | ENUMERATED (m50, m80, m180, m200, m350, m400, m500, m700, m1000, …) | Only applies for groupcast. |
| PC5 Link Aggregated Bit Rates | O |  | Bit Rate 9.2.11 | Only applies for non-GBR QoS Flows. |

| Range bound | Explanation |
| --- | --- |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flows allowed towards one UE. Value is 2048. |

### 9.2.161	TNL Capacity Indicator

The TNL Capacity Indicator IE indicates the available capacity of the Transport Network experienced by the NR cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL TNL Maximum Offered Capacity | M |  | INTEGER (1.. 16777216,...) | Maximum capacity offered by the transport portion of the cell in kbps |
| DL TNL Available Capacity | M |  | INTEGER (0.. 100,...) | Available capacity over the transport portion serving the cell in percentage relative to the DL TNL Maximum Offered Capacity. Value 100 corresponds to the Maximum offered capacity. |
| UL TNL Maximum Offered Capacity | M |  | INTEGER (1.. 16777216…) | Maximum capacity offered by the transport portion of the cell in kbps |
| UL TNL Available Capacity | M |  | INTEGER (0.. 100,...) | Available capacity over the transport portion serving the cell in percentage relative to the DL TNL Maximum Offered Capacity. Value 100 corresponds to the Maximum offered capacity. |

### 9.2.162	NR Radio Resource Status

The NR Radio Resource Status IE indicates the usage of the PRBs per cell for MIMO and per SSB area for all traffic in Downlink and Uplink and the usage of PDCCH CCEs for Downlink and Uplink scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SSB Area Radio Resource Status List |  | 1 |  |  |
| >SSB Area Radio Resource Status Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>SSB Index | M |  | 9.2.167 |  |
| >>SSB Area DL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL GBR PRB usage |
| >>SSB Area UL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL GBR PRB usage |
| >>SSB Area DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL non-GBR PRB usage |
| >>SSB Area UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL non-GBR PRB usage |
| >>SSB Area DL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area DL Total PRB usage |
| >>SSB Area UL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area UL Total PRB usage |
| >>DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| >>UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| MIMO PRB usage Information | O |  |  |  |
| >DL GBR PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell DL GBR PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [45]. |
| >UL GBR PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell UL GBR PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [45]. |
| >DL non-GBR PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell DL non-GBR PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [45]. |
| >UL non-GBR PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell UL non-GBR PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [45]. |
| >DL Total PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell DL Total PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [45]. |
| >UL Total PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell UL Total PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [45]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

### 9.2.163	NR Composite Available Capacity Group

The NR Composite Available Capacity Group IE indicates the overall available resource level per cell and per SSB area in the cell in Downlink and Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Composite Available Capacity Downlink | M |  | NR Composite Available Capacity 9.2.164 | For the Downlink |
| Composite Available Capacity Uplink | M |  | NR Composite Available Capacity 9.2.164 | For the Uplink |

### 9.2.164	NR Composite Available Capacity

The NR Composite Available Capacity IE indicates the overall available resource level in the cell in either Downlink or Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Capacity Class Value | O |  | NR Cell Capacity Class Value 9.2.165 |  |
| Capacity Value | M |  | NR Capacity Value 9.2.166 |  |

### 9.2.165	NR Cell Capacity Class Value

The NR Cell Capacity Class Value IE indicates the value that classifies the cell capacity with regards to the other cells. The NR Cell Capacity Class Value IE only indicates resources that are configured for traffic purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR Capacity Class Value | M |  | INTEGER (1..100,...) | Value 1 shall indicate the minimum cell capacity, and 100 shall indicate the maximum cell capacity. There should be a linear relation between cell capacity and Cell Capacity Class Value. |

### 9.2.166	NR Capacity Value

The NR Capacity Value IE indicates the amount of resources per cell and per SSB area that are available relative to the total en-gNB resources. The capacity value should be measured and reported so that the minimum en-gNB resource usage of existing services is reserved according to implementation. The NR Capacity Value IE can be weighted according to the ratio of cell capacity class values, if available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity with respect to the whole cell. Capacity Value should be measured on a linear scale. |
| SSB Area Capacity Value List |  | 0..1 |  |  |
| >SSB Area Capacity Value Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>SSB Index | M |  | 9.2.167 |  |
| >>SSB Area Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity . SSB Area Capacity Value should be measured on a linear scale. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

### 9.2.167	SSB Index

The SSB Index IE identify an SSB area of an NR cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SSB Index | M |  | INTEGER (0..63) |  |

### 9.2.168	NR Carrier List

This IE indicates the SCS-specific carriers per TDD, per DL, per UL or per SUL of an NR cell.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR Carrier Item |  | 1..<maxnoofNRSCSs> |  |  |
| >NR SCS | M |  | ENUMERATED (scs15, scs30, scs60, scs120, …) | SCS for the corresponding carrier. |
| >Offset to Carrier | M |  | INTEGER (0.. 2199, ...) | Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the NR SCS IE defined for this carrier). The maximum value corresponds to 275×8−1. See TS 38.211 [42], clause 4.4.2. |
| >Carrier Bandwidth | M |  | INTEGER (1.. maxnoofNRPhysicalResourceBlocks, ...) | Width of this carrier in number of PRBs (using the NR SCS IE defined for this carrier). See TS 38.211 [42], clause 4.4.2. |

| Range bound | Explanation |
| --- | --- |
| maxnoofNRSCSs | Maximum no. of SCS-specific carriers per TDD, per DL, per UL or per SUL of an NR cell. Value is 5. |
| maxnoofNRPhysicalResourceBlocks | Maximum no. of Physical Resource Blocks of an NR Cell. Value is 275. |

### 9.2.169	SSB Positions In Burst

Indicates the time domain positions of the transmitted SS-blocks in a half frame with SS/PBCH blocks as defined in TS 38.213 [43], clause 4.1.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE ssb-PositionsInBurst | M |  |  | The first/ leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted. |
| >ShortBitmap |  |  |  |  |
| >>ShortBitmap | M |  | BIT STRING (SIZE(4)) |  |
| >MediumBitmap |  |  |  |  |
| >>MediumBitmap | M |  | BIT STRING (SIZE(8)) |  |
| >LongBitmap |  |  |  |  |
| >>LongBitmap | M |  | BIT STRING (SIZE(64)) |  |

### 9.2.170	NPRACH Configuration

This IE indicates the NPRACH Configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE FDDorTDD | M |  |  |  |
| >FDD |  |  |  |  |
| >>NPRACH-CP-Length | M | ENUMERATED {us66dot7, us266dot7, …} |  |  |
| >>Anchor Carrier NPRACH Configuration | M |  | OCTET STRING | Includes the NPRACH-ParametersList-NB-r13 IE as defined in 6.7.3.2 of TS 36.331 [9]. |
| >>Anchor Carrier EDT NPRACH Configuration | O |  | OCTET STRING | Includes the NPRACH-ParametersList-NB-r14 IE as defined in 6.7.3.2 of TS 36.331 [9]. |
| >>Anchor Carrier Format 2 NPRACH Configuration | O |  | OCTET STRING | Includes the NPRACH-ParametersListFmt2-NB-r15 IE as defined in 6.7.3.2 of TS 36.331 [9] |
| >>Anchor Carrier Format 2 EDT NPRACH Configuration | O |  | OCTET STRING | Includes the NPRACH-ParametersListFmt2-NB-r15 IE as defined in 6.7.3.2 of TS 36.331 [9] |
| >>Non Anchor Carrier NPRACH Configuration | O |  | OCTET STRING | Includes the UL-ConfigCommonList-NB-r14 IE as defined in 6.7.3.1 of TS 36.331 [9]. |
| >>Non Anchor Carrier Format 2 NPRACH Configuration | O |  | OCTET STRING | Includes the UL-ConfigCommonList-NB-v1530 IE as defined in 6.7.3.1 of TS 36.331 [9]. |
| >TDD |  |  |  |  |
| >>nprach-PreambleFormat | M | ENUMERATED {fmt0, fmt1, fmt2, fmt0-a, fmt1-a, …} |  |  |
| >>Anchor Carrier NPRACH Configuration TDD | M |  | OCTET STRING | Includes the NPRACH-ParametersListTDD-NB-r15 IE as defined in 6.7.3.2 of TS 36.331 [9] |
| >>Non Anchor Carrier Frequency Configuration list |  | 0..< maxnoofNonAnchorCarrierFreqConfig> |  |  |
| >>>Non Anchor Carrier Frequency | M |  | OCTET STRING | Includes the DL-CarrierConfigCommon-NB-r14 IE as defined in 6.7.3.2 of TS 36.331 [9] |
| >>Non Anchor Carrier NPRACH Configuration TDD | O |  | OCTET STRING | Includes the UL-ConfigCommonListTDD-NB-r15 IE as defined in 6.7.3.1 of TS 36.331 [9]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofNonAnchorCarrierFreqConfig | Maximum no. of non-Anchor Carrier Frequency Configurations. Value is 15. |

### 9.2.171	UE Radio Capability ID

This IE contains the UE Capability ID as defined in TS 23.003[29].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Radio Capability ID | M |  | OCTET STRING |  |

### 9.2.172	QoS Mapping Information

This IE indicates the DSCP and/or IPv6 Flow Label field(s) of IP packets sent in the corresponding GTP-U tunnel for IAB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DSCP | O |  | BIT STRING (SIZE(6)) |  |
| Flow label | O |  | BIT STRING (SIZE(20)) |  |

### 9.2.173	UE Radio Capability

This IE contains UE Radio Capability information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Radio Capability | M |  | OCTET STRING | Includes the UERadioAccessCapabilityInformation message as defined in 10.2.2 of TS 36.331 [9]. |

### 9.2.174	URI

This IE is a URI.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| URI | M |  | VisibleString | String representing URI (Uniform Resource Identifier) |

### 9.2.175	SFN Offset

This IE contains the time offset between an absolute time reference and the SFN0 start. The IE is calculated assuming that the SFN transmission started at the absolute time reference. The absolute time reference chosen is 1980-01-06 T00:00:19 International Atomic Time (TAI).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SFN Time Offset | M |  | BIT STRING (SIZE(24)) | Time offset in microseconds between the absolute time reference "1980-01-06 T00:00:19 International Atomic Time (TAI)” and the SFN0 start. The maximum usable value is (1024*10^4-1). Values higher than the maximum are discarded. |  |  |

### 9.2.176	Global RAN Node ID

This IE is used to globally identify an NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE NG-RAN node | M |  |  |  |
| >gNB |  |  |  |  |
| >>Global gNB ID | M |  | Global en-gNB ID 9.2.112 |  |

### 9.2.177	SCG UE History Information

This IE contains information about the PSCells served by the secondary node in an active state.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerSN> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. |
| >Last Visited PSCell Information | M |  | OCTET STRING | The PSCell related information. Defined in TS 36.413 [4]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerSN | Maximum number of last visited PSCell information records that can be reported in the IE. Value is 8 |

### 9.2.178	SCG Activation Status

The SCG Activation Status IE indicates the status of the SCG resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SCG Activation Status | M |  | ENUMERATED (SCG activated, SCG deactivated, ...) |  |

### 9.2.179	SCG Activation Request

The SCG Activation Request IE indicates whether the SCG resources are required to be activated or deactivated.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SCG Activation Request | M |  | ENUMERATED (Activate SCG, Deactivate SCG, ...) |  |

### 9.2.180	Served Cell Specific Info Request

The Served Cell Specific Info Request IE is used by the NG-RAN node to request specific information about NR cells.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| List of Requested NR Cells |  | 1 |  | List of NR cells. |
| >List of Requested NR Cells item |  | 1 .. < maxCellinengNB> |  |  |
| >>NR CGI | M |  | 9.2.111 | NR cell for which specific served NR cell information is requested. |
| >>Additional Measurement Timing Configuration List Request Indicator | O |  | ENUMERATED (AdditionalMTCListRequested, …) | Included when the NG-RAN node requests the Additional Measurement Timing Configuration List IE to be included in the Served Cell Information NR IE for the requested cells. |

| Range bound | Explanation |
| --- | --- |
| maxCellinengNB | Maximum no. cells that can be served by an en-gNB. Value is 16384. |

### 9.2.181	Security Indication

This IE contains the user plane integrity protection indication which indicates the requirements on UP integrity protection for the corresponding E-RAB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Integrity Protection Indication | M |  | ENUMERATED (required, preferred, not needed, …) | Indicates whether UP integrity protection shall apply, should apply, or shall not apply for the concerned E-RAB. |

### 9.2.182	Security Result

This IE indicates whether the security policy indicated as "preferred" in the Integrity Protection Indication IE is performed or not.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Integrity Protection Result | M |  | ENUMERATED (performed, not performed, …) | Indicates whether UP integrity protection is performed or not for the concerned E-RAB. |

### 9.2.183	Sensor Measurement Configuration

This IE defines the parameters for Sensor measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Sensor Measurement Configuration | M |  | ENUMERATED (Setup, …) |  |
| Sensor Measurement Configuration Name List |  | 0..1 |  |  |
| >Sensor Measurement Configuration Name Item |  | 1..<maxnoofSensorName> |  |  |
| >>CHOICE Sensor Name | M |  |  |  |
| >>>Uncompensated Barometric |  |  |  |  |
| >>>>Uncompensated Barometric Configuration | M |  | ENUMERATED (true, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSensorName | Maximum no. of Sensor local name used for Sensor measurement collection. Value is 3. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>