3GPP TSG-RAN WG3 Meeting #119	R3-230572

Athens, GR, 27 Feb – 03 Mar, 2023

Title: 	(TPs for SON BLCR for TS 38.473): Remaining issues for RACH optimisation

Source: 	Huawei

Agenda item:	10.2.3

Document Type:	Other

1. Introduction

In the RAN3#118 meeting, there were some progresses for RACH enhancement as the following:

RAN3 supports a network-based solution for RACH report retrieval over F1AP based on an indication from the gNB-DU to the gNB-CU of successful RACH procedures which are not known to the gNB-CU (e.g., when RACH is triggered due to beam failure recovery, no PUCCH resource available, UL sync issue)

Define a new class-2 F1AP message (e.g., RACH INDICATION) to indicate certain RACH occurrence(s) from gNB-DU to gNB-CU

FFS whether the new F1AP message is UE-associated or non-UE associated

SN should indicate the potential availability of RA report to the MN, MN can fetch the RA report and transfer it to SN

Define a new class-2 message (e.g., RACH INDICATION) over Xn so that the S-NG-RAN can inform M-NG-RAN that one or more RACH reports are available at the UE.

The new Xn message should be non-UE associated.

Proposal 7: Agree the XnAP and F1AP TPs (with FFS as needed).

- TP to F1AP (E///): R3-226481 rev in R3-22xxxx

- TP to XnAP (HW): R3-226300 rev in R3-226858

In this paper, we give the corresponding TPs for the agreed solutions, and give potential solutions to the FFS to support the objective. Meanwhile, we prefer to give our further analysis for the LS [1] from RAN3 to RAN2.

2. Discussion

2.1 TPs for agreements

Since RAN3# has confirmed the support of XnAP and F1AP enhancement to enable network-based RA report retrieval. We also accordingly provide the TP to the BL CR of TS 38.300 and TS 38.401.

To agree on the TPs to SON BLCR for TS 38.300 and TS 38.401 to capture the agreed solutions in[1] and [2].

2.2 F1AP message

Also, it is noted that there is a leftover issue to be discussed from the last meeting

FFS whether the new F1AP message is UE-associated or non-UE associated

A common signalling framework in both F1AP and XnAP is expected. So the SN could directly use the RA occurrences informed over the F1AP to indicate the associated MN. It just makes a simple solution for the involved SN-DU and SN-CU. Since new XnAP message with non-UE associated message is already confirmed in RAN3, we propose to make F1AP align with XnAP. Besides, another reason for F1AP with non-UE associated message is that gNB-DU need information regarding random access events associated to multiple UE to optimize the RA configuration of gNB-DU. So it is natural for gNB-DU to report RA occurrences associated to multiple UEs one time, which also reduce the unnecessary signalling overhead.

To define a non-UE associated F1 procedure for UE RACH occurrence indication.

To agree on the TP for TS 38.473 in the Annex.

4. Conclusion

In this paper, we discuss miscellaneous enhancements for RACH enhancement, we have the following observations and proposals:

- To agree on the TPs to SON BLCR for TS 38.300 and TS 38.401 to capture the agreed solutions in[1] and [2].

- To define a non-UE associated F1 procedure for UE RACH occurrence indication.

- To agree on the TP for TS 38.473 in the Annex.

5. Reference

[1] R3-230573	(TPs for SON BLCRs for TS 38.300): RACH report retrieval,

[2] R3-230574	(TPs for SON BLCRs for TS 38.401): RACH report retrieval.

Annex – TP for SON BLCR for TS 38.473

Start of the next change

Table 2: Class 2 procedures

| Elementary Procedure |  | Message |  |
| --- | --- | --- | --- |
| Error Indication |  | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) |  | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer |  | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer |  | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer |  | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification |  | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery |  | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging |  | PAGING |  |
| Notify |  | NOTIFY |  |
| PWS Restart Indication |  | PWS RESTART INDICATION |  |
| PWS Failure Indication |  | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication |  | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report |  | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction |  | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |  |
| Deactivate Trace |  | DEACTIVATE TRACE |  |
| DU-CU Radio Information Transfer |  | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer |  | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting |  | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication |  | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |  |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |  |
| Access Success |  | ACCESS SUCCESS |  |
| Cell Traffic Trace |  | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control |  | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback |  | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report |  | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort |  | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication |  | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update |  | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation |  | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication |  | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report |  | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination |  | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update |  | POSITIONING INFORMATION UPDATE |  |
| Multicast Group Paging |  | MULTICAST GROUP PAGING |  |
| Broadcast Context Release Request |  | BROADCAST CONTEXT RELEASE REQUEST |  |
| Multicast Context Release Request |  | MULTICAST CONTEXT RELEASE REQUEST |  |
| PDC Measurement Report |  | PDC MEASUREMENT REPORT |  |
| PDC Measurement Termination |  | PDC MEASUREMENT TERMINATION COMMAND |  |
| PDC Measurement Failure Indication |  | PDC MEASUREMENT FAILURE INDICATION |  |
| PDC Measurement Termination |  | PDC MEASUREMENT TERMINATION COMMAND |  |
| Measurement Activation |  | MEASUREMENT ACTIVATION |  |
| QoE Information Transfer |  | QOE INFORMATION TRANSFER |  |
| Positioning System Information Delivery |  | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |  |
| RACH Indication |  | RACH INDICATION |  |

Start of the next change

## 8.2	Interface Management procedures

8.2.x	RACH Indication

8.2.x.1		General

The purpose of the RACH Indication procedure is to inform the gNB-CU of the occurrences of successful random access event in gNB-DU.

The procedure uses non-UE associated signalling.

8.2.x.2		Successful Operation

Figure 8.2.x.2-1: RACH Indication procedure.

The gNB-DU initiates the procedure by sending the RACH Indication message to gNB-CU.

The RACH INDICATION message contains the existence of one or more successful random access event occurring in the gNB-DU and the UE F1AP IDs for all associated UEs.

Upon reception of the RACH INDICATION message the gNB-CU may fetch the RACH report from the UE.

8.2.x.3 	Abnormal Conditions

Not applicable.

Start of the next change

9.2.1.x	RACH INDICATION

This message is sent by the S-NG-RAN node to inform the M-NG-RAN node that one or more RACH reports are available at the UE.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RACH Indication List |  | 1 |  |  | YES | ignore |
| >UE ID List Item |  | 1 .. <maxnoofRAEvents> |  |  | EACH | ignore |
| >> gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| >> gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAEvents | Maximum no. of successful random access events. Value is 1024. |

End of the change