3GPP TSG-RAN WG3 Meeting #119	R3-230966

Athens, GR, 27 Feb – 03 Mar, 2023

Title: 	(TP for SON BLCRs for TS 38.401): RACH report retrieval

Source: 	Huawei, Orange, China Telecom, CMCC, Ericsson

Agenda item:	10.2.3

Document Type:	Other

1. Introduction

The whole RACH report retrieval mechanism in CU DU split and MR-DC were agreed at last meeting. The following agreements are captured in chair minutes:

RAN3 supports a network-based solution for RACH report retrieval over F1AP based on an indication from the gNB-DU to the gNB-CU of successful RACH procedures which are not known to the gNB-CU (e.g., when RACH is triggered due to beam failure recovery, no PUCCH resource available, UL sync issue).

Define a new class-2 F1AP message (e.g., RACH INDICATION) to indicate certain RACH occurrence(s) from gNB-DU to gNB-CU.

FFS whether the new F1AP message is UE-associated or non-UE associated.

SN should indicate the potential availability of RA report to the MN, MN can fetch the RA report and transfer it to SN

Define a new class-2 message (e.g., RACH INDICATION) over Xn so that the S-NG-RAN can inform M-NG-RAN that one or more RACH reports are available at the UE.

The new Xn message should be non-UE associated.

It is proposed to agree on the stage 2 TP in Annex to reflect the agreements mentioned above.

Annex – TP for SON BLCR for TS 38.401

7.5	RACH Optimisation Function

The RACH Optimization Function in non-split gNB case is specified in TS 38.300 [2].

In case of split gNB architecture, RACH configuration conflict detection and resolution function is located at the gNB-DU. To perform RACH optimisation at gNB-DU, gNB-CU sends the RACH report reported by the UE to gNB-DU via F1AP signalling. The gNB-DU signals the PRACH configuration per-cell to gNB-CU. The gNB-CU may forward a limited set of neighbour cell’s PRACH configurations received from neighbour gNBs and other gNB-DUs to the gNB-DU to resolve the configuration conflict.

RA Report retrieval

When a UE performs successful random access attempts which are only known by the gNB-DU (e.g., beam failure recovery, UL synchronization issue, scheduling request failure, no PUCCH resource available), the gNB-DU may inform the gNB-CU about the occurrences of successful random access procedures in the gNB-DU via a RACH Indication. The gNB-CU may then retrieve the RA Report from the UE(s) based on the RACH indication received via F1AP signalling from gNB-DU.