3GPP TSG-RAN WG3 Meeting #119	R3-231026

Athens, GR, 27 Feb – 03 Mar, 2023

Title: 	(TP for L1L2Mob BLCR for TS 38.401): BLCR update with latest agreements for inter-DU LTM

Source: 	Huawei, Ericsson, Nokia, Nokia Shanghai Bell, ZTE, CMCC, Qualcomm Incorporated

Agenda item:	14.2

Document Type:	Other

1. Introduction

This document is a TP to stage 2 BLCR to reflect the following agreement and some other refinements on the wording:

#118 meeting

For inter-DU case, the target gNB-DU indicates the gNB-CU about the UE successful access to the target cell by Access Success message.

Annex – TP for LTM BLCR for TS 38.401

--------------------------------------------------------- Start of change -------------------------------------------------------------

#### 8.2.1.Y	Inter-gNB-DU L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for L1/L2 Triggered Mobility. Figure 8.2.1.Y-1 shows the inter-gNB-DU L1/L2 Triggered Mobility procedure for intra-NR.

Figure 8.2.1.Y-1: inter gNB-DU L1/L2 Triggered Mobility

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the source gNB-DU containing  measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate  L1/L2 triggered mobility configuration.

- The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing the target candidate cells.

FFS for Step 3 and Step 4: either a single or multiple UE Context Setup procedure(s) should be used.

- If the candidate gNB-DU accepts the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration for the accepted target candidate cell(s).

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the L1/L2 triggered mobility configuration.

FFS: whether it is DL RRC MESSAGE TRANSFER message or UE Context Modification Request message.

- The source gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

- The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

FFS: whether it is UL RRC MESSAGE TRANSFER message or UE Context Modification Response message.

- The UE sends the lower layer measurement result to the source gNB-DU.

- The source gNB-DU decides to execute L1/L2 triggered mobility to a candidate target cell.

FFS: Notifying the LTM triggering decision to the other nodes as well.

- The source gNB-DU sends LTM command to the UE.

Editor’s note:  The LTM command needs to be updated according to RAN2’s discussion.

- The source gNB-DU signals the gNB-CU about the initiation of the L1/L2 triggered mobility command to the UE including the Target cell ID.

FFS: introduce new message or reuse legacy message.

- FFS:  how the target gNB-DU detects the UE access and whether there is an F1 impact.

- The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

- FFS: For inter-DU L1/L2 Triggered Mobility, whether and how to release the source cell/prepared cells’ resources in the source gNB-DU is FFS.

--------------------------------------------------------- the end of change -------------------------------------------------------------