3GPP TSG-RAN3 Meeting #119	R3-230007

Athens, Greece, 27th February - 3rd March 2023

3GPP TSG RAN1 WG1 Meeting #111	R1-2212715

Toulouse, France, November 14th – 18th, 2022

# Title:	Reply LS on Positioning Reference Units

# Response to:	LS (R1-2210824/S2-2209590) on Positioning Reference Units

# Release:	Release 18

# Work Item:	FS_eLCS_Ph3

Source:	RAN1

To:	SA2

Cc:	RAN2, RAN3

Contact Person:

Name:	Ren Da

E-mail Address:	renda@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN1 would like to thank SA2 for the LS (R1-2210824/S2-2209590) on Positioning Reference Units, in which SA2 asks RAN1’s view on the following conclusion.

SA2 has concluded that the PRU information stored in core network at least includes PRU location, mobility state (i.e. fixed or mobile) and PRU capability. The PRU capability indicates the capability of a PRU for supporting positioning activities, which includes the positioning signal transmission capability and positioning measurement capability on Uu and possibly PC5. Based on that information, the PRU could be selected by an LMF to obtain measurements of RAN nodes to help improve location accuracy for all UEs and/or to assist the positioning of specific other UEs.

SA2 has also asked RAN1 the following questions:

- Question#1: Whether there is additional information that needs to be stored in the core network? If yes, what is the information?

- Question#2: Whether a PRU can be allowed to support the positioning signal transmission capability and signal measurement capability on PC5?

RAN1 has discussed the SA2’s LS in RAN1#111 and would like to provide the following responses.

- Regarding SA2’s conclusion on PRU, RAN1 would suggest the following modification:

  - “Based on that information, the PRU could be selected by an LMF to obtain measurements of RAN nodes, or to transmit the reference signals for positioning on Uu and possibly PC5, to help improve location accuracy for all UEs and/or to assist the positioning of specific other UEs”.

- Regarding SA2’s first question, RAN1 would suggest SA2 to check with RAN2 or RAN3 for the answer.

- Regarding SA2’s second question, RAN1 responds as follows:

  - From RAN1's perspective, a UE (which could be a PRU) that supports SL positioning can be allowed to support the positioning reference signal transmission capability and signal measurement capability on PC5, if the capability is introduced in R18.

2. Actions:

To SA2.

ACTION: 	RAN1 respectfully requests SA2 to take the above information into account during the work for PRU.

3. Date of Next RAN1 Meetings:

RAN1 Meeting #112 		27th Feb – 3th March 2023	Athens, GR.

RAN1 Meeting #112bis-e		17th – 26th April 2023		e-meeting.