3GPP TSG-RAN3 Meeting #119	R3-230052

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0961 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of reporting disaster PLMN of Ues to the peer gNB over Xn interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | RadiSys |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | MINT |  |  |  |  |  | Date: |  |  | 2023-02-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In case target gNodeB needs to trigger resource management for reprioritizing the disaster UE when it is experiencing load condition, there is no way to identify the disaster affected UEs. In the proposal, a new optional IE for PLMN needs to be introduced in HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE and S-NODE ADDITION REQUEST message which will indicate disaster PLMN of the UE, if applicable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new IE in HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE and S-NODE ADDITION REQUEST to indicate disaster affected PLMN on which UE was registered |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no way for target gNodeB to know if the UE is disaster affected UE and hence it will be treated in the same way as rest of the UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2, 8.2.4.2, 8.3.1.2, 9.1.1.1, 9.1.1.98, 9.1.2.12, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR R3-230050 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 38.473 CR R3-230054 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<< First Change Begins>>>>>>>>>>>>>>>>>>>>>>>>>>

## 8.2	Basic mobility procedures

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the request concerns a conditional handover and shall include the Conditional Handover Information Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Target NG-RAN node UE XnAP ID IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node shall remove the existing prepared conditional HO identified by the Target NG-RAN node UE XnAP ID IE and the Target Cell Global ID IE. It is up to the implementation of the target NG-RAN node when to remove the HO information.

Upon reception of the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall stop the timer TXnRELOCprep and terminate the Handover Preparation procedure. If the procedure was initiated for an immediate handover, the source NG-RAN node shall start the timer TXnRELOCoverall. The source NG-RAN node is then defined to have a Prepared Handover for that Xn UE-associated signalling.

For each E-RAB ID IE included in the QoS Flow To Be Setup List IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the content of the IE in the UE context and use it for subsequent inter-system handover.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

At reception of the HANDOVER REQUEST message the target NG-RAN node shall prepare the configuration of the AS security relation between the UE and the target NG-RAN node by using the information in the UE Security Capabilities IE and the AS Security Information IE in the UE Context Information IE, as specified in TS 33.501 [28].

Upon reception of the PDU Session Resource Setup List IE, contained in the HANDOVER REQUEST message, the target NG-RAN node shall behave the same as specified in TS 38.413 [5] for the PDU Session Resource Setup procedure. The target NG-RAN node shall report in the HANDOVER REQUEST ACKNOWLEDGE message the successful establishment of the result for all the requested PDU session resources. When the target NG-RAN node reports the unsuccessful establishment of a PDU session resource, the cause value should be precise enough to enable the source NG-RAN node to know the reason for the unsuccessful establishment.

For each PDU session if the PDU Session Aggregate Maximum Bit Rate IE is included in the PDU Session Resources To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall store the received PDU Session Aggregate Maximum Bit Rate in the UE context and use it when enforcing traffic policing for Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [7].

For each QoS flow for which the source NG-RAN node proposes to perform forwarding of downlink data, the source NG-RAN node shall include the DL Forwarding IE set to "DL forwarding proposed" within the Data Forwarding and Offloading Info from source NG-RAN node IE in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message. The source NG-RAN node shall include the DL Forwarding IE set to "DL forwarding proposed" for all the QoS flows mapped to a DRB, if it requests a DAPS handover for that DRB.

For each PDU session for which the target NG-RAN node decides to admit the data forwarding for at least one QoS flow, the target NG-RAN node may include the PDU Session level DL data forwarding UP TNL Information IE within the Data Forwarding Info from target NG-RAN node IE in the PDU Session Resource Admitted Info IE contained in the PDU Session Resources Admitted List IE in the HANDOVER REQUEST ACKNOWLEDGE message.

For each QoS flow for which the source NG-RAN node has not yet received the SDAP end marker packet if QoS flow re-mapping happened before handover, the source NG-RAN node shall include the UL Forwarding Proposal IE within the Data Forwarding and Offloading Info from source NG-RAN node IE in the HANDOVER REQUEST message, and if the target NG-RAN node decides to admit uplink data forwarding for at least one QoS flow, the target NG-RAN node may include the PDU Session Level UL Data Forwarding UP TNL Information IE in the Data Forwarding Info from target NG-RAN node IE in the PDU Session Resources Admitted Item IE contained in the PDU Session Resources Admitted List IE in the HANDOVER REQUEST ACKNOWLEDGE message to indicate that it accepts the uplink data forwarding.

For each PDU session resource successfully setup at the target NG-RAN, the target NG-RAN node may allocate resources for additional Xn-U PDU session resource GTP-U tunnels, indicated in the Secondary Data Forwarding Info from target NG-RAN node List IE.

For each PDU session in the HANDOVER REQUEST message, if the Alternative QoS Parameters Set List IE is included in the GBR QoS Flow Information IE in the PDU Session Resources To Be Setup List IE, the target NG-RAN node may accept the setup of the involved QoS flow when notification control has been enabled if the requested QoS parameters set or at least one of the alternative QoS parameters sets can be fulfilled at the time of handover as specified in TS 23.501 [7]. In case the target NG-RAN node accepts the handover fulfilling one of the alternative QoS parameters it shall indicate the alternative QoS parameters set which it can currently fulfil in the Current QoS Parameters Set Index IE within the PDU Session Resources Admitted List IE of the HANDOVER REQUEST ACKNOWLEDGE message while setting the QoS parameters towards the UE according to the requested QoS parameters set as specified in TS 23.501 [7].

For each DRB for which the source NG-RAN node proposes to perform forwarding of downlink data, the source NG-RAN node shall include the DRB ID IE and the mapped QoS Flows List IE within the Source DRB to QoS Flow Mapping List IE contained in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message. The source NG-RAN node may include the QoS Flow Mapping Indication IE in the Source DRB to QoS Flow Mapping List IE to indicate that only the uplink or downlink QoS flow is mapped to the DRB. If the target NG-RAN node decides to use the same DRB configuration and to map the same QoS flows as the source NG-RAN node, the target NG-RAN node includes the DL Forwarding GTP Tunnel Endpoint IE within the Data Forwarding Response DRB List IE in the HANDOVER REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this DRB.

The target NG-RAN node may additionally include the Redundant DL Forwarding UP TNL Information IE if at least one of the QoS flow mapped to the DRB is eligible to the redundant transmission feature as indicated in the Redundant QoS Flow Indicator IE within the PDU Session Resource To Be Setup List IE received in the HANDOVER REQUEST message for the QoS flow.

If the HANDOVER REQUEST ACKNOWLEDGE message contains the UL Forwarding UP TNL Information IE for a given DRB in the Data Forwarding Response DRB List IE within Data Forwarding Info from target NG-RAN node IE in the PDU Session Resources Admitted List IE and the source NG-RAN node accepts the data forwarding proposed by the target NG-RAN node, the source NG-RAN node shall perform forwarding of uplink data for the DRB.

If the HANDOVER REQUEST includes PDU session resources for PDU sessions associated to S-NSSAIs not supported by target NG-RAN, the target NG-RAN node shall reject such PDU session resources. In this case, and if at least one PDU Session Resource To Be Setup Item IE is admitted, the target NG-RAN node shall send the HANDOVER REQUEST ACKNOWLEDGE message including the PDU Session Resources Not Admitted List IE listing corresponding PDU sessions rejected at the target NG-RAN.

If the Mobility Restriction List IE is

-	contained in the HANDOVER REQUEST message, the target NG-RAN node shall

-	store the information received in the Mobility Restriction List IE in the UE context;

-	use this information to determine a target for the UE during subsequent mobility action for which the NG-RAN node provides information about the target of the mobility action towards the UE, except when one of the PDU sessions has a particular ARP value (TS 23.501 [7]) in which case the information shall not apply;

-	use this information to select a proper SCG during dual connectivity operation.

-	use this information to select proper RNA(s) for the UE when moving the UE to RRC_INACTIVE.

-	not contained in the HANDOVER REQUEST message, the target NG-RAN node shall

-	consider that no roaming and no access restriction apply to the UE except for the PNI-NPN mobility as described in TS 23.501 [7].

The target NG-RAN node shall consider that roaming or access to CAG cells is only allowed if the Allowed PNI-NPN ID List IE is contained in the HANDOVER REQUEST message, as described in TS 23.501 [7].

If the Trace Activation IE is included in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, initiate the requested trace function as specified in TS 32.422 [23].

If the Index to RAT/Frequency Selection Priority IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall store this information and use it as defined in TS 23.501 [7].

If the UE Context Reference at the S-NG-RAN IE is contained in the HANDOVER REQUEST message the target NG-RAN node may use it as specified in TS 37.340 [8]. In this case, the source NG-RAN node may expect the target NG-RAN node to include the UE Context Kept Indicator IE set to "True" in the HANDOVER REQUEST ACKNOWLEDGE message, which shall use this information as specified in TS 37.340 [8].

For each PDU session, if the Network Instance IE is included in the PDU Session Resource To Be Setup List IE and the Common Network Instance IE is not present, the target NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [7].

Redundant transmission:

-	For each PDU session, if the Redundant UL NG-U UP TNL Information at UPF IE is included in the PDU Session Resource To Be Setup List IE, the target NG-RAN node shall, if supported, use it as the uplink termination point for the user plane data for the redundant transmission for the concerned PDU session.

-	For each PDU session, if the Additional Redundant UL NG-U UP TNL Information at UPF List IE is included in the PDU Session Resource To Be Setup List IE, the target NG-RAN node shall, if supported, use them as the uplink termination points for the user plane data for the redundant transmission for the concerned PDU session.

-	For each PDU session, if the Redundant Common Network Instance IE is included in the PDU Session Resource To Be Setup List IE, the target NG-RAN node shall, if supported, use it when selecting transport network resource for the redundant transmission as specified in TS 23.501 [7].

-	For each PDU session, if the Redundant PDU Session Information IE is included in the PDU Session Resource To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received information in the UE context and set up the redundant user plane for the concerned PDU session, as specified in TS 23.501 [7]. If the PDU Session Pair ID IE is included in the Redundant PDU Session Information IE, the target NG-RAN node may store and use it to identify the paired PDU sessions.

If the TSC Traffic Characteristics IE is included in the QoS Flows To Be Setup List in the PDU Session Resource To Be Setup List IE, the target NG-RAN node shall, if supported, use it as specified in TS 23.501 [7].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource To Be Setup List IE or in the Additional UL NG-U UP TNL Information at UPF List IE, or in the Additional Redundant UL NG-U UP TNL Information at UPF List IE, the target NG-RAN node shall, if supported, use it when selecting transport network resource for the concerned NG-U transport bearer as specified in TS 23.501 [7].

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Be Setup List IE and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "required", the target NG-RAN node shall perform user plane integrity protection or ciphering, respectively. If the NG-RAN node is not able to perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU Session Resources with an appropriate cause value.

If the NG-RAN node is an ng-eNB, it shall reject all PDU sessions for which the Integrity Protection Indication IE is set to "required".

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Be Setup List IE and the Integrity Protection Indication IE or the Confidentiality Protection Indication IE is set to "preferred", the target NG-RAN node should, if supported, perform user plane integrity protection or ciphering, respectively and shall notify the SMF whether it succeeded the user plane integrity protection or ciphering or not for the concerned security policy.

For each PDU session for which the Maximum Integrity Protected Data Rate IE is included in the Security Indication IE in the PDU Session Resources To Be Setup List IE, the NG-RAN node shall store the respective information and, if integrity protection is to be performed for the PDU session, it shall enforce the traffic corresponding to the received Maximum Integrity Protected Data Rate IE, for the concerned PDU session and concerned UE, as specified in TS 23.501 [7].

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Be Setup List IE and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "not needed", the target NG-RAN node shall not perform user plane integrity protection or ciphering, respectively, for the concerned PDU session.

For each PDU session, if the Additional UL NG-U UP TNL Information List IE is included in the PDU Session Resources To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node may forward the UP transport layer information to the target S-NG-RAN node as the uplink termination point for the user plane data for this PDU session split in different tunnel.

If the Location Reporting Information IE is included in the HANDOVER REQUEST message, then the target NG-RAN node should initiate the requested location reporting functionality as defined in TS 38.413 [5].

Upon reception of UE History Information IE in the HANDOVER REQUEST message, the target NG-RAN node shall collect the information defined as mandatory in the UE History Information IE and shall, if supported, collect the information defined as optional in the UE History Information IE, for as long as the UE stays in one of its cells, and store the collected information to be used for future handover preparations.

If the Trace Activation IE is included in the HANDOVER REQUEST message which includes

-	the MDT Activation IE set to "Immediate MDT and Trace", then the target NG-RAN node shall if supported, initiate the requested trace session and MDT session as described in TS 32.422 [23].

-	the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", the target NG-RAN node shall, if supported, initiate the requested MDT session as described in TS 32.422 [23] and the target NG-RAN node shall ignore the Interfaces To Trace IE, and the Trace Depth IE.

-	the MDT Location Information IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, store this information and take it into account in the requested MDT session.

-	the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", and if the Signalling based MDT PLMN List IE is included in the MDT Configuration IE, the target NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [43].

-	the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the WLAN Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the Sensor Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall take it into account for MDT Configuration as described in TS 37.320 [43].

-	the MDT Configuration IE and if the target NG-RAN node is a gNB receiving a MDT Configuration-EUTRA IE, or the target NG-RAN node is a ng-eNB receiving a MDT Configuration-NR IE, the target NG-RAN node shall store it as part of the UE context, and use it as described in TS 37.320 [43].

If the Management Based MDT PLMN List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received information in the UE context, and use this information to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [23].

If the HANDOVER REQUEST message includes the Management Based MDT PLMN List IE, the target NG-RAN node shall, if supported, store it in the UE context, and take it into account if it includes information regarding the PLMN serving the UE in the target NG-RAN node.

If the Mobility Information IE is provided in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information. The target NG-RAN shall, if supported, store the C-RNTI assigned at the source cell as received in the HANDOVER REQUEST message.

Upon reception of the UE History Information from the UE IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the collected information and use it for future handover preparations.

For each QoS flow which has been successfully established in the target NG-RAN node, if the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall store this information, and shall, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall store this information, and shall, if supported, use it for RAN part delay reporting.

If the 5GC Mobility Restriction List Container IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as specified in TS 38.300 [9].

V2X:

-	If the NR V2X Services Authorized IE is included in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the LTE V2X Services Authorized IE is included in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the NR UE Sidelink Aggregate Maximum Bit Rate IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services.

-	If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for LTE V2X services.

5G ProSe:

-	If the 5G ProSe Authorized IE is included in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the 5G ProSe UE PC5 Aggregate Maximum Bit Rate IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services.

- 	If the 5G ProSe PC5 QoS Parameters IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it as defined in TS 23.304 [48].

If the PC5 QoS Parameters IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it as defined in TS 23.287 [38].

If the DAPS Request Information IE is included for a given DRB in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the request concerns a DAPS handover for that DRB, as described in TS 38.300 [9]. Accordingly, the target NG-RAN node shall include the DAPS Response Information IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Maximum Number of CHO Preparations IE is included in the Conditional Handover Information Acknowledge IE contained in the HANDOVER REQUEST ACKNOWLEDGE message, then the source NG-RAN node should not prepare more candidate target cells for a CHO for the same UE towards the target NG-RAN node than the number indicated in the IE.

If the Estimated Arrival Probability IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node may use the information to allocate necessary resources for the incoming CHO.

If the IAB Node Indication IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the handover is for an IAB node. In addition:

-	If the No PDU Session Indication IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider the UE as an IAB-node which does not have any PDU sessions activated, and ignore the PDU Session Resources To Be Setup List IE, and shall not take any action with respect to PDU session setup. Subsequently, the source NG-RAN node shall, if supported, ignore the PDU Session Resources Admitted To Be Added List IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the UE Radio Capability ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

If for a given QoS Flow the Source DL Forwarding IP Address IE is included within the Data Forwarding and Offloading Info from source NG-RAN node IE in the PDU Session Resources To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information and use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the MBS Session Information List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, establish an NG-RAN MBS session resources context as specified in TS 23.247 [46] and TS 38.300 [9], if applicable.

If the HANDOVER REQUEST message includes the MBS Area Session ID IE, the target NG-RAN, if supported, shall use this information as an indication from which MBS Area Session ID the UE is handed over. For each MBS session for which the Active MBS Session Information IE is included in the MBS Session Information Item List IE, the target NG-RAN shall, if supported, use this information to setup respective MBS session resources. The target NG-RAN node shall, if supported, consider that the MBS sessions for which the Active MBS Session Information IE is not included are inactive.

If the HANDOVER REQUEST ACKNOWLEDGE message contains in the MBS Session Information Response List IE the MBS Data Forwarding Response Info IE that the source NG-RAN node shall use the information for forwarding MBS traffic to the target NG-RAN node.

If the MBS Session Associated Information List IE is included in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the information contained in the Associated QoS Flows Information List IE as specified in TS 23.247 [46].

For each MRB indicated in the MBS Mapping and Data Forwarding Request Info from source NG-RAN node IE, the target NG-RAN node shall use the MRB ID IE and, if included, the MRB Progress Information IE which includes the highest PDCP SN of the packet which has already been delivered to the UE for the MRB, to decide whether to apply data forwarding for that MRB and to establish respective resources.

The source NG-RAN shall, for each MRB in the MBS Data Forwarding Response Info from target NG-RAN node IE in the HANDOVER REQUEST ACKNOWLEDGE message, start data forwarding to the indicated DL Forwarding UP TNL Information. If the MRB Progress Information IE is included the source NG-RAN node may use the information to determine when to stop data forwarding.

If the Time Synchronisation Assistance Information IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7].

If the QMC Configuration Information IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, take it into account for QoE measurements handling, as described in TS 38.300 [9].

If the UE Slice-Maximum Bit Rate List IE is contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

If the DisasterPLMNIdentity IE is contained in HANDOVER REQUEST message, the target NG-RAN node, shall store and use the same for reducing the UE scheduling priority in load condition.

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the DRBs transferred to MN IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the S-NG-RAN node in the SN Status Transfer procedure towards the target NG-RAN node, as specified in TS 37.340 [8].

<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes  >>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

### 8.2.4	Retrieve UE Context

#### 8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer, or to request for small data transmission.

The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

If the old NG-RAN node is able to identify the UE context by means of the UE Context ID, and to successfully verify the UE by means of the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message, and decides to provide the UE context to the new NG-RAN node, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT RESPONSE message.

If the Trace Activation IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, initiate the requested trace function as specified in TS 32.422 [23].

If the Index to RAT/Frequency Selection Priority IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall store this information and use it as defined in TS 23.501 [7].

If the Location Reporting Information IE is included in the RETRIEVE UE CONTEXT RESPONSE message, then the new NG-RAN node should initiate the requested location reporting functionality as defined in TS 38.413 [5].

If the Trace Activation IE is included in the RETRIEVE UE CONTEXT RESPONSE message which includes

-	the MDT Activation IE set to "Immediate MDT and Trace", then the new NG-RAN node shall if supported, initiate the requested trace session and MDT session as described in TS 32.422 [23].

-	the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", the new NG-RAN node shall, if supported, initiate the requested MDT session as described in TS 32.422 [23] and the target NG-RAN node shall ignore the Interfaces To Trace IE, and the Trace Depth IE.

-	the MDT Location Information IE, within the MDT Configuration IE, the new NG-RAN node shall, if supported, store this information and take it into account in the requested MDT session.

-	the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", and if the Signalling based MDT PLMN List IE is included in the MDT Configuration IE, the new NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [43].

-	the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, the new NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the WLAN Measurement Configuration IE, within the MDT Configuration IE, the new NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the Sensor Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the MDT Configuration and if the new NG-RAN node is a gNB receiving a MDT Configuration-EUTRA IE, or the target NG-RAN node is a ng-eNB receiving a MDT Configuration-NR IE, the new NG-RAN node shall store it as part of the UE context, and use it as described in TS 37.320 [43].

For each QoS flow in the RETRIEVE UE CONTEXT RESPONSE message, if the QoS Monitoring Request IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resources To Be Setup List IE, the new NG-RAN node shall store this information, and shall, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resources To Be Setup List IE, the new NG-RAN node shall store this information, and shall, if supported, use it for RAN part delay reporting.

If the 5GC Mobility Restriction List Container IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store this information in the UE context and use it as specified in TS 38.300 [9].

V2X:

-	If the NR V2X Services Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and it contains one or more IEs set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the LTE V2X Services Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and it contains one or more IEs set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the NR UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE Context Information Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services.

-	If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE Context Information Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for LTE V2X services.

5G ProSe:

-	If the 5G ProSe Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and it contains one or more IEs set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the 5G ProSe UE PC5 Aggregate Maximum Bit Rate IE is included in the UE Context Information - Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services.

- 	If the 5G ProSe PC5 QoS Parameters IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use it as defined in TS 23.304 [48].

If the PC5 QoS Parameters IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use it as defined in TS 23.287[38].

In case of RRC Re-establishment, the old NG-RAN may include the UE History Information IE or the UE History Information from the UE IE in the RETRIEVE UE CONTEXT RESPONSE message. Upon reception of the UE History Information IE or the UE History Information from the UE IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store the collected information and use it for future handover preparations.

If the UE Radio Capability ID IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG- RAN node shall, if supported store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

If the Management Based MDT PLMN List IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store it in the UE context, and use this information to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [23].

If the MBS Session Information List IE is included in the UE Context Information – Retrieve UE Context Response IE contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use this information to establish an NG-RAN MBS session resources context, if applicable.

If the RETRIEVE UE CONTEXT RESPONSE message includes the MBS Area Session ID IE, the new NG-RAN node shall, if supported, use this information as an indication in which MBS Area Session ID the UE has been suspended. For each MBS session for which the Active MBS Session Information IE is included in the MBS Session Information Item IE, the new NG-RAN node shall, if supported, use this information to setup respective MBS session resources. The new NG-RAN node shall, if supported, consider that the MBS sessions for which the Active MBS Session Information IE is not included are inactive.

If the IAB Node Indication IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, consider that the procedure is performed for an IAB-node. In addition:

-	If the No PDU Session Indication IE is contained in the UE Context Information – Retrieve UE Context Response IE of the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, consider the UE as an IAB-node which does not have any PDU sessions activated, and ignore the PDU Session Resources To Be Setup List IE in the UE Context Information – Retrieve UE Context Response IE, and shall not take any action with respect to PDU session setup.

If the Time Synchronisation Assistance Information IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7].

If the QMC Configuration Information IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, take it into account for QoE measurements handling, as described in TS 38.300 [9].

If the SDT Support Request IE is included in the RETRIEVE UE CONTEXT REQUEST message, the old NG-RAN node shall, if supported, consider that the UE has requested for SDT as defined in TS 38.300 [9].

If the UE Slice-Maximum Bit Rate List IE is contained in RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

If the Positioning Information IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, take it into account to allocate proper SRS resources and make corresponding response to LMF when positioning a UE.

If the DisasterPLMNIdentity IE is contained in RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, store and use the same for reducing the UE scheduling priority in load condition.

Interaction with the Retrieve UE Context Confirm procedure

If the UE Context Reference at the S-NG-RAN IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node may use it to establish dual connectivity with the S-NG-RAN node and shall trigger the Retrieve UE Context Confirm procedure to the old NG-RAN node when the UE successfully resumes on the new NG-RAN node.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes  >>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

## 8.3	Procedures for Dual Connectivity

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE ADDITION REQUEST message, it shall start the timer TXnDCprep.

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the QoS Flow Level QoS Parameters IE for each QoS flow shall follow the principles specified for the PDU Session Resource Setup procedure in TS 38.413 [5].

The S-NG-RAN node shall choose the ciphering algorithm based on the information in the UE Security Capabilities IE and locally configured priority list of AS encryption algorithms and apply the key indicated in the S-NG-RAN node Security Key IE as specified in TS 33.501 [28].

If the TSC Traffic Characteristics IE is included for a QoS flow in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [5].

If the Additional QoS Flow Information IE is included for a QoS flow in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [5].

For each GBR QoS flow, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE, the S-NG-RAN node shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure specified in TS 38.413 [5].

For each PDU session, if the Network Instance IE is included in the PDU Session Resource Setup Info – SN terminated IE contained in the PDU Session Resources To Be Added List IE and the Common Network Instance IE is not present, the S-NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [7].

For each GBR QoS flow, if the Offered GBR QoS Flow Information IE is included in the QoS Flows To Be Setup List IE contained in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node may request the M-NG-RAN node to configure the DRB to which that QoS flow is mapped with MCG resources.

For each PDU session, if the Non-GBR Resources Offered IE is included in the PDU Session Resource Setup Info – SN terminated IE contained in the PDU Session Resources To Be Added List IE and set to "true", the S-NG-RAN node may request the M-NG-RAN node to configure DRBs to which non-GBR QoS flows of the PDU session are mapped with MCG resources.

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource Setup Info – SN terminated IE contained in the PDU Session Resources To Be Added List IE, the S-NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [7].

Redundant transmission:

-	For each PDU session, if the Redundant UL NG-U UP TNL Information at UPF IE is included in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall, if supported, use it as the uplink termination point for the user plane data for this PDU session for the redundant transmission and it shall include the Redundant DL NG-U UP TNL Information at NG-RAN IE in the PDU Session Resource Setup Response Info – SN terminated IE as described in TS 23.501 [7].

-	For each PDU session, if the Redundant Common Network Instance IE is included in the PDU Session Resource Setup Info – SN terminated IE the S-NG-RAN node shall, if supported, use it when selecting transport network resource for the redundant transmission as specified in TS 23.501 [7].

-	For each PDU session for which the Redundant QoS Flow Indicator IE is include in QoS Flows To Be Setup List IE contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store and use it as specified in TS 23.501 [7].

-	For each PDU session, if the Redundant PDU Session Information IE is included in the PDU Session Resource Setup Info - SN terminated IE in the S-NODE ADDITION REQUEST message, the S-NODE-RAN node shall, if supported, store the received information in the UE context and setup the redundant user plane resources for the concerned PDU session, as specified in TS 23.501 [7].

-	For each PDU session resource successfully setup for which the Redundant PDU Session Information IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, include the Used RSN Information IE in the PDU Session Resource Setup Response Info – SN terminated IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message. If the PDU Session Pair ID IE is included in the Redundant PDU Session Information IE, the S-NG-RAN node may store and use it to identify the paired PDU sessions.

If the S-NODE ADDITION REQUEST message contains the Selected PLMN IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NODE ADDITION REQUEST message contains the Expected UE Behaviour IE, the S-NG-RAN node shall, if supported, store this information and may use it to optimize resource allocation.

If the S-NODE ADDITION REQUEST message contains the Mobility Restriction List IE, the S-NG-RAN node, if supported, shall store this information and use it to select an appropriate SCG.

If the S-NODE ADDITION REQUEST message contains the Index to RAT/Frequency Selection Priority IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NG-RAN node is a gNB and the S-NODE ADDITION REQUEST message contains the PCell ID IE, the S-NG-RAN node shall search for the target NR cell among the NR neighbour cells of the PCell indicated, as specified in the TS 37.340 [8].

If the S-NODE ADDITION REQUEST message contains the S-NG-RAN node PDU Session Aggregate Maximum Bit Rate IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NODE ADDITION REQUEST message contains the MR-DC Resource Coordination Information IE, the S-NG-RAN node should forward it to lower layers and it may use it for the purpose of resource coordination with the M-NG-RAN node, or to coordinate with sidelink resources used in the M-NG-RAN node. The S-NG-RAN node shall consider the value of the received UL Coordination Information IE valid until reception of a new update of the IE for the same UE. The S-NG-RAN node shall consider the value of the received DL Coordination Information IE valid until reception of a new update of the IE for the same UE. If the E-UTRA Coordination Assistance Information IE or the NR Coordination Assistance Information IE is contained in the MR-DC Resource Coordination Information IE, the S-NG-RAN node shall, if supported, use the information to determine further coordination of resource utilisation between the S-NG-RAN node and the M-NG-RAN node.

If the S-NODE ADDITION REQUEST message contains the NE-DC TDM Pattern IE, the S-NG-RAN node should forward it to lower layers and use it for the purpose of single uplink transmission. The S-NG-RAN node shall consider the value of the received NE-DC TDM Pattern IE valid until reception of a new update of the IE for the same UE.

If the S-NODE ADDITION REQUEST message contains the QoS Flow Mapping Indication IE, the S-NG-RAN node may take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

For each bearer for which allocation of the PDCP entity is requested at the S-NG-RAN node:

-	the M-NG-RAN node may propose to apply forwarding of downlink data by including the DL Forwarding IE within PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message. For each bearer that it has decided to admit, the S-NG-RAN node may include the DL Forwarding GTP Tunnel Endpoint IE within the PDU Session Resource Setup Response Info – SN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this bearer.

-	the S-NG-RAN node may include for each bearer in the PDU Session Resource Setup Response Info – SN terminated IE the UL Forwarding GTP Tunnel Endpoint IE to indicates it request data forwarding of uplink packets to be performed for that bearer.

-	the M-NG-RAN node shall include RLC Mode IE for each bearer offloaded from M-NG-RAN node to S-NG-RAN node in the DRBs to QoS Flow Mapping List IE within the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDTION REQUEST message, and the RLC Mode IE indicates the mode that the M-NG-RAN used for the DRB when it was hosted at the M-NG-RAN node.

For each bearer for which the PDCP entity is at the M-NG-RAN node:

-	the M-NG-RAN node shall include the RLC mode IE for each bearer in the DRBs To Be Setup List IE within the PDU Session Resource Setup Info – MN terminated IE of the S-NODE ADDTION REQUEST message to indicate the RLC mode has been configured at the M-NG-RAN node, so that the S-NG-RAN node shall configure the same RLC mode for this MN terminated split bearer.

The M-NG-RAN node may also propose to apply forwarding of UL data when offloading QoS flows for which in-order delivery is requested by including the UL Forwarding Proposal IE in the Data Forwarding and Offloading Info from source NG-RAN node IE within the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message. The S-NG-RAN node may include the PDU Session Level UL Data Forwarding UP TNL Information IE in the Data Forwarding Info from target NG-RAN node IE within the PDU Session Resource Setup Response Info – SN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding.

If the Masked IMEISV IE is contained in the S-NODE ADDITION REQUEST message the S-NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

If the UE Radio Capability ID IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

The S-NG-RAN node shall report to the M-NG-RAN node, in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the result for all the requested PDU session resources in the following way:

-	A list of PDU session resources which are successfully established shall be included in the PDU Session Resources Admitted To Be Added List IE.

-	A list of PDU session resources which failed to be established shall be included in the PDU Session Resources Not Admitted List IE.

Upon reception of the S-NODE ADDITION REQUEST ACKNOWLEDGE message the M-NG-RAN node shall stop the timer TXnDCprep.

If the S-NODE ADDITION REQUEST ACKNOWLEDGE message contains the MR-DC Resource Coordination Information IE, the M-NG-RAN node may use it for the purpose of resource coordination with the S-NG-RAN node. The M-NG-RAN node shall consider the value of the received UL Coordination Information IE valid until reception of a new update of the IE for the same UE. The M-NG-RAN node shall consider the value of the received DL Coordination Information IE valid until reception of a new update of the IE for the same UE. If the E-UTRA Coordination Assistance Information IE or the NR Coordination Assistance Information IE is contained in the MR-DC Resource Coordination Information IE, the M-NG-RAN node shall, if supported, use the information to determine further coordination of resource utilisation between the M-NG-RAN node and the S-NG-RAN node.

The S-NG-RAN node may include for each bearer in the DRBs To Be Setup List IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message the PDCP SN Length IE to indicate the PDCP SN length for that DRB.

If the S-NG-RAN node UE XnAP ID IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store this information and use it as defined in TS 37.340 [8].

If the S-NODE ADDITION REQUEST message contains the PDCP SN Length IE, the S-NG-RAN node shall, if supported, store this information and use it for lower layer configuration of the concerned MN terminated bearer.

If the S-NODE ADDITION REQUEST message contains the SN Addition Trigger Indication IE, the S-NG-RAN node shall include the RRC config indication IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message to inform the M-NG-RAN node if the S-NG-RAN node applied full or delta configuration, as specified in TS 37.340 [8].

If the S-NODE ADDITION REQUEST message contains the S-NG-RAN node Maximum Integrity Protected Data Rate Uplink IE or the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE, the S-NG-RAN node shall use the received information when enforcing the maximum integrity protected data rate for the UE.

If the Security Indication IE is included in the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message, the behaviour of the S-NG-RAN node shall be the same as specified for the same IE in the PDU Session Resources To Be Setup List IE in the Handover Preparation procedure, for the concerned PDU session, and the S-NG-RAN node shall include the Security Result IE in the PDU Session Resource Setup Response Info – SN terminated IE.

If the Security Result IE is included in the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message, the S-NG-RAN node may take the information into account when deciding whether to perform user plane integrity protection or ciphering for the DRBs that it establishes for the concerned PDU session, except if the Split Session Indicator IE is included in the PDU Session Resource Setup Info – SN terminated IE and set to "split", in which case it shall perform user plane integrity protection or ciphering according to the information in the Security Result IE. If the S-NG-RAN node is an ng-eNB, it shall reject all PDU sessions for which the Integrity Protection Indication IE is set to "required" as specified in TS 33.501 [28]. If either the S-NG-RAN node or the M-NG-RAN node is an ng-eNB, the S-NG-RAN node shall behave according to clause 6.10.4 of TS 33.501 [28] for PDU sessions for which the Integrity Protection Indication IE is set to "preferred".

The S-NG-RAN node may include the Location Information at S-NODE IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, if respective information is available at the S-NG-RAN node.

If the Location Information at S-NODE Reporting IE set to "pscell" is included in the S-NODE ADDITION REQUEST, the S-NG-RAN node shall, start providing information about the current location of the UE. If the Location Information at S-NODE IE is included in the S-NODE ADDITION REQUEST ACKNOWLEDGE, the M-NG-RAN node shall store the included information so that it may be transferred towards the AMF.

If the Default DRB Allowed IE is included in the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message and set to "true", the S-NG-RAN node may configure the default DRB for the PDU session.

If the S-NODE ADDITION REQUEST ACKNOWLEDGE message includes the DRB IDs taken into use IE, the M-NG-RAN node, if applicable, shall act as specified in TS 37.340 [8].

If Trace Activation IE has previously been received for this UE, it shall be included in the S-NODE ADDITION REQUEST message. If the Trace Activation IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, initiate the requested trace function as described in TS 32.422 [23].

If the Trace Activation IE is included in the S-NODE ADDITION REQUEST message which includes

-	the MDT Activation IE set to "Immediate MDT and Trace", then the S-NG-RAN node shall if supported, initiate the requested trace session and MDT session as described in TS 32.422 [23].

-	the MDT Activation IE set to "Immediate MDT Only", the S-NG-RAN node shall, if supported, initiate the requested MDT session as described in TS 32.422 [23] and the S-NG-RAN node shall ignore the Interfaces To Trace IE, and the Trace Depth IE.

-	the MDT Location Information IE, within the MDT Configuration IE, the S-NG-RAN node shall, if supported, store this information and take it into account in the requested MDT session.

-	the MDT Activation IE set to "Immediate MDT Only", and if the Signalling based MDT PLMN List IE is included in the MDT Configuration IE, the S-NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [43].

-	the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, the S-NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the WLAN Measurement Configuration IE, within the MDT Configuration IE, the S-NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the Sensor Measurement Configuration IE, within the MDT Configuration IE, the S-NG-RAN node shall take it into account for MDT Configuration as described in TS 37.320 [43].

-	the MDT Configuration IE and if the S-NG-RAN node is a gNB at least the MDT Configuration-NR IE shall be present, while if the S-NG-RAN Node is an ng-eNB at least the MDT Configuration-EUTRA IE shall be present.

If the Requested Fast MCG recovery via SRB3 IE set to "true" is included in the S-NODE ADDITION REQUEST message and the S-NG-RAN node decides to configure fast MCG link recovery via SRB3 as specified in TS 37.340 [8], the S-NG-RAN node shall, if supported, include the Available fast MCG recovery via SRB3 IE set to "true" in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the QoS Monitoring Request IE is included in the QoS Flow Level QoS Parameters IE for a QoS flow contained in the DRBs To Be Setup List IE of the PDU Session Resource Setup Info – MN terminated IE, the S-NG-RAN node shall, if supported, use it to configure lower layers for the purpose of delay measurement and QoS monitoring as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE is included in the QoS Flow Level QoS Parameters IE for a QoS flow contained in the DRBs To Be Setup List IE of the PDU Session Resource Setup Info – MN terminated IE, the S-NG-RAN node shall, if supported, use it for RAN part delay reporting.

For each QoS flow which has been successfully established in the S-NG-RAN node, if the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall store this information, and shall, if supported, perform delay measurement and QoS monitoring as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall store this information, and shall, if supported, use it for RAN part delay reporting. In case such a QoS flow is included in the DRBs To Be Setup List IE of the PDU Session Resource Setup Response Info – SN terminated IE, the M-NG-RAN node shall, if supported, use it to configure lower layers for the purpose of delay measurement and QoS monitoring. If the QoS Monitoring Reporting Frequency IE is included in the DRBs To Be Setup List IE of the PDU Session Resource Setup Response Info – SN terminated IE, the M-NG-RAN node shall, if supported, use it for RAN part delay reporting.

For each DRB configured as MN-terminated split bearer/SCG bearer, if the QoS Mapping Information IE is included in the DRBs Admitted List IE in the PDU Session Resource Setup Response Info – MN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it to set DSCP and/or IPv6 flow label fields for the downlink IP packets which are transmitted from M-NG-RAN node to S-NG-RAN node through the GTP tunnels indicated by the UP Transport Layer Information IE.

If the Source NG-RAN Node ID IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, use it to decide the direct data path availability with the indicated source NG-RAN node, and if the direct data forwarding path is available, include the Direct Forwarding Path Availability IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If for a given QoS Flow the Source DL Forwarding IP Address IE or both, the Source DL Forwarding IP Address IE and the Source Node DL Forwarding IP Address IE are included within the Data Forwarding and Offloading Info from source NG-RAN node IE in the PDU Session Resource Setup Info – SN terminated IE contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store this information and use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If for a given QoS Flow the Source DL Forwarding IP Address IE is included within the QoS Flows Mapped To DRB List IE in the PDU Session Resource Setup Response Info – SN terminated IE contained in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, store this information and use it as part of its ACL functionality to identify source TNL address for data forwarding in case of subsequent handover preparation, if such ACL functionality is deployed.

If the Management Based MDT PLMN List IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store the received information in the UE context, and use this information to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [23].

Upon reception of the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, start collecting SCG information and continue for as long as the UE stays in one of its cells.

If the UE History Information from the UE IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store this information.

If the PSCell Change History IE set to "reporting full history" is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, signal the latest SCG UE History Information upon each PSCell change, to the M-NG-RAN node, using the S-NG-RAN node initiated S-NG-RAN node Modification procedure.

If the IAB Node Indication IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider that dual connectivity operation is requested for an IAB-node. In addition:

-	If the No PDU Session Indication IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider the UE as an IAB-node which does not have any PDU sessions activated, and ignore the PDU Session Resources To Be Added List IE, and shall not take any action with respect to PDU session setup. Subsequently, the M-NG-RAN node shall, if supported, ignore the PDU Session Resources Admitted To Be Added List IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

-	If the F1-terminating IAB-donor Indicator IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, assume that it will become the F1-terminating IAB-donor of the IAB-node, and act as described in TS 38.401 [2].

If the CHO Information SN Addition IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall consider that the S-NG-RAN node Addition Preparation procedure has been triggered as part of a conditional handover. It may use the Source M-NG-RAN node ID IE and the Source M-NG-RAN node UE XnAP ID IE to identify other active S-NG-RAN node Addition Preparations related to this UE. If the Estimated Arrival Probability IE is contained in the CHO Information SN Addition IE included in the S-NODE ADDITION REQUEST message, then the S-NG-RAN node may use the information to allocate necessary resources for the UE.

If the SCG Activation Request IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node may use it to configure SCG resources as specified in TS 37.340 [8], and shall, if supported, include the SCG Activation Status IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message. If the SCG Activation Request IE in the S-NODE ADDITION REQUEST message is set to "Activate SCG", the S-NG-RAN node shall, if supported, activate the SCG resources and set the SCG Activation Status IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message to "SCG activated".

If the Conditional PSCell Addition Information Request IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider that the request concerns CPAC, as described in TS 37.340 [8]. Accordingly, the S-NG-RAN node shall, if supported, include the Conditional PSCell Addition Acknowledge IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the Conditional PSCell Addition Information Acknowledge is included in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, shall, if supported, consider the indicated PSCells are selected by the target SN as candidate PSCells for CPAC.

If the CG-CandidateList is included in the S-NG-RAN node to M-NG-RAN node Container IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it for the purpose of CPAC.

If the Estimated Arrival Probability IE is contained in the Conditional PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, then the candidate target S-NG-RAN node may use the information to allocate necessary resources for the incoming CPAC procedure.

If the S-NG-RAN node UE Slice Maximum Bit Rate IE for a specific S-NSSAI is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store and use the received S-NG-RAN node UE Slice Maximum Bit Rate for all PDU sessions associated with the S-NSSAI for the concerned UE as defined in TS 23.501 [7].

If the DisasterPLMNIdentity IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, store and use the same for reducing the UE scheduling priority in load condition.

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

Interaction with the Activity Notification procedure

Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes  >>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | Ignore |
| DisasterPLMNIdentity | O |  | PLMN Identity 9.2.2.4 | Disaster affected PLMN to which UE belongs. | YES | Ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes  >>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.9	RETRIEVE UE CONTEXT RESPONSE

This message is sent by the old NG-RAN node to transfer the UE context to the new NG-RAN node.

Direction: old NG-RAN node  new NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node | YES | ignore |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information – Retrieve UE Context Response | M |  | 9.2.1.13 |  | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | Ignore |
| Disaster PLMN Identity | O |  | PLMN Identity 9.2.2.4 | Disaster affected PLMN to which UE belongs | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes  >>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 |  | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | reject |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  | – |  |
| >>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10] | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | 9.2.3.81 |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | reject |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  | YES | ignore |
| Available DRB IDs | C-ifSNterminated |  | DRB List 9.2.1.29 | Indicates the list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  | YES | ignore |
| SN Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-MN HO, intra-MN HO, ...) | This IE indicates the trigger for S-NG-RAN node Addition Preparation procedure | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| Source NG-RAN Node ID | O |  | Global NG-RAN Node ID 9.2.2.3 | The NG-RAN Node ID of the source NG-RAN node or the source SN. | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| PSCell Change History | O |  | ENUMERATED (reporting full history, ...) |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| CHO Information SN Addition | O |  |  |  | YES | reject |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source M-NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| SCG Activation Request | O |  | 9.2.3.154 |  | YES | ignore |
| Conditional PSCell Addition Information Request | O |  |  |  | YES | reject |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | reject |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | Reject |
| Disaster PLMN Identity | O |  | PLMN Identity 9.2.2.4 | Disaster affected PLMN to which UE belongs | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes  >>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

AMF-UE-NGAP-ID,

AS-SecurityInformation,

AssistanceDataForRANPaging,

BitRate,

Cause,

CellAndCapacityAssistanceInfo-EUTRA,

CellAndCapacityAssistanceInfo-NR,

CellAssistanceInfo-EUTRA,

CellAssistanceInfo-NR,

CHOinformation-Req,

CHOinformation-Ack,

CHOinformation-AddReq,

CHOinformation-ModReq,

CHO-MRDC-EarlyDataForwarding,

CHO-MRDC-Indicator,

CPTransportLayerInformation,

TNLA-To-Add-List,

TNLA-To-Update-List,

TNLA-To-Remove-List,

TNLA-Setup-List,

TNLA-Failed-To-Setup-List,

CriticalityDiagnostics,

XnUAddressInfoperPDUSession-List,

DAPSResponseInfo-List,

DataTrafficResourceIndication,

DeliveryStatus,

DesiredActNotificationLevel,

DRB-ID,

DRB-List,

DRB-Number,

DRBsSubjectToDLDiscarding-List,

DRBsSubjectToEarlyStatusTransfer-List,

DRBsSubjectToStatusTransfer-List,

DRBToQoSFlowMapping-List,

E-UTRA-CGI,

ExpectedUEActivityBehaviour,

ExpectedUEBehaviour,

ExtendedUEIdentityIndexValue,

FiveGCMobilityRestrictionListContainer,

GlobalCell-ID,

GlobalNG-RANNode-ID,

GlobalNG-RANCell-ID,

GUAMI,

InterfaceInstanceIndication,

I-RNTI,

Local-NG-RAN-Node-Identifier,

LocationInformationSNReporting,

LocationReportingInformation,

LowerLayerPresenceStatusChange,

LTEUESidelinkAggregateMaximumBitRate,

LTEV2XServicesAuthorized,

MR-DC-ResourceCoordinationInfo,

ServedCells-E-UTRA,

ServedCells-NR,

ServedCellsToUpdate-E-UTRA,

ServedCellsToUpdate-NR,

MAC-I,

MaskedIMEISV,

MDT-Configuration,

MDTPLMNList,

MobilityRestrictionList,

Neighbour-NG-RAN-Node-List,

NG-RAN-Cell-Identity,

NG-RANnodeUEXnAPID,

NR-CGI,

NE-DC-TDM-Pattern,

NRUESidelinkAggregateMaximumBitRate,

NRV2XServicesAuthorized,

PagingDRX,

EUTRAPagingeDRXInformation,

PagingPriority,

PartialListIndicator,

PLMN-Identity,

PDCPChangeIndication,

PDUSessionAggregateMaximumBitRate,

PDUSession-ID,

PDUSession-List,

PDUSession-List-withCause,

PDUSession-List-withDataForwardingFromTarget,

PDUSession-List-withDataForwardingRequest,

PDUSessionResourcesAdmitted-List,

PDUSessionResourcesNotAdmitted-List,

PDUSessionResourcesToBeSetup-List,

PDUSessionResourceChangeRequiredInfo-SNterminated,

PDUSessionResourceChangeRequiredInfo-MNterminated,

PDUSessionResourceChangeConfirmInfo-SNterminated,

PDUSessionResourceChangeConfirmInfo-MNterminated,

PDUSessionResourceSecondaryRATUsageList,

PDUSessionResourceSetupInfo-SNterminated,

PDUSessionResourceSetupInfo-MNterminated,

PDUSessionResourceSetupResponseInfo-SNterminated,

PDUSessionResourceSetupResponseInfo-MNterminated,

PDUSessionResourceModificationInfo-SNterminated,

PDUSessionResourceModificationInfo-MNterminated,

PDUSessionResourceModificationResponseInfo-SNterminated,

PDUSessionResourceModificationResponseInfo-MNterminated,

PDUSessionResourceModConfirmInfo-SNterminated,

PDUSessionResourceModConfirmInfo-MNterminated,

PDUSessionResourceModRqdInfo-SNterminated,

PDUSessionResourceModRqdInfo-MNterminated,

PDUSessionType,

PC5QoSParameters,

QoSFlowIdentifier,

QoSFlowNotificationControlIndicationInfo,

QoSFlows-List,

RANPagingArea,

ResetRequestTypeInfo,

ResetResponseTypeInfo,

RFSP-Index,

RRCConfigIndication,

RRCResumeCause,

SCGConfigurationQuery,

SCGreconfigNotification,

SecurityIndication,

S-NG-RANnode-SecurityKey,

SpectrumSharingGroupID,

SplitSRBsTypes,

S-NG-RANnode-Addition-Trigger-Ind,

S-NSSAI,

TargetCellList,

TAISupport-List,

Target-CGI,

TimeToWait,

TraceActivation,

UEAggregateMaximumBitRate,

UEContextID,

UEContextInfoRetrUECtxtResp,

UEContextKeptIndicator,

UEHistoryInformation,

UEIdentityIndexValue,

UERadioCapabilityForPaging,

UERadioCapabilityID,

UERANPagingIdentity,

UESecurityCapabilities,

UPTransportLayerInformation,

UserPlaneTrafficActivityReport,

XnBenefitValue,

RANPagingFailure,

TNLConfigurationInfo,

MaximumCellListSize,

MessageOversizeNotification,

NG-RANTraceID,

MobilityInformation,

InitiatingCondition-FailureIndication,

HandoverReportType,

TargetCellinEUTRAN,

C-RNTI,

UERLFReportContainer,

Measurement-ID,

RegistrationRequest,

ReportCharacteristics,

CellToReport,

ReportingPeriodicity,

CellMeasurementResult,

UEHistoryInformationFromTheUE,

MobilityParametersInformation,

MobilityParametersModificationRange,

RACHReportInformation,

IABNodeIndication,

SNTriggered,

SCGIndicator,

UESpecificDRX,

DirectForwardingPathAvailability,

TransportLayerAddress,

PrivacyIndicator,

URIaddress,

MBS-Session-ID,

UEIdentityIndexList-MBSGroupPaging,

MBS-SessionInformation-List,

MBS-SessionInformationResponse-List,

SuccessfulHOReportInformation,

PSCellHistoryInformationRetrieve,

SSBOffsets-List,

NG-RANnode2SSBOffsetsModificationRange,

Coverage-Modification-List,

SCGFailureReportContainer,

SNMobilityInformation,

PSCellChangeHistory,

CHOConfiguration,

SCGUEHistoryInformation,

F1CTrafficContainer,

NoPDUSessionIndication,

IAB-TNL-Address-Request,

IAB-TNL-Address-Response,

TrafficIndex,

TrafficProfile,

TrafficToBeReleaseInformation,

F1-TerminatingTopologyBHInformation,

Non-F1-TerminatingTopologyBHInformation,

BHInfoList,

IABTNLAddress,

IABCellInformation,

IABTNLAddressException,

TimeSynchronizationAssistanceInformation,

SCGActivationRequest,

SCGActivationStatus,

CPAInformationRequest,

CPAInformationAck,

CPCInformationRequired,

CPCInformationConfirm,

CPAInformationModReq,

CPAInformationModReqAck,

CPC-DataForwarding-Indicator,

CPCInformationUpdate,

CPACInformationModRequired,

QMCConfigInfo,

FiveGProSeAuthorized,

FiveGProSePC5QoSParameters,

ServedCellSpecificInfoReq-NR,

NRPagingeDRXInformation,

NRPagingeDRXInformationforRRCINACTIVE,

SDTSupportRequest,

SDT-Termination-Request,

SDTPartialUEContextInfo,

SDTDataForwardingDRBList,

PEIPSassistanceInformation,

UESliceMaximumBitRateList,

PagingCause,

MDTPLMNModificationList,

F1-terminatingIAB-donorIndicator,

SRB-ID,

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

id-ActivatedServedCells,

id-ActivationIDforCellActivation,

id-AdditionalDRBIDs,

id-AMF-Region-Information,

id-AMF-Region-Information-To-Add,

id-AMF-Region-Information-To-Delete,

id-AssistanceDataForRANPaging,

id-AvailableDRBIDs,

id-Cause,

id-cellAssistanceInfo-EUTRA,

id-cellAssistanceInfo-NR,

id-CellAndCapacityAssistanceInfo-EUTRA,

id-CellAndCapacityAssistanceInfo-NR,

id-ConfigurationUpdateInitiatingNodeChoice,

id-UEContextID,

id-CriticalityDiagnostics,

id-XnUAddressInfoperPDUSession-List,

id-DesiredActNotificationLevel,

id-DRBsSubjectToStatusTransfer-List,

id-ExpectedUEBehaviour,

id-ExtendedUEIdentityIndexValue,

id-FiveGCMobilityRestrictionListContainer,

id-GlobalNG-RAN-node-ID,

id-GUAMI,

id-indexToRatFrequSelectionPriority,

id-List-of-served-cells-E-UTRA,

id-List-of-served-cells-NR,

id-LocationInformationSN,

id-LocationInformationSNReporting,

id-LocationReportingInformation,

id-LTEUESidelinkAggregateMaximumBitRate,

id-LTEV2XServicesAuthorized,

id-MAC-I,

id-MaskedIMEISV,

id-MDT-Configuration,

id-MDTPLMNList,

id-MN-to-SN-Container,

id-MobilityRestrictionList,

id-M-NG-RANnodeUEXnAPID,

id-new-NG-RAN-Cell-Identity,

id-newNG-RANnodeUEXnAPID,

id-NRUESidelinkAggregateMaximumBitRate,

id-NRV2XServicesAuthorized,

id-oldNG-RANnodeUEXnAPID,

id-OldtoNewNG-RANnodeResumeContainer,

id-PagingCause,

id-PagingDRX,

id-EUTRAPagingeDRXInformation,

id-PagingPriority,

id-PartialListIndicator-EUTRA,

id-PartialListIndicator-NR,

id-PCellID,

id-PDUSessionResourceSecondaryRATUsageList,

id-PDUSessionResourcesActivityNotifyList,

id-PDUSessionResourcesAdmitted-List,

id-PDUSessionResourcesNotAdmitted-List,

id-PDUSessionResourcesNotifyList,

id-PDUSessionToBeAddedAddReq,

id-PDUSessionToBeReleased-RelReqAck,

id-procedureStage,

id-RANPagingArea,

id-requestedSplitSRB,

id-RequiredNumberOfDRBIDs,

id-ResetRequestTypeInfo,

id-ResetResponseTypeInfo,

id-RespondingNodeTypeConfigUpdateAck,

id-RRCResumeCause,

id-SCGreconfigNotification,

id-selectedPLMN,

id-ServedCellsToActivate,

id-servedCellsToUpdate-E-UTRA,

id-ServedCellsToUpdateInitiatingNodeChoice,

id-servedCellsToUpdate-NR,

id-sourceNG-RANnodeUEXnAPID,

id-SpareDRBIDs,

id-S-NG-RANnodeMaxIPDataRate-UL,

id-S-NG-RANnodeMaxIPDataRate-DL,

id-S-NG-RANnodeUEXnAPID,

id-TAISupport-list,

id-Target2SourceNG-RANnodeTranspContainer,

id-targetCellGlobalID,

id-targetNG-RANnodeUEXnAPID,

id-TimeToWait,

id-TNLA-To-Add-List,

id-TNLA-To-Update-List,

id-TNLA-To-Remove-List,

id-TNLA-Setup-List,

id-TNLA-Failed-To-Setup-List,

id-TraceActivation,

id-UEContextInfoHORequest,

id-UEContextInfoRetrUECtxtResp,

id-UEContextKeptIndicator,

id-UEContextRefAtSN-HORequest,

id-UEHistoryInformation,

id-UEIdentityIndexValue,

id-UERANPagingIdentity,

id-UESecurityCapabilities,

id-UserPlaneTrafficActivityReport,

id-XnRemovalThreshold,

id-PDUSessionAdmittedAddedAddReqAck,

id-PDUSessionNotAdmittedAddReqAck,

id-SN-to-MN-Container,

id-RRCConfigIndication,

id-SplitSRB-RRCTransfer,

id-UEReportRRCTransfer,

id-PDUSessionReleasedList-RelConf,

id-BearersSubjectToCounterCheck,

id-PDUSessionToBeReleasedList-RelRqd,

id-ResponseInfo-ReconfCompl,

id-initiatingNodeType-ResourceCoordRequest,

id-respondingNodeType-ResourceCoordResponse,

id-PDUSessionToBeReleased-RelReq,

id-PDUSession-SNChangeRequired-List,

id-PDUSession-SNChangeConfirm-List,

id-PDCPChangeIndication,

id-PC5QoSParameters,

id-SCGConfigurationQuery,

id-UEContextInfo-SNModRequest,

id-requestedSplitSRBrelease,

id-PDUSessionAdmitted-SNModResponse,

id-PDUSessionNotAdmitted-SNModResponse,

id-admittedSplitSRB,

id-admittedSplitSRBrelease,

id-PDUSessionAdmittedModSNModConfirm,

id-PDUSessionReleasedSNModConfirm,

id-s-ng-RANnode-SecurityKey,

id-PDUSessionToBeModifiedSNModRequired,

id-S-NG-RANnodeUE-AMBR,

id-PDUSessionToBeReleasedSNModRequired,

id-target-S-NG-RANnodeID,

id-S-NSSAI,

id-MR-DC-ResourceCoordinationInfo,

id-RANPagingFailure,

id-UERadioCapabilityForPaging,

id-PDUSessionDataForwarding-SNModResponse,

id-Secondary-MN-Xn-U-TNLInfoatM,

id-NE-DC-TDM-Pattern,

id-InterfaceInstanceIndication,

id-S-NG-RANnode-Addition-Trigger-Ind,

id-SNTriggered,

id-DRBs-transferred-to-MN,

id-TNLConfigurationInfo,

id-MessageOversizeNotification,

id-NG-RANTraceID,

id-FastMCGRecoveryRRCTransfer-SN-to-MN,

id-FastMCGRecoveryRRCTransfer-MN-to-SN,

id-RequestedFastMCGRecoveryViaSRB3,

id-AvailableFastMCGRecoveryViaSRB3,

id-RequestedFastMCGRecoveryViaSRB3Release,

id-ReleaseFastMCGRecoveryViaSRB3,

id-CHOinformation-Req,

id-CHOinformation-Ack,

id-targetCellsToCancel,

id-requestedTargetCellGlobalID,

id-DAPSResponseInfo-List,

id-CHO-MRDC-EarlyDataForwarding,

id-CHO-MRDC-Indicator,

id-MobilityInformation,

id-InitiatingCondition-FailureIndication,

id-UEHistoryInformationFromTheUE,

id-HandoverReportType,

id-HandoverCause,

id-SourceCellCGI,

id-TargetCellCGI,

id-ReEstablishmentCellCGI,

id-TargetCellinEUTRAN,

id-SourceCellCRNTI,

id-UERLFReportContainer,

id-NGRAN-Node1-Measurement-ID,

id-NGRAN-Node2-Measurement-ID,

id-RegistrationRequest,

id-ReportCharacteristics,

id-CellToReport,

id-ReportingPeriodicity,

id-CellMeasurementResult,

id-NG-RANnode1CellID,

id-NG-RANnode2CellID,

id-NG-RANnode1MobilityParameters,

id-NG-RANnode2ProposedMobilityParameters,

id-MobilityParametersModificationRange,

id-RACHReportInformation,

id-IABNodeIndication,

id-UERadioCapabilityID,

id-SCGIndicator,

id-UESpecificDRX,

id-PDUSessionExpectedUEActivityBehaviour,

id-DirectForwardingPathAvailability,

id-SourceNG-RAN-node-ID,

id-TargetNodeID,

id-ManagementBasedMDTPLMNList,

id-PrivacyIndicator,

id-TraceCollectionEntityIPAddress,

id-TraceCollectionEntityURI,

id-MBS-Session-ID,

id-UEIdentityIndexList-MBSGroupPaging,

id-MulticastRANPagingArea,

id-MBS-SessionInformation-List,

id-MBS-SessionInformationResponse-List,

id-SuccessfulHOReportInformation,

id-PSCellHistoryInformationRetrieve,

id-SSBOffsets-List,

id-NG-RANnode2SSBOffsetsModificationRange,

id-Coverage-Modification-List,

id-SourcePSCellCGI,

id-FailedPSCellCGI,

id-SCGFailureReportContainer,

id-SNMobilityInformation,

id-SourcePSCellID,

id-SuitablePSCellCGI,

id-PSCellChangeHistory,

id-CHOConfiguration,

id-SCGUEHistoryInformation,

id-F1CTrafficContainer,

id-NoPDUSessionIndication,

id-F1-Terminating-IAB-DonorUEXnAPID,

id-nonF1-Terminating-IAB-DonorUEXnAPID,

id-IAB-TNL-Address-Request,

id-IAB-TNL-Address-Response,

id-TrafficToBeAddedList,

id-TrafficToBeModifiedList,

id-TrafficToBeReleaseInformation,

id-TrafficAddedList,

id-TrafficModifiedList,

id-TrafficNotAddedList,

id-TrafficNotModifiedList,

id-TrafficRequiredToBeModifiedList,

id-TrafficRequiredModifiedList,

id-TrafficReleasedList,

id-IABTNLAddressToBeAdded,

id-IABTNLAddressToBeReleasedList,

id-BoundaryNodeCellsList,

id-ParentNodeCellsList,

id-IABTNLAddressException,

id-CHOinformation-AddReq,

id-CHOinformation-ModReq,

id-TimeSynchronizationAssistanceInformation,

id-SCGActivationRequest,

id-SCGActivationStatus,

id-CPAInformationRequest,

id-CPAInformationAck,

id-CPCInformationRequired,

id-CPCInformationConfirm,

id-CPAInformationModReq,

id-CPAInformationModReqAck,

id-CPC-DataForwarding-Indicator,

id-CPCInformationUpdate,

id-CPACInformationModRequired,

id-QMCConfigInfo,

id-Local-NG-RAN-Node-Identifier,

id-Neighbour-NG-RAN-Node-List,

id-Local-NG-RAN-Node-Identifier-Removal,

id-FiveGProSeAuthorized,

id-FiveGProSePC5QoSParameters,

id-FiveGProSeUEPC5AggregateMaximumBitRate,

id-ServedCellSpecificInfoReq-NR,

id-NRPagingeDRXInformation,

id-NRPagingeDRXInformationforRRCINACTIVE,

id-SDTSupportRequest,

id-SDT-SRB-between-NewNode-OldNode,

id-SDT-Termination-Request,

id-SDTPartialUEContextInfo,

id-SDTDataForwardingDRBList,

id-PEIPSassistanceInformation,

id-UESliceMaximumBitRateList,

id-S-NG-RANnodeUE-Slice-MBR,

id-ManagementBasedMDTPLMNModificationList,

id-F1-terminatingIAB-donorIndicator,

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells

FROM XnAP-Constants;

-- **************************************************************

--

#### -- HANDOVER REQUEST

--

-- **************************************************************

HandoverRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverRequest-IEs}},

...

}

HandoverRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID			CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY reject	TYPE Cause										PRESENCE mandatory}|

{ ID id-targetCellGlobalID					CRITICALITY reject	TYPE Target-CGI									PRESENCE mandatory}|

{ ID id-GUAMI								CRITICALITY reject	TYPE GUAMI										PRESENCE mandatory}|

{ ID id-UEContextInfoHORequest				CRITICALITY reject	TYPE UEContextInfoHORequest						PRESENCE mandatory}|

{ ID id-TraceActivation						CRITICALITY ignore	TYPE TraceActivation							PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore	TYPE MaskedIMEISV								PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore	TYPE UEHistoryInformation						PRESENCE mandatory}|

{ ID id-UEContextRefAtSN-HORequest			CRITICALITY ignore	TYPE UEContextRefAtSN-HORequest				PRESENCE optional }|

{ ID id-CHOinformation-Req					CRITICALITY reject	TYPE CHOinformation-Req							PRESENCE optional }|

{ ID id-NRV2XServicesAuthorized				CRITICALITY ignore	TYPE NRV2XServicesAuthorized					PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized			CRITICALITY ignore	TYPE LTEV2XServicesAuthorized					PRESENCE optional }|

{ ID id-PC5QoSParameters					CRITICALITY ignore	TYPE PC5QoSParameters							PRESENCE optional }|

{ ID id-MobilityInformation					CRITICALITY ignore	TYPE MobilityInformation						PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE		CRITICALITY ignore	TYPE UEHistoryInformationFromTheUE			PRESENCE optional }|

{ ID id-IABNodeIndication					CRITICALITY reject	TYPE IABNodeIndication							PRESENCE optional }|

{ ID id-NoPDUSessionIndication				CRITICALITY ignore	TYPE NoPDUSessionIndication						PRESENCE optional }|

{ ID id-TimeSynchronizationAssistanceInformation	CRITICALITY ignore	TYPE TimeSynchronizationAssistanceInformation		PRESENCE optional }|

{ ID id-QMCConfigInfo						CRITICALITY ignore	TYPE QMCConfigInfo								PRESENCE optional }|

{ ID id-FiveGProSeAuthorized				CRITICALITY ignore	TYPE FiveGProSeAuthorized						PRESENCE optional }|

{ ID id-FiveGProSePC5QoSParameters			CRITICALITY ignore	TYPE FiveGProSePC5QoSParameters				PRESENCE optional}|

{ ID id-DisasterPLMNIdentity				CRITICALITY ignore	TYPE PLMN-Identity PRESENCE optional }

,

...

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes  >>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT RESPONSE

--

-- **************************************************************

RetrieveUEContextResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RetrieveUEContextResponse-IEs}},

...

}

RetrieveUEContextResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID						CRITICALITY ignore	TYPE NG-RANnodeUEXnAPID									PRESENCE mandatory}|

{ ID id-oldNG-RANnodeUEXnAPID						CRITICALITY ignore	TYPE NG-RANnodeUEXnAPID									PRESENCE mandatory}|

{ ID id-GUAMI										CRITICALITY reject	TYPE GUAMI													PRESENCE mandatory}|

{ ID id-UEContextInfoRetrUECtxtResp					CRITICALITY reject	TYPE UEContextInfoRetrUECtxtResp						PRESENCE mandatory}|

{ ID id-TraceActivation								CRITICALITY ignore	TYPE TraceActivation										PRESENCE optional }|

{ ID id-MaskedIMEISV								CRITICALITY ignore	TYPE MaskedIMEISV											PRESENCE optional }|

{ ID id-LocationReportingInformation				CRITICALITY ignore	TYPE LocationReportingInformation						PRESENCE optional }|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional }|

{ ID id-NRV2XServicesAuthorized						CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized					CRITICALITY ignore	TYPE LTEV2XServicesAuthorized							PRESENCE optional }|

{ ID id-PC5QoSParameters							CRITICALITY ignore	TYPE PC5QoSParameters										PRESENCE optional }|

{ ID id-UEHistoryInformation						CRITICALITY ignore	TYPE UEHistoryInformation								PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE				CRITICALITY ignore	TYPE UEHistoryInformationFromTheUE					PRESENCE optional }|

{ ID id-MDTPLMNList									CRITICALITY ignore	TYPE MDTPLMNList											PRESENCE optional }|

{ ID id-IABNodeIndication							CRITICALITY reject	TYPE IABNodeIndication									PRESENCE optional }|

{ ID id-UEContextRefAtSN-HORequest					CRITICALITY ignore	TYPE UEContextRefAtSN-HORequest						PRESENCE optional }|

{ ID id-TimeSynchronizationAssistanceInformation	CRITICALITY ignore	TYPE TimeSynchronizationAssistanceInformation		PRESENCE optional }|

{ ID id-QMCConfigInfo								CRITICALITY ignore	TYPE QMCConfigInfo											PRESENCE optional }|

{ ID id-FiveGProSeAuthorized						CRITICALITY ignore	TYPE FiveGProSeAuthorized								PRESENCE optional }|

{ ID id-FiveGProSePC5QoSParameters					CRITICALITY ignore	TYPE FiveGProSePC5QoSParameters						PRESENCE optional}|

|

{ ID id-DisasterPLMNIdentity				CRITICALITY ignore	TYPE PLMN-Identity PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes  >>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST

--

-- **************************************************************

SNodeAdditionRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequest-IEs}},

...

}

SNodeAdditionRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-UESecurityCapabilities				CRITICALITY reject		TYPE UESecurityCapabilities						PRESENCE mandatory}|

{ ID id-s-ng-RANnode-SecurityKey			CRITICALITY reject		TYPE S-NG-RANnode-SecurityKey					PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUE-AMBR					CRITICALITY reject		TYPE UEAggregateMaximumBitRate					PRESENCE mandatory}|

{ ID id-selectedPLMN						CRITICALITY ignore		TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-MobilityRestrictionList				CRITICALITY ignore		TYPE MobilityRestrictionList						PRESENCE optional }|

{ ID id-indexToRatFrequSelectionPriority	CRITICALITY reject		TYPE RFSP-Index										PRESENCE optional }|

{ ID id-PDUSessionToBeAddedAddReq			CRITICALITY reject		TYPE PDUSessionToBeAddedAddReq					PRESENCE mandatory}|

{ ID id-MN-to-SN-Container					CRITICALITY reject		TYPE OCTET STRING									PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE optional }|

{ ID id-ExpectedUEBehaviour					CRITICALITY ignore		TYPE ExpectedUEBehaviour							PRESENCE optional }|

{ ID id-requestedSplitSRB					CRITICALITY reject		TYPE SplitSRBsTypes									PRESENCE optional }|

{ ID id-PCellID								CRITICALITY reject		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-DesiredActNotificationLevel			CRITICALITY ignore		TYPE DesiredActNotificationLevel				PRESENCE optional }|

{ ID id-AvailableDRBIDs						CRITICALITY reject		TYPE DRB-List										PRESENCE conditional}

-- The IE shall be present if there is at least one  PDUSessionResourceSetupInfo-SNterminated included --|

{ ID id-S-NG-RANnodeMaxIPDataRate-UL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-DL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-LocationInformationSNReporting		CRITICALITY ignore		TYPE LocationInformationSNReporting			PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo		CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore		TYPE MaskedIMEISV									PRESENCE optional }|

{ ID id-NE-DC-TDM-Pattern					CRITICALITY ignore		TYPE NE-DC-TDM-Pattern								PRESENCE optional }|

{ ID id-S-NG-RANnode-Addition-Trigger-Ind	CRITICALITY reject		TYPE S-NG-RANnode-Addition-Trigger-Ind		PRESENCE optional }|

{ ID id-TraceActivation						CRITICALITY ignore		TYPE TraceActivation								PRESENCE optional }|

{ ID id-RequestedFastMCGRecoveryViaSRB3		CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3			PRESENCE optional }|

{ ID id-UERadioCapabilityID					CRITICALITY reject		TYPE UERadioCapabilityID							PRESENCE optional }|

{ ID id-SourceNG-RAN-node-ID				CRITICALITY ignore		TYPE GlobalNG-RANNode-ID							PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNList			CRITICALITY ignore		TYPE MDTPLMNList									PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore		TYPE UEHistoryInformation							PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE		CRITICALITY ignore		TYPE UEHistoryInformationFromTheUE				PRESENCE optional }|

{ ID id-PSCellChangeHistory					CRITICALITY ignore		TYPE PSCellChangeHistory							PRESENCE optional }|

{ ID id-IABNodeIndication					CRITICALITY reject		TYPE IABNodeIndication								PRESENCE optional }|

{ ID id-NoPDUSessionIndication				CRITICALITY ignore		TYPE NoPDUSessionIndication						PRESENCE optional }|

{ ID id-CHOinformation-AddReq				CRITICALITY reject		TYPE CHOinformation-AddReq						PRESENCE optional }|

{ ID id-SCGActivationRequest				CRITICALITY ignore		TYPE SCGActivationRequest							PRESENCE optional }|

{ ID id-CPAInformationRequest				CRITICALITY reject		TYPE CPAInformationRequest						PRESENCE optional }|

{ ID id-S-NG-RANnodeUE-Slice-MBR				CRITICALITY reject		TYPE UESliceMaximumBitRateList					PRESENCE optional }|

{ ID id-F1-terminatingIAB-donorIndicator		CRITICALITY reject		TYPE F1-terminatingIAB-donorIndicator			PRESENCE optional }|

{ ID id-DisasterPLMNIdentity				CRITICALITY ignore	TYPE PLMN-Identity PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes  >>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-DefaultDRB-Allowed,

id-DLCarrierList,

id-EndpointIPAddressAndPort,

id-ExtendedReportIntervalMDT,

id-ExtendedTAISliceSupportList,

id-FiveGCMobilityRestrictionListContainer,

id-SecondarydataForwardingInfoFromTarget-List,

id-LastE-UTRANPLMNIdentity,

id-IntendedTDD-DL-ULConfiguration-NR,

id-MaxIPrate-DL,

id-SecurityResult,

id-OldQoSFlowMap-ULendmarkerexpected,

id-PDUSessionCommonNetworkInstance,

id-PDUSession-PairID,

id-BPLMN-ID-Info-EUTRA,

id-BPLMN-ID-Info-NR,

id-DRBsNotAdmittedSetupModifyList,

id-Secondary-MN-Xn-U-TNLInfoatM,

id-ULForwardingProposal,

id-DRB-IDs-takenintouse,

id-SplitSessionIndicator,

id-NonGBRResources-Offered,

id-MDT-Configuration,

id-TraceCollectionEntityURI,

id-NPN-Broadcast-Information,

id-NPNPagingAssistanceInformation,

id-NPNMobilityInformation,

id-NPN-Support,

id-LTEUESidelinkAggregateMaximumBitRate,

id-NRUESidelinkAggregateMaximumBitRate,

id-ExtendedRATRestrictionInformation,

id-QoSMonitoringRequest,

id-QoSMonitoringDisabled,

id-QosMonitoringReportingFrequency,

id-DAPSRequestInfo,

id-OffsetOfNbiotChannelNumberToDL-EARFCN,

id-OffsetOfNbiotChannelNumberToUL-EARFCN,

id-NBIoT-UL-DL-AlignmentOffset,

id-TDDULDLConfigurationCommonNR,

id-CarrierList,

id-ULCarrierList,

id-FrequencyShift7p5khz,

id-SSB-PositionsInBurst,

id-NRCellPRACHConfig,

id-Redundant-UL-NG-U-TNLatUPF,

id-Redundant-DL-NG-U-TNLatNG-RAN,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-ExtendedPacketDelayBudget,

id-Additional-Redundant-UL-NG-U-TNLatUPF-List,

id-RedundantCommonNetworkInstance,

id-TSCTrafficCharacteristics,

id-RedundantQoSFlowIndicator,

id-Additional-PDCP-Duplication-TNL-List,

id-RedundantPDUSessionInformation,

id-UsedRSNInformation,

id-RLCDuplicationInformation,

id-CSI-RSTransmissionIndication,

id-UERadioCapabilityID,

id-secondary-SN-UL-PDCP-UP-TNLInfo,

id-pdcpDuplicationConfiguration,

id-duplicationActivation,

id-NPRACHConfiguration,

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated,

id-DL-scheduling-PDCCH-CCE-usage,

id-UL-scheduling-PDCCH-CCE-usage,

id-SFN-Offset,

id-QoS-Mapping-Information,

id-AdditionLocationInformation,

id-dataForwardingInfoFromTargetE-UTRANnode,

id-Cause,

id-SecurityIndication,

id-RRCConnReestab-Indicator,

id-SourceDLForwardingIPAddress,

id-SourceNodeDLForwardingIPAddress,

id-M4ReportAmount,

id-M5ReportAmount,

id-M6ReportAmount,

id-M7ReportAmount,

id-BeamMeasurementIndicationM1,

id-Supported-MBS-FSA-ID-List,

id-MBS-SessionAssociatedInformation,

id-MBS-SessionInformation-List,

id-SliceRadioResourceStatus-List,

id-CompositeAvailableCapacitySupplementaryUplink,

id-SSBOffsets-List,

id-NG-RANnode2SSBOffsetsModificationRange,

id-NR-U-Channel-List,

id-NR-U-ChannelInfo-List,

id-MIMOPRBusageInformation,

id-UEAssistantIdentifier,

id-IAB-MT-Cell-List,

id-NoPDUSessionIndication,

id-permutation,

id-UL-GNB-DU-Cell-Resource-Configuration,

id-DL-GNB-DU-Cell-Resource-Configuration,

id-tdd-GNB-DU-Cell-Resource-Configuration,

id-Additional-Measurement-Timing-Configuration-List,

id-SurvivalTime,

id-Local-NG-RAN-Node-Identifier,

id-Neighbour-NG-RAN-Node-List,

id-FiveGProSeUEPC5AggregateMaximumBitRate,

id-Redcap-Bcast-Information,

id-UESliceMaximumBitRateList,

id-PositioningInformation,

id-ServedCellSpecificInfoReq-NR,

id-TAINSAGSupportList,

id-earlyMeasurement,

id-BeamMeasurementsReportConfiguration,

id-CoverageModificationCause,

id-UERLFReportContainerLTEExtension,

id-ExcessPacketDelayThresholdConfiguration,

id-DisasterPLMNIdentity,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

maxnoofCAGs,

maxnoofCAGsperPLMN,

maxnoofCellsinAoI,

maxnoofCellsinNG-RANnode,

maxnoofCellsinRNA,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEPLMNs,

maxnoofEPLMNsplus1,

maxnoofEUTRABands,

maxnoofEUTRABPLMNs,

maxnoofForbiddenTACs,

maxnoofMBSFNEUTRA,

maxnoofMultiConnectivityMinusOne,

maxnoofNeighbours,

maxnoofNIDs,

maxnoofNRCellBands,

maxnoofPDUSessions,

maxnoofPLMNs,

maxnoofProtectedResourcePatterns,

maxnoofQoSFlows,

maxnoofQoSParaSets,

maxnoofRANAreaCodes,

maxnoofRANAreasinRNA,

maxnoofSCellGroups,

maxnoofSCellGroupsplus1,

maxnoofSliceItems,

maxnoofExtSliceItems,

maxnoofSNPNIDs,

maxnoofsupportedTACs,

maxnoofsupportedPLMNs,

maxnoofTAI,

maxnoofTAIsinAoI,

maxnoofTNLAssociations,

maxnoofUEContexts,

maxNRARFCN,

maxNrOfErrors,

maxnoofRANNodesinAoI,

maxnooftimeperiods,

maxnoofslots,

maxnoofExtTLAs,

maxnoofGTPTLAs,

maxnoofCHOcells,

maxnoofPC5QoSFlows,

maxnoofSSBAreas,

maxnoofNRSCSs,

maxnoofPhysicalResourceBlocks,

maxnoofRACHReports,

maxnoofAdditionalPDCPDuplicationTNL,

maxnoofRLCDuplicationstate,

maxnoofBluetoothName,

maxnoofCellIDforMDT,

maxnoofMDTPLMNs,

maxnoofTAforMDT,

maxnoofWLANName,

maxnoofSensorName,

maxnoofNeighPCIforMDT,

maxnoofFreqforMDT,

maxnoofNonAnchorCarrierFreqConfig,

maxnoofDataForwardingTunneltoE-UTRAN,

maxnoofUEIDIndicesforMBSPaging,

maxnoofMBSFSAs,

maxnoofMBSQoSFlows,

maxnoofMRBs,

maxnoofCellsforMBS,

maxnoofMBSServiceAreaInformation,

maxnoofTAIforMBS,

maxnoofAssociatedMBSSessions,

maxnoofMBSSessions,

maxnoofSuccessfulHOReports,

maxnoofPSCellsPerSN,

maxnoofNR-UChannelIDs,

maxnoofCellsinCHO,

maxnoofCHOexecutioncond,

maxnoofServingCells,

maxnoofBHInfo,

maxnoofTLAsIAB,

maxnoofTrafficIndexEntries,

maxnoofBAPControlPDURLCCHs,

maxnoofServedCellsIAB,

maxnoofDUFSlots,

maxnoofSymbols,

maxnoofHSNASlots,

maxnoofRBsetsPerCell,

maxnoofChildIABNodes,

maxnoofIABSTCInfo,

maxnoofPSCellCandidates,

maxnoofTargetSNs,

maxnoofUEAppLayerMeas,

maxnoofSNSSAIforQMC,

maxnoofCellIDforQMC,

maxnoofPLMNforQMC,

maxnoofTAforQMC,

maxnoofMTCItems,

maxnoofCSIRSconfigurations,

maxnoofCSIRSneighbourCells,

maxnoofCSIRSneighbourCellsInMTC,

maxnoofNeighbour-NG-RAN-Nodes,

maxnoofSRBs,

maxnoofSMBR,

maxnoofNSAGs,

maxnoofRBsetsPerCell1,

maxnoofTargetSNsMinusOne,

maxnoofThresholdsForExcessPacketDelay

FROM XnAP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM XnAP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Single-Container{},

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES

FROM XnAP-Containers;

<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes  >>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= 27

id-traceStart													ProcedureCode ::= 28

id-handoverSuccess												ProcedureCode ::= 29

id-conditionalHandoverCancel									ProcedureCode ::= 30

id-earlyStatusTransfer											ProcedureCode ::= 31

id-failureIndication												ProcedureCode ::= 32

id-handoverReport												ProcedureCode ::= 33

id-resourceStatusReportingInitiation							ProcedureCode ::= 34

id-resourceStatusReporting										ProcedureCode ::= 35

id-mobilitySettingsChange										ProcedureCode ::= 36

id-accessAndMobilityIndication									ProcedureCode ::= 37

id-cellTrafficTrace 											ProcedureCode ::= 38

id-RANMulticastGroupPaging										ProcedureCode ::= 39

id-scgFailureInformationReport									ProcedureCode ::= 40

id-ProcedureCode41-NotToBeUsed									ProcedureCode ::= 41

id-scgFailureTransfer											ProcedureCode ::= 42

id-f1CTrafficTransfer											ProcedureCode ::= 43

id-iABTransportMigrationManagement								ProcedureCode ::= 44

id-iABTransportMigrationModification							ProcedureCode ::= 45

id-iABResourceCoordination										ProcedureCode ::= 46

id-retrieveUEContextConfirm										ProcedureCode ::= 47

id-cPCCancel													ProcedureCode ::= 48

id-partialUEContextTransfer										ProcedureCode ::= 49

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 262143

maxnoofAllowedAreas							INTEGER ::= 16

maxnoofAMFRegions							INTEGER ::= 16

maxnoofAoIs									INTEGER ::= 64

maxnoofBluetoothName						INTEGER ::= 4

maxnoofBPLMNs								INTEGER ::= 12

maxnoofCAGs									INTEGER ::= 12

maxnoofCAGsperPLMN							INTEGER ::= 256

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofCellsinAoI							INTEGER ::= 256

maxnoofCellsinUEHistoryInfo					INTEGER ::= 16

maxnoofCellsinNG-RANnode					INTEGER ::= 16384

maxnoofCellsinRNA							INTEGER ::= 32

maxnoofCellsUEMovingTrajectory				INTEGER ::= 16

maxnoofDRBs									INTEGER ::= 32

maxnoofEUTRABands							INTEGER ::= 16

maxnoofEUTRABPLMNs							INTEGER ::= 6

maxnoofEPLMNs								INTEGER ::= 15

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofEPLMNsplus1							INTEGER ::= 16

maxnoofForbiddenTACs							INTEGER ::= 4096

maxnoofFreqforMDT							INTEGER ::= 8

maxnoofMBSFNEUTRA							INTEGER ::= 8

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofMultiConnectivityMinusOne			INTEGER ::= 3

maxnoofNeighbours							INTEGER ::= 1024

maxnoofNeighPCIforMDT						INTEGER ::= 32

maxnoofNIDs									INTEGER ::= 12

maxnoofNRCellBands							INTEGER ::= 32

maxnoofPLMNs								INTEGER ::= 16

maxnoofPDUSessions							INTEGER ::= 256

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoofQoSFlows								INTEGER ::= 64

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofRANAreaCodes							INTEGER ::= 32

maxnoofRANAreasinRNA						INTEGER ::= 16

maxnoofRANNodesinAoI						INTEGER ::= 64

maxnoofSCellGroups							INTEGER ::= 3

maxnoofSCellGroupsplus1						INTEGER ::= 4

maxnoofSensorName							INTEGER ::= 3

maxnoofSliceItems							INTEGER ::= 1024

maxnoofSNPNIDs								INTEGER ::= 12

maxnoofsupportedPLMNs						INTEGER ::= 12

maxnoofsupportedTACs						INTEGER ::= 256

maxnoofTAforMDT								INTEGER ::= 8

maxnoofTAI									INTEGER ::= 16

maxnoofTAIsinAoI 							INTEGER ::= 16

maxnooftimeperiods							INTEGER ::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofUEContexts							INTEGER ::= 8192

maxNRARFCN									INTEGER ::= 3279165

maxNrOfErrors								INTEGER ::= 256

maxnoofslots								INTEGER ::= 5120

maxnoofExtTLAs								INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofCHOcells								INTEGER ::= 8

maxnoofPC5QoSFlows 							INTEGER ::= 2064

maxnoofSSBAreas								INTEGER ::= 64

maxnoofRACHReports							INTEGER ::= 64

maxnoofNRSCSs								INTEGER ::= 5

maxnoofPhysicalResourceBlocks				INTEGER ::= 275

maxnoofAdditionalPDCPDuplicationTNL			INTEGER ::= 2

maxnoofRLCDuplicationstate					INTEGER ::= 3

maxnoofWLANName								INTEGER ::= 4

maxnoofNonAnchorCarrierFreqConfig			INTEGER ::= 15

maxnoofDataForwardingTunneltoE-UTRAN		INTEGER ::= 256

maxnoofMBSFSAs								INTEGER ::= 256

maxnoofUEIDIndicesforMBSPaging				INTEGER ::= 4096

maxnoofMBSQoSFlows							INTEGER ::= 64

maxnoofMRBs									INTEGER ::= 32

maxnoofCellsforMBS							INTEGER ::= 8192

maxnoofMBSServiceAreaInformation				INTEGER ::= 256

maxnoofTAIforMBS 							INTEGER ::= 1024

maxnoofAssociatedMBSSessions				INTEGER ::= 32

maxnoofMBSSessions							INTEGER ::= 256

maxnoofSuccessfulHOReports					INTEGER ::= 64

maxnoofPSCellsPerSN 						INTEGER ::= 8

maxnoofNR-UChannelIDs						INTEGER ::= 16

maxnoofCellsinCHO							INTEGER ::= 8

maxnoofCHOexecutioncond						INTEGER ::= 2

maxnoofServedCellsIAB						INTEGER ::= 512

maxnoofServingCells							INTEGER ::= 32

maxnoofBHInfo								INTEGER ::= 1024

maxnoofTrafficIndexEntries					INTEGER	::=	1024

maxnoofTLAsIAB								INTEGER	::=	1024

maxnoofBAPControlPDURLCCHs					INTEGER ::= 2

maxnoofIABSTCInfo							INTEGER ::= 45

maxnoofSymbols								INTEGER ::= 14

maxnoofDUFSlots								INTEGER ::= 320

maxnoofHSNASlots							INTEGER ::= 5120

maxnoofRBsetsPerCell						INTEGER ::= 8

maxnoofRBsetsPerCell1						INTEGER ::= 7

maxnoofChildIABNodes						INTEGER ::= 1024

maxnoofPSCellCandidates						INTEGER ::= 8

maxnoofTargetSNs 							INTEGER ::= 8

maxnoofUEAppLayerMeas						INTEGER ::= 16

maxnoofSNSSAIforQMC							INTEGER ::= 16

maxnoofCellIDforQMC							INTEGER ::= 32

maxnoofPLMNforQMC							INTEGER ::= 16

maxnoofTAforQMC								INTEGER ::= 8

maxnoofMTCItems								INTEGER ::= 16

maxnoofCSIRSconfigurations					INTEGER ::= 96

maxnoofCSIRSneighbourCells					INTEGER ::= 16

maxnoofCSIRSneighbourCellsInMTC				INTEGER ::= 16

maxnoofNeighbour-NG-RAN-Nodes				INTEGER ::= 256

maxnoofSRBs									INTEGER ::= 5

maxnoofSMBR									INTEGER ::= 8

maxnoofNSAGs								INTEGER ::= 256

maxnoofTargetSNsMinusOne					INTEGER ::= 7

maxnoofThresholdsForExcessPacketDelay		INTEGER ::= 255

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation																	ProtocolIE-ID ::= 1

id-admittedSplitSRB																					ProtocolIE-ID ::= 2

id-admittedSplitSRBrelease																			ProtocolIE-ID ::= 3

id-AMF-Region-Information																			ProtocolIE-ID ::= 4

id-AssistanceDataForRANPaging																		ProtocolIE-ID ::= 5

id-BearersSubjectToCounterCheck																		ProtocolIE-ID ::= 6

id-Cause																							ProtocolIE-ID ::= 7

id-cellAssistanceInfo-NR																			ProtocolIE-ID ::= 8

id-ConfigurationUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 9

id-CriticalityDiagnostics																			ProtocolIE-ID ::= 10

id-XnUAddressInfoperPDUSession-List																	ProtocolIE-ID ::= 11

id-DRBsSubjectToStatusTransfer-List																	ProtocolIE-ID ::= 12

id-ExpectedUEBehaviour																				ProtocolIE-ID ::= 13

id-GlobalNG-RAN-node-ID																				ProtocolIE-ID ::= 14

id-GUAMI																							ProtocolIE-ID ::= 15

id-indexToRatFrequSelectionPriority																	ProtocolIE-ID ::= 16

id-initiatingNodeType-ResourceCoordRequest															ProtocolIE-ID ::= 17

id-List-of-served-cells-E-UTRA																		ProtocolIE-ID ::= 18

id-List-of-served-cells-NR																			ProtocolIE-ID ::= 19

id-LocationReportingInformation																		ProtocolIE-ID ::= 20

id-MAC-I																							ProtocolIE-ID ::= 21

id-MaskedIMEISV																						ProtocolIE-ID ::= 22

id-M-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 23

id-MN-to-SN-Container																				ProtocolIE-ID ::= 24

id-MobilityRestrictionList																			ProtocolIE-ID ::= 25

id-new-NG-RAN-Cell-Identity																			ProtocolIE-ID ::= 26

id-newNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 27

id-UEReportRRCTransfer																				ProtocolIE-ID ::= 28

id-oldNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 29

id-OldtoNewNG-RANnodeResumeContainer																ProtocolIE-ID ::= 30

id-PagingDRX																						ProtocolIE-ID ::= 31

id-PCellID																							ProtocolIE-ID ::= 32

id-PDCPChangeIndication																				ProtocolIE-ID ::= 33

id-PDUSessionAdmittedAddedAddReqAck																	ProtocolIE-ID ::= 34

id-PDUSessionAdmittedModSNModConfirm																ProtocolIE-ID ::= 35

id-PDUSessionAdmitted-SNModResponse																	ProtocolIE-ID ::= 36

id-PDUSessionNotAdmittedAddReqAck																	ProtocolIE-ID ::= 37

id-PDUSessionNotAdmitted-SNModResponse																ProtocolIE-ID ::= 38

id-PDUSessionReleasedList-RelConf																	ProtocolIE-ID ::= 39

id-PDUSessionReleasedSNModConfirm																	ProtocolIE-ID ::= 40

id-PDUSessionResourcesActivityNotifyList															ProtocolIE-ID ::= 41

id-PDUSessionResourcesAdmitted-List																	ProtocolIE-ID ::= 42

id-PDUSessionResourcesNotAdmitted-List																ProtocolIE-ID ::= 43

id-PDUSessionResourcesNotifyList																	ProtocolIE-ID ::= 44

id-PDUSession-SNChangeConfirm-List																	ProtocolIE-ID ::= 45

id-PDUSession-SNChangeRequired-List																	ProtocolIE-ID ::= 46

id-PDUSessionToBeAddedAddReq																		ProtocolIE-ID ::= 47

id-PDUSessionToBeModifiedSNModRequired																ProtocolIE-ID ::= 48

id-PDUSessionToBeReleasedList-RelRqd																ProtocolIE-ID ::= 49

id-PDUSessionToBeReleased-RelReq																	ProtocolIE-ID ::= 50

id-PDUSessionToBeReleasedSNModRequired																ProtocolIE-ID ::= 51

id-RANPagingArea																					ProtocolIE-ID ::= 52

id-PagingPriority																					ProtocolIE-ID ::= 53

id-requestedSplitSRB																				ProtocolIE-ID ::= 54

id-requestedSplitSRBrelease																			ProtocolIE-ID ::= 55

id-ResetRequestTypeInfo																				ProtocolIE-ID ::= 56

id-ResetResponseTypeInfo																			ProtocolIE-ID ::= 57

id-RespondingNodeTypeConfigUpdateAck																ProtocolIE-ID ::= 58

id-respondingNodeType-ResourceCoordResponse															ProtocolIE-ID ::= 59

id-ResponseInfo-ReconfCompl																			ProtocolIE-ID ::= 60

id-RRCConfigIndication																				ProtocolIE-ID ::= 61

id-RRCResumeCause																					ProtocolIE-ID ::= 62

id-SCGConfigurationQuery																			ProtocolIE-ID ::= 63

id-selectedPLMN																						ProtocolIE-ID ::= 64

id-ServedCellsToActivate																			ProtocolIE-ID ::= 65

id-servedCellsToUpdate-E-UTRA																		ProtocolIE-ID ::= 66

id-ServedCellsToUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 67

id-servedCellsToUpdate-NR																			ProtocolIE-ID ::= 68

id-s-ng-RANnode-SecurityKey																			ProtocolIE-ID ::= 69

id-S-NG-RANnodeUE-AMBR																				ProtocolIE-ID ::= 70

id-S-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 71

id-SN-to-MN-Container																				ProtocolIE-ID ::= 72

id-sourceNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 73

id-SplitSRB-RRCTransfer																				ProtocolIE-ID ::= 74

id-TAISupport-list																					ProtocolIE-ID ::= 75

id-TimeToWait																						ProtocolIE-ID ::= 76

id-Target2SourceNG-RANnodeTranspContainer															ProtocolIE-ID ::= 77

id-targetCellGlobalID																				ProtocolIE-ID ::= 78

id-targetNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 79

id-target-S-NG-RANnodeID																			ProtocolIE-ID ::= 80

id-TraceActivation																					ProtocolIE-ID ::= 81

id-UEContextID																						ProtocolIE-ID ::= 82

id-UEContextInfoHORequest																			ProtocolIE-ID ::= 83

id-UEContextInfoRetrUECtxtResp																		ProtocolIE-ID ::= 84

id-UEContextInfo-SNModRequest																		ProtocolIE-ID ::= 85

id-UEContextKeptIndicator																			ProtocolIE-ID ::= 86

id-UEContextRefAtSN-HORequest																		ProtocolIE-ID ::= 87

id-UEHistoryInformation																				ProtocolIE-ID ::= 88

id-UEIdentityIndexValue																				ProtocolIE-ID ::= 89

id-UERANPagingIdentity																				ProtocolIE-ID ::= 90

id-UESecurityCapabilities																			ProtocolIE-ID ::= 91

id-UserPlaneTrafficActivityReport																	ProtocolIE-ID ::= 92

id-XnRemovalThreshold 																				ProtocolIE-ID ::= 93

id-DesiredActNotificationLevel																		ProtocolIE-ID ::= 94

id-AvailableDRBIDs 																					ProtocolIE-ID ::= 95

id-AdditionalDRBIDs 																				ProtocolIE-ID ::= 96

id-SpareDRBIDs 																						ProtocolIE-ID ::= 97

id-RequiredNumberOfDRBIDs 																			ProtocolIE-ID ::= 98

id-TNLA-To-Add-List																					ProtocolIE-ID ::= 99

id-TNLA-To-Update-List																				ProtocolIE-ID ::= 100

id-TNLA-To-Remove-List																				ProtocolIE-ID ::= 101

id-TNLA-Setup-List																					ProtocolIE-ID ::= 102

id-TNLA-Failed-To-Setup-List																		ProtocolIE-ID ::= 103

id-PDUSessionToBeReleased-RelReqAck																	ProtocolIE-ID ::= 104

id-S-NG-RANnodeMaxIPDataRate-UL																		ProtocolIE-ID ::= 105

id-PDUSessionResourceSecondaryRATUsageList															ProtocolIE-ID ::= 107

id-Additional-UL-NG-U-TNLatUPF-List																	ProtocolIE-ID ::= 108

id-SecondarydataForwardingInfoFromTarget-List														ProtocolIE-ID ::= 109

id-LocationInformationSNReporting																	ProtocolIE-ID ::= 110

id-LocationInformationSN																			ProtocolIE-ID ::= 111

id-LastE-UTRANPLMNIdentity																			ProtocolIE-ID ::= 112

id-S-NG-RANnodeMaxIPDataRate-DL																		ProtocolIE-ID ::= 113

id-MaxIPrate-DL																						ProtocolIE-ID ::= 114

id-SecurityResult																					ProtocolIE-ID ::= 115

id-S-NSSAI																							ProtocolIE-ID ::= 116

id-MR-DC-ResourceCoordinationInfo																	ProtocolIE-ID ::= 117

id-AMF-Region-Information-To-Add																	ProtocolIE-ID ::= 118

id-AMF-Region-Information-To-Delete																	ProtocolIE-ID ::= 119

id-OldQoSFlowMap-ULendmarkerexpected																ProtocolIE-ID ::= 120

id-RANPagingFailure																					ProtocolIE-ID ::= 121

id-UERadioCapabilityForPaging																		ProtocolIE-ID ::= 122

id-PDUSessionDataForwarding-SNModResponse															ProtocolIE-ID ::= 123

id-DRBsNotAdmittedSetupModifyList																	ProtocolIE-ID ::= 124

id-Secondary-MN-Xn-U-TNLInfoatM																		ProtocolIE-ID ::= 125

id-NE-DC-TDM-Pattern																				ProtocolIE-ID ::= 126

id-PDUSessionCommonNetworkInstance																	ProtocolIE-ID ::= 127

id-BPLMN-ID-Info-EUTRA																				ProtocolIE-ID ::= 128

id-BPLMN-ID-Info-NR																					ProtocolIE-ID ::= 129

id-InterfaceInstanceIndication																		ProtocolIE-ID ::= 130

id-S-NG-RANnode-Addition-Trigger-Ind																ProtocolIE-ID ::= 131

id-DefaultDRB-Allowed																				ProtocolIE-ID ::= 132

id-DRB-IDs-takenintouse																				ProtocolIE-ID ::= 133

id-SplitSessionIndicator																			ProtocolIE-ID ::= 134

id-CNTypeRestrictionsForEquivalent																	ProtocolIE-ID ::= 135

id-CNTypeRestrictionsForServing																		ProtocolIE-ID ::= 136

id-DRBs-transferred-to-MN																			ProtocolIE-ID ::= 137

id-ULForwardingProposal																				ProtocolIE-ID ::= 138

id-EndpointIPAddressAndPort 																		ProtocolIE-ID ::= 139

id-IntendedTDD-DL-ULConfiguration-NR																ProtocolIE-ID ::= 140

id-TNLConfigurationInfo																				ProtocolIE-ID ::= 141

id-PartialListIndicator-NR																			ProtocolIE-ID ::= 142

id-MessageOversizeNotification																		ProtocolIE-ID ::= 143

id-CellAndCapacityAssistanceInfo-NR																	ProtocolIE-ID ::= 144

id-NG-RANTraceID																					ProtocolIE-ID ::= 145

id-NonGBRResources-Offered																			ProtocolIE-ID ::= 146

id-FastMCGRecoveryRRCTransfer-SN-to-MN																ProtocolIE-ID ::= 147

id-RequestedFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 148

id-AvailableFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 149

id-RequestedFastMCGRecoveryViaSRB3Release															ProtocolIE-ID ::= 150

id-ReleaseFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 151

id-FastMCGRecoveryRRCTransfer-MN-to-SN																ProtocolIE-ID ::= 152

id-ExtendedRATRestrictionInformation																ProtocolIE-ID ::= 153

id-QoSMonitoringRequest																				ProtocolIE-ID ::= 154

id-FiveGCMobilityRestrictionListContainer															ProtocolIE-ID ::= 155

id-PartialListIndicator-EUTRA																		ProtocolIE-ID ::= 156

id-CellAndCapacityAssistanceInfo-EUTRA																ProtocolIE-ID ::= 157

id-CHOinformation-Req																				ProtocolIE-ID ::= 158

id-CHOinformation-Ack																				ProtocolIE-ID ::= 159

id-targetCellsToCancel																				ProtocolIE-ID ::= 160

id-requestedTargetCellGlobalID																		ProtocolIE-ID ::= 161

id-procedureStage																					ProtocolIE-ID ::= 162

id-DAPSRequestInfo																					ProtocolIE-ID ::= 163

id-DAPSResponseInfo-List																			ProtocolIE-ID ::= 164

id-CHO-MRDC-Indicator																				ProtocolIE-ID ::= 165

id-OffsetOfNbiotChannelNumberToDL-EARFCN															ProtocolIE-ID ::= 166

id-OffsetOfNbiotChannelNumberToUL-EARFCN															ProtocolIE-ID ::= 167

id-NBIoT-UL-DL-AlignmentOffset																		ProtocolIE-ID ::= 168

id-LTEV2XServicesAuthorized																			ProtocolIE-ID ::= 169

id-NRV2XServicesAuthorized																			ProtocolIE-ID ::= 170

id-LTEUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 171

id-NRUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 172

id-PC5QoSParameters																					ProtocolIE-ID ::= 173

id-AlternativeQoSParaSetList																		ProtocolIE-ID ::= 174

id-CurrentQoSParaSetIndex																			ProtocolIE-ID ::= 175

id-MobilityInformation 																				ProtocolIE-ID ::= 176

id-InitiatingCondition-FailureIndication															ProtocolIE-ID ::= 177

id-UEHistoryInformationFromTheUE																	ProtocolIE-ID ::= 178

id-HandoverReportType																				ProtocolIE-ID ::= 179

id-HandoverCause																					ProtocolIE-ID ::= 180

id-SourceCellCGI																					ProtocolIE-ID ::= 181

id-TargetCellCGI																					ProtocolIE-ID ::= 182

id-ReEstablishmentCellCGI																			ProtocolIE-ID ::= 183

id-TargetCellinEUTRAN																				ProtocolIE-ID ::= 184

id-SourceCellCRNTI																					ProtocolIE-ID ::= 185

id-UERLFReportContainer																				ProtocolIE-ID ::= 186

id-NGRAN-Node1-Measurement-ID																		ProtocolIE-ID ::= 187

id-NGRAN-Node2-Measurement-ID																		ProtocolIE-ID ::= 188

id-RegistrationRequest																				ProtocolIE-ID ::= 189

id-ReportCharacteristics																				ProtocolIE-ID ::= 190

id-CellToReport																								ProtocolIE-ID ::= 191

id-ReportingPeriodicity																					ProtocolIE-ID ::= 192

id-CellMeasurementResult																				ProtocolIE-ID ::= 193

id-NG-RANnode1CellID																						ProtocolIE-ID ::= 194

id-NG-RANnode2CellID																				ProtocolIE-ID ::= 195

id-NG-RANnode1MobilityParameters																	ProtocolIE-ID ::= 196

id-NG-RANnode2ProposedMobilityParameters															ProtocolIE-ID ::= 197

id-MobilityParametersModificationRange																ProtocolIE-ID ::= 198

id-TDDULDLConfigurationCommonNR																		ProtocolIE-ID ::= 199

id-CarrierList																						ProtocolIE-ID ::= 200

id-ULCarrierList																					ProtocolIE-ID ::= 201

id-FrequencyShift7p5khz																				ProtocolIE-ID ::= 202

id-SSB-PositionsInBurst																				ProtocolIE-ID ::= 203

id-NRCellPRACHConfig																				ProtocolIE-ID ::= 204

id-RACHReportInformation																			ProtocolIE-ID ::= 205

id-IABNodeIndication																				ProtocolIE-ID ::= 206

id-Redundant-UL-NG-U-TNLatUPF																		ProtocolIE-ID ::= 207

id-CNPacketDelayBudgetDownlink																		ProtocolIE-ID ::= 208

id-CNPacketDelayBudgetUplink																		ProtocolIE-ID ::= 209

id-Additional-Redundant-UL-NG-U-TNLatUPF-List														ProtocolIE-ID ::= 210

id-RedundantCommonNetworkInstance																	ProtocolIE-ID ::= 211

id-TSCTrafficCharacteristics																		ProtocolIE-ID ::= 212

id-RedundantQoSFlowIndicator																		ProtocolIE-ID ::= 213

id-Redundant-DL-NG-U-TNLatNG-RAN																	ProtocolIE-ID ::= 214

id-ExtendedPacketDelayBudget																		ProtocolIE-ID ::= 215

id-Additional-PDCP-Duplication-TNL-List																ProtocolIE-ID ::= 216

id-RedundantPDUSessionInformation																	ProtocolIE-ID ::= 217

id-UsedRSNInformation																				ProtocolIE-ID ::= 218

id-RLCDuplicationInformation																		ProtocolIE-ID ::= 219

id-NPN-Broadcast-Information																		ProtocolIE-ID ::= 220

id-NPNPagingAssistanceInformation																	ProtocolIE-ID ::= 221

id-NPNMobilityInformation																			ProtocolIE-ID ::= 222

id-NPN-Support																						ProtocolIE-ID ::= 223

id-MDT-Configuration																				ProtocolIE-ID ::= 224

id-MDTPLMNList																						ProtocolIE-ID ::= 225

id-TraceCollectionEntityURI																			ProtocolIE-ID ::= 226

id-UERadioCapabilityID																				ProtocolIE-ID ::= 227

id-CSI-RSTransmissionIndication																		ProtocolIE-ID ::= 228

id-SNTriggered																						ProtocolIE-ID ::= 229

id-DLCarrierList																					ProtocolIE-ID ::= 230

id-ExtendedTAISliceSupportList																		ProtocolIE-ID ::= 231

id-cellAssistanceInfo-EUTRA																			ProtocolIE-ID ::= 232

id-ConfiguredTACIndication																			ProtocolIE-ID ::= 233

id-secondary-SN-UL-PDCP-UP-TNLInfo																	ProtocolIE-ID ::= 234

id-pdcpDuplicationConfiguration																		ProtocolIE-ID ::= 235

id-duplicationActivation																			ProtocolIE-ID ::= 236

id-NPRACHConfiguration																				ProtocolIE-ID ::= 237

id-QosMonitoringReportingFrequency																	ProtocolIE-ID ::= 238

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated													ProtocolIE-ID ::= 239

id-DL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 240

id-UL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 241

id-SFN-Offset																						ProtocolIE-ID ::= 242

id-QoSMonitoringDisabled																				ProtocolIE-ID ::= 243

id-ExtendedUEIdentityIndexValue																		ProtocolIE-ID ::= 244

id-EUTRAPagingeDRXInformation																		ProtocolIE-ID ::= 245

id-CHO-MRDC-EarlyDataForwarding																		ProtocolIE-ID ::= 246

id-SCGIndicator																						ProtocolIE-ID ::= 247

id-UESpecificDRX																					ProtocolIE-ID ::= 248

id-PDUSessionExpectedUEActivityBehaviour															ProtocolIE-ID ::= 249

id-QoS-Mapping-Information																			ProtocolIE-ID ::= 250

id-AdditionLocationInformation																		ProtocolIE-ID ::= 251

id-dataForwardingInfoFromTargetE-UTRANnode															ProtocolIE-ID ::= 252

id-DirectForwardingPathAvailability																	ProtocolIE-ID ::= 253

id-SourceNG-RAN-node-ID																				ProtocolIE-ID ::= 254

id-SourceDLForwardingIPAddress																		ProtocolIE-ID ::= 255

id-SourceNodeDLForwardingIPAddress																	ProtocolIE-ID ::= 256

id-ExtendedReportIntervalMDT																			ProtocolIE-ID ::= 257

id-SecurityIndication																				ProtocolIE-ID ::= 258

id-RRCConnReestab-Indicator																			ProtocolIE-ID ::= 259

id-TargetNodeID																						ProtocolIE-ID ::= 260

id-ManagementBasedMDTPLMNList																		ProtocolIE-ID ::= 261

id-PrivacyIndicator																					ProtocolIE-ID ::= 262

id-TraceCollectionEntityIPAddress																	ProtocolIE-ID ::= 263

id-M4ReportAmount																					ProtocolIE-ID ::= 264

id-M5ReportAmount																					ProtocolIE-ID ::= 265

id-M6ReportAmount																					ProtocolIE-ID ::= 266

id-M7ReportAmount																					ProtocolIE-ID ::= 267

id-BeamMeasurementIndicationM1																		ProtocolIE-ID ::= 268

id-MBS-Session-ID																					ProtocolIE-ID ::= 269

id-UEIdentityIndexList-MBSGroupPaging																	ProtocolIE-ID ::= 270

id-MulticastRANPagingArea																			ProtocolIE-ID ::= 271

id-Supported-MBS-FSA-ID-List																		ProtocolIE-ID ::= 272

id-MBS-SessionInformation-List																		ProtocolIE-ID ::= 273

id-MBS-SessionInformationResponse-List																ProtocolIE-ID ::= 274

id-MBS-SessionAssociatedInformation																	ProtocolIE-ID ::= 275

id-SuccessfulHOReportInformation																	ProtocolIE-ID ::= 276

id-SliceRadioResourceStatus-List																	ProtocolIE-ID ::= 277

id-CompositeAvailableCapacitySupplementaryUplink													ProtocolIE-ID ::= 278

id-SCGUEHistoryInformation																			ProtocolIE-ID ::= 279

id-SSBOffsets-List																					ProtocolIE-ID ::= 280

id-NG-RANnode2SSBOffsetModificationRange															ProtocolIE-ID ::= 281

id-Coverage-Modification-List																		ProtocolIE-ID ::= 282

id-NR-U-Channel-List																				ProtocolIE-ID ::= 283

id-SourcePSCellCGI																					ProtocolIE-ID ::= 284

id-FailedPSCellCGI																					ProtocolIE-ID ::= 285

id-SCGFailureReportContainer																		ProtocolIE-ID ::= 286

id-SNMobilityInformation																			ProtocolIE-ID ::= 287

id-SourcePSCellID																					ProtocolIE-ID ::= 288

id-SuitablePSCellCGI																				ProtocolIE-ID ::= 289

id-PSCellChangeHistory																				ProtocolIE-ID ::= 290

id-CHOConfiguration																					ProtocolIE-ID ::= 291

id-NR-U-ChannelInfo-List																			ProtocolIE-ID ::= 292

id-PSCellHistoryInformationRetrieve																	ProtocolIE-ID ::= 293

id-NG-RANnode2SSBOffsetsModificationRange															ProtocolIE-ID ::= 294

id-MIMOPRBusageInformation																			ProtocolIE-ID ::= 295

id-F1CTrafficContainer																				ProtocolIE-ID ::= 296

id-IAB-MT-Cell-List																					ProtocolIE-ID ::= 297

id-NoPDUSessionIndication																			ProtocolIE-ID ::= 298

id-IAB-TNL-Address-Request																			ProtocolIE-ID ::= 299

id-IAB-TNL-Address-Response																			ProtocolIE-ID ::= 300

id-TrafficToBeAddedList																				ProtocolIE-ID ::= 301

id-TrafficToBeModifiedList																			ProtocolIE-ID ::= 302

id-TrafficToBeReleaseInformation																	ProtocolIE-ID ::= 303

id-TrafficAddedList																					ProtocolIE-ID ::= 304

id-TrafficModifiedList																				ProtocolIE-ID ::= 305

id-TrafficNotAddedList																				ProtocolIE-ID ::= 306

id-TrafficNotModifiedList																			ProtocolIE-ID ::= 307

id-TrafficRequiredToBeModifiedList																	ProtocolIE-ID ::= 308

id-TrafficRequiredModifiedList																		ProtocolIE-ID ::= 309

id-TrafficReleasedList																				ProtocolIE-ID ::= 310

id-IABTNLAddressToBeAdded																			ProtocolIE-ID ::= 311

id-IABTNLAddressToBeReleasedList																	ProtocolIE-ID ::= 312

id-nonF1-Terminating-IAB-DonorUEXnAPID																ProtocolIE-ID ::= 313

id-F1-Terminating-IAB-DonorUEXnAPID																	ProtocolIE-ID ::= 314

id-BoundaryNodeCellsList																			ProtocolIE-ID ::= 315

id-ParentNodeCellsList																				ProtocolIE-ID ::= 316

id-tdd-GNB-DU-Cell-Resource-Configuration															ProtocolIE-ID ::= 317

id-UL-GNB-DU-Cell-Resource-Configuration															ProtocolIE-ID ::= 318

id-DL-GNB-DU-Cell-Resource-Configuration															ProtocolIE-ID ::= 319

id-permutation																						ProtocolIE-ID ::= 320

id-IABTNLAddressException																			ProtocolIE-ID ::= 321

id-CHOinformation-AddReq																			ProtocolIE-ID ::= 322

id-CHOinformation-ModReq																			ProtocolIE-ID ::= 323

id-SurvivalTime																						ProtocolIE-ID ::= 324

id-TimeSynchronizationAssistanceInformation															ProtocolIE-ID ::= 325

id-SCGActivationRequest																				ProtocolIE-ID ::= 326

id-SCGActivationStatus																				ProtocolIE-ID ::= 327

id-CPAInformationRequest																			ProtocolIE-ID ::= 328

id-CPAInformationAck																				ProtocolIE-ID ::= 329

id-CPCInformationRequired																			ProtocolIE-ID ::= 330

id-CPCInformationConfirm																			ProtocolIE-ID ::= 331

id-CPAInformationModReq																				ProtocolIE-ID ::= 332

id-CPAInformationModReqAck																			ProtocolIE-ID ::= 333

id-CPC-DataForwarding-Indicator																		ProtocolIE-ID ::= 334

id-CPCInformationUpdate																				ProtocolIE-ID ::= 335

id-CPACInformationModRequired																		ProtocolIE-ID ::= 336

id-QMCConfigInfo																					ProtocolIE-ID ::= 337

id-ProtocolIE-ID338-NotToBeUsed																		ProtocolIE-ID ::= 338

id-Additional-Measurement-Timing-Configuration-List													ProtocolIE-ID ::= 339

id-PDUSession-PairID																					ProtocolIE-ID ::= 340

id-Local-NG-RAN-Node-Identifier																		ProtocolIE-ID ::= 341

id-Neighbour-NG-RAN-Node-List																		ProtocolIE-ID ::= 342

id-Local-NG-RAN-Node-Identifier-Removal																ProtocolIE-ID ::= 343

id-FiveGProSeAuthorized																				ProtocolIE-ID ::= 344

id-FiveGProSePC5QoSParameters																		ProtocolIE-ID ::= 345

id-FiveGProSeUEPC5AggregateMaximumBitRate															ProtocolIE-ID ::= 346

id-ServedCellSpecificInfoReq-NR																		ProtocolIE-ID ::= 347

id-NRPagingeDRXInformation																			ProtocolIE-ID ::= 348

id-NRPagingeDRXInformationforRRCINACTIVE																ProtocolIE-ID ::= 349

id-Redcap-Bcast-Information																			ProtocolIE-ID ::= 350

id-SDTSupportRequest																				ProtocolIE-ID ::= 351

id-SDT-SRB-between-NewNode-OldNode																	ProtocolIE-ID ::= 352

id-SDT-Termination-Request																			ProtocolIE-ID ::= 353

id-SDTPartialUEContextInfo																			ProtocolIE-ID ::= 354

id-SDTDataForwardingDRBList																			ProtocolIE-ID ::= 355

id-PagingCause																						ProtocolIE-ID ::= 356

id-PEIPSassistanceInformation																		ProtocolIE-ID ::= 357

id-UESliceMaximumBitRateList																			ProtocolIE-ID ::= 358

id-S-NG-RANnodeUE-Slice-MBR																			ProtocolIE-ID ::= 359

id-PositioningInformation																			ProtocolIE-ID ::= 360

id-UEAssistantIdentifier																			ProtocolIE-ID ::= 361

id-ManagementBasedMDTPLMNModificationList															ProtocolIE-ID ::= 362

id-F1-terminatingIAB-donorIndicator																	ProtocolIE-ID ::= 363

id-TAINSAGSupportList																				ProtocolIE-ID ::= 364

id-SCGreconfigNotification																			ProtocolIE-ID ::= 365

id-earlyMeasurement																					ProtocolIE-ID ::= 366

id-BeamMeasurementsReportConfiguration																ProtocolIE-ID ::= 367

id-CoverageModificationCause																			ProtocolIE-ID ::= 368

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated									ProtocolIE-ID ::= 369

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-DisasterPLMNIdentity 																		ProtocolIE-ID ::= 372

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes  >>>>>>>>>>>>>>>>>>>>>>>>>>