3GPP TSG-RAN WG3 Meeting #119	R3-230092

# Athens, Greece, 27th Feb – 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1907 | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on IAB Authorized IE in UE Context Modification procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Qualcomm, CATT, Huawei, Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2023-01-31 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | IAB’s authorization can be changed to “not authorized” during its normal operation. MME may choose to detach the IAB-MT (including MME initiate the UE Context Release procedure to MeNB), or MME may keep the IAB-MT attached as a normal UE (e.g. has an optional PDN connection for OAM). In the later case, MME initiates the UE Context Modification procedure to inform the MeNB that the IAB authorized is changed to “not authorized”. MeNB need to ensure the IAB’s operation is stopped, e.g. initiate Secondary Node Release procedure, etc. However, current specification only defines “the eNB shall store the received IAB Authorized IE, if supported, in the UE context”, but does not define the MeNB to take actions to stop the IAB operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the eNB need to take actions to ensure the IAB node will not serve the UE(s). Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact under functional point of view. The eNB need to take actions to ensure the IAB node will not serve the UE(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When the IAB authorized information is changed to “not authorized”, the IAB incorrectly continue its operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR 0927 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.3.4	UE Context Modification

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to partly modify the established UE Context, e.g., with the Security Key or the Subscriber Profile ID for RAT/Frequency priority. The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation.

** Unchanged text skipped **

Upon receipt of the UE CONTEXT MODIFICATION REQUEST message, the eNB shall

-	store the received Security Key IE, take it into use and associate it with the initial value of NCC as defined in TS 33.401 [15]

-	store the UE Security Capabilities IE and take them into use together with the received keys according to TS 33.401 [15].

-	if supported, store the NR UE Security Capabilities IE and use it as defined in TS 33.401 [15]

-	store the Subscriber Profile ID for RAT/Frequency priority IE and use it as defined in TS 36.300 [14].

-	if supported, store the Additional RRM Policy Index IE and use it as defined in TS 36.300 [14].

-	store the received IAB Authorized IE, if supported, in the UE context. If the IAB Authorized IE is set to "not authorized" for an IAB-MT, the eNB shall, if supported, initiate actions to ensure that the IAB node will not serve the UE(s).

If the UE Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message the eNB shall:

-	replace the previously provided UE Aggregate Maximum Bit Rate by the received UE Aggregate Maximum Bit Rate in the UE context;

-	use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>