3GPP TSG-RAN WG3 Meeting #119	R3-230105

Athens, GR, 27 Feb – 03 Mar, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0272 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of MT-SDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MT_SDT |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of MT-SDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the stage2 call flow of MT-SDT procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support MT-SDT in case of disaggregated architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.18.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------Start of the First Change---------------------

## 8.18	Overall procedure for Small Data Transmission during RRC Inactive

----------------Start of the First Change---------------------

### 8.18.x	Paging triggered SDT (MT-SDT)

The procedure for paging triggered SDT (MT-SDT) is shown in Figure 8.18.x-1.

Figure 8.18.x-1: Paging triggered MT-SDT.

1.	The gNB-CU-UP and gNB-CU-CP receive DL SDT data and signalling from 5GC, respectively.

2.	In case of DL SDT data arrival, the gNB-CU-UP sends DL DATA NOTIFICATION message including an indication of DL SDT data arrival, and may together with SDT assistance information (e.g. the DL SDT data volume) to the gNB-CU-CP.

3.	Based on the DL SDT signalling received in step 1, and the DL SDT related information received in step 2, the gNB-CU-CP sends to the gNB-DU the PAGING message including an indication of MT-SDT, and may together with the SDT assistance information, e.g. the volume of both DL SDT data and DL SDT signalling.

4.	The gNB-DU makes decision on whether to page the UE with MT-SDT indicator, takes the assistance information received in step 3 in to account.

5.	The gNB-DU sends Paging message to the UE, and may provide MT-SDT indicator.

6.	Upon receiving MT-SDT indicator in paging, the UE initiates the MO-SDT (the RACH based SDT with/without anchor relocation, or CG based SDT), with the following differences:

-	The UE sends RRCResumeRequest message with the resume cause mt-SDT, and may without user data and/or signalling.

Note 1: in case the DU triggers Paging message without MT-SDT indicator in step 5, if the UE has been successfully reached, it attempts to resume from RRC_INACTIVE, as described in clause 8.9.6.2.

----------------End of the Change---------------------