3GPP TSG-RAN WG3 #119		R3-230143

27th Feb – 3rd Mar 2023

Athens, Greece

Agenda Item:	11.3

Source:	CATT

Title:	(TP to BLCR for TS38.423) Support for legacy QoE in NR-DC

Document for:	Approval

- Introduction

As discussed in R3-230142, it is proposed to introduce class 1 non-UE associated procedure c for the UE selection and configuration sending node selection for m-based QoE if two steps solution adopted

# 2 	TP for TS38.423

//////////////////////////////////////////////////////////////irrelevant operations skipped//////////////////////////////////////////////////////////

#### 9.1.2.x1	QoE INFORMATION COORDINATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node or the S-NG-RAN node to the M-NG-RAN node to request the QoE information Coordination.

Direction: S-NG-RAN node  M-NG-RAN node and M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| QoE Reference List |  | 1 |  |  | YES | reject |
| >QoE Reference Item |  | 1..<maxnoofUEAppLayerMeas> |  |  | - |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [45]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3-bytes Octet String. | - |  |
| >>Container for Application Layer Measurement Configuration | O |  | OCTET STRING (SIZE(1..8000)) | Contains the signalling based QoE measurement configuration, see Annex L in TS 26.247 [47], clause 16.5 in TS 26.114 [53] and clause 9 in TS 26.118 [54]. | – |  |
| >>>Selected UE for M-based QoE Item |  | 1.. < maxnoofSelectedUE> |  | List of selected UE for management-based QoE. | – |  |
| >>>>M-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | - |  |
| >>>>S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | - |  |
| >>>>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, …) | The IE indicates the identity of the application layer measurement configuration, as defined in TS 38.331 [18]. | - |  |
| >>>>Configuration Node | O |  | ENUMERATED (MN, SN, ...) | The IE indicates which node sends the QoE configuration to UE | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of UE application layer measurement. Value is 16. |
| maxnoofSelectedUE | Maximum no. of selected UE for management -based QoE. Value is 232 . |

#### 9.1.2.x2	QoE INFORMATION COORDINATION RESPONSE

This message is sent by the M-NG-RAN node to the S-NG-RAN node or the S-NG-RAN node to the M-NG-RAN node as response to QoE INFORMATION COORDINATION REQUEST

Direction: M-NG-RAN node  S-NG-RAN node and S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| QoE Reference List |  | 1 |  |  | YES | reject |
| >QoE Reference Item |  | 1..<maxnoofUEAppLayerMeas> |  |  | - |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [45]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3-bytes Octet String. | - |  |
| >>Container for Application Layer Measurement Configuration | O |  | OCTET STRING (SIZE(1..8000)) | Contains the signalling based QoE measurement configuration, see Annex L in TS 26.247 [47], clause 16.5 in TS 26.114 [53] and clause 9 in TS 26.118 [54]. | – |  |
| >>Selected UE for M-based QoE Accepted List |  | 1 |  | List of selected UE for management-based QoE. | – |  |
| >>>Selected UE for M-based QoE Accepted Item |  | 1.. < maxnoofSelectedUE> |  | List of selected UE for management-based QoE. | – |  |
| >>>>M-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | - |  |
| >>>>S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | - |  |
| >>>>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, …) | The IE indicates the identity of the application layer measurement configuration, as defined in TS 38.331 [18]. | - |  |
| >>>>Configuration Node | O |  | ENUMERATED (MN, SN, ...) | The IE indicates which node sends the QoE configuration to UE | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of UE application layer measurement. Value is 16. |
| maxnoofSelectedUE | Maximum no. of selected UE for management -based QoE. Value is 232 . |