3GPP TSG-RAN WG3#119                                             	                     R3-230210

Athens, Greece, 27 February – 03 March 2023

Agenda item:	8.1

Source:	Nokia, Nokia Shanghai Bell

Title: 	RAN impacts of NPN release 18 enhancements

Document for:	Approval

# Introduction

RAN3 has received the incoming LS in [3] whereby SA2 reports that SA2 has started the specification work on TS 23501 and TS 23502 concerning NPN enhancements in release 18.

RAN3 is impacted by key issue 1 and key issue 2 as follows:

1.	Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection, including:

2.	Support for non-3GPP access for SNPN.

This paper provides an early analysis of the impacts foreseen on RAN to evaluate the work to be done by RAN3.

# Discussion

- First evaluation of impact of key issue 1

NG Impacts

The objective related to key issue 1 deals with the support of idle and connected mode mobility between SNPNs.

For idle mode mobility, the UE will select the target SNPN. RAN3 is not impacted by this mobility in idle mode.

For connected mode mobility, we assume that source gNB will be the entity to select the target SNPN.

In order for the source gNB to select the target SNPN, it must receive the list of equivalent SNPNs from the AMF.

The list can be sent in the Initial Context setup request. This can be included in the mobility restriction list, alike the list of equivalent PLMNs today.

Observation 1: the AMF can send the list of equivalent PLMNs to the NG-RAN within the Mobility restriction list in the Initial context setup over NGAP.

Another key aspect once target source gNB has selected the target SNPN is to indicate this choice to 5GC at the time of the mobility of the UE.

In current NGAP protocol, the target ID is included in the Handover Required message to be handled by the AMF. For example, the target TAI can be used by the AMF to select the target AMF.

In case the target is target SNPN, the selection of the target AMF should be done not only based on the target TAI but also based on target SNPN ID (or alternatively the target NID).

It is thus required that the target ID is enhanced with this information.

Observation 2: the target ID should be enhanced with including the target SNPN ID (or target NID) and be included in handover required.

As a result, at minimum the following impacts can be anticipated onto the NGAP protocol as a starting point:

- Mobility restriction list (TS 38.413 section 9.3.1.85) present in HO Request, DL NAS Transport, Initial Context setup.

- Target ID (TS 38.413 section 9.3.1.25) present in Handover Required.

Xn Impacts

Same as for NG interface, the involved information elements must also be propagated over Xn interface during Xn handover. This means:

- The source gNB needs to send the list of equivalent PLMNs to the target gNB within the Mobility restriction list in the Handover Request message over XnAP.

- The source gNB needs to send the selected NID in the Handover Request over XnAP.

Observation 3: the Xn Handover Request should be enhanced including the selected NID or SNPN ID (or target NID) and also the list of equivalent PLMNs within the Mobility restriction list.

- First evaluation of impact of key issue 2

The objective related to key issue deals with support of non-3GPP access to SNPN.

The selected SNPN ID (or alternatively the selected NID) needs to be indicated to AMF.

When the UE accesses through the TNGF the Initial UE Message should indicate to the AMF which SNPN ID is involved.

The NGAP Initial UE Message already contains the SNPN ID, however this is used today for 3GPP access as per release 16.

TS 38.413 therefore would not need any impact as such however there is a need to discuss reflecting in TS 29.248 that the IE also applies to non-3GPP access from release 18 onwards.

Observation 4:  the sending of selected SNPN ID needs to be extended over the NGAP Initial UE Message for non-3GPP access.

# Conclusion and Proposal

Following the LS received from SA2 in [3] this paper has provided a first analysis of the impacts on RAN foreseen from the work on NPN enhancements in release 18 corresponding to the two following objectives originating from SA2:

1.	Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection, including:

2.	Support for non-3GPP access for SNPN.

The analysis result into the following observations:

Observation 1: for key issue 1, the AMF can send the list of equivalent PLMNs to the NG-RAN within the Mobility restriction list in the Initial context setup over NGAP.

Observation 2: for key issue 1, the target ID should be enhanced with including the target SNPN ID (or target NID) and be included in handover required.

Observation 3: for key issue 1, the Xn Handover Request should be enhanced including the selected NID or SNPN ID (or target NID) and also the list of equivalent PLMNs within the Mobility restriction list.

Observation 4:  for key issue 2, the sending of selected SNPN ID needs to be extended over the NGAP Initial UE Message for non-3GPP access.

As RAN3 work will progress these observations can become agreements.

At this stage, only a first feedback can be given to SA2 that RAN3 has identified the RAN impacts to be manageable and can complete the work in time if there is timely approval of a WID.

Proposal 1: RAN3 to initiate necessary work based on approval of associated WID.

Proposal 2: RAN3 to provide a high level reply LS to SA2 concerning RAN3 impact analysis.

A draft response LS is available in [4].

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO, Inc.

- SP-221340, Work Item on NPN Enhancements

- R3-230036, LS on RAN Impacts for NPN Enhancement in release 18

- R3-230211, Response LS on RAN Impacts for NPN Enhancements in release 18