3GPP TSG-RAN WG3 Meeting #119	R3-230235

Athens, Greece, February 27th – March 3rd, 2023

Agenda item: 		10.2.3

Source:	Intel Corporation

Title:	RA report enhancement for RACH partitioning

Document for:		Discussion and Decision

# Introduction

This contribution makes a further analysis on the RACH partitioning related parameters that need to be included in RA report for RACH optimization.

# Discussion

Two parameters, i.e. feature or feature combination that triggered the RACH and Used feature combination, were agreed to be added into RA report for RACH partitioning [1]. Besides, there are some other parameters proposed to be added into the RA report for RACH partitioning in the previous RAN3 meetings, e.g. feature priorities configured by network, RA resources configured with feature indicator. These parameters are configured by network nodes which could be the same node the UE sends the RA report to. Even it is not case, the UE can send the RA report after connecting to a different node that is not the one who configured the UE, the configuration information could be retrieved the between network nodes, for example, a Retrieve UE Context-like procedure can be used to retrieve these configuration information from the old gNB, and a UE ID can be included to identify the UE. If proven necessary, RAN3 still needs further discussion on the detailed mechanism.

In order to save air interface resources, we think it is not needed to include any network configured parameters in the RA report.

Proposal 1: Network configured parameters are not necessary to be included in the RA report.

Proposal 2: If proven necessary, RAN3 to further discuss the retrieving mechanism for the feature configuration information between network nodes.

Besides the RA report, in order to facilitate the network to evaluate the network resource utilization for each feature or feature combination, it is also beneficial to include the feature combination related information for each RACH procedure in RLF report and SHR.

Proposal 3: To add feature combination related information in RLF report and SHR.

# Conclusion

In this contribution, we analyzed the RACH report optimization for RACH partitioning, and the following proposals were drawn from the analysis.

Proposal 1: Network configured parameters are not necessary to be included in the RA report.

Proposal 2: If proven necessary, RAN3 to further discuss the retrieving mechanism for the feature configuration information between network nodes.

Proposal 3: To add feature combination related information in RLF report and SHR.

# References

- R2_119bis-e_meeting_report