3GPP TSG-RAN WG3 Meeting #119	R3-230237

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:		10.2.4

Source:	Intel Corporation

Title:	TP to TS 38.413 on area scope for NPN

Document for:		Other

# Introduction

This contribution provides TP to TS 38.413 on area scope for NPN.

# TP to TS 38.413

<Start of change>

#### 9.3.1.169	MDT Configuration-NR

This IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace, …) |  |
| CHOICE Area Scope of MDT | M |  |  |  |
| >Cell based |  |  |  |  |
| >>Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  |
| >>>NR CGI | M |  | 9.3.1.7 |  |
| >TA based |  |  |  |  |
| >>TA List for MDT |  | 1..<maxnoofTAforMDT> |  |  |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. |
| >PLMN wide |  |  | NULL |  |
| >TAI based |  |  |  |  |
| >>TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  |
| >>>TAI | M |  |  |  |
| >CAG based |  |  |  |  |
| >>CAG ID List for MDT |  | 1..<maxnoofCAGIDforMDT> |  |  |
| >>>CAG ID | O |  | 9.3.3.43 |  |
| >NID based |  |  |  |  |
| >>NID List for MDT |  | 1..<maxnoofNIDforMDT> |  |  |
| >>>NID | O |  | 9.3.3.42 |  |
| CHOICE MDT Mode | M |  |  |  |
| >Immediate MDT |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [41]. First Bit = M1, Second Bit= M2, Third Bit = M4, Fourth Bit = M5, Fifth Bit = M6, Sixth Bit = M7, Seventh Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, other bits reserved for future use. Value “1” indicates “activate” and value “0” indicates “do not activate”. |
| >>M1 Configuration | C-ifM1 |  | 9.3.1.171 |  |
| >>M4 Configuration | C-ifM4 |  | 9.3.1.172 |  |
| >>M5 Configuration | C-ifM5 |  | 9.3.1.173 |  |
| >>M6 Configuration | C-ifM6 |  | 9.3.1.174 |  |
| >>M7 Configuration | C-ifM7 |  | 9.3.1.175 |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  |
| >>MDT Location Information | O |  | 9.3.1.176 |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  |
| >Logged MDT |  |  |  |  |
| >>Logging interval | M |  | ENUMERATED (320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms, 20480ms, 30720ms, 40960ms, 61440ms, infinity, …) | This IE is defined in TS 38.331 [18]. |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90,120, …) | This IE is defined in TS 38.331 [18]. Unit: [minute]. |
| >>CHOICE Report Type | M |  |  |  |
| >>>Periodical |  |  | NULL |  |
| >>>Event Triggered |  |  |  |  |
| >>>>Event Trigger Logged MDT Configuration | M |  | 9.3.1.180 |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  |
| >>Area Scope of Neighbour Cells | O |  | 9.3.1.182 |  |
| Signalling Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.168 |  |
| Area Scope of MDT for NPN | M |  | 9.3.3.x |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |
| maxnoofCAGIDforMDT | Maximum no. of CAG ID for MDT scope. Value is FFS. |
| maxnoofNIDforMDT | Maximum no. of NID for MDT scope. Value is FFS. |

| Condition | Explanation |
| --- | --- |
| C-ifM1 | This IE shall be present if the Measurements to Activate IE has the first bit set to “1”. |
| C-ifM4 | This IE shall be present if the Measurements to Activate IE has the third bit set to “1”. |
| C-ifM5 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”. |
| C-ifM6 | This IE shall be present if the Measurements to Activate IE has the fitth bit set to “1”. |
| C-ifM7 | This IE shall be present if the Measurements to Activate IE has the sixth bit set to “1”. |

<End of change>

<Start of change>

#### 9.3.3.x	Area Scope of MDT for NPN

This IE indicates the area scope of MDT for NPI-NPN and SNPN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Area Scope of MDT for NPN | M |  |  |  |
| >NPI-NPN based |  |  |  |  |
| >>NPI-NPN ID List for MDT |  | 1..<maxnoofNPINPNIDforMDT> |  |  |
| >>>PLMN ID |  |  |  |  |
| >>>CAG ID List for MDT |  | 1..<maxnoofCAGIDforMDT> |  |  |
| >>>>CAG ID | O |  | 9.3.3.43 |  |
| >SNPN ID based |  |  |  |  |
| >> SNPN ID List for MDT |  | 1..<maxnoofSNPNIDforMDT> |  |  |
| >>>PLMN ID |  |  |  |  |
| >>> NID List for MDT |  | 1..<maxnoofNIDforMDT> |  |  |
| >>>NID | O |  | 9.3.3.42 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofNPINPNIDforMDT | Maximum no. of NPI-NPN ID for MDT scope. Value is FFS. |
| maxnoofCAGIDforMDT | Maximum no. of CAG ID for MDT scope. Value is FFS. |
| maxnoofSNPNIDforMDT | Maximum no. of SNPN ID for MDT scope. Value is FFS. |
| maxnoofNIDforMDT | Maximum no. of NID for MDT scope. Value is FFS. |

<End of change>