3GPP TSG-RAN WG3 #119	R3-230250

Athens, Greece, 27th February – 3rd March 2023

Source:	CATT

Title:	Discussion on XnAP impacts of validity time and accuracy

Agenda Item:	12.2.2.1

Document for:	Discussion and decision

- Introduction

This discussion paper focuses on validity time and accuracy of prediction of load balancing, although it is also applicable for quantised predictions in other use cases.

- Discussion

  - Validity time of prediction

Load balancing predictions are typically averages over a given time duration.

In SON load metrics are defined that the time duration is as long as the periodicity. Assume that the time to report is t0 and the periodicity is T, the content of it is typically the average measured between t0-T and t0. The next time to report is t0+T and its content is the average between t0 and t0+T.

Now in AI/ML we are to introduce predictions, which talk about the future rather than the past. The most straightforward method could be a mirror image of load reports in SON, i.e.:

- The prediction provided at t0 is (ideally) the average between t0 and t0+T;

- The prediction provided at t0+T is (ideally) the average between t0+T and t0+2T.

However such straightforward method may not be sufficient enough.

RAN nodes generally wish any decision is suitable as long as possible, e.g. it should not be changed or rolled back within ½T. On the other hand, RAN nodes may take RRM decision at any moment, e.g. possibly at t0+¾T. If such case occurs, the RAN node will naturally wish it has got the prediction covering the duration between t0+¾T and t0+1¼T. But according to the method above, it has not. What it got is only for the duration between t0 and t0+T, received at t0.

Maybe a better approach is that, the node making such prediction provides at t0 not only the one for the duration between t0 and t0+T, but also the one for the duration between t0+T and t0+2T. Another approach is to provide the prediction simply for the duration between t0 and t0+2T. We slightly prefer the former one.

Proposal 1: Assume that the time to report prediction is t0 and the periodicity is T, Either of the following two bullets should be supported:
-	In addition to the one for t0 till t0+T, the one for t0+T till t0+2T or even further is also provided;
-	The validity duration is longer than the period, e.g. the prediction is for t0 till t0+2T.

  - Accuracy and confidence of prediction

Load balancing predictions (including the ones agreed last meeting and the ones tagged FFS) are all quantised ones. Theoretically the most accurate expression of accuracy/confidence of quantised predictions should be probability distributions (this is indeed available if one runs multiple models for a lot of times), but maybe we includes only the accuracy part (maybe more precisely, uncertainty) is sufficient enough.

Accuracies are ideally not constants. It can be drastically different depending on situations.

In weather forecast the accuracy can be usually fairly good, but it may not be that good if a storm, especially a tropical cyclone, is nearby. For the latter case meteorologists may run multiple models (typically in super computers) and observes that the predicted trajectories of this storm are different one another, and as one result, the predicted weather of a city are totally different.

Here in RAN AI the case can be similar. A smart AI may predict the load or energy saving metrics quite well in usual case, but if the case is not usual (e.g. holiday, demonstration) there can be much uncertainty.

So in principle, it should be supported that the prediction is provided with an accuracy IE.

Proposal 2: Every quantised prediction can be accompanied with an accuracy IE.

- Conclusion

Proposal 1: Assume that the time to report prediction is t0 and the periodicity is T, Either of the following two bullets should be supported:
-	In addition to the one for t0 till t0+T, the one for t0+T till t0+2T or even further is also provided;
-	The validity duration is longer than the period, e.g. the prediction is for t0 till t0+2T.

Proposal 2: Every quantised prediction can be accompanied with an accuracy IE.

Based on the proposal, we draft 2 Stage 3 TPs [1][2].

- Reference

[1] R3-230251; (TP for 38.423) Support of load balance and energy saving; CATT.

[2] R3-230255; (BL CR for 38.420) Support of AI/ML for NG-RAN; CATT, CMCC.