3GPP TSG-RAN WG3 Meeting #119	R3-230261

Athens, GR, 27 Feb – 03 Mar, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0973 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Missing Criticality Diagnostics [Add_Crit_Diag] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, BT, Deutsche Telekom, Orange, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Criticality Diagnostics is used to inform the peer node that parts of a received message have not been comprehended or were missing, or if the message contained logical errors and indicate which IEs were not comprehended or were missing. Therefore for Class1 procedure, in RAN3 interfaces, we include the Criticality Diagnostics in the response messages and the failure messages. But in XnAP, the Criticality Diagnostics IE is missing in some response messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the Criticality Diagnostics IE in the following XnAP messages: - E-UTRA – NR CELL RESOURCE COORDINATION RESPONSE - IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE - IAB TRANSPORT MIGRATION MODIFICATION RESPONSE - IAB RESOURCE COORDINATION RESPONSE Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it only adds the missing Criticality Diagnostics as optional IE with criticality ignore in some response messages.The impact can be considered isolated because the change only adding the Criticality Diagnostics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Criticality Diagnostics IE is missing in many response messages, the receiving node is not able to inform the related errors to the sendng node. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.24, 9.1.4.3, 9.1.4.5, 9.1.4.7, 9.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.473 CR1112 TS 37.473 CR0019 TS 36.423 CR1732 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------Start of the First Change-----------

#### 9.1.2.24	E-UTRA – NR CELL RESOURCE COORDINATION RESPONSE

This message is sent by a neighbouring ng-eNB to a peer gNB or by a neighbouring gNB to a peer ng-eNB, both nodes able to interact, as a response to the E-UTRA – NR CELL RESOURCE COORDINATION REQUEST.

Direction: ng-eNB  gNB, gNB  ng-eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Responding NodeType | M |  |  |  | YES | reject |
| >ng-eNB |  |  |  |  |  |  |
| >>Data Traffic Resource Indication | M |  | 9.2.2.30 | Indicates resource allocations for data traffic. | – |  |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxnoofCellsinNG-RANnode) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | – |  |
| >>List of E-UTRA Cells in E-UTRA Coordination Response |  | 1.. < maxnoofCellsinNG-RANnode > |  | List of applicable E-UTRA cells | – |  |
| >>>EUTRA Cell ID | M |  | E-UTRA CGI 9.2.2.8 |  | – |  |
| >gNB |  |  |  |  |  |  |
| >>Data Traffic Resource Indication | M |  | 9.2.2.30 | Indicates resource allocations for data traffic. | – |  |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxnoofCellsinNG-RANnode) | Indicates the NR cells involved in resource coordination with the E-UTRA cells affiliated with the same Spectrum Sharing Group ID. | – |  |
| >>List of NR Cells in NR Coordination Response |  | 1.. < maxnoNRcellsSpectrumSharingwithE-UTRA > |  | List of applicable NR cells | – |  |
| >>>NR Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoNRcellsSpectrumSharingwithE-UTRA | Maximum no. of NR cells affiliated to a Spectrum Sharing Group ID involved in cell resource coordination with a number of E-UTRA cells affiliated with the same Spectrum Sharing Group ID. Value is 64. |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

-----------Start of the Next Change-----------

#### 9.1.4.3	IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE

This message is sent by the non-F1-terminating IAB-donor to the F1-terminating IAB-donor of a boundary IAB-node to provide inter-donor transport related configurations for the offloaded traffic.

Direction: non-F1-terminating IAB-donor  F1-terminating IAB-donor.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| F1-Terminating IAB-Donor UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | This IE refers to the Source NG-RAN node UE XnAP ID or to the M-NG-RAN node UE XnAP ID, or to the S-NG-RAN node UE XnAP ID. | YES | reject |
| Non-F1-Terminating IAB-Donor UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | This IE refers to the Target NG-RAN node UE XnAP ID or to the S-NG-RAN node UE XnAP ID, or to the M-NG-RAN node UE XnAP ID. | YES | reject |
| Traffic Added List |  | 0..1 |  |  | YES | reject |
| >Traffic Added Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.80 |  | – |  |
| >>Non-F1-terminating Topology BH Information | M |  | 9.2.2.83 |  | – |  |
| Traffic Modified List |  | 0..1 |  |  | YES | reject |
| >Traffic Modified Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.80 |  | – |  |
| >>Non-F1-terminating Topology BH Information | M |  | 9.2.2.83 |  | – |  |
| Traffic Not Added List |  | 0..1 |  |  | YES | reject |
| >Traffic Not Added Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.80 |  | – |  |
| >>Cause | O |  | 9.2.3.2 |  | – |  |
| Traffic Not Modified List |  | 0..1 |  |  | YES | reject |
| >Traffic Not Modified Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.80 |  | – |  |
| >>Cause | O |  | 9.2.3.2 |  | – |  |
| IAB TNL Address Response | O |  | 9.2.2.86 |  | YES | reject |
| Traffic Released List |  | 0..1 |  |  | YES | reject |
| >Traffic Released Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.80 |  | – |  |
| >>BH Info List | O |  | 9.2.2.99 |  | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTrafficIndexEntries | Maximum no. of traffic offloaded to the non-F1-terminating IAB-donor. The value is 1024. |

-----------Start of the Next Change-----------

#### 9.1.4.5	IAB TRANSPORT MIGRATION MODIFICATION RESPONSE

This message is sent by the F1-terminating IAB-donor to the non-F1-terminating IAB-donor of a boundary IAB-node to acknowledge the update of configuration requested by the non-F1-terminating IAB-donor.

Direction: F1-terminating IAB-donor  non-F1-terminating IAB-donor.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| F1-Terminating IAB-donor UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | This IE refers to the Source NG-RAN node UE XnAP ID or to the M-NG-RAN node UE XnAP ID, or to the S-NG-RAN node UE XnAP ID. | YES | reject |
| Non-F1-Terminating IAB-donor UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | This IE refers to the Target NG-RAN node UE XnAP ID or to the S-NG-RAN node UE XnAP ID, or to the M-NG-RAN node UE XnAP ID. | YES | reject |
| Traffic Required Modified List |  | 0..1 |  |  | YES | reject |
| >Traffic Required Modified Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.80 |  | – |  |
| Traffic Released List IE |  | 0..1 |  |  | YES | reject |
| >Traffic Released Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.80 |  | – |  |
| >>BH Info List | O |  | 9.2.2.99 |  | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTrafficIndexEntries | Maximum no. of traffic offloaded to the non-F1-terminating IAB-donor. The value is 1024. |

-----------Start of the Next Change-----------

#### 9.1.4.7	IAB RESOURCE COORDINATION RESPONSE

This message is sent by a non-F1-terminating/F1-terminating IAB-donor to an F1-terminating/non-F1-terminating IAB-donor of a boundary IAB-node, in response to an IAB RESOURCE COORDINATION REQUEST message.

Direction: non-F1-terminating IAB-donor  F1-terminating IAB-donor, F1-terminating IAB-donor  non-F1-terminating IAB-donor.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| F1-terminating IAB-Donor node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | This IE refers to the Source NG-RAN node UE XnAP ID or to the M-NG-RAN node UE XnAP ID or to the S-NG-RAN node UE XnAP ID. | YES | reject |
| Non F1-terminating IAB-Donor node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | This IE refers to the Target NG-RAN node UE XnAP ID or to the S-NG-RAN node UE XnAP ID or to the M-NG-RAN node UE XnAP ID. | YES | reject |
| Boundary Node Cells List |  | 0..1 |  | List of cells served by the boundary IAB-node IAB-DU. | YES | reject |
| >Boundary Node Cells List Item |  | 1 .. <maxnoofServedCellsIAB > |  |  | EACH | reject |
| >>Boundary Node cell Information | M |  | IAB Cell Information 9.2.2.94 |  | – |  |
| Parent-Node Cells List |  | 0..1 |  | List of cells served by the parent node IAB-DU. | YES | reject |
| >Parent-Node Cells List Item |  | 1 .. < maxnoofServingCells > |  |  | EACH | reject |
| >>Parent Node Cell Information | M |  | IAB Cell Information 9.2.2.94 |  | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedCellsIAB | Maximum number of cells served by an IAB-DU. Value is 512. |
| maxnoofServingCells | Maximum no. of serving cells for an IAB-MT. Value is 32, as defined by the maxNrofServingCells in TS 38.331 [10]. |

-----------Start of the Next Change-----------

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

AMF-UE-NGAP-ID,

AS-SecurityInformation,

//skip unchanged part

-- **************************************************************

--

#### -- E-UTRA NR CELL RESOURCE COORDINATION RESPONSE

--

-- **************************************************************

E-UTRA-NR-CellResourceCoordinationResponse::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{E-UTRA-NR-CellResourceCoordinationResponse-IEs}},

...

}

E-UTRA-NR-CellResourceCoordinationResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-respondingNodeType-ResourceCoordResponse  	CRITICALITY reject	TYPE RespondingNodeType-ResourceCoordResponse 	PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication					CRITICALITY reject	TYPE InterfaceInstanceIndication						PRESENCE optional }|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

RespondingNodeType-ResourceCoordResponse ::= CHOICE {

ng-eNB								ResourceCoordResponse-ng-eNB-initiated,

gNB									ResourceCoordResponse-gNB-initiated,

choice-extension					ProtocolIE-Single-Container { {RespondingNodeType-ResourceCoordResponse-ExtIEs} }

}

RespondingNodeType-ResourceCoordResponse-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ResourceCoordResponse-ng-eNB-initiated ::= SEQUENCE {

dataTrafficResourceIndication		DataTrafficResourceIndication,

spectrumSharingGroupID				SpectrumSharingGroupID,

listofE-UTRACells					SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI								OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ResourceCoordResponse-ng-eNB-initiated-ExtIEs} }		OPTIONAL,

...

}

ResourceCoordResponse-ng-eNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResourceCoordResponse-gNB-initiated ::= SEQUENCE {

dataTrafficResourceIndication		DataTrafficResourceIndication,

spectrumSharingGroupID				SpectrumSharingGroupID,

listofNRCells						SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF NR-CGI									OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ResourceCoordResponse-gNB-initiated-ExtIEs} }		OPTIONAL,

...

}

ResourceCoordResponse-gNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

//skip unchanged part

-- **************************************************************

--

#### -- IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE

--

-- **************************************************************

IABTransportMigrationManagementResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ IABTransportMigrationManagementResponse-IEs}},

...

}

IABTransportMigrationManagementResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-F1-Terminating-IAB-DonorUEXnAPID		CRITICALITY reject		TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-nonF1-Terminating-IAB-DonorUEXnAPID		CRITICALITY reject		TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-TrafficAddedList						CRITICALITY reject		TYPE TrafficAddedList							PRESENCE optional }|

{ ID id-TrafficModifiedList						CRITICALITY reject		TYPE TrafficModifiedList						PRESENCE optional }|

{ ID id-TrafficNotAddedList						CRITICALITY reject		TYPE TrafficNotAddedList						PRESENCE optional }|

{ ID id-TrafficNotModifiedList					CRITICALITY reject		TYPE TrafficNotModifiedList					PRESENCE optional }|

{ ID id-IAB-TNL-Address-Response				CRITICALITY reject		TYPE IAB-TNL-Address-Response				PRESENCE optional }|

{ ID id-TrafficReleasedList						CRITICALITY reject		TYPE TrafficReleasedList						PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics					PRESENCE optional },

...

}

TrafficAddedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficAdded-Item

TrafficAdded-Item ::= SEQUENCE {

trafficIndex			TrafficIndex,

non-F1-TerminatingTopologyBHInformation		Non-F1-TerminatingTopologyBHInformation,

iE-Extensions			ProtocolExtensionContainer { {TrafficAdded-Item-ExtIEs} }	OPTIONAL,

...

}

TrafficAdded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TrafficModifiedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficModified-Item

TrafficModified-Item ::= SEQUENCE {

trafficIndex			TrafficIndex,

non-F1-TerminatingTopologyBHInformation		Non-F1-TerminatingTopologyBHInformation,

iE-Extensions			ProtocolExtensionContainer { {TrafficModified-Item-ExtIEs} }	OPTIONAL,

...

}

TrafficModified-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TrafficNotAddedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficNotAdded-Item

TrafficNotAdded-Item ::= SEQUENCE {

trafficIndex			TrafficIndex,

casue					Cause			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {TrafficNotAdded-Item-ExtIEs} }	OPTIONAL,

...

}

TrafficNotAdded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TrafficNotModifiedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficNotModified-Item

TrafficNotModified-Item ::= SEQUENCE {

trafficIndex			TrafficIndex,

cause					Cause		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {TrafficNotModified-Item-ExtIEs} }	OPTIONAL,

...

}

TrafficNotModified-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TrafficReleasedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficReleased-Item

TrafficReleased-Item ::= SEQUENCE {

trafficIndex			TrafficIndex,

bHInfoList				BHInfoList		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { TrafficReleased-Item-ExtIEs} }	OPTIONAL,

...

}

TrafficReleased-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

//skip unchanged part

-- **************************************************************

--

#### -- IAB TRANSPORT MIGRATION MODIFICATION RESPONSE

--

-- **************************************************************

IABTransportMigrationModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ IABTransportMigrationModificationResponse-IEs}},

...

}

IABTransportMigrationModificationResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-F1-Terminating-IAB-DonorUEXnAPID			CRITICALITY reject	TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-nonF1-Terminating-IAB-DonorUEXnAPID			CRITICALITY reject	TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-TrafficRequiredModifiedList					CRITICALITY reject	TYPE TrafficRequiredModifiedList			PRESENCE optional }|

{ ID id-TrafficReleasedList							CRITICALITY reject	TYPE TrafficReleasedList						PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics					PRESENCE optional },

...

}

TrafficRequiredModifiedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficRequiredModified-Item

TrafficRequiredModified-Item ::= SEQUENCE {

trafficIndex			TrafficIndex,

iE-Extensions			ProtocolExtensionContainer { { TrafficRequiredModified-Item-ExtIEs} }	OPTIONAL,

...

}

TrafficRequiredModified-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

//skip unchanged part

-- **************************************************************

--

#### -- IAB RESOURCE COORDINATION RESPONSE

--

-- **************************************************************

IABResourceCoordinationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ IABResourceCoordinationResponse-IEs}},

...

}

IABResourceCoordinationResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-F1-Terminating-IAB-DonorUEXnAPID		CRITICALITY reject		TYPE NG-RANnodeUEXnAPID					PRESENCE mandatory}|

{ ID id-nonF1-Terminating-IAB-DonorUEXnAPID		CRITICALITY reject		TYPE NG-RANnodeUEXnAPID					PRESENCE mandatory}|

{ ID id-BoundaryNodeCellsList					CRITICALITY reject		TYPE BoundaryNodeCellsList				PRESENCE optional }|

{ ID id-ParentNodeCellsList						CRITICALITY reject		TYPE ParentNodeCellsList					PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics				PRESENCE optional },

...

}

-----------End of the Changes-----------