3GPP TSG-RAN WG3#119	R3-230266

Athens, Greece, 27 February – 03 March, 2023

Agenda item:		9.2.4

Title:		Further discussion on the Clarifications of prepared PSCell addition by candidate SN

Source:		NEC, ZTE, CATT, Ericsson, Huawei

Document for:		Discussion and Decision

# 1. Introduction

For the discussed issue in RAN3#118 [R3-226370], it has been confirmed that for SN initiated CPC, when the target (candidate) SN adds prepared PSCells within the limit given by the source SN, using SN initiated SN modification procedure, the MN may decide to forward to the source SN the request from the target SN.

The remaining issue to check is the way to capture in the specification.

PScell addition by candidate SN:

Two alternatives in R3-226800 and R3-226867 to be continued…

# 2. Discussion

For the SN initiated CPC, when the target (candidate) SN adds some more prepared PSCells within the limit given by the source SN, using SN initiated SN modification procedure, it has been confirmed in RAN3#118 that the MN may decide to forward to the source SN the request from the target SN.

Then for the wording to be captured in the specification, during RAN3#118 in Toulouse, we discussed two options (option A and option B).

Option A is to add into the chapter 10.3 Secondary Node Modification (MN/SN initiated), a NOTE to show the case when the candidate SN includes some prepared PSCells with short description.

Option B is to add into the chapter 10.5 Secondary Node Change (MN/SN initiated), a NOTE to show the case when the candidate SN includes some prepared PSCells, with also some description of the steps to execute.

Further checking later after the meeting, there is another alternative (to call Option C) which is to add in chapter 10.5 in the figure some new optional steps with its description.

(It was also found later that not only the candidate SN to add some more prepared PSCells, but also the candidate SN may want to remove some prepared PScells, this case is also considered with using the same procedure)

See corresponding draft CR in:

- Description option A: This is shown in R3-230267

- Description option B: This is shown in R3-230268

- Description option C: This is shown in R3-230269

Some comparison of all options:

- Option C is the clearest one which ease readers to understand. While adding optional steps to the figure with detailed descriptions could lead to the inclusion of more related functions or features in the future, given that the function is already complete, the likelihood of such risks may be relatively low.

- Option B is to put in a NOTE the description includes all related steps in one paragraph, with no adding optional steps in the figure. On the other hand, it may need to careful read since the description does not exactly correspond to the steps in the figure.

- Option A is to put a short description.

=>but in any case, any option is workable, we just need to choose either Option A or Option B or Option C.

Proposal: For the SN initiated CPC, when the target (candidate) SN adds some more prepared PSCells or removes some prepared PSCells, the way to describe in the stage 2 spec, to select either the way in Option A or Option B or Option C.

# 3. Conclusion

In this contribution we discussed for the SN initiated CPC when the candidate SN adds some more prepared PSCells or to remove some prepared PSCells, the way(some options) to capture the wording in the stage 2 specification.

The following proposals is provided.

Proposal: For the SN initiated CPC, when the target (candidate) SN adds some more prepared PSCells or removes some prepared PSCells, the way to describe in the stage 2 spec, to select either way in Option A or Option B or Option C.

# References

[1] R3-222754==R2-2204493, “Reply LS on CPAC”, RAN3 to RAN2

[2] TS37.340 v17.2.0

[3] R2-2207494 “Clarifications on prepared PSCell addition by candidate SN in CPC”, NEC

[4] R2-2208713 “Report of [AT119-e][220][DCCA] Stage-2 corrections to DCCA”

[5] R3-226370 Clarifications on prepared PSCell addition by candidate SN, NEC/ZTE

ANNEX: quoted from TS 37.340

Figure 10.3.1-2: SN Modification procedure - SN initiated with MN involvement

Figure 10.5.1-4: Conditional SN Change – SN initiated