3GPP TSG-RAN WG3#119	　　　　　　　　　　　　　　　　　　　　R3-230272

Athens, Greece, 27 February – 03 March, 2023

Agenda Item:  22.2

Source:       NEC, KDDI Corporation, Rakuten Mobile, Inc

Title:		(TP to TS38.413 on NCR) Support of Network-Control Repeater (NCR)

Document for:	decision

Introduction

Last RAN3#118 meeting has discussed and progressed the critical issue of the NR network-controlled repeaters (NCR). Following the progress, this contribution intends to further discuss more detail aspect and give TP to progress the work.

22.2. Support Network-Controlled Repeater Management Specify the solution of network-controlled repeater management (i.e., the identification and authorization/validation of NCR) [RAN3, RAN2] - NOTE: Down-selection of solutions in section 8 of TR 38.867 is needed taking into account the feedback of other working groups (i.e., SA3 and SA5). From a security point of view, the feasibility of NCR validation procedure in solution 1 and the feasibility of solution 2 will be decided by SA3.The selected solution shall provide inter-vendor interoperability. The NCR authorization indicator is provided from AMF to gNB explicitly over the NG interface. The discussion on RAN impact on validation function is pending to SA3 reply LS. gNB-CU knows whether the connected gNB-DU supports NCR based on OAM configuration. Down selection on all solutions which takes the feedback from SA3 and SA5 into account can be discussed in next RAN3 meeting. The NCR-OAM connectivity requirement should be supported, further details can be discussed. RAN3#118: Exclude the solution 2. OAM-NCR connectivity can be provided via PDU session. gNB-DU needs to know the authorization status of NCR. Take Solution 3 as the basis for NCR management. FFS on any additional aspects. The NCR may be configured with a list of allowed and/or forbidden cells. Discussion on further stage 2 related aspects agreed by RAN1. Whether the needs for gNB-CU or gNB-DU to configure which cell(s) can be used for NCR device accessing. Work on stage2 and stage3 CRs

Discussion

NCR management solution

Last RAN3#118 meeting has agreed “Take Solution 3 as the basis for NCR management.”. The solution 3 as described in TR 38.867 Figure 8.1.3-1, is having the NCR support indicator from the AMF to the gNB in NG Setup procedure, and also an NCR indication info received from the NCR-MT then transfer from the gNB to the AMF in the Initial UE Message, then the AMF gives the NCR authorized indicator in the UE Context Setup and UE Context Modification procedure.

Quoted from TR38.867 ### 8.1.3 Solution 3 In this solution, NCR identification is done at RAN side, and NCR authorization is done at CN side, similar to the handling of IAB-MT. The general procedure of the solution is illustrated in below figure: Figure 8.1.3-1. Call flow for solution 3 Sequence of this solution: 1. During NG-C setup procedure, the AMF should inform the gNB whether it supports NCR, e.g. by including "NCR-supported" indicator in NG SETUP RESPONSE message. 2. NCR establishes RRC connection and NCR identification can be implemented by reporting an NCR indicator in Msg5 (in addition to sending any NCR-related radio capability) and/or by reporting an NCR indicator (implicitly or explicitly) in UE's radio capability signaling. The gNB selects an AMF which supports NCR function, and forwards the NCR indicator to the AMF. 3. AMF and other CN entities do further authorization, and provides "NCR authorized" to the gNB.

The baseline CR [R3-226825] has included the Network Controlled Repeater Authorized IE in the UE CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST message, the rest that base on solution 3 are not yet included.

Proposal 1: include a NCR device Support indicator from the AMF to the gNB in NG SETUP RESPONSE message.

Proposal 2: include a NCR device Indication in the INITIAL UE MESSAGE.

NCR management impact

For the issue of “The NCR may be configured with a list of allowed and/or forbidden cells.”, if we will agree to have such, e.g. to allow the NCR-MT to access the intended cells, or to avoid the NCR-MT to access not-intended cells e.g. due to temporary strong signal from not-intended cells, still this can be configured by OAM, like LTE RN, to have an OAM connection with NCR-MT provided by the NCR-MT PDU session.

Proposal 3: If the NCR is configured with a list of allowed and/or forbidden cells and its use case is to allow to access intended-cells or to avoid to access non-intended cells due to temporary strong signal from not intended cells, this is configured by OAM by using OAM connection with NCR-MT provided by the NCR-MT PDU session.

For the issue of “Whether the needs for gNB-CU or gNB-DU to configure which cell(s) can be used for NCR device accessing”, if this will be related with the RAN2 agreement on the broadcasting of NCR support indicator in system information, then from RAN3 perspective we need to decide which node (either gNB-CU or gNB-DU) to generate such system information. Then follow the existing way, this system information is informed over F1AP in node management procedure (F1 Setup procedure, gNB-DU/gNB-CU Configuration Update procedure).  Cell configuration is normally done by OAM in gNB-DU, the system information broadcast for NCR support indicator is then to be configured by OAM in gNB-DU. If there will be any impact on the specification, it is likely then either no new impact on F1AP or only small update of wording to be added in F1AP node management procedures.

Proposal 4: It is the gNB-DU to configure the cell whether cell(s) can be used for NCR device accessing and with broadcasting of NCR support indicator in the system information, this system information is exchanged over F1AP from gNB-DU to gNB-CU.

A related issue is, if to further consider whether during the NCR operation, the gNB-DU or gNB-CU will need to disable temporary NCR access due to temporary reason e.g. cell overloading. This may be not needed if only cell overloading is the reason, as access restriction can be done by restricting of normal end-UE access instead of disable the whole NCR. However, explicit indication to show whether any cell can be used for NCR device accessing, can be considered if its use case is confirmed.

Proposal 5: discuss whether any use case to temporary disable the cell(s) used for NCR access and whether explicitly indicator from gNB-CU/gNB-DU to gNB-DU/gNB-CU is needed or not.

Proposals

Proposal 1: include a NCR device Support indicator from the AMF to the gNB in NG SETUP RESPONSE message.

Proposal 2: include a NCR device Indication in the INITIAL UE MESSAGE.

Proposal 3: If the NCR is configured with a list of allowed and/or forbidden cells and its use case is to allow to access intended-cells or to avoid to access non-intended cells due to temporary strong signal from not intended cells, this is configured by OAM by using OAM connection with NCR-MT provided by the NCR-MT PDU session.

Proposal 4: It is the gNB-DU to configure the cell whether cell(s) can be used for NCR device accessing and with broadcasting of NCR support indicator in the system information, this system information is exchanged over F1AP from gNB-DU to gNB-CU.

Proposal 5: discuss whether any use case to temporary disable the cell(s) used for NCR access and whether explicitly indicator from gNB-CU/gNB-DU to gNB-DU/gNB-CU is needed or not.

Proposal 6: propose to agree TP to 38.413.

References

[1] TR38.867

[2] R3-226923 Summary of Offline Discussion on CB: # 26_NCR

[3] R3-226926 (BL CR) 38300 BL CR for Network Controlled Repeater management

[4] R3-226825 (BL CR 38.413) Network-Controlled Repeaters Authorization

TP to 38.413 for NCR

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

ACL	Access Control List

AMF	Access and Mobility Management Function

CAG	Closed Access Group

CGI	Cell Global Identifier

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DC	Dual Connectivity

DL	Downlink

EPC	Evolved Packet Core

FN-RG	Fixed Network Residential Gateway

GUAMI	Globally Unique AMF Identifier

HFC	Hybrid Fiber-Coax

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

MBS	Multicast/Broadcast Service

N3IWF	Non 3GPP InterWorking Function

NB-IoT	Narrow Band Internet of Things

NCR	Network-Controlled Repeater

NID	Network Identifier

NGAP	NG Application Protocol

NPN	Non-Public Network

NRPPa	NR Positioning Protocol Annex

NSAG	Network Slice AS Group

NSCI	New Security Context Indicator

NSSAI	Network Slice Selection Assistance Information

OTDOA	Observed Time Difference of Arrival

PEIPS	Paging Early Indication with Paging Subgrouping

PNI-NPN	Public Network Integrated Non-Public Network

ProSe	Proximity Services

PSCell	Primary SCG Cell

PTP	Point to Point

PTM	Point to Multipoint

QMC	QoE Measurement Collection

QoE	Quality of Experience

RedCap	Reduced Capability

RIM	Remote Interference Management

RIM-RS	RIM Reference Signal

RSN	Redundancy Sequence Number

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SgNB	Secondary gNB

SMF	Session Management Function

S-NG-RAN node	Secondary NG-RAN node

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

TAC	Tracking Area Code

TAI	Tracking Area Identity

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNLA	Transport Network Layer Association

TWAP	Trusted WLAN Access Point

TWIF	Trusted WLAN Interworking Function

UL	Uplink

UP	User Plane

UPF	User Plane Function

V2X	Vehicle-to-Everything

W-AGF	Wireline Access Gateway Function

WUS	Wake Up Signal

Skip unchanged part

### 8.6.1	Initial UE Message

#### 8.6.1.1	General

The Initial UE Message procedure is used when the NG-RAN node has received from the radio interface the first uplink NAS message to be forwarded to an AMF.

#### 8.6.1.2	Successful Operation

Figure 8.6.1.2-1: Initial UE message

The NG-RAN node initiates the procedure by sending an INITIAL UE MESSAGE message to the AMF. The NG-RAN node shall allocate a unique RAN UE NGAP ID to be used for the UE and the NG-RAN node shall include this identity in the INITIAL UE MESSAGE message.

The NAS-PDU IE contains a UE – AMF message that is transferred without interpretation in the NG-RAN node.

In case of network sharing, the selected PLMN is indicated by the PLMN Identity IE within the TAI IE included in the INITIAL UE MESSAGE message.

When the NG-RAN node has received from the radio interface the 5G-S-TMSI IE, it shall include it in the INITIAL UE MESSAGE message.

If the AMF Set ID IE is included in the INITIAL UE MESSAGE message this indicates that the message is a rerouted message and the AMF shall, if supported, use the IE as described in TS 23.502 [10].

If the UE Context Request IE is included in the INITIAL UE MESSAGE message the AMF shall trigger an Initial Context Setup procedure towards the NG-RAN node.

If the Allowed NSSAI IE is included in the INITIAL UE MESSAGE message the AMF shall use the IE as defined in TS 23.502 [10].

If the Source to Target AMF Information Reroute IE is included in the INITIAL UE MESSAGE message the AMF shall use the IE as defined in TS 23.502 [10].

If the IAB Node Indication IE is included in the INITIAL UE MESSAGE message, the AMF shall consider that the message is related to an IAB node.

If the CE-mode-B Support Indicator IE is included in the INITIAL UE MESSAGE message and set to "supported", the AMF shall, if supported, use the extended NAS timer settings for the UE as specified in TS 23.501 [9].

If the LTE-M indication IE is included in the INITIAL UE MESSAGE message the AMF shall, if supported, use it according to TS 23.501 [9].

If the EDT Session IE set to "true" is included in the INITIAL UE MESSAGE message and the NG-RAN node is an ng-eNB, the AMF shall, if supported, consider that the message has been received as a result of an EDT session initiated by the UE.

If PNI-NPN related information within the NPN Access Information IE is received in the INITIAL UE MESSAGE message, the AMF shall, if supported, consider that the included information is associated to the cell via which the UE has sent the first NAS message, and to the PLMN Identity which is indicated within the TAI IE, and use the included information as specified in TS 23.501 [9].

In case of network sharing for SNPNs, the selected SNPN is indicated within the User Location Information IE included in the INITIAL UE MESSAGE message by the PLMN Identity IE within the TAI IE and the NID IE.

If the RedCap Indication IE is included in the INITIAL UE MESSAGE message, the AMF shall, if supported, use it according to TS 23.501 [9].

If the NCR device Indication IE is included in the INITIAL UE MESSAGE message, the AMF shall consider that the message is related to an NCR device.

#### 8.6.1.3	Abnormal Conditions

If the 5G-S-TMSI is not received by the AMF in the INITIAL UE MESSAGE message whereas expected, the AMF shall consider the procedure as failed.

Skip unchanged part

### 8.7.1	NG Setup

#### 8.7.1.1	General

The purpose of the NG Setup procedure is to exchange application level data needed for the NG-RAN node and the AMF to correctly interoperate on the NG-C interface. This procedure shall be the first NGAP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes, replaces it by the one received and clears AMF overload state information at the NG-RAN node. If the NG-RAN node and AMF do not agree on retaining the UE contexts this procedure also re-initialises the NGAP UE-related contexts (if any) and erases all related signalling connections in the two nodes like an NG Reset procedure would do.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation

The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF. The AMF responds with an NG SETUP RESPONSE message including the appropriate data.

If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the NG SETUP REQUEST message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

If the UE Retention Information IE set to “ues-retained“ is included in the NG SETUP REQUEST message, the AMF may accept the proposal to retain the existing UE related contexts and signalling connections by including the UE Retention Information IE set to “ues-retained“ in the NG SETUP RESPONSE message.

If the AMF supports IAB, the AMF shall include the IAB Supported IE in the NG SETUP RESPONSE message.

The AMF shall include the Backup AMF Name IE, if available, in the Served GUAMI List IE in the NG SETUP RESPONSE message. The NG-RAN node shall, if supported, consider the AMF as indicated by the Backup AMF Name IE when performing AMF reselection, as specified in TS 23.501 [9].

If the GUAMI Type IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall store the received value and use it for further AMF selection as defined in TS 23.501 [9].

If the RAN Node Name IE is included in the NG SETUP REQUEST message, the AMF may use this IE as a human readable name of the NG-RAN node. If the Extended RAN Node Name IE is included in the NG SETUP REQUEST message, the AMF may use this IE as a human readable name of the NG-RAN node and shall ignore the RAN Node Name IE if also included.

If the AMF Name IE is included in the NG SETUP RESPONSE message, the NG-RAN node may use this IE as a human readable name of the AMF. If the Extended AMF Name IE is included in the NG SETUP RESPONSE message, the NG-RAN node may use this IE as a human readable name of the AMF and shall ignore the AMF Name IE if also included.

If the NB-IoT Default Paging DRX IE is included in the NG SETUP REQUEST message, the AMF shall take it into account for paging.

If the RAT Information IE is included in the NG SETUP REQUEST message, the AMF shall handle this information as specified in TS 23.502 [10].

If the NID IE within the NPN Support IE is included within a Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall consider that the NG-RAN node supports the indicated S-NSSAI(s) for the corresponding tracking area code for the SNPN identified by the PLMN Identity IE and the NID IE.

If the NID IE within the NPN Support IE is included within a PLMN Support Item IE in the NG SETUP RESPONSE message, the NG-RAN node shall consider that the AMF supports the SNPN identified by the PLMN Identity IE and the NID IE.

If the Onboarding Support IE is also included within the same PLMN Support Item IE, the NG-RAN node shall, if supported, consider that the AMF supports UE onboarding for the identified SNPN, as specified in TS 23.501 [9].

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the AMF supports NCR device, the AMF shall include the NCR device Supported IE in the NG SETUP RESPONSE message.

Skip unchanged part

### 9.2.5	NAS Transport Messages

#### 9.2.5.1	INITIAL UE MESSAGE

This message is sent by the NG-RAN node to transfer the initial layer 3 message to the AMF over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| NAS-PDU | M |  | 9.3.3.4 |  | YES | reject |
| User Location Information | M |  | 9.3.1.16 |  | YES | reject |
| RRC Establishment Cause | M |  | 9.3.1.111 |  | YES | ignore |
| 5G-S-TMSI | O |  | 9.3.3.20 |  | YES | reject |
| AMF Set ID | O |  | 9.3.3.12 |  | YES | ignore |
| UE Context Request | O |  | ENUMERATED (requested, ...) |  | YES | ignore |
| Allowed NSSAI | O |  | 9.3.1.31 |  | YES | reject |
| Source to Target AMF Information Reroute | O |  | 9.3.3.27 |  | YES | ignore |
| Selected PLMN Identity | O |  | PLMN Identity 9.3.3.5 | Indicates the selected PLMN id for the non-3GPP access. | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) | Indication of an IAB node | YES | reject |
| CE-mode-B Support Indicator | O |  | 9.3.1.156 |  | YES | reject |
| LTE-M Indication | O |  | 9.3.1.157 |  | YES | ignore |
| EDT Session | O |  | ENUMERATED (true, …) |  | YES | ignore |
| Authenticated Indication | O |  | ENUMERATED (true, …) | Indicates the FN-RG has been authenticated by the access network. | YES | ignore |
| NPN Access Information | O |  | 9.3.3.46 |  | YES | reject |
| RedCap Indication | O |  | 9.3.1.228 |  | YES | ignore |
| NCR device Indication | O |  | ENUMERATED (true, ...) | Indication of an NCR device | YES | reject |

Skip unchanged part

#### 9.2.6.2	NG SETUP RESPONSE

This message is sent by the AMF to transfer application layer information for an NG-C interface instance.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| Served GUAMI List |  | 1 |  |  | YES | reject |
| >Served GUAMI Item |  | 1..<maxnoofServedGUAMIs> |  |  | - |  |
| >>GUAMI | M |  | 9.3.3.3 |  | - |  |
| >>Backup AMF Name | O |  | AMF Name 9.3.3.21 |  | - |  |
| >>GUAMI Type | O |  | ENUMERATED (native, mapped, …) |  | YES | ignore |
| Relative AMF Capacity | M |  | 9.3.1.32 |  | YES | ignore |
| PLMN Support List |  | 1 |  |  | YES | reject |
| >PLMN Support Item |  | 1..<maxnoofPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >>Slice Support List | M |  | 9.3.1.17 | Supported S-NSSAIs per PLMN or per SNPN. | - |  |
| >>NPN Support | O |  | 9.3.3.44 | If NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>Extended Slice Support List | O |  | 9.3.1.191 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >>Onboarding Support | O |  | ENUMERATED (true, ...) | Indication of onboarding support. | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| IAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for IAB. | YES | ignore |
| Extended AMF Name | O |  | 9.3.3.51 |  | YES | ignore |
| NCR device Supported | O |  | ENUMERATED (true, ...) | Indication of support for NCR device. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedGUAMIs | Maximum no. of GUAMIs served by an AMF. Value is 256. |
| maxnoofPLMNs | Maximum no. of PLMNs per message. Value is 12. |