3GPP TSG-RAN WG3 Meeting #119	R3-230314

Athens, GR, 27 Feb – 03 Mar, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0977 | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of SFN offset in served cell information E-UTRA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Deutsche Telekom, Orange, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2023-02-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For E-UTRA cell, the SFN Offset IE was introduced in Served Cell Information E-UTRA IE in RP-210124, in order for the receiving NG-RAN node to deduce the SFN0 time offset of the reported cell for measurement configuration. However, in ASN.1 this IE was erroneously included in the ServedCells-E-UTRA-Item, which should be included in the ServedCellInformation-E-UTRA instead. This also leads to the misalignment between the ASN.1 and Tabular. This CR is NBC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In ASN.1, to remove the SFN offset from the ServedCells-E-UTRA-Item and includ it under ServedCellInformation-E-UTRA. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only affects the SFN offset for E-UTRA cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between the tabular and ASN.1. The SFN offset for E-UTRA cell can not be provided correctly for each cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

<Below is provided just for reference>

#### 9.2.2.12	Served Cell Information E-UTRA

This IE contains cell configuration information of an E-UTRA cell that a neighbour NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA PCI | M |  | INTEGER (0..503, …) | E-UTRA Physical Cell ID | – |  |
| ECGI | M |  | E-UTRA CGI 9.2.2.8 |  | – |  |
| TAC | M |  | 9.2.2.5 | Tracking Area Code | – |  |
| RANAC | O |  | RAN Area Code 9.2.2.6 |  | – |  |
| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB1 associated to the E-UTRA Cell Identity in the ECGI IE. NOTE: In this version of the specification, it is possible to broadcast only up to 6 PLMN IDs. | – |  |
| >PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| CHOICE E-UTRA-Mode-Info | M |  |  |  | – |  |
| >FDD |  |  |  |  | – |  |
| >>FDD Info |  | 1 |  |  | – |  |
| >>>UL EARFCN | M |  | E-UTRA ARFCN 9.2.2.21 | Corresponds to NUL in TS 36.104 [25] for E-UTRA operating bands for which it is defined; ignored for E-UTRA operating bands for which NUL is not defined | – |  |
| >>>DL EARFCN | M |  | E-UTRA ARFCN 9.2.2.21 | Corresponds to NDL in TS 36.104 [25] | – |  |
| >>>UL E-UTRA Transmission Bandwidth | M |  | E-UTRA Transmission Bandwidth 9.2.2.22 | Same as DL Transmission Bandwidth in this release; ignored in case UL EARFCN value is ignored | – |  |
| >>>DL E-UTRA Transmission Bandwidth | M |  | E-UTRA Transmission Bandwidth 9.2.2.22 |  | – |  |
| >>>Offset of NB-IoT Channel Number to DL EARFCN | O |  | Offset of NB-IoT Channel Number to EARFCN 9.2.2.47 | Corresponds to MDL in TS 36.104 [25] | YES | reject |
| >>>Offset of NB-IoT Channel Number to UL EARFCN | O |  | Offset of NB-IoT Channel Number to EARFCN 9.2.2.47 | Corresponds to MUL in TS 36.104 [25] | YES | reject |
| >TDD |  |  |  |  | – |  |
| >>TDD Info |  | 1 |  |  | – |  |
| >>>EARFCN | M |  | E-UTRA ARFCN 9.2.2.21 | Corresponds to NDL/NUL in TS 36.104 [25] | – |  |
| >>>E-UTRA Transmission Bandwidth | M |  | 9.2.2.22 |  | – |  |
| >>>Subframe Assignment | M |  | ENUMERATED (sa0, sa1, sa2, sa3, sa4, sa5, sa6, ...) | Uplink-downlink subframe configuration information defined in TS 36.211 [26] | – |  |
| >>>Special Subframe Info |  | 1 |  | Special subframe configuration information defined in TS 36.211 [26] | – |  |
| >>>>Special Subframe Patterns | M |  | ENUMERATED (ssp0, ssp1, ssp2, ssp3, ssp4, ssp5, ssp6, ssp7, ssp8, ssp9, ssp10, ...) |  | – |  |
| >>>>Cyclic Prefix DL | M |  | ENUMERATED (Normal, Extended,…) |  | – |  |
| >>>>Cyclic Prefix UL | M |  | ENUMERATED (Normal, Extended, ...) |  | – |  |
| >>>Offset of NB-IoT Channel Number to DL EARFCN | O |  | Offset of NB-IoT Channel Number to EARFCN 9.2.2.47 | Corresponds to MDL in TS 36.104 [25] | YES | reject |
| >>>NB-IoT UL DL Alignment Offset | O |  | 9.2.2.48 | Corresponds to the TDD-UL-DL-AlignmentOffset-NB in TS 36.331 [14]. | YES | reject |
| Number of Antenna Ports E-UTRA | O |  | 9.2.2.23 |  | – |  |
| PRACH Configuration | O |  | E-UTRA PRACH Configuration 9.2.2.25 |  | – |  |
| MBSFN Subframe Info |  | 0..<maxnoofMBSFN> |  | MBSFN subframe defined in TS 36.331 [14] | – |  |
| >Radioframe Allocation Period | M |  | ENUMERATED (n1, n2, n4, n8, n16, n32, …) |  | – |  |
| >Radioframe Allocation Offset | M |  | INTEGER (0..7, ...) |  | – |  |
| >MBSFN Subframe Allocation E-UTRA | M |  | 9.2.2.26 |  | – |  |
| E-UTRA Multiband Info List | O |  | 9.2.2.24 |  | – |  |
| FreqBandIndicatorPriority | O |  | ENUMERATED (not-broadcast, broadcast, ...) | This IE indicates that the eNodeB supports FreqBandIndicationPriority, and whether FreqBandIndicatorPriority is broadcast in SIB 1 (see TS 36.331 [14]) | – |  |
| BandwidthReducedSI | O |  | ENUMERATED (scheduled, ...) | This IE indicates that the SystemInformationBlockType1-BR is scheduled in the cell (see TS 36.331 [14]) | – |  |
| Protected E-UTRA Resource Indication | O |  | 9.2.2.29 | This IE indicates which E-UTRA control/reference signal resources are protected and are not subject to E-UTRA - NR Cell Resource Coordination. | – |  |
| Broadcast PLMN Identity Info List E-UTRA |  | 0..<maxnoofEUTRABPLMNs> |  | This IE corresponds to the cellAccessRelatedInfoList-5GC IE in SIB1 as specified in TS 36.331 [14]. All PLMN Identities and associated information contained in the cellAccessRelatedInfoList-5GC IE are included and provided in the same order as broadcast in SIB1. | YES | ignore |
| >Broadcast PLMNs |  | 1..<maxnoofEUTRABPLMNs> |  | Broadcast PLMNs in SIB1 associated to the E-UTRA Cell Identity IE. | – |  |
| >>PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| >TAC | M |  | 9.2.2.5 |  | – |  |
| >E-UTRA Cell Identity | M |  | BIT STRING (SIZE(28)) |  | – |  |
| >RANAC | O |  | RAN Area Code 9.2.2.6 |  | – |  |
| NPRACH Configuration | O |  | NPRACH Configuration 9.2.2.74 |  | YES | ignore |
| SFN Offset | O |  | 9.2.2.75 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. The value is 12. |
| maxnoofMBSFN | Maximum no. of MBSFN frame allocation with different offset. Value is 8. |
| maxnoofEUTRABPLMNs | Maximum no. of PLMN Ids.broadcast in an E-UTRA cell. Value is 6. |

<Unchanged Text Omitted>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<Unchanged Text Omitted>

##### -- Served Cells E-UTRA IEs

ServedCellInformation-E-UTRA ::= SEQUENCE {

e-utra-pci								E-UTRAPCI,

e-utra-cgi								E-UTRA-CGI,

tac										TAC,

ranac									RANAC																					OPTIONAL,

broadcastPLMNs							SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF ServedCellInformation-E-UTRA-perBPLMN,

e-utra-mode-info						ServedCellInformation-E-UTRA-ModeInfo,

numberofAntennaPorts					NumberOfAntennaPorts-E-UTRA																OPTIONAL,

prach-configuration						E-UTRAPRACHConfiguration																OPTIONAL,

mBSFNsubframeInfo						MBSFNSubframeInfo-E-UTRA																OPTIONAL,

multibandInfo							E-UTRAMultibandInfoList																	OPTIONAL,

freqBandIndicatorPriority				ENUMERATED {not-broadcast, broadcast, ...} 												OPTIONAL,

bandwidthReducedSI						ENUMERATED {scheduled, ...}																OPTIONAL,

protectedE-UTRAResourceIndication		ProtectedE-UTRAResourceIndication														OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-ExtIEs} }					OPTIONAL,

...

}

ServedCellInformation-E-UTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-BPLMN-ID-Info-EUTRA		CRITICALITY ignore	EXTENSION BPLMN-ID-Info-EUTRA		PRESENCE optional }|

{ ID id-NPRACHConfiguration		CRITICALITY ignore	EXTENSION	NPRACHConfiguration		PRESENCE optional}|

{ ID id-SFN-Offset				CRITICALITY ignore EXTENSION SFN-Offset					PRESENCE optional },

...

}

ServedCellInformation-E-UTRA-perBPLMN ::= SEQUENCE {

plmn-id					PLMN-Identity,

iE-Extensions			ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-perBPLMN-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-E-UTRA-perBPLMN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCellInformation-E-UTRA-ModeInfo ::= CHOICE {

fdd					ServedCellInformation-E-UTRA-FDDInfo,

tdd					ServedCellInformation-E-UTRA-TDDInfo,

choice-extension	ProtocolIE-Single-Container{ {ServedCellInformation-E-UTRA-ModeInfo-ExtIEs} }

}

ServedCellInformation-E-UTRA-ModeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ServedCellInformation-E-UTRA-FDDInfo ::= SEQUENCE {

ul-earfcn			E-UTRAARFCN,

dl-earfcn			E-UTRAARFCN,

ul-e-utraTxBW		E-UTRATransmissionBandwidth,

dl-e-utraTxBW		E-UTRATransmissionBandwidth,

iE-Extensions		ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-FDDInfo-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-E-UTRA-FDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-OffsetOfNbiotChannelNumberToDL-EARFCN	CRITICALITY reject	EXTENSION OffsetOfNbiotChannelNumberToEARFCN		PRESENCE optional}|

{ ID id-OffsetOfNbiotChannelNumberToUL-EARFCN	CRITICALITY reject	EXTENSION OffsetOfNbiotChannelNumberToEARFCN		PRESENCE optional},

...

}

ServedCellInformation-E-UTRA-TDDInfo ::= SEQUENCE {

earfcn					E-UTRAARFCN,

e-utraTxBW				E-UTRATransmissionBandwidth,

subframeAssignmnet		ENUMERATED {sa0,sa1,sa2,sa3,sa4,sa5,sa6,...},

specialSubframeInfo		SpecialSubframeInfo-E-UTRA,

iE-Extensions			ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-TDDInfo-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-E-UTRA-TDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-OffsetOfNbiotChannelNumberToDL-EARFCN	CRITICALITY reject	EXTENSION OffsetOfNbiotChannelNumberToEARFCN		PRESENCE optional}|

{ ID id-NBIoT-UL-DL-AlignmentOffset				CRITICALITY reject	EXTENSION NBIoT-UL-DL-AlignmentOffset					PRESENCE optional},

...

}

ServedCells-E-UTRA ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-E-UTRA-Item

ServedCells-E-UTRA-Item ::= SEQUENCE {

served-cell-info-E-UTRA		ServedCellInformation-E-UTRA,

neighbour-info-NR			NeighbourInformation-NR									OPTIONAL,

neighbour-info-E-UTRA		NeighbourInformation-E-UTRA								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServedCells-E-UTRA-Item-ExtIEs} } OPTIONAL,

...

}

ServedCells-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCellsToUpdate-E-UTRA ::= SEQUENCE {

served-Cells-ToAdd-E-UTRA		ServedCells-E-UTRA																OPTIONAL,

served-Cells-ToModify-E-UTRA	ServedCells-ToModify-E-UTRA														OPTIONAL,

served-Cells-ToDelete-E-UTRA	SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI 					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ServedCellsToUpdate-E-UTRA-ExtIEs} }	OPTIONAL,

...

}

ServedCellsToUpdate-E-UTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCells-ToModify-E-UTRA ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-ToModify-E-UTRA-Item

ServedCells-ToModify-E-UTRA-Item ::= SEQUENCE {

old-ECGI					E-UTRA-CGI,

served-cell-info-E-UTRA		ServedCellInformation-E-UTRA,

neighbour-info-NR			NeighbourInformation-NR											OPTIONAL,

neighbour-info-E-UTRA		NeighbourInformation-E-UTRA										OPTIONAL,

deactivation-indication		ENUMERATED {deactivated, ...}									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {Served-cells-ToModify-E-UTRA-Item-ExtIEs} }	OPTIONAL,

...

}

Served-cells-ToModify-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Change Ends