3GPP TSG-RAN WG3 Meeting #119	R3-230326

Athens, Greece, 27 February-3 March, 2023

Agenda item:	 10.2.2

Source:		Lenovo, CMCC

Title:		(TP for SON BLCR for 38.300) MRO for inter-system handover for voice fallback

Document for:		Discussion and Decision

1	Introduction

RAN3#117-e meeting discussed MRO enhancement for inter-system handover for voice fallback and achieved some agreements [1]:

Consider Case 1-2 for MRO enhancements for inter-system inter-RAT handover for voice fallback:

-	Case 1: after failure (HOF/RLF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell.

-	Case 2: after failure (HOF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, none suitable E-UTRAN cell can be selected, the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in NR.

WA: The RLF Report needs to indicate that the last failed inter-system inter-RAT HO was triggered due to voice fallback. FFS on whether an explicit or implicit method is needed or not.

Further agreements were achieved in RAN3#117bis-e meeting [2]:

Deprioritize Case 5 for MRO enhancements for inter-system inter-RAT handover for voice fallback:

-	Case 5: the UE successfully performs inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, but the handover is about to failure.

Deprioritize MRO enhancements for redirection for voice fallback.

Introduce stage 2 descriptions of failure type definition for inter-system inter-RAT HO from NR to E-UTRA for voice fallback. The detailed descriptions are FFS.

Turn the WA into an agreement: The RLF Report needs to indicate that the last failed inter-system inter-RAT HO was triggered due to voice fallback.

Left issues are listed as below:

FFS MRO for inter-system handover for voice fallback:

- whether to consider Case 4 for MRO for inter-system handover for voice fallback;

- details on failure type definition for inter-system inter-RAT HO from NR to E-UTRA for voice fallback in stage 2;

- network interface to deliver RLF report for inter-system inter-RAT HO from NR to E-UTRA for voice fallback.

In this paper, we would further discuss the left issues for MRO for inter-system handover for voice fallback.

2	Discussion

Until now, Case 1 and Case 2 are agreed for MRO consideration:

-	Case 1: after failure (HOF/RLF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, a suitable E-UTRAN cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRAN cell.

-	Case 2: after failure (HOF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, none suitable E-UTRAN cell can be selected, the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in NR.

Furthermore, for Case 1, it can be split into two cases:

-	Case 1a: a suitable E-UTRAN cell2 is selected after HOF of inter-system inter-RAT handover from source NR cell1 to target E-UTRAN cell1;

-	Case 1b: a suitable E-TURAN cell2 is selected, after a RLF occurs shortly after a successful handover from source NR cell1 to target E-UTRAN cell1.

From RAN3 point of view, we would further discuss how to report and deliver RLF report for inter-system inter-RAT HO from NR to E-UTRAN for voice fallback.

For Case 1a, there are two potential options:

- Option i: UE reports NR RLF report and source NR cell1 ID out of the NR RLF report to E-UTRAN cell2, E-UTRAN cell2 would not decode the NR RLF report, but based on the source NR cell1 ID, E-UTRAN cell2 would forward the NR RLF report to source NR cell1. In this option, the UE RLF Report Container is sent to the source NR cell1 over S1 and NG interfaces, for example, the existing ENB CONFIGURATION TRANSFER message can be reused to transfer the UE RLF Report Container from E-UTRAN cell2 to MME, MME shall transparently transfer the UE RLF Report Container towards the AMF serving the source NR cell1, then AMF shall transfer the UE RLF Report Container to the source NR cell1 via the existing DOWNLINK RAN CONFIGURATION TRANSFER message.

- Option ii: if UE is back to a NR cell3, UE reports NR RLF report to NR cell3, NR cell3 forwards the NR RLF report to source NR cell1. In this Option, the NR cell3 may transfer the UE RLF Report Container to the source NR cell1 directly via Xn interface, e.g. the existing FAILURE INDICATION message can be reused. Also, if there is no direct Xn interface, the NR cell3 may transfer the UE RLF Report Container to the source NR cell1 via NG interface, e.g. the existing UPLINK RAN CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message can be reused.

Proposal 1: RAN3 discuss RLF report delivery for the case that a suitable E-UTRAN cell is selected after HOF of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback:

- The UE RLF Report Container may be transferred from the re-connected E-UTRAN node to the source NG-RAN node via the existing ENB CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message.

- The UE RLF Report Container may be transferred from the connected NG-RAN node to the source NG-RAN node via the existing FAILURE INDICATION message, or UPLINK RAN CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message.

For Case 1b, UE may report LTE RLF report to E-UTRAN cell2, E-UTRAN cell2 forwards the LTE RLF report to target E-UTRAN cell1, target E-UTRAN cell1 decodes the LTE RLF report and explicitly transfers failure related information which is decoded from the LTE RLF report to source NR cell1. For example, the E-UTRA cell2 may transfer the UE RLF Report Container to the target E-UTRA cell1 via X2 interface e.g. the existing RLF INDICATION message can be reused, or via S1 interface e.g. the existing eNB CONFIGURATION TRANSFER message and MME CONFIGURATION TRANSFER message can be reused. Then, target E-UTRAN cell1 would forward the failure related informatin which is decoded from the UE RLF Report Container to the source NR cell1 over S1 and NG interfaces, e.g. the existing ENB CONFIGURATION TRANSFER message can be reused to transfer the failure related informatin which is decoded from the UE RLF Report Container from target E-UTRAN cell1 to MME, MME shall transparently transfer the failure related informatin which is decoded from the UE RLF Report Container towards the AMF serving the source NR cell1, then AMF shall transfer the failure related informatin which is decoded from the UE RLF Report Container to the source NR cell1 via the existing DOWNLINK RAN CONFIGURATION TRANSFER message.

Proposal 2: For the case that a suitable E-UTRAN cell is selected after a RLF occurs shortly after a successful inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, the UE RLF Report Container may be transferred from the re-connected E-UTRAN node to the target E-UTRAN node via the existing RLF INDICATION message, or eNB CONFIGURATION TRANSFER message and MME CONFIGURATION TRANSFER message. And, target E-UTRAN node may forward the failure related informatin which is decoded from the UE RLF Report Container to the source NG-RAN node over S1 and NG interfaces via the existing ENB CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message.

For Case 2: UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in a NR cell 2, after HOF of inter-system inter-RAT handover from source NR cell1 to target E-UTRAN cell1 and none suitable E-UTRA cell is selected. In the case, UE may report NR RLF report to NR cell 2, the NR cell2 may transfer the UE RLF Report Container to the source NR cell1 directly via Xn interface, e.g. the existing FAILURE INDICATION message can be reused. Also, if there is no direct Xn interface, the NR cell2 may transfer the UE RLF Report Container to the source NR cell1 via NG interface, e.g. the existing UPLINK RAN CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message can be reused.

Proposal 3: For the case that the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in a NR cell after HOF of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, the UE RLF Report Container may be transferred from the re-established NG-RAN node to the source NG-RAN node via the existing FAILURE INDICATION message, or UPLINK RAN CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message.

For both Case1 and Case2, based on the received indication on whether the failed inter-system inter-RAT handover was triggered for voice fallback within the UE RLF Report Container, the source NG-RAN node may distinguish failure case due to inter-system inter-RAT handover for voice fallback from legacy failure case in inter-system inter-RAT handover when performing MRO analysis, furthermore, source NG-RAN node may modify handover related parameters to enable the UE perform a successful inter-system inter-RAT handover for voice fallback.

Considering the existing Inter-system/ Too Late Handover or Inter-system/ Too Early Handover as defined in TS38.300 can’t cover the failure case of inter-system inter-RAT HO for voice fallback, RAN3#117bis-e meeting agree to introduce stage 2 descriptions of failure type definition for inter-system inter-RAT HO from NR to E-UTRAN for voice fallback, but the detailed descriptions are FFS. Since failure due to inter-system inter-RAT HO from NR to E-UTRAN for voice fallback is totally different from legacy Too Late Inter-system Handover or Too Early Inter-system Handover, it is better to have separate failure type definition rather than update on top of the legacy stage 2 definition for connection failure due to inter-system mobility.

Proposal 4: Introduce a separate failure type definition for inter-system inter-RAT HO from NR to E-UTRAN for voice fallback in stage 2.

Based on above discussion, the corresponding TP for TS38.300 is attached in the Annex.

3	Conclusion

In this contribution, we discussed MRO for inter-system inter-RAT handover for voice fallback, the following proposals are given:

Proposal 1: RAN3 discuss RLF report delivery for the case that a suitable E-UTRAN cell is selected after HOF of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback:

- The UE RLF Report Container may be transferred from the re-connected E-UTRAN node to the source NG-RAN node via the existing ENB CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message.

- The UE RLF Report Container may be transferred from the connected NG-RAN node to the source NG-RAN node via the existing FAILURE INDICATION message, or UPLINK RAN CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message.

Proposal 2: For the case that a suitable E-UTRAN cell is selected after a RLF occurs shortly after a successful inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, the UE RLF Report Container may be transferred from the re-connected E-UTRAN node to the target E-UTRAN node via the existing RLF INDICATION message, or eNB CONFIGURATION TRANSFER message and MME CONFIGURATION TRANSFER message. And, target E-UTRAN node may forward the failure related informatin which is decoded from the UE RLF Report Container to the source NG-RAN node over S1 and NG interfaces via the existing ENB CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message.

Proposal 3: For the case that the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in a NR cell after HOF of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, the UE RLF Report Container may be transferred from the re-established NG-RAN node to the source NG-RAN node via the existing FAILURE INDICATION message, or UPLINK RAN CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message.

Proposal 4: Introduce a separate failure type definition for inter-system inter-RAT HO from NR to E-UTRAN for voice fallback in stage 2.

Reference

[1] Chairman notes for RAN3#117-e meeting

[2] Chairman notes for RAN3#117bis-e meeting

Annex: TP for TS38.300

15.5.2.2.x	Connection failure due to inter-system inter-RAT HO from NR to E-UTRAN for voice fallback

One of the functions of Mobility Robustness Optimization is to detect connection failures that occurred due to inter-system inter-RAT HO from NR to E-UTRAN for voice fallback. The problem is defined as follow:

-	A failure (HOF/RLF) occurs during the handover procedure from a cell belonging to an NG-RAN node to a cell belonging to an E-UTRAN node, the UE tries RRC connection setup procedure for the voice service in a cell belonging to an E-UTRAN node; or a failure (HOF) occurs during the handover procedure from a cell belonging to an NG-RAN node to a cell belonging to an E-UTRAN node, the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in NG-RAN.

Detection mechanism

A failure indication may be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

In case the last serving node is an NG-RAN node, the detection mechanisms for inter-system inter-RAT HO failure from NR to E-UTRAN for voice fallback is carried out through the following:

-	There is a recent handover procedure from a cell belonging to an NG-RAN node to a cell belonging to an E-UTRAN node for the UE prior to the connection failure, and, the successful re-connect cell is a cell belonging to an E-UTRAN node where the UE attempts a RRC connection setup procedure, or the re-establishment attempt cell is a cell belonging to an NG-RAN node where the UE attempts a RRC re-establishment procedure.

An indication on whether a failure in a recent handover procedure from a cell belonging to an NG-RAN node to a cell belonging to an E-UTRAN node was triggered by EPS fallback for IMS voice may be included in the RLF Report.

In case the re-establishment attempt cell is a cell belonging to an NG-RAN node where the UE attempts a RRC re-establishment procedure, the UE may make the RLF Report available to an NG-RAN node. The NG-RAN node may forward the information using the FAILURE INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG to the node that served the UE before the connection failure.

In case the re-connect cell is a cell belonging to an E-UTRAN node where the UE attempts a RRC connection setup procedure:

- the UE may make the RLF Report available to an E-UTRAN node. The E-UTRAN node may forward the information by means of the ENB configuration transfer procedure over S1 and Downlink RAN configuration transfer over NG to the node that served the UE before the connection failure;

- or, the UE may make the RLF Report available to an NG-RAN node. The NG-RAN node may forward the information using the FAILURE INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG to the node that served the UE before the connection failure.