3GPP TSG-RAN WG3 Meeting #119	R3-230343

Athens, Greece, 27th February – 3rd March 2023

Title: 	(TP to TS 38.401 & TS 38.470) Support of L1/L2 based inter-cell mobility

Source: 	Lenovo

Agenda item:	14.2

Document Type:	Other

1. Introduction

This document provides the TP to TS 38.401 and TS 38.470 for the Support of L1/L2 based inter-cell mobility.

2. Text Proposal

Annex 1: (TP to TS 38.401) Support of L1/L2 based inter-cell mobility

--------------------------------------------------------- the start of change -------------------------------------------------------------

#### 8.2.1.X	Intra-gNB-DU L1/L2 based inter-cell Mobility

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for L1/L2 based inter-cell mobility. Figure 8.2.1.x-y shows the intra-gNB-DU L1/L2 based inter-cell mobility procedure for intra-NR.

Figure 8.2.1.x-1: Intra-gNB-DU L1/L2 based inter-cell Mobility

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the gNB-DU containing  measurements of neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 inter-cell mobility configuration.

- WA: The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing the target candidate cells.

- WA: If the request for configuring L1/L2 inter-cell mobility is accepted, the gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations of the accepted target candidate cell(s).

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes a generated RRCReconfiguration message with the L1/L2 inter-cell mobility configuration, which may contain a configuration per target candidate (e.g. including the lower layer RRC configurations of the candidate cells for L1/L2 inter-cell mobility. Details are FFS).

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- The UE sends the L1 measurement result to the gNB-DU. The gNB-DU decides to execute L1/L2 inter-cell mobility.

- The gNB-DU sends the L1/L2 inter-cell mobility command to the UE.

- The gNB-DU sends the L1/L2 triggered mobility notification to the gNB-CU to inform the LTM has been triggered. The candidate target cell ID is included.

NOTE: It is up to implementation when to start sending L1/L2 triggered mobility notification to the gNB-CU.

- FFS: How the gNB-DU detects the UE access to the target cell is up to RAN2.

- The gNB-DU indicates the gNB-CU about the UE successful access to the target cell by Access Success message. Th target cell ID is included.

- The gNB-CU may send the UE Context Modification message to the gNB-DU to update or release the resources of prepared cells.

- The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

---------------------------------------------------------Next change -------------------------------------------------------------

#### 8.2.1.Y	Inter-gNB-DU L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for L1/L2 Triggered Mobility. Figure 8.2.1.Y-1 shows the inter-gNB-DU L1/L2 Triggered Mobility procedure for intra-NR.

Figure 8.2.1.Y-1: inter gNB-DU L1/L2 Triggered Mobility

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the source gNB-DU containing  measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 inter-cell mobility configuration.

- The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing the target candidate cells.

- If the candidate gNB-DU decides to accept the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration for the accepted target candidate cell(s).

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU, which includes the generated RRCReconfiguration message with the L1/L2 inter-cell mobility configuration.

- The source gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

- The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via a UE CONTEXT MODIFICATION RESPONSE message.

FFS: whether it is UL RRC MESSAGE TRANSFER message or UE Context Modification Response message.

- The UE sends the L1 measurement result to the source gNB-DU.

- The source gNB-DU decides to execute L1/L2 triggered mobility to a candidate target cell.

FFS: Notifying the LTM cell switch decision to the other nodes as well.

- The source gNB-DU sends L1/L2 inter-cell mobility command to the UE.

Editor’s note: the order of step 10 and 11 is not defined. The cell switch command message needs to be updated according to RAN2’s discussion.

- The source gNB-DU sends the L1/L2 triggered mobility notification to the gNB-CU to inform the LTM has been triggered. The candidate target cell ID is included.

NOTE: It is up to implementation when to start sending L1/L2 triggered mobility notification to the gNB-CU.

FFS:  how the target gNB-DU detects the UE access and whether there is an F1 impact.

FFS: For inter-DU L1/L2 Triggered Mobility, whether and how to release the source cell/prepared cells’ resources in the source gNB-DU is FFS.

--------------------------------------------------------- the end of change -------------------------------------------------------------

Annex 2: (TP to TS 38.470) Support of L1/L2 based inter-cell mobility

3GPP TSG-RAN WG3 Meeting #119	R3-230343

Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.470 | CR | <CR#> | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP to TS 38.470) Support of L1/L2 based inter-cell mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2 |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To introduce L1/L2 based inter-cell mobility. |  |  |  |  |  |  |  |  |
| Summary of change: |  | L1/L2 based inter-cell mobility is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | L1/L2 based inter-cell mobility is not supported in case of split gNB architeture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.2.3	F1 UE context management function

The F1 UE context management function supports the establishment and modification of the necessary overall UE context.

The establishment of the F1 UE context is initiated by the gNB-CU and accepted or rejected by the gNB-DU based on admission control criteria (e.g., resource not available).

The modification of the F1 UE context can be initiated by either gNB-CU or gNB-DU. The receiving node can accept or reject the modification. The F1 UE context management function also supports the release of the context previously established in the gNB-DU. The release of the context is triggered by the gNB-CU either directly or following a request received from the gNB-DU. The gNB-CU request the gNB-DU to release the UE Context when the UE enters RRC_IDLE or RRC_INACTIVE. In case of configured grant small data transmission, this function allows the gNB-CU to request the gNB-DU to keep part of the UE context, including at least the lower layer configuration for SDT DRB(s)/SRB(s), C-RNTI, CS-RNTI, F1AP association, and F1 tunnel information, when the gNB-CU sends the UE into RRC_INACTIVE state. In case of positioning when the gNB-CU sends the UE into RRC_INACTIVE state, this function allows the gNB-CU to request the gNB-DU to keep part of the positioning context, including the SRS configuration for the UE.

This function can be also used to manage DRBs, SRBs and SL DRBs, i.e., establishing, modifying and releasing DRB, SRB and SL DRB resources. The establishment and modification of DRB, or SL DRB resources are triggered by the gNB-CU and accepted/rejected by the gNB-DU based on resource reservation information and QoS information to be provided to the gNB-DU. For each DRB to be setup or modified, the S-NSSAI may be provided by gNB-CU to the gNB-DU in the UE Context Setup procedure and the UE Context Modification procedure. In case of NG-RAN sharing, the gNB-CU includes the serving PLMN ID (for SNPNs the serving SNPN ID) in the UE Context Setup procedure.

This function is also used to manage a UE's configuration with multicast resources and to provide information about the multicast MRBs which are established for the multicast MBS Sessions the UE has joined.

For Uu, the mapping between QoS flows and radio bearers is performed by gNB-CU and the granularity of bearer related management over F1 is radio bearer level. For NG-RAN, the gNB-CU decides an aggregated DRB QoS profile for each radio bearer based on received QoS flow profile, and provides both aggregated DRB QoS profile and QoS flow profile to the gNB-DU, and the gNB-DU either accepts the request or rejects it with appropriate cause value. With this function, gNB-DU could also notify gNB-CU whether the QoS for already established DRBs is not fulfilled any longer or it is fulfilled again. The function can be also used to inform the gNB-DU the alternative QoS Parameters Sets when available for a QoS flow. To support packet duplication for intra-gNB-DU CA as described in TS 38.300 [8], one data radio bearer should be configured with at least two GTP-U tunnels between gNB-CU and a gNB-DU.

For SL, the mapping between QoS flows and radio bearers is performed by gNB-CU and the granularity of bearer related management over F1 is radio bearer level. For NG-RAN, the gNB-CU decides an aggregated SL DRB QoS profile for each radio bearer based on received QoS flow profile, and provides both aggregated SL DRB QoS profile and QoS flow profile to the gNB-DU, and the gNB-DU either accepts the request or rejects it with appropriate cause value.

With this function, gNB-CU requests the gNB-DU to setup or change of the SpCell (as defined in TS 38.321 [10]) for the UE, and the gNB-DU either accepts or rejects the request with appropriate cause value. This function also enables the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional mobility.

With this function, the gNB-CU requests the setup of the SCell(s) at the gNB-DU side, and the gNB-DU accepts all, some or none of the SCell(s) and replies to the gNB-CU. The gNB-CU requests the removal of the SCell(s) for the UE.

With this function, the gNB-CU indicates the UL UE AMBR limit to the gNB-DU, and the gNB-DU enforces the indicated limit.

With this function, the gNB-DU indicates that a bearer, or a UE is inactive or active. The gNB-CU consolidates all the serving gNB-DUs for the UE and takes further action.

With this function, the gNB-CU indicates the gNB-DU that the UE context concerns mobility enhancement operation, and the gNB-DU takes corresponding actions.

With this function, the gNB-CU provides suggested candidate cells to enable the gNB-DU to determine the candidate cells configuration used for LTM. This function also enables the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during LTM.

In addition, for IAB-nodes and IAB-donors:

-	The F1 UE context management function is used for managing BH RLC channels, i.e. establishing, modifying and releasing BH RLC channel resources. The establishment of BH RLC channels is triggered by the IAB-donor-CU. The establishment and modification is accepted/rejected by the IAB-node’s parent, based on e.g. resource reservation information and QoS information provided to the IAB-node’s parent.

-	The DRB QoS profile framework is reused for BH RLC channels carrying DRBs. Prioritization of traffic on the F1-C interface is based on traffic type (e.g. UE-associated F1AP signalling, non-UE-associated F1AP signalling) and is enforced in the IAB-donor-DU and in IAB-nodes, considering that the traffic on the F1-C interface has higher priority than other traffic; in-sequence delivery over the signaling connection is always ensured.

-	The IAB-donor-CU associates each BH RLC channel carrying control plane traffic with one of the signaled control plane traffic type values.

For L2 U2N Relay:

-	The F1 UE context management function is used for managing Uu Relay RLC channels and PC5 Relay RLC channels for L2 U2N relay, i.e. establishing, modifying and releasing Uu Relay RLC channel and PC5 Relay RLC channel resources. The establishment of Uu Relay RLC channels and PC5 Relay RLC channels are triggered by the gNB-CU. The establishment and modification is accepted/rejected by the gNB-DU based on resource reservation information and QoS information provided to the gNB-DU. The modification of Uu/PC5 Relay RLC channels can be triggered by the gNB-CU or the gNB-DU.

-	The gNB-CU determines the QoS for the PC5 Relay RLC channel and the QoS for the Uu Relay RLC channel based on the received QoS profile for the L2 U2N Remote UE, and provides the QoS information to the gNB-DU.

-	The gNB-CU configures the gNB-DU about the SRB(s)/DRB(s) to Uu Relay RLC channel(s) mapping, which is used by the gNB-DU to perform data transfer of L2 U2N Remote UE. The mapping between DRB and Uu Relay RLC channel is configured at the granularity of GTP-U tunnel.

-	The gNB-CU is responsible for the local ID allocation and update for L2 U2N Remote UE.